3GPP TSG RAN WG1 #106-e	R1- 2107743

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.5.4

Source:	Apple Inc.

Title:	Views on Rel-17 positioning latency reduction

Document for:	Discussion, Decision

# Introduction

In 3GPP RAN#90-e, a work item for Rel-17 NR positioning enhancements was approved and was updated in RAN#91-e [1]. One of the objectives of Rel-17 positioning WI, is to specify solutions to reduce positioning latency, as it can be found below [1]:

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

In this contribution, we present our views on potential techniques and required signalling to reduce NR positioning latency.

# Latency Reduction for NR Positioning

## PRS measurements and MG-less/based enhancements

In Rel-16 specification, the UE is not expected to perform DL PRS positioning measurements outside the measurement gap (MG). On the other hand, MG configuration needs the exchange of couple of RRC messages between UE and gNB, where each message is in the order of 10ms. Indeed, within the study item phase, couple of companies analysed and reported the required time for MG configuration to be more than 20ms. In addition, after UE is configured with MG, misalignment between PRS configurations and MG offset/duration, is another source of latency in current specification. Thus, optimizing MG and/or required time for UE to start measurements is essential to reduce positioning latency.

To reduce latency, PRS measurement without/outside MG configuration was discussed in RAN1#105e, where the following agreement was made [2]:

Agreement:

- Further study the following options (with the same numerology) to support PRS measurement without MGs for latency reduction in Rel-17

  - Option 1: The PRS is from the serving cell and UE measurement is inside the active DL BWP

  - Option 2: The PRS can be from the serving cell and non-serving cell, and UE measurement is inside the active DL BWP

  - Option 3: The PRS (from the serving cell or non-serving cell) used for UE measurement may extend outside or be completely outside the active DL BWP (including with potentially a different numerology)

  - Note: RAN1 strives not to increase the PRS measurement time compared with Rel-16 MG-based measurement

- The following aspects are FFS

  - PRS processing prioritization window

  - Mechanism to trigger UE DL PRS measurements and report

  - UE/gNB assumptions on processing of DL PRS and other DL physical channels / signals

  - UE DL PRS processing capabilities

- Note: Companies are encouraged to compare the latency benefits of introducing MG-less PRS measurements over MG-based PRS measurements

- Note: Depending on the comparison of latency benefits (and other considerations such as complexity) between introducing MG-less PRS measurements and MG-based PRS measurements, none/one/multiple of the above options should be adopted in Rel-17.

In our view, that the purpose of the above agreement is to replace MG-less to MG-based PRS measurements, with similar functionalities already defined for MG-based PRS measurements. One of those features is UE within MG may receive PRS from serving and/or neighbouring gNBs. Thus, we propose to support a similar feature for MG-less measurements, i.e., Option 2.

Proposal 1: For PRS measurement without/outside MGs, support subject to UE capability, the PRS from the serving cell and non-serving cell can be received and measured within UE’s active DL BWP.

With the above agreement, also as mentioned by some companies in RAN1#105-e, the missing part is related to higher level coordination between different TRPs which is needed to allow (at least the intersection of) PRS reception from serving and non-serving cells, inside UE’s active BWP. Our solution is a special, coordinated among serving and non-serving cells, BWP for PRS measurement where UE is indicated to switch to in order to perform PRS reception and measurements from serving and non-serving cells. Based on current specification for BWP switching, if the switching indication is DCI-based, BWP switching will take up to 3ms. That in comparison with more than 20ms of MG configuration is a big reduction of positioning latency. To be more specific, switching to M-BWP can be dynamically indicated to UE by UE-specific DCI or GC-DCI, or UE can be indicated a periodic switch to BWP. For the latter case, M-BWP can be RRC indicated by gNB or it can be configured by LMF on a LPP message. Based on what we discussed, we make the following proposal:

Proposal 2: support under UE capability an indication to switch to a BWP associated with positioning measurements, by

- Alt1: UE-specific DCI

- Alt2: GC-DCI

- Alt3: In a periodic higher layer configured by LMF

Next aspect related to M-BWP switching is about how long UE will stay in the M-BWP. Few options are listed below.

Proposal 3: M-BWP configuration may include the time duration which M-BWP will last

- In this case, once the time is expired, UE would switch to a default BWP or back to the active BWP before switching to M-BWP

- Alternatively, UE would stay in M-BWP until further indication to switch to another (regular) BWP is received

Given that M-BWP is anyway using one of BWP-Ids, it is reasonable to question when UE is indicated to switch to M-BWP, whether or not it is expected to receive signals other than PRS. We should note that reception of other signals and channels within M-BWP, requires more specification effort. For example, priority rules may need to be defined in case of PRS is collided with other channels/signals within the M-BWP. Additionally, UE capability numbers may need to be redefined for measurements within M-BWP, given that in current specification UE’s positioning capability numbers are determined under the assumption that no other channel/signal besides PRS is processed within the PRS measurement and processing window.

Based on what we discussed, we have the following proposal:

Proposal 4: For PRS measurement outside/without MG, e.g. once UE receives the indication to switch to Measurement BWP (M-BWP), UE is not expected to receive or transmit data within the M-BWP

In addition to MG-less enhancements, RAN1 also discussed in 105-e proposals on latency reduction for MG-based PRS measurements, where the following was agreed [2]:

Agreement:

RAN1 to further study at least the following aspects for MG enhancement with regards to MG requesting and configuration/activation/triggering for the purpose of latency reduction for positioning:

- Preconfiguration of multiple MGs

- Triggering/activation of MG(s) with lower layer signalings (DCI or DL MAC CE)

- Request of MG(s) with lower layer signaling by the UE to the gNB

- Request/determination of MG(s) by LMF indication to the gNB/UE

- Note: The combination of the above items is possible.

In our view, MG-based positioning with some lower layer signaling to reduce latency should be coupled with A/SP-PRS configuration/demand/activation. Otherwise, the misalignment between MG and PRS reception is a well-known source of latency, as observed within the SI phase. While with joint configuration of MGs and PRSs, where MG is tied with A-PRSs associated with a MG, such misalignment issue is resolved. We have the following proposal:

Proposal 5: NW provides assistance data to UE based on which UE is configured with one or more MG configurations and A-PRSs associated with each MG.

- A MG and PRS resources associated with that MG may be triggered/activated by UE specific DCI, or GC-DCI or MAC-CE signaling

With A/SP-PRS and regular P-PRS reception colliding within the MG, there should be some prioritization rules defined to specify which PRS UE is expected to measure and process. Given that A/SP-PRS comes with dynamic or MAC-CE indication, A-PRS should be higher priority than SP-PRS, itself higher priority than P-PRS.

Proposal 6: In case of A-PRS, triggered by lower layer signaling, collides with a SP/P-PRS, SP/P-PRS is fully or partially (only overlapping symbols) cancelled.

## Latency reduction for UE to report positioning measurement

Another source of positioning latency corresponds to the time that UE needs to be granted with an UL resource to transmit PUSCH, which carries positioning measurements. At worst, UE may need to send SR, gNB receive and process the SR, next gNB sends a PDCCH to schedule UE with a dynamic PUSCH. For some scenarios, the PUSCH resource to report PRS measurement can be associated with other signallings, so no further indication from network is needed. An example is once UE is indicated to M-BWP, the M-BWP configuration includes PUSCH resource determination, which is tied with the end of M-BWP with a configured Nx symbols from the last PRS symbol within M-BWP, as shown Figure 1, or Ny symbols after the end of M-BWP. Indeed, the matter was discussed in RAN1#105-e, where some companies were against to define a prescheduled UL resources to carry the LPP measurement report [2]. While the benefit of similar enhancements in cutting the SR/BSR and reducing latency is already observed by RAN1 within the SI phase, some opponents of such enhancements think that the feature should be left to RAN2. Here it should be noted that RAN1 is also an assigned WG to this objective. Besides, RAN1 shall discuss phy-layer aspects related to this feature, like minimum number of symbols, based on UE capability, between the last PRS symbol and PUSCH grant, etc.

Based on what we discussed above, we have the following proposal:

Proposal 7: At least for the case of M-BWP switching, NW configures (as part of M-BWP configuration and/or indication) PUSCH resource for UE to report positioning measurements and/or location information

- The grant is specifically configured for positioning measurement report, e.g. Nx symbols after the end of last symbol of last DL-PRS resource, or after the end of M-BWP

- Nx is determined based on UE capability

Figure 1: PUSCH resource indication for UE to report positioning measurement, as part of signaling indicating M-BWP configuration

# Conclusions

In this contribution, we provided our thoughts on potential techniques to reduce the positioning latency for the NR positioning techniques. Based on views shared in this paper, we make the following proposals:

Proposal 1: For PRS measurement without/outside MGs, support subject to UE capability, the PRS from the serving cell and non-serving cell can be received and measured within UE’s active DL BWP.

Proposal 2: support under UE capability an indication to switch to a BWP associated with positioning measurements, by

- Alt1: UE-specific DCI

- Alt2: GC-DCI

- Alt3: In a periodic higher layer configured by LMF

Proposal 3: M-BWP configuration may include the time duration which M-BWP will last

- In this case, once the time is expired, UE would switch to a default BWP or back to the active BWP before switching to M-BWP

- Alternatively, UE would stay in M-BWP until further indication to switch to another (regular) BWP is received

Proposal 4: For PRS measurement outside/without MG, e.g. once UE receives the indication to switch to Measurement BWP (M-BWP), UE is not expected to receive or transmit data within the M-BWP

Proposal 5: NW provides assistance data to UE based on which UE is configured with one or more MG configurations and A-PRSs associated with each MG.

- A MG and PRS resources associated with that MG may be triggered/activated by UE specific DCI, or GC-DCI or MAC-CE signaling

Proposal 6: In case of A-PRS, triggered by lower layer signaling, collides with a SP/P-PRS, SP/P-PRS is fully or partially (only overlapping symbols) cancelled.

Proposal 7: At least for the case of M-BWP switching, NW configures (as part of M-BWP configuration and/or indication) PUSCH resource for UE to report positioning measurements and/or location information

- The grant is specifically configured for positioning measurement report, e.g. Nx symbols after the end of last symbol of last DL-PRS resource, or after the end of M-BWP

- Nx is determined based on UE capability

# References

- RP-210903, “New WID on NR Positioning Enhancements”, March 2021

- R1-2106334, FL summary #4 of 8.5.4 latency improvements for DL and DL+UL methods, Huawei