3GPP TSG-RAN WG1 #106-e		R1-2107745

e-Meeting, August 16th – 27th, 2021

Source: 	Apple Inc.

Title:                     Reduced maximum UE bandwidth for Redcap

Agenda item:	8.6.1.1

Document for:	Discussion and Decision

# 1 Introduction

In RAN1 104-bis e-meeting, the following agreements were reached [1]:

Working assumption: - During initial access, the bandwidth of the initial DL BWP for RedCap UEs is not expected to exceed the maximum RedCap UE bandwidth. - The bandwidth and location of the initial DL BWP for RedCap UEs can be the same as the bandwidth and location of the MIB-configured initial DL BWP for non-RedCap UEs. - This does not preclude a SIB-configured initial DL BWP for non-RedCap UEs only with a wider bandwidth than the maximum RedCap UE bandwidth. - This does not preclude separate or additional bandwidth and location for initial DL BWP for RedCap UEs (FFS). Working assumption: After initial access, at least for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth. - FFS: BWP#0 configuration option 2 (as in 38.331, Appendix B2)

In RAN1 #105 e-meeting, further progress was made with the following agreements:

Agreements: Replace the RAN1#104bis-e working assumption with the following working assumption (for option 1) and working assumption (for option 2): - Working assumption: After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth. - Working assumption: After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 2 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth. Agreements: - Both during and after initial access, the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the maximum RedCap UE bandwidth is allowed. - Working assumption: Both during and after initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - FFS: whether/how to avoid or minimize PUSCH resource fragmentation due to PUCCH transmission for the above case - Support the case when the centre frequency is assumed to be the same for the initial DL and UL BWPs in TDD. - FFS whether or not to additionally support the case when the centre frequency is different; if so, how to minimize centre frequency retuning Agreement: Take the following as an agreement, revised from the RAN1#104bis-e working assumption: - A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE. - At least for FR1, FG 6-1 (“Basic BWP operation with restriction” as described in TR 38.822) is used as a starting point for the mandatory RedCap UE type capability. - This does not preclude support of FG 6-1a (“BWP operation without restriction on BW of BWP(s)” as described in TR 38.822) as a UE capability for RedCap UEs. Working assumption: Both during and after initial access, even for the scenario where the initial UL BWP for non-RedCap UEs is not configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP can optionally be configured/defined for RedCap UEs. - RO sharing between RedCap and non-RedCap is not precluded. Working assumption: For enabling/supporting that the RACH occasion (RO) associated with the best SSB falls within the RedCap UE bandwidth, support separate initial UL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth), and this separate initial UL BWP for RedCap includes ROs for RedCap UEs. - Note: these ROs can be dedicated for RedCap UEs or shared with non-RedCap UEs. Working assumption: - For enabling/supporting that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access, support separate initial UL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth). - FFS: whether/how the specification also supports separate PUCCH/Msg3/[MsgA] PUSCH configuration/indication or a different interpretation of the same configuration/indication for RedCap (e.g., disabled frequency hopping or different frequency hopping) Working assumption: At least for TDD, an initial DL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth) can be optionally configured/defined separately from the initial DL BWP for non-RedCap UEs at least after initial access - FFS the details of the configuration/definition - The configuration for a separately configured initial DL BWP for RedCap UEs is signaled in SIB. - whether to support that separate initial DL BWP for RedCap UEs can include a configuration of CORESET and CSS(s) - whether part of the configuration can be defined instead of signaled - If a separate initial DL BWP for RedCap UEs is configured/defined, this separate initial DL BWP for RedCap UEs can be used at least after initial access (i.e., at least after RRC Setup, RRC Resume, or RRC Reestablishment). - FFS during the initial access - FFS: whether a separately configured initial DL BWP for RedCap UEs needs to contain the entire CORESET #0, and, if not, the Redcap UE behaviour for CORESET #0 monitoring - FFS: supported bandwidths in the separate initial DL BWP - FFS: whether additional SSB is transmitted in the separately configured initial DL BWP for RedCap UEs - FFS: FDD case

In this contribution, we discuss remaining issues related to operate Redcap device with a reduced BW compared to normal devices according to the RAN1 104bis and RAN1 105e agreements.

# 2. Discussions

## 2.1 Initial DL BWP

In RAN1 104 bis and 105 e-meetings, it was extensively discussed whether a Redcap UE is allowed to operate with an initial BWP wider than the maximum Redcap UE bandwidth during and after initial access. Three working assumptions were made for initial DL BWP (during initial access and after initial access with BWP#0 configuration option 1 and Option 2). These working assumptions should be confirmed such that the unnecessary RF retuning operation across consecutive transmission/receptions can be mitigated for Redcap UEs, especially after initial access.  We therefore propose the following:

Proposal 1: Confirm the following working assumptions for initial DL BWP Operation for Redcap UEs:

- During initial access, the bandwidth of the initial DL BWP for RedCap UEs is not expected to exceed the maximum RedCap UE bandwidth.

  - The bandwidth and location of the initial DL BWP for RedCap UEs can be the same as the bandwidth and location of the MIB-configured initial DL BWP for non-RedCap UEs.

  - This does not preclude a SIB-configured initial DL BWP for non-RedCap UEs only with a wider bandwidth than the maximum RedCap UE bandwidth.

- After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

- After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 2 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

In Rel-15/16, CORESET#0 provided by MIB serves as default initial DL BWP, which has the BW below maximum BW of Redcap UEs. To avoid unnecessary signaling overhead in SIB1, CORESET#0 can be inferred by Redcap UE as initial DL BWP if the SIB-configured initial DL BWP has a wider bandwidth than the maximum Redcap UE bandwidth and no dedicated initial DL BWP is configured in SIB1.

Proposal 2:

- Separate or additional initial DL BWP for Redcap UEs can be configured by SIB1.

- If SIB-configured initial DL BWP has a wider bandwidth than the maximum Redcap UE bandwidth and additional initial DL for Redcap UEs is not configured, a Redcap UE derives initial DL BWP corresponding to CORESET #0.

## 2.2 Initial UL BWP

For initial UL BWP, a few work assumptions was made to allow configuring initial UL BWP to be wider than the maximum RedCap UE bandwidth for non-Redcap UEs. For this case, a separate initial UL BWP no wider than the RedCap UE maximum bandwidth should be configured/defined for RedCap UEs such that the PUCCH and PUSCH with frequency hopping as well as FDMed ROs always fall within the Redcap UE BW and the RF retuning is avoided.

Proposal 3: Confirm the working assumption for initial UL BWP that

- Both during and after initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs.

  - Support the case when the centre frequency is assumed to be the same for the initial DL and UL BWPs in TDD.

One FFS aspect related to separate UL BWP with smaller BW is whether/how to avoid or minimize PUSCH resource fragmentation due to the PUCCH transmission. The PUSCH resource fragmentation issue is more pronounced during initial access phase in which the PUCCH frequency hopping is always enabled according to the current specification to improve the decoding performance of HARQ-ACK for the Msg4 reception. This PUSCH fragmentation problem can be minimized by placing PUCCH resource at one edge of non-Redcap initial UL BWP with disabling the frequency hopping.

Proposal 4: Disabling the PUCCH frequency hopping in a separate initial UL BWP for Redcap UE can be considered to address the PUSCH resource fragmentation.

Another related FFS aspect for TDD system is whether to additionally support the case that the centre frequencies of initial DL and UL BWP are different for TDD system; if so, how to minimize centre frequency retuning. In general, support different centre frequencies for DL and UL BWP in TDD system should be avoided since it potentially requires a Redcap UE to implement two local oscillators (LOs) and therefore increases the UE complexity and cost, which is contradicted with overall design target. In addition, it causes frequent RF retuning between DL and UL BWP and results in significantly increased latency (e.g., at least 3ms for BWP switching as illustrated in FIG.1 assuming 15kHz BWP SCS), huge power consumption and difficult to support latency-sensitive traffic for wearable devices. We therefore propose the following:

Proposal 5: For Redcap device in TDD system, NOT support the case that the centre frequencies for DL and UL BWP are different, including both initial DL/UL BWP and non-initial DL/UL BWP.

Figure 1: Problem of different central frequency for initial DL/UL BWP for Redcap UE in TDD

For TDD system, it is essential to have a seperate initial DL BWP that maintains a same central frequency as the cell-edge initial UL BWP. As one consequence, the cell-edge initial DL BWP, as illustrated in FIG.2, may not include the bandwidth of CORESET #0 and SSB. In Rel-15/16, BWP configuration without SSB/COREST#0 (i.e., FG 6-1A) is optionally supported due to additional complexities for intra-frequency RRM measurement, RLM and Time/Frequency tracking. Enabling the Redcap UE to operate with cell-edge initial DL BWP without SSB and CORESET#0 essentially makes FG 6-1A to be mandatory feature for Redcap UEs, which causes additional power consumption and throughput degradation. These issues should be addressed first. One possible solution is to transmit additional SSB or CSI-RS/TRS in the cell-edge initial DL BWP, as shown in FIG.2, to avoid RF retuning to perform Time/Frequency tracking, intra-frequency and inter-frequency RRM measurements as well as mitigating throughput loss caused by RF retuning. The additional SSB in a separate Redcap DL BWP maybe placed on a non-synchronization raster such that potential impacts on legacy UE can be eliminated.  It should be noted that a measurement gap is required for SSB-based RRM measurement if SSB of target cell is not in active BWP, see FIG.2, which leads to throughput loss and increased power consumption for Redcap UEs.

Figure 2: Separate Initial DL BWP for Redcap UEs with additional SSB

Proposal 6: Additional SSB should be transmitted in a DL BWP if it does not include the cell-defining SSB (CD-SSB).

Proposal 7: For SSB-based RRM measurement, a measurement gap is required if the SSB of target cell is not in active BWP.

## 2.3 Other considerations for separate Initial DL BWP

Based on agreement made in RAN1 105 e-meeting, one more FFS aspect is whether to additionally support configurating CORESET and CSS(s) for the separate initial DL BWP. Configuring additional CORESET and CSS, as illustrated in FIG.3, can solve the potential congestion problem caused by the limited bandwidth size of CORESET#0 if a large number of UEs generates a considerable amount of traffic during the initial/random access procedures, mainly including Msg2/Msg4/Paging and scheduling DCIs. For TDD system with central-frequency-aligned initial DL/UL BWP, it avoids unnecessary RF retuning to receive RAR/MsgB during initial access and reduces power consumption.

Figure 3: Separate Initial DL BWP for Redcap UEs with additional CORESETS for offloading purpose

Proposal 8: For a Redcap-dedicated initial DL BWP, additional CORESETs can be configured and used for scheduling Msg2/4, paging and SIB messages for Redcap UEs before RRC connection completion.

# 3. Conclusion

In this contribution, we have presented our views on the reduced maximum UE bandwidth for Redcap. Based on the discussions, we proposed the following:

Proposal 1: Confirm the following working assumptions for initial DL BWP Operation for Redcap UEs:

- During initial access, the bandwidth of the initial DL BWP for RedCap UEs is not expected to exceed the maximum RedCap UE bandwidth.

  - The bandwidth and location of the initial DL BWP for RedCap UEs can be the same as the bandwidth and location of the MIB-configured initial DL BWP for non-RedCap UEs.

  - This does not preclude a SIB-configured initial DL BWP for non-RedCap UEs only with a wider bandwidth than the maximum RedCap UE bandwidth.

- After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

- After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 2 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

Proposal 2:

- Separate or additional initial DL BWP for Redcap UEs can be configured by SIB1.

- If SIB-configured initial DL BWP has a wider bandwidth than the maximum Redcap UE bandwidth and additional initial DL for Redcap UEs is not configured, a Redcap UE derives initial DL BWP corresponding to CORESET #0.

Proposal 3: Confirm the working assumption for initial UL BWP that

- Both during and after initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs.

  - Support the case when the centre frequency is assumed to be the same for the initial DL and UL BWPs in TDD.

Proposal 4: Disabling the PUCCH frequency hopping in a separate initial UL BWP for Redcap UE can be considered to address the PUSCH resource fragmentation.

Proposal 5: For Redcap device in TDD system, NOT support the case that the centre frequencies for DL and UL BWP are different, including both initial DL/UL BWP and non-initial DL/UL BWP.

Proposal 6: Additional SSB should be transmitted in a DL BWP if it does not include the cell-defining SSB (CD-SSB).

Proposal 7: For SSB-based RRM measurement, a measurement gap is required if the SSB of target cell is not in active BWP.

Proposal 8: For a Redcap-dedicated initial DL BWP, additional CORESETs can be configured and used for scheduling Msg2/4, paging and SIB messages for Redcap UEs before RRC connection completion.

# References

- RAN1 104bis e-meeting Chairman notes

- RAN1 105 e-meeting Chairman notes