3GPP TSG RAN WG1 #106-e		R1-2107750

e-Meeting, August 16th – August 27th, 2021

Agenda Item:	8.7.1.1

Source:	Apple Inc.

Title:	Paging enhancements for idle/inactive-mode UE

Document for:	Discussion/Decision

# Introduction

A work item on UE Power Saving Enhancements for NR was approved in RAN#86 (Dec 2019, RP-193239), and the WID was further updated in RP-200938 (RAN#88, June 2020) [1]. One objective is about paging enhancements for idle/inactive-mode UEs:

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1]

  - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1]

- NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting

In RAN1#103-e, it was agreed to support paging early indication. In RAN1#104b-e, some observations were made regarding the different options on the coexistence, performance, and overhead.

Agreements: For NR idle/inactive-mode paging enhancement, paging early indication before paging occasion is supported from RAN1 perspective

- FFS: Physical layer design based on DCI, SSS or TRS/CSI-RS

In this contribution, we provide our views on the detailed design of paging early indication.

# Paging Early Indication Design

## Physical channel/signal for PEI

Regarding the different channel/signal to carry PEI, the high-level comparison is provided in Table 1.

Table 1 Comparison between PDCCH-based and sequence-based PEI design

|  | PDCCH | Sequence-based (TRS, CSI-RS, SSS-like) |
| --- | --- | --- |
| Resource overhead | AL 4 = 288 REs AL 8 = 576 REs AL 16 = 1152 REs | TRS: 48 RBs x 6 = 288 REs (2 symbols in 1 slot) SSS: 127 REs with 1 symbol (or 254 REs with 2 symbols) |
| Decoding/Detection Performance | More sensitive to frequency offset | Less sensitive to frequency offset compared to PDCCH |
| Ability to carry additional info | Easy to carry other info, such as sub-group indication, SI update, ETWS notification. | More challenging to carry additional info, would need to use multiple sequences. |
| Power consumption |  | Using multiple symbols can assist in frequency sync, which saves power with less SSB detection. |

Power saving comparison

The power saving gain is one of the most important metrics. Comparing PDCCH-based and sequence-based solutions, sequence-based solution can potentially achieve more power saving due to the following two reasons:

- Sequence detection itself is easier compared to PDCCH decoding, which should consume less power in baseband processing.

- If the sequence is further used for time/frequency tracking, it reduces the number of SSBs needed to achieve the required time/frequency synchronization accuracy. This also saves UE power.

There is no agreed power modeling for sequence-based solution. Companies generally assume that PDCCH and sequence consume the same power, which may not reflect the reality. Moreover, given that we currently assume the power consumption of 45 for micro-sleep and 50 for PDCCH-only (the difference could be rather small compared to the real UE power consumption), there is not much room in between even if we introduce additional assumption for sequence-based solution to account for the fact that it consumes less power than PDCCH. This in some sense is the shortcoming of the agreed power model, and it does not allow the sequence-based solution to be evaluated properly.

However, if we check the break-down of the UE power consumption when PEI is used, it is seen that the PEI detection itself only occupies a very small percentage of the total UE power consumption. Fig. 1 and Fig. 2 provides the break-down for low SINR case (3 SSBs before PO, with intra- and inter-freq RRM measurement) and high SINR case (1 SSB before PO, without intra- or inter-freq RRM measurement), respectively. The group paging rate is 10%. For the low SINR case, PEI (PDCCH) consumes ~3% of total power; while for the high SINR case, PEI (PDCCH) consumes ~6% of total power. This means that even if the UE can save some power by using sequence-based PEI instead of PDCCH-based PEI, the overall power saving is very limited.

Figure 1 UE power consumption break-down for low SINR case with PEI

Figure 2 UE power consumption break-down for high SINR case with PEI

Observation 1: The power consumption for PEI detection is 3~6% of total UE power consumption in idle/inactive state. This means that the overall power saving in PEI detection from different PEI designs is very small.

Moreover, we evaluate the impact of PEI-assisted time/frequency tracking (e.g. if TRS-like sequence or two-symbol SSS-like sequence is used) on UE power consumption. For simplicity, it is assumed that the power consumption in the slot for sequence-based PEI detection is the same as that for the PDCCH-only slot. For low SINR case, the baseline PEI case (i.e., PEI is not used for time/frequency tracking, and the UE still needs to monitor 3 SSBs for PO reception) provides 26.3% UE power saving gain. If we assume PEI can replace one SSB for time/frequency tracking (i.e., the UE only needs to monitor two SSBs plus PEI for PO reception), the UE power saving gain is 27.9%. The difference is only 1.6%, which is small. A side effect is that the sequence detection is not 100% reliable, and false alarm event may cause the tracking loop at the UE to be adjusted based on wrong signal, which will take some time to recover.

Observation 2: For low SINR case, if sequence-based PEI can replace one SSB for time/frequency synchronization purpose, it provides an additional saving of 1.6% compared to PDCCH-based PEI.

Putting these two aspects together, even though using sequence-based PEI may provide additional power saving compared to PDCCH-based PEI, the difference appears to be small. So this should not be considered as the main reason to favor one solution vs another.

Overhead comparison

If we assume total system bandwidth of 100 MHz, the worst case that there are 4 POs in every paging frame (10ms), 10% group paging rate with Behv-A, and PEI using PDCCH AL 8 (576 REs), the PEI overhead for the system is ~0.05%. Note that this is a very extreme case. From the calculation, we can see that the overhead is quite small.

In addition, if we compare PDCCH-based vs sequence-based solution, the overhead difference should be quite minor when similar performance is considered.

From the perspective of multiplexing with PDSCH, there are existing mechanisms to provide either semi-static or dynamic rate matching around CORESET or TRS/CSI-RS resources, which also depends on UE capabilities. For SSS-based solution, as discussed in RAN1#104b-e, there could also be ways to support multiplexing, e.g. by using CORESET to cover the REs for SSS, but this could be more complicated and less efficient.

In any case, we do not see the overhead as the main decision factor to choose between the two types of solutions.

In terms of performance, it is a tradeoff with resource overhead. Given sufficient number of REs (e.g. AL4 for PDCCH, 2-symbol SSS, or 48-RB TRS, all with similar overhead), all these options should be able to provide sufficient reliability not to impact overall paging performance. This can be seen from the observations agreed in RAN1#104b-e.

However, PDCCH-based solution has advantages in some other aspects:

- Relatively speaking, it has less specification impact, and less gNB/UE implementation impact.

- The multiplexing with legacy signals is easier to handle from gNB implementation point of view, because it can be handled in the same way as legacy PDCCH.

- It is easier to use PDCCH to carry other information, such as sub-grouping information, the TRS/CSI-RS availability indication.

Therefore, we think PDCCH-based solution should be adopted.

Proposal 1: Use PDCCH to carry paging early indication.

## Behavior A vs. B

The following was agreed in RAN1#104-e:

Agreements:

For the evaluation and comparison of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, the following are assumed:

- Behv-A:

  - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged

  - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

- Behv-B:

  - PEI indicates whether or not UE should monitor a PO

  - UE is required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

Related to the consideration between Behv-A vs Behv-B, one question is whether we should use an on-off design (transmitted only if it is positive) or expect the PEI (positive or negative) to be always transmitted (at least in the normal situation).

Using on-off design means that PEI is required to be transmitted only if there is paging. Given that the typical group paging rate is low (e.g. 10% as assumed in the evaluation), PEI is transmitted with a small probability in typical cases. Therefore, the on-off design avoids unnecessary negative PEI transmission when comparing to the always-transmitted signal. If the on-off design is used, it is obvious that it can only work together with Behv-A, not Behv-B.

It has been pointed that Behv-B has the advantage that the network has the flexibility not to transmit PEI without impacting paging performance. However, this comes at the cost of more UE power consumption with decoding PEI. So instead of saving UE power, the UE may end up burning more power, which is completely against the motivation of the feature.

As the calculation showed above, the overhead is rather small, so it should not have much impact to require the gNB to always transmit positive PEI.

Proposal 2: Adopt Behv-A, i.e.,

- PEI indicates UE should monitor a PO if UE’s group/subgroup is paged.

- UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO.

## PO configuration

If we follow the legacy approach to determine a UE’s PO, it is natural that in each PO we will see a mix of legacy UEs and new UEs. This works but we think there is some benefit to support separate POs for legacy UEs and new UEs. It means that in each PO, there are only legacy UEs or only new UEs. The potential advantages include:

- If we consider the support of sub-grouping, the idea is that we can have a high group paging rate but a much lower sub-group paging rate, so that each UE belonging to a certain sub-group needs to wake up only when the sub-group is paged. However, if a PO has a mix of legacy and new UEs, even though the new UEs has a reduced sub-group paging rate, the legacy UEs still need to receive the paging PDSCH according to the much higher group paging rate, which results in more power consumption for the legacy UEs. If we allow the legacy and new UEs to be separated, the sub-grouping feature can be fully utilized for the new UEs, without any negative impact on the legacy UEs.

- If a PO has legacy UEs only, there is no need for a network to transmit PEI. This is in contrast to the case where each PO has a mix of legacy and new UEs, and PEI always needs to be transmitted. This further reduces the unnecessary overhead.

This can be achieved by providing separate parameters for the number of paging frames and offset for the new UEs.

Proposal 3: Support of separate PO configurations for legacy UEs and new UEs.

# Conclusion

In this contribution, we evaluated the power saving gain for different paging enhancements, and made the following observations and proposal:

Proposal 1: Use PDCCH to carry paging early indication.

Proposal 2: Adopt Behv-A, i.e.,

- PEI indicates UE should monitor a PO if UE’s group/subgroup is paged.

- UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO.

Proposal 3: Support of separate PO configurations for legacy UEs and new UEs.

# Reference

- RP-200938, Revised WID UE Power Saving Enhancements for NR, MediaTek Inc., RAN#88e, June 2020.

# Appendix A: Evaluation Assumptions

| Parameter | Value |
| --- | --- |
| SCS | 30 kHz |
| Paging cycle | 1.28 sec |
| Group paging rate | 10% unless specified otherwise |
| SSB periodicity | 20 ms |
| SSB duration | 2 ms (for time/frequency tracking and serving cell measurement) |
| Intra-frequency RRM measurement | Assumes synchronized deployment, 20 ms SMTC periodicity, 2ms SMTC window that overlaps with serving cell SSB |
| Inter-frequency RRM measurement | 5 ms SMTC window and 6 ms measurement gap |
| PO duration | 2 ms |

# Appendix B: Power Consumption Model

| Power State | Relative Power (per slot) |
| --- | --- |
| Deep sleep | 1 |
| Light sleep | 20 |
| Micro sleep | 45 |
| PDCCH-only | 50 |
| PDCCH+PDSCH | 120 |
| PDSCH-only | 112 |
| SSB/CSI-RS processing | 50 |
| Intra-frequency RRM measurement | 60 for measurement only, 80 for combined search and measurement (synchronous) Probability for intra-frequency cell search = ¼ |
| Inter-frequency RRM measurement | 60 for measurement only, 60 for neighbor cell search Probability for inter-frequency cell search = ¼ Micro sleep power assumed for switch in/out a frequency layer |