3GPP TSG RAN WG1 #106-e			R1-2107754

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.8.1.2

Source:	Apple Inc.

Title:	Discussion on TB processing over multi-slot PUSCH

Document for:	Discussion/Decision

# Introduction

In RAN1#105-e meeting, the following agreements were reached on TBoMS.

Working assumption:  Agreement:

For TBS determination of TBoMS:

- NohPRB is configured by xOverhead and represents the overhead per slot.

- NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated.

Note: xOverhead configuration is as per Rel-15/16

Working Assumption

A transmission occasion for TBoMS (TOT) is constituted of at least one slot or multiple consecutive physical slots for UL transmission

- FFS: whether the concept of TOT will be used for designing aspects related to signal generation, e.g., rate-matching, power control, etc.

- FFS: whether such concept will be specified or not.

Agreement:

- The structure of TBoMS will be according to only one of these two options (to be down-selected in RAN1#106-e)

  - Option 3, if a design based on single RV is adopted.

  - Option 4, if a design based on different RVs is adopted.

- FFS: other details, e.g., rate-matching, TBS determination, collision handling, etc.

- The single RV is not constrained to have only the same coded bits in each slot or in each TOT

- The concept of TOT as per the corresponding Working assumption is used to define Option 3 and Option 4 and may or may not be used to design other details, e.g., rate-matching, TBS determination, collision handling and so on.

Agreement:

Time domain resource determination for TBoMS can be performed only via PUSCH repetition Type A like TDRA.

- FFS: details

- FFS: whether or not optimizations for time domain resource determination are necessary for allocating resource in the S slots (for the unpaired spectrum case)

Working assumption

Allocating resources for TBoMS in the special slot in TDD is possible according to the agreed time domain resource determination for TBoMS.

Agreement:

The following three options for rate-matching for TBoMS are considered for down-selection during RAN1 #106-e, where only one option will be selected:

- Option a: Rate-matching is performed per slot;

- Option b: Rate matching is performed continuously across all the allocated slot(s) per TOT;

- Option c: Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS

Note: “rate-matching is performed per X” means that the time unit for the bit selection and bit interleaving is X.

Note2: the above 3 options imply that the UL resource in the time unit may or may not be consecutive (depending on the given option)

Agreement:

Number of slots allocated for TBoMS is determined by using a row index of a TDRA list, configured via RRC.

- FFS: details.

Agreement:

The following approach is used to calculate NInfo for TBoMS:

- Approach 2: Based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated, scaled by K≥1.

  - FFS: the definition of K.

L is the number of symbols determined using the SLIV of PUSCH indicated via TDRA

FFS: impacts and further details if repetitions of TBoMS is supported.

FFS: whether the symbols over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed, and details on how to handle such scenarios.

In this contribution, we discuss the open issues of TB processing over multi-slot aspects and give our proposals.

# TB processing over multi-slot

The number of slots for TB transmission and transmission occasion

According to the evaluation result during the study in [2], the number of slots used for TB transmission is from 2 to 10 from different companies, the SNR gain and the power boosting gain could be achieved. Considering the TB overhead for the re-transmission, the maximum number of slots for TB transmission could be 8.

Proposal 1: Considering the maximum number of usable slots for TB transmission is 8.

As discussed in repetition type A enhancement, the available slot is defined to increase the repetition number. Similarly, how to count the slot for TBoMS transmission need to be determined, i.e., counting the slot based on physical slot or available slot. The difference between two counting methods is the DL slot will not be counted in the assigned number of slots for TBoMS transmission for available slot based counting, the assigned slot number is closing to the slots for actual transmission.

Proposal 2:  The number of slots for TBoMS transmission is counted based on available slot.

One working assumption was made in last RAN1 meeting on special slot usage for TBoMS. It was agreed the repetition type A like TDRA is applied for TBoMS time domain resource determination. If the UL symbols in special slot are aligned with SLIV indication, it’s natural the special slot could be used for TBoMS transmission, otherwise the special slot is skipped in the counting and there is no strong motivation to revise the TDRA scheme to enable the special slot usage.

Proposal 3: Confirm the following working assumption:

- Allocating resources for TBoMS in the special slot in TDD is possible according to the agreed time domain resource determination for TBoMS.

If joint channel estimation is applied to improve the PUSCH coverage, the TBoMS transmission occasion will be restricted by the available UL slots in the TDD UL/DL configuration, which cause the gain of TBoMS limited, repeating the TB transmission will be helpful to get the additional repetition gain on top of TB processing gain.

Proposal 4: For TB transmission over consecutive UL slots, repetition can be supported on top of TBoMS.

TB processing scheme

There were long discussions on TB processing scheme in RAN1#104bis-e meeting. Four options were down-selected to two options in RAN1#105-e meeting. Basically, the option 1 and option 3 are in the same category, only difference is TBS is determined by one TOT or multiple TOTs. Option 2 and option 4 are the similar case. Multiple TOTs are beneficial to facilitate the discussion with joint channel estimation, such as the TOT can be equal to time domain window for joint channel estimation, from this perspective, Option 3 and Option 4 are preferred than Option 1 and Option 2, and correspondingly the TOT is constituted of consecutive slots. Thus, it was agreed in RAN1#105-e meeting to down-select further between Option 3 and Option 4 in this meeting.

- Option 3:  Transmit a TB within multiple TOTs and  with single RV

To support transmission on non-consecutive UL slots, UE would  stop/resume the transmission before/after the DL slots boundary, additional baseband handling would be required for implementation. One issue is if the TB is not decoded correctly, then the whole TB will be retransmitted over multiple slots. If the number of slot for TB transmission is larger, re-transmission overhead is the concern.

- Option 4: Transmit a TB within multiple TOTs and with multiple RVs

The whole TB is rate matched in the slots of one TOT. RV index is refreshed for every TOT according to the RV pattern. To support transmission on non-consecutive UL slots, there is no special handling from UE side. If the TB is not correctly decoded, per TOT re-transmission is enough.

The rate matching options were discussed in the last meeting. For Option b, rate matching is performed continuously across all the allocated slot(s) per TOT, it is more suitable for Option4. Another rate matching option is performed continuously across all the allocated slots/TOTs for TBoMS, i.e., option c, it’s suitable for Option 3.

Proposal 5: A transmission block for TBoMBS is transmitted on multiple TOTs using different RVs, and the rate matching is performed across all slots per TOT.

# Summary

In this contribution, we discuss the TB transmission over multiple slots and have the following proposals:

Proposal 1: Considering the maximum number of usable slots for TB transmission is 8.

Proposal 2:  The number of slots for TBoMS transmission is counted based on available slot.

Proposal 3: Confirm the following working assumption:

- Allocating resources for TBoMS in the special slot in TDD is possible according to the agreed time domain resource determination for TBoMS.

Proposal 4: For TB transmission over consecutive UL slots, repetition can be supported on top of TBoMS.

Proposal 5: A transmission block for TBoMBS is transmitted on multiple TOTs using different RVs, and the rate matching is performed across all slots per TOT.

# References

- RP-202928, “New WID on NR coverage enhancements”, China Telecom

- R1-2106402, “Report of RAN1#105-e meeting”, ETSI MCC