3GPP TSG-RAN WG1 #106-e		R1-2107767

e-Meeting, August 16th – 27th, 2021

Source: 	Apple Inc.

Title:                     On efficient SCell Activation/Deactivation

Agenda item:	8.13.2

Document for:	Discussion and Decision

# 1 Introduction

In RAN1 104-bis e-meeting, the following was agreed for efficient activation/deactivation mechanism for one SCG and SCells [1]:

Agreement For efficient activation of SCells - Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s) - Details FFS including timeline design for receiving temporary RS Note: Separate from the support of Option 1a, it is up to RAN4 whether or not to consider an activation time enhancement for Option 2 without requiring further RAN1 work - Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding Rel-15/16 A-TRS(s) Send an LS to RAN4. The LS is endorsed in R1-2104110

In RAN1 105 e-meeting, good progress was made on the signaling designs for efficient SCell activation operation and the following was agreed [2]:

Agreement For efficient activation of Scells, the triggered temporary RS is aperiodic. Agreement For efficient activation of a Scell (in known Scell case), at least the number of temporary RS bursts is indicated by a field in new MAC-CE - The number of temporary RS bursts is RRC configurable. - FFS: which field in MAC-CE is used and how this field is associated with the number of bursts - For the purpose of designing temporary RS Scell activation, there is no RAN1 specification impact for the case where the number of indicated temporary RS bursts is smaller than what is expected by the UE Agreement To trigger temporary RS for efficient activation of SCells, the contents of the triggering MAC-CE(s) in a single PDSCH provide at least the following information (explicitly or implicitly): - Whether or not temporary RS is triggered - FFS detailed Information of temporary RS, e.g.: - Resources used for triggered Temporary RS - Triggering time offset of triggered Temporary RS - QCL source for triggered Temporary RS - FFS: Detailed signaling structure of the triggering MAC-CE(s) including the down-selection between the following example options and whether the decision should be made in RAN1 or RAN2 - Opt. 1.1: One new MAC CE for both SCell activation triggering and corresponding temporary RS triggering - Opt. 1.2: One R15/16 SCell activation MAC CE for SCell activation triggering and one new MAC CE (in the same PDSCH) for corresponding temporary RS triggering Agreement For efficient activation of a Scell (in known Scell case), the triggering offset of temporary RS is indicated by a field in new MAC-CE - The candidate value(s) of triggering offset(s) is RRC configurable - FFS: which field in MAC-CE is used and how this field is associated with the value of triggering offset Agreement For the reference slot for triggering offset of temporary RS - Option 2: the last DL slot of the to-be-activated Scell overlapping with slot n+k as defined in 38.213 sub-clause 4.3 - FFS: the earliest slot no earlier than the reference slot for a UE to receive a triggered temporary RS Agreement If a UE measures a temporary RS triggered by a MAC-CE during SCell activation procedure, the measurement is performed within the BWP bandwidth of BWP indicated by firstActiveDownlinkBWP-Id

In this contribution, we present our views on the remaining open issues regarding fast SCell activation signaling details.

# 2. Discussion

In RAN1 105 e-meeting, it was agreed that triggering MAC-CE(s) for efficient activation of SCell is included in a single PDSCH. It is FFS regarding the number of triggering MAC-CE. Two options were identified as follows:

- FFS: Detailed signaling structure of the triggering MAC-CE(s) including the down-selection between the following example options and whether the decision should be made in RAN1 or RAN2 - Opt. 1.1: One new MAC CE for both SCell activation triggering and corresponding temporary RS triggering - Opt. 1.2: One R15/16 SCell activation MAC CE for SCell activation triggering and one new MAC CE (in the same PDSCH) for corresponding temporary RS triggering

According to current MAC PDU structure, each MAC CE is associated with a MAC sub-header, which consists of R/F/LCID fields and size is at least one Octet. Depending on the MAC CE is fixed size or variable size, a new ‘L’ field maybe added with additional one Octet length. From signaling overhead perspective, Opt.1.1 (i.e., a single MAC CE) is clearly more preferrable to mitigate unnecessary MAC sub-headers for triggering MAC-CE. It should be noted that the specification impact or standard efforts of these two options are identical since anyhow the IEs for TRS triggering need to be specified regardless of integrated MAC CE (Opt.1.1) or separate MAC CE (Opt.1.2). Furthermore, Opt.1.2 requires one more LCID compared to Opt.1.1 and consume more LCID resource. One concern raised on Opt.1.1 is how it allows aperiodic TRS to be utilized for RRC-based SCell activation. It is our view that it is inefficient to trigger A-TRS using additional MAC CE for RRC-based SCell activation. Instead, it is more nature to provide separate TRS resource in the same RRC message that triggers SCell activation if the function or need is justified.

Proposal 1: Specify a new MAC CE for both SCell activation triggering and corresponding temporary RS triggering.

Another FFS aspect is the contents of the triggering MAC CE to trigger temporary RS for efficient activation of SCells. It was agreed in RAN1 105 e-meeting that MAC CE signaling should support whether or not temporary RS is triggered for a SCell activation i.e., TRS-based vs. SSB-based approaches, which provides flexibility for gNB to tradeoff between activation latency and RS signaling overhead and is useful in CA case. Other information was identified without reaching consensus [2]

Agreement To trigger temporary RS for efficient activation of SCells, the contents of the triggering MAC-CE(s) in a single PDSCH provide at least the following information (explicitly or implicitly): - Whether or not temporary RS is triggered - FFS detailed Information of temporary RS, e.g.: - Resources used for triggered Temporary RS - Triggering time offset of triggered Temporary RS - QCL source for triggered Temporary RS Agreement For efficient activation of a Scell (in known Scell case), at least the number of temporary RS bursts is indicated by a field in new MAC-CE - The number of temporary RS bursts is RRC configurable. - FFS: which field in MAC-CE is used and how this field is associated with the number of bursts - For the purpose of designing temporary RS Scell activation, there is no RAN1 specification impact for the case where the number of indicated temporary RS bursts is smaller than what is expected by the UE Agreement For efficient activation of a Scell (in known Scell case), the triggering offset of temporary RS is indicated by a field in new MAC-CE - The candidate value(s) of triggering offset(s) is RRC configurable - FFS: which field in MAC-CE is used and how this field is associated with the value of triggering offset

Table 1 summarized the information of MAC-CE based on the agreements made in RAN1 105-e meeting, including FFS aspects for each candidate.

Table 1: Information candidates for MAC-CE for efficient activation of SCells

| Index | Information | Agreed to be included in MAC-CE | FFS |
| --- | --- | --- | --- |
| 1 | Whether or not temporary RS is triggered | Yes | Detailed signaling |
| 2 | Triggering time offset of temporary RS | Yes The candidate value(s) of triggering offset(s) is RRC configurable | Which field in MAC-CE is used and how this field is associated with the value of triggering offset |
| 3 | Resources used for triggered Temporary RS | NOT yet |  |
| 4 | QCL source for triggered Temporary RS | NOT yet |  |

In our view, the information of row #3/#4 in Table 1 is beneficial to provide important flexibility for gNB. For example, indicating TRS resource (1 or 2 bursts) by MAC-CE is useful to enable sharing a 2-bursts TRS resource transmitted by gNB to a UE that requires 1-burst TRS for AGC and T/F tracking, which is achieved by configuring TRS resource with both 1 and 2 bursts for a single UE and then uses MAC-CE to select one of them. It should be noted that supporting all of row #2/#3/#4 does not necessarily cause MAC-CE overhead, which is subject to design details. As one example, for each TRS triggering state in MAC-CE, a TRS configuration with three columns as in Table 2 can be configured by gNB. The values for each configurable parameter (e.g., triggering offset ∆) that is associated with different triggering state (e.g., ‘00’ and ‘01’) can be same or different subject to gNB scheduler to properly setting based on use case and deployment scenarios.

Table 2: Configuration for TRS triggering State in MAC-CE

One remaining question is the indication whether temporary RS is triggered for a to-be-activated SCell. It may depend on the ‘TRS triggering state’ in the MAC-CE. Two options exist for ‘TRS triggering state’ information in MAC-CE:

- Opt.1: A common ‘TRS triggering state’ IE for to-be-activated SCells.

  - Similar as ‘CSI request field’ usage, the configuration for each ‘TRS triggering state’ to associate with <time offset, resources, QCL source> by RRC (as in Table 2) is per CC.

- Opt.2: A CC-specific ‘TRS triggering state’ IE for each to-be-activated SCell.

Figure 1: ‘CC-common’ and ‘CC-specific’ TRS triggering State field in MAC-CE

For Opt.2, as shown in FIG.2 one of ‘TRS triggering state’ code points e.g., ‘00’ can be reserved to indicate that no aperiodic TRS is triggered for a to-be-activated CC. In this case, SSB is used as in Rel-15/16. However, it does not work to reserve a single value for the shared field to indicate some CCs using TRS and others using SSB. To support this function, a bit-map field ‘Fi’ can be added to the MAC-CE with one-to-one mapping with each SCell, as shown in FIG.1. If a SCell is indicated to be activated by ‘Ci’ field, SSB is used for SCell activation if the corresponding ‘Fi’ field is set to be ‘0; Otherwise, TRS is used. The exact TRS resource is indicated by the ‘CC-common’ Triggering State field in the same MAC-CE.

Proposal 2: Support the following additional information to be configured by RRC on a per CC basis and then selected by MAC-CE by using ‘Triggering State’ field

- Resources used for triggered Temporary RS

- QCL source for triggered Temporary RS

Proposal 3: The mapping between ‘Triggering State’ field values and A-TRS configuration is configured by RRC on a per CC basis.

Proposal 4: Consider the following alternatives to indicate whether TRS is triggered for SCell activation:

- Alt.1: A common TRS 'trigger state' for to-be-activated SCells and additional bitmap to indicate a subset of these SCells to use SSB i.e., without TRS.

- Alt.2: CC-specific TRS 'trigger state' for to-be-activated SCells with one state (e.g., '00') indicating 'no TRS is triggered' i.e., using SSB as in Rel-16.

On the TCSI-reporting, the value depends on the periodicity configuration of P-CSI-RS or SP-CSI-RS used for CSI reporting. It was widely observed that 20ms/40ms periodicity is typically configured in NR system, which makes TCSI-reporting dominant for SCell activation procedure. As one example, if we count the additional delay from periodic report, there could still be up to 25ms delay even with A-TRS to expedite TO/FO/AGC refinement.

Observation 1: The CSI measurement and reporting are one of dominant factors for the latency of SCell activation procedure.

Proposal 5: The new MAC-CE command that triggers the SCell activation and A-TRS transmission is used to additionally trigger A-CSI-RS transmission to reduce the latency.

# 3. Conclusion

In this contribution, we have presented our views with the following proposal and observations:

Proposal 1: Specify a new MAC CE for both SCell activation triggering and corresponding temporary RS triggering.

Proposal 2: Support the following additional information to be configured by RRC on a per CC basis and then selected by MAC-CE by using ‘Triggering State’ field

- Resources used for triggered Temporary RS

- QCL source for triggered Temporary RS

Proposal 3: The mapping between ‘Triggering State’ field values and A-TRS configuration is configured by RRC on a per CC basis.

Proposal 4: Consider the following alternatives to indicate whether TRS is triggered for SCell activation:

- Alt.1: A common TRS 'trigger state' for to-be-activated SCells and additional bitmap to indicate a subset of these SCells to use SSB i.e., without TRS.

- Alt.2: CC-specific TRS 'trigger state' for to-be-activated SCells with one state (e.g., '00') indicating 'no TRS is triggered' i.e., using SSB as in Rel-16.

Observation 1: The CSI measurement and reporting are one of dominant factors for the latency of SCell activation procedure.

Proposal 5: The new MAC-CE command that triggers the SCell activation and A-TRS transmission is used to additionally trigger A-CSI-RS transmission to reduce the latency.

# References

- RAN1 104 bis e-meeting Chairman notes

- RAN1 105 e-meeting Chairman notes