3GPP TSG RAN WG1 #106-e	R1-2107788

e-Meeting, August 16th – 27th, 2021

Source:	Sharp

Title:	Enhancements on SRS

Agenda Item:	8.1.3

Document for:	Discussion and Decision

# Instruction

At RAN#86, a new work item on “Further enhancements on MIMO for NR” was agreed [1], and the revised WID was agreed at RAN#89-e [2]. In the WID, the following objectives were agreed for SRS enhancement.

3. Enhancement on SRS, targeting both FR1 and FR2: - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8}) - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

In this contribution, we share our views on SRS enhancement.

# Discussions

# Reference slot of SRS trigger

At RAN1#103 meeting, the following agreements were made for flexible trigger of aperiodic SRS, and which option to take was not agreed yet. In addition, at RAN1#104 meeting, RAN1 was not able to reach consensus about this.

Agreement A given aperiodic SRS resource set is transmitted in the (t+1)-th available slot counting from a reference slot, where t is indicated from DCI, or RRC (if only one value of t is configured in RRC), and the candidate values of t at least include 0. Adopt at least one of the following options for the reference slot. - Opt. 1: Reference slot is the slot with the triggering DCI. - Opt. 2: Reference slot is the slot indicated by the legacy triggering offset. - FFS the detailed definition of “available slot” considering UE processing complexity and timeline to determine available slot, potential co-existence with collision handling, etc., e.g., - Based on only RRC configuration, “available slot” is the slot satisfying: there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set - FFS explicit or implicit indication of t - FFS whether updating candidate triggering offsets in MAC CE may be beneficial

In our views, we still believe we don’t need to redefine the legacy triggering offset, and Option 2 has less the specification impacts. Therefore,

Proposal 1: RAN1 should adopt Option 2

# Conclusion

In this contribution, we have the following proposal:

Proposal 1: RAN1 should adopt Option 2

# References

- RAN1 chairman’s note, RAN1#105 e-meeting, May 2021