3GPP TSG RAN WG1 #106-e	R1-2107793

e-Meeting, August 16th – 27th, 2021

Source:	Sharp

Title:	Enhancements of intra-UE UCI multiplexing on PUCCH and PUSCH

Agenda Item:	8.3.3

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide our views on remaining issues for intra-UE multiplexing with different priorities on PUCCH and PUSCH.

# Timeline requirements for multiplexing

With the working assumption from in RAN1 #104 [1], the existing PUCCH/PUSCH multiplexing timeline requirements can be used as a baseline. In general, to multiplex UCI with different priorities on a PUCCH, the status or transmission of the high priority (HP) PUCCH should be known before the low priority (LP) PUCCH transmission. If the multiplexing timeline is not satisfied, e.g. the LP PUCCH transmission is already started, the Rel-16 channel dropping rule should be applied, so that the HP PUCCH is transmitted and the LP PUCCH transmission is dropped without resume.

Proposal 1: Reusing Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline for Rel-17 intra-UE PUCCH/PUSCH multiplexing with different priorities.

- The Rel-16 dropping rule is reused if the multiplexing timeline requirements are not met.

- UCI multiplexing enable/disable signalling

To support multiplexing of UCIs with different priorities on PUCCH, since PUCCH collision and timeline requirements should be evaluated first, RRC configuration to enable/disable the feature on PUCCH may be enough. If DCI indication is supported, the timeline requirement may be redundant and may have conflict with the DCI indication. For example, if the PUCCH carrying LP HARQ-ACK transmission is already started, and the UE receives a DCI indicates HP HARQ-ACK and LP HARQ-ACK is enabled, the UE may not know what behavior should be performed.

Similarly, for the UCI multiplexing on PUSCH with different priorities, because timeline requirements should be evaluated first, the RRC configuration for the enabled cases is sufficient. If DCI indication for UCI multiplexing is supported, it only complicates the UE behaviour. For example, if DCI indication contradicts with the timeline requirements for multiplexing on PUSCH, the UE behaviour is unclear.

Therefore, a RRC configuration to enable/disable the multiplexing is simpler with consistent UE behavior. The enable/disable for UCI multiplexing on PUCCH and/or different scenarios of UCI multiplexing on PUSCH may be configured separately.

Proposal 2: Multiplexing of UCI of different priorities on PUCCH, and different scenarios of UCI multiplexing on PUSCH can be separately enabled/disabled by RRC configuration.

# UCI multiplexing on PUCCH

As agreed in RAN1 #105-e [3], for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2, treat the two bits as HARQ-ACK bits with high priority. When the total number of LP HARQ-ACK and HP HARQ-ACK bits is more than 2 bits, separate coding should be applied on each HARQ-ACK codebook. In all cases, the HP PUCCH resource should be used.

  - Code rate determination and PUCCH selected with separate coding

In case of separate coding when the total paylod is greater than 2 bits, the coding rate of HARQ-ACK with different priorities should be different to provide different level of BER requirements.  Several methods can be considered to determine the maximum code rate for the UCI of each priority, e.g.

The existing maxCoderates in the PUCCH-Config for each priority can be used for the HARQ-ACK of the corresponding priority. Thus, when two PUCCH-Config are configured, the maxCodetate in PUCCH-Config for priority index 1 is applied on the HP HARQ-ACK, and the maxCodetate in PUCCH-Config for priority index 0 is applied on the LP HARQ-ACK.

Alternatively, two maxCoderate parameters can be configured for the PUCCH-Config with priority index 1, i.e. add another macCodetate for the LP HARQ-ACK to the PUCCH-Config for HP HARQ-ACK resources. The new maxCodetate parameter is used to determine the maxCodetate for LP HARQ-ACK in case of separate coding of HARQ-ACK with different priorities.

In another method, the UE can be configured vy higher layer signaling with a scaling factor between the coding rate of HP HARQ-ACK and LP HARQ-ACK. For example, a ratio or a beta offset value can be configured to provide the relative coding rate between the HARQ-ACK with different priorities.

With the determined maximum code rates of HARQ-ACK with different priorities, the UE can then select a HP PUCCH based on an effective payload, derived from the payload of HP HARQ-ACK and the payload of LP HARQ-ACK with consideration of the relative coding rate ratio between them.

For example, define a ratio as r = coderate of HP UCI/ coderate of LP UCI, thus r<1.

The effective payload can be given as

Effective HARQ-ACK payload = Payload of HP HARQ-ACK + r * Payload of LP HARQ-ACK

Proposal 3: For multiplexing a HP HARQ-ACK and a LP HARQ-ACK into a PUCCH with separate coding,

- Code rate for HARQ-ACK with different priorities are determined based on existing or additional maxCoderate parameters, or separate scaling factor configurations.

- A HP PUCCH resource is selected based on a derived effective payload.

  - HARQ-ACK and SR multiplexing on PUCCH

For multiplexing a LP HARQ-ACK and a HP SR, all PUCCH format combinations can be considered. In the last several meetings, the HARQ-ACK using PF0 and PF1 were discussed with many options listed for further discussion.

For up to 2 bits of LP HARQ-ACK with PF0 or PF1, a basic principle should be agreed first, i.e. no impact on the HARQ-ACK PUCCH transmission in case of negative HP SR. In case of positive HP SR, different methods can be considered for different PUCCH format combinations, as given below.

    - HP SR with PF1 and LP HARQ-ACK with PF0

Since a positive SR is indicated by the present of SR PUCCH transmission, and a SR PUCCH with PF1 has a capacity to carry two bits on the PUCCH resource. Furthermore, the HP PUCCH resource should be prioritized if possible since it normally provides better BER performance. For LP HARQ-ACK with PF1 collision with HP positive SR using PF 1, the high priority SR PUCCH resource can be used to carry the LP HARQ-ACK. The same method can be extended to the case of LP HARQ-ACK with PF0 collision with HP positive SR using PF 1.

Proposal 4: When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0,

- Transmit LP HARQ-ACK on the HP SR resource for positive HP SR, and

- Transmit LP HARQ-ACK on the LP HARQ-ACK resource for negative HP SR.

    - HP SR with PF0 and LP HARQ-ACK with PF0

For LP HARQ-ACK with PUCCH format 0 with HP positive SR using PUCCH format 0, the SR multiplexed on the HARQ-ACK PUCCH is a natural extension. However, it is better to have some differentiation between a position HP SR and a positive LP SR if there are overlapping HP SR and LP SR PUCCH resources with the HARQ-ACK PUCCH. Several methods can be considered.

In one method, different cyclic shift values can be used for HP positive SR and LP positive SR. For example, with 2 bits of HARQ-ACK, CS set of {1, 4, 7, 10} are used to indicate LP positive SR as in the current specifications, and CS set of {2, 5, 8, 11} are used to indicate HP positive SR.

In another method, the HP positive SR and LP HARQ-ACK may be multiplexed on the HP SR PUCCH resource. In current systems, a SR PUCCH resource is configured with only one CS value, there is no extra space to carry the HARQ-ACK bits.

However, if the HP SR PUCCH resource can be configured with reduce multiple reserved CSs (e.g. 2 CSs reserved to carry 1 extra bits and 4 CSs reserved to carry 2 extra bits), this method can be considered. Note this will reduce the multiplexing capabilities of SR PUCCH.

Thus, in case of negative SR, the LP HARQ-ACK is reported on LP HARQ-ACK PUCCH. And in case of positive HP SR, the LP HARQ-ACK is reported on HP SR PUCCH by applying a corresponding CS.

Proposal 5: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0,

- Multiplex positive HP SR on LP HARQ-ACK PUCCH by a CS is preferred.

- Alternatively, multiplex positive HP SR and LP HARQ-ACK on a HP SR PUCCH can be considered if a HP SR PUCCH PF0 resource is configured with multiple reserved CS values.

    - HP SR with PF0 and LP HARQ-ACK with PF1

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, multiplexing on a PUCCH may not be always possible due to a potential timing issue. If the LP HARQ-ACK PUCCH with PF1 transmission is already started when the HP positive SR is triggered on a HP SR PUCCH with PF0, multiplexing SR and HARQ-ACK on a PUCCH is not feasible. Thus, the Rel-16 dropping behaviour should be maintained.

On the other hand, if the positive HP SR is known/triggered before the LP HARQ-ACK PUCCH transmission, the same method as given before can be considered, i,e., multiplexing HP positive SR and LP HARQ-ACK on a HP SR PUCCH with PF0 only if a HP SR PUCCH PF0 resource is configured with multiple reserved CS values. Therefore, as a unified solution, the Rel-16 dropping behaviour may be applied without enhancement.

Proposal 6: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, the Rel-16 dropping behaviour may be applied.

- Alternatively, positive HP SR and LP HARQ-ACK multiplexing on the HP SR PUCCH with PF0 can be considered if the HP SR PUCCH PF0 resource is configured with multiple reserved CS values.

    - HP SR and LP HARQ-ACK with PUCCH format 2/3/4

For multiplexing of SR and HARQ-ACK with the same priority on PUCCH format 2/3/4, a number of bits for SR is generated based on the number of overlapping SR PUCCHs, then appended to the HARQ-ACK codebook. The bits for SR are included regardless of positive or negative SR status. Similarly, for multiplexing of HP SR with LP HARQ-ACK with more than 2 bits on PUCCH format 2/3/4, the HP SR bits can be generated based on the number of overlapping HP SR PUCCH resources. The generated HP SR bits can then be reported together with the LP HARQ-ACK codebook.

More details need to be discussed on the ordering of HP SR bits and the LP HARQ-ACK bits, and whether LP SRs should also be considered. For example, prepend high priority UCI bits before the LP UCI, i.e. HP SR + LP HARQ-ACK, or append the HP SR bits, i.e. LP HARQ-ACK + HP SR. In both cases, LP SR bits may also be appended to avoid dropping of LP positive SR while all HP SRs are negative.

Proposal 7: For multiplexing of HP SR with LP HARQ-ACK with PUCCH format 2/3/4,

- HP SR bits can be generated based on the number of overlapping HP SR PUCCH resources.

- FFS on the ordering of HP SR and LP HARQ-ACK

- FFS on if LP SR bits should also be included.

  - HP HARQ-ACK and LP HARQ-ACK and SR multiplexing on PUCCH

In case of multiplexing a LP HARQ-ACK, a HP HARQ-ACK and a HR SR, the total payload is always more than 2 bits, thus, a HP HARQ-ACK PUCCH resource with PF 2/3/4 should be selected.

The order of multiplexing within a priority before the multiplexing between different priorities should be maintained. Thus, the multiplexing order is defined as HP HARQ-ACK + HP SR + LP HARQ-ACK. To avoid dropping of a positive LP SR with negative HP SR, the LP SR may be configured by higher layer signalling and reported with the multiplexing order of HP HARQ-ACK + HP SR + LP HARQ-ACK + LP SR.

In all case, several issues need to be discussed on how to generate the SR bits. For a codebook with no more than 2 bits. In current HARQ-ACK and SR multiplexing on PUCCH with PUCCH format 0 or 1, no extra bit is added to the HARQ-ACK codebook.

For multiplexing of HARQ-ACK with different priorities and SR on PF 2/3/4, how to represent the SR and how many bits should be generated for the SR should be studied. For collision handling within each priority, SR bits are determined based on the number of overlapping SRs with the HARQ-ACK PUCCH resource of the same priority, i.e. the original PUCCH resource of each priority. In case of multiplexing of HARQ-ACK with different priorities, how to determine the SR overlapping conditions should be decided first, e.g.

- The SR bits may be determined based on the number of overlapping SRs with the original PUCCH resource with the same priority, or

- The SR bits may be determined based on the number of overlapping SRs with the determined PUCCH resource for UCI multiplexing.

Proposal 8: For multiplexing of HP HARQ-ACK, LP HARQ-ACK and SR, the multiplexing order and SR bit generation methods should be further clarified.

# UCI multiplexing on PUSCH

  - Set of beta-offset and beat-offset values

There are 4 different combinations between the UCI and PUSCH priorities. Thus, up to 4 different beta-offset can be configured for different cases.

For LP HARQ-ACK on HP PUSCH, the HARQ-ACK timing can always be satisfied. Thus, the existing Rel-16 method can be reused by applying a separate beta offset value, including beta-offset value smaller than 1. However, since the RRC signalling can be used to enable/disable UCI multiplexing on PUSCH, beat offset value of 0 to indicate the same function is somehow redundant and unnecessary.

If both HP HARQ-ACK and LP HARQ-ACK are reported on HP PUSCH, the HP HARQ-ACK can be multiplexed first, followed by the LP HARQ-ACK using different beta offsets.

Proposal 9: Support up to 4 different beta-offsets for different UCI and PUSCH priority combinations, and do not support beta offset = 0 for UCI disabling signalling.

  - HP HARQ-ACK on LP PUSCH

For HP HARQ-ACK on LP PUSCH, beside a different beta offset from LP HARQ-ACK, more details need to be discussed on the timeline requirements and the locations for HARQ-ACK multiplexing. For example, to maintain the HARQ-ACK timing,

- The HARQ-ACK can be multiplexed in a single hop only.

- The HARQ-ACK can only be multiplexed within the overlapping symbols between the HP PUCCH and the LP PUSCH.

- The starting symbol of HARQ-ACK multiplexing may be more flexible, e.g. may not be always immediately after a DMRS symbol.

If both HP HARQ-ACK and LP-HARQ-ACK need to be reported on LP-PUSCH, the multiplexing methods should be further clarified. For example,

- The LP HARQ-ACK can be multiplexed on the PUSCH first as in Rel-16, then the HP-HARQ is multiplexed by puncturing on the PUSCH with LP HARQ-ACK.

- The HP HARQ-ACK and LP-HARQ-ACK can be multiplexed on LP-PUSCH so that the HP-HARQ-ACK is multiplexed first within a single hop, and the LP-HARQ-ACK may be multiplexed on two hops.

Proposal 10: For HP HARQ-ACK with or without LP HARQ-ACK on LP PUSCH, details should be further studied on

- Multiplexing location, e.g. starting/ending symbol

- Detailed multiplexing methods, etc.

  - High priority SR multiplexing on LP PUSCH

With current rules, SR is not reported on PUSCH. Thus, in case of a HP SR overlapping with LP PUSCH, the LP PUSCH will be dropped. However, if there is a HP HARQ-ACK multiplexed on the LP PUSCH, dropping the LP PUSCH may also drops the more important HP HARQ-ACK.

Therefore, the HP SR reporting needs to be enhanced for overlapping between a PUCCH for HP SR and a LP PUSCH. For example, the positive HP SR multiplexing on a LP PUSCH may be supported by a RRC configuration or UE capability. Detailed methods of HP SR multiplexing on LP PUSCH can be FFS, e.g. the HP SR may be reported in a similar was as HP HARQ-ACK or jointly with HARQ-ACK.

Furthermore, even if SR multiplexing on PUSCH is supported, if the multiplexing timeline cannot be satisfied, e.g. a positive SR is triggered after the start of a LP PUSCH transmission, SR multiplexing on PUSCH may be not possible and channel dropping behaviour should be performed.

To avoid dropping of potential HP UCI on a LP PUSCH, the channel dropping rules should be enhanced. For example, if there is HP HARQ-ACK multiplexing on the LP PUSCH, the LP PUSCH with HP HARQ-ACK should be treated as a HP channel and be transmitted, and the HP SR PUCCH should be dropped. On the other hand, if there is no HP UCI multiplexed on the LP PUSCH, HP SR PUCCH should be transmitted, and the LP PUSCH should be dropped. The enhanced channel dropping rule should also be supported if SR multiplexing on PUSCH is not supported.

Proposal 11: HP SR multiplexing on LP PUSCH can be supported, and enhanced channel dropping rule should be defined based on whether HP UCI is multiplexed on LP PUSCH.

# Conclusions

In this contribution, we provide the following proposals for potential enhancement of intra-UE UCI multiplexing on PUCCH and PUSCH.

For UCI multiplexing timeline,

Proposal 1: Reusing Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline for Rel-17 intra-UE PUCCH/PUSCH multiplexing with different priorities.

- The Rel-16 dropping rule is reused if the multiplexing timeline requirements are not met.

For enable/disable signaling of UCI multiplexing with different priorities,

Proposal 2: Multiplexing of UCI of different priorities on PUCCH, and different scenarios of UCI multiplexing on PUSCH can be separately enabled/disabled by RRC configuration.

For PUCCH resource for UCI multiplexing,

Proposal 3: For multiplexing a HP HARQ-ACK and a LP HARQ-ACK into a PUCCH with separate coding,

- Code rate for HARQ-ACK with different priorities are determined based on existing or additional maxCoderate parameters, or separate scaling factor configurations.

- A HP PUCCH resource is selected based on a derived effective payload.

For different cases of HP SR overlapping with LP HARQ-ACK on PUCCH, and for HP SR multiplexing with HP HARQ-ACK and LP HARQ-ACK, we propose

Proposal 4: When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0,

- Transmit LP HARQ-ACK on the HP SR resource for positive HP SR, and

- Transmit LP HARQ-ACK on the LP HARQ-ACK resource for negative HP SR.

Proposal 5: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0,

- Multiplex positive HP SR on LP HARQ-ACK PUCCH by a CS is preferred.

- Alternatively, multiplex positive HP SR and LP HARQ-ACK on a HP SR PUCCH can be considered if a HP SR PUCCH PF0 resource is configured with multiple reserved CS values.

Proposal 6: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, the Rel-16 dropping behaviour may be applied.

- Alternatively, positive HP SR and LP HARQ-ACK multiplexing on the HP SR PUCCH with PF0 can be considered if the HP SR PUCCH PF0 resource is configured with multiple reserved CS values.

Proposal 7: For multiplexing of HP SR with LP HARQ-ACK with PUCCH format 2/3/4,

- HP SR bits can be generated based on the number of overlapping HP SR PUCCH resources.

- FFS on the ordering of HP SR and LP HARQ-ACK

- FFS on if LP SR bits should also be included.

Proposal 8: For multiplexing of HP HARQ-ACK, LP HARQ-ACK and SR, the multiplexing order and SR bit generation methods should be further clarified.

For UCI multiplexing on PUSCH, we propose

Proposal 9: Support up to 4 different beta-offsets for different UCI and PUSCH priority combinations, and do not support beta offset = 0 for UCI disabling signalling.

Proposal 10: For HP HARQ-ACK with or without LP HARQ-ACK on LP PUSCH, details should be further studied on

- Multiplexing location, e.g. starting/ending symbol

- Detailed multiplexing methods, etc.

Proposal 11: HP SR multiplexing on LP PUSCH can be supported, and enhanced channel dropping rule should be defined based on whether HP UCI is multiplexed on LP PUSCH.

# References

- Chairman’s Note RAN#104-e final

- Chairman’s Note RAN#104-bis-e final

- Chairman’s Note RAN#105-e final