3GPP TSG RAN WG1 Meeting #106-e	R1-2107804

e-Meeting, August 16th – 27th, 2021

Source:	Sharp

Title:	Discussion on resource allocation for power saving

Agenda Item:	8.11.1.1

Document for:	Discussion and Decision

Introduction

A revised work item on NR sidelink enhancement was approved in RAN#90-e meeting [1], with one of the objectives to specify resource allocation to reduce power consumption of the UEs, as follows:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - This work should consider the impact of sidelink DRX, if any.

In this document, we share our views on a few aspects relating to resource allocation for power saving.

Discussion

Periodic-based partial sensing

(Pre-)configuration of k

One of the remaining issues for periodic-based partial sensing is the details of (pre-)configuration of the value k. In LTE V2X, (pre-)configuration of k is via a bitmap. Therefore, we prefer a bitmap similar as LTE V2X partial sensing is re-used in NR sidelink.

Proposal 1: Adopt a bitmap for (pre-)configuration of k.

In RAN1#105e, it was agreed for resource selection, UE at least monitors the most recent sensing occasion for a given reservation periodicity before the resource selection trigger slot n or the first slot of the set of Y candidate slots. For a candidate slot tySL, the most recent sensing occasions before the slot n or the first slot of Y candidate slots for two different reservation periodicities (Preserve1 and Preserve2) may correspond to two different values of k. With the increasing number of (pre-)configured reservation periodicities, it is not flexible to accommodate different values of k corresponding to each periodicity. One method to address the issue is to interpret the bitmap of k from the most recent sensing occasion before slot n or the first slot of the Y candidate slots, i.e. denote the most recent sensing occasion before slot n or the first slot of the Y candidate slots as ty-k0×PreserveSL, UE shall monitor slots ty-(k-1+k0)×PreserveSL if the k-th bit is (pre-)configured as ‘1’.

Proposal 2: For a candidate slot tySL, UE shall monitor slots ty-(k-1+k0)×PreserveSL if the k-th bit in the bitmap is (pre-)configured as ‘1’, where ty-k0×PreserveSL is the most recent sensing occasion before the resource selection triggering slot n or the first slot of the Y candidate slots.

Pre-emption check and re-evaluation

In previous RAN1 e-meetings, details of pre-emption check and re-evaluation when performing periodic-based partial sensing remain undecided yet. In NR V2X, the procedure for pre-emption check or re-evaluation can be regarded as part of full sensing procedure. The resource(s) subject to pre-emption check or re-evaluation are within the resource selection window for current TB and after performing identification of candidate resources (i.e. full sensing), if the resource(s) subject to pre-emption check and re-evaluation are excluded from the candidate resource set, then UE would report re-evaluation for the resource(s), or additionally determine whether to report pre-emption for the resource(s) based on the priority information. Similar mechanism can be adopted for NR periodic-based partial sensing, which means if UE selects Y candidate slots up to implementation, the Y candidate slots shall include at least the slots corresponding to the resource(s) subject to pre-emption check or re-evaluation provided by MAC layer, otherwise UE would be unable to determine whether to report re-evaluation or pre-emption check.

Proposal 3: For periodic-based partial sensing, the selected Y candidate slots include at least the slots corresponding to the resources subject to pre-emption check and/or re-evaluation.

In Rel-16 NR V2X, resource(s) of a sidelink grant subject to pre-emption check has been indicated by a prior SCI. It is also noted that each candidate slot among the Y candidate slots in NR periodic-based partial sensing corresponds to a set of periodical sensing occasions. For the default case (i.e. no configuration), the set of periodical sensing occasions include sensing occasions corresponding to every periodicity in sl-ResourceReservePeriodList, which may include the slot of the prior SCI which indicates the resource subject to pre-emption check. Due to half-duplex constraint, the slot of the prior SCI cannot be monitored. Therefore, it is straightforward to preclude the slot which indicates the resource subject to pre-emption check from the periodical sensing occasions, if any.

Proposal 4:  If tySLis included in the set of Y candidate slots and at least a resource subject to pre-emption check is in slot tySL, the UE monitors slots of periodic sensing occasions ty-k×PreserveSL except for the slot of a prior SCI which indicates the resource.

Contiguous partial sensing

Monitored slots in contiguous partial sensing

Besides the periodic-based partial sensing, the contiguous partial sensing was introduced in NR sidelink, mainly for the aperiodic traffic. The contiguous partial sensing related agreements are as follows,

Agreements: - In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option: - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable). - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.) - FFS whether n can be replaced by e.g., index of some of Y candidate slots - FFS condition(s) in which contiguous partial sensing is performed by UE - FFS interaction with SL-DRX, if any - FFS interaction with periodic-based partial sensing, if any - Other options are not precluded - Note: This option is not to replace random resource selection only without sensing or re-evaluation and pre-emption checking Agreement: - In contiguous partial sensing for resource (re)selection, TA and TB values can be zero, positive or negative - TA and TB values or range depend on different operating scenarios or conditions (e.g., periodic/aperiodic traffic, predictability of triggering slot n, remaining PDB, re-evaluation/pre-emption checking, HARQ feedback, CBR/CR parameter, power saving, etc) - FFS details - FFS: details of how periodic-based partial sensing and contiguous partial sensing are used for re-evaluation and pre-emption checking. Including how to reduce UE’s power consumption (caused by additional sensing operation of re-evaluation/pre-emption) after its resource selection, with the considerations of different operating scenarios or conditions (e.g., pre-emption enabled/disabled, HARQ-ACK enabled/disabled, etc)

One of the remaining issues is the value of TA/TB and whether n can be replaced by index of some candidate slots. In Rel-16 NR V2X, when the MAC layer triggers sensing, i.e. to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission, the MAC layer may also provide a set of resources {ri} for re-evaluation, which means {ri} are regarded as candidate resources in the specified sensing procedure, i.e. Step 1-7 in clause 8.1.4 of TS38.214.

Regarding NR V2X, e.g. a resource r0 among {ri} is going to be signalled at first time in slot ty0, the UE would trigger the re-evaluation procedure in slot (ty0-T3) to check potential reservation by other UEs after the moment when the UE performed sensing and selected the resource in slot ty0, i.e. after the initial sensing procedure. The re-evaluation procedure for a resource in slot ty0 is shown in Figure 1 as follows. Note that the duration between slot n and slot (ty0-T3) (i.e. the red arrow) is for the re-evaluation.

Figure 1. Re-evaluation procedure in NR V2X

With only periodic-based partial sensing, for the initial sensing when UE firstly selects r0 in slot ty0, the candidate slot ty0 corresponds to ty0-Preserve and ty0-2*Preserve(e.g. k=1,2). For the sensing of re-evaluation, the candidate slot ty0 corresponds to ty0-Preserve and ty0-2*Preserve as well, as shown in Figure 2. Thus, w/o the contiguous partial sensing, the duration for re-evaluation as illustrated in Figure 1 is missing and the re-evaluation fails to work any longer, similar for pre-emption check. With the introduced contiguous partial sensing and n is replaced with the index of the candidate slot ty0, accordingly TA is set as (-kmin×Preserve) and subject to processing time, the contiguous partial sensing achieves the same function as the sensing duration for re-evaluation, where kmin is the minimum of k values. Since an SCI can indicate a resource within 31 resource pool slots, TB would be 31 resource pool slots later than TA.

Figure 2. Re-evaluation procedure when applying only periodic-based partial sensing (k=1, 2)

Proposal 5: Replace n with index of the Y candidate slots, set TA as (-kmin×Preserve) subject to the processing time and TB as 31 resource pool slots later than TA, where kmin is the minimum of k values.

Handling of mixed resource allocation schemes in a resource pool

It was agreed in RAN1#103-e that a resource pool can be configured to enable any combination of full sensing, partial sensing, and random resource selection, indicating that a mixed use of resource allocation schemes in a same resource pool is allowed. A few questions related to this agreement arise.

Question #1: How does a UE capable of multiple resource allocation schemes configured to enable in a TX pool determines which one to use?

In LTE V2X, V2X sidelink transmissions are divided into the following:

- “Non-P2X related V2X sidelink communication” where full sensing is used, unless in exceptional cases where the UE falls back to random resource selection.

- “P2X related V2X sidelink communication” where either partial sensing or random resource selection is used as per configuration or it is left to UE implementation which one to use if both are configured. Similarly to the non-P2X case, the UE falls back to random resource selection in case of exception and when partial sensing is chosen.

In NR sidelink, with the support for a much wider range of scenarios / use cases, it is highly undesirable to hard-code “P2X related V2X sidelink communication” into the specs. On the other hand, it seems natural to follow the principle in LTE V2X that if multiple resource allocation schemes are enabled, instead of (pre-) configuring a fixed resource allocation scheme for all UEs, each UE should be able to choose one of the configured schemes, depending on at least UE capability and the usage scenario. And we think this should not be completely left to UE implementation, otherwise the system performance may fluctuate dramatically as the resource allocation schemes chosen by the UEs communicating with each other vary. For example, the system performance might be severely degraded if a large number of UEs use random resource selection. It is proposed that some kind of prioritization is applied for selection of resource allocation schemes in case multiple resource allocation schemes are configured to enable in a resource pool, e.g. partial sensing should be prioritized if the UE is in power-saving mode, and full sensing should be prioritized if the UE is in non-power-saving mode.

Proposal 6: Prioritization is applied for selection of resource allocation schemes in case the UE is capable of multiple resource allocation schemes configured to enable in a resource pool.

Question #2: Should a Rel-16 UE use full sensing even if a chosen resource pool is not configured to enable full sensing?

In our view, in order to be backward compatible with Rel-16 UEs, full sensing should be always allowed in a resource pool, even if not explicitly configured to enable in the resource pool configuration.

Proposal 7: Full sensing is always allowed in a resource pool.

Conclusion

In this contribution, we discuss a few aspects relating to relating to resource allocation for power saving, and make the following proposals:

Proposal 1: Adopt a bitmap for (pre-)configuration of k.

Proposal 2: For a candidate slot tySL, UE shall monitor slots ty-(k-1+k0)×PreserveSL if the k-th bit in the bitmap is (pre-)configured as ‘1’, where ty-k0×PreserveSL is the most recent sensing occasion before the resource selection triggering slot n or the first slot of the Y candidate slots.

Proposal 3: For periodic-based partial sensing, the selected Y candidate slots include at least the slots corresponding to the resources subject to pre-emption check and/or re-evaluation.

Proposal 4: If tySLis included in the set of Y candidate slots and at least a resource subject to pre-emption check is in slot tySL, the UE monitors slots of periodic sensing occasions ty-k×PreserveSL except for the slot of a prior SCI which indicates the resource.

Proposal 5: Replace n with index of the Y candidate slots, set TA as (-kmin×Preserve) subject to the processing time and TB as 31 resource pool slots later than TA, where kmin is the minimum of k values.

Proposal 6: Prioritization is applied for selection of resource allocation schemes in case the UE is capable of multiple resource allocation schemes configured to enable in a resource pool.

Proposal 7: Full sensing is always allowed in a resource pool.

References

RP-202846, “WID revision: NR Sidelink enhancement”, LG Electronics, RAN#90-e.

RAN1 agreements on SL resource allocation for power saving

RAN1#103-e

Conclusion

- SL reception Type A and Type D should be used as the reference for evaluation and designing of SL power saving features in R17.

  - Type A: UE is not capable of performing reception of any SL signals and channels, FFS with exception of performing PSFCH and S-SSB reception (aim to conclude in RAN1#104-e)

  - Type D: UE is capable of performing reception of all SL signals and channels defined in R16. It does not preclude UE to perform reception of a subset of SL signals/channels

  - If there are evaluations with assumptions other than the above reference, the detailed assumptions need to be reported

  - Note: the types and the associated capability defined here are not intended to be defined as Rel-17 UE features as is.

Agreements:

- Partial sensing based RA is supported as a power saving RA scheme

  - FFS details

- Random resource selection is supported as a power saving RA scheme

  - FFS any changes or enhancement

  - FFS on conditions to apply random resource selection

Agreements:

- In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof

  - FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc.

Agreements:

- Re-evaluation and pre-emption checking are not supported by UEs that do not perform any sensing (i.e. PSCCH reception)

- Re-evaluation and pre-emption checking are supported by UEs that perform sensing

  - FFS details and any conditions(s) in which re-evaluation and pre-emption can be performed

- FFS whether/how re-evaluation and pre-emption can be supported by UEs performing random resource selection that do perform sensing

- Note: details about sensing in this context, including when it is performed, are not decided yet.

Agreements:

- Further study congestion control based on CBR and CR for power saving RA schemes

  - Identify necessary changes from R16 CBR/CR (if any), including transmission resource selection and transmission parameters that can be adjusted and applicable to power savings RA schemes

  - Note: this is not intended to require all UEs to perform sensing for the purpose of CBR measurement

RAN1#104-e

Agreements:

- Random resource selection is applicable to both periodic and aperiodic transmissions

  - FFS conditions for random resource selection

Conclusion:

- PSFCH reception is not included for Type A UE

- S-SSB reception is not included for Type A UE

- SL reception Type B is additionally added

  - Type B: Same as Type A with an exception of performing PSFCH and S-SSB reception

- Note: the same conditions as in RAN1#103-e regarding the context of the discussion of Type A and Type D still apply (also applicable to type B)

Agreements:

In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where

- FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE

- The resource selection window is [n+T1, n+T2]

  - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4]

  - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that

    - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB

    - T2-T1 ≤ (pre-)configured threshold

- A minimum value for Y is (pre-)configured from a range of values, FFS details

- FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX)

- FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17.

Agreements:

In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, the UE monitors slots of at least one a set of periodic sensing occasions, where a periodic sensing occasion is a set of slots according to

if tvSL is included in the set of Y candidate slots.

- Preserve is a periodicity value from the configured set of possible resource reservation periods allowed in the resource pool (sl-ResourceReservePeriodList). Down select to one:

  - Option 1:  Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

  - Option 2:  PreservePreserve Preserve corresponds to a subset of values from the configured set sl-ResourceReservePeriodList

    - FFS how to determine the subset (e.g., by (pre-)configuration, UE determination)

  - Option 3: PreservePreserve Preserve is a common divisor among values in the configured set sl-ResourceReservePeriodList

  - Option 4: FFS others

- k equals tois selected according to (down select to one)

  - Option 1: Only the most recent sensing occasion within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction

  - Option 2: The two most recent sensing occasions within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction

  - Option 3: All possible sensing occasions after n –T0

  - Option 4: Only one periodic sensing occasion for one reservation period. The k value is up to UE implementation. Max value for k is (pre-)configured.

  - Option 5: k is (pre-)configured, including multiple values

  - Option 6: (pre-)configuration of a bitmap, same as in LTE-V

  - Option 7: FFS others

- FFS relationship between periodic sensing occasions and SL-DRX

- FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE

- Note: companies are encouraged to show performance data for the down selections

Agreements:

- In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option:

  - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable).

    - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.)

    - FFS whether n can be replaced by e.g., index of some of Y candidate slots

  - FFS condition(s) in which contiguous partial sensing is performed by UE

  - FFS interaction with SL-DRX, if any

  - FFS interaction with periodic-based partial sensing, if any

  - Other options are not precluded

  - Note: This option is not to replace random resource selection only without sensing or re-evaluation and pre-emption checking

RAN1#104b-e

Conclusion:

- In periodic-based partial sensing,

  - It is not necessary to further discuss whether or not to introduce a threshold to re-define T1 and T2.

Agreements:

- In periodic-based partial sensing,

- For the set of Preserve values, down-select to one of the following in RAN1#105-e

    - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

    - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList

      - FFS if support multiple sets of Preserve values based on one or more metrics

      - FFS whether/how to restrict the set of values

- For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible)

        - Alt 1: Option 1 as in RAN1#104-e

        - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1

          - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration

        - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc.

      - Note: companies are encouraged to provide more evaluations

Agreement:

- When periodic-based partial sensing is potentially performed by UE in a mode 2 Tx resource pool provided by higher layer, at least all of the followings are met:

  - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

  - The resource pool is (pre-)configured to enable partial sensing

  - Partial sensing configured by higher layer in the UE

RAN1#105-e

Agreem Agreement:

- For the set of Preserve values in periodic-based partial sensing,

  - If no (pre-)configuration (i.e., by default), Preserve corresponds to all values from the (pre-)configured set sl-ResourceReservePeriodList.

  - Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList

    - This is per mode 2 Tx resource pool (pre-)configuration

    - A UE by implementation may also monitor other sl-ResourceReservePeriodList values not part of the restricted subset

      - In particular, the UE may additionally monitor occasions corresponding to P_RSVP_Tx

        - FFS whether the monitoring can be mandatory

Agreement:

- In periodic-based partial sensing for resource (re)selection, the UE at least monitors in periodic sensing occasion(s) for a given reservation periodicity before the first slot of the selected Y candidate slots subject to processing time restriction for the identification of candidate resources.

  - The processing time restriction includes Tproc,0SL  and Tproc,1SL.

  - Aspects relating to sensing during SL DRX are to be discussed separately

- Relationship to re-evaluation and pre-emption operation for periodic-based partial sensing to be discussed separately

  - FFS details including whether monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection or re-evaluation and pre-emption checking

Agreement:

- For the k value in periodic-based partial sensing for resource (re)selection,

  - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction.

  - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value

    - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included.

    - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions)

    - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring.

  - FFS relationship between periodic-based partial sensing occasions and SL-DRX

  - Note:

    - This is for the case when the resource (re)selection triggering slot n is expected by UE

Agreement:

- For random resource selection,

  - Reuse the maximum distance separation of 32 logical slots for a HARQ retransmission resource reserved by a prior SCI for the same TB, which was defined in R16 for full sensing operation.

  - SL HARQ feedback enabled transmission is supported (FFS applicable conditions if any)

    - The minimum HARQ feedback time gap (Z) shall be respected between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected.

- FFS the impact of resource collision when random resource selection is performed by a UE which does not perform sensing / re-evaluation and pre-emption checking in a resource pool with mixed RA schemes (e.g. for low priority or any priority transmissions).

  - Including study potential solution(s) if the impact is not negligible (e.g. threshold based, raising priority, minimum time gap, pattern based, a priori SCI reserving initial transmissions, resource pool partitioning, and etc.).

Agreement:

In contiguous partial sensing for resource (re)selection, TA and TB values can be zero, positive or negative

- TA and TB values or range depend on different operating scenarios or conditions (e.g., periodic/aperiodic traffic, predictability of triggering slot n, remaining PDB, re-evaluation/pre-emption checking, HARQ feedback, CBR/CR parameter, power saving, etc)

  - FFS details

- FFS: details of how periodic-based partial sensing and contiguous partial sensing are used for re-evaluation and pre-emption checking. Including how to reduce UE’s power consumption (caused by additional sensing operation of re-evaluation/pre-emption) after its resource selection, with the considerations of different operating scenarios or conditions (e.g., pre-emption enabled/disabled, HARQ-ACK enabled/disabled, etc).