3GPP TSG RAN WG1 #106-e		                                    R1-2107809

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.6.1.1

Source:	InterDigital, Inc.

Title:	Reduced maximum bandwidth for RedCap UEs

Document for:	Discussion

# Introduction

In RAN1 #105-e, reduced maximum bandwidth for RedCap UEs was discussed and the following agreements were made [1]:

Agreements: Replace the RAN1#104bis-e working assumption with the following working assumption (for option 1) and working assumption (for option 2):

- Working assumption: After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

- Working assumption: After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 2 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

Agreements:

- Both during and after initial access, the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the maximum RedCap UE bandwidth is allowed.

- Working assumption: Both during and after initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs.

  - FFS: whether/how to avoid or minimize PUSCH resource fragmentation due to PUCCH transmission for the above case

  - Support the case when the center frequency is assumed to be the same for the initial DL and UL BWPs in TDD.

    - FFS whether or not to additionally support the case when the center frequency is different; if so, how to minimize center frequency retuning

Agreement: Take the following as an agreement, revised from the RAN1#104bis-e working assumption:

- A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE.

  - At least for FR1, FG 6-1 (“Basic BWP operation with restriction” as described in TR 38.822) is used as a starting point for the mandatory RedCap UE type capability.

    - This does not preclude support of FG 6-1a (“BWP operation without restriction on BW of BWP(s)” as described in TR 38.822) as a UE capability for RedCap UEs.

Working assumption: Both during and after initial access, even for the scenario where the initial UL BWP for non-RedCap UEs is not configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP can optionally be configured/defined for RedCap UEs.

- RO sharing between RedCap and non-RedCap is not precluded.

Working assumption: For enabling/supporting that the RACH occasion (RO) associated with the best SSB falls within the RedCap UE bandwidth, support separate initial UL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth), and this separate initial UL BWP for RedCap includes ROs for RedCap UEs.

- Note: these ROs can be dedicated for RedCap UEs or shared with non-RedCap UEs.

Working assumption:

- For enabling/supporting that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access, support separate initial UL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth).

  - FFS: whether/how the specification also supports separate PUCCH/Msg3/[MsgA] PUSCH configuration/indication or a different interpretation of the same configuration/indication for RedCap (e.g., disabled frequency hopping or different frequency hopping)

- Working assumption: At least for TDD, an initial DL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth) can be optionally configured/defined separately from the initial DL BWP for non-RedCap UEs at least after initial access

  - FFS the details of the configuration/definition

    - The configuration for a separately configured initial DL BWP for RedCap UEs is signaled in SIB.

    - whether to support that separate initial DL BWP for RedCap UEs can include a configuration of CORESET and CSS(s)

    - whether part of the configuration can be defined instead of signaled

  - If a separate initial DL BWP for RedCap UEs is configured/defined, this separate initial DL BWP for RedCap UEs can be used at least after initial access (i.e., at least after RRC Setup, RRC Resume, or RRC Reestablishment).

    - FFS during the initial access

  - FFS: whether a separately configured initial DL BWP for RedCap UEs needs to contain the entire CORESET #0, and, if not, the Redcap UE behaviour for CORESET #0 monitoring

  - FFS: supported bandwidths in the separate initial DL BWP

  - FFS: whether additional SSB is transmitted in the separately configured initial DL BWP for RedCap UEs

  - FFS: FDD case

In this contribution, we discuss some remaining issues of reduced maximum bandwidth for RedCap UEs.

# Reduced maximum bandwidth

### Initial DL BWP and CORESET for RedCap UEs

It has already been agreed that, after initial access, an initial DL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth) can be optionally configured/defined separately from the initial DL BWP for non-RedCap UEs. Similarly, during the access, initial DL BWP for RedCap UEs can be configured to manage network congestion. The transmission of msg2/msg4/paging/SI can be offloaded to a new CORESET defined for RedCap UEs in the new initial DL BWP.

Figure 2-1 illustrates four possible arrangements for CORESET#0 and the new RedCap-CORESET. If the total frequency span of the CORESET#0 and RedCap-CORESET is within the maximum RedCap BW, the Redcap UE does not need to retune its frequency, e.g., to receive SIB1 in CORESET#0 and other information in the new CORESET. Note that as the CORESET#0 BW gets larger, CORESET#0 and the RedCap-CORESET may have to overlap in order to keep the total span within the maximum RedCap BW, possibly reducing the offloading benefit.

On the other hand, if the total frequency span of the CORESET#0 and RedCap-CORESET is larger than the maximum RedCap BW, the Redcap UE would need to retune its frequency. However, given that SIB1 reception is not frequent, this retuning operation is not expected to meaningfully increase RedCap UE complexity. If a new RedCap-CORESET is not defined, the additional DL BWP does not need to fully contain the CORESET#0 since in this case the new DL BWP and the initial DL BWP will overlap significantly, and the benefits of offloading would diminish.

In addition to the above, SSB may be transmitted in the new DL BWP for the RedCap UE to perform measurements for synchronization, intra-frequency RRM, RLM, etc.

Figure 2-1 Possible arrangements of CORESET#0 and RedCap-CORESET

Proposal 1: Separate initial DL BWP for RedCap UEs is configured/defined during initial access.

Proposal 2: Support a configuration of CORESET and CSS(s) in the initial DL BWP for RedCap UEs.

Proposal 3: Separately configured initial DL BWP for RedCap UEs does not need to contain the entire CORESET #0.

Proposal 4: Support SSB transmission in the initial DL BWP for RedCap UEs.

### UL BWP

It has been agreed to support configuration of a separate initial UL BWP no wider than the RedCap UE maximum bandwidth both during and after initial access. One specific issue that needs to be addressed is the PUCCH transmission after msg4 during initial access. According to 38.213, the common PUCCH resource set to be used in the initial uplink BWP is determined as follows:

If the UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a DCI format scheduling a PDSCH reception or a SPS PDSCH release, the UE determines a PUCCH resource with index rPUCCH, 0≤rPUCCH≤15, as rPUCCH=2⋅nCCE,0NCCE+2⋅∆PRI, where NCCE is a number of CCEs in a CORESET of a PDCCH reception with the DCI format, as described in Clause 10.1, nCCE,0 is the index of a first CCE for the PDCCH reception, and ∆PRI is a value of the PUCCH resource indicator field in the DCI format. If rPUCCH8=0 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon - the UE determines the PRB index of the PUCCH transmission in the first hop as RBBWPoffset+rPUCCHNCS and the PRB index of the PUCCH transmission in the second hop as NBWPsize-1-RBBWPoffset-rPUCCHNCS, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS If rPUCCH8=1 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon - the UE determines the PRB index of the PUCCH transmission in the first hop as NBWPsize-1-RBBWPoffset-rPUCCH-8NCS and the PRB index of the PUCCH transmission in the second hop as RBBWPoffset+rPUCCH-8NCS - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as

According to the above, the frequency range of the initial uplink BWP between the first and second hop may exceed the maximum BW of the RedCap UE, possibly causing PUSCH fragmentation as shown in (a) of Figure 2-2. The gNB may mitigate the impact of PUSCH fragmentation by proper scheduling; this, however, may not be always possible. As an additional option, the gNB may choose to disable PUCCH frequency hopping for RedCap UE(s), preventing the fragmentation issue as shown in (b) of the same figure.

Figure 2-2 PUSCH fragmentation and prevention by disabling PUCCH frequency hopping

Proposal 5: Support disabling PUCCH frequency hopping during initial access.

# Conclusion

This contribution has discussed several remaining issues of reduced maximum bandwidth for RedCap UEs. The following proposals have been made:

Proposal 1: Separate initial DL BWP for RedCap UEs is configured/defined during initial access.

Proposal 2: Support a configuration of CORESET and CSS(s) in the initial DL BWP for RedCap UEs.

Proposal 3: Separately configured initial DL BWP for RedCap UEs does not need to contain the entire CORESET #0.

Proposal 4: Support SSB transmission in the initial DL BWP for RedCap UEs.

Proposal 5: Support disabling PUCCH frequency hopping during initial access.

# References

- [1] Chairman’s Notes, 3GPP WG1 #105-e