3GPP TSG RAN WG1 #106-e		                                    R1-2107810

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.6.1.2

Source:	InterDigital, Inc.

Title:	Reduced number of Rx branches for RedCap UEs

Document for:	Discussion

# Introduction

In RAN1 #105-e, reduced number of Rx branches for RedCap UEs was discussed and the following were agreed [1]:

Agreement:

- Redcap UE is mandated to support at least DCI format 0_0/1_0.

Agreement:

For UE capability signalling, the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework.

- Detailed signalling is up to RAN2

Conclusion

- No consensus to support early identification of the number of Rx branches in Msg1/Msg3/MsgA for Redcap UE in Rel-17

Agreement:

Regarding DCI format 0_1/1_1 and DCI format 0_2 and 1_2,

- DCI format 0_1/1_1 are mandatory as in legacy. DCI 0_2/1_2 are optionally supported.

In this contribution, we discuss further details of reduced number of Rx branches for RedCap UEs.

# Discussion

One issue that has been discussed is whether reducing the number of Rx antennas causes a noteworthy increase in PDCCH blocking probability and if a mitigation mechanism is necessary. To elaborate on this issue, Figure 2-1 illustrates the blocking probability for 1 Rx and 2 Rx antenna branches and as a function of the number of UEs in good (solid curves) and poor (dashed curves) coverage. In the simulations, 32 CCEs and 18 search space candidates are assumed with the distribution [6 6 2 2 2] for aggregation levels [1 2 4 8 16]. We can see from the figure that the blocking probability is higher for larger number of UEs and the relative increase in blocking probability is more significant for UEs in good coverage. We should note that the exact blocking probability will depend on many factors such as the deployment scenario, number of RedCap UEs, SINR distribution, etc. In addition, existing mechanisms including flexible CORESET/search space configuration as well as cross-slot scheduling could handle the blocking probability issues as network could configure search spaces in non-overlapping time/frequency regions for different group of RedCap UEs if needed. Therefore, no additional mechanisms to reduce blocking probability seems to be needed at this stage.

Proposal 1: No additional mechanism is introduced to optimize PDCCH blocking probability.

Figure 2-1 PDCCH blocking probability for good and poor coverage UEs with 1 and 2 RX antenna branches

It has been agreed to support DCI formats 0_1/1_1 as mandatory and DCI formats 0_2/1_2 as optional for RedCap UEs. As these formats can already provide a high degree of configurability, we think DCI field optimization is not necessary at this stage.

Proposal 2: DCI field optimization is not introduced for RedCap UEs.

# Conclusion

This contribution has discussed reduced Rx branches for RedCap UEs. The following have been proposed:

Proposal 1: No additional mechanism is introduced to optimize PDCCH blocking probability.

Proposal 2: DCI field optimization is not introduced for RedCap UEs.

# References

- Chairman Notes, RAN1 #105-e