3GPP TSG RAN WG1 Meeting #106-e									R1-2107834

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.3.3

Source:	TCL Communication

Title:	Intra-UE Multiplexing/Prioritization

Document for:	Discussion and decision

- Introduction

In RAN1#105 e-meeting[1] and RAN1#104bis e-meeting[2] , agreements of Intra-UE Multiplexing and Prioritization are achieved as follows:

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2,

- For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding. Down-select from the two options:

  - Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit.

  - Option 2: Reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding.

- For HP HARQ-ACK or LP HARQ-ACK >2 bit(s), HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.3.3 or Clause 5.3.1.

- FFS rate matching equation and RE mapping rules for PF2/3/4. Rel-15 is baseline if available.

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2, treat the two bits as HARQ-ACK bits with High priority.

·           Rel-15 design (for PF0 and PF1) is baseline.

·           Note: QC has strong concern on above scheme. The scheme cannot provide unequal error protection between the HP bit and LP bit hence could suffer from performance degradation for the HP bit. QC accept the scheme for the sake of progress in RAN 1 with the concern on the performance reserved.

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs.

- FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s).

- (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

  - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

  - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs.

- It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

Based on the above agreements, we present some further analyses on intra-UE multiplexing and prioritization.

- Discussion

Intra-UE multiplexing/prioritization has been discussed in Release 16. However, due to the time limitation, only the prioritization of intra-UE collision handling was discussed in Release 16, and intra-UE multiplexing is one of the Work items in Release 17. In the previous release, it has been concluded that two-level priority is supported for different services (URLLC/eMBB) when the collision happens between uplink transmissions the corresponding subsequent operations could be determined based on different priorities. In previous meetings, several agreements have been achieved, and the discussed scenarios could be divided into two categories, which are UCI multiplexing on PUCCH and UCI multiplexing on PUSCH. In this section, we will analyze the details and solutions for these two directions and the simultaneous PUCCH/PUSCH transmissions.

UCI multiplexing on PUCCH

Explicit indication for enabling multiplexing

Intra-UE prioritization and multiplexing have always been critical issues for URLLC. In Release-16, intra-UE prioritization has been specified and intra-UE multiplexing is in discussion in Release-17. First of all, considering the compatibility of the two releases, it should support one indication to enable multiplexing procedures between UCIs with different priorities. Furthermore, the main principle of intra-UE uplink collision handling is to guarantee the latency and reliability of high priority transmission. To this end, it would bring more benefits if the network could indicate whether multiplexing should be performed for UL transmissions with different priorities. For example, the enabling indication can be given according to the payload size of the LP UCI, when the LP UCI occupies a certain number of bits, intra-UE multiplexing should be enabled. For another example, if the priority or channel conditions in the UE result in the dropping behavior of low-priority UL transmissions frequently, intra-UE multiplexing should be enabled. And the indication could be indicated explicitly, and multiplexing of UCIs could be scheduled by DCI or RRC configuration. However, regarding the two approaches, dynamically indicated in DCI would cost more DCI resources, and it is not feasible for the fallback DCI format. DCI miss detection may also lead to extra issues. In addition, the benefit is not clear if the enabling/disabling indication is indicated by DCI together with RRC signaling, and the DCI miss detection issue still exists. Therefore, semi-statically indicated by RRC configuration is preferred.

Proposal 1: Support explicit indication to enable multiplexing procedure between HP UCI and LP UCI via RRC configuration.

Resource mapping rules

For multiplexing high-priority UCI and low-priority UCI on one PUCCH, there may exist one scenario where the total UCI bits exceed the payload of the multiplexed PUCCH resource. The solution for the above scenario was discussed in the previous meeting and the following options were proposed.

- Option 1a: LP UCI is dropped

- Option 1b: LP HARQ-ACK is partially dropped

- Option 1c: LP HARQ-ACK is compressed/bundled

For option 1a, drop the LP UCI may cause less specification impact and be easy for implementation. However, always dropping low-priority UCI may lead to a negative impact on the performance of low-priority services. Therefore, option 1b and option 1c are preferred. At least low-priority UCI has an opportunity to be transmitted, and at the same time there is no reliability degradation for high-priority UCI transmission.

In addition, for Option 1b, partially dropping of low-priority UCI transmission should not only focus on HARQ-ACK feedback but all UCI types. When PUCCH resources are insufficient, a specific UCI type or some UCI types can be dropped first. The dropping behavior should be performed in a certain dropping order. For example, the dropping order of UCI of the same priority may be as follows, CSI->SR->HARQ-ACK. In this case, when the PUCCH resources are insufficient to satisfy all the multiplexed HP UCI and LP UCI, the low-priority CSI should be dropped first.

For Option 1c, compressing or bundling low-priority HARQ-ACK feedback can reduce signaling overhead, and has no effect on the reliability of high-priority transmission. In this case, bundling low-priority HARQ-ACK bits is a feasible solution. Instead of dropping HARQ-ACK feedback with low priority if the PUCCH resource is not available, bundling the low priority HARQ-ACK feedback into one bit could save a lot of signaling overhead, and the logical ‘OR’ operation could be considered.

Furthermore, the combination of Option 1b and Option 1C is also considerable.

Proposal 2: If the total UCI bits exceed the payload of the multiplexed PUCCH resource, partially dropped low priority UCI and/or compressed/bundled low-priority HARQ-ACK should be supported.

Multiplexing timeline and latency

The main purpose of supporting multiplexing for UCIs with different priorities is to improve the transmission performance and reliability of low-priority UL transmissions. However, regarding the strict requirements on latency and reliability of URLLC traffic service, multiplexing for different priorities should be based on the fact that this multiplexing will not have any negative impact on high-priority UL transmissions. Otherwise, low-priority transmissions should be dropped. For this reason, the multiplexing of UCIs of different priorities should be allowed only when the ending symbol of the PUCCH carrying the multiplexed UCI is not later than the ending symbol of the PUCCH carrying the high-priority UCI.

For example, as specified in Figure 1, when LP UCI overlaps with HP UCI and multiplexing is performed between these two UCIs, the multiplexed UCI should end no later than high-priority UCI. As shown in Figure 1, multiplexed UCI 1 is accepted. However, for multiplexed UCI2, which ends after HP UCI is unacceptable.

Figure 1 Multiplexed UCI should end no later than HP UCI

Proposal 3: Multiplexing for UCIs with different priorities should only be allowed when the PUCCH carrying the multiplexed UCI ends no later than the PUCCH carrying high-priority UCI.

More than two overlapping PUCCH/PUSCH

In Rel-16, during RAN1#99 meeting, we already have one conclusion to resolve collision between UL transmissions, and a UE performs the following:

- Step 1: Resolve collision between UL transmissions with same priority.

- Step 2: Resolve collision between UL transmissions with different priorities.

This conclusion could be used to deal with more than two overlapping channels. However, it is not a generic approach that applies well in all scenarios. For example, when three UL transmissions overlap with each other, and two of them are high-priority transmissions, the collision handling between these two high-priority UL transmissions with no latency and reliability impact needs to be discussed. For another example, as shown in Figure 2, two UL UCIs overlap with one LP UCI, respectively. If HP UCI1 and LP UCI do not satisfy the timeline condition to be multiplexed and LP UCI should be canceled by HP UCI1. In this case, even if the LP UCI and HP UCI2 satisfy the timeline condition to be multiplexed, the LP UCI has no chance to be transmitted.

Figure 2 Processing order for more than 2 overlapping channels

In order to improve the performance of low priority transmission, one checking/multiplexing step between channels of different priorities could be considered. One principle should be noticed that this checking/multiplexing step should have no negative influence on the high priority transmission. And we assume that the multiplexing/prioritization (if any) between overlapping channels of the same priority is already done. On the basis of the existing agreement, after resolving the collision between UL transmissions with the same priority, a single checking/multiplexing step between channels with different priorities can be performed. As the example in Figure 2, the LP UCI can select the proper HP UCI to be multiplexed. In addition, suitable adjustments for some specific UCI types should be further studied, i.e., the multiplexing between HARQ-ACK with different priorities can be performed firstly.

Proposal 4: Regarding the scenario of multiplexing more than two overlapping channels, one checking/multiplexing step between channels of different priorities after multiplexing/prioritization (if any) between overlapping channels of the same priority is already done should be further studied.

UCI multiplexing on PUSCH

As discussed above, an explicit indication to enable multiplexing procedures is necessary. The network can indicate to disable multiplexing behavior between UL transmissions of different priorities and to drop lower priority UL transmissions to reduce the negative impact on higher priority UL transfers. However, it is not concluded that how to configure this enabling indication. An alternative is to configure the multiplexing procedure by means of the beta-offset. When the beta-offset is set to 0, it means that there are no UCI to be multiplexed on the PUSCH. However, to be consistent with the enabling/disabling multiplexing indication between PUCCHs with different priorities, an explicit indication is desirable. Similar to the discussion in 2.1.1, this indication can be scheduled via DCI or RRC configuration. With respect to the two approaches, dynamically indicated in DCI would cost more DCI resources, and DCI miss detection may also lead to extra issues. Therefore, semi-statically indicated by RRC configuration is preferred.

Proposal 5: RRC configuration for enabling UCI multiplexing on PUSCH with different priorities should be supported.

When multiplexing a low-priority UCI on a high-priority PUSCH, there may exist a problem that the resource for multiplexing may not be sufficient for the entire transmission of UCI and PUSCH. Dropping the entire low-priority UCI is the simplest and most straightforward solution, but it also leads to the negative impact on the performance of low priority services and unnecessary retransmissions. In this case, partially dropping or bundling of low-priority UCIs can be considered. For partially dropping low priority UCI, the multiplexing priority order for different UCI types or the priority order within the same UCI type needs to be clarified. Similar to the analysis in section 2.1.2, LP UCI compressed/bundling could be considered either.

Proposal 6: For the multiplexing between low priority UCI and high priority PUSCH, if the resource is not sufficient for the multiplexing, considering bundling or partially drop the low priority UCI.

One agreement has been achieved, which is to reuse Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline requirements for Rel-17 intra-UE PUCCH/PUSCH multiplexing with different priorities. In addition, for the latency requirement of multiplexed PUSCH, multiplexing for UCI and PUSCH with different priorities should only be allowed when the ending symbol of multiplexed PUSCH is no later than the ending symbol of high-priority UCI. The principle of multiplexing behavior for different priorities should base on the fact that this multiplexing does not have any impact on high-priority UL transmission. Otherwise, low-priority transmission should be dropped.

Proposal 7: Multiplexing for UCI and PUSCH with different priorities should only be allowed when the ending symbol of multiplexed PUSCH is no later than the ending symbol of high-priority UCI.

Simultaneous PUCCH/PUSCH transmissions

In previous RAN1 meetings, the following agreements are achieved for simultaneous PUCCH/PUSCH transmissions.

Agreements:

Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA.

- FFS how to trigger this function.

- FFS for intra-band CA.

Agreements:

Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group

- FFS: dynamic indication

It is agreed to support simultaneous PUCCH/PUSCH transmissions on different cells for inter-band CA, and this capability could be RRC configured within the same PUCCH group. Regarding the dynamic indication for enabling the simultaneous PUCCH/PUSCH transmissions, the benefit is not clear. And additional solutions are required to avoid DCI miss detection. Therefore, dynamic indication for simultaneous PUCCH/PUSCH should not be supported.

Proposal 8: Dynamic indication for simultaneous PUCCH/PUSCH should not be supported.

Furthermore, the simultaneous PUCCH and PUSCH transmission is supported in Release-17. Accordingly, the PHR for PUCCH and PUSCH transmission should be discussed. There is no available PHR type for the PUCCH transmission only in NR. Therefore, PHR for simultaneous PUCCH and PUSCH transmission should be further studied.

Proposal 9: PHR for simultaneous PUCCH and PUSCH transmission should be further studied.

- Conclusions

The following proposals have been made in this document.

Proposal 1: Support explicit indication to enable multiplexing procedure between HP UCI and LP UCI via RRC configuration.

Proposal 2: If the total UCI bits exceed the payload of the multiplexed PUCCH resource, partially dropped low priority UCI and/or compressed/bundled low-priority HARQ-ACK should be supported.

Proposal 3: Multiplexing for UCIs with different priorities should only be allowed when the PUCCH carrying the multiplexed UCI ends no later than the PUCCH carrying high-priority UCI.

Proposal 4: Regarding the scenario of multiplexing more than two overlapping channels, one checking/multiplexing step between channels of different priorities after multiplexing/prioritization (if any) between overlapping channels of the same priority is already done should be further studied.

Proposal 5: RRC configuration for enabling UCI multiplexing on PUSCH with different priorities should be supported.

Proposal 6: For the multiplexing between low priority UCI and high priority PUSCH, if the resource is not sufficient for the multiplexing, considering bundling or partially drop the low priority UCI.

Proposal 7: Multiplexing for UCI and PUSCH with different priorities should only be allowed when the ending symbol of multiplexed PUSCH is no later than the ending symbol of high-priority UCI.

Proposal 8: Dynamic indication for simultaneous PUCCH/PUSCH should not be supported.

Proposal 9: PHR for simultaneous PUCCH and PUSCH transmission should be further studied.

- References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #105-e.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104b-e.