3GPP TSG RAN WG1 Meeting #106-e			     R1-2107848

Electronic Meeting, August 16th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Beam based operation for new SCSs for NR from 52.6 to 71 GHz

Agenda Item:	8.2.4

Document for: 	Discussion and Decision

- Introduction

At RAN#91-e meeting, the revised WID on NR operation from 52.6 to 71GHz was approved with the objectives as follows [1]:

- Physical layer aspects including [RAN1]: - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation - Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided

In this contribution, beam based operation for new SCSs for NR 52.6- 71GHz is discussed.

- Discussion

  - Timing associated with beam based operation

With shorter symbol/slot length due to higher SCSs applied in NR52.6-71GHz, timing parameters associated with beam operation need to be investigated. Following agreements were made at RAN1#104-e and RAN1#104b-e.

Agreement (RAN1#104-e): - Further study new parameter values for at least the following parameters: - maxNumberRxTxBeamSwitchDL - Additional beam switching time delay d for triggering AP-CSI-RS when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS - Study whether/how to introduce a beam switching gap between signals/channels - FFS: condition to apply including potential UE capability definition - Study should account for inputs from RAN4 Agreement (RAN1#104b-e): Introduce new parameter values for additional beam switching time delay d, when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS or PDSCH Agreement(RAN1#104b-e): For timeDurationForQCL, beamSwitchTiming and beamReportTiming, - Following candidate values of FR2 are reused for 120 kHz: - timeDurationForQCL: 14 and 28 symbols - beamSwitchTiming: 14, 28, 48, 224 and 336 symbols - beamReportTiming: 14, 28 and 56 symbols - For 480 kHz - Support at least the candidate values for 120 kHz scaled by 4x - FFS: Support for additional candidate value(s) less than maximum of 4x (candidate values for 120 kHz) - For 960 kHz - Support at least the candidate values for 120 kHz scaled by 8x - FFS: Support for additional candidate values(s) less than maximum of 8x (candidate values for 120 kHz) - FFS: UE capability signaling details - Note: The scaled values 224 and 336 symbols for beamSwitchTiming are used as in Rel-16 (defined in Rel-15 with updates in Rel-16).

As per the agreements, candidate values scaled based on values for 120kHz SCS for FR2-1 were agreed. We are open to introduce additional values, e.g., smaller values, if necessary. Smaller values can be beneficial as it would improve scheduling efficiency, while it may be difficult to support from implementation perspective in practice. Whether/how much it is practical should be taken into account carefully.

For parameter maxNumberRxTxBeamSwitchDL, it defines the number of Tx and Rx beam changes UE can perform on the band within a slot. UE shall report one value per each subcarrier spacing supported by the UE. Value range is defined per SCS from 15 kHz to 240 kHz SCS in the current specification as follows. Values for new SCSs in 52.6 – 71 GHz, i.e., 480 and 960 kHz SCS, need to be defined in this WI. Considering shorter slot length with higher SCS, different value range for maxNumberRxTxBeamSwitchDL than the one for the existing SCSs may need to be considered for 480/960 kHz SCS depending on the RAN 4 conclusion on “Minimum duration between beam switches”.

maxNumberRxTxBeamSwitchDL                   SEQUENCE {

scs-15kHz                       ENUMERATED {n4, n7, n14}                OPTIONAL,

scs-30kHz                       ENUMERATED {n4, n7, n14}                OPTIONAL,

scs-60kHz                       ENUMERATED {n4, n7, n14}                OPTIONAL,

scs-120kHz                      ENUMERATED {n4, n7, n14}                OPTIONAL,

scs-240kHz                      ENUMERATED {n4, n7, n14}                OPTIONAL

}

For beam switching time delay d for triggering AP-CSI-RS by a PDCCH with smaller SCS than that for the AP-CSI-RS, it is currently defined for PDCCH with SCS from 15 to 60 kHz only, as shown on the table below. Considering that 480 and 960 kHz SCS are optionally supported for NR in 52.6 – 71 GHz, it should be possible to trigger AP-CSI-RS with 480/960 kHz by a DCI with smaller SCS (e.g. 120 kHz). Therefore, we believe it is necessary to add the definitions for PDCCH with at least 120 kHz SCS.

| µPDCCH | d [PDCCH symbols] |
| --- | --- |
| 0 | 8 |
| 1 | 8 |
| 2 | 14 |

Nothing is specified for beam switching between contiguous transmissions in Rel-15/16 from RAN1 perspective since the scheduling strategy and the CP duration are considered to be sufficient to avoid any issue related to the beam switching time. However, with 480/960 kHz SCS, the CP length may not be long enough to cover the whole beam switching time duration. For beam switching between SRS/PUCCH/PUSCH, the needed gap depends on “transient period” and “UE beam switching time (beam direction switch only)”, which is still under discussion in RAN4 for 52.6 – 71 GHz band. If such time durations required for beam switching are relatively longer compared to CP duration with 480/960 kHz SCS, guard period should be defined to ensure sufficient time for beam switching between SRS/PUCCH/PUSCH. In that case, as well as the duration of such guard period, how to define/ensure such guard periods also need to be discussed, e.g., in scheduling point of view specified in RAN1 or in RF requirements specified in RAN4.

SRS antenna switching gap also needs to be discussed for NR in 52.6 – 71 GHz. In FR1 and FR2-1, minimum guard period between two SRS resources of an SRS resource set for antenna switching is defined for each SCS from 15 kHz to 120 kHz (as in the following table). Value(s) for the SRS antenna switching gap should also be defined for 480 and 960 kHz SCS.

Table: The minimum guard period between two SRS resources of an SRS resource set for antenna switching

|  |  | Y [symbol] |
| --- | --- | --- |
| 0 | 15 | 1 |
| 1 | 30 | 1 |
| 2 | 60 | 1 |
| 3 | 120 | 2 |

Proposal 1: For timing parameters associated with beam based operation,

- New value range for maxNumberRxTxBeamSwitchDL may need to be considered for 480/960kHz SCS depending on the RAN 4 conclusion on “Minimum duration between beam switches”.

- New parameter values need to be defined for beam switching time delay d for triggering AP-CSI-RS by a PDCCH with a smaller subcarrier spacing than that for AP-CSI-RS.

- For beam switching between SRS/PUCCH/PUSCH, whether/how to define the beam switching gap depends on RAN4 conclusion on “transient period” and “UE beam switching time (beam direction switch only)”.

- Value(s) for the SRS antenna switching gap should be defined for 480 and 960kHz SCS.

  - Beam indication/application for multi-PDSCH/PUSCH scheduled by single DCI

How to indicate the beam (i.e., TCS state/SRI) for multi-PDSCH/PUSCH scheduled by one DCI was discussed at RAN1#104b-e. It was agreed to use single TCI state/SRI in DCI for multiple PDSCHs/PUSCHs for single TRP case. The remaining issue is the one for multi-TRP case. In our view, similar principle for single TRP case should be applied for the multi-TRP case, i.e. single TCI state/SRI for each TRP.

Agreement: For multiple PDSCHs/PUSCHs scheduled by a single DCI, at least for single TRP, support indication of only a single TCI state/SRI in DCI FFS: number of TCI states/SRIs in a single DCI scheduling multiple PDSCHs/PUSCHs for multi-TRP

Another issue for multi-PDSCH scheduling is beam application considering the offsets between PDSCH and PDCCH may be different for different PDSCHs. In Rel-16, if the offset between PDSCH and PDCCH is less than timeDurationForQCL, UE will assume the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE.

Agreement: Further study the following: - For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL. - For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when all of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL - Note: If the current Rel-16 behavior would be extended to multiple-PDSCH scheduling, it could result in a different QCL assumption for each PDSCH due to the fact the that the CORESET with the lowest ID can be different for different slots, resulting in a potentially different TCI state for each slot - Note: Applicability to multi-TRP can be discussed further

The key point is whether UE determines TCI state for each PDSCH independently based on corresponding PDCCH-PDSCH offset and/or corresponding default TCI state. In Rel-16, it was specified that the same TCI state indicated by DCI will be applied for each PDSCH if multi-slot PDSCH is configured. There is no specification on whether to apply the same TCI state for each PDSCH if the indicated TCI state can’t be applied for the first PDSCH due to the offset between PDSCH and PDCCH is less than the threshold. However, it was clarified for MTRP case in section 5.1.5 of TS 38.214 that “When the UE is configured by higher layer parameter repetitionScheme set to 'tdmSchemeA' or is configured with higher layer parameter repetitionNumber, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion.”. Therefore, we think similar approach for the beam application conclusion of MTRP PDSCH repetition should be applied for discussed cases here, i.e. the same TCI state as the one for the first PDSCH should be applied to each scheduled PDSCH.

Proposal 2: For beam indication/application for multi-PDSCH/PUSCH scheduled by single DCI,

- For multi-PDSCH/PUSCH scheduling with a single DCI for multi-TRP, support indication of single TCI state/SRI for each TRP.

- For multi-PDSCH scheduling, regardless of the offset between scheduling DCI and PDSCHs relative to timeDurationForQCL, TCI states for PDSCHs scheduled by a single DCI follows the TCI state applied for the first PDSCH.

  - Beam management enhancement

In 52.6 – 71 GHz, using narrower beams can help to obtain more beamforming gain to cope with large propagation loss. Beam management enhancement with narrower beam operation can be considered for 52.6-71GHz system. In Rel-16, the number of CSI-RS resources for beam management over all resource sets is no more than 128, and the number of CSI-RS resources in a resource set is up to 64. With narrower beams, whether the number of beams for a resource set and/or across all resource sets are sufficient or not may need to be discussed.

Moreover, in Rel-16, the number of reported beams for each reporting setting in one report instance is up to 4. If more beams are configured in 52.6 – 71 GHz, the number of reported beams may not be sufficient to perform beam management properly. It may also need to be discussed in this WI.

Proposal 3: For beam management in 52.6-71GHz, discuss the following:

- whether to increase the number of configured CSI-RS resources for beam management.

- whether to support reporting more than 4 beams for beam reporting in one report instance, if the number of configured CSI-RS resources in a resource set for beam management is increased.

  - Beam failure detection and recovery enhancement

Beam failure recovery mechanism (BFR) is supported in Rel-15/16 NR (for PCell/PSCell in Rel-15 and for SCell in Rel-16) to cope with beam mis-matching within a coverage. In 52.6 – 71 GHz, assuming narrower beam width than that in lower frequency, there may be more opportunities where BFR needs to be performed, i.e. beam mis-matching may happen more frequently. Thus, the current BFR procedure may need to be enhanced to improve the efficiency of the BFR procedure. For example, in the current BFR procedure a candidate beam set can be configured, where the number of beams is limited to at most 64. More beams may need to be included for BFR in 52.6 – 71 GHz.

Another point is the gap time to apply new beam after the reception of BFR response from gNB. In Rel-15/16, UE can apply the new beam indicated by gNB after 28 symbols from the reception of BFR response. Since higher SCS will be used in 52.6 – 71 GHz, the absolute time duration of 28 symbols guard period would be shorter than that in case with lower SCS, and such shorter guard period may be inappropriate for UE. Whether the 28 symbols work or not may also need to be discussed.

During BFR procedure, UE monitors periodic CSI-RS and/or SSB to detect beam failure based on estimated link quality. However, such periodic transmission may not be ensured in unlicensed spectrum due to channel access procedure (e.g., LBT failure). Monitoring RS or channel other than such periodic transmissions may need to be considered. On the other hand, the probability of LBT failure is unclear, and it may be quite low in higher frequency band assuming the use of narrower beam for transmissions. Then the necessity itself of such aperiodic signal monitoring should be discussed at first.

Proposal 4: Beam failure detection/recovery procedure in NR 52.6-71GHz can consider following potential enhancements,

- whether to increase the number of candidate beams included in set

- whether to introduce a new time gap to apply new beam configuration after receiving BFR response from gNB

- whether to introduce aperiodic RS monitoring for beam failure detection

- Conclusion

In this contribution, we discussed the beam based operation in 52.6-71GHz. Based on the discussion, we made following proposals.

Proposal 1: For timing parameters associated with beam based operation,

- New value range for maxNumberRxTxBeamSwitchDL may need to be considered for 480/960kHz SCS depending on the RAN 4 conclusion on “Minimum duration between beam switches”.

- New parameter values need to be defined for beam switching time delay d for triggering AP-CSI-RS by a PDCCH with a smaller subcarrier spacing than that for AP-CSI-RS.

- For beam switching between SRS/PUCCH/PUSCH, whether/how to define the beam switching gap depends on RAN4 conclusion on “transient period” and “UE beam switching time (beam direction switch only)”.

- Value(s) for the SRS antenna switching gap should be defined for 480 and 960kHz SCS.

Proposal 2: For beam indication/application for multi-PDSCH/PUSCH scheduled by single DCI,

- For multi-PDSCH/PUSCH scheduling with a single DCI for multi-TRP, support indication of single TCI state/SRI for each TRP.

- For multi-PDSCH scheduling, regardless of the offset between scheduling DCI and PDSCHs relative to timeDurationForQCL, TCI states for PDSCHs scheduled by a single DCI follows the TCI state applied for the first PDSCH.

Proposal 3: For beam management in 52.6-71GHz, discuss the following:

- whether to increase the number of configured CSI-RS resources for beam management.

- whether to support reporting more than 4 beams for beam reporting in one report instance, if the number of configured CSI-RS resources in a resource set for beam management is increased.

Proposal 4: Beam failure detection/recovery procedure in NR 52.6-71GHz can consider following potential enhancements,

- whether to increase the number of candidate beams included in set

- whether to introduce a new time gap to apply new beam configuration after receiving BFR response from gNB

- whether to introduce aperiodic RS monitoring for beam failure detection

References

- CMCC, RP-210862, Revised WID: Extending current NR operation to 71 GHz, March 2021.