3GPP TSG RAN WG1 #106e			R1-2107855

e-Meeting, August 16th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on timing relationship enhancements for NTN

Agenda Item:	8.4.1

Document for: 	Discussion and Decision

- Introduction

According to the WID [1], the following objectives are listed to be specified in Rel-17 NTN.

Enhancing features to address the identified issues due to long propagation delays, large Doppler effects, and moving cells in NTN, the following should be specified (see TR 38.821): - Timing relationship enhancements[RAN1,RAN2] - Enhancements on UL time and frequency synchronization [RAN1,RAN2] - HARQ - Number of HARQ process [RAN1] - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2] In addition, the following topics should be specified if beneficial and needed - Enhancement on the PRACH sequence and/or format and extension of the ra-ResponseWindow duration (in the case of UE with GNSS capability but without pre-compensation of timing and frequency offset capabilities) [RAN1/2]. - Feeder link switch [RAN2,RAN1] - Beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse [RAN1/2] - Including signaling of polarization mode

Also, at the RAN1#105e meeting, the following was agreed [2].

Agreement: If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH. Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized. Agreement: The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT. - The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac. Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by TTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of NTA,common×Tc and K_mac. How to treat NTA and NTA,offset can be further discussed. Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0. Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed. Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed. Agreement: The K_offset value signaled in system information is always used for - The transmission timing of RAR / fallbackRAR grant scheduled PUSCH - The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI - The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI - FFS: The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI - The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI - FFS: The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI FFS: how to treat additional transmission timings related to fallback DCI formats FFS: how to update this formulation with beam-specific K_offset if beam-specific K_offset is agreed to be supported

In this document, we discuss on timing relationship enhancements for NTN.

- Discussion on Timing relationship

  - On the signaling of K_offset in initial access

For K_offset configured in system information and used in initial access, at least a cell specific K_offset　configuration, which is used in all beams of a cell, has been agreed [5]. One of the remaining issues is how to signal K_offset for initial access. As agreed in [6], for the transmission timing of RAR grant scheduled PUSCH (i.e. Msg3), the slot allocated for the PUSCH is determined by K_offset in addition to K2 and Δ. To achieve this, we consider that K_offset should be signaled before Msg3 and the following options can be considered as in which system information the K_offset should be sent.

- Option 1: K_offset is signaled in MIB.

- Option 2: K_offset is signaled in SIB1.

- Option 3: K_offset is signaled in SIB following SIB1.

Since MIB has only 1 bit left in the current NR specification, it is necessary to redefine MIB for NTN to realize option 1. We do not see any benefits of signaling K_offset in MIB compared to options 2 and 3. Option 2 is a reasonable option because SIB1 is received before Msg3 transmission and resources are surplus compared to MIB. SIB1 can be scheduled in the same way as PDSCH. Option 3 is also reasonable when SIB1 resources are insufficient. On the other hands, if K_offset is signalled in SIB following SIB1, UE may not recognise by SIB1 detection whether the signal is transmitted by TN or NTN. Therefore how UE knows the necessity of receiving K_offset in SIB should be considered. It is also noted that the signalling of K_offset needs to be discussed together with the signalling mechanism of TA related parameters, e.g. TA common. Since they are necessary for initial access procedure and unified signalling mechanism should be considered.

Proposal 1: K_offset is signaled in SIB1 or in SIB following SIB1.

To consider whether K_offset should be beam-specific, let us confirm the relationship between SSB beams and contents of SIB in Rel-15/16 TN firstly. It seems that SIB is cell-specific in NR Rel-15/16 and hence contents of SIB are common among SSB beams. Otherwise, i.e., if SIB1 is beam-specific, UE has to reread SIB1 every time the UE moves the SSB beam, which becomes a problem in the initial access for example.

When considering beam-specific K_offset, it is necessary to clarify the relationship between the satellite SSB beam and the terrestrial cell. As shown in Figure 1, the NW implementation can choose whether (a) mapping one satellite beam to one terrestrial SSB or (b) mapping one satellite beam to one terrestrial cell. In the implementation of (a) (i.e. when making K_offset beam-specific), it is necessary to put K_offset for all SSB beams in SIB in order not to change the above relationship between SSB beams and contents of SIB. This idea is not preferred from a payload perspective. Meanwhile, if gNB puts K_offset only for a part of SSB beams in SIB, the above relationship is broken and many spec efforts would be required to handle initial access. Besides, it is unclear if there is any merit in the implementation of (a) going so far as changing the relationship between SSB beams and contents of SIB. If it is claimed that a benefit is lower scheduling latency in initial access since the appropriate K_offset value for each beam is different in NTN, we consider there is no motivation to optimize the access latency in the initial access stage.

On the other hand, the operation of beam-specific K_offset in system information can be realized easily by the implementation of (b) because the beam-specific K_offset looks cell-specific system information from the terrestrial cell, and the relationship between SSB beams and contents of SIB is maintained in NTN as well. For these reasons, we believe that beam-specific K_offset is unnecessary for initial access.

Figure 1. Relationship between the satellite beam and the terrestrial cell

Observation 1: Beam-specific K_offset can be achieved by mapping one satellite beam to one terrestrial cell by NW implementation.

Proposal 2: K_offset in initial access is a cell-specific parameter. Beam-specific K_offset is not supported.

  - Updating K_offset after initial access

It was agreed to update the K_offset after initial access at the RAN1#104-e meeting [4], but further discussion is needed on how to update. We support UE-specific update of K_offset via RRC signaling as a result of considering the required update interval of K_offset.

To confirm that there is no problem updating K_offset via RRC signaling, let us consider the required update interval of it. Appropriate K_offset might be changed due to high-speed movement of LEO, which directly affects the interval of K_offset updates. In particular, K_offset needs to be updated if LEO moves away from the UE and the propagation delay increases by the amount corresponding to K1 or K2. Also, considering the efficiency of resources, NW implementation that updates K_offset more frequently is conceivable. However, this update interval is on the order of seconds at the earliest. The reason is that the movement time of LEO corresponding to the slot length is about 2.8 s even in the worst case. Specifically, as an extreme example that is most affected by satellite movement, we consider the case of LEO at an altitude of 600 m as shown in Figure 2. In the current 3GPP standard, a slot length is 125 μs, which is the shortest at SCS 120 kHz. This duration corresponds to a propagation distance of 37.5 km. Considering when the elevation angle of UE and gNB from the satellite is 10 degrees as the situation most affected by the movement of LEO, the movement time of LEO corresponding to the change of the propagation distance (feeder link plus service link) by 37.5 km corresponds to about 2.8 s. From this analysis, an update interval of several seconds would be feasible. Note that in actual system non-optimal K_offset can still be used as long as K1+K_offset indicates an appropriate slot for the UE. This means that gNB can adopt a more update interval at the cost of a bit degradation of latency performance.

Figure 2. Analysis of the movement time of LEO corresponding to the slot length when SCS is 120kHz

Therefore, there is no problem updating K_offset via RRC signaling on the order of seconds at the earliest. A RRC parameter is defined for this purpose and UE uses the parameter instead of K_offset for initial access if provided; otherwise, K_offset used in initial access can be used.

Observation 2:  Update interval of K_offset is on the order of seconds at the earliest, thus there is no problem updating K_offset via RRC signaling.

Proposal 3: A RRC parameter to configure UE-specific K_offset.

- If this parameter is provided, the UE uses the parameter as K_offset.

- Otherwise, the UE uses K_offset provided in initial access.

  - K_offset value determination

For determination of cell-specific K_offset in system information discussed at the RAN1#104bis-e meeting [3], two options were agreed as described in Section 1, while further down-selection is discussed in this meeting.

We believe that Option 1 (Signal one offset value for K_offset) is enough because there seems no motivation for this function with separate parameters (i.e. Option 2). We understand that K_offset is sum of feeder link and service link while TA discussion is separate. TA is compensated per link, so parameter is separate.

Though there is a claim that Option 2 can reduce overhead by deciding the feeder link K_offset based on common TA, we expect little gain from adopting option 2 to reduce overhead since update interval of K_offset is on the order of seconds at the earliest as described in Section 2.2.

Proposal 4: For determination of cell-specific K_offset in system information, signal one offset value for K_offset.

  - On K1/K2 range extension

It was agreed to extend the value of K1 from (0..15) to (0..31) for unpaired spectrum at the RAN1#103-e meeting [5]. The extension is beneficial especially to guarantee all 32 HARQ processes’ feedback can be multiplexed in a HARQ-ACK codebook at least for unpaired spectrum.

On the other hand, we believe it is not needed to extend the value of K1 range for paired spectrum because an UL HARQ feedback corresponding to any DL packet can always be transmitted regardless of the length of propagation delay. Specifically, an UL HARQ feedback corresponding to a DL packet received in DL slot "n" can always be transmitted in UL slot "n+ K1 + K_offset". Similarly, an UL grant in DCI received in DL slot "n" can always find a corresponding UL resource at UL slot "n+K2+K_offset" to transmit a packet on PUSCH.

Proposal 5: Keep the K1/K2 range for paired spectrum.

To increase K1 value range for unpaired spectrum, a new RRC parameter (e.g., dl-DataToUL-ACK-r17) should be adopted at least for DCI format 1_1 because it is impossible to change an existing parameter, i.e. dl-DataToUL-ACK and dl-DataToUL-ACK-r16. For DCI format 1_2, further discussion is needed whether the DCI field size should be reduced.

Proposal 6: A new RRC parameter (e.g., dl-DataToUL-ACK-r17) is adopted at least for DCI format 1_1.

Another remaining issue is whether there is a need to increase the field size for K1 indication in DCI. We believe there is no need to change the DCI field size. To explain a reason, let us confirm the advantages and disadvantages of changing the DCI field size. As an example of TDD frame structure, we consider the structure in ATG mentioned by CMCC [7]. As shown in Figure 3, there are two main options whether (a) multiplexing all HARQ-ACK bits on a single PUCCH by using finer K1 value or (b) allowing multiple HARQ-ACK timings and keep the number of bits required for the DCI field. In option (a), while there is an advantage that UL resources can be used effectively, it will be necessary to increase the DCI field size up to 5 bits. On the other hand, in option (b), while there is an advantage that the DCI field size can be the existing 3 bits, the efficiency of UL resources is reduced. According to the discussion of paired spectrum in RAN1 #104e and RAN1#104bis-e, it seems RAN1 do not strive to support multiplexing all HARQ-ACK bits on a single PUCCH while beneficial from perspective of resource efficiency and power saving. Following the flow of this discussion, we prefer option (b) to option (a).

Proposal 7: Keep the field size for K1 indication in DCI.

Figure 3. Examples of K1 indication

  - PDCCH ordered PRACH

At the RAN1 meetings introducing K_offset for PDCCH ordered PRACH has been discussed. Figure 4 shows an illustration of RO timing with/without K_offset. According the current specification [8], UE shall transmit PRACH at the next PRACH occasion (RO) when RACH procedure is triggered by PDCCH. Therefore, the RO depends on the location of the UE, e.g. UE located at position #1 transmits PRACH at RO of #2 and UE located at position #2 transmits RO of #3 as shown in Figure 4, so that blind detection of gNB would be necessary. In order to avoid the blind decoding, K_offset can be introduced for the selection of RO, so that UE transmit PRACH at the first RO after K_offset from the reception of PDCCH, e.g. UE located at either position #1 or #2 transmits PRACH at RO of #3 as shown in Figure 4. Therefore we conclude a following proposal.

Proposal 8: K_offset is introduced for PDCCH ordered PRACH.

Figure 4. Illustration of RO timing with/without K_offset

  - Beam failure recovery

At the RAN1 #105 e-meeting, it was proposed to study whether timing relationship enhancement is necessary or not for beam failure recovery. According to the current specification [7], BFR timer can be configured up to 200 msec, and 200 msce may not be long enough with considering the RTT of NTN scenario, e.g. more than 200 msec RTT is expected for GEO scenario. On the other hands, considering the typical NTN deployment/scenario, e.g. wide area can be covered by a single beam, and LoS environment between satellite and UE can be expected, updating a beam with BFR may not be a typical case for the NTN scenario. Therefore we conclude a following proposal.

Proposal 9: Extension of BFR timer is not necessary for NTN.

- Conclusion

In this contribution, we discuss timing relationship enhancements for NTN. Based on the discussion we made following observations and proposals.

Observation 1: Beam-specific K_offset can be achieved by mapping one satellite beam to one terrestrial cell by NW implementation.

Observation 2:  Update interval of K_offset is on the order of seconds at the earliest, thus there is no problem updating K_offset via RRC signaling.

Observation 3: When the residual TA error is smaller than a certain level, there are many scheduling timings to prevent the ambiguity for the RO selection.

Observation 4: When the residual TA error is larger than a certain level, there is less/no scheduling timing to prevent the ambiguity for the RO selection.

Proposal 1: K_offset is signaled in SIB1 or in SIB following SIB1.

Proposal 2: K_offset in initial access is a cell-specific parameter. Beam-specific K_offset is not supported.

Proposal 3: A RRC parameter to configure UE-specific K_offset.

- If this parameter is provided, the UE uses the parameter as K_offset.

- Otherwise, the UE uses K_offset provided in initial access.

Proposal 4: For determination of cell-specific K_offset in system information, signal one offset value for K_offset.

Proposal 5: Keep the K1/K2 range for paired spectrum.

Proposal 6: A new RRC parameter (e.g., dl-DataToUL-ACK-r17) is adopted at least for DCI format 1_1.

Proposal 7: Keep the field size for K1 indication in DCI.

Proposal 8: K_offset is introduced for PDCCH ordered PRACH.

Proposal 9: Extension of BFR timer is not necessary for NTN.

References

- RP-201256, “WID: Solutions for NR to support non-terrestrial networks (NTN),” Thales, June 2020.

- 3GPP, RAN1#105e, RAN1 Chairman’s Notes, May 2021.

- 3GPP, RAN1#104bis-e, RAN1 Chairman’s Notes, April 2021.

- 3GPP, RAN1#104e, RAN1 Chairman’s Notes, March 2021.

- 3GPP, RAN1#103e, RAN1 Chairman’s Notes, Nov. 2020.

- 3GPP, RAN1#102e, RAN1 Chairman’s Notes, Sept. 2020.

- 3GPP TS 38.331 V16.4.1, “NR; Radio Resource Control (RRC); Protocol specification,” March 2021.

- 3GPP TS 38.321 V16.4.0, “NR; Medium Access Control (MAC) protocol specification,” March 2021.