3GPP TSG RAN WG1 #106-e			R1-2107877

e-Meeting, August 16th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Resource multiplexing between child and parent links of an IAB node

Agenda Item:	8.10.1

Document for: 	Discussion and Decision

- Introduction

At RAN1#105-e meeting, following agreements on IAB resource multiplexing were made [1].

Agreement For frequency domain multiplexing, H/S/NA configurations for an IAB-node are provided separately in addition to the Rel-16 H/S/NA Agreement DCI Format 2_5 is reused to support soft resource availability indications for frequency-domain resources - FFS: If additional enhancements are necessary Agreement The parent IAB-node is dynamically provided with conditions/parameters to facilitate adaptation between multiplexing operation modes: - FFS: Required number of guard symbols for switching of multiplexing mode (FFS: per timing mode or per multiplexing mode) for IAB-DU - FFS: Signalling procedure - FFS: Required guard band for FDM - FFS: other conditions, e.g. required timing mode, required power control parameters, and preferred TCI. Agreement If an IAB node is configured with a frequency-domain H/S/NA configuration down select between the following options: - Alt. 1 Either the Rel-16 H/S/NA configuration or frequency domain configuration is applied for a given resource - FFS: Whether configurations are switched with per-slot, per-resource type within a slot, or per-symbol granularity - Alt. 2 The Rel-16 H/S/NA configuration and frequency domain configuration are jointly applied Agreement The minimum resource size for configuring the frequency domain granularity is a set of N RBs: - Candidate values for N: {4, 8, 16, other values TBD} - N is at least the # PRBs that are corresponding to the MT’s # PRBs of an RBG). - FFS: Scaling or configuration of N based on system BW or size of IAB-MT BWP Agreement In case of intra-band inter-carrier dual connectivity for both inter-donor and intra-donor scenarios the following are supported: - Reusing the Rel-16 CA TDD prioritization rules in case of UL/DL conflict when applicable - FFS: Whether all prioritization rules apply in case of NR-DC - FFS: Need of new prioritization rules in case of NR-DC - Coordinating the IAB-MT’s TDD configurations to avoid conflicts from different parent nodes in case the child IAB-MT does not support simultaneous TX and RX on different carries - FFS: Coordination for scheduling conflicts involving at least DCI Format 2_0 usage (e.g. usage of flexible symbols) - Exchanging H/S/NA configurations between parent nodes/donors Agreement In case of simultaneous MT/DU operation, - the parent node can dynamically indicate to the child node at least a set of restricted beams at the IAB-DU of the child node - the child node can dynamically report to the parent node a set of recommended beams, not preferred beams, or both recommended and not preferred beams of the IAB-MT of the child node - FFS: Whether the specification supports all reporting combinations. - FFS: Applicability to specific multiplexing cases or specific time-frequency resources - FFS: Additional semi-static signaling - FFS: Per-panel granularity in addition to per-beam granularity - FFS: Relationship between child IAB-MT beam indication and parent IAB-DU beam indication - Note: This does not preclude any enhancements for either DU or MT-based CLI measurement and reports Agreement For an IAB-MT with multiple serving cells (including the case with two parent nodes), a per-cell IAB-DU soft resource is considered as available if the resource is either explicitly indicated (via DCI 2_5), or implicitly determined as available with respect to all serving cells. - If the IAB-DU per-cell soft resource neither explicitly indicated as Available, nor implicitly determined as Available by the IAB-DU with respect to at least one serving cell - Alt 1. The IAB-DU per-cell resource is assumed to be not available - This agreement does not necessarily mean the Rel-16 spec does not support what is described in the main bullet

In this contribution, we discuss enhancements on resource multiplexing between child and parent links of an IAB node to support simultaneous operation of transmission and/or reception.

- Resource management in frequency domain

It was agreed that frequency domain H/S/NA configurations are provided in addition to Rel-16 H/S/NA. Considering different frequency resources may be needed for DU on its child link in different slots and different D/U/F resource types, H/S/NA resource types in frequency domain can be configured per RB set per slot per D/U/F resource type, as shown in Fig. 1.

Proposal 1: Semi-static configuration of H/S/NA resource type in frequency domain can be provided per RB set (e.g., N RBs) per slot per D/U/F resource type.

Whether H/S/NA resource type in frequency domain is configured for every time unit needs to be discussed. To save signaling overhead, frequency domain H/S/NA can be provided only for time units where FDM is needed. For a time unit where FDM is not needed, frequency domain H/S/NA can be absent. Explicit signaling, e.g., one bit for each time unit indicating whether frequency domain H/S/NA is provided, and/or predefined rules, e.g., based on time domain H/S/NA resource type to determine whether frequency domain H/S/NA is provided, can be considered.

Regarding how to apply time domain H/S/NA configuration and frequency domain H/S/NA configuration, for a given symbol, if frequency domain H/S/NA is not provided, time domain H/S/NA can be applied. For a given symbol, if both frequency domain H/S/NA and time domain H/S/NA are provided, whether frequency domain H/S/NA or time domain H/S/NA is applied needs to be determined. From our perspective, some rules taking into account e.g. multiplexing capability, MT/DU’s DL/UL direction, timing mode, power control, and beam indication, can be used. When certain condition is met, e.g. if simultaneous MT-Tx/DU-Tx is supported by IAB node capability, and case6 timing mode is indicated for MT-UL Tx, frequency domain H/S/NA and FDM behavior is applied; otherwise, e.g. if case6 timing mode is not indicated, time domain H/S/NA and TDM behavior is applied.

Figure 1: semi-static configuration of H/S/NA in frequency domain

Proposal 2:  To save signaling overhead, H/S/NA in freq. domain can be provided only for some time units indicated by explicit signaling and/or determined by predefined rules.

- For a given symbol, if frequency domain H/S/NA is not provided, time domain H/S/NA can be applied

- For a given symbol, if both frequency domain H/S/NA and time domain H/S/NA are provided, whether frequency domain H/S/NA or time domain H/S/NA is applied can be determined by predefined rules considering e.g., multiplexing capability, MT/DU’s DL/UL direction, timing mode, power control, beam indication, etc.

It was agreed that DCI format 2_5 is reused to support soft resource availability indications for frequency-domain resources. DCI format 2_5 can provide a joint soft resource availability indication in time and frequency domain as shown in Fig. 2. The soft resource availability can be indicated with the same granularity as semi-static configuration, i.e., per RB set per slot per D/U/F resource type. With DCI format 2_5 framework, ResourceAvailability can be enhanced to indicate availability per RB set per slot per D/U/F resource type instead of per slot per D/U/F resource type in Rel-16. As discussed previously, for a symbol, frequency domain H/S/NA or time domain H/S/NA may be applied. If frequency domain H/S/NA and FDM behavior is applied, the soft resource availability indicated for each RB set on the symbol can be applied. For a symbol, if time domain H/S/NA and TDM behavior is applied, the soft resource availability for the symbol can also be determined from the availability indication per RB set, e.g., a symbol is determined as “IA”, if all the RB sets on the symbol are indicated as “IA”, or if at least one of the RB sets on the symbol is indicated as “IA”.

Figure 2: dynamic indication of soft resource availability in time and frequency domain

Proposal 3: DCI format 2_5 framework can be reused, and ResourceAvailability can be enhanced to indicate availability per RB set per slot per D/U/F resource type.

- Resource management in spatial domain

It was agreed that child node can dynamically report to parent node a set of recommended beams and/or not preferred beams of IAB-MT. For child node’s beam reporting, CSI reporting framework can be reused. With legacy beam reporting, IAB-MT can report N best beams according to L1-RSRP/SINR. Considering the case where no beam suitable for simultaneous MT and DU operation may be found among the N best beams selected and reported with legacy mechanism, it is beneficial that a separate set of beams e.g., M best beams suitable for simultaneous operation are reported in addition to legacy beam reporting, so that parent node can determine and indicate best MT beam separately for the case of simultaneous MT/DU operation and the case of non-simultaneous operation.

Proposal 4: In addition to legacy beam reporting, IAB node can report M best beams that are suitable for simultaneous MT/DU operation via CSI reporting framework.

It was also agreed that parent node can dynamically indicate to child node a set of restricted beams of IAB-DU. In our understanding, the condition of interference between MT and DU may be different when different MT beam is used, so that restricted beams may be different when different MT beam is used. Semi-static configuration of restricted DU beams for each SSB/CSI-RS of MT serving cell via RRC signaling can be considered. For a MT Tx indicated with a spatial relation containing an SSB/CSI-RS as reference RS, restricted DU beams configured for the reference SSB/CSI-RS can be applied.

Proposal 5: Restricted DU beams can be configured for each SSB/CSI-RS of MT serving cell via RRC signaling.

- Adaptation of multiplexing operation

4.1. Impact on multiplexing capability from timing mode

Case#6 and case#7 timing alignment are required for simultaneous MT-Tx/DU-Tx and simultaneous MT-Rx/DU-Rx, respectively. For simultaneous MT-Rx/DU-Rx with case#7 timing, timing alignment can be controlled by IAB node itself i.e., by controlling TA for its child nodes/UEs to align DU-Rx timing with MT-Rx timing. For simultaneous MT-Tx/DU-Tx with case#6 timing, it was agreed at case#6 timing mode is controlled by the parent node. Thus, whether simultaneous MT-Tx/DU-Tx can be performed is impacted by UL Tx timing indicated by parent node, i.e., simultaneous MT-Tx/DU-Tx can be performed only if case#6 UL Tx timing is indicated by parent node, otherwise, simultaneous Tx cannot be performed.

Proposal 6: If case#6 timing mode is required for simultaneous MT-Tx/DU-Tx, simultaneous MT-Tx/DU-Tx can be performed only if case#6 UL Tx timing is indicated by parent node; otherwise, simultaneous Tx cannot be performed.

4.2. Multiplexing capability on DU flexible symbol

For a given multiplexing capability, MT and DU can simultaneously Tx/Rx if transmission/reception direction of MT and DU are aligned. In Rel-16, transmission/reception direction of a DU flexible symbol is decided by IAB node and is not informed to the parent node. In that case, whether/how to perform simultaneous operation with parent node on DU flexible symbol needs to be discussed.

To support simultaneous operation on DU flexible symbol, one of the potential solutions is that IAB node reports transmission/reception directions of DU flexible symbols to the parent node. However, a disadvantage of this method is that the parent node cannot configure/indicate a resource for IAB node MT Tx/Rx unless the report on the DU flexible symbol is available. In order to perform simultaneous operation, IAB node needs to decide and report the transmission/reception direction of the DU flexible resource before the same resource is configured/indicated for MT Tx/Rx. Moreover, additional signaling overhead will be introduced.

Another possible solution is that the parent node can configure/indicate/schedule IAB node MT Tx or Rx on a DU flexible symbol, and IAB node DU will Tx/Rx on the flexible symbol based on MT transmission/reception direction and its multiplexing capability. In this case, IAB node does not need to report transmission/reception direction of DU flexible symbol to the parent node, with saving signaling overhead. Moreover, simultaneous operation is always possible, which is beneficial for resource utilization efficiency although the resource usage of DU flexible symbol may be restricted depending on its multiplexing capability and the resource usage at the parent node.

Proposal 7: On a DU flexible symbol, parent node can configure/indicate IAB node MT Tx/Rx, and IAB node DU will Tx/Rx on the symbol based on MT transmission/reception direction and its multiplexing capability.

4.3. Enhancement on guard symbol

In Rel-16, to resolve a resource conflict between IAB-MT and IAB-DU caused by time misalignment, guard symbols are inserted at MT Tx/Rx and DU Tx/Rx switching for case#1 timing mode. In Rel-17, case#6 and case#7 timing modes are additionally supported. Different timing modes may result in different MT Tx timing and DU Tx timing and therefore result in different numbers of guard symbols are needed at MT Tx/Rx and DU Tx/Rx switching.

In Rel-16, 0~4 guard symbols can be reported by IAB node and provided by parent node. In Rel-17, with case#6 and case#7 timing, whether different value range of the number of guard symbols is needed for the new timing modes needs to be studied.

Additionally, if dynamic switching among different timing modes is supported, an enhancement on multiple indications of guard symbols for different timing modes needs to be studied. For MT Tx timing, case1/6/7 timing need to be considered, and different numbers of guard symbols can be indicated for case1/6/7 MT Tx timing respectively. In last meeting, whether the number of guard symbols is indicated per timing mode or per multiplexing mode was discussed. From our perspective, indicating number of guard symbols per timing mode is preferred, because compared to indication of per multiplexing mode, indication of per timing mode results in less overhead.

Proposal 8: Whether different value range of the number of guard symbols is needed for the new timing modes needs to be studied.

Proposal 9: Guard symbol is provided for each combination of MT case1 Tx/case6 Tx/case7 Tx/Rx and DU Tx/Rx.

4.4. Enhancement on guard band

Guard band was discussed to alleviate interference. In case of simultaneous MT-Tx/DU-Rx, MT-Rx/DU-Rx, DU-Tx/MT-Rx, the interference can be measured by IAB node, and the guard band can be decided by IAB node. Following option can be considered.

- Option1: DU will not Tx/Rx at the guard band and it can be handled by implementation. DU does not need to report the desired guard band to parent node.

- Option2: MT will not Tx/Rx at the guard band and DU needs to report desired guard band to parent node. Meanwhile parent node may need to indicate provided guard band to IAB node.

From our perspective, option1 is preferred which saves signaling of reporting and indication of guard band.

Proposal 10: Guard band can be handled by IAB node implementation.

- Conclusion

In this contribution, we discussed enhancements on resource multiplexing between child and parent links of an IAB. Based on the discussion, we made following proposals.

Proposal 1: Semi-static configuration of H/S/NA resource type in frequency domain can be provided per RB set (e.g., N RBs) per slot per D/U/F resource type.

Proposal 2:  To save signaling overhead, H/S/NA in freq. domain can be provided only for some time units indicated by explicit signaling and/or determined by predefined rules.

- For a given symbol, if frequency domain H/S/NA is not provided, time domain H/S/NA can be applied

- For a given symbol, if both frequency domain H/S/NA and time domain H/S/NA are provided, whether frequency domain H/S/NA or time domain H/S/NA is applied can be determined by predefined rules considering e.g., multiplexing capability, MT/DU’s DL/UL direction, timing mode, power control, beam indication, etc.

Proposal 3: DCI format 2_5 framework can be reused, and ResourceAvailability can be enhanced to indicate availability per RB set per slot per D/U/F resource type.

Proposal 4: In addition to legacy beam reporting, IAB node can report M best beams that are suitable for simultaneous MT/DU operation via CSI reporting framework.

Proposal 5: Restricted DU beams can be configured for each SSB/CSI-RS of MT serving cell via RRC signaling.

Proposal 6: If case#6 timing mode is required for simultaneous MT-Tx/DU-Tx, simultaneous MT-Tx/DU-Tx can be performed only if case#6 UL Tx timing is indicated by parent node; otherwise, simultaneous Tx cannot be performed.

Proposal 7: On a DU flexible symbol, parent node can configure/indicate IAB node MT Tx/Rx, and IAB node DU will Tx/Rx on the symbol based on MT transmission/reception direction and its multiplexing capability.

Proposal 8: Whether different value range of the number of guard symbols is needed for the new timing modes needs to be studied.

Proposal 9: Guard symbol is provided for each combination of MT case1 Tx/case6 Tx/case7 Tx/Rx and DU Tx/Rx.

Proposal 10: Guard band can be handled by IAB node implementation.

References

- RAN1 Meeting #105-e Chairman’s Notes, May 2021