3GPP TSG RAN WG1 #106-e		    R1-2107904

e-Meeting, August 16th – 27th, 2021

Source: 	     Xiaomi

Title:		Discussion on efficient activation and de-activation mechanism for SCell in NR CA

Agenda item:    8.13.2

Document for:  Discussion

Introduction

In RAN1#105e meeting, the efficient activation and de-activation mechanism for SCell was heatedly discussed. Good progress was achieved especially for the temporary RS triggering command. The following agreements were achieved during last meeting:

Agreement For efficient activation of Scells, the triggered temporary RS is aperiodic. Agreement For efficient activation of a Scell (in known Scell case), at least the number of temporary RS bursts is indicated by a field in new MAC-CE - The number of temporary RS bursts is RRC configurable. - FFS: which field in MAC-CE is used and how this field is associated with the number of bursts - For the purpose of designing temporary RS Scell activation, there is no RAN1 specification impact for the case where the number of indicated temporary RS bursts is smaller than what is expected by the UE Agreement To trigger temporary RS for efficient activation of SCells, the contents of the triggering MAC-CE(s) in a single PDSCH provide at least the following information (explicitly or implicitly): - Whether or not temporary RS is triggered - FFS detailed Information of temporary RS, e.g.: - Resources used for triggered Temporary RS - Triggering time offset of triggered Temporary RS - QCL source for triggered Temporary RS - FFS: Detailed signaling structure of the triggering MAC-CE(s) including the down-selection between the following example options and whether the decision should be made in RAN1 or RAN2 - Opt. 1.1: One new MAC CE for both SCell activation triggering and corresponding temporary RS triggering - Opt. 1.2: One R15/16 SCell activation MAC CE for SCell activation triggering and one new MAC CE (in the same PDSCH) for corresponding temporary RS triggering Agreement For efficient activation of a Scell (in known Scell case), the triggering offset of temporary RS is indicated by a field in new MAC-CE - The candidate value(s) of triggering offset(s) is RRC configurable - FFS: which field in MAC-CE is used and how this field is associated with the value of triggering offset Agreement For the reference slot for triggering offset of temporary RS - Option 2: the last DL slot of the to-be-activated Scell overlapping with slot n+k as defined in 38.213 sub-clause 4.3 - FFS: the earliest slot no earlier than the reference slot for a UE to receive a triggered temporary RS Agreement If a UE measures a temporary RS triggered by a MAC-CE during SCell activation procedure, the measurement is performed within the BWP bandwidth of BWP indicated by firstActiveDownlinkBWP-Id

Additionally, the temporary RS structure including QCL source of temporary RS was also discussed without any conclusion because of the absence of detailed inputs from RAN4.

In this contribution, we provide our views on remaining issues of efficient SCell activation/de-activation.

Discussion

In the last meeting, it was agreed to adopt aperiodic manner in time domain for temporary RS in order to expedite SCell activation procedure. However, the structure of temporary RS is still open as lack of complete inputs from RAN4. In RAN1#103 e-meeting, the following agreements was concluded:

Agreements: As working assumption, with respect to efficient SCell activation, reuse existing Rel-15/16 TRS structure for temporary RS - FFS: how many burst/symbols are required for both AGC settling and Time/Frequency tracking for different cases, e.g. FR1 and FR2, known and unknown SCell - A burst of temporary RS is notated as in S5.1.6.1.1 of TS 38.214 - “2-slot with four CSI-RSs resources (4 samples)” for FR1 - either “1-slot with two CSI-RSs resources (2 samples)” or “2-slot with four CSI-RSs resources (4 samples)” for FR2 - The working assumption can be confirmed after RAN4 check. (A LS for such request is planned).

RAN1 and RAN4 now have a better understanding for each other after 2-meeting-long back-and-forth discussion. In [1] and [2], RAN4 provide information related to temporary RS design for different scenarios, e.g. frequency range 1 and frequency range 2, known cell and unknown cell, etc. However, there are still some holes need to be fixed before we have a big picture on temporary RS structure.  From RAN1 perspective, the fundamental functionality of temporary RS is to stratify the requirement on fast AGC settling and T/F tracking. From the current reply from RAN4, the requirement on T-RS burst in different scenarios are variable. Additionally, RAN4 still works on some leftover issues, e.g. unknown cell without continuous active cell in the same band.  In order to pursue a unified solution across different scenarios, the discussion on temporary structure should be deprioritized.

Proposal 1: The structure of temporary RS depends on the inputs from RAN4 and should be deprioritized until RAN4 provides more inputs on the leftover scenarios.

In RAN1#104e meeting, the following working assumption is achieved but is not confirmed in the last meeting:

Working Assumption For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell - FFS: QCL type - FFS: the case of unknown SCell - FFS: other QCL source, e.g. the SSB/P-TRS of another active cell

The main concern is the possible misalignment between network and UE on the cell status, i.e. known cell and unknown cell. However, it is not a standalone case which is also discussed under the umbrella of T-RS burst indication. Accordingly, there is a note to address the concerns related to misalignment issue, which says ‘For the purpose of designing temporary RS Scell activation, there is no RAN1 specification impact for the case where the number of indicated temporary RS bursts is smaller than what is expected by the UE’. If unknown cell and unknown cell is not an issue for RS burst indication, the same logic should be equally applied to the above working assumption.

For a known cell, it is reasonable to take the SSB of the to-be-activated cell as a QCL source for the temporary RS as it is certainly recognized by a UE staying on this cell. There is no additional effort as it is exactly the same as the current QCL configuration. However, SSB cannot provide accurate information for Doppler spread and delay spread as CSI-RS because of the continuous property in time domain.[3] Furthermore, QCL type C and QCL type D is sufficient for the purpose of T/F tracking. Accordingly, QCL type C and Type D can be configured for the temporary RS which is QCLed with SSB.

Proposal 2: For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell

- QCL type C and when applicable, QCL type D can be configured

It was agreed in the last meeting that the temporary RS indication should be carried by MAC CE. There are two related issues needs to be fixed, i.e. what information needs to be conveyed by the MAC CE(s) and how to design MAC CE. From our understanding, the fundamental issue is to realize the two functionalities required by fast SCell activation, i.e. temporary RS triggering and SCell activation. There is no much difference from RAN1 perspective whether the temporary RS triggering and SCell activation are carried by an integrated MAC CE or separate MAC CEs.

Proposal 3: The MAC CE detailed design, i.e. RS triggering and SCell activation are carried by an integrated MAC CE or separate MAC CE, should depend on RAN2’ decision.

From RAN1 perspective, the key point is to guarantee the temporary RS works well. In the other word, we should focus on identifying what information is needed for temporary RS transmission and reception. During RAN1#105 e-meeting, the content of MAC CE was heatedly discussed and some potential information was proposed, which is shown as below：

- Number of TRS burst(agreed)

- Whether or not the TRS is triggered(agreed)

- Resources used for triggered Temporary RS(FFS)

- Triggering time offset of triggered Temporary RS(FFS)

- QCL source for triggered Temporary RS(FFS)

There is no doubt that all the above information is needed for temporary RS. However, it would be a huge burden for RAN2 if all of them are explicitly indicated via MAC CE. On the other hand, we think the situation is pretty similar to the current aperiodic CSI RS triggering, i.e. RRC signalling configure a trigger list for A-CSI which includes the NZP CSI RS resource, the timeline, the QCL information, etc. gNB trigger the A-CSI RS and corresponding CSI (if applicable) via the CSI request information field included in DCI. The CSI request information indicates an entry index of the CSI-AperiodicTriggerStateList. The same mechanism can be applied for temporary RS triggering. To be specific, the aforementioned information can be pre-configured by RRC signalling and MAC CE only needs to indicate an entry index. Consequently, the standard effort for both physical layer and high layer can be reduced as much as possible. One example reflecting the above procedure is shown in Figure 1. Here we are trying to reuse the current mechanism for triggering aperiodic CSI RS as much as possible. An integrated MAC CE including SCell activation indication and RS triggering indication is assumed. In this example, higher layer configures a temporary RS trigger list wherein each entry points to a set of complete information for temporary RS. The information flow starts from temporary RS trigger list is also shown in Figure 1.  It can be observed that the resources used for triggered temporary RS, triggering time offset and QCL source can already be configured by RRC signalling. The flexibility can also be guaranteed via the configured temporary RS trigger list. Furthermore, it would be a big burden for MAC PDU if the above information are indicated by separate information fields in MAC CE. Not only the overhead but also the standard effort would be unexpected high.

Proposal 4: The current mechanism for triggering NZP aperiodic CSI RS should be reused for temporary RS triggering.

- A trigger state list for temporary RS is pre-configured by RRC signalling.

- MAC CE indicates one of the entry index included in the trigger state list.

Figure 1: Example for two-step Temporary RS triggering with integrated MAC CE

Regarding to the timeline of temporary RS triggering, it was agreed that the las DL slot of the to-be-activated SCell overlapping with slot n+k as defined in 38.213 should be taken as the reference slot. It is still a FFS on how to define the earliest slot no earlier than the reference slot for a UE to receive a triggered temporary RS. At the current reference slot, UE already finishes the MAC CE parsing and has the completed information for temporary RS. It again quite similar to the case of triggering NZP aperiodic CSI RS via DCI. The minimum triggering offset in this case is zero, i.e. the triggered NZP aperiodic CSI RS can be transmitted in the same slot wherein the DCI is transmitted. From this point of view, we think the slot on the heels of reference slot should be available for temporary RS. It not only inherits the logic applied in Rel-15/16, but also carter to the motivation of expediting the SCell activation procedure. Of course it depends on network’s decision that which trigger offset is associated with each trigger state.

Proposal 5: The earliest slot for a UE to receive a triggered temporary RS could be the DL slot immediately following the reference slot.

Conclusion

In this contribution, we discuss the remaining issues on efficient SCell activation/de-activation.  Based on the discussion, our proposals are summarized as follows:

Proposal 1: The structure of temporary RS depends on the inputs from RAN4 and should be deprioritized until RAN4 provides more inputs on the leftover scenarios.

Proposal 2: For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell

- QCL type C and when applicable, QCL type D can be configured

Proposal 3: The MAC CE detailed design, i.e. RS triggering and SCell activation are carried by an integrated MAC CE or separate MAC CE, should depend on RAN2’ decision.

Proposal 4: The current mechanism for triggering NZP aperiodic CSI RS should be reused for temporary RS triggering.

- A trigger state list for temporary RS is pre-configured by RRC signalling.

- MAC CE indicates one of the entry index included in the trigger state list.

Proposal 5: The earliest slot for a UE to receive a triggered temporary RS could be the DL slot immediately following the reference slot.

Reference

  - R1-2102300, Reply LS on temporary RS for efficient SCell activation in NR CA, RAN4, Huawei

  - R1-2104170, Reply LS on temporary RS for efficient SCell activation in NR CA, RAN4, Huawei

  - R1-2104497, Discussion on efficient activation and de-activation mechanism for SCell in NR CA, CATT