3GPP TSG RAN WG1 Meeting #106-e                                                                 	   R1-2107949

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.6.2

Source:	Lenovo, Motorola Mobility

Title:	RAN1 aspects for RAN2-led features for RedCap

Document for:	Discussion and decision

# Introduction

In RAN#91e, the updated RedCap WID [1] was agreed. Following objectives are RAN2-led and need RAN1 inputs,

- Specify definition of one RedCap UE type including capabilities for RedCap UE identification and for constraining the use of those RedCap capabilities only for RedCap UEs, and preventing RedCap UEs from using capabilities not intended for RedCap UEs including at least carrier aggregation, dual connectivity and wider bandwidths. [RAN2, RAN1]

- Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1]

- Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

In this contribution, we provide analysis on these RAN2-led objectives, including RedCap UE identification and UE type definition.

# Discussion

## UE identification

Regarding UE identification for RedCap UEs, following working assumption and agreement were agreed in RAN1#105-e,

Working assumption:

- For 4-step RACH, support the early indication of RedCap UEs at least in Msg1.

  - The early indication in Msg1 can be configured to be enabled/disabled

    - FFS How to support enable/disable the early indication

  - FFS details e.g.:

    - separate initial UL BWP

    - separate PRACH resource

    - PRACH preamble partitioning

  - FFS the possibility of supporting Msg3 for the early indication

Agreement: (if the above working assumption is confirmed)

- Early indication of RedCap UEs in Msg1 can be enabled/disabled via SIB

To move forward, it is preferred to confirm the above working assumption on early identification of RedCap UEs.

Proposal 1: Confirm the RAN1#105-e working assumption on early identification of RedCap UEs.

In general, early identification of RedCap UEs through Msg1 is achieved by using separate preambles for RedCap UEs. This includes separate preambles by configuring separate ROs, or separate preambles by preamble partitioning in the shared ROs.

One option in the working assumption is through separate initial UL BWP, which however, cannot achieve early identification alone when ROs are shared between RedCap UEs and non-RedCap UEs even they are in different initial UL BWPs.  For example, the separate initial UL BWP for RedCap UEs is configured to contain legacy ROs of the initial UL BWP for non-RedCap UEs.

Therefore, early identification of RedCap UEs in Msg1 could be discussed regardless of whether separate initial UL BWP is configured for the RedCap UEs, but from if ROs are shared between RedCap UEs and non-RedCap UEs. Specifically,

- If ROs are shared between RedCap and non-RedCap UEs, early identification of RedCap UEs in Msg1 is through preamble partitioning. Similar with RACH partitioning for 2-step RACH, a set of contention-free preambles for non-RedCap UEs are allocated for RedCap UEs for contention-based RACH procedure.

- If ROs are not shared, early identification of RedCap UEs in Msg1 is through the received preambles in the separate ROs for RedCap UEs.

Depending on if the capacity is an issue.

Proposal 2: Support early identification of RedCap UEs in Msg1 through,

- Preamble partitioning with shared ROs,

  - Similar with RACH partitioning for 2-step RACH, a set of contention-free preambles for non-RedCap UEs are allocated for RedCap UEs for contention-based RACH procedure.

- Configuring separate ROs for RedCap UEs in initial UL BWP.

Early identification of RedCap UEs in Msg1 is enabled if separate RACH resource is configured for RedCap UEs. There is no need to have explicit signaling on enable/disable of early identification of RedCap UEs.

Proposal 3: There is no need to have explicit signaling on enable/disable of early identification of RedCap UEs in Msg1.

As in the WID, uplink coverage enhancement solutions specified in the NR Coverage Enhancement WI shall be assumed to be available also to RedCap UEs by default. Rel.17 CE supports requesting Msg3 repetition by configuring separate PRACH resources based on a DL RSRP threshold. Such scheme could be optionally supported by RedCap UEs, and the PRACH resources for RedCap UEs are partitioned correspondingly. It should be noted that although the intention of separate PRACH resource in CE study is for UE requesting Msg3 repetition, it helps also link adaptation for DL data transmission (e.g., for Msg2/Msg4). Besides, identifying RedCap UEs with bad channel quality could help gNB determine whether to restrict the access of such UEs.

Proposal 4: Support early identification of RedCap UEs with different channel conditions for the purpose of link adaptation during initial access and access control of RedCap UEs.

## RedCap UE type definition

Following working assumption on RedCap UE type definition was agreed in RAN1#105-e,

Working assumption:

- RedCap UE type is defined based on one of the following options

  - Option 2: Only include the reduced capabilities that the network needs to know during initial access, if any.

  - Option 4: The corresponding minimum set of the reduced capabilities that one RedCap UE type shall mandatorily support

  - FFS: details of the set of reduced capabilities

To move forward, it is preferred to confirm this working assumption on RedCap UE type definition.

Proposal 5: Confirm RAN1#105-e working assumption on UE type definition for RedCap UEs.

In our view, there is almost no difference between option 2 and option 4 considering that

- The number of Rx branches will not be identified during initial access (see below RAN1#105-e conclusion).

- Legacy 64QAM MCS tables are mandatory while 64QAM low SE MCS table is optionally supported (see below RAN1#105-e agreements). And we don’t think low SE MCS table will be used during initial access.

- Whether UE is FD-FDD or HD-FDD UE will not impact the data transmission and reception during initial access.

Conclusion

- No consensus to support early identification of the number of Rx branches in Msg1/Msg3/MsgA for Redcap UE in Rel-17

Agreements:

- For a RedCap UE, 64QAM MCS tables (Table 5.1.3.1-1 in TS 38.214 for DL and UL OFDM and Table 6.1.4.1-1 in TS 38.214 for UL w/ transform precoding respectively) are the “default” ones and are mandatory.

- The following is optionally supported by RedCap UEs:

  - 256QAM MCS tables (Table 5.1.3.1-2 in TS 38.214 for DL and UL OFDM)

  - 64QAM low SE MCS tables (Table 5.1.3.1-3 in TS 38.214 for DL and UL OFDM and Table 6.1.4.1-2 in TS 38.214 for UL w/ transform precoding respectively)

In our view, option 4 is a clearer definition than option 2 and should be used for RedCap UE type definition. The exact definition of RedCap UEs could include maximum UE BW with 20MHz/100MHz, minimum 1 Rx branch and legacy 64QAM MCS tables.

Proposal 6: Support option 4 as for RedCap UE type definition, i.e., the corresponding minimum set of the reduced capabilities that one RedCap UE type shall mandatorily support.

# Conclusions

As a summary, we have the following proposals on RAN1 aspects for RAN2-led features for RedCap UEs,

Proposal 1: Confirm RAN1#105-e working assumption on early identification of RedCap UEs.

Proposal 2: Support early identification of RedCap UEs in Msg1 through,

- Preamble partitioning with shared ROs,

  - Similar with RACH partitioning for 2-step RACH, a set of contention-free preambles for non-RedCap UEs are allocated for RedCap UEs for contention-based RACH procedure.

- Configuring separate ROs for RedCap UEs in initial UL BWP.

Proposal 3: There is no need to have explicit signaling on enable/disable of early identification of RedCap UEs in Msg1.

Proposal 4: Support early identification of RedCap UEs with different channel conditions for the purpose of link adaptation during initial access and access control of RedCap UEs.

Proposal 5: Confirm RAN1#105-e working assumption on UE type definition for RedCap UEs.

Proposal 6: Support option 4 as for RedCap UE type definition, i.e., the corresponding minimum set of the reduced capabilities that one RedCap UE type shall mandatorily support.

# References

RP-210918, “Revised WID on support of reduced capability NR devices”, RAN#91e