3GPP TSG RAN WG1 #106-e		R1-2107959

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Discussion on RAN4 Reply LS on PUCCH and PUSCH repetition

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the DMRS bundling related questions raised in the reply LS [1] from RAN4.

- Discussion

In [1], RAN4 provided Reply LS addressing questions from RAN1 on conditions for DMRS bundling for PUCCH and PUSCH repetitions. Besides, RAN4 asked RAN1 to provide more information for the following scenario.

… RAN4 also asks RAN1 to provide more information on the scenario when “downlink reception” from UE point of view includes downlink symbols without actual DL transmission from gNB to UE and UE is not assumed to do DL monitoring?

From RAN1 perspective, symbols can be indicated as downlink symbols, while these symbols are not configured or scheduled with any downlink reception, these symbols are downlink symbols without DL transmission from gNB to UE and UE is not assumed to do DL monitoring.

Besides, even if for DL reception, e.g. PDCCH monitoring, PDSCH, or CSI-RS is configured on the semi-static flexible symbols, the DL reception can also be overridden by dynamic signalling, e.g, DCI 2-0 indicating these semi-static flexible symbols as ‘flexible’ or ‘uplink’.

RAN1 can inform these cases to RAN4 in the reply LS.

Proposal 1: Inform RAN4 the following cases for ‘downlink reception’ on symbols without actual transmission from gNB to UE and UE is not assumed to do DL monitoring.

- Downlink symbols not configured or scheduled with any downlink reception; and

- Downlink receptions are configured but canceled by dynamic signaling.

- Conclusion

In this contribution, we discussed the questions raised in RAN4 reply LS on PUCCH and PUSCH repetitions, and we have the following proposals.

Proposal 1: Inform RAN4 the following cases for ‘downlink reception’ on symbols without actual transmission from gNB to UE and UE is not assumed to do DL monitoring.

- Downlink symbols not configured or scheduled with any downlink reception; and

- Downlink receptions are configured but canceled by dynamic signaling.

- References

- R4-2107880, Reply LS on PUCCH and PUSCH repetition, RAN4.