3GPP TSG RAN WG1 #106-e		R1-2107967

e-Meeting, August 16th – 27th, 2021

Title:	Draft reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities

Response to:	R1-2104162 (R2-2104550)

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	vivo

To:	RAN2

Cc:                RAN4

Contact Person:

Name:

E-mail Address:

- Overall Description

RAN1 thanks RAN2 for the LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities. Regarding the questions related to RAN1 FG 6-24 ul-TimingAlignmentEUTRA-NR, and FG 6-23 pa-PhaseDiscontinuityImpacts, RAN1 provides the answers as the following:

Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable?

[RAN1 answer]: The RAN1 FG 6-24 ul-TimingAlignmentEUTRA-NR, and FG 6-23 pa-PhaseDiscontinuityImpacts are applicable to intra-band (NG)EN-DC/NE-DC capabilities, including the BC Type 1, 2 and 5 as mentioned in the RAN2 LS R2-2104550.

Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /ul-dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part?

[RAN1 answer]: If the RAN1 FG 6-24 ul-TimingAlignmentEUTRA-NR, or FG 6-23 pa-PhaseDiscontinuityImpacts are indicated by the UE in an inter-band (NG)EN-DC/NE-DC BC with an intra-band (NG)EN-DC/NE-DC BC part, the capability is used to indicate the restriction to the intra-band (NG)EN-DC BC par

# 2. Actions:

ACTION: 	RAN1 sincerely requests RAN2 to consider the above answers in their future work:

3. Date of Next TSG-RAN1 Meetings:

3GPP  RAN1#106bis-e 	        11 October – 19 October 2021	    Electronic Meeting

3GPP  RAN1#107e 	        11 November – 19 November 2021	Electronic Meeting