3GPP TSG RAN WG1 #106-e     	                                       R1-2107978

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Clarification on HARQ reporting on PUCCH for an ‘incomplete’ PSFCH period

Agenda Item:	7.2.4

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the type1 HARQ-ACK reporting on PUCCH for an ‘incomplete’ PSFCH period.

- Discussion

Agreements: - Option B is agreed for the number of logical slots in a resource pool. - Option A: UE is not expected to be configured with a resource pool where the number of logical slots of the resource pool, counted within 10240 ms period, is not an integer multiple of the PSFCH resource period - Option B: It is clarified in the specification that a PSFCH slot is associated with up to N PSSCH slots where N is the PSFCH resource period. - Option 1 is agreed for the location of PSFCH slots where the PSFCH resource period is N. - Option 1: Logical slot index #0, #N, #2N, …. within 10240 ms period

According to the previous agreement, the number of SL slots of a resource pool in a DFN/SFN cycle may not always be multiple of NPSSCHPSFCH, which means there may be an ‘incomplete’ PSFCH period that has less than NPSSCHPSFCH associated PSSCH occasions at the DFN/SFN cycle boundary.

An example of an ‘incomplete’ PSFCH period is shown below. With PSFCH period NPSSCHPSFCH and minimum PSSCH-PSFCH gap K, it can be derived that the time gap between a PSFCH occasion and its NPSSCHPSFCH corresponding candidate PSSCH occasions should be within the range [K, NPSSCHPSFCH+K-1]. However, when the number of SL slots in a 10240ms period is not a multiple of NPSSCHPSFCH, the interval between the last PSFCH in the previous cycle and the first PSFCH in the next cycle is less than NPSSCHPSFCH. In this case, the number of PSSCH occasions that are actually associated with the latter PSFCH among the NPSSCHPSFCH candidate PSSCH occasions is less than NPSSCHPSFCH.

Figure 1. Example of an incomplete PSFCH period

This ‘incomplete’ PSFCH occasion that has fewer than NPSSCHPSFCH associated PSSCH occasions is usually the first PSFCH occasion in a 10240ms period. Donating the number of candidate PSSCH occasions associated with a PSFCH occasion by NPSSCH_actualPSFCH, as illustrated in Figure 3, the PSFCH occasion in red has only NPSSCH_actualPSFCH=1 associated PSSCH occasion while NPSSCHPSFCH=4.

TS 38.213 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel ==================Omitted================== Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH MA=MA∪j; j=j+1; nF=nF+1; end while ==================Omitted================== If the UE does not transmit a PSSCH in an occasion for candidate PSSCH transmission with corresponding PSFCH reception occasions, due to the UE not detecting a corresponding DCI format 3_0, the UE generates a NACK value for the occasion for candidate PSSCH transmission with corresponding PSFCH reception occasions.

According to the current spec, NPSSCHPSFCH HARQ-ACK bits should be reported for each PSFCH occasion involved in a type1 SL HARQ codebook, based on the PSFCH detection result corresponding to each candidate PSSCH occasion. More specifically, the pseudo-code of type1 codebook (highlighted above) implies that a constant size of the codebook is generated as it is determined by the length of PSFCH period.

Observation 1. According to the spec, for a PSFCH occasion, there are NPSSCHPSFCH candidate PSSCH occasions, and NPSSCHPSFCH HARQ-ACK bits should be generated to keep the constant type1 codebook size, where NPSSCHPSFCH is the PSFCH period.

If a candidate PSSCH occasion that is not scheduled for SL, NACK is reported. Similarly, for the candidate PSSCH occasion(s) that are not associated with the corresponding PSFCH occasion, one straightforward way is to generate NACK so that the codebook size remains unchanged over time. While on the other hand, it is also possible for UE not to generate HARQ-ACK bit for such PSSCH occasions.

In conclusion, there are two different interpretations to determine the HARQ-ACK reporting for an incomplete PSFCH period:

Interpretation 1: TX UE reports NPSSCHPSFCH HARQ-ACK bits for the ‘incomplete’ PSFCH occasion (i.e., the same number of HARQ-ACK bits as for a normal PSFCH occasion).

Interpretation 2: TX UE reports NPSSCH_actualPSFCH HARQ-ACK bits for the ‘incomplete’ PSFCH occasion, which means UE only generates HARQ-ACK for the actually associated PSSCH occasions.

Figure 2. Example of option1 and option2

If interpretation1 is the correct understanding, how the UE sets the values of the HARQ-ACK bits for the candidate PSSCH occasion(s) that are not associated with the corresponding PSFCH occasion should be specified. Otherwise, if interpretation2 is the correct understanding, the pseudo-code of type1 codebook generation should be fixed to construct different type1 codebook sizes for ‘normal’ and ‘incomplete’ PSFCH occasions, respectively. Two draft CRs are attached for each interpretation.

Proposal 1. The spec should be clarified on how to generate the type 1 codebook for the ‘incomplete’ PSFCH period, i.e., whether NPSSCHPSFCH or NPSSCH_actualPSFCH HARQ-ACK bits should be reported, where NPSSCHPSFCH is the PSFCH period and  NPSSCH_actualPSFCH is the number of PSSCH occasion(s) that are associated with the  ‘incomplete’ PSFCH. Draft CR is provided for each interpretation.

From our perspective, interpretation2 has the disadvantage that the type1 codebook size would dynamically change. To be specific, if the gNB allocates two K1 values, and one of them corresponds to a normal PSFCH occasion and the other one corresponds to an incomplete PSFCH occasion, the TX UE will report NPSSCHPSFCH+NPSSCH_actualPSFCH HARQ-ACK bits on the corresponding PUCCH, while if both K1 values correspond to normal PSFCHs for another scheduled PUCCH, the TX UE shall feedback 2NPSSCHPSFCH HARQ-ACK bits. This dynamically changing codebook size will not only increase the complexity of gNB detection but also deviates from the original intent of introducing type1 codebook. Therefore, interpretation1 is suggested.

Proposal 2. Adopt the CR of interpretation1 of clarification on HARQ-ACK reporting for ‘incomplete’ PSFCH period, i.e., TX UE reports NPSSCHPSFCH HARQ-ACK bits for the ‘incomplete’ PSFCH occasion, including NPSSCHPSFCH-NPSSCH_actualPSFCH bit(s) of NACK and NPSSCH_actualPSFCH HARQ-ACK bit(s) corresponding to the actually associated PSSCH occasions.

- Conclusion

Based on the above discussions, we have the following observations and proposals:

Observation 1. According to the spec, for a PSFCH occasion, there are NPSSCHPSFCH candidate PSSCH occasions, and NPSSCHPSFCH HARQ-ACK bits should be generated to keep the constant type1 codebook size, where NPSSCHPSFCH is the PSFCH period.

Proposal 1. The spec should be clarified on how to generate the type 1 codebook for the ‘incomplete’ PSFCH period, i.e., whether NPSSCHPSFCH or NPSSCH_actualPSFCH HARQ-ACK bits should be reported, where NPSSCHPSFCH is the PSFCH period and  NPSSCH_actualPSFCH is the number of PSSCH occasion(s) that are associated with the  ‘incomplete’ PSFCH. Draft CR is provided for each interpretation.

Proposal 2. Adopt the CR of interpretation1 of clarification on HARQ-ACK reporting for ‘incomplete’ PSFCH period, i.e., TX UE reports NPSSCHPSFCH HARQ-ACK bits for the ‘incomplete’ PSFCH occasion, including NPSSCHPSFCH-NPSSCH_actualPSFCH bit(s) of NACK and NPSSCH_actualPSFCH HARQ-ACK bit(s) corresponding to the actually associated PSSCH occasions.