3GPP TSG-RAN WG1 Meeting #106-e	Tdoc R1-2108007

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.14.3

Source:	Ericsson

Title:	XR performance evaluation results

Document for:	Discussion

# 1	Introduction

In this contribution, we provide system simulation results evaluation XR capacity and power consumption based on the agreed evaluation methodology.

Following cases are considered:

- DL capacity in Dense Urban Macro for cloud gaming and AR/VR

  - FR1 and FR2

- DL capacity in Indoor hotspot for cloud gaming and AR/VR

  - FR1 and FR2

- UL capacity in Dense Urban in FR1 for

  - pose traffic

  - scene traffic

  - pose + scene traffic

- UL capacity in Urban Macro in FR1 for

  - pose traffic

  - scene traffic

  - pose+scene traffic

- Power consumption in Dense Urban Macro

For the traffic model parameters, we rely on below parameters agreed in RAN1#105-e:

Agreement

When companies are submitting evaluation results to RAN1, it is recommended to submit results at least the following parameters in the below table.

- Note 1: This is only intended to have more results from more companies at least for the corresponding configuration. RAN1 agreements regarding baseline vs. optional for simulation scenarios, configurations, parameters, remain the same.

- Note 2: Companies are encouraged to submit results for other baseline/optional configurations as much as they can.

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

# 2	Discussion

## 2.1	DL capacity for CG and AR/VR

For cloud gaming and AR/VR, RAN1 agreed to use a frame-based model, where the frame sizes followed a truncated gaussian distribution. Also, there is frame level jitter, where the jitter follows a truncated gaussian distribution. The capacity has been evaluated for Dense Urban (FR1: Figure 1 and FR2: Figure 4), and Indoor Hotspot (FR1: Figure 3 and FR2: Figure 5).

Figure 1: Performance for CG and AR/VR in FR1 Dense Urban.

It is interesting to also study the utilization for the different load points. This is depicted in Figure 2

Figure 2: Utilization for CG and AR/VR in FR1 Dense Urban.

From Figure 2, it is seen that the average utilization at the capacity limit is around 30% for AR/VR and around 45% for CG. This is the trade-off required to support tight latency bounds.

Figure 3: Performance for CG and AR/VR in FR1 Indoor hotspot.

a

Figure 4: Performance for CG and AR/VR in FR2 Dense Urban.

Figure 5: Performance for CG and AR/VR in FR2 Indoor hotspot.

## 2.3	UL capacity

During RAN1#104-e, the pose traffic model was also discussed. This traffic model is meant to represent the traffic that is sent from an XR device to describe the movement of, e.g., an HMD, so that a game server can change the view displayed on the device. The traffic characterized by small but frequent packets transmitted in UL, each with a fixed size. Furthermore, the scene traffic has been agreed, which is an UL video traffic model, quite similar to the DL video traffic model.

In this section, we provide results for the pose traffic model, the scene traffic model, and the combination of pose and scene traffic. Dense urban and Urban macro are considered for FR1.

### 2.3.1	UL capacity for the pose traffic model

The performance with pose-only traffic is shown in Figure 6. Both for Dense Urban and Urban Macro, the capacity exceeds 15 users per cell.

Figure 6: Performance for the pose traffic model for Dense Urban and Urban macro.

### 2.3.2	UL capacity for the scene traffic model

The performance with scene-only traffic is shown in Figure 7. For Dense Urban, the capacity is close to 5 users per cell, but for Urban Macro, less than 90% of the UEs are satisfied for all loads.

Figure 7: Performance for the scene traffic model for Dense urban and Urban Macro.

### 2.3.3	UL capacity for the pose+scene traffic model

The performance with pose and scene traffic is shown in Figure 8. For Dense Urban, the capacity is close to 5 users per cell, but for Urban Macro, less than 90% of the UEs are satisfied for all loads.

Figure 8: Performance for the pose+scene traffic model for Dense urban and Urban Macro.

Not surprisingly, capacity is limited by the scene traffic. The presence of the scene traffic has some impact on the pose traffic as well. In Figure 9, we look at user satisfaction for pose traffic separately.

Figure 9: Performance for the pose traffic in the pose+scene traffic scenario for Dense urban and Urban Macro.

If we compare Figure 9 with Figure 6, we see that the scene traffic has impact on the performance for the pose traffic.

## 2.3	Power consumption

In this section we present simulation results evaluating impact of UE power consumption reduction techniques on XR capacity. Results are shown for Dense Urban Macro deployment. To be able to align data and DRX period start for eCDRX parameters, the start of the data period for each user is selected based on the UE id and evenly distributed over the data period. This is slightly different compared to the previous section.

The following scenarios are evaluated:

- Traffic models

  - DL only -- XR CG & AR/VR traffic with 30Mbps average data rate,10/15ms PDB

  - DL+UL – XR CG & AR/VR traffic with 30Mbps average data rate + UL pose information (4ms periodicity and 100bytes payload)

- DRX configured with below settings

  - Baseline case with no DRX is configured

    - labelled as ‘drx-0’ in the figures

  - DRX with different settings labelled as ‘drx-p-On-q-iat-r’ in the figures with below settings for p,q,r

    - DRX Cycle p = 10ms; On duration timer q = 5ms; IAT r = 5ms

    - Enhanced CDRX, DRX period adjusted with data period DRX Cycle p = 16.66ms (16.5,16.5,17, …); On duration timer q = 8ms; IAT r = 3ms

- Power consumption is also shown for ‘genie’ case

  - Labelled as ‘genie’ in the figures.

  - For this case it is assumed that PDCCH monitoring is turned off for the UE using a ‘genie assisted’ mechanism whenever there is no data reception (sleep if no UL) and the UE is sent to sleep state. Power consumption for the sleep state is based on available sleep duration.

- For the DL+UL evaluations, UE sleep state is determined considering both DL Rx and UL Tx activity for the UE.

### 2.3.1	DL CG and AR/VR model

Figure 2.3.1-1 shows XR performance for baseline (no DRX) case vs. different DRX settings

X) case vs. different DRX settings

Figure 2.3.1-1: XR performance for baseline (no DRX) case vs. different DRX settings

Figure 2.3.1-2a and 2.3.1-2b show CDFs of UE power consumption for low/medium loads (i.e., 1,4 UEs/cell respectively) for different DRX settings (for all UEs).

Figures 2.3.1-2a (left), 2.3.1-2b (right) : XR Power consumption for different cell loading (DL CG & AR/VR)

Figure 2.3.1-3a and 2.3.1-3b show the relative fraction of time consumed by the UE in different power consumption states (‘ulNoTrans’ & ‘dlNoDetect’ indicate inactive UL or DL slots but the slot may be active in the other direction). Figure 2.3.1-3a shows results for low load (1UEs/cell) and 2.3.1-3b for medium load (4UEs/cell)

Figures 2.3.1-3a (left), 2.3.1-3b (right) : Relative time fraction of different UE power consumption states (DL CG & AR/VR)

Figure 2.3.1-4a, 2.3.1-4b, show the relative fraction of energy consumed by the UE in different power consumption states. Figure 2.3.1-4a shows results for low load (1UEs/cell) and 2.3.1-4b for medium load (4UEs/cell)

Figures 2.3.1-4a (left), 2.3.1-4b (right) : Relative energy fraction of different UE power consumption states (DL CG & AR/VR)

Table 2.3-1 below shows the XR capacity vs. UE power savings tradeoff for the different DRX settings that are considered in the evaluations. The power gains are for all users at 4 UE/cell.

Table 2.3.1-1: XR capacity vs. UE power savings tradeoff (DL – 30Mbps),

| DRX configuration (DRX cycle, On duration, IAT) | Mean PS gain compared to baseline at 4 UE/cell (%) | 95%-tile PS gain (highest Energy) | 50%-tile PS gain (lowest Energy) | 5%-tile PS gain (lowest Energy) | Capacity Cell load when 90% of the users are satisfied PDB 10ms | Capacity Cell load when 90% of the users are satisfied PDB 15ms | Satisfied users @ baseline PDB 10ms | Satisfied users @ baseline PDB 15ms |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No DRX (baseline) | N/A | N/A | N/A | N/A | 4.0 | 4.7 | 90% | 90% |
| (10ms,5ms,5ms) | 8% | 14% | 9% | 3% | 0.6 | 4.1 | 27% | 76% |
| (10ms,8ms,3ms) | 4% | 6% | 5% | 2% | 3.4 | 4.6 | 82% | 87% |
| (16.66ms,8ms,3ms) | 21% | 27% | 23% | 13% | 3.5 | 4.3 | 84% | 80% |
| Genie | 40% | 52% | 43% | 26% | N/A | N/A | N/A | N/A |

### 2.3.2	DL CG and AR/VR + UL Pose model

Here we show UE power consumption also considering UL transmissions for pose updates.

The following scenarios are evaluated:

- DRX configured with below settings

  - Baseline case with no DRX is configured: labelled as ‘drx-0’ in the figures

  - DRX with different settings labelled as ‘data-8-drx-p-On-q-iat-r’ in the figures with below settings for p,q,r

  - DRX Cycle p = 10ms; On duration timer q = 5ms; IAT r = 5ms

- DRX configured with below settings

  - Baseline case with no DRX is configured

    - labelled as ‘drx-0’ in the figures

  - DRX with different settings labelled as ‘drx-p-On-q-iat-r’ in the figures with below settings for p,q,r

    - DRX Cycle p = 4ms; On duration timer q = 3ms; IAT r = 0ms

    - Enhanced CDRX, DRX period adjusted with data period DRX Cycle p = 16.66ms (16.5,16.5,17, …); On duration timer q = 13ms; IAT r = 0ms

  - genie case, labelled as ‘genie’ in the figures

Figure 2.3.1-1: XR performance for baseline (no DRX) case vs. different DRX settings

Figure 2.3.2-1a and 2.3.2-1b show CDFs of UE power consumption for low/medium loads (i.e., 1,4 UEs/cell respectively)

Figures 2.3.2-1a (left), 2.3.2-1b (right) : XR Power consumption for different cell loading (DL CG and AR/VR + UL pose)

Figure 2.3.2-2a, 2.3.2-2b, show the relative fraction of time consumed by the UE in different power consumption states (‘ulNoTrans’ & ‘dlNoDetect’ indicate inactive UL or DL slots but the slot may be active in the other direction). Figure 2.3.2-2a shows results for low load (1UEs/cell) and 2.3.2-2b for medium load (4UEs/cell)

Figures 2.3.2-2a (left), 2.3.2-2b (right) : Relative time fraction of different UE power consumption states (DL CG and AR/VR + UL pose)

Figure 2.3.2-3a, 2.3.2-3b, show the relative energy fraction of energy consumed by the UE in different power consumption states (‘ulNoTrans’ & ‘dlNoDetect’ indicate inactive UL or DL slots but the slot may be active in the other direction). Figure 2.3.2-3a shows results for low load (1UEs/cell) and 2.3.2-3b for medium load (4UEs/cell)

Figures 2.3.2-3a (left), 2.3.2-3b (right) : Relative energy fraction of different UE power consumption states (DL CG and AR/VR + UL pose)

Table 2.3-2-1 below shows XR UE power savings for genie case compared to baseline no DRX for all users at 4 UE/cell.

Table 2.3.2-1: XR UE power savings (DL 30Mbps + UL pose)

| DRX configuration (DRX cycle, On duration, IAT) | Mean PS gain compared to baseline at 4 UE/cell (%) | 95%-tile PS gain (highest Energy) | 50%-tile PS gain (lowest Energy) | 5%-tile PS gain (lowest Energy) | Capacity Cell load when 90% of the users are satisfied PDB 10ms | Capacity Cell load when 90% of the users are satisfied PDB 15ms | Satisfied users @ baseline PDB 10ms | Satisfied users @ baseline PDB 15ms |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No DRX (baseline) | N/A | N/A | N/A | N/A | 3.9 | 4.8 | 90% | 90% |
| (4ms,3ms,0ms) | 7% | 12% | 7% | 5% | 3.2 | 4.4 | 78% | 84% |
| (16.66ms,13ms,0ms) | 6% | 7% | 6% | 5% | 3.7 | 4.6 | 88% | 88% |
| Genie | 17% | 23% | 18% | 11% | N/A | N/A | N/A | N/A |

We make the following observations from evaluations.

- When DL XR traffic is considered without taking into account the UL impact

  - contribution of “PDCCH monitoring” (i.e., PDCCH monitoring without scheduled data) to overall UE power consumption is larger (as shown in Figures 2.3.1-4a/4b)

  - Results show that at least for the evaluated cases, appropriate DRX settings can achieve some UE power savings gain with reduction in capacity

    - For DL 30Mbps

      - 8% UEPS gain with 85%/15% capacity loss for 10/15ms PDB with Rel15 settings (10ms,5ms,5ms)

      - 21% UEPS gain with 9%/7% capacity loss for 10/15ms PDB with enhanced DRX settings (16.66ms,8ms,3ms)

- When both DL and UL XR traffic is considered

  - PUSCH transmissions contribute a significant fraction to overall UE power consumption (e.g. as shown in Figures 2.3.2-3a/3b) and frequent UL transmissions prevent the UE from transitioning to light sleep state.

  - For DL 30Mbps+UL pose

    - 6%/7% UEPS gain is observed for DRX with Rel15/enhanced settings, and with ideal ‘genie’ case 17% avg power savings potential compared to baseline ‘always on’ is observed (as shown in Table 2.3.2-2)

# 3	References

RP-193241, Study on XR Evaluations for NR, Qualcomm, RAN#86, Sitges, December 2019.

R1-2105829, Traffic model for XR, Ericsson, RAN1#105-e, May 2021

# 4	Appendix

| Parameters | Values |
| --- | --- |
| Frequency Range | FR1 @ 4 GHz, SCS: 30 kHz, BW: 100 MHz |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (8, 2, 2, 1, 1). (dV, dH) = (0.5, 0.5) λ. (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1). (dV, dH) = (0.5, 0.5) λ. gNB Antenna has mechanical down-tilt 12 degrees (DU) or 6 degrees (UMa) |
| Duplex method | TDD DDDSU |
| UE Antenna Configuration | (M, N, P) = (1, 2, 2), |
| Link adaptation | Outer loop adaptation + 32-port CSI-RS Type I codebook |
| Scheduling | PF, SU-MIMO |
| Control and RS overhead | Explicit modelling of control and RS overhead |
| Transmission scheme | Up to rank 4 transmission |
| Other potential impairments | Not modelled (assumed ideal) |
| Target BLER | 10% |
| Scenario | Dense Urban (macro-layer only, TR 38.913), 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site), 20% outdoor Urban macro, 500m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site), 20% outdoor |
| Channel model | UMa 5G (TR 38.901) |
| BS Tx power | 51dBm (DMa) 56dBm (UMa) |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| UE distribution | Randomly over the system |
| UE receiver | MMSE-IRC |
| CSI feedback | Realistic, including CSI quantization |