3GPP TSG RAN WG1 #106-e	                                	      R1-2108025

e-Meeting, August 16th – 27th, 2021

Agenda Item:	7.2.5

Source:	Lenovo, Motorola Mobility

Title:	Explanation of Changes in Draft CR on Correction on SPS overlap with dynamic PDSCH in TS 38.214

Document for:	Decision

# Introduction

This contribution outlines the changes done in the companion draft CR on Correction on SPS overlap with dynamic PDSCH in TS 38.214.

# Changes to TS 38.214

The change is to subclause 5.1. The motivation for the change is described below.

If a 1st SPS PDSCH overlaps with a 2nd SPS PDSCH, and the UE determines to decode only the 1st SPS PDSCH  because of a lower configured sps-ConfigIndex for the 1st SPS PDSCH, the 14-symbol requirement (DCI needs to be received 14 symbols prior to the earliest SPS PDSCH symbol) is applied to the 1st SPS PDSCH (survivor SPS PDSCH) as shown in Figure 1:

Figure 1: dynamic PDSCH is decoded if the DCI scheduling the dynamic PDSCH is received at least 14 symbols prior to the first symbol of the SPS PDSCH overlapping with the scheduled dynamic PDSCH, wherein the SPS PDSCH is determined after resolving overlap amongst the overlapped SPS PDSCHs (i.e., 1st SPS PDSCH which has a lower sps-ConfigIndex than 2nd SPS PDSCH)

The current specification text is unclear and ambiguous on conditions for requiring reception of dynamic PDSCH when overlapping with SPS PDSCHs.

# Summary

We propose to clarify that a gap of 14 symbols between the PDCCH and the earliest starting symbol of SPS PDSCHs applies only with respect to SPS PDSCHs that are survivors according to the survivor PDSCH procedure described in Clause 5.1. The companion draft CR provides a text proposal for TS 38.214.