3GPP TSG RAN WG1 #106-e																R1-2108057

e-meeting, August 16th – 27th, 2021

Agenda item:		8.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhancements on SRS flexibility, Coverage and Capacity

Document for:		Discussion and Decision

# 1	Introduction

In RAN#86, enhancements on SRS operation were approved to be considered and specified as part of the further enhancements on MIMO WID [1]. The objectives for the SRS enhancements are stated as follows [1]:

The work item aims to specify the enhancements identified for NR MIMO. The detailed objectives are as follows: - Extend specification support in the following areas [RAN1] … - Enhancement on SRS, targeting both FR1 and FR2: - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8}) - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency …

# 2	Discussion on SRS Enhancements

## 2.1	Triggering Enhancements

Working Assumption For DCI indication of “t” in Rel-17 SRS triggering offset enhancement - For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request - t is indicated by adding a new configurable DCI field (up to 2 bits) - Applies only when there are multiple candidate values of t configured - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured

Agreement - Up to 4 “t” values can be configured per SRS resource set.

In RAN1-104bis-e meeting, a working assumption was made that a new DCI field ( up to 2 bits)  is to be added to DCI scheduling PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request. From our perspective, we can confirm this working assumption. Additionally, it was agreed that  each aperiodic SRS resource set can be configured with RRC signaling up to 4 different scheduling offsets.

In RAN1-103-e meeting, two different reference slot options ( option 1 and option 2) for aperiodic SRS triggering were introduced. Figure 1 shows an example of option 2 where a legacy resource set specific slot offset defines a reference slot. Aperiodic UL SRS resource set is configured with four different scheduling offsets values (t_offset). Here new DCI field “t” is associated as index to the set specific triggering offset values. When the reference slot is determined as available slot, i.e.uplink or flexible and satisfying UE capability on minimum timing requirement, the scheduling offset is equal to the value of “t_offset” (instead of t_offset +1). As a result of this, it is possible to trigger the aperiodic UL SRS resource set also into the first available slot. From our perspective, the option 2 can be seen as the most feasible one by providing required configuration flexibility to distribute different UL SRS transmission in time with reduced congestion probability and minimal specification changes.

Observation 1: Option 2 (i.e. slot offset as reference slot) is the most feasible option for Rel-17.

Figure 1 an example of Option 2 regarding to reference slot definition.

Proposal 1: Confirm the following working assumption: For DCI indication of “t” in Rel-17 SRS triggering offset enhancement

- For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request

  - t is indicated by adding a new configurable DCI field (up to 2 bits)

    - Applies only when there are multiple candidate values of t configured

  - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured

Proposal 2: Support option 2 (i.e. slot offset as a reference slot).

## 2.2	Coverage and Capacity Enhancements

Agreement Study the maximum number of cyclic shifts for Comb-8 in Rel-17, with the following alternatives as starting points - Alt 1: The maximum number of CSs for Comb-8 is 6 - Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

Based on the RAN1-104bis-e meeting agreement, two different alternatives, i.e. Alt-1 and Alt-2, with different number of maximum number of cyclic shifts for comb-8 have been proposed to be studied. Alt-1 is already supported in Rel-16 specification for positioning where the number of cyclic-shifts is equal to 6 with a minimum sequence length of 6. For comb-8, multiple of 4 PRBs is required to fulfill the agreed minimum sequence length.  When CS=6 and comb-8 is configured, any of the ports within SRS resource, the cyclic shifted sequence for a port is orthogonal to each because same base sequence is used for all ports within the SRS resource. CS=6 provides 6 orthogonal antenna ports that is sufficient to support either 1, 2 or 4 antenna ports to be multiplexed within the SRS resource the Rel-17.

An another aspect to consider if two or more resources from same or different user share the same comb-type such away that the first cyclic values are different for two different resources. This enables to share the same base sequence with between two different resources by using sub-set of the maximum cyclic shifts. From this perspective, Alt-2 (CS=12) may have a larger number of candidate cyclic shift combinations to be paired with respect CS=6. However, the minimum sequence length requirements of 6 limits the number of orthogonal cyclic shifts with CS=12 such that only some specific cyclic shift values are orthogonal to each other’s. Hence, to determine those specific cyclic shift value combinations, some specific rules would be needed that would increase the specification efforts. Due to fact that only the subset of maximum number of cyclic shifts (CS=12) with minimum sequence length are orthogonal to each other, the number of possible paired cyclic shift combinations may be close to each other’s with Alt-1 and Alt-2. In other words, Rel-17 minimum sequence length limits the number of orthogonal antenna ports for small PRB allocations leading to similar multiplexing capability as with comb-4 and CS=12.

Furthermore, delay spread of the channel needs to be taken into account in design such that phase shift spacing associated with different cyclic shift values is not too small in comparison with the delay spread of the channel. It is worth noting with CS=12 phase-shifts between different cyclic shift values are more closely place with respect to CS=6. From this perspective, CS = 6 can provide better robustness compared with CS=12 against the delay spread of the channel. Based on all the aspects discussed in above text, the Alt-1 is more feasible options for Rel-17.

Observation 2: Alt-1 (maximum number of CS equals to 6 for comb-8) is more feasible option compared with Alt-2 for Rel-17.

Proposal 3: Support Alt-1 (maximum number of CS equals to 6 for comb-8) for Rel-17.

## 2.2.1 Repetition

Agreement For increased repetition in Rel-17, support the following N_symbol (number of OFDM symbols in one SRS resource) and R (repetition factor) values - N_symbol = 8, R = {1, 2, 4, 8} - N_symbol = 12, R = {1, 2, [3], 4, 6, 12} - FFS the following configurations - N_symbol = 10, R = {1, 2, 5, 10} - N_symbol = 14, R = {1, 2, 7, 14} - FFS options to reduce SRS BW for R>1

Based on the RAN1-104bis-e meeting agreement, it remained for further study to consider N_symbol =10 and 14 with R={1,2,4,10} and R=1,2,7,14}, respectively. To enhance further configuration flexibility, from our perspective we support both of the aforementioned options.

Proposal 4: Support both options: N_symbol =10 and R={1,2,4,10} as well as N_symbol =14 and R={1,2,7,14}.

## 2.2.2 Partial Frequency Sounding

Agreement For Rel-17 SRS capacity and coverage enhancement, support the following - Increase the maximum number of repetition symbols in one slot and one SRS resource to S - Support at least one S value from {8, 10, 12, 14} - FFS other candidate values - Support to transmit SRS only in contiguous RBs in one OFDM symbol, where indicates the number of RBs configured by BSRS and CSRS - Support at least one PF value from {2, [3], 4, 8} - FFS other candidate values, e.g., non-integer values for PF - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - No new sequence including length is introduced - FFS it is applicable to frequency hopping and non-frequency hopping - FFS detailed signaling mechanism to determine PF and the location of the RBs - Support Comb 8 - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - FFS whether and if needed, how to use harmonized approach to define the three supported schemes - Note: other schemes for SRS capacity and coverage enhancements are not supported in Rel-17.

Agreement - For RPFS in Rel-17, support PF = {2, 4}. - FFS 3, 8, 12, 16 or fractional numbers - Support at least one of the following alternatives (to be decided in RAN1#105-e) - Alt 1: 1PFmSRS, BSRS is an integer value - Alt 2: 1PFmSRS, BSRS is an integer value with minimum value 4 - Alt 3: is a multiple of 4 - Alt 4: Round 1PFmSRS, BSRS to a multiple of 4 in case of Alt 1 or Alt 2

Based on the RAN1-104bis-e meeting agreement, different partial frequency sounding SRS bandwidth alternatives have been proposed where at least one of the alternative needs to be supported. Furthermore, additional partial frequency values PF=3,8,12,16 remained for further study. When a comb-8 is considered with PF= 3, it leads to configuration where the non-integer number of PRBs exist with some specific SRS bandwidths, e.g. 2,4,16,20,28,32,40,52, etc.  The same problem occurs also with comb-8 when PF>3 or fractional number of PF is configured. Additionally, the agreed minimum sequence length of 6, can not be fulfilled with PF=3 and SRS bandwidths, e.g. 4,8. As a result of this, it is not feasible to introduce any new PF values in addition to already agreed ones. Regarding to the minimum sequence length, the problem occurs also with PF=2 and SRS bandwidth of 4 PRBs as well as with PF=4 and SRS bandwidth of 4, 8 and 12 PRBs. Therefore, the with comb-8 minimum partial frequency sounding SRS band should be 8 PRBs for PF=2 and 16 PRBs for PF = 4. Furthermore, partial frequency sounding SRS bandwidth should be multiple of 4 for PF =4 such that minimum sequence length is fulfilled. In the case of PF=2, it is not a necessity to have partial frequency sounding to be multiple of 4. Therefore, Alt-3 is the most feasible option with addition where the minimum sequence length requirement needs to be fulfilled.

Observation 3: When UE is configured with comb-8 and  PF≥3 or fractional number of PF, problems occurs regarding to non-integer amount of PRBs and minimum sequence length.

Observation 4: It is problematic to fulfill the minimum sequence length with the following configurations: PF=2 and SRS bandwidth of 4 PRBs, PF=4 and SRS bandwidth of 4,8, 12 PRBs.

Observation 5: Alt-3 is the most feasible option with an addition where minimum sequence length requirement needs to be fulfilled.

Agreement For RPFS SRS in Rel-17, adopt one of the following alternatives for sequence generation, where no new sequence length other than the ones supported in the current spec is introduced (to be decided in RAN1#105-e) - Alt 1: Generate length-12PFmSRS, BSRSComb ZC sequence - Alt 2: Truncate from legacy length-12⋅mSRS, BSRSComb sequence according to the location of RPFS SRS

Based on the RAN1-104bis-e meeting agreement, two different ways to generate ZC sequence have been proposed. Figure 2 shows an example of PAPR performance for ZC Alt-1 “short” independent blockwise sequence and ZC Alt-2 “long” non-blockwise sequence with PF=4 and PF=8. As can be seen Alt-1 outperforms Alt-2 in PAPR performance. To enhance coverage and power efficiency of UL SRS transmission, Alt-1 is more preferable option for Rel-17.

Observation 6: Alt-1 outperforms Alt-2 in PAPR performance.

Figure 2 PAPR performance for Alt-1 (“short” independent block-wise seq.) and Alt-2 (“long” non-blockwise seq.)

Additionally, Alt-1 enables to generate independent block-specific sequence for each of the PF block. In Alt-2 a single long sequence is generated over multiple PF blocks out of which channel estimates are computed per block. Since in Alt-1 ZC sequence is generated per block-wise better channel estimation performance is obtained with respect to Alt-2. Based on the above discussion, we prefer to provide support for Alt-1.

Observation 7: Alt-1 can provide better channel estimation performance with respect to Alt-2.

Agreement For RB-level partial frequency sounding (RPFS) in Rel-17 - The start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs is Noffset=kFPFmSRS, BSRS, where kF = {0, …, PF-1} - FFS support start RB location (Noffset) hopping in different SRS occasions, symbols or frequency hopping periods, and if supported, detailed hopping pattern - Support to determine PF and Noffset at least via RRC configuration per SRS resource. - FFS whether to introduce DCI and/or MAC CE in addition

Based on the RAN1-104bis-e meeting agreement, it remained for further study whether to support  start PRB location Noffset hopping for partial frequency sounding in different SRS transmissions occasions. Figure 3 shows an example of partial frequency sounding with hopping and two symbols, PF=2 and without repetition. By enabling partial frequency sounding block with specific start PRB location Noffset hopping, further flexibility for multiplexing other UL signals/reference/channels transmissions from same/other user(s) can be provided.

Observation 8: To enhance flexibility for multiplexing other UL signals/reference/channels transmissions from same/other user(s), it is beneficial to introduce hopping offset Noffset.

Figure 3 An example of partial frequency sounding with hopping.

Regarding to whether to introduce DCI and/or MAC CE to adapt PF and Noffset, from our perspective there is no need to introduce any additional signaling with respect to agreed RRC signaling.

Observation 9: No need to introduce any additional signaling for PF and Noffset with respect to RRC signaling.

To further enhance coverage of partial frequency sounding block-wise transmission w/ or w/o repetition and/or frequency hopping or non-hopping, it is highly important to consider maximum output power reduction (MPR) aspects.  To avoid exceeding out-of-band emissions (e.g., ACLR, Spectrum emission mask) to mainly adjacent radio systems within a band and provide extra band-specific robustness against additional spurious emissions such as a protection of neighboring radio systems outside a band with a reasonable implementations such as PA linearity, RAN4 defines several cases allowing for a UE to reduce its maximum output power to comply with regulations and/or due to some UE RF impairments (Rel-15 NR RAN4 specification (TS 38.101-1, sect. 6.2.2/6.2.3 and TS 38.101-2, sect. 6.2.2 and 6.2.3)).

MPR: Maximum output Power Reduction

UE is allowed to use the maximum power reduction (i.e. power back-off) values to meet out of band emissions. When the power reduction is realized, the UE is assumed to reduce evenly the total TX power across PRBs assigned for an active UL BWP in a UE channel bandwidth. Though different MPR(s) are specified depending on combinations of many factors such that UE power class, modulations, waveforms(CP-OFDM or DFT-s-OFDM), UE channel bandwidth, frequency range (i.e., FR1(410 MHz – 7125 MHz) or FR2(24250 MHz – 52600 MHz)) and so on, ultimately, the values are a function of the number of RBs and its position in a channel bandwidth under a certain combination. Moreover, MPR values and the applicable conditions consisting of the number of PRBs and its position are symmetrical within the channel bandwidth.

A-MPR: Additional Maximum Power Reduction

To enable protection of a specific band for out-of-band emissions as well as fulfill additional spurious emission requirements, NR specification provides support for the UE to use extra UL TX power reduction (i.e. power back-off). Unlike MPR, A-MPR values and the applicable conditions consisting of the number of RBs and its position are not symmetrical within the channel bandwidth. It means that if the same number of RBs is located at lower edge of a UE channel bandwidth where the edge is closer to a victim system to be protected than the higher edge, the required A-MPR at the lower edge RBs allocation is larger than that at the higher edge RBs allocation even with the same number of RBs.

Observation 10: To avoid out-of-band emissions (i.e. inter-modulation interference) to neighboring radio systems within a band and provide extra band-specific robustness, RAN4 defines maximum allowed UL TX output power reduction requirements (i.e. MPR and A-MPR) that can severely limit the coverage of UL SRS.

To avoid coverage degradation of the UL SRS due to MPR/A-MPR requirements, it is possible to use SRS configurations where UL SRS resources are not configured at band edges. However, this approach is very inefficient way to utilize of the spectrum. Hence, this approach is not pragmatic and desirable one. Furthermore, due to required TX power reduction, still some portion of the UL BWP may require the UE introduce TX power imbalance within the total SRS bandwidth.

Figure 4 shows an example of the power imbalance problem associated with UL SRS with frequency hopping. As shown, UE is configured with UL SRS where the total SRS transmission bandwidth is divided into four different SRS transmissions (64 PRBs in each) covering entire UL BWP (272 PRBs). Furthermore, the SRS resource is configured with repetition factor of 4. To avoid out-of band emissions according to RAN4 specification, the UE reduces its UL SRS transmissions power at the edges of a channel bandwidth with respect to the center of the channel bandwidth. Here, it is assumed that the UE reduces 4dB at edges and 1.5 dB at center of UL BWP. Due to resource specific repetition (i.e. 4), the impact of power reduction can be partly compensated with repetition, but power imbalance still remains across configured maximum transmitted SRS bandwidth.

Figure 4 an example of received power imbalance between center and edge PRBs of the total SRS bandwidth

Even though the potential coverage problem of the UL SRS can be partly covered with Rel-15 SRS resource configuration, the power imbalance problem remains between different UL SRS transmission instants. As a result, the quality of UL CSI or DL CSI associated with active UL/DL BWP can be severely degraded. Due to Rel-15 UL SRS resource level repetition, each UL SRS symbol uses the same repetition factor leading to excessive UL SRS overhead which is not desirable from the system perspective.

The problem of power imbalance between PRBs across total UL SRS sounding bandwidth can significantly limit the coverage and quality of UL/DL CSI acquisition resulting in significant performance degradation of MU-MIMO transmissions in both UL and DL. Especially, when DL CSI acquisition for MU-MIMO transmission is carried out based on UL SRS sounding, the quality of UL SRS based DL channel estimates play a key role for non-codebook based precoded MU-MIMO PDSCH transmission.

Observation 11: Due to MPR/A-MPR requirements, TX power imbalance is introduced across total UL SRS transmission bandwidth resulting degradation for coverage and quality of UL CSI/DL CSI, impacting to, e.g. DL MU-MIMO performance.

To reduce the problem of TX power imbalance, and degradation of UL/DL CSI acquisition as well as UL SRS signaling overhead in presence of different MPR/A-MPR requirements, Rel-17 UL SRS resource configuration for coverage enhancements, e.g. partial frequency sounding, need to take into account flexibly about these requirements. More specifically, Rel-17 SRS configuration needs to provide support for approach that enables to control UL TX power reduction for each UL SRS transmission occasion. To reduce specification and implementation impacts, enabling support for repetition of partial frequency sounding block/PRBs block specifically or symbol specific repetition factors for different symbols within one UL SRS resource can provide a straightforward approach for this.

Observation 12:	Rel-17 SRS configuration for partial frequency sounding need to provide support for approach that enables to control flexibly UL TX power reduction for each UL SRS transmission occasion.

Observation 13: It would be beneficial to enable repetition of partial frequency sounding block specifically or symbol specific repetition factors for different symbols within one UL SRS resource.

To minimize Rel-17 partial frequency sounding UL SRS resource transmission overhead with frequency hopping, e.g. in the presence of MPR/-A-MPR requirements, it would be beneficial also to enable the adaptation of the size of SRS transmission bandwidth for those PRBs/partial frequency sounding blocks that impacted by the MPR/A-MPR requirements.

Figure 5 shows throughput performance of PDSCH w/ and w/o MPR requirements with Ns=4, 8 and R=4, 8. Here, the solid lines illustrates the performance with MPR requirement. As can be observed, the throughput performance is degraded in the presence of MPR requirement. The performance is degraded more at high SNR range with respect to low SNR range. This is due to reason that high MCSs are more sensitive for PRB power imbalance caused by MPR than low MCSs used at low SNR range.

Figure 5 Throughput performance of PDSCH in the presence ofMPR (dashead lines = no MPR, solid lines = MPR).

Figure 6 shows throughput performance of PDSCH with MPR specific repetition. As can be seen, the performance can be improved by introducing repetition for set of PRBs specifically being under MPR. It is worth noting that the result do not capture impact of power boost for repeated PRBs/blocks that are subset of the total SRS bandwidth. Hence, further performance improvement can be expected when enabling power boosting for MPR specifically repeated PRB/blocks.

Figure 6 Throughput performance of PDSCH with MPR specific repetition.

An another highly important aspect impacting the coverage of UL SRS is that different TX RF chains may have TX power imbalance to which UL SRS antenna ports are associated at a UE-side. Clearly, by having power imbalance between different UL SRS antenna ports will have significant impact to system performance, for example to DL CSI acquisition with UL SRS antenna switching. Therefore,  it is highly import to take into account these aspects while designing Rel-17 UL SRS. Especially, in Rel-17 UL SRS antenna-switching is extended from 4T4R to 4T8R.

Figure 7 shows an example of throughput performance of PDSCH in the presence of power imbalance between different UL SRS antenna ports. Note that for example 2 x16 means that UE has 2 TX antennas and gNB 16 RX antennas. Additionally, for example, IB3 means second antenna ports has implementation loss of 3 dB with respect to first antenna port used as reference. As can be observed, power imbalance between different UL SRS antenna ports significantly degrades the throughput performance of PDSCH.

Observation 14: Power imbalance between different UL SRS antenna ports significantly degrades the throughput performance of PDSCH in Rel-17.

Figure 7 Throughput performance of PDSCH in the presence of power imbalance between different UL SRS antenna ports with phase error.

To avoid the impact of the power imbalance between different UL SRS antenna ports for DL or UL performance, RAN1 needs to study UL SRS transmission procedures in Rel-17 how to compensate/minimize the impact of power imbalance between different UL SRS antenna ports.

Proposal 5: Support Alt-1 for ZC sequence generation.

Proposal 6: Do not support any additional PF values, i.e. 3, 8, 12, 16 or fractional numbers.

Proposal 7: Support Alt-3 (partial frequency sounding bandwidth is multiple of 4) with an addition that minimum sequence length requirement needs to be fulfilled.

Proposal 8: Support Alt-1 to generate the length of ZC sequence.

Proposal 9: Support to specify hopping offset Noffset.

Proposal 10: Not to support to any additional signaling for PF and Noffset with respect to RRC signaling.

Proposal 11: Support repetition of partial freuquency sounding block specifically or symbol specific repetition factors for different symbols within one UL SRS resource.

Proposal 12: Support Rel-17 UL SRS resource configuration that enables the use of different size of SRS bandwidths for different symbols within one UL SRS resource.

Proposal 13: RAN1 needs to study UL SRS transmission procedures in Rel-17 how to compensate the impact of power imbalance between different UL SRS antenna ports.

## 2.3	Antenna Switching Enhancements

Agreement On aperiodic SRS configuration for antenna switching with > 4Rx, support the following N_max values - 1T6R: N_max = 3 - 1T8R: N_max = 4 - 2T6R: N_max = 3 - 2T8R: N_max = 4 - [4T8R: N_max = 2] - The support of N_max value does not imply the support of N value that is smaller than N_max. This is FFS. - FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed

Agreement On supported values of N for Rel-17 aperiodic SRS antenna switching with >4Rx, down-select at least one of the following alternatives in RAN1#105e - Alt 1: All the non-zero integer values <= N_max are supported for N - Alt 2: Support N=N_max only - Alt 3: Support specific N values <= N_max - FFS whether different alternatives may be selected for the same xTyR configuration subject to the UE capability on maximum number of symbols that can be used for SRS in a slot - FFS: whether different alternatives may be selected for different xTyR configuration

Based on the RAN1-104bis-e meeting agreements, different alternatives for the maximum number of UL SRS resource sets have been proposed where at least one of the alternative needs to be supported. From a network perspective, enabling UL SRS antenna switching configuration flexibility for is highly important aspect especially in different TDD deployments. Otherwise, UL SRS resource transmissions will restrict UL resource utilization for PUSCH and PUCCH.

Since slot-offset parameter is resource set specific, it is a necessity that different SRS resources can be distribute across different resources to enable resources to be configured over different slots, especially with some configurations, e.g. 1T6R, 1T8R, 2T6R, 2T8R. It is also worth noting that minimum switching gaps between resources within resource set imposes additional challenge for resource configuration flexibility with different TDD patterns. Therefore, it is not feasible to restrict into a single value or some specific sub-set numbers of maximum one. Based on above discussion, from our perspective to enable maximum configuration flexibility, Alt-1 is preferred solution.

Observation 15: Alt-1 is a feasible option to enable configuration flexibility for antenna-switching.

Additionally, it remained for further study whether different alternative may selected for the same xTyR configuration subject to the UE capability on maximum number of symbols that can be used for SRS in a slot. As discussed in the above text, different TDD deployments require different antenna-switching configurations, from our point of view additional UE capability constraint regarding to the maximum number of resource sets for antenna-switching is not preferred solution.

Observation 16: Additional UE capability constraint regarding to the maximum number of resource sets for antenna-switching is not needed.

Agreement On aperiodic SRS configuration for antenna switching with 4T8R, support N_max = 2

Agreement For antenna switching, support one of the following - Alt 1: Support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Alt 2: Support up to two semi-persistent SRS resource sets in addition to a periodic SRS resource set - Note: the two SP-SRS resource sets are not activated at the same time. - FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed - FFS whether configurations on SRS repetitions have impact - FFS relevant UE capability design

Based on the RAN1-104bis-e meeting agreements, two different alternatives for the maximum number of resource sets for periodic and semi-persistent UL SRS resources have been proposed. To enhance configuration flexibility of antenna-switching, Alt-2 is preferred alternative.

Agreement - For aperiodic antenna switching SRS, support to configure N <=N_max resource sets, where totally K resources are distributed in the N resource sets flexibly based on RRC configuration. - For 1T6R, K=6, N_max = [4], and each resource has 1 port. - For 1T8R, K=8, N_max = [4], and each resource has 1 port. - For 2T6R, K=3, N_max = [3], and each resource has 2 ports. - For 2T8R, K=4, N_max = [4], and each resource has 2 ports. - (Working Assumption) For 4T8R, K=2, N_max = [2], and each resource has 4 ports. - FFS the number of supported candidate values of N for each xTyR. - FFS extension to increase N_max for 1T4R, 2T4R, T=R and 1T2R cases for aperiodic, periodic and semi-persistent SRS resources - FFS the number of resources and resource sets for semi-persistent and periodic antenna switching SRS - Note: SRS could be transmitted over the last 6 OFDM symbols, or over any OFDM symbols within the slot subject to UE capability.

Based on the RAN1-104bis-e meeting agreements, it remained for further study whether increase the number of resource sets for 1T4R, 2T4R, T=R and 1T2R cases for aperiodic, periodic and semi-persistent SRS resources. As discussed earlier in this section, antenna-switching configuration flexibility is a highly important aspect for efficient operation with TDD Rel-17 and beyond releases.

To enable efficient utilization of flexible slots in conjunction with UL slots, it would be important to increase the number of aperiodic/semi-persistent/periodic resource sets for 1T4R with configuration where 4 sets (N_max=4) with 1 resource within each set is configured. Furthermore, based on aforementioned reasons, it would be important to increase the number of aperiodic/semi-persistent/periodic resource sets for 1T2R with configuration where 2 (N_max=2) with one resource within each set is configured. Similarly, the number of aperiodic/semi-persistent/periodic  resource sets for 2T4R with configuration where 2 (N_max=2) with 1 resource (2-antenna port) within each set.

Observation 17: To enhance the flexibility of legacy antenna-switching configuration, new configurations need to be added for 1T2R, 1T4R and 2T4R  aperiodic, semi-persistent and periodic resources as follows: 1T2R: N_max=2 with one resource within each set, 1T4R: N_max=4 with one resource within each set, 2T4R: N_max=2 with one resources (2-AP) within each set.

Regarding to guard periods associated with UL SRS antenna switching configuration, there is also a related discussion ongoing in RAN4 regarding to requirements for interruption with UL SRS antenna-switching. Based on this discussion, it would be good for RAN1 to provide further clarification on a resource configuration where UL SRS antenna-switching resources of resources are transmitted in non-consecutive symbols, shown in Figure 8. More specifically, current RAN1 specification does not define how UE should handle OFDM symbols including potential guard period(s) associated with UL SRS antenna switching configuration between non-consecutive UL SRS symbols. Based on this information, RAN4 can continue for defining requirements for aforementioned scenario.

Observation 18: It would be beneficial for RAN1 to clarify how UE should handle OFDM symbols including potential guard period(s) associated with UL SRS antenna switching configuration between non-consecutive UL SRS symbols. Based on this information, RAN4 can define requirements accordingly.

Figure 8 UL SRS resources of a resource set are transmitted in non-consecutive symbols in the same slot.

Proposal 14: Support Alt-1 (All the non-zero integer values <= N_max are supported for N).

Proposal 15: Not to support additional UE capability constraint regarding to the maximum number of resource sets for antenna-switching.

Proposal 16: Support Alt-2 (Support up to two semi-persistent SRS resource sets in addition to a periodic SRS resource set).

Proposal 17: Support new configurations for 1T2R, 1T4R and 2T4R  aperiodic, semi-persistent and periodic resources as follows: 1T2R: N_max=2 with one resource within each set, 1T4R: N_max=4 with one resource within each set, 2T4R: N_max=2 with one resource (2-AP) within each set.

Proposal 18: RAN1 to clarify how UE should handle OFDM symbols including potential guard period(s) associated with UL SRS antenna switching configuration between non-consecutive UL SRS symbols.

# 3	Conclusions

In the previous sections, the following observations and proposal have been made:

Observation1: Option 2 (i.e. slot offset as reference slot) is the most feasible option for Rel-17.

Observation 2: Alt-1 (maximum number of CS equals to 6 for comb-8) is more feasible option compared with Alt-2 for Rel-17.

Observation 3: When UE is configured with comb-8 and  PF≥3 or fractional number of PF, problems occurs regarding to non-integer amount of PRBs and minimum sequence length.

Observation 4: It is problematic to fulfill the minimum sequence length with the following configurations: PF=2 and SRS bandwidth of 4 PRBs, PF=4 and SRS bandwidth of 4,8, 12 PRBs.

Observation 5: Alt-3 is the most feasible option with an addition where minimimun sequence length requirement needs to be fulfilled.

Observation 6: Alt-1 outperforms Alt-2 in PAPR performance.

Observation 7: Alt-1 can provide better channel estimation performance with respect to Alt-2.

Observation 8: To enhance flexibility for multiplexing other UL signals/reference/channels transmissions from same/other user(s), it is beneficial to introduce hopping offset Noffset.

Observation 9: No need to introduce any additional signaling for PF and Noffset with respect to RRC signaling.

Observation 10: To avoid out-of-band emissions (i.e. inter-modulation interference) to neighboring radio systems within a band and provide extra band-specific robustness, RAN4 defines maximum allowed UL TX output power reduction requirements (i.e. MPR and A-MPR) that can severely limit the coverage of UL SRS.

Observation 11: Due to MPR/A-MPR requirements, TX power imbalance is introduced across total UL SRS transmission bandwidth resulting degradation for coverage and quality of UL CSI/DL CSI, impacting to, e.g. DL MU-MIMO performance.

Observation 12: Rel-17 SRS configuration for partial frequency sounding need to provide support for approach that enables to control flexibly UL TX power reduction for each UL SRS transmission occasion.

Observation 13: It would be beneficial to enable repetition of partial frequency sounding block specifically or symbol specific repetition factors for different symbols within one UL SRS resource.

Observation 14: Power imbalance between different UL SRS antenna ports significantly degrades the throughput performance of PDSCH in Rel-17.

Observation 15: Alt-1 is a feasible option to enable configuration flexibility for antenna-switching.

Observation 16: Additional UE capability constraint regarding to the maximum number of resource sets for antenna-switching is not needed.

Observation 17: To enhance the flexibility of legacy antenna-switching configuration, new configurations need be to added for 1T2R, 1T4R and 2T4R  aperiodic, semi-persistent and periodic resources as follows: 1T2R: N_max=2 with one resource within each set, 1T4R: N_max=4 with one resource within each set, 2T4R: N_max=2 with one resources (2-AP) within each set.

Observation 18: It would be beneficial for RAN1 to clarify how UE should handle OFDM symbols including potential guard period(s) associated with UL SRS antenna switching configuration between non-consecutive UL SRS symbols. Based on this information, RAN4 can define requirements accordingly.

Proposal 1: Confirm the following working assumption: For DCI indication of “t” in Rel-17 SRS triggering offset enhancement

- For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request

  - t is indicated by adding a new configurable DCI field (up to 2 bits)

    - Applies only when there are multiple candidate values of t configured

  - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured

Proposal 2:Support option 2 (i.e. slot offset as a reference slot).

Proposal 3: Support Alt-1 (maximum number of CS equals to 6 for comb-8) for Rel-17.

Proposal 4: Support both options: N_symbol =10 and R={1,2,4,10} as well as N_symbol =14 and R={1,2,7,14}.

Proposal 5: Support Alt-1 for ZC sequence generation.

Proposal 6: Do not support any additional PF values, i.e. 3, 8, 12, 16 or fractional numbers.

Proposal 7: Support Alt-3 (partial frequency sounding bandwidth is multiple of 4) with an addition that minimum sequence length requirement needs to be fulfilled.

Proposal 8: Support Alt-1 to generate the length of ZC sequence.

Proposal 9: Support to specify hopping offset Noffset.

Proposal 10: Not to support to any additional signaling for PF and Noffset with respect to RRC signaling.

Proposal 11: Support repetition of partial freuquency sounding block specifically or symbol specific repetition factors for different symbols within one UL SRS resource.

Proposal 12: Support Rel-17 UL SRS resource configuration that enables the use of different size of SRS bandwidths for different symbols within one UL SRS resource.

Proposal 13: RAN1 needs to study UL SRS transmission procedures in Rel-17 how to compensate the impact of power imbalance between different UL SRS antenna ports.

Proposal 14: Support Alt-1 (All the non-zero integer values <= N_max are supported for N)

Proposal 15: Not to support additional UE capability constraint regarding to the maximum number of resource sets for antenna-switching.

Proposal 16: Support Alt-2 (Support up to two semi-persistent SRS resource sets in addition to a periodic SRS resource set).

Proposal 17: Support new configurations for 1T2R, 1T4R and 2T4R  aperiodic, semi-persistent and periodic resources as follows: 1T2R: N_max=2 with one resource within each set, 1T4R: N_max=4 with one resource within each set, 2T4R: N_max=2 with one resource (2-AP) within each set.

Proposal 18: RAN1 to clarify how UE should handle OFDM symbols including potential guard period(s) associated with UL SRS antenna switching configuration between non-consecutive UL SRS symbols.

# 4  References

- RP-193133, “Further enhancements on MIMO for NR”, Samsung.

# 5 Appendix: Simulation Parameters

| Carrier frequency | 3.5 GHz |
| --- | --- |
| Bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz |
| BS antenna configuration | (M, N, P, Mg, Ng) = (8,2,2,1,1), (0.5, 0.5)λ (V,H)-element spacing |
| UE antenna configuration | Omni: (M,N,P)= (1,2,2) with 0.5λ spacing with omni-directional antenna elements. |
| PDSCH Precoding | Reciprocity Based SVD Wideband Feedback Based (Type I) Wideband |
| Channel Model | CDL-B UE speed 3 kmph 300 ns RMS delay spread |
| DL/UL SNR offset | 20 dB DL/UL power offset, 14dB DL/UL power offset with ¼ BW frequency hopping |
| Link adaptation | link adaptation used in DL |
| CSI periodicity | 5ms |
| CSI-delay | 4ms |
| Modulation and coding schemes | 256 QAM MCS table |
| SRS Antenna Switching Scheme | 2t2r |
| SRS repetition | 1, 2, 4, 8 or 12 |
| SRS COMB | 2, 4 or 8 |
| TDD configuration | DL: SDDD |