3GPP TSG RAN WG1 Meeting #106-e	R1-2108069

E-meeting, August 16th - 27th, 2021

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	On inter-donor resource multiplexing for IAB

Document for:	Discussion and Decision

Introduction

RAN3 sent the following LS to RAN1 [1]

RAN3 discussed the inter-donor resource multiplexing, and agreed to consider the following two scenarios for Rel-17 IAB:

- Scenario 1: Inter-donor migration/RLF recovery for single connected IAB-node

- Scenario 2: Inter-donor topology redundancy for dual-connected IAB-node

(a) Scenario 1                                                 (b) Scenario 2

In scenario 1, for the resource multiplexing of IAB-node 3 (i.e. the boundary node), some kinds of coordination may be needed between the resource for its parent link and the child link. Similarly, in scenario 2, resource coordination may be necessary among the parent link 1, parent link 2, and child link for the IAB-node 3 (i.e. the boundary node).

For scenario 1 and 2, RAN3 considers the following solutions (other solutions are not precluded) for the resource coordination between the parent link and the child link:

- Option 1: The child node’s gNB-DU cell resource configuration is matched to the parent node’s gNB-DU’s resource configuration.

- Option 2: The parent node’s gNB-DU resource configuration is matched to the child node’s gNB-DU’s resource configuration.

- Option 3: A boundary node should connect only to a new parent with which it has a non-conflicting TDD and H/S/NA pattern beforehand.

For Scenario 2, RAN3 considers the following solutions (other solutions are not precluded) for the coordination between two parent links:

- Option 1: The gNB-DU cell resource configuration of the parent node controlled by the F1-terminating donor of the boundary node, is matched to another parent’s gNB-DU’s resource configuration.

- Option 2: The gNB-DU cell resource configuration of the parent node controlled by the non-F1-terminating donor of the boundary node, is matched to another parent’s gNB-DU’s resource configuration.

- Option 3: The secondary leg of a boundary node is established only towards a secondary parent whose H/S/NA configuration is compatible with the H/S/NA configuration of the master parent beforehand.

Furthermore, RAN3 has the following related agreements:

- The IAB-DU’s resource configuration can be provided by the F1-terminating donor. This also applies if child and parent DUs connect to different donors.

- The non-F1-terminating donor of the boundary node should be aware of the boundary node’s multiplexing info and the boundary IAB-DU’s activated cell list.

- The F1-terminating donor of the boundary node forwards the boundary IAB node’s multiplexing info and the boundary IAB-DU’s activated cell list to the non-F1-terminating donor.

- The parent node, which is controlled by the non-F1 terminating donor of the boundary IAB node, should also be aware of the boundary IAB-DU’s cell configurations.

- The content of the boundary IAB-DU’s cell configurations to be sent to boundary node’s parent node should include: H/S/NA resource configurations, DL/UL resource configurations, the multiplexing info, and cell specific signal/channel configurations of boundary IAB-DU’s cells.

RAN3 assumes that the inter-donor resource multiplexing should be handled by RAN1 first, and then RAN3 design some signalling to achieve such coordination accordingly. So RAN3 would like to ask RAN1 to discuss solutions for such issue first, and inform RAN3 about the outcome.

In this contribution, we discussed some details on inter-donor resource multiplexing for IAB.

Discussion

Resource multiplexing for inter-donor migration/RLF recovery for single connected IAB-node

For inter-donor migration/RLF recovery, the following two modes have been considered in RAN3

- Partial Migration: the boundary IAB-MT is migrated to the 2nd IAB-donor-CU, while the boundary IAB-DU and descendant IAB node(s) (if any) are terminated to the 1st IAB-donor-CU.

- Full Migration: the boundary IAB node and the descendant IAB node(s) (if any) are migrated (both RRC and F1 connection) to the 2nd IAB-donor-CU from 1st IAB-donor-CU.

RAN3 has agreed to support Partial Migration and RAN3 is discussing whether to support the Full Migration. Despite of this, it seems that there is no fundamental issue for Full Migration (if supported) since the 2nd IAB-donor-CU will configure the resources for the boundary IAB-DU and descendant IAB node(s) (if any).

For partial migration, the boundary IAB-MT is migrated to the 2nd IAB-donor-CU, while the boundary IAB-DU and descendant IAB node(s) (if any) are terminated to the 1st IAB-donor-CU. In this case, the 1st IAB-donor-CU may need to reconfigure the resources for the boundary IAB-DU and descendant IAB node(s) (if any) since the parent link of the boundary node has changed. This may be the intention of Option 1 even though it will be good to clarify what “matched to” means by RAN3. On the other hand, the 2nd IAB-donor-CU may reconfigure the resources for parent node gNB-DU, which might be the intention of Option 2. Based on the above understanding, Option 1 seems to target to minimize the impact to the parent/ancestor node while Option 2 seems to target to minimize the impact to child/descendant nodes. For Technically, both Option 1 and Option 2 are feasible. However, Option 1 has some advantages over Option 2 in the sense that if the parent node have other child nodes. The DU resources for the child nodes may need to be reconfigured as well. Option 3 seems a bit restrictive and it may be difficult to fine such a new parent node.

Observation 1: For resource multiplexing in case of inter-donor migration/RLF recovery for single connected IAB-node, both Option 1 and Option 2 are feasible while Option 3 is bit restrictive.

Resource multiplexing for inter-donor topology redundancy for dual-connected IAB-node

For Scenario 2, the implication of resource coordination is a bit similar to Scenario 1 where both Option 1 and Option 2 are feasible while Option 3 is bit restrictive. For the proposed resource coordination between two parent links, it would be good to clarify what “matched to” means in the context of dual-connectivity.

Observation 2: For resource multiplexing in case of inter-donor topology redundancy for dual-connected IAB-node, both Option 1 and Option 2 are feasible while Option 3 is bit restrictive.

Given the RAN3 agreement, the two donors involved in inter-donor migration or dual-connectivity can exchange the following information: boundary node multiplexing info, boundary IAB-DU’s activated cell list, boundary IAB-DU’s cell configurations which includes the H/S/NA resource configurations, DL/UL resource configurations, the multiplexing info, and cell specific signal/channel configurations of boundary IAB-DU’s cells. From resource coordination point of view, this is already sufficient. How to configure or update the IAB-DU resource configurations can be left to implementation.

Proposal: Send an LS to RAN3 to confirm both Option 1 and Option 2 are technically feasible and it is RAN1’s understanding that the information exchange agreed in RAN3 is sufficient for resource coordination.

Conclusions

In this paper, we observe and propose:

Observation 1: For resource multiplexing in case of inter-donor migration/RLF recovery for single connected IAB-node, both Option 1 and Option 2 are feasible while Option 3 is bit restrictive.

Observation 2: For resource multiplexing in case of inter-donor topology redundancy for dual-connected IAB-node, both Option 1 and Option 2 are feasible while Option 3 is bit restrictive.

Proposal: Send an LS to RAN3 to confirm both Option 1 and Option 2 are technically feasible and it is RAN1’s understanding that the information exchange agreed in RAN3 is sufficient for resource coordination.

- References

- R1-2106419, “LS on IAB resource multiplexing”, RAN3, Huawei