3GPP TSG-RAN WG1 Meeting #106-e 	R1-2108088

Online, 16th – 27th August 2021

Agenda Item:	8.6.1.4

Source: 	u-blox AG

Title:	RedCap UE Further Complexity Reduction Considerations

Document for:	Discussion / Decision

# Introduction

Release 17 NR_redcap WI [1] is expected to “specify a UE feature and parameter list with lower end capabilities, relative to Release 16 eMBB and URLLC NR” to serve three use cases: “industrial wireless sensors”, “video surveillance”, and “wearables”.

More specifically [1][2]:

- Industrial wireless sensors: reference bit rate is less than 2 Mbps (potentially asymmetric e.g. UL heavy traffic) for all use cases and the device is stationary.

- Video surveillance: reference economic video bitrate would be 2-4 Mbps; High-end video e.g. for farming would require 7.5-25 Mbps

- Wearables: reference bitrate for smart wearable application can be 5-50 Mbps in DL and 2-5 Mbps in UL, and peak bit rate of the device can be higher, up to 150 Mbps for downlink and up to 50 Mbps for uplink

This contribution discusses some additional aspects to consider for complexity reduction for RedCap UE to address the above target use cases.

# Discussions

The justifications of additional scaling factors were discussed in RAN1, and a list of scalingFactor was introduced in Rel-15 to resolve RF and baseband capabilities mismatch for non-RedCap UEs [4].

- Scaling factor is used to reflect the association of capability mismatch between the baseband capability and RF capability for both SA UE and NSA UE.

- Scaling factor is used to scale down maximum throughput of NSA UEs operating in EN-DC scenario that shares hardware resources between LTE and NR.

Specifically, four scalingFactor values are specified in [5]: 1, 0.8, 0.75, and 0.4. Applying these to the maximum bit rate calculation in [5] yields a minimum supported full-duplex FDD downlink data rate in the range of 64-68Mbps, depending on the number of receive antennas the UE supports.

These data rates exceed the reference bit rates specified in the above use cases, in particular for the aforementioned industrial wireless sensors and video surveillance use cases. The minimum supported TDD data rate varies depending on the vendor’s DL:UL configuration, but they still exceed the reference bit rates mentioned above.

Furthermore, the UE supported data rate directly impacts the memory size and memory bandwidth for L2 and HARQ buffers, and therefore implementation complexity and cost.

To help the uptake of Rel-17 Redcap UE in a wider range of use cases specified in [1], we suggest relaxing buffer size and buffer bandwidth requirements for both HARQ and for L2 buffers by reducing the minimum value of scalingFactor that can be signalled as UE capability, for RedCap UE only.

Observation 1: Current Rel-16 specifications require a supported minimum FDD downlink data rate of at least 64Mbps, which exceeds the target rates for the use cases specified in the RedCap WID.

Observation 2: Reducing the scalingFactor for RedCap will relax L2 and HARQ buffer requirements and can help with further UE complexity reduction.

Observation 3: Reduction in the minimum allowable scalingFactor can enable Rel-17 RedCap to be used in a wider range of applications, including industrial wireless sensors.

Proposal 1: Lower scalingFactor, e.g., 0.1, 0.2 shall be adopted for Rel-17 RedCap in order to relax HARQ and L2 buffer requirements for the industrial wireless sensor and the video surveillance use cases identified in [1].

The other approach is to add new separate L2 buffer scaling factors and the associated signalling mechanisms. We acknowledge the merits of such an approach. However, the above approach (Proposal 1) gives the minimum specification impact, and it also explicitly scales the HARQ bandwidth as well as the L2 buffer size, which directly contributes to RedCap UE complexity reduction.

Finally, the requirement in Section 4.1.2 in [5] requires a minimum scalingFactor dependent on the number of MIMO layers and the maximum supported QAM order. We propose that this requirement does not apply to RedCap UE.

Proposal 2: Exclude RedCap UE from the minimum scalingFactor requirement in [5], Section 4.1.2.

# Conclusion

In this contribution, we presented our views on aspects to further reduce RedCap UE complexity, by introducing smaller values to the scalingfactor capability field. Based on the discussion, we summarise our views through the following observations and proposals.

Observation 1: Current Rel-16 specifications require a supported minimum FDD downlink data rate of at least 64Mbps, which exceeds the target rates for the use cases specified in the RedCap WID.

Observation 2: Reducing the scalingFactor for RedCap will relax L2 and HARQ buffer requirements and can help with further UE complexity reduction.

Observation 3: Reduction in the minimum allowable scalingFactor can enable Rel-17 RedCap to be used in a wider range of applications, including industrial wireless sensors.

Proposal 1: Lower scalingFactor, e.g., 0.1, 0.2 shall be adopted for Rel-17 RedCap in order to relax HARQ and L2 buffer requirements for the industrial wireless sensor and the video surveillance use cases identified in [1].

Proposal 2: Exclude RedCap UE from the minimum scalingFactor requirement in [5], Section 4.1.2.

# References

- RP-211574, “Revised WID on support of reduced capability NR devices”, Nokia and Ericsson, 3GPP TSG RAN #92e, June 2021

- TR 22.804, “Study on Communication for Automation in Vertical Domains”, Release 16

- R1-1802438, “Discussion on L1 Data rate”, Intel, TSG RAN WG1 Meeting #92, March 2018

- R1-1807651, “Reply LS on formula or table for L1 data rate”, May 2018

- 38.306, “User Equipment (UE) radio access capabilities”, Release 16