3GPP TSG RAN WG1 #106-e	R1-2108124

e-Meeting, August 16th – 27th, 2021

Agenda item:		8.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	UE power saving enhancements for Active Time

Document for:		Discussion and Decision

# Introduction

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4]

  - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1]

- NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized.

The release 17 work item on UE power saving enhancements [1] includes the following objective:

- Specify at least one of the following options for Rel-17 dynamic PDCCH adaptation in time-domain for active time,​

  - Option 1: Search space set group switching,e.g.,  including explicit and implicit search spaceset group switching specified in R16 for NR-U ​

  - Option 2: PDCCH skipping for a certain duration / DRX cycle​

- FFS: which option(s)

- Candidate DCI formats for dynamic PDCCH adaptation include DCI formats 1_1(including scheduling and non-scheduling DCI), 0_1, 1_2, 0_2, 2_0, 2_6.​

- Note:​ Companies are encouraged to provide analysis on specification impact, power saving benefit and system impact (e.g., packet latency, system overhead)​

- FFS: other schemes are not precluded for further study​

At the RAN1 #103-e it was agreed that

Specify at least one of Option 1 and Option 2. Note that other schemes are not excluded.

And the remaining open issues include

This discussion was continued in RAN1#104 where following agreement was made:

Agreements: - Strive for a common design for DCI based PDCCH monitoring adaptation in active time for an active BWP to support functionalities inclusive of both SSSG switching and PDCCH skipping for a duration. - Details FFS Agreements: - The following alternatives can be considered for DCI based PDCCH monitoring adaptation in active time for an active BWP for power saving - Alt 1: Enhancement of Rel-16 SSSG switching to support PDCCH monitoring adaptation including skipping for a duration - Alt 2a: Enhancement of DCI(s) utilized for Rel-16 power saving adaptation for supporting both skipping PDCCH monitoring for a duration and SSSG switching - Alt 2b: Enhancement of DCI(s) utilized for Rel-16 power saving adaptation for supporting both skipping PDCCH monitoring for a duration and PDCCH monitoring periodicity adaptation - Others not precluded

Further agreement was made in RAN1#105e:

Agreement: - At least one of Alt 1 and Alt 2 is supported, to be decided in RAN1#106, - Alt 1: Supporting SSSG switching to emulate PDCCH skipping functionality, - Alt 1-1: by an ‘empty’ SSSG which no SS set(s) is configured for the ‘empty’ SSSG, UE does not monitoring PDCCH on the ‘empty’ SSSG, - Alt1-2: by a ‘dormant SSSG’ which may have associated SS sets, and monitored conditionally (e.g., depending on HARQ NACK or RTT/ReTx timers) - Alt 2: PDCCH schedules data and also indicates PDCCH monitoring adaptation by PDCCH skipping for a duration is supported. - FFS details, including - e.g., joint / separate indication of SSSG switching and PDCCH skipping - Determination of the duration(s) for PDCCH skipping, e.g., - by RRC signaling, - by DCI indication - Implicitly, to the end of C-DRX active time

Based on the last meeting discussion, the debate seems not be anymore as which functionalities to support, but how to support them. Some companies, as illustrated in Alt1 would like to use common frame work, building upon Rel-16 functionalities to enable the discussed power saving methods, while other companies would prefer to separate these to different features. As discussed earlier our preference is to build upon existing frame work, based on Alt 1. This would enable, through timer based SSSG switching similar power saving mechanisms in Active time as are currently possible through C-DRX. As this would be autonomous (through the timers), this circumvents the UE-NW interaction, while retaining both synchronised on the PDCCH monitoring occasions.

Proposal: Adopt Alt1 from last meetings agreement for further work.

In this contribution we consider further the method and enhancement to Rel-16 SSSG switching to support PDCCH monitoring adaptation for power saving purposes covering also enabling stopping PDCCH monitoring for a duration.

# System level aspects

As the power saving gains attainable from SS switching and PDCCH skipping are similar and use similar mechanism to achieve the power saving (reduce the PDCCH monitoring) they can be considered overlapping schemes. From system perspective, PDCCH skipping implies larger PDCCH load to achieve the power saving. Table 1 provides an estimate of the number of PDCCH skipping and SS switching commands required based on the DRX configuration per traffic scenario. While the SS switching can always be achieved via a single PDCCH DCI, the PDCCH skipping requires 2-4 times as many messages per UE for most scenarios. This increases the PDCCH load, and furthermore it is worth observing that the PDCCH skipping command energy saving gains are conditioned on the scheduler to be able to re-send the skipping command immediately after skipping duration has expired. In practical scenarios this may not be always feasible, thus the evaluated UE energy saving gains may not realize in practice for the scheme based on PDCCH skipping command.

Observation: SSSG switching has lower signalling overhead than PDCCH skipping for most of the evaluated traffic scenarios.

Table 1 Estimate of PDCCH signaling commands for PDCCH skipping and SS switching. SCS is 30 kHz and the TDD frame has 6 DL slots and 4 UL slots. Assume a single packet is sent in the first slot, which also contains the first PDCCH skip/SS switch command.

| Traffic scenario | On Duration [ms] | Inactivity timer [ms] | Number of PDCCH skip commands |  | Number of SS switch commands |
| --- | --- | --- | --- | --- | --- |
|  |  |  | No data, i.e. no inactivity timer | Data, i.e. inactivity timer |  |
| FTP3 | 8 | 20 | 2 | 4 | 1 |
| VoIP | 8 | 10 | 2 | 2 | 1 |
| Video | 8 | 10 | 2 | 2 | 1 |
| Data intensive | 8 | 4 | 2 | 1 | 1 |

# Enhancements to SSSG switching

In RAN1#105e following agreements were made:

Agreement: - PDCCH schedules data and also indicates PDCCH monitoring adaptation by SSSG switching and PDCCH skipping for a duration is supported. - At least DCI format(s) 1-1, 0-1, 1-2 and 0-2 can be used for the indication(s) Agreement: - At least one of Alt 1 and Alt 2 is supported, to be decided in RAN1#106, - Alt 1: Supporting SSSG switching to emulate PDCCH skipping functionality, - Alt 1-1: by an ‘empty’ SSSG which no SS set(s) is configured for the ‘empty’ SSSG, UE does not monitoring PDCCH on the ‘empty’ SSSG, - Alt1-2: by a ‘dormant SSSG’ which may have associated SS sets, and monitored conditionally (e.g., depending on HARQ NACK or RTT/ReTx timers) - Alt 2: PDCCH schedules data and also indicates PDCCH monitoring adaptation by PDCCH skipping for a duration is supported. - FFS details, including - e.g., joint / separate indication of SSSG switching and PDCCH skipping - Determination of the duration(s) for PDCCH skipping, e.g., - by RRC signaling, - by DCI indication - Implicitly, to the end of C-DRX active time Agreement: At least SSSG#0 and SSSG#1 switching is supported for Rel-17 SSSG switching indicated by PDCCH scheduling data and/or timer. - FFS: support of more than 2 SSSGs

In last meeting it was agreed to support PDCCH monitoring adaptation indication at least with the scheduling DCIs. The Rel-16 SS set group switching functionality supports DCI format 2_0 using field SearchSpaceSwitchTrigger-r16 to trigger change between search space set group 0 and 1. Similarly an additional field could be introduced in the agreed scheduling DCIs, enabling the target SSSG to be indicated.

Observation: A field indicating the target SSSG index could be added to the scheduling DCI.

In some scenarios, it may be beneficial to facilitate SSSG switching even though the UE is not scheduled. The support of DCI format 2_0 for SSSG switching is applicable to this purpose and could be supported in addition of scheduling DCIs. In case of multi-cell operation, to allowing PCell to control the PDCCH monitoring adaptation of Scells, similar approach as for Scell dormancy via e.g. DCI format 1_1 (Scell dormancy Case 2) could be considered. For the multi-cell cell operation it should be further discussed how the control is handled jointly with dormant BWP operation.

Proposal: Support Rel-17 SSSG switching, in addition to scheduling DCIs also for DCI format 2_0. For multi cell operation consider introducing similar mechanism as for Scell dormancy e.g. via DCI format 1_1 (SCell dormancy case 2).

A NR UE can monitor 3 different DCI sizes scrambled with C-RNTI and 1 DCI size scrambled with other RNTI. Thus, if a non-scheduling DCI is to be monitored for SSSG switching the DCI size needs to be aligned with other DCIs monitored during Active Time.

It was agreed in last meeting that for SS set group switching functionality at least with two SS set groups, 0 and 1, will be supported. As part of the study for WID objective 2a, it could be considered whether to extend the number of possible SS set groups from 2 to e.g. 3. This could facilitate further adaptation to the traffic for example by enabling gradual relaxation of the PDCCH monitoring. Three groups would allow network to better account the latency requirements for later arriving packets (of a data burst) i.e. by applying first less aggressive reduction in PDCCH monitoring (e.g. SSSG#1 with PDCCH monitoring every 2nd slot) followed by more aggressive reduction in PDCCH monitoring (e.g. SSSG#2 with PDCCH monitoring every 10th slot), as illustrated in Figure 1 (a). In practical scenarios the configuration of C-DRX and PDCCH monitoring adaptation will be a trade of between latency requirements and attainable power saving, thus most aggressive values cannot be applied without any safe guard.

In addition, a timer-based SS set group switching is also supported by Rel-16 (searchSpaceSwitchingTimer-r16). Hence, to enable UE autonomous adaptation when UE is not being scheduled (provided no DL nor UL grant) for a certain time this could also be considered for active time power saving operation. Also if the number of SS set groups is increased, timer(s) could be applied so that gradual change from more frequent PDCCH monitoring towards relaxed PDCCH monitoring is enabled. Hence, with three SSSGs and timer based adaptation behaviour similar to C-DRX (short and long DRX cycle) can be achieved, where prolonged inactivity leads to reduced PDCCH monitoring, as illustrated in Figure 1 (b).

(a)

(b)

Figure 1. Illustration of gradual PDCCH monitoring reduction to minimize the impact to packet latency based (a) DCI and (b) timer.

It can be further discussed that if more SS set groups are supported e.g. 3, that whether common ‘inactivity’ timer is used for the adaptation or whether it can be separately configured for groups N, N>0. Figure 2 provides an illustration of how DCI and different timers could be used to switch from SSSG to another.

The timer-based mechanism would also enable the stopping the PDCCH monitoring for a duration with corresponding SS set configuration (e.g. Timer 2). Stopping the PDCCH monitoring for the duration of the timer could be achieved by having an ‘empty’ SSSG to which no USS is mapped to (but may include Type3-CSS). Hence, for example, UE, with DCI, is steered to monitor SSSG#3, to which no USS is mapped to, resulting UE stopping the PDCCH monitoring (based on USS). Alternatively, this could be achieved with only two SSSG groups, but it would prevent configuring PDCCH monitoring reduction and PDCCH skipping at a same time. As show in Section 2.1, attaining power saving through PDCCH monitoring reduction with PDCCH skipping, would require network to ‘trigger’ skipping with DCI, which is not desirable. The preferred way would be to enable ‘one shot’ skipping via SSSG#3, followed by timer-based change to SSSG#2 with reduced PDCCH monitoring periodicity.

Figure 2 Timers for SSSG switching.

Proposal: Increase the number of SS set groups at least from 2 to 3.

Proposal: Support timer-based UE autonomous SS set group switching for active time power saving.

Like discussed above, with the timer based SSSG switching, also the stopping the PDCCH monitoring (for USS) could be supported. This could be defining a {empty} SS set group to which no USS set is configured to, or alternatively via configuring separate SS sets with new {infinite} or very long periodicity (2560 slots).

Observation: Through use of timer based SSSG switching and proper configuration of SS set stopping of PDCCH monitoring for a duration can be achieved.

In terms of the alternatives listed in RAN1#105e, our preference would be to select Alt1-1;

Proposal: Supporting SSSG switching with ‘empty’ SSSG to embed PDCCH skipping functionality to SSSG framework.

As discussed in context of minimum scheduling offset restriction, initiating certain procedures, transmitting RACH or SR, could also trigger implicit switch from one SS set group to another or re-initating the PDCCH monitoring (i.e. stopping the PDCCH monitoring reduction). When UE transmit SR, it implies that there is a need for data activity, hence more frequent PDCCH monitoring to support higher scheduling activity would seem justified. Likewise, if BSR could be considered to trigger SSSG switch. Also, if UE transmit RACH e.g. due to BFD, monitoring the response and as well scheduling the new configuration should proceed without delay, hence more frequent PDCCH monitoring should be applied in this case. It could be further also considered how the SSSG switching would be reflected in uplink activity in general.

Proposal: Procedures such as SR transmission, BSR or beam failure recovery should result UE to stop PDCCH monitoring adaptation and resume normal PDCCH monitoring.

Regarding the interaction with C-DRX it would need to be agreed what is the UE behaviour. As described above, the baseline behaviour could be envisioned based on the timers, so that during the inactivity the UE can based on timer expiry move to less frequent PDCCH monitoring. This would allow UE to maintain the reduced PDCCH monitoring during the On Duration, when UE is not scheduled. If UE is scheduled during the On Duration, UE could be either explicitly (with scheduling DCI) or implicitly (triggered by PDCCH detection) indicated to change the SS set group.

Observation: In case of C-DRX, timer-based SSSG switching could be applied during the inactivity and SSSG would be switched if UE is scheduled during On Duration.

When UE is configured to monitor DCP outside the active time, and if network sends wake-up signal to the UE, it would imply that UE is to be scheduled during On Duration and therefore frequent PDCCH monitoring is beneficial. Hence, the DCP should implicitly result SSSG switch to more frequent PDCCH monitoring. When considering this scenario it seems beneficial to allow the network to define a default SS set group, which the UE applies at the start of the On Duration, at least when DCP is used to trigger the On Duration monitoring.

Proposal: Support configuring a default SS set group that is applied during On Duration, at least when DCP is configured.

Handling of CSS

The discussions for active time power saving through PDCCH monitoring reduction has been mainly focused to monitoring of USS. Type0-Type3-PDCCH CSS provide different functionalities for the system operation, and from CONNECTED mode UE perspective Type3-CSS is most relevant, while Type2-CSS (paging) is expected to be monitored periodically (albeit not very frequently) and also Type1-CSS (e.g. if TAT has expired and SR is triggered). Type3-CSS can carry different group common DCIs, thus it should be considered whether could can be applied to PDCCH monitoring reduction, or whether these should be precluded and UE should always be assumed to monitor these according to the configured periodicity. As the reason to change the behavior/assumed configuration via group common DCI may be triggered by external reason e.g. SFI-RNTI or TPC-X-RNTI, it would seem preferable to ensure that all UEs can be provided the adaptation in due time. Correspondingly, albeit as a relatively infrequent occurrence, CONNECTED mode UEs should monitor paging DCI based on the configured search space set.

Proposal: PDCCH monitoring adaptation should not be applied to Type0/0A/1 or 2 PDCCH CSS. For Type3-PDCCH CSS adaptation could be considered, but the monitoring should not be stopped completely.

Application delay (and starting point)

The application delay options were discussed in last meeting and different options were listed as below:

[High] proposal 4-1: - Further consider the following application delay for PDCCH adaptation indication by DCI, - Option a: the application timelines provided in Table 10.4-1 in TS38.213 for search-space group switching for unlicensed band form is reused. - FFS: Pswitch=[25X] symbols for SCS configuration μ=3 - Option b: the application delay needed for PDCCH processing for Rel-16 minimum application delay for K0min/K2min indication was reusedis reused/extended. - Option c: PDCCH skipping command applies after PUSCH transmission if triggered by UL DCI - Option d: PDCCH skipping command applies after ACK/NACK transmission. - Option e: after successfully decoding TB. - Option f: Application delay should be “ZERO” for PDCCH monitoring adaptation. PDCCH monitoring adaptation would be applied after UE receive the additional PDCCH monitoring adaptation control signaling bit(s) in DCI - Others not precluded. - FFS reference points for the application time - FFS whether the same or different application delay(s) should be used for SSSG switching and PDCCH skipping functions

In context of PDCCH monitoring adaptation via SSSG switching, it would seem most straight forward to use the application delay timelines introduced in Rel-16. However as in difference to Rel-16 operation the PDCCH monitoring adaptation can be provided by scheduling DCI, thus using the minimum scheduling offset restriction could also be considered. In terms of absolute value from the end of DCI both result similar processing delay value, hence it could be considered using the time introduced in Rel-16, at least for DL scheduling DCI.

Proposal: Use the application delay timeline introduced in Rel-16 for SSSG switching.

It is good to note that for most parts (when PDCCH is received in the first 3 symbols of the slot) the application delay only matters when UE is indicated via DCI to change from less frequent monitoring to more frequent. In the case when UE is allowed to reduce the PDCCH monitoring, the application delay is mostly UE internal issue, and does not thus affect the attainable power saving. Correspondingly when timer based PDCCH monitoring is triggered e.g. to {empty} SSSG the delay is UE internal issue.

As discussed in last meeting the application delay has a relation to the handling of the HARQ re-transmissions. Hence, how the reference/starting point of the application delay is determined. Now, in principle for cases when the UE is still monitoring PDCCH, albeit less frequently, it may not be absolutely necessary to have any special handling of HARQ, but the scheduling of the re-transmission can happen based on the periodicity determined by applied SS set(s). It is under network configuration to determine the applied PDCCH monitoring reduction so that traffic quality criteria’s can be met

Observation: For PDCCH monitoring adaptation case, where UE still continues to monitor PDCCH, albeit at reduced rate, there may not be any need to have special handling of HARQ re-transmissions scheduling, but scheduling can follow the applied SS set(s).

Only in the case when UE stops the PDCCH monitoring for an extended duration, e.g. via skipping duration that results IC-DRX inactivity timer to expire, there can be need for special handling of the HARQ re-transmissions.

Observation: Special handling of HARQ re-transmissions is only needed when UE stops the PDCCH monitoring for extended time.

Therefore it is proposed that when UE is triggered to change to {empty} SSSG/skip PDCCH monitoring, special handling of scheduling open a re-transmission (DL/UL) can be configured. This could be achieved in simplest way using similar configuration as in C-DRX, so that timers, such as drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDL, can be provided to the UE, allowing determining time window for handling the possible scheduling of re-transmissions.

Proposal: For stopping PDCCH monitoring based on {empty} SSSG or PDCCH skipping, define timers similarly as in C-DRX operation to enable configuring time windows for handling the open re-transmissions.

Other aspects

In RAN1 #104 it was discussed that relaxing the PDCCH monitoring can be connected to the minimum scheduling offset. I.e. if minimum cross-slot offset scheduling restriction (e.g. K0min>0) is applied, this would result implicitly to a corresponding reduction in PDCCH monitoring. While in Rel-16 evaluations it was assumed that K0min=1 would be sufficient to ensure power saving, this may not be the case in practise. Therefore, binding implicitly the PDCCH monitoring to e.g. the applied K0min would result in higher overall latency, by delaying both, DCI and the PDSCH. This would also limit throughput that could be achieved with minimum cross-slot offset scheduling restriction, because the network would only be able to schedule the UE according to the minimum scheduling offset x, i.e. every K0min th PDCCH. However, if power saving benefit is seen sufficient, it may be possible to consider to link different SSSGs with different K0,min and/or K2,min. So, for example, for a certain SSSG with relaxed PDCCH monitoring the Kx, min-value(s) could also be larger than 0 slots. One approach can be to associate the Kx, min-value(s) with the SSSG, such that when the UE switches SSSG it will automatically also apply the associated Kx, min-value(s) without need for further configuration signalling. This would enable to separate the ‘periodicities’ assumed for DCI and PDSCH/PUSCH.

Observation: Associating minimum cross-slot scheduling restriction to certain SSSGs could be considered.

# Conclusion

This contribution we have presented power saving evaluations for SS set groups switching and PDCCH skipping in different scenarios.

In Section 2 were present the evaluation of signalling overhead related to the PDCCH monitoring adaptation considering SSSG switching and PDCCH skipping:-

Observation: SSSG switching has lower signalling overhead than PDCCH skipping for most of the evaluated traffic scenarios.

In the continued discussion in RAN1#105e it was agreed that SSSG switching at least with 2 SSSGs wil be supported in Rel-17 for PDCCH monitoring adaptation and the method to introduce stopping the PDCCH monitoring, will be either introduced as a part of the SSSG frame work or as a separate functionality. Based on this we made following observations and proposals in Section 3:-

Observation: A field indicating the target SSSG index could be added to the scheduling DCI.

Proposal: Support Rel-17 SSSG switching, in addition to scheduling DCIs also for DCI format 2_0. For multi cell operation consider introducing similar mechanism as for Scell dormancy e.g. via DCI format 1_1 (SCell dormancy case 2).

Proposal: Increase the number of SS set groups at least from 2 to 3.

Proposal: Support timer-based UE autonomous SS set group switching for active time power saving.

Observation: Through use of timer based SSSG switching and proper configuration of SS set stopping of PDCCH monitoring for a duration can be achieved.

Proposal: Supporting SSSG switching with ‘empty’ SSSG to embed PDCCH skipping functionality to SSSG framework.

Proposal: Procedures such as SR transmission, BSR or beam failure recovery should result UE to stop PDCCH monitoring adaptation and resume normal PDCCH monitoring.

Observation: In case of C-DRX, timer-based SSSG switching could be applied during the inactivity and SSSG would be switched if UE is scheduled during On Duration.

Proposal: Support configuring a default SS set group that is applied during On Duration, at least when DCP is configured.

Proposal: PDCCH monitoring adaptation should not be applied to Type0/0A/1 or 2 PDCCH CSS. For Type3-PDCCH CSS adaptation could be considered, but the monitoring should not be stopped completely.

Proposal: Use the application delay timeline introduced in Rel-16 for SSSG switching.

Observation: For PDCCH monitoring adaptation case, where UE still continues to monitor PDCCH, albeit at reduced rate, there may not be any need to have special handling of HARQ re-transmissions scheduling, but scheduling can follow the applied SS set(s).

Observation: Special handling of HARQ re-transmissions is only needed when UE stops the PDCCH monitoring for extended time.

Proposal: For stopping PDCCH monitoring based on {empty} SSSG or PDCCH skipping, define timers similarly as in C-DRX operation to enable configuring time windows for handling the open re-transmissions.

Observation: Associating minimum cross-slot scheduling restriction to certain SSSGs could be considered.

# References

- RP-193239, ”New WID: UE Power Saving Enhancements”, Mediatek

- TR 38.840 v16.0.0, ” Study on User Equipment (UE) power saving in NR”, 3GPP

- R1-2006898, ” UE power saving enhancements for Active Time”, Nokia, Nokia Shanghai Bell

- R1-2101666, “Evaluation of Active Time enhancements”, Nokia, Nokia Shanghai Bell, 2021