3GPP TSG RAN WG1 #106-e		R1-2108132

e-Meeting, August 16th – 27th, 2021

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on RAN2 LS on resource reselection trigger sl-reselectAfter

Document for:	Discussion, Decision

# 1	Introduction

Question 1: In NR V2X, whether “unused retransmission opportunities in case of HARQ feedback is enabled” shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection?

Question 2: In NR V2X, according to RAN1 agreement, is it the correct understanding that only the resources already indicated in SCI shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection?

RAN WG1 received a LS from RAN WG2 regarding whether to count as unused retransmission opportunities for the case where HARQ feedback is enabled, and it has been indicated that re-transmission of the MAC PDU is not needed. An additional question regarding which resources have to be counted for unused transmission opportunities is also included as follows in R2-2106625:

# 2	Discussion

In our view, skipped retransmissions opportunities when HARQ feedback has indicated that retransmission of the MAC PDU is not needed shall not be counted in order to trigger resource re-selection. The unused retransmission opportunities when HARQ feedback is enabled but the transmission has been successful, i.e., re-transmission of the MAC PDU is not needed shall not be treated as unused opportunities since the MAC PDU has already indicated that these opportunities are not needed. Therefore, the UE will just ignore these retransmissions opportunities regardless of any other condition and should not be counted as unused or to increase the parameter sl-reselectAfter.

Unused retransmissions opportunities – when HARQ feedback has indicated that retransmission of the MAC PDU is not needed – shall not be counted in order to trigger resource re-selection.

The second topic to address is when to consider resources to be counted towards consecutive unused transmission opportunities. In this case, we have the same understanding as RAN WG2 and only the resources already indicated in SCI shall be counted as consecutive unused transmission opportunities.

Confirm the understanding of RAN WG2 and count towards the consecutive unused transmission opportunities only those already indicated in SCI.

Regarding the issue discussed in this contribution, we have prepared a draft reply LS in R1-2108127.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Unused retransmissions opportunities – when HARQ feedback has indicated that retransmission of the MAC PDU is not needed – shall not be counted in order to trigger resource re-selection.

Proposal 2	Confirm the understanding of RAN WG2 and count towards the consecutive unused transmission opportunities only those already indicated in SCI.