3GPP TSG RAN WG1 Meeting #106-e	R1-2108183

E-Meeting, August 16th – 27th, 2021

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on RAN2 LS on resource reselection trigger sl-reselectAfter

Document for:	Discussion and Decision

- Introduction

RAN2 sent an LS to RAN1 [1], where the following two questions were asked:

Question 1: In NR V2X, whether “unused retransmission opportunities in case of HARQ feedback is enabled” shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection?

Question 2: In NR V2X, according to RAN1 agreement, is it the correct understanding that only the resources already indicated in SCI shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection?

RAN2 requested RAN1 to provide feedback on Q1 and Q2.

- Discussions

Answer to Q1

Since the current value range of sl-reselectAfter, which is reused from LTE-V, is relatively small, when the number of consecutive unused transmission opportunities is calculated in case of HARQ feedback enabled, the unused retransmission opportunities should not be counted. Otherwise, it may result in frequent resource re-selection, which leads to over-booking of resources and degrades the performance of mode 2.

Proposal 1:  Reply to Q1 as follows:

- From RAN1 perspective, the “unused retransmission opportunities in case of HARQ feedback is enabled” shall not be counted towards “consecutive unused transmission opportunities” to trigger resource reselection.

Answer to Q2

According to the following agreements in RAN1#98, resources that are not indicated in SCI are either not detectable by other UEs or not actually to be used by the UE itself, thus counting those resources towards “consecutive unused transmission opportunities” does not help either itself or other UEs in terms of avoiding resource collision.

Agreements:

- In Mode-2, SCI payload indicates sub-channel(s) and slot(s) used by a UE and/or reserved by a UE for PSSCH (re-)transmission(s)

- SL minimum resource allocation unit is a slot

- FFS whether when the resource allocation is multiple slots, the slots can be aggregated

- FFS whether in case of multiple slots, the indicated slots are contiguous or not

Proposal 2:  Reply to Q2 as follows:

- From RAN1 perspective, only the resources already indicated in SCI shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection.

- Conclusions

Following proposals were made:

Proposal 1:  Reply to Q1 as follows:

- From RAN1 perspective, the “unused retransmission opportunities in case of HARQ feedback is enabled” shall not be counted towards “consecutive unused transmission opportunities” to trigger resource reselection.

Proposal 2:  Reply to Q2 as follows:

- From RAN1 perspective, only the resources already indicated in SCI shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection.

# References

- R2-2106625, “LS on resource reselection trigger sl-reselectAfter”, RAN2#114-e, May 19 – May 27, 2021.