3GPP TSG-RAN WG1 Meeting #106-e	R1-2108189

e-Meeting, August 16th – 27th, 2021

Agenda Item:	7.2.8

Source:	Huawei, HiSilicon

Title:	Aligning PRS duration calculation with RAN4

Document for:	Discussion and decision

# Introduction

In RAN4#98b, RAN4 made the following agreement in the WF R4-2105851 [1]

P2 - If muting option 1 is applied, the periodicity of a PRS resource is scaled by N_muting - N_muting = X * dl-prs-MutingBitRepetitionFactor - X is the size of NR-MutingPattern-r16 for mutingOption1-r16. P4 - For information: the following steps are used in deriving the measurement period requirements - A): The PRS periodicity indicated by “NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16” - B): Scale the PRS periodicity based on inter-period muting pattern - C): Derive the frequency layer specific periodicity (TPRS,i) if multiple periodicities are configured in this layer - D): Derive the available periodicity within MGs (Tavailable_PRS,i) - E): Derive the effective periodicity based on PRS processing time (Teffect,i) - Note: step B) is pending on conclusion on issue 1-1-1. If requirements with PRS resource muting is not defined, step B) is not needed P5 - LPRS,i for PFL i should be calculated by aggregating the duration of all the PRS resources that fall within MGs and are not muted

In RAN4#99, RAN4 made the following agreements in the WF R4-2108294 [2].

Agreement: Lprs is calculated by referring to PRS duration K defined in TS 38.214

The related discussion was documented in [3].

In the latest update of TS 38.133, the PRS measurement period was updated as below [4].

TRSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below: TRSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast , where: NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1; and in FR2, NRxBeam,i = 8. CSSFPRS,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in clause 9.1.5.2. NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot. Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. Nsample is the number of PRS RSTD samples and Nsample= 4. Tlast is the measurement duration for the last PRS RSTD sample, including the sampling time and processing time, Tlast = Ti + Tavailable_PRS,i , Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as: Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i Where, Ti corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34], Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi. MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the PRS frequency layer i. TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i. If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive the measurement period of that positioning frequency layer i. Where, TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource, TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity. Nmuting is the scaling factor considering PRS resource muting. If bitmap b1 for higher-layer parameter DL-PRS-MutingPattern is provided, and TperPRS*TmutingPRS ≤10240ms, then Nmuting=TmutingPRS*min(L,10240TperPRS*TmutingPRS); otherwise, if bitmap b1 is not provided or TperPRS*TmutingPRS>10240ms, then Nmuting=1. TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and L is the size of the bitmap b1. - Note: For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

The existing PRS duration calculation K in TS 38.214 is based on maximum PRS periodicity in a positioning frequency layer.

For the case when measurement gap is configured, the UE DL PRS processing capability is defined in [TS 37.355]. For the purpose of DL PRS processing capability, the duration K msec of DL PRS symbols within P msec window corresponding to the maximum PRS periodicity in a positioning frequency layer, is calculated by

# Cover sheet

## Reason of change

Since RAN4 already changed calculation of the effective periodicity of a positioning frequency layer taking into account the following aspects, RAN1 may also do the change to the K calculation to align with RAN4 [4], as already proposed by [5][6].

Muting impact

MG alignment

The periodicity profile among TPRs (using LCM instead of maximum value)

## Summary of changes

The change in our view should include the following

First, the evaluation window of P should no longer be corresponding to the maximum PRS periodicity, and the simplest way is to cite the RAN4 terminology TPRS,i.

Second, in the duration calculation equation, the counted PRS slot/symbol should be mapping to those PRS that are neither muted nor outside the MG. From RAN1 perspective, those PRS could be referred to as “PRS to process”, as UE is not expected to process those PRS outside the MG or muted.

Third, we think that the wording “For the purpose of DL PRS processing capability” is no longer required, and may sometimes be interpreted inaccurately. The suggested wording can be “For the purpose of DL PRS processing”.

## Consequence if not approved

The consequence is straightforward, in that RAN1 spec is not aligned with RAN4 spec, which creates ambiguity.

## Isolated impacts

Since the change is to align with RAN4 specification, there should be no inter-operability issue between UE and LMF.

# TP

Based on the discussion above, we have the following TP.

Proposal 1:  Adopt the following TP for PRS duration calculation in clause 5.1.6.5 of TS 38.214.

- =========================== Unchanged parts ===========================

For the case when measurement gap is configured, the UE DL PRS processing capability is defined in [TS 37.355]. For the purpose of DL PRS processing, the duration K msec of DL PRS symbols within P msec window corresponding to TPRS,i as defined in clause 9.9 of [11, TS 38.133] in positioning frequency layer i, is calculated by

-	Type 1 duration calculation with UE symbol level buffering capability

K=s∈SKs
Ks=Tsend-Tsstart

-	Type 2 duration calculation with UE slot level buffering capability

K=12μS

-	S is the set of slots based on the numerology of the DL PRS of a serving cell within the P msec window in the positioning frequency layer that contains potential DL PRS resources to process considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets.

-	For Type 1, Tsstart, Tsend is the smallest interval in msec within slot s corresponding to an integer number of OFDM symbols based on the numerology of the DL PRS of a serving cell that covers the union of the potential PRS symbols to process and determines the PRS symbol occupancy within slot s, where the interval Tsstart, Tsend considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS resource sets (target and reference).

-	For Type 2, μ is the numerology of the DL PRS, and S is the cardinality of the set S.

- =========================== Unchanged parts ===========================

# Reference

- R4-2105851, WF on UE PRS measurement requirements, Huawei, HiSilicon, TSG-RAN WG4 Meeting #98-bis-e, Electronic Meeting, 12 – 25 April, 2021.

- R4-2108294, WF on UE PRS measurement requirements, Huawei, HiSilicon, TSG-RAN WG4 Meeting #99-e
Electronic Meeting, 19 – 27 May, 2021.

- R4-2108036, Email discussion summary: [99-e][214] NR_pos_1, Huawei, HiSilicon, TSG-RAN WG4 Meeting #99-e, Electronic Meeting, 19 – 27 May, 2021.

- 3GPP TS 38.133, NR; Requirements for support of radio resource management.

- R1-2102948, Maintenance on Rel-16 NR positioning, vivo, RAN1#104bis-e, e-Meeting, 12 – 20 April, 2021.

- R1-2105470, Maintenance on Rel-16 NR positioning, vivo, RAN1#105-e, e-Meeting, 10 – 27 May, 2021.