3GPP TSG RAN WG1 Meeting #106-e	  R1-2108201

E-meeting, August 16th – 27th, 2021

Agenda Item:	7.2.5

Source:	Moderator (Huawei)

Title:	Summary of preparation phase email discussion for Rel-16 eURLLC

Document for:	Discussion and Decision

Introduction

The contributions submitted for agenda item 7.2.5 (Rel-16 URLLC/IIoT) have been reviewed and a number of draft CRs have been selected and proposed for further email discussion in RAN1#106-e. The criteria for the selection of the draft CRs for further email discussion was whether the issue raised was critical or not. The full list of issues considered in the preparation phase can be found in the attached excel spreadsheet along with comments from individual companies.

Summary of detailed issues

A brief summary of the issues are given in the following tables with initial recommendations before preparation phase email discussion.

| Issue | Description | Source | Initial recommendations (before prep phase email discussion) |
| --- | --- | --- | --- |
| #1 | UE PDSCH processing time for DCI format 1_2 | Ericsson (R1-2106672), ZTE (R1-2106732) | Postpone to next meeting. The rel-16 discussion for DCI format 1_2 depends on the clarification of Rel-15/16 behavior in Rel-15 maintenance session. |
| #2 | Draft CR on PDCCH monitoring capability | OPPO R1-2107265 | Discuss over email in RAN1#106-e. 1. The first change on 'per slot' and 'per span' seems unnecessary, since there seems no misunderstanding if you look at all related definitions. However, let's hear more views from companies and if possible it is always good to make the spec clearer. 2. The second change is necessary and simple RRC parameter change. |
| #3 | Correction on SRS resource configuration | Qualcomm R1-2107318 | More views from other companies are needed before making decision whether to discuss this issue, since it is kind of revisit the agreement achieved in RAN1#105-e. |
| #4 | Correction on RNTI in Table 6.1.2.1.1-1 for determining PUSCH time domain resource allocation for DCI format 0_0 | Sharp R1-2107787 | Discuss over email in RAN1#106-e. The change is necessary and seems simple correction. |
| #5 | Draft 38.214 CR on non-Codebook based UL transmission with DCI format 0_2 | Vivo R1-2107986 | Discuss over email in RAN1#106-e. The change is necessary and seems simple correction. |
| #6 | Sub-slot Based HARQ-ACK Feedback for MAC CE Activation/deactivation | Ericsson R1-2106674, CATT R1-2106931, OPPO R1-2107263, Vivo R1-2107985 | Discuss over email in RAN1#106-e. |
| #7 | HARQ-ACK timing for sub-slot based HARQ-ACK feedback | OPPO R1-2107266, Huawei, HiSilicon R1-2107681 Apple R1-2107713, Ericsson R1-2107063 | Discuss over email in RAN1#106-e. Follow up from issues identified from previous meeting in Rel-15 maitenance session on HARQ-ACK timing for sub-slot based HARQ-ACK feedback. |
| #8 | Conflict between the first PUCCH repetition and semi-static configuration in Rel-15/16 | Nokia, Nokia Shanghai Bell (R1-2107555) | Discuss over email in RAN1#106-e. Follow up from issue identified in the previous meeting on conflict between the first PUCCH repetition and semi-static configuration in Rel-15/16. |
| #9 | Remaining issues on UL prioritization and UL skipping | Huawei, HiSilicon (R1-2106487) Ericsson R1-2106675, ZTE R1-2106731, Samsung R1-2106861, CATT R1-2106930, OPPO R1-2107269, Qualcomm R1-2107319, Nokia R1-2107556, Apple R1-2107714, Vivo R1-2107984 | Discuss over email in RAN1#106-e. Follow up from issues identified from previous meetings on UL prioritization and UL skipping. |
| #10 | UE Procedures for UCI Multiplexing and Prioritization | Ericsson R1-2106673, Nokia R1-2106825/ R1-2107557, OPPO R1-2107270/ R1-2107271, Huawei R1-2107680, Apple R1-2107715, Vivo R1-2107982 | Discuss over email in RAN1#106-e. Follow up from issues from previous meetings on UE Procedures for UCI Multiplexing and Prioritization |
| #11 | Correction on overlapping between SPS HARQ-ACK with HP and SPS HARQ-ACK with LP | Huawei, HiSilicon (R1-2106489) | Discuss over email in RAN1#106-e. The proposed change is to capture one missing case of the agreements achieved in RAN1#101-e for handling collision between a high priority configured UL transmission and low priority channels. |
| #12 | Correction for PUSCH repetition Type B in 38.213 (38.214) | Huawei, HiSilicon (R1-2108199, R1-2106512) | Discuss over email in RAN1#106-e. 1. For collison handling between a low priority PUSCH transmission and a high priority PUCCH transmission defined in 38.213, when the PUSCH transmission is configured with PUSCH repetition type B, it is not clear whether an actual repetition or a nominal repetition should be cancelled, which may result in different understanding between gNB and UE. 2. For the third termination condition for configured UL transmission defined in 38.214, it is not clear whether it is related to an actual repetition or to a nominal repetition. |
| #13 | Correction on UL cancellation time | Huawei, HiSilicon (R1-2106513) | Discuss over email in RAN1#106-e. For the UL cancellation, there are two timelines for the UE. This first is for the determination of the RUR time region and the second is for the determination of the earliest actual symbol to be cancelled. For the second timeline, it is not clear how T_(proc,2) is calculated. |
| #14 | Correction on invalid symbol determination for PUSCH repetition Type B | CATT | Discuss over email in RAN1#106-e. CRs in R1-2104010 and R1-2106346 on half-duplex operation in CA with unpaired spectrum were captured in TS38.213 v16.6.0. Similar changes are needed for invalid symbol determination for PUSCH repetition type B in TS38.214. |
| #15 | PHR issues related to URLLC/IIOT WI | LG Electronics R1-2107434 | (Resubmission from RAN1#105-e) It was considered as low priority from the preparation phase in RAN1#105-e. Companies views are needed on whether to discuss it in this meeting. |
| #16 | multi-CSI-PUCCH-ResourceList where SPS HARQ-ACK multiplexed | Huawei, HiSilicon (R1-2106488), Ericsson R1-2106677, Nokia, Nokia Shanghai Bell (R1-2106826, R1-2107557), Vivo R1-2107983 | Discuss over email in RAN1#106-e. Follow up from issue identified from previous meetings on multi-CSI-PUCCH-ResourceList where SPS HARQ-ACK multiplexed. |
| #17 | SPS Release and SPS PDSCH Receptions with Slot Aggregation | Ericsson R1-2106676, Nokia R1-2106827, Nokia Shanghai Bell R1-2106862, Samsung R1-2107557, Vivo R1-2107983 | Discuss over email in RAN1#106-e. To capture the agreements and conclusions from previous meetings. On SPS Release and SPS PDSCH Receptions w/wo Slot Aggregation |
| #18 | Draft CR on collison handling for overlapping SPS PDSCHs with SPS release | OPPO R1-2107264 | Discuss over email in RAN1#106-e. It seems clarifications/discussions needed on whether/how to include processing time for SPS PDSCH release when handling collision among overlapping SPS PDSCHs. |
| #19 | Discussion on Type-1 HARQ-ACK codebook generation for SPS release and SPS PDSCH | OPPO R1-2107267, R1-2107268 | Discuss over email in RAN1#106-e. Clarify that if a UE is configured to receive more than one SPS PDSCHs and indicated to receive SPS PDSCH release in a slot whose HARQ-ACK information would be multiplexed in a same PUCCH and their HARQ-ACK information share same location in the Type-1 HARQ-ACK codebook, UE reports HARQ-ACK information for SPS PDSCH release and does not generate HARQ-ACK information for the more than one SPS PDSCHs. |
| #20 | Correction on SPS overlap with dynamic PDSCH in TS 38.214 | Lenovo, Motorola Mobility (R1-2108025) | R1-2108025 trying to clarify that a gap of 14 symbols between the PDCCH and the earliest starting symbol of SPS PDSCHs applies only with respect to SPS PDSCHs that are survivors according to the survivor PDSCH procedure described in this Clause 5.1. It seems the change is unnecessary since it should be clear that it is survivor PDSCH (s) from the following definition in the spec on the steps for a UE to receive one or more PDSCHs without corresponding PDCCH transmissions in the slot. However, let's hear more views from companies and if possible it is always good to make the spec clearer. |

Email discussion topics in RAN#106-e (Outcome of the prep phase)

The following email threads were taken as the outcome of the email discussion on the scope of email threads:

[106-e-NR-L1enh-URLLC-01] Issue#3: Correction on SRS resource configuration by August 20 – Wei (Qualcomm)

[106-e-NR-L1enh-URLLC-02] Issue#6: Sub-slot Based HARQ-ACK Feedback for MAC CE Activation/deactivation by August 20 – Yufei (Ericsson)

[106-e-NR-L1enh-URLLC-03] Issue#7: HARQ-ACK timing for sub-slot based HARQ-ACK feedback by August 20 – Sigen (Apple)

[106-e-NR-L1enh-URLLC-04] Issue#8: Conflict between the first PUCCH repetition and semi-static configuration in Rel-15/16 – Klaus (Nokia)

[106-e-NR-L1enh-URLLC-05] Issue#9: Remaining issues on UL prioritization and UL skipping by August 20 – Lihui (vivo)

[106-e-NR-L1enh-URLLC-06] Issue#10: UE Procedures for UCI Multiplexing and Prioritization by August 20 – Kianoush (Qualcomm)

[106-e-NR-L1enh-URLLC-07] Issue#11: Correction on overlapping between SPS HARQ-ACK with HP and SPS HARQ-ACK with LP by August 20 – Thorsten (Huawei)

[106-e-NR-L1enh-URLLC-08] Issue#12: Correction for PUSCH repetition Type B in 38.213 (38.214) by August 20 – Thorsten (Huawei)

[106-e-NR-L1enh-URLLC-09] Issue#14: Correction on invalid symbol determination for PUSCH repetition Type B by August 20 – Yanping (CATT)

[106-e-NR-L1enh-URLLC-10] Issue#16: multi-CSI-PUCCH-ResourceList where SPS HARQ-ACK multiplexed by August 20 – Duckhyun (LG)

[106-e-NR-L1enh-URLLC-11] Issue#17: SPS Release and SPS PDSCH Receptions with Slot Aggregation by August 20 – Duckhyun (LG)

[106-e-NR-L1enh-URLLC-12] Issue#18: Draft CR on collision handling for overlapping SPS PDSCHs with SPS release by August 20 – Yi (OPPO)

[106-e-NR-L1enh-URLLC-13] Discussion on editorial spec changes for recommendation to the editors (Second change on RRC parameter BlindDetectionCA1 of issue #2, Issue#4, Issue#5) by August 20 – Chengyan (Huawei)

- References

- R1-2106487	Remaining issues on UL prioritization and UL skipping	Huawei, HiSilicon

- R1-2106488	CSI-PUCCH-ResourceList where SPS HARQ-ACK multiplexed	Huawei, HiSilicon

- R1-2106489	Correction on overlapping between SPS HARQ-ACK with HP and SPS HARQ-ACK with LP	Huawei, HiSilicon

- R1-2106511	Correction for PUSCH repetition Type B in 38.213	Huawei, HiSilicon

- R1-2106512	Correction for PUSCH repetition Type B in 38.214	Huawei, HiSilicon

- R1-2106513	Correction on UL cancellation time	Huawei, HiSilicon

- R1-2106672	UE PDSCH processing time for DCI format 1_2	Ericsson

- R1-2106673	UE Procedures for UCI Multiplexing and Prioritization	Ericsson

- R1-2106674	Sub-slot Based HARQ-ACK Feedback for MAC CE Activation/deactivation	Ericsson

- R1-2106675	Intra-UE Multiplexing/Prioritization and UL Skipping	Ericsson

- R1-2106676	SPS Release and SPS PDSCH Receptions with Slot Aggregation	Ericsson

- R1-2106677	Multi-CSI-PUCCH-ResourceList with Multiplexed SPS HARQ-ACK	Ericsson

- R1-2106730	Draft CR on Type-2 HARQ-ACK codebook size calculation	ZTE

- Withdrawn

- R1-2106731	Remaining issues on intra-UE multiplexing in Rel-16 URLLC	ZTE

- R1-2106732	Discussion on PDSCH processing timeline for DCI format 1_2	ZTE

- R1-2106825	[Draft CR] Correction on UE procedure for intra-UE prioritization/multiplexing	Nokia, Nokia Shanghai Bell

- R1-2106826	Draft CR] Clarification on Inter-sub-slot multiplexing of low-priority UCIs	Nokia, Nokia Shanghai Bell

- R1-2106827	[Draft CR] Handling of HARQ-ACK feedback for SPS release	Nokia, Nokia Shanghai Bell

- R1-2106861	Discussion on PUSCH skipping for URLLC	Samsung

- R1-2106862	Draft CR on SPS release with aggregation factor	Samsung

- R1-2106930	Discussion on overlapping between CG PUSCH and DG PUSCH	CATT

- R1-2106931	Correction on MAC CE effective time for spatial setting for a PUCCH transmission	CATT

- R1-2106932	Correction on invalid symbol determination for PUSCH repetition Type B	CATT

- R1-2107263	Draft CR on PUCCH spatial setting when subslotLength-ForPUCCH is provided	OPPO

- R1-2107264	Draft CR on collison handling for overlapping SPS PDSCHs with SPS release	OPPO

- R1-2107265	Draft CR on PDCCH monitoring capability	OPPO

- R1-2107266	Remaining issues on HARQ-ACK timing for sub-slot based HARQ-ACK feedback	OPPO

- R1-2107267	Draft CR on Type-1 HARQ-ACK codebook generation for SPS release and SPS PDSCH	OPPO

- R1-2107268	Discussion on Type-1 HARQ-ACK codebook generation for SPS release and SPS PDSCH	OPPO

- R1-2107269	Remaining issues on intra-UE prioritization or multiplexing	OPPO

- R1-2107270	Draft CR on  scheduling and HARQ	OPPO

- R1-2107271	Discussion on  scheduling and HARQ	OPPO

- R1-2107318	Remaining issues on PDCCH enhancements for URLLC	Qualcomm Incorporated

- R1-2107319	Remaining issues on eCG enhancements for URLLC	Qualcomm Incorporated

- R1-2107434	PHR issues related to URLLC/IIOT WI	LG Electronics

- R1-2107555	UCI enhancements maintenance: Conflict between the first PUCCH repetition and semi-static configuration in Rel-15/16	Nokia, Nokia Shanghai Bell

- R1-2107556	Rel-16 URLLC/IIoT PUSCH skipping (with LCH and/or PHY prioritization configured)	Nokia, Nokia Shanghai Bell

- R1-2107557	Discussion on Nokia draft CRs on Rel-16 URLLC/IIoT Scheduling/HARQ and SPS enhancements	Nokia, Nokia Shanghai Bell

- R1-2107680	Remaining issues on Scheduling and HARQ	Huawei, HiSilicon

- R1-2107681	Correction for HARQ-ACK timing in Rel-16	Huawei, HiSilicon

- R1-2107713	Discussions on HARQ-ACK timing for sub-slot-based HARQ-ACK feedback in Rel-16	Apple

- R1-2107714	UCI multiplexing and PUSCH skipping design for eURLLC	Apple

- R1-2107715	Remaining issues on intra-UE multiplexing/prioritization for eURLLC	Apple

- R1-2107787	Correction on RNTI in Table 6.1.2.1.1-1 for determining PUSCH time domain resource allocation for DCI format 0_0	Sharp

- R1-2107982	Clarification on UE procedure for prioritization	vivo

- R1-2107983	Maintenance on SPS enhancements	vivo

- R1-2107984	Maintenance on intra-UE prioritization and multiplexing	vivo

- R1-2107985	Draft CR on PUCCH spatial relation update	vivo

- R1-2107986	Draft 38.214 CR on non-Codebook based UL transmission with DCI format 0_2	vivo

- R1-2108025	Correction on SPS overlap with dynamic PDSCH in TS 38.214	Lenovo, Motorola Mobility

- R1-2108199	Correction for PUSCH repetition Type B in 38.213	Huawei, HiSilicon