3GPP TSG RAN WG1 #106-e		R1-2108377

e-Meeting, Aug 16th – 27th, 2021

Source:	OPPO

Title:	Evaluation results for XR evaluation

Agenda Item:	8.14.3

Document for:	Discussion and Decision

- Introduction

A study item of XR and could game (CG) evaluations for NR was approved in RAN#88e with the following objectives [1]:

- Confirm XR and Cloud Gaming applications of interest - Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc. - Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios - Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs

To be specific, there are four typical applications suggested as the starting points for this study [1]:

- VR1: “Viewport dependent streaming”

- VR2: “Split Rendering: Viewport rendering with Time Warp in device”

- AR1: “XR Distributed Computing”

- AR2: “XR Conversational”

- CG: Cloud Gaming

In previous RAN1 meetings, there were some discussions on the traffic models and evaluation methodologies. Some pertinent agreements for traffic models and simulation assumptions were made in [2][3][4].

In this contribution, we present some initial evaluation results of the system capacity based on the agreed traffic models and simulation assumptions.

- System capacity

In this section, we will show some initial evaluation results of the system capacity in Dense Urban scenarios.

In our simulator, there are some common processing for all evaluation cases:

- Although a packet is exceeding the delay, it is still delivered.

- Equal number of UEs per cell is used for each drop.

- A UE is declared a satisfied UE if more than 99% of packets are successfully delivered within a given air interface PDB

- DL video stream for 60 frame per second (fps)

- Average data rate of DL video stream: 30Mpbs

- Packet size is generated according to the truncated Gaussian distribution with both upper and lower bounds, where the following parameters are used: [STD, Max, Min]: [10.5, 150, 50] % of Mean packet size.

Evaluation for Dense Urban scenario

In this section, we will provide simulation results for Dense Urban scenario. The details of simulation parameters are summarized in Table 1.

Table 1 simulation parameters for Dense Urban

| Parameters | Value |
| --- | --- |
| Scenario | Dense Urban hexagonal layout with (19, 3) Sectors |
| Inter-BS distance | 200m |
| Carrier frequency | 4GHz |
| Duplex Mode / Simulation bandwidth | 100MHz |
| SCS | 30KHz |
| TDD pattern | DDDSU |
| BS Antenna Configuration | 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) (dH, dV) = (0.5λ, 0.5λ) |
| UE Antenna Configuration | 4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| Transmit Power | 51dBm |
| Antenna Height | 3 m for BS and 1.5 m for UE |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |
| UE speed | 3km/h |
| Scheduling Algorithm | SU-MIMO+PF |
| Channel estimation | Realist |
| BS receiver | MMSE-IRC |

Evaluation result for PDB=10ms

In this simulation, air interface PDB is assumed as 10ms and three packet arrival models are evaluated. Figure 1 is an illustration for packet arrival model for multiple UEs.

- Case 1: the interval of packet arrival among UEs are random

- Case 2: the interval of packet arrival among UEs are equal

- Case 3: the interval of packet arrival among UEs are zero, i.e. packet arrival among UEs are synchronized

Figure 1 Packet arrival model for multiple UEs

The simulation results of system capacity for NR supporting VR/AR DL video stream with jitter range of [-4ms, 4ms] are shown in Figure 2.

Figure 2: Ratio of the satisfied UE with the PDB of 10ms (30 Mbps) with Jitter

According to the simulation results shown in the figure, we can make the following observations:

Observation 1: In FR1, for XR/VR DL video stream with 30Mbps @60 fps with jitter range of [-4ms, 4ms] and PDB=10ms in Dense Urban scenarios

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 8.4 users for case 1

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 9.2  users for case 2

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 7.4 users for case 3

The simulation results of system capacity for NR supporting VR/AR DL video stream without jitter are shown in Figure 3.

Figure 3: Ratio of the satisfied UE with the PDB of 10ms (30 Mbps) without jitter

According to the simulation results shown in the figure, we can make the following observations:

Observation 2: In FR1, for XR/VR DL video stream with 30Mbps @60 fps without jitter and PDB=10ms in Dense Urban scenarios

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 9 users for case 1

  - If 90% of UEs are satisfied (baseline), the UE number per cell about 10.5 users for case 2

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 7.1 users for case 3

Evaluation result for PDB=15ms

In this simulation, air interface PDB is assumed as 15ms and three packet arrival models are evaluated. The simulation results of system capacity for NR supporting CG DL video stream with jitter range of [-4ms, 4ms] are shown in Figure 4.

Figure 4: Ratio of the satisfied UE with the PDB of 15ms (30 Mbps) with Jitter

According to the simulation results shown in the figure, we can make the following observations:

Observation 3: In FR1, for XR/VR DL video stream with 30Mbps @60 fps with jitter range of [-4ms, 4ms] and PDB=15ms in Dense Urban scenarios

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about  10.2 users for case 1

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 10.3 users for case 2

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 10.3 users for case 3

The simulation results of system capacity for NR supporting VR/AR DL video stream without jitter are shown in Figure 5.

Figure 5: Ratio of the satisfied UE with the PDB of 15ms (30 Mbps) without jitter

According to the simulation results shown in the figure, we can make the following observations:

Observation 4: In FR1, for XR/VR DL video stream with 30Mbps @60 fps without jitter and PDB=15ms in Dense Urban scenarios

  - If 90% of UEs are satisfied (baseline), the UE number per cell is  about 10.5 users for case 1

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 11 users for case 2

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 10.1 users for case 3

Analysis

Simulation result for all cases are summarized in Table 2.

Table 2 Simulation results for combination of jitter model, packet arrival model and PDB

| Jitter | Packet arrival | PDB=10ms | PDB=15ms |
| --- | --- | --- | --- |
| Jitter range of [-4ms, 4ms] | Case1: Random | 8.4 | 10.2 |
|  | Case 2: Equal | 9.2 | 10.3 |
|  | Case 3: Syn | 7.4 | 10.3 |
| No Jitter | Case1: Random | 9 | 10.5 |
|  | Case 2: Equal | 10.5 | 11 |
|  | Case 3: Syn | 7.1 | 10.1 |

Comparing PDB=10ms and PDB=15ms, for most cases, the larger PDB, the larger capacity. For random packet arrival and equal packet arrival, the capacity gap between PDB=10ms and PDB=15ms is small, i.e.1-2 users. Scattered packet arrival reduces packet transmission time for each UE due to plenty of resource in one slot can be allocated for one UE. Packet transmission time for each UE is usually smaller than PDB, so larger PDB does not increase capacity significantly. For synchronized packet arrival, the capacity gap between PDB=10ms and PDB=15ms is significant, i.e. 3 users. Crowded packet arrival increases packet transmission time for each UE due to very limited resource in one slot can be allocated for each UE. Packet transmission time for some UEs is around PDB, so relaxed PDB could increase the number of satisfied UEs.

Observation 5: For crowded packet arrival, relax PDB could increase capacity.

Comparing different packet arrival model, the scattered packet arrival, the larger capacity. The capacity of equal packet arrival and the capacity of random packet arrival are similar, i.e. the capacity gap is 0.1-1.5 users. And the capacity of equal packet arrival or random packet arrival is larger than the capacity of synchronized packet arrival with PDB=10ms. Similar as above analysis, scattered packet arrival, including random packet arrival and equal packet arrival, is benefit to increase capacity. And crowded packet arrival, i.e. synchronized packet arrival, decrease capacity.

Observation 6: Scattered packet arrival, including random packet arrival and equal packet arrival, is benefit to increase capacity. And crowded packet arrival, i.e. synchronized packet arrival, decreases capacity, especially for smaller PDB.

Comparing jitter and non-jitter, for equal packet arrival and random packet arrival, the capacity of jitter case and non-jitter case is similar and the capacity of non-jitter case is about one larger than the capacity of jitter case due to packet arrival is a little more even for non-jitter case.

Observation 7: For equal packet arrival and random packet arrival, the capacity of jitter case and capacity of non-jitter case are similar.

- Conclusions

In this contribution, we provide some initial simulation results for XR/CG service in Dense Urban scenario and have the following observation:

Observation 1: In FR1, for XR/VR DL video stream with 30Mbps @60 fps with jitter range of [-4ms, 4ms] and PDB=10ms in Dense Urban scenarios

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 8.4 users for case 1

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 9.2 users for case 2

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 7.4 users for case 3

Observation 2: In FR1, for XR/VR DL video stream with 30Mbps @60 fps without jitter and PDB=10ms in Dense Urban scenarios

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 9 users for case 1

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 10.5 users for case 2

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 7.1 users for case 3

Observation 3: In FR1, for XR/VR DL video stream with 30Mbps @60 fps with jitter range of [-4ms, 4ms] and PDB=15ms in Dense Urban scenarios

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about  10.2 users for case 1

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 10.3 users for case 2

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 10.3 users for case 3

Observation 4: In FR1, for XR/VR DL video stream with 30Mbps @60 fps without jitter and PDB=15ms in Dense Urban scenarios

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 10.5 users for case 1

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 11 users for case 2

  - If 90% of UEs are satisfied (baseline), the UE number per cell is about 10.1 users for case 3

Observation 5: For crowded packet arrival, relax PDB could increase capacity significantly.

Observation 6: Scattered packet arrival, including random packet arrival and equal packet arrival, is benefit to increase capacity. And crowded packet arrival, i.e. synchronized packet arrival, decreases capacity, especially for smaller PDB.

Observation 7: For equal packet arrival and random packet arrival, the capacity of jitter case and capacity of non-jitter case are similar.

- Reference

RP-201145 Study on XR Evaluations for NR

Chairman’s Notes of RAN1#103e

Chairman’s Notes of RAN1#104e

Chairman’s Notes of RAN1#104bis-e

Chairman’s Notes of RAN1#105-e