3GPP TSG-RAN WG1 Meeting #106-e	R1-2108510

# E-meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0250 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on SCell dormancy indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, OPPO, Ericsson, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core, NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2021-08-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed e?planations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Change#1: With the description “UE has not received any PDCCH within the monitoring occasions for DCI formats scheduling PDSCH receptions, or SPS PDSCH release, or DCI format 1_1 indicating SCell dormancy on any serving cell c" in section 9.1.3.2, it is not possible for UE to have HARQ-ACK information in response to a non-detected DCI format 1_1 indicating SCell dormancy. Change#2: It is not clear how to determine PDSCH-to-HARQ_feedback timing indicator field values for DCI indicating SCell dormancy. It is straightforward to reuse the same solution as DCI scheduling a PDSCH reception or a SPS PDSCH release, i.e. configured by high layer signaling. Also considering Type3 HARQ-ACK triggering without PDSCH, the two additional cases can be implicitly covered by not explicitly mentioning each of the cases. Change#3: The interpretation of the bitmap values for SCell dormancy indication in case of absence or value of 0 for carrier indicator field in DCI format 0_1 or a DCI format 1_1 is unclear, due to incorrect indentations. Change#4: Considering a SPS PDSCH release indication is carried by a DCI with CRC scrambled by a CS-RNTI, and a PDCCH is considered as a Case 2 PDCCH only if the CRC of DCI format 1_1 is scrambled by a RNTI which includes C-RNTI and MCS-C-RNTI, therefore, the description of “the UE considers the DCI format 1_1 as not indicating a SPS PDSCH release” should be removed to avoid confusion on the understanding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change#1: Delete “in response to a detection of a DCI format 1_1 indicating SCell dormancy”. Change#2: Delete “scheduling a PDSCH reception or a SPS PDSCH release,” to also cover SCell dormancy indication and Type 3 codebook. Change#3: Correct the indentation such that the apprppriate subbullets are under the condition related to the carrier indicator field. Change#4: Delete the description that Case 2 PDCCH does not indicate a SPS PDSCH release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Change#1: The descrption of specification impose unnecessary conditions for UE to check for codebook generation and is redundant. Change#2: PDSCH-to-HARQ_feedback timing indicator field value for DCI indicating SCell dormancy and Type 3 codebook is undefined. Change#3: Incorrect/unclear bit value interpretion. Change#4: Description in the specification causes confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.2, 9.2.3, 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R1-2108418 with correction on format |  |  |  |  |  |  |  |  |

9.1.3.2	Type-2 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field, then

-	if the UE has not received any PDCCH within the monitoring occasions for DCI formats scheduling PDSCH receptions, or SPS PDSCH release, or DCI format 1_1 indicating SCell dormancy on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else, the UE generates the HARQ-ACK codebook as described in clause 9.1.3.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

<Unchanged part omitted>

9.2.3	UE procedure for reporting HARQ-ACK

<Unchanged part omitted>

For DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values map to {1, 2, 3, 4, 5, 6, 7, 8}. For a DCI format, other than DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field values, if present, map to values for a set of number of slots provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, as defined in Table 9.2.3-1.

<Unchanged part omitted>

## 10.3	PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

<Unchanged part omitted>

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a Scell dormancy indication field,

-	the Scell dormancy indication field is a bitmap with size equal to a number of groups of configured Scells, provided by dormancyGroupWithinActiveTime,

-	each bit of the bitmap corresponds to a group of configured Scells from the number of groups of configured Scells

-	if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0

-	a ‘0’ value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated Scell in the corresponding group of configured Scells

-	a ‘1’ value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated Scell in the corresponding group of configured Scells, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for each activated Scell in the corresponding group of configured Scells, if the current active DL BWP is not the dormant DL BWP

-	the UE sets the active DL BWP to the indicated active DL BWP

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if

-	the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if

-	a one-shot HARQ-ACK request field is not present or has a '0' value, and if

-	the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1

the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception, and for transport block 1 interprets the sequence of fields of

-	modulation and coding scheme

-	new data indicator

-	redundancy version

and of

-	HARQ process number

-	antenna port(s)

-	DMRS sequence initialization

as providing a bitmap to each configured SCell, in an ascending order of the SCell index, where

-	a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for a corresponding activated SCell

-	a '1' value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for a corresponding activated SCell, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for a corresponding activated SCell, if the current active DL BWP is not the dormant DL BWP

-	the UE sets the active DL BWP to the indicated active DL BWP

<Unchanged part omitted>