3GPP TSG RAN WG1 #106-e	R1-2108641

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.12.2

Source:	Moderator (Huawei)

Title:	FL summary#7 on improving reliability for MBS for RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

This summary summarizes the contributions submitted in AI 8.12.2 to discuss how to improve the reliability for MBS for RRC_CONNECTED UEs.

This summary includes HARQ-ACK feedback specific issues, NACK-only specific issues, HARQ-ACK feedback common issues, HARQ-ACK feedback for SPS multicast, PDSCH repetition, and CSI feedback. In each of high level issue, a sub-level list of issues are organized.

For each of listed issue, proposal(s) is/are suggested from moderator’s perspective according to the submitted individual company’s proposal(s). Companies are welcome to make comments in the table “collect views”. The proposals may be updated in subsequent rounds according to the comments collected in previous rounds so as to strive to converge to consensus. Note that moderator may only tend to collect concerns when time is right for some specific rounds, for which companies are expected to only provide concerns in the table “collect concerns” if any instead of inputting views again and again to alleviate efforts.

People can use “navigation pane” to quickly overview the organization of the summary and proposal(s) for each issue for discussion and provide views/comments into the table of “collect view”/“collect concerns” under each proposal.

Note: for all proposals FL suggests, companies are encouraged to input views, situation can be known better, so as to progress fruitfully.

# ACK/NACK-based feedback specific

## Separate HARQ-ACK codebook

Submitted Proposals

“separate codebooks for Type-1”

(Huawei) Proposal 1:

  - UE can be configured to generate separate Type-1 HARQ-ACK codebooks for unicast and multicast with different priorities, respectively.

(Spreadtrum) proposal 2:

  - For type-1 HARQ-ACK codebook, if unicast and multicast are with different priorities, separate codebooks could be generated. If unicast and multicast are with same priority, a single HARQ-ACK codebook can be generated.

(FUTUREWEI) proposal 1:

  - The signalling for URLLC feature to configure the separate codebooks for unicast and multicast is applicable also when they are of the same priority and for Type-1 HARQ codebook.

(CMCC) proposal 6:

  - Separate codebooks for unicast and multicast are constructed for the case of different priority, for Type-1 HARQ codebook.

(Intel) Proposal 7:

  - For Type-1 HARQ codebook, joint codebook is used for same priority and different codebooks are used for different priorities.

“codebook for the same priority”

separate codebooks

(Huawei) Proposal 2:

  - UE can be configured to generate separate HARQ-ACK codebooks (including both Type-1 and Type-2 codebooks cases) for unicast and multicast with the same priority.

(Spreadtrum) proposal 1:

  - For type-2 HARQ-ACK codebook, no matter whether unicast and multicast are with same or different priorities, separate codebooks should be generated.

(Spreadtrum) proposal 2:

  - For type-1 HARQ-ACK codebook, if unicast and multicast are with different priorities, separate codebooks could be generated. If unicast and multicast are with same priority, a single HARQ-ACK codebook can be generated.

(FUTUREWEI) proposal 1:

  - The signalling for URLLC feature to configure the separate codebooks for unicast and multicast is applicable also when they are of the same priority and for Type-1 HARQ codebook.

(OPPO) Proposal 1:

  - Proposal 1: For the case of same priority for multicast and uncast, separate codebook can be configured for multicast and unicast.

Single codebook

(CMCC) proposal 5:

  - Single codebook for unicast and multicast is constructed for the case of same priority, for Type-1 and Type-2 HARQ codebook.

(MediaTek) Proposal 1:

  - For multicast HARQ-ACK codebook construction, Type-1 HARQ codebook is only applicable for the case of same priority between unicast and multicast.

(Intel) Proposal 6:

  - For Type-2 HARQ codebook for the case of same priority of unicast and multicast, a joint codebook is configured.

- “PUCCH-config codebook”

- Separate config-> separate codebook

(Huawei) Proposal 3:

  - When UE is configured with a separate PUCCH-Config for multicast from that for unicast and slot-based PUCCH and subslot-based PUCCH are configured wherein, respectively, two separate codebooks are generated independently and one of them is transmitted when the PUCCH resources are overlapped including the cases of same or different priorities.

(Huawei) Proposal 4:

  - The separate HARQ-ACK codebooks for unicast and multicast should apply to separate PUCCH transmissions only.

(CATT) Proposal 12:

  - When PUCCH-ConfigurationList for multicast and PUCCH-ConfigurationList for unicast are separately configured, UE is expected to construct separate codebooks for multicast and unicast.

(OPPO) Proposal 2:

  - If PUCCH-config/PUCCH-ConfigurationList for unicast is also applied for MBS, HARQ-ACK bits of unicast and MBS with same priority are multiplexed and transmitted in the PUCCH of the same priority level.

(OPPO) Proposal 4:

  - If separate PUCCH-config/PUCCH-ConfigurationList is configured for MBS, HARQ-ACK codebook for MBS and unicast are constructed separately and transmitted in PUCCH resource dedicated for MBS or unicast.

- Separate config-> same codebook

(ZTE) proposal 7:

  - If separate HARQ-ACK codebook generation for unicast and multicast is configured

    - UE multiplexes HARQ-ACK codebooks for unicast and multicast with the same priority in one PUCCH resource from PUCCH-config /PUCCH-ConfigurationList configured for unicast instead of multicast.

    - UE supports transmitting two non-overlapping PUCCH resources in one slot with different priorities for unicast and multicast, respectively.

(MediaTek) Proposal 2:

  - When separate PUCCH-ConfigurationList_mcast is configured for multicast:

    - If Type-1 codebook is supported, UE is expected to generate a joint HARQ codebook with the same priority between unicast and multicast, and the PUCCH resource for the Type-1 HARQ codebook is determined according to across PUCCH-config_ucast of PUCCH-ConfigurationList_ucast and PUCCH-config_mcast of PUCCH-ConfigurationList_mcast.

    - If Type-2 codebook is supported, the separated HARQ-ACK codebook will be constructed independently and be transmitted in respective resource.

- “Configuration for codebook type ”

(vivo) Proposal 4:

  - The HARQ-ACK codebook type for multicast and unicast should be separately configured.

(Nokia) Proposal 22:

  - Signaling of the type of HARQ-ACK codebooks to be produced (which codebook type to use when low / high priority service is received) is performed separately for unicast and multicast.

(Samsung) Proposal 4:

  - HARQ-ACK codebook type configuration is separate for unicast and multicast, regardless of priority.

(Samsung) Proposal 5:

  - A UE is provided separate pdsch-HARQ-ACK-CodebookList for unicast and multicast (for two priorities).

(CMCC) proposal 4:

  - A separate pdsch-HARQ-ACK-CodebookList can be configured for multicast service.

(LGE) Proposal 7:

  - Clarify whether HP HARQ-ACK can be configured for multicast in case that HP HARQ-ACK is not configured for unicast.

“whether multiplexing different priorities”

(Intel) Proposal 8:

  - Multiplexing across priorities is not allowed in a single PUCCH occasion. For overlapping PUCCH transmissions in time, dropping rule is used.

- others

(ZTE) proposal 2:

  - For Type-1 HARQ codebook, if separate codebooks for unicast and multicast is configured for the case of different priorities, the intended UE behaviour is as following.

    - For unicast codebook, UE reports ACK value(s) for HARQ-ACK information bit(s) only if UE correctly decodes a unicast PDSCH in this SLIV candidate; UE reports NACK value(s) for HARQ-ACK information bit(s) if UE successfully decodes a multicast PDSCH, doesn’t receive any PDSCH or failed to decode the unicast PDSCH.

    - For multicast codebook, UE reports ACK value(s) for HARQ-ACK information bit(s) only if UE correctly decodes a multicast PDSCH in this SLIV candidate; UE reports NACK value(s) for HARQ-ACK information bit(s) if UE successfully decodes a unicast PDSCH, doesn’t receive any PDSCH or failed to decode the multicast PDSCH.

### Round-1 (closed)

FL’s comment:

Regarding whether/when UE generates separate HARQ-ACK codebooks for unicast and multicast, the agreement achieved in the last meeting is as follows:

Agreement:

The signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast, respectively, at least for the case of different priorities, at least for Type-2 HARQ codebook

- FFS: The case for the same priority.

- FFS: The case of Type-1 HARQ codebook

- FFS: Whether this applies to separate PUCCH transmissions only

Extending this agreement to the case of Type-1 codebook seems straightforward and is clearly proposed by some companies.

Whether UE generates a single codebook or can also generate separate codebooks for unicast and multicast with the same priority is discussed by some companies. [4] companies explicitly proposed separate codebooks can be configured for the case of “same priority”. One company proposed a single codebook for the same priority, one company proposed Type-1 codebook is only applicable for the case of the same priority and one company proposed Type-2 codebook for the same priority, a joint codebook is configured.

Whether separate PUCCH-config for multicast means separate codebooks for unicast and multicast especially for the case of “same priority” is discussed by [5] companies. [3] view that separate PUCCH-config could mean separate codebooks but [2] view that UE still constructs a single codebook for the same priority even though the PUCCH-config is separately configured.

There are also [5] companies proposing the HARQ-ACK codebook type can be configured separately for unicast and multicast.

FL’s Proposal:

#### Proposal 2.1.1-1

- UE can be configured to generate separate Type-1 codebooks for unicast and multicast with different priorities, respectively.

- separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast, respectively.

Collect views:

| Company | Comments |
| --- | --- |
| CATT | A classification is needed for the proposal. What the ‘UE can be’ in the main bullet means? In our understanding, it can imply that two separate Type-1 codebooks or a joint Type-1 codebook can be generated for unicast and multicast with different priorities. If so, we are not support to generate a joint Type-1 codebook for ‘different priorities’. In R16, two HARQ-ACK codebooks can be constructed for both Type-1 and Type-2 codebooks when the UE is indicated by pdsch-HARQ-ACK-CodebookList for unicast to generate two HARQ-ACK codebooks with different priorities. Thus, separate Type-1 codebooks for unicast and multicast with different priorities should also be constructed. Moreover, we think the separate PUCCH-config for multicast does not mean separate codebooks for unicast and multicast at least for ‘different priorities’. For unicast and multicast with different priorities, separate codebooks should always be generated regardless of the PUCCH-config/PUCCH-ConfigurationList. Thus, we suggest the proposal can be updated as following: - UE can be is configured to generate separate Type-1 codebooks for unicast and multicast with different priorities, respectively. - separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast, respectively. |
| Nokia, NSB | We believe that such an agreement is unnecessary and sub-bullet is misleading. Based on the agreements made so far, the UE multiplexes the HARQ-ACK feedback of MBS and unicast with same priority when the scheduled PUCCH resources overlap (different multiplexing rules for TDM-ed and FDM-ed transmissions are discussed). In case of overlapping PUCCH resources for different priorities, low priority feedback is dropped (Rel-16 multiplexing / prioritization). In addition, UE supports two non-overlapping slot-based PUCCHs for ACK / NACK feedback for multicast and unicast with different priorities in a slot subject to UE capability. Considering these agreements, for different priorities, it is natural that the UE generates separate Type-1 (and even Type-2) codebooks for unicast and multicast. It does not depend on the configuration of a separate PUCCH-config (note that if separate PUCCH-config is not configured, UE uses unicast PUCCH-config for multicast). |
| Samsung | Agree with Nokia. Whether the codebook generation is joint or separate does not relate to whether the PUCCH-Config is joint or separate. |
| ZTE | We are ok with the main bullet. However, even if we have such agreement, the UE behavior is still not clear since UE doesn’t check the priority and GRNTI when constructing type-1 codebook based on the current spec. Thus, we suggest to add the following two bullets to clarify the desired UE behavior. #### Proposal 2.1.1-1 - UE can be configured to generate separate Type-1 codebooks for unicast and multicast with different priorities, respectively. - separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast, respectively. - For codebook for unicast, UE reports ACK value(s) for HARQ-ACK information bit(s) only if UE correctly decodes a unicast PDSCH with the desired priority in this SLIV candidate; otherwise, UE reports NACK value(s) for HARQ-ACK information bit(s). - For codebook for multicast, UE reports ACK value(s) for HARQ-ACK information bit(s) only if UE correctly decodes a multicast PDSCH with the desired priority in this SLIV candidate; otherwise, UE reports NACK value(s) for HARQ-ACK information bit(s). |
| Qualcomm | Similar view as Nokia |
| Ericsson | OK, considering the previous agreement for type 2. |
| NTT DOCOMO | Similar view as Nokia |
| CMCC | Similar view as Nokia, we only support the main bullet. |
| Lenovo, Motorola Mobility | Similar view as Nokia. Considering Rel-17 URLLC is also considering supporting HARQ-ACK feedback for different priorities in same codebook, I wonder whether such impacts MBS discussion. |
| Spreadtrum | Share the same view with NOKIA. But fine with one agreement or conclusion to capture this. |
| MediaTek | We prefer to generate the Type-1 codebook with the same priority. For the different priority, why do we consider the Type-1 CB? If it means the different PUCCH resource for multicast and unicast with different priority, we can live with it. If it means the shared PUCCH resource for multicast and unicast with different priority, dropping rule can be used. |
| OPPO | Similar view as Nokia |
| Apple | We share the similar view as Nokia. If a new agreement is needed, CATT’s update is fine. |
| vivo | Similar view as Nokia |

#### Proposal 2.1.1-2

- For UE supporting both unicast and multicast, the pdsch-HARQ-ACK-CodebookList is separately configured for multicast from that for unicast.

Collect views:

| Company | Comments |
| --- | --- |
| CATT | Agree. - The separate pdsch-HARQ-ACK-CodebookList can be configured to indicate the number of codebooks (e.g. one or two) and the type of codebooks (e.g. Type-1 orType-2 codebooks) for multicast services. |
| Nokia, NSB | Support. |
| Samsung | Support. Codebooks for unicast and multicast should not be mandated to be same (e.g. Type-1 may be used for unicast with CA, Type-2 is sufficient for multicast and Rel-17 single-cell operation). |
| ZTE | Ok with the above proposal. |
| Qualcomm | We prefer to say “…pdsch-HARQ-ACK-CodebookList can be separately configured for multicast from that for unicast.” |
| Ericsson | Agree with Qualcomm rewording. |
| NTT DOCOMO | Support. If separate codebooks are constructed for unicast and multicast, the codebooks with the same priority index should be multiplexed and sent on a single PUCCH. |
| CMCC | Support. |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | Support. Seems “can be” is better. |
| MediaTek | Support. |
| OPPO | Support the proposal |
| Apple | Ok with this proposal. |
| vivo | - Support the proposal in general. We think it seems better if it can be modified as“pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList is separately configured for multicast from that for unicast. ” |

### Round-2 (closed)

#### Proposal 2.1.2-1 (H)

- For UE supporting both unicast and multicast, the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList can be separately configured for multicast from that for unicast.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | This issue is prioritized because it affects RRC parameters which will be soon provided to RAN2. The proposal is updated per comments from CATT, QC, Ericsson, Lenovo, … |
| Nokia, NSB | Support |
| Samsung | Support |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | OK |
| vivo | Ok |
| Chengdu TD Tech, TD Tech | Ok |
| Qualcomm | Fine |
| ZTE | OK |
| Spreadtrum | OK |
| CMCC | OK |
| OPPO | OK |
| CATT | OK with this proposal. |
| NTT DOCOMO | Support |
| MediaTek | OK. |
| - Ericsson | - Not very clear with the motivation to configure codebook type separately for multicast from that for unicast if priority is the same. As this mean multicast might be configured with semi-static codebook, unicast might be configured with dynamic codebook. Considering that for the same priority, we also agree to send unicast/multicast harq feedback in a joint codebook, then this will bring a bit complicated issue, i.e. multiplexing semistatic and dynamic into one codebook. - Assuming a pdsch-HARQ-ACK-Codebook is configured for unicast and pdsch-HARQ-ACK-Codebook-MBS is configured for multicast, there is a possibility to configure two different codebook types between unicast and multicast. But this IE does not support priority handling, therefore, we wonder how the UE will choose the codebook type to multiplex the multicast and unicast HARQ feedback. - Similarly, configuring separate pdsch-HARQ-ACK-CodebookList means that there could be potentially two different HARQ feedback type for the same indicated priority in DCI between multicast and unicast. |
| - Apple | - OK |

#### Proposal 2.1.2-2 (H)

- UE is configured to generate separate Type-1 codebooks for unicast and multicast with different priorities, respectively.

- separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast, respectively.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | The proposal is updated but with sub-bullet kept. Let me explain a bit more about the logic. Last meeting we agreed signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast with different priorities. The intention is to extend the agreement to cover the case of Type-1 codebook. By signaling for URLLC, we basically mean a pdsch-HARQ-ACK-CodebookList can configure two codebooks with priority index 0 and index 1 and accordingly UE is provided first and second for each of PUCCH-Config by PUCCH-ConfigurationList for use with the first and second HARQ-ACK codebooks, respectively. (see 38.213 section 9.1), and the different PUCCH Resource IDs are configured in different PUCCH-Config within the pucch-ConfigurationList if configured. (see 38.331 field description for pucch-ConfigurationList). Please correct me if I understand the spec not correctly. This proposal is exactly following the light of agreement made for Type-2 codebook in the last meeting. |
| Nokia, NSB | We believe that the understanding of the FL regarding URLLC signaling is correct. However, the sub-bullet is still misleading. Assume that the UE does not get configured with a separate PUCCH-config for multicast. In that case, the UE would utilize the unicast PUCCH-config, based on latest agreements, also for multicast feedback. Thus, a separate configuration of pdsch-HARQ-ACK-CodebookList for multicast is also needed even in that case (even when PUCCH-config of unicast and multicast are the same). Since the UE supports two non-overlapping slot-based PUCCHs for ACK / NACK feedback for multicast and unicast with different priorities in a slot subject to UE capability, separate Type-1 codebooks for unicast and multicast with different priorities are possible even when PUCCH-config are the same. Thus, the statement “separate PUCCH-config” in the sub-bullet is not a requirement. On the other hand, if the scheduled PUCCH resources are in different slots for unicast and multicast PUCCH, anyway, separate codebooks would be produced. |
| Samsung | OK in principle with the proposal but the wording can be improved. The UE is not “configured to generate separate Type-1 CBs for different priorities”, that is default operation, it is not based on configuration (at least in Rel-17) as there is no configuration for a joint codebook in that case. Maybe something like “When a UE is configured Type-1 codebooks for unicast and multicast with different priorities, respectively, the UE separately generates each of the Type-1 codebooks”. OK with the sub-bullet as different priorities cannot be linked to a same PUCCH-config. |
| Lenovo, Motorola Mobility | Generally is fine with us. The logic is not clear to us. Seemingly, the sub-bullet is the prerequisite of the main bullet, our understanding is listed below: - When separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast, UE is configured to generate separate Type-1 codebooks for unicast and multicast with different priorities. |
| vivo | Share the view as Samsung. The rewording from Samsung is fine to us. |
| Chengdu TD Tech, TD Tech | ok |
| ZTE | Some clarification is needed from our perspective. Based on the current spec, UE doesn’t check the priority for PDSCH when constructing type-1 codebook based on the current spec. Some bullets may be needed to clarify the intended UE behavior. For example, unicast can be indicated as high or low priority by DCI, similarly multicast can also be indicated as high or low priority by DCI. In this case, UE needs to check whether the scheduled PDSCH corresponds to low or high priority, whether the scheduled PDSCH corresponds to unicast or multicast. This part needs to be clarified. |
| CMCC | We support the principle of this proposal. Regarding the interpretation of “separate PUCCH-config”, the first is a separate PUCCH-config/PUCCH-ConfigurationList is configured multicast which was agreed in previous meetings. The second is there is no separate PUCCH-config/PUCCH-ConfigurationList configured for multicast, i.e., sharing the PUCCH-config/PUCCH-ConfigurationList with unicast, but unicast and multicast have different priorities which corresponding to different PUCCH-config, which is similar to Rel-16 two PUCCH-config for different priorities. We think the intention of this proposal includes these two interpretations and modify it as the following to address the concern from Nokia - Different separate PUCCH-config/PUCCH-ConfigurationList is configured for unicast and multicast, respectively. |
| Lenovo 2 | Further clarification on the main bullet from our side: Does it imply two Type-1 codebooks are generated with one for unicast and another for multicast OR maximum four Type-1 codebooks may be generated (e.g. a first codebook for LP unicast, a second codebook for HP unicast, a third codebook for LP multicast, a fourth codebook for HP multicast)? - On the other hand, for Type-1 codebook determination, based on K1 set, UE knows HARQ-ACK feedback for a multicast PDSCH in one slot should be generated. However, UE can’t know whether it is LP or HP PDSCH. Then how to generate the Type-1 codebook for multicast should be further studied. |
| OPPO | We also prefer the wording suggested by Samsung. |
| CATT | We thank FL’s explanation and believe the FL’s understanding is right. The sub-bullet is still misleading us. We believe that with different priorities, the configuration of separate codebooks does not depend on the configuration of a separate PUCCH-config. Thus, we think the sub-bullet is unnecessary or needs few modifications. |
| NTT DOCOMO | The wording from Samsung is fine to us. |
| MediaTek | We share the similar with Samsung/vivo. |
| - Ericsson | - There is no one to one mapping between separate codebook and different priorities. Even same priority, UE can also generate two codebooks as long as the PUCCH resource for unicast and multicast are not overlapped in time and UE has such capability. - In our understanding, only the two PUCCH configs in PUCCH ConfigurationList are of different priority. A separate PUCCH-Config for MBS does not mean different priorities, therefore it is a bit strange to include separate PUCCH-Config in the sub-bullet. |
| - Apple | - We are ok with the wording from Samsung in main bullets. The sub-bullet is better to separated into two sub-bullet with / without PUCCH-config/PUCCH-ConfigurationList is configured for unicast. |

### Round-3 (new)

- When we were discussing proposals in section 2.1.2 regarding pdsch-HARQ-ACK-Codebook /pdsch-HARQ-ACK-CodebookList for multicast, there were questions/comments whether codebook/codebooklist is per G-RNTI or applied to multiple G-RNTI.

- Refer to 38.331 regarding codebook/codebooklist configuration as follows:

PhysicalCellGroupConfig ::=         SEQUENCE {

harq-ACK-SpatialBundlingPUCCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

harq-ACK-SpatialBundlingPUSCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

p-NR-FR1                            P-Max                                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-Codebook             ENUMERATED {semiStatic, dynamic},

- …

pdsch-HARQ-ACK-CodebookList-r16        SetupRelease {PDSCH-HARQ-ACK-CodebookList-r16}               OPTIONAL,   -- Need M

- …

PDSCH-HARQ-ACK-CodebookList-r16 ::=     SEQUENCE (SIZE (1..2)) OF ENUMERATED {semiStatic, dynamic}

pdsch-HARQ-ACK-Codebook The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to both CA and none CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3). If pdsch-HARQ-ACK-Codebook-r16 is signalled, UE shall ignore the pdsch-HARQ-ACK-Codebook (without suffix). For the HARQ-ACK for sidelink, if pdsch-HARQ-ACK-Codebook-r16 is signalled, the UE uses pdsch-HARQ-ACK-Codebook (without suffix) and ignores pdsch-HARQ-ACK-Codebook-r16. If the field pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup is present, pdsch-HARQ-ACK-Codebook is applied to primary PUCCH group. Otherwise, this field is applied to the cell group (i.e. for all the cells within the cell group). For the HARQ-ACK for sidelink, if the field pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup is present, pdsch-HARQ-ACK-Codebook is applied to primary and secondary PUCCH group and the UE ignores pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup.

pdsch-HARQ-ACK-CodebookList A list of configuration for at least two simultaneously constructed HARQ-ACK codebooks. Each configuration in the list is defined in the same way as pdsch-HARQ-ACK-Codebook (see TS 38.212 [17], clause 7.3.1.2.2 and TS 38.213 [13], clauses 7.2.1, 9.1.2, 9.1.3 and 9.2.1). If this field is present, the field pdsch-HARQ-ACK-Codebook is ignored for the case at least two HARQ-ACK codebooks are simultaneously constructed. If this field is present, the value of this field is applied for primary PUCCH group and for secondary PUCCH group (if configured). For the HARQ-ACK for sidelink, the UE uses pdsch-HARQ-ACK-Codebook and ignores pdsch-HARQ-ACK-CodebookList if this field is present.

- Refer to 38.213

9.1	 HARQ-ACK codebook determination

If a UE is provided pdsch-HARQ-ACK-CodebookList, the UE can be indicated by pdsch-HARQ-ACK-CodebookList to generate one or two HARQ-ACK codebooks. If the UE is indicated to generate one HARQ-ACK codebook, the HARQ-ACK codebook is associated with a PUCCH of priority index 0. If a UE is provided pdsch-HARQ-ACK-CodebookList, the UE multiplexes in a same HARQ-ACK codebook only HARQ-ACK information associated with a same priority index. If the UE is indicated to generate two HARQ-ACK codebooks

-	a first HARQ-ACK codebook is associated with a PUCCH of priority index 0 and a second HARQ-ACK codebook is associated with a PUCCH of priority index 1

-	the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by {PUCCH-ConfigurationList, UCI-OnPUSCH-ListDCI-0-1, PDSCH-CodeBlockGroupTransmissionList} or {PUCCH-ConfigurationList, UCI-OnPUSCH-ListDCI-0-2, PDSCH-CodeBlockGroupTransmissionList}, respectively, for use with the first and second HARQ-ACK codebooks, respectively

- Based on 38.331 and 38.213, the codebook type is configured per cell group and is mandatorily configured for MCG for unicast. Two codebooks with different priories are configured via the pdsch-HARQ-ACK-CodebookList.

- We have agreed the following:

Agreement:

For UE supporting both unicast and multicast, the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList can be separately configured for multicast from that for unicast.

- Whether it is applied to per G-RNTI or applied to all G-RNTIs configured to UE was not discussed ever but it also affects RRC parameters. The down-selection is expected to be in this meeting if possible, but let’s see companies’ views first.

#### Proposal 2.1.3-1 (H)

- The pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList configured for multicast is to down-select from:

- Alt 1: per G-RNTI

- Alt2: applied to all G-RNTIs configured to UE

- Note: the down-selection if possible is expected to be in this meeting.

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Considering the time limitation of the WI, we support Alt. 2. Type-1 codebook has more redundancy and Type-2 codebook is more prone to errors when DCI is missed. Therefore, MBSs that have high reliability requirements could prefer Type-1 and others could prefer Type-2. Thus, a list per G-RNTI would help. On the other hand, if Type-1 codebook is configured for some MBSs that the UE receives and Type-2 for others, some further specification impact is expected. For example (assuming same priority of different G-RNTIs), will the UE also send a Type-1 codebook on top of the Type-2 codebook (multiplexed), in case Type-2 codebook is scheduled to be transmitted? Based on current specification, the UE would transmit Type-1 codebook in all PUCCH occasions possible. Another spec impact would be on multiplexing different types codebooks (e.g., one G-RNTI is configured with Type-1, the other with Type-2, and corresponding PUCCHs should be multiplexed), however, as we mentioned in our contribution, this is anyway needed when unicast and multicast are configured with different types of codebooks and are to be multiplexed. |
| vivo | We prefer Alt2. Type 1 codebook is more reliable on the codebook size. Type 2 codebook has less redundancy. If there is no multiplexing among codebooks for different g-RNTIs with different codebook types, it will be helpful to configure codebook type per G-RNTI. However, when HARQ-ACK for multiple G-RNTI of the same priority are scheduled in the same slot, they will be multiplexed on one codebook, the reliability benefit of type 1 codebook will be lost. In addition, for alt 1, we need to discuss how to generate codebook of different G-RNTI with different codebook types and it can be expected that it will be very complex to generate a such codebook. For simplicity, Alt 2 is preferred. |
| Samsung | Alt.2 is sufficient because Rel-17 multicast operation is for single-cell. Type-1 was basically designed to support some particular CA cases – no practical need to have Type-1 for single-cell multicast (the specs will of course support it, but also no real or perceived optimizations are needed to Type-1 due to multicast), or to have different CBs for different G-RNTIs. We don’t see any meaningful complexity issue with configuring the codebook type per G-RNTI, will not be against it, but it is not necessary in Rel-17. |
| CMCC | Prefer Alt 2. Alt 1 needs more spec impact on how to multiplexing different type codebooks with the same priority. |
| Lenovo, Motorola Mobility | Alt 2 is supported. Since there are only two priority levels, i.e., low priority and high priority, and we have agreed that HARQ-ACK feedback for same priorities can be multiplexed in one HARQ-ACK codebook, it seems not necessary to configure HARQ-ACK codebook list per G-RNTI. For different multicast services, if they are of same priority, then their corresponding HARQ-ACK feedback can be indicated in same HARQ-ACK codebook. |
| LG | We also prefer Alt.2 |
| ZTE | We acknowledge that Alt.1 has more flexibility than Alt.2. However, considering the limited remaining time of Rel-17 and complexity of Alt.1, we would suggest to go with Alt.2. |
| Qualcomm | Technically, if the CB of unicast and multicast services can be configured separately, different multicast services can be treated independently as well. The UE only multiplex the feedback with same priority and same CB. Alt2 may be used to control the CB size by limiting the number of G-RNTIs sharing same CB. Alt2 is more flexible, while Alt1 is simpler with less signaling overhead. We will not object common configuration for G-RNTIs, as long as the agreement is clear, and companies have common understanding on the impact. |
| CATT | Prefer Alt 2. Similar views with CMCC, if Alt 1 is supported, how to multiplexing different type codebooks should be considered, which may lead more spec works than Alt 2. |
| NTT DOCOMO | We prefer Alt2. Similar view with vivo. |
| OPPO | Support Alt 2. We did not see strong technical reason for Alt 1, rather it could increase the complexity significantly. |
| MediaTek | We prefer Alt 2. Share the similar view with OPPO. |

## Non-overlapping/sub-slot PUCCH

Submitted Proposals

“sub-slot based PUCCH”

(vivo) Proposal 11: Support sub-slot based PUCCH transmission for multicast HARQ-ACK.

(Nokia) proposal 2:

  - Rel-16 sub-slot based PUCCH configurations and mechanisms are supported also for at least UE-specific ACK / NACK based feedback for MBS.

(Spreadtrum) proposal 4:

  - Sub-slot based PUCCH transmission for HARQ-ACK is supported.

(ZTE) Proposal 6:

  - Regarding the case of non-overlapping PUCCHs resources for HARQ-ACK in the same UL slot:

- HARQ-ACK PUCCH overriding mechanism is reused for multicast with the same priority at least for the same MBS service..

- if HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast are determined to be in the same UL slot, then

•	The overriding/multiplexing should be used for the two HARQ-ACK PUCCHs for the same priority (regardless of whether they overlap in the time domain).

•	The final PUCCH resource is determined based on the PRI in the last DCI corresponding to the unicast HARQ-ACK PUCCH.

- sub-slot based PUCCH transmission for HARQ-ACK is supported.

(CMCC) proposal 1:

  - Support sub-slot based PUCCH transmission for multicast HARQ-ACK.

“Non-overlapping slot-based PUCCH”

(vivo) Proposal 9:

  - For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast,

    - For the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot, separate transmission of HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast is supported.

(Nokia) proposal 3:

  - As in Rel-16 unicast operation, the UE receiving MBS services, based on UE capability, support multi-TRP operation configured with ackNackFeedbackMode-r16 = separate, i.e., the UE can transmit two non-overlapping PUCCHs including HARQ-ACK feedback per (sub-)slot, i.e., one for each TRP.

(CMCC) proposal 2:

  - If separate PUCCH(s) is configured for HARQ-ACK for multicast, the PUCCH structure for PUCCH-Config of multicast and unicast HARQ-ACK with the same priority index should be the same. The PUCCH structure for PUCCH-Config for multicast and unicast HARQ-ACK with different priority index can be different.

(DOCOMO) Proposal 8:

  - Support at most one PUCCH with HARQ-ACK in a slot/sub-slot per priority index per TRP.

### Round-1 (pending)

“sub-slot based PUCCH”

FL’s comment:

Regarding whether UE supports sub-slot based PUCCH and non-overlapping PUCCHs for multicast was discussed in the last meeting, which ends up the agreement as follows:

Agreement:

NR supports at least the following cases for UE supporting multicast:

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast with different priorities in a slot subject to UE capability.

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast with different priorities, respectively, in a slot subject to UE capability.

Supporting sub-slot based PUCCH was controversial but there are still five companies proposing to support it in this meeting. We can have a discussion in this meeting whether it is agreeable.

Regarding whether UE transmits two non-overlapping PUCCHs for the same priority or in the case of m-TRP configured with ackNackFeedbackMode-r16 = separate, too few input for this meeting, so no FL proposal is suggested.

FL’s Proposal:

#### Proposal 2.2.1-1 (H)

Support sub-slot based PUCCH for ACK/NACK based HARQ-ACK feedback for multicast subject to UE capability.

Collect views:

| Company | Comments |
| --- | --- |
| - CATT | - Agree. |
| - Nokia, NSB | - Support. |
| - Samsung | Not support. - There is no need and the specification impact is substantial. |
| - ZTE | Support the above proposal. Sub-slot based ACK/NACK mechanism is already in the spec for unicast. From our perspective, supporting sub-slot based PUCCH for multicast won’t cause much spec impact. |
| - Qualcomm | Ok in principle |
| - Ericsson | Ok |
| - NTT DOCOMO | Support |
| - CMCC | Support |
| - MediaTek | Generally OK. |
| - OPPO | OK |
| - vivo | support |
| - FL’s reply | Regarding the concern from Samsung about spec impact. I’d like to point out in Huawei’s paper R1-2106439 one issue is discussed when UE does not support sub-slot PUCCH for multicast but support it for unicast. The issue is how UE constructs the HARQ codebook for unicast and multicast with the same priority: if only support multiplexing the HARQ for the same priority case, we may need to discuss the processing timeline whether UE has sufficient time to multiplex it when UE is configured with sub-slot PUCCH (e.g., PUCCH3) for unicast but slot-based PUCCH (e.g., PUCCH5) for multicast. I am assuming the discussion for such issue is also time consuming. To solve this issue, UE is either enabled to generate separate codebooks for the same priority case or UE supports sub-slot PUCCH for multicast. I’d like to hear views from the group including Aris as editor which approach needs less spec efforts. [Samsung2]: There are several aspects for the support of “sub-slot” based PUCCH transmissions for multicast, ranging from specification impact, to existence of actual use-cases, to realistic deployments. For the issue mentioned by Jinhuan above, unless all UEs in a multicast group have/support same “sub-slot” configuration, there will be no difference between “slot based” and “sub-slot based” PUCCH (which, unlike “slot based” that all Ues support, will be undefined if some Ues do not support and for the given sub-slot configuration). Regarding the multiplexing timeline in the above figure, it is not clear why the Rel-15 procedure cannot apply, and the pseudo-code in 9.2.5 cannot be executed, by considering all PUCCHs to be in the same set Q. Of course, regardless of “slot” or “sublsot”, Rel-17 multicast introduces the case where overlapping needs to be resolved for 2 PUCCHs with HARQ-ACK (unicast, multicast) in a slot – that is trivial to add/specify and does not affect the procedure in 9.2.5. Also, if for whatever reason a Gnb wants to have “sub-slot” based PUCCH, a “sub-slot” effect can be achieved by Gnb implementation for example by configuring some PUCCH resources in the first 7 symbols and remaining ones in the second 7 symbols of a slot – 1 bit from the PRI may be effectively lost but that is probably OK (subject to having a capability for a UE to transmit multiple PUCCHs with HARQ-ACK in a slot, as would also be the case for “sub-slot”). For non-spec/implementation related aspects: - It is not reasonable to expect Rel-17 UEs for multicast to support sub-slot based PUCCH transmissions. That is also beyond the Rel-17 WID. If a need, a proper “multicast for URLLC applications” WI should be endorsed. - Latency is the likely reason why “sub-slot” PUCCH is suggested (it is understood there are some proposals for other purposes relying on “sub-slot” but that cannot be a reason). However, there are several other aspects then need to be defined for low latency. Coverage and data rates will also shrink. For spec/implementation related aspects, just based on an initial/“top-of-the-head” list below, they will not be simple: - “Sub-slot”-based Type-1 codebook construction (assuming a joined construction for unicast/multicast in case of sub-slot PUCCH) - If span-based CG-PDCCH needs to be supported (otherwise, sub-slot for the PUCCH makes little sense) - Inter-MBS service prioritizations for same/different priorities (unless same “sub-slot” configurations apply to all G-RNTIs). |
| - Nokia, NSB | We support the initial proposal. Based on the current specifications and previous agreements, the UE cannot transmit more than one HARQ-ACK including PUCCH in a slot with the same priority (and this holds per sub-slot in case of sub-slot configurations). Therefore, generating separate codebooks for the same priority (without multiplexing) is not feasible. |
| - Lenovo, Motorola Mobility | One question for clarification: Can NACK-only based HARQ-ACK feedback be supported in sub-slot PUCCH? |
| - Chengdu TD Tech, TD Tech | ok |

## Type-1 HARQ codebook

Submitted Proposals

“FDM-ed for more than one G-RNTI”

(Nokia) Proposal 24:

  - If FDM-ed transmissions of multiple multicast / broadcast services are allowed, the UE constructs separate Type-1 HARQ-ACK sub-codebooks using Rel-15 / 16 mechanisms for each multicast / broadcast service..

(Nokia) Proposal 25:

  - The UE concatenates the constructed Type-1 sub-codebooks and sends them in the same PUCCH resource in case their HARQ-ACK feedback is scheduled for the same time instance.

(Nokia) Proposal 26:

  - The PHY identification of PDSCH HARQ-ACK to MBS Type-1 sub-codebook mapping is the group-common RNTI value.

(Nokia) Proposal 27:

  - The order of concatenation of the MBS sub-codebooks to construct a HARQ-ACK codebook, when the HARQ-ACK feedback of different services are scheduled for the same time instance, follows the increasing order of the G-RNTI values that are used to map PDSCH HARQ-ACK to MBS sub-codebook. MBS sub-codebooks are preceded by unicast sub-codebook, as agreed before.

(ZTE) Proposal 4:

  - Type-1 HARQ-ACK codebook construction for overlapped multicast and multicast or overlapped unicast and multicast, for a PDSCH SLIV group, the number of HARQ-ACK bits generated by the UE takes the UE capability of PDSCH reception into account.

(CATT) Proposal 13:

  - when the number of services configured including unicast and multicast is greater than number of FDM-ed PDSCHs reported by UE, the enhancement on Type-1 HARQ-ACK codebook generation shall be studied.

(Lenovo) Proposal 9:

  - For HARQ-ACK feedback for unicast and multicast to be multiplexed in the same PUCCH, two separate Type-1 HARQ-ACK codebooks are generated for unicast and multicast, respectively, according to the respective K1 set and TDRA table.

(Lenovo) Proposal 10:

  - For HARQ-ACK feedback for unicast and multicast to be multiplexed in the same PUCCH, Type-1 HARQ-ACK codebook for unicast is placed firstly then followed by Type-1 HARQ-ACK codebook for multicast.

(Qualcomm) Proposal 5:

  - For multiplexing of ACK/NACK feedback for unicast + multicast, or multicast + multicast

    - For Type-1 codebook,

      - For a UE capable of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4); otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed

    - For Type-2 codebook,

      - DAI per multicast G-RNTI is separately counted

      - The concatenation of Type-2 HARQ-ACK codebook for multicast and multicast is based on G-RNTI order

“TDM-ed cases”

“intersection k1”

(vivo) Proposal 5: For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on Alt 1.

  - for slot timing values K_1 in the intersection of K_1 set for unicast (termed set A) and K_1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K_1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K_1 in set B but not in set A, based on PDSCH TDRA set for multicast.

(Nokia) Proposal 29:

  - Alt. 1 is selected for determining the PDSCH reception candidate occasions.

(CATT) Proposal 14:

  - Alt 1 can be used as enhancement scheme on TDM-ed Type-1 HARQ-ACK codebook construction, and Alt 2 can be used as baseline scheme.

(Lenovo) Proposal 11:

  - For Type-1 HARQ-ACK feedback for unicast and multicast to be multiplexed in the same PUCCH, determine candidate PDSCH reception occasions based on below steps:

    - if a slot is included in intersection of downlink association set (DAS) for unicast and DAS for multicast, the candidate PDSCH reception occasions in the slot are determined according to the union of the configured TDRA table for unicast and the configured TDRA table for multicast;

    - if the slot is only included in DAS for unicast, the candidate PDSCH reception occasions in the slot are determined only according to the configured TDRA table for unicast;

    - if the slot is only included in DAS for multicast, the candidate PDSCH reception occasions in the slot are determined only according to the configured TDRA table for multicast

(Qualcomm) Proposal 4:

  - For HARQ-ACK codebook construction of ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, prefer Alt1 to determine PDSCH reception candidate occasions.

    - Alt 1 assuming K1 set for unicast termed as set A and K1 set for multicast termed as set B

      - for slot timing values K1 in the intersection of A and B, based on union of the PDSCH TDRA sets,

      - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

      - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

(CMCC) Proposal 8:

  - For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is:

    - for slot timing values K_1 in the intersection of K_1 set for unicast (termed set A) and K_1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

    - for slot timing values K_1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

    - for slot timing values K_1 in set B but not in set A, based on PDSCH TDRA set for multicast.

(Apple) Proposal 5:

  - For Type-1 HARQ-ACK codebook construction for TMD-ed unicast and multicast multiplexed in the same slot, K1 set is in the intersection K1 set of unicast and multicast is based on union of the PDSCH TDRA sets, otherwise PDSCH TDRA set of unicast K1 set or multicast K1 set is applied.

(Ericsson) Proposal 10:

  - Alt 1 is selected as the method to construct type-1 HARQ-ACK codebook for TDMed unicast and multicast.

“Union k1”

(Spreadtrum) proposal 3:

  - For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, PDSCH reception candidate occasions are based on the union of the PDSCH TDRA sets for slot timing values K_1 in the union of K_1 set for unicast and K_1 set for multicast.

(ZTE) Proposal 3:

  - For Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is for slot timing values K_1 in the union of K_1 set for unicast and K_1 set for multicast, based on the union of the PDSCH TDRA sets.

(CATT) Proposal 14:

  - Alt 1 can be used as enhancement scheme on TDM-ed Type-1 HARQ-ACK codebook construction, and Alt 2 can be used as baseline scheme.

(NEC) Proposal 2:

  - For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on Alt2, i.e.,

    - Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

(OPPO) Proposal 5:

  - For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, PDSCH reception candidate occasions are based on Alt 2.

(Intel) Proposal 11:

  - For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions, Alt 2 is supported.

“TDM-ed or FDM-ed” for Type-1

(Huawei) Proposal 5:

  - UE generates the Type-1 codebook via “FDM-ed” manner only when UE is configured to do so. Otherwise, UE will not expect unicast and multicast to be scheduled in FDM.

(Nokia) Proposal 23:

  - RAN1 discusses whether FDM-ed / TDM-ed multicast transmissions of different multicast / broadcast services are allowed.

(Nokia) Proposal 28:

  - If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4).

    - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

(NEC) Proposal 1:

  - If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner. Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed manner.

(Qualcomm) Proposal 5:

  - For multiplexing of ACK/NACK feedback for unicast + multicast, or multicast + multicast

    - For Type-1 codebook,

      - For a UE capable of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4); otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed

    - For Type-2 codebook,

      - DAI per multicast G-RNTI is separately counted

      - The concatenation of Type-2 HARQ-ACK codebook for multicast and multicast is based on G-RNTI order

(CMCC) Proposal 9:

  - If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner. Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

(DOCOMO) Proposal 6:

  - For Type-1 HARQ-ACK codebook construction, whether to use TDM manner or FDM manner is indicated by an RRC parameter.

(Ericsson) Proposal 9:

  - FDMed unicast and multicast capable UE can be indicated via RRC signaling whether type-1 codebook should be generated as FDMed manner or not.

### Round-1 (2&3 pending)

FL’s Comments

How Type-1 codebook for FDM-ed unicast and multicast is agreed in the last meeting as follows:

Agreement:

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast with the same priority from the same TRP, support

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for Mtrp).

- FFS: If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4).

  - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

Whether UE generates Type-1 HARQ codebook based on FDM-ed manner (i.e., Opt4) or TDM-ed manner (i.e., union TDRA tables) was discussed in the end of last meeting but without progress due to limited time. Based on the submitted proposals in this meeting, it seems the common understanding that UE generates it based on FDM-ed manner only when network configures UE to do so and otherwise UE will not expect network to schedule unicast and multicast in FDMed.

Regarding FDM-ed Type-1 codebook construction for more than one G-RNTI, extending the above agreement to cover this case seems straightforward.

For TDM-ed Type-1 codebook, this is the third meeting to discuss whether the construction is based on the intersection of K1 sets or based on the union of K1 sets, the views are still controversial based on the submitted proposals:

- Supportive to Alt 1 (intersection): [8, vivo, Nokia, CATT, Lenovo, Qualcomm, CMCC, Apple, Ericsson]

- Supportive to Alt 2 (union): [6, Spreadtrum, ZTE, CATT, NEC, OPPO, Intel]

If we have to make a decision which I hope to do, I would suggest the slight majority view, i.e., Alt 1.

#### Proposal 2.3.1-1

If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4). Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

Collect views:

| Company | Comments |
| --- | --- |
| - CATT | - Agree. |
| - Nokia, NSB | - Support. |
| - Samsung | - The proposal seems unnecessary (although OK in principle) as the intention is to prevent a Gnb misconfiguration. |
| - ZTE | - OK with the above proposal. |
| - Qualcomm | - ok |
| - Ericsson | - Support |
| - NTT DOCOMO | - Support |
| - CMCC | - Support |
| LG | We think that it would be beneficial to discuss expected UE behavior in the latter case (i.e. when unicast and multicast are scheduled in FDM-ed even after report of the capability of supporting the FDM-ed unicast and multicast in the same slot), considering that whether to schedule them in FDM seems up to Gnb implementation in any case. In details, we want to clarify whether to drop all FDMed PDSCHs in the latter case. For example, UE could simply receive unicast only depending on the reported capability, instead of dropping all PDSCHs. Besides, we want to clarify how to generate HARQ-ACK codebook in the latter case. Accordingly, we propose to change to: #### Proposal 2.3.1-1 If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4). Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed. - FFS: how the UE not supporting the capability receives PDSCH(s) in the slot. - FFS; how the UE not supporting the capability generates Type-1 HARQ-ACK codebook for the FDM-ed unicast and multicast in the same slot. |
| Lenovo, Motorola Mobility | OK. |
| MediaTek | Agree. |
| OPPO | Agree with the proposal |
| Apple | Ok with this proposal. |
| Vivo | Agree in principle. One question for the last sentence. If unicast PDSCH and multicast PDSCH are scheduled in different time and different frequency, can it be called they are scheduled in FDM-ed? We think when UE is indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner, unicast PDSCH and multicast PDSCH can be scheduled in different time and different frequency. |

#### Proposal 2.3.1-2

For UE supports more than one G-RNTI scheduled in FDM-ed, Type-1 codebook is constructed by concatenating Type-1 sub-codebook of each G-RNTI following the ascending order of the G-RNTI value.

Collect views:

| Company | Comments |
| --- | --- |
| - CATT | We believe that this proposal is trying to explain how the Type-1 sub-codebook is constructed among multicast. If unicast is supported, the ‘Type-1 codebook’ for multicast becomes the ‘Type-1 sub-codebook’. Thus, we suggest the proposal can be updated as following: #### Proposal 2.3.1-2 - For UE supports more than one G-RNTI scheduled in FDM-ed, Type-1 (sub-)codebook for multicast is constructed by concatenating Type-1 sub-codebook of each G-RNTI following the ascending order of the G-RNTI value. |
| - Nokia, NSB | Support with some editing: For a UE supporting FDM-ed transmissions scrambled by more than one G-RNTI, when the corresponding HARQ-ACK feedback are to be multiplexed, the UE constructs separate Type-1 HARQ-ACK sub-codebooks for each G-RNTI and concatenates them in an ascending order of the G-RNTI value. |
| - Samsung | Not support. FDMed multicast PDSCHs has not been agreed and is out of scope. |
| - ZTE | Seems we need to first discuss whether to support FDM-ed multicast and multicast. |
| - Qualcomm | Even if max one multicast and one unicast are FDMed in a slot, we could have the case G-RNTI1+C-RNTI in slot1 and G-RNTI2+C-RNTI in slot2. For such case, we need to decide the rules of FDM pattern to concatenate the sub-codebooks for different G-RNTIs. Therefore, we prefer to revise the proposal as: #### Proposal 2.3.1-2 For a UE using FDM-ed manner for Type-1 codebook of more than one G-RNTI with same priority from the same TRP, it is constructed by concatenating Type-1 sub-codebook of each G-RNTI following the ascending order of the G-RNTI value |
| - Ericsson | - Support |
| NTT DOCOMO | It is questionable whether it is necessary to consider FDM between multiple multicast PDSCHs. Whether to support it is discussed in AI 8.12.1, but few companies support it. |
| CMCC | Can be deprioritized after the progress of AI 8.12.1. |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | Whether to support FDM based multicast and multicast should be discussed firstly. |
| Spreadtrum | That multiple group-common PDSCHs are FDMed transmission has not been supported. Thus, we suggest to delay this issue. |
| MediaTek | Not support. We share the similar view with Samsung/NTT DOCOMO. Maybe we can postpone the discussion until whether to supporting FDM-ed multicast have clear conclusion in AI 8.12.1. |
| OPPO | Whether to support FDM between multicast and multicast should be decided firstly. |
| Apple | Whether FDMed multicast and multicast is supported is still open in AI8.12.1. |
| vivo | support |
| Chengdu TD Tech, TD Tech | ok |

#### Proposal 2.3.1-3

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on Alt 1 from the following alternatives:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

Collect views:

| Company | Comments |
| --- | --- |
| CATT | OK. |
| Nokia, NSB | Support. |
| Samsung | Not support. There are no overhead savings as there is no reason for a NW to configure different K1 sets for unicast and multicast and, even if that was the case, same K1 values will practically always remain after removing K1 values based on the TDD UL-DL configuration. Also, in RAN1#104-e, it was agreed that Type-1 CB is based on the union of the TDRA sets with “FFS whether/how to optimize the Type-1 CB”. Changing a HARQ-ACK CB construction is a major change and requires clear benefits that do not exist in Alt.1. |
| ZTE | Considering that the overhead saving for HARQ-ACK codebook using option 1 is determined by relation of K1 set for unicast and multicast, it can be unnecessary in cases where K1 sets for unicast and multicast are overlapped. And the usage of the union of the PDSCH TDRA sets provides a unified solution for Type-1 HARQ-ACK codebook construction for unicast and multicast, which results in almost few modification for specification. Thus, Option 2 is preferred from our perspective. Maybe one way to go is to first agree on Alt.2 and then further discuss Alt.1. |
| Qualcomm | ok |
| - Ericsson | - Support |
| - NTT DOCOMO | - Support |
| - CMCC | - Support |
| LG | We prefer Alt 2. |
| Spreadtrum | Alt 2 is baseline. Alt 1 is one optimization and the optimization seems to be unnecessary as Samsung pointed. |
| MediaTek | Generally OK. |
| OPPO | In our understanding, Alt 2 has already been supported according to the agreement made in RAN1#104-e. |
| Apple | Support |
| vivo | support |
| Chengdu TD Tech, TD Tech | Alt 1 |
| CMCC2 | As the necessity concern from Samsung and Spreadtrum, we think it is possible gNB configure different K1 sets for unicast and multicast. For example considering the MBS services periodicity is predictable and gNB can configure the PDCCH monitoring periodicity of GC-PDCCH as per multiple slots, but for unicast, the PDCCH monitoring periodicity is slot-based. That is the K1 sets of multicast is also a subset of unicast. |

### Round-2 (closed)

#### Proposal 2.3.2-1 (H)

If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4). Otherwise, down-select from the following alternatives:

Alt1: UE does not expect unicast and multicast are to be scheduled in FDM-ed.

Alt2: Defining UE to receive one or up to UE implementation when unicast and multicast are scheduled in FDM-ed

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - The proposal is updated per LG’s comment saying the scheduling is up to gNB and basically gNB should not be restricted to do some scheduling. - As FL, I have to say Alt1 seems simplest and proposed by vast majority. I’d like to hear companies’ views regarding this updated proposal. |
| - Nokia, NSB | - We prefer Alt 1. - Why would the gNB schedule unicast in and FDM-ed manner with multicast, if the UE indicated that it cannot support FDM-ed transmissions? |
| - Samsung | - Alt.2 - Alt.1 would be nice if it was possible, but it is not. A UE can be scheduled unicast and then the gNB may need to schedule multicast to a group of UEs that includes the UE (e.g. unicast is with multi-slot repetitions, but also for single slot). The reverse can also apply. At least for the case of different priorities, Alt.2 is supported in R16 for unicast. The issue is whether the applicability of Alt.2 should extend to same priority for multicast vs. unicast. We think it should because multicast scheduling is not controlled per single UE. |
| - LG | - We support Alt 2. - When UE does not support FDMed unicast and multicast, we expect that gNB would try to avoid FDMed unicast and multicast. However, we wonder if gNB can always avoid it. Whether to schedule them in FDM will be anyway up to gNB implementation. Thus, we want to clarify whether to always drop all FDMed PDSCHs or simply receive unicast only depending on the reported capability, instead of dropping all PDSCHs. Besides, we want to clarify how to generate HARQ-ACK codebook, if this case suddenly happens. |
| - Lenovo, Motorola Mobility | - We think Alt 2 can be split to two options: - Alt 2-1: define UE to receive one; - Alt 2-2: up to UE implementation. - Then we prefer Alt 2-2. |
| - vivo | - We prefer Alt 1. - Similar view as Nokia, gNB should not schedule unicast in FDM-ed manner with multicast if UE does not support FDM-ed transmissions. |
| - Chengdu TD Tech, TD Tech | - We prefer to alt 1. |
| - Qualcomm | - We think Alt1 is sufficient. - Our understanding is that Alt1 means for UE not capable of FDMed unicast and multicast, it will be up to UE implementation when gNB schedule an error case of FDMed unicast and multicast. We don’t support Alt2 to define the UE behaviour of such error case. |
| - ZTE | - After further checking, it seems difficult/impossible for network to fully avoid scheduling FDMed unicast and multicast if UEs supporting FDM and UEs not supporting FDM are in the same multicast group. From this perspective, Alt.2 seems more appropriate. |
| - Spreadtrum | - Support Alt 1. - In our understanding, for UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, if gNB doesn’t indicate to generate Type-1 codebook as FDM manner, it implys gNB would not schedule with FDMed multiplexing pattern. If gNB wants to schedule in FDMed manner, it can be done by RRC reconfiguring codebook indication information. |
| - CMCC | - Prefer Alt 1 - gNB should consider all UEs’ capabilities to make sure the multiplexing pattern between unicast and multicast is suitable for each UE. |
| - OPPO | - Support Alt 1, or modified Alt 2 as below, we do not think there is a need to define UE receiving behaviour when there is FDM unicast and multicast. Alt2: Defining UE to receive one or up to UE implementation when unicast and multicast are scheduled in FDM-ed |
| - CATT | - Prefer Alt1. - Similar view as Nokia/vivo, if the UE does not report the capability of supporting the FDM-ed, it is naturally means the UE cannot support FDM-ed transmissions. Then, gNB should not schedule the FDM-ed transmissions. If gNB can still schedule the FDM-ed transmissions for UE, then what is the meaning for the report? |
| - NTT DOCOMO | - We prefer Alt1. Similar view as CMCC. |
| - MediaTek | - Support Alt 1. - If UE tell the gNB does not support the capability, gNB should consider the UE’s processing capability when scheduling the multicast services. |
| - Ericsson | - Option 1 is simpler. In our view, it is unclear what would the gNB would expect from the UE if feedback is left to implementation. Should the network test multiple codebooks feedback? |
| - Apple | - Alt 1 is straightforward. If UE doesn’t support FDM reception, UE should not be configured the related MBS session resources. |

## Type-2 HARQ codebook

Submitted Proposals

“Separate DAI per G-RNTI”

(vivo) Proposal 7:

  - For ACK/NACK based HARQ-ACK feedback, for type 2 HARQ-ACK codebook for RRC_CONNECTED Ues receiving multicast.

    - If UE is configured with multiple g-RNTIs

      - If PTM transmission scheme 1 is used for group-common PDSCH, separate DAI counting for different g-RNTIs is used.

        - Concatenating more than one Type-2 HARQ-ACK codebook for multicast is supported.

(Nokia) Proposal 30:

  - When Type-2 HARQ-ACK codebook is used, the DAI counter is separate for each multicast / broadcast service, since the UEs in the PTM group may be interested in different services. Separate DAI counters are used to construct separate sub-codebooks.

(ZTE) Proposal 5:

  - Regarding Type-2 HARQ-ACK codebook construction for MBS

    - The DAI count is performed per MBS service.

    - If the UE receives multiple MBS services, multiple sub-codebooks is generated separately for each MBMS service, then UE cconcatenates these sub-codebooks based on the G-RNTI of each MBS service in ascending order to form a type-2 codebook for the multiple MBS services.

    - The maximum number of Type-2 HARQ-ACK sub-codebooks for multicast is determined by UE capability for MBS.

“Codebook for more than one per G-RNTI”

(Nokia) Proposal 31:

  - When Type-2 HARQ-ACK codebook is used the PHY identification of PDSCH HARQ-ACK to MBS sub-codebook mapping is the group-common RNTI value.

(Nokia) Proposal 32:

  - When Type-2 HARQ-ACK codebook is used, the order of concatenation of the sub-codebooks to construct a HARQ-ACK codebook, when the HARQ-ACK feedback of different services are scheduled for the same time instance, follows the increasing order of the RNTI values that are used to map PDSCH HARQ-ACK to MBS sub-codebook.

(ZTE) Proposal 5:

  - Regarding Type-2 HARQ-ACK codebook construction for MBS

    - The DAI count is performed per MBS service.

    - If the UE receives multiple MBS services, multiple sub-codebooks is generated separately for each MBMS service, then UE cconcatenates these sub-codebooks based on the G-RNTI of each MBS service in ascending order to form a type-2 codebook for the multiple MBS services.

    - The maximum number of Type-2 HARQ-ACK sub-codebooks for multicast is determined by UE capability for MBS.

(Samsung) Proposal 6:

  - For a same priority and a same TRP, Type-2 HARQ-ACK codebooks for multicast are placed in ascending order of a corresponding G-RNTI after a Type-2 HARQ-ACK codebook for unicast and are jointly coded.

(CATT) Proposal 16:

  - For concatenating more than one Type-2 HARQ-ACK codebooks for multicast, the order between sub-codebooks can be based on the increasing order of G-RNTI.

- “max number of G-RNTI”

(CATT) Proposal 15:

  - Support concatenating no more than 3 Type-2 HARQ-ACK codebooks for multicast scheduling.

(Lenovo) Proposal 8:

  - For multicast PDSCHs, don’t support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

(OPPO) Proposal 3:

  - At most one Type 2 MBS HARQ-ACK sub-codebook is concatenated with type 2 unicast HARQ-ACK sub-codebook.

### Round-1 (pending)

FL’s Comments

RAN2 LS in (R1-2106408) indicates RAN2 agreed that in order to receive multiple MBS services, UE need to support multiple G-RNTIs and/or G-CS-RNTIs. And it is FFS whether this depends on UE capability.

RAN1 discussion includes three issues based on the submitted proposals, which are DAI counting, Type-2 codebook construction, and the maximum number of G-RNTI UE supports.

At least the proposals for the first two issues seem straightforward. The last issue could be handled in UE feature discussion phase in FL’s opinion but more discussion could happen first in this meeting.

FL’s Proposal:

#### Proposal 2.4.1-1

When UE supports and is configured with more than one G-RNTI,

- for Type-2 codebook construction, DAI is separately counted per G-RNTI.

- Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each RNTI following the ascending order of the G-RNTI value.

Collect views:

| Company | Comments |
| --- | --- |
| CATT | Agree. |
| Nokia, NSB | Support. |
| Samsung | Support – there is no other meaningful way. |
| ZTE | Support the above proposal. |
| Qualcomm | Ok |
| - Ericsson | - Support. Is RNTI not supposed to say “G-RNTI”? |
| - NTT DOCOMO | - Support |
| - CMCC | - Support |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | OK |
| Spreadtrum | Support |
| MediaTek | OK |
| OPPO | OK |
| Apple | OK |
| vivo | support |
| Chengdu TD Tech, TD Tech | ok |

#### Proposal 2.4.1-2

The maximum G-RNTI UE supports is up to UE capability reporting and the candidate values are  [2] and [3].

Collect views:

| Company | Comments |
| --- | --- |
| - CATT | - Agree. - As the number of G-RNTI that configured increasing, the probability of last DCI missing issue and the overhead of the UL DAI increase. The number of G-RNTI shall be limited within no more than 3 so that the probability last DCI missing issue and the overhead of the UL DAI is acceptable. |
| - Nokia, NSB | Should be handled in UE feature discussion phase. |
| - Samsung | The value of 1 should be included. Can be discussed in UE features. |
| - ZTE | We are not sure whether we need to discuss this issue since from RAN1 perspective, the complexity mainly comes from the PDSCH scheduled in the same slot, instead of the number of G-RNTI. Even if the UE capability is needed, seems more like a RAN2 UE capability. |
| - Qualcomm | Similar concern as ZTE. |
| - Ericsson | - Support in principle. Agree that this could be discussed in capability/UE feature discussion. |
| - NTT DOCOMO | - The maximum number is something to be decided later in the discussion of UE capability. |
| - CMCC | - Discuss this issue in UE capability/feature. |
| LG | We guess that ‘the maximum G-RNTI UE supports’ in this proposal does not mean the maximum number of G-RNTIs that UE are interested or receives. Rather, we guess that ‘the maximum G-RNTI UE supports’ in this proposal means ‘the maximum number of G-RNTIs that UE supports for concatenating sub-codebooks for different RNTIs for multicast’. We think that the number of sub-codebooks for multicast may depend on whether sub-codebook for unicast is concatenated with sub-codebook(s) for multicast. Accordingly, we could change to: #### Proposal 2.4.1-2 The maximum number of G-RNTIs that UE supports for concatenating sub-codebooks for different RNTIs for multicast only G-RNTI UE supports is up to UE capability reporting and the candidate values are [2] and [3]. - FFS: whether this capability depends on whether sub-codebook for unicast is concatenated with sub-codebook(s) for multicast |
| Spreadtrum | Fine in principle |
| MediaTek | This issue can be discussed in UE feature phase. |
| OPPO | Agree to define maximum number of G-RNTI that UE supports as a UE feature, the exact value should be discussed in UR feature/capability. |
| Apple | General ok with this proposal, or this UE capability can be discussed later. |
| vivo | We support that “The maximum G-RNTI UE supports is up to UE capability reporting”. We think the candidate values can be FFS in UE feature discussion. |
| Chengdu TD Tech, TD Tech | The number of the G-RNTIs supported by a UE is 2 or 3? Why? |

## Enh Type-2 / Type 3 HARQ codebook

Submitted Proposals

(Nokia) Proposal 16:

  - For ACK / NACK based feedback for RRC_CONNECTED Ues receiving multicast, do not support

    - Enhanced Type-2 HARQ-ACK codebook.

    - Type-3 HARQ-ACK codebook

### Round-1 (closed)

FL’s Comments

FL will not suggest a proposal at this stage since there is a single proposal for this issue.

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Ok. |
| Samsung | The proposal is not necessary. Type-3 is already supported and can be used (e.g. as part of the unicast configuration). Fine to not pursue any optimizations in this WI. Possibly a more suitable proposal is that enhancements to eType-2/Type-3 CBs are not pursued in this WI. |
| ZTE | We have already support type-1, type-2 HARQ-ACK codebook and NACK-only for MBS. Looking at the current discussion, so many remaining issues may even jeopardize the completion of the already agreed HARQ mechanisms. In this sense, additionally supporting other HARQ mechanisms at this stage may not be an appropriate decision from our perspective. |

## UCI multiplexing/prioritizing

Submitted Proposals

“last unicast DCI”

(vivo) Proposal 8:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI for unicast.

(Nokia) Proposal 21:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI for unicast.

(Samsung) Proposal 2:

  - When a UE multiplexes HARQ-ACK associated with unicast DCIs and HARQ-ACK associated with multicast DCIs in a PUCCH, the UE determines a PUCCH resource from the PRI value in the last unicast DCI.

(CMCC) Proposal 7:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission should be based on the PRI indicated in the last DCI for unicast.

(DOCOMO) Proposal 7:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the PUCCH resource is determined by the last unicast DCI.

    - The order is first indexed in an ascending order across serving cells indexes for a same PDCCH monitoring occasion and is then indexed in an ascending order across PDCCH monitoring occasion indexes.

(Ericsson) Proposal 11:

    - When multicast and unicast traffic HARQ feedback can be multiplexed into the same PUCCH resource, the specific PUCCH resource is determined by the PRI in DCI associated with unicast traffic.

“last across DCI”

(Spreadtrum) Proposal 5:

  - For ACK/NACK based HARQ-ACK feedback, determining the PUCCH resources for transmission is based on the PRI indicated in the last DCI across unicast and multicast.

(CATT) Proposal 17:

  - For multiplexing the HARQ-ACKs of multicast and unicast, the last DCI refers to the last DCI across unicast and multicast.

(Qualcomm) Proposal 3:

  - For multiplexing of ACK/NACK feedback for unicast and multicast, determining the PUCCH resources for transmission is based on the PRI indicated in the last DCI across unicast and multicast.

(LGE) Proposal 21:

  - For multiplexing HARQ-ACKs for dynamically scheduled multicast and dynamically scheduled unicast or for dynamically scheduled multicast PDSCHs only, UE determines a PUCCH resource based on the PRI of the last DCI scheduling either multicast or unicast.

(MediaTek) Proposal 3:

  - When multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the “last DCI” refers to the last DCI across unicast and multicast.

(Apple) Proposal 4:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the PRI in last DCI across the unicast and multicast is used for PUCCH resource indication.

- “multiplexing for overlap or not”

(Nokia) Proposal 20:

  - Multiplexing of HARQ-ACK feedback of same priority is supported in case PUCCH transmissions are in the same slot, not only when the corresponding PUCCH resources physically overlap.

(DOCOMO) Proposal 9:

  - When multicast transmission is performed from a single TRP and HARQ-ACK feedback for unicast and multicast is indicated as transmitted at the same slot/sub-slot, the UE multiplexes HARQ-ACK with the same priority regardless whether PUCCH resources are overlapped or not.

- “HARQ with other UCI”

(Intel) Proposal 5:

  - When a PUCCH resource carrying HARQ ACK for MBS overlaps in time domain with a PUCCH resource carrying other UCI types, a default priority order for PUCCH dropping can be defined in specification where HARQ-ACK feedback of unicast transmission > HARQ-ACK feedback of MBS transmission > SR > CSI report.

Multiplexing of different priorities:

(Samsung) Proposal 1:

  - A UE does not support multiplexing of multicast HARQ-ACK and unicast UCI/data of different priorities – the UE transmits only the information of larger priority..

Others

(vivo) Proposal 10:

  - For PUCCH including multicast HARQ-ACK overlaps with PUSCH, reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and PUSCH.

    - FFS UL-DAI indication when both unicast HARQ-ACK and multicast HARQ-ACK are multiplexed on one PUSCH

### Round-1 (pending)

FL’s Comments

Whether the PRI is based on the last unicast DCI or the last DCI across unicast and multicast when determining the PUCCH resources after multiplexing HARQ bits for unicast and multicast was discussed in the last meeting without no progress due to controversial.

From the submitted proposals for this meeting:

- Supportive to “last unicast DCI”: [6, vivo, Nokia, Samsung, CMCC, DOCOMO, Ericsson]

- Supportive to “last across DCI”: [6, Spreadtrum, CATT, Qualcomm, LGE, MediaTek, Apple]

The arguments from both sides from submitted contributions in RAN1#105 and RAN1#106 are summarized as follows:

From proponents of Alt1 (last unicast DCI), the issues for Alt2 include:

  - No closed-loop power control for unicast PUCCH (CMCC).

  - one-to-one mapping between PUCCH resources for any resource set of multicast HARQ-ACK and PUCCH resources for any resource set of unicast (and multicast) HARQ-ACK, across all PUCCH resource sets (CMCC, Samsung).

  - PUCCH resource allocation flexibility of Alt.1 is higher than Alt.2 (CMCC, vivo, Nokia, DOCOMO).

  - PUCCH configuration for multicast may only include configuration for PUCCH formats 0/1 (Ericsson).

From proponents of Alt2 (“last across DCI”), less spec impact (Spreadtrum, CATT), and the issues for Alt1 include:

  - Alt.1 requires a unicast PDSCH always to be scheduled and the number of unicast DCIs shall be more or equal to four (CATT, Apple).

  - Alt.1 has codebook size ambiguity due to UE missing last DCI for multicast coming after unicast DCI (QC)

The views still diverge and this issue is hopefully to be solved in this meeting. I would suggest looking at the concerns from companies for each alternative in order to identify whether there is a big concern that companies cannot live. Also, companies are encouraged to continue the discussion addressing others comments/concerns without the limit of one-come-one-round for more efficient discussion.

FL’s Proposal:

#### Proposal 2.6.1-1

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, continue discussing the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;

Collect views:

| Company | Concerns to Alt1 | Concerns to Alt2 |
| --- | --- | --- |
| CATT |  | - No closed-loop power control for unicast PUCCH (CMCC). - Since different UEs may need different PUCCH transmission power, it is not possible to indicate the same close loop TPC command in group PDCCH, so the ‘TPC command for scheduled PUCCH’ is not need. If the last DCI is for multicast, there is no closed-loop power control for multicast PUCCH. It means the power is unchanged based on the ‘TPC command for scheduled PUCCH’ in the recent unicast DCI. Thus, it is acceptable that there is no no closed-loop power control for multicast PUCCH. - PUCCH resource allocation flexibility of Alt.1 is higher than Alt.2. - When the last DCI is multicast, gNB will guarantee the PUCCH resources are orthogonal among different UEs in a same group, which was already supported. If it is difficult for gNB to assign the orthogonal PUCCH resource, based on gNB’s implementation, gNB can make unicast DCI as last DCI. - PUCCH configuration for multicast may only include configuration for PUCCH formats 0/1. - The PUCCH configuration for multicast can also include the configuration for PUCCH formats 2/3. We do not see the reason why only PUCCH formats 0/1 can be configured. Also, no related agreement is achieved. |
| Nokia, NSB |  | The proposal discusses the case where there are both unicast and multicast HARQ-ACK feedback, therefore, anyway multicast PDSCH was scheduled in such a scenario. Thus, concern from CATT and Apple does not exist. - We believe that the last DCI missing problem is not unique to Alt. 2. There are further scenarios where the codebook size ambiguity can occur, as we have presented in Error! Reference source not found.. One solution can be including an index to represent which service the HARQ-ACK feedback belongs to. |
| Samsung |  | Does not work. It is impossible for a NW to allocate multicast PUCCH resources to support unicast HARQ-ACK. Several other disadvantages also exist. |
| ZTE | We support Alt.1 | Alt.1 has the flexibility of indicating different PUCCH resource and TPC. In practice, it is difficult or even impossible to guarantee that all the PUCCH resources with the same PRI for different UEs in the same group are orthogonal. Some specification work must be introduced to avoid such implementation complexity. Regarding companies’ concern “Alt.1 requires a unicast PDSCH always to be scheduled”, note that the main bullet says “For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast”, there is already unicast scheduling. Network don’t need to additionally schedule any unicast PDSCH. |
| Qualcomm | It does not work if last DCI is not unicast. We didn’t see clear solution of Alt1 if the last multicast DCI is missing, unless we mandate the gNB always schedules a unicast DCI as the last DCI. Based on the discussion, the concerns on Alt2 are mostly because of the PUCCH resource for multicast. We don’t think Alt1 has higher flexibility than Alt2. Based on RAN1 agreement, the PUCCH resources for multicast are optionally configured. - If not configured, the UE can use PUCCH for unicast. - If configured, gNB can make sure the PUCCH resource of the last DCI across multicast and unicast is sufficient for multiplexing. But for progress, we propose: Alt2 is used at least if PUCCH resources are not configured for multicast. |  |
| Ericsson | We support Alt1 From the discussion from our Tdoc: "The rule to select the corresponding PUCCH resource can be similar as what is defined before R17. First decide the PUCCH resource set according to the number of HARQ-ACK feedback bits, second determine the specific PUCCH resource according to PRI in DCI. Here, the number of HARQ-ACK feedback bits is the sum of both multicast and unicast traffic. Considering that multicast may be configured to use NACK-only feedback, and NACK-only may need to be converted into ACK/NACK feedback so that multicast and unicast can be transmitted in one joint codebook, it is therefore more reasonable to use the PRI from the last DCI of unicast to indicate PUCCH resource." |  |
| CMCC | Support Alt 1 | 1) Alt 1 can indicate TCP command 2) when multicast and unicast HARQ-ACK are configured with separate PUCCH-Config, the PUCCH resource after HARQ-ACK multiplexing based on Alt 1 can reduce PUCCH overhead. This is because if only the HARQ-ACK feedback for multicast needs to be transmitted on the PUCCH resources of multicast, RRC signaling only needs to configure few PUCCH resources for HARQ-ACK for multicast. If HARQ-ACK feedback for unicast multiplexes on the PUCCH resources of multicast, the UCI payload size after multiplexing will increase, RRC signaling needs to configure more PUCCH resources or PRBs for multicast, which brings huge PUCCH overhead. |
| Lenovo, Motorola Mobility | Alt 1 is preferred for more flexibility for UE although it leads to limitation to gNB. |  |
| OPPO |  | Support Alt 2. 1. PUCCH power control should not be a problem, as anyhow UE may have multicast only, and a power control mechanism for PUCCH conveying multicast HARQ-ACK only should be supported. 2. For Alt 1, if the last unicast DCI is followed by multiple multicast transmissions, gNB has to predict the payload size when it transmit the unicast DCI, which may not be possible all the time. |
| Apple | Alt2 is preferred. Regarding the PUCCH resource allocation flexibility, even the PRI for unicast is used, the complexity of PUCCH resource allocation is increased to keep resource orthogonal among users with same PRI. So, the same resource allocation principle applying to multicast is not the issue, i.e., keep PUCCH resource orthogonal with the same PRI for multicast. Some companies argued this proposal is for the case of HARQ-ACK multiplexing between unicast and multicast. For the case of no multiplexing, i.e., multicast only case, gNB still need to keep the PUCCH resource among different MBS users orthogonal with the same PRI indication. |  |
| Chengdu TD Tech, TD Tech | Alt 1 |  |

## PUCCH resource

Submitted Proposals

“Resource sets/resources”

(ZTE) Proposal 1:

  - Regarding configuration of PUCCH resource sets for multicast (reuse unicast mechanism):

- A maximum of 4 PUCCH resource sets are configured for multicast for the UE.

- NACK-only and ACK/NACK feedback share the same PUCCH resource set.

- UCI size limit for each PUCCH resource sets is configured, except for the PUCCH resource set 0, which only supports no more than 2 UCI bits.

(Lenovo) Proposal 7:

  - For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for ACK/NACK based feedback can be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast.

(OPPO) Proposal 13:

  - For ACK/NACK based HARQ-ACK feedback for multicast, both shared and separate PUCCH resources among UEs within the group are supported.

(OPPO) Proposal 14:

  - For ACK/NACK based feedback and NACK only based feedback for multicast, RSRP based PUCCH resource configuration is supported.

(OPPO) Proposal 15:

  - Whether shared or separate PUCCH resources are used can be up to gNB configuration or scheduling.

(CMCC) Proposal 3:

  - For the separate PUCCH-ConfigurationList configured for multicast for two simultaneously constructed HARQ-ACK codebooks with different priorities,

    - different PUCCH Resource IDs are configured in the two PUCCH-Config within the PUCCH-ConfigurationList.

(LGE) Proposal 1:

  - PUCCH-config for multicast can be configured per CFR or per UE’s active UL BWP associated to the CFR.

- “k1 indication”

(LGE) Proposal 3:

  - For PTM scheme 1, group common DCI indicates a single PUCCH resource indicator and a single PDSCH-to-HARQ_feedback timing indicator at least for ACK/NACK based HARQ-ACK.

(LGE) Proposal 4:

  - For UE specific PUCCH resource allocation for ACK/NACK based HARQ-ACK to group common DCI, different UEs in the group can be configured with different values of at least PUCCH-Resource and dl-DataToUL-ACK in UE dedicated PUCCH-config for multicast or for unicast (unless PUCCH-config for multicast is configured).

    - Different UEs can be allocated with different PUCCH resources by the same PUCCH resource indicator and the same PDSCH-to-HARQ_feedback timing indicator of the group common DCI.

(LGE) Proposal 5:

  - For PTP retransmission, the PUCCH resource indicator and the PDSCH-to-HARQ_feedback timing indicator in UE specific DCI are interpreted based on PUCCH-config for unicast, regardless of whether PUCCH-config for multicast is configured or not.

(DOCOMO) Proposal 5:

  - A list of k1 values for DCI format 1_0 for multicast is configurable.

### Round-1 (closed)

FL’s comment:

Regarding PUCCH resources for ACK/NACK-based HARQ-ACK feedback for multicast, what we have agreed are as follows:

Agreement:

For ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies.

Agreement:

For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively.

- FFS other configurations

The question/issue we can discuss for this meeting is whether the maximum number of PUCCH resources sets, the maximum PUCCH resource in the first PUCCH resource set, the maximum number of UCI information bits for the first PUCCH resource set are unchanged or increased, especially when UE is configured with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.

FL’s Proposal:

#### Proposal 2.7.1-1

For UE supporting ACK/NACK-based HARQ-ACK feedback for multicast, the maximum values that are configured to UE as follows are unchanged:

- the maximum number of PUCCH resources sets,

- the maximum PUCCH resource in the first PUCCH resource set,

- the maximum number of UCI information bits for the first PUCCH resource set.

- Note:

  - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.

  - The case of NACK-only based is discussed separately.

Collect views:

| Company | Comments |
| --- | --- |
| CATT | OK with it. |
| Nokia, NSB | Support. We do not need changes in this context. |
| Samsung | Support (assuming that the intention is “… are unchanged compared to unicast in Rel-16”). |
| ZTE | We are ok with the above proposal. |
| Qualcomm | Ok in principle |
| Ericsson | OK in principle. Second bullet could be reworded "the maximum number of PUCCH resources in the first PUCCH resource set," |
| NTT DOCOMO | Support |
| CMCC | support |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | OK |
| Spreadtrum | Support |
| MediaTek | Support |
| OPPO | OK |
| Apple | Ok with this proposal. |
| vivo | Ok in principle (assuming that the intention is “… are unchanged compared to unicast in Rel-16”). It would be better to reword the proposal. E.g., the maximum number of PUCCH resources sets in each PUCCH-config The current wording is not very clear to us. For the case of UE is configured with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast. It is not clear that the maximum number of PUCCH resources sets is 4 or 8. In our view, the maximum number of PUCCH resources sets within in each PUCCH-config is 4, and the maximum number of PUCCH resources sets for a UE is increased when more than one PUCCH-config is configured. |

### Round-2 (closed)

#### Proposal 2.7.2-1 (H)

For UE supporting ACK/NACK-based HARQ-ACK feedback for multicast, the maximum values that are configured to UE as follows are unchanged compared to unicast in Rel-16:

- the maximum number of PUCCH resources sets,

- the maximum number of PUCCH resources in the first PUCCH resource set,

- the maximum number of UCI information bits for the first PUCCH resource set.

- Note:

  - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.

  - The case of NACK-only based is discussed separately.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | The proposal is updated per the Round-1 comments and prioritized due to impact to RRC parameters. |
| Nokia, NSB | Support |
| Samsung | OK to progress although no good reason to have the same maximum number of PUCCH resources in the first PUCCH resource set as for unicast. |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | OK |
| vivo | ok |
| Chengdu TD Tech, TD Tech | Ok |
| Qualcomm | ok |
| ZTE | OK |
| CMCC | OK |
| OPPO | OK |
| CATT | OK |
| NTT DOCOMO | Support |
| MediaTek | OK |
| Ericsson | It is still not very clear. In R-16, when there are two PUCCH-Configs, then the total number of PUCCH resource sets is increased, from 4 to 8. In other words, maximum number of PUCCH resource set is from per PUCCH-Config perspective. While the total number of PUCCH resources are not increased. If we allow separate PUCCH-Config for multicast, what do we really want to keep unchanged? Maximum number of PUCCH resource set per PUCCH-config, or total number of PUCCH resource set from all PUCCH-Config? We think we shall at least add that the maximum number of PUCCH resources are kept unchanged compared to R-16. |
| Apple | OK |

# NACK-only based feedback specific

## PUCCH format and HARQ bits

Submitted Proposals

“format and HARQ bits”

(Huawei) Proposal 10:

  - Support only one HARQ-ACK bit for NACK-only feedback for PUCCH format 0 or format 1.

(Huawei) Proposal 11:

  - The sequence cyclic shift for NACK-only is .

(OPPO) Proposal 6:

  - The PUCCH format used for NACK-only based HARQ-ACK feedback is semi-statically configured by RRC.

(OPPO) Proposal 8:

  - In NACK-only based HARQ feedback scheme, only 1 HARQ-ACK bit is transmitted in one PUCCH.

(Chengdu TD Teck) Proposal 1:

  - Support PUCCH format 2/3/4 for NACK-only based HARQ-ACK feedback.

(Chengdu TD Teck) Proposal 2:

  - If PUCCH format 2/3/4 is used to feed back the NACK information for an MBS session, N is configured as a part of the configuration information of the MBS session. UE maps the NACK information of 1 bit long onto a sequence of N>2 bits long. Then send the NACK information with PUCCH format 2/3/4.

(Ericsson) Proposal 1:

  - FFS: Use the PUCCH format 0 or format 1 phase rotations and for format 1 the DFT spreading as dimension in addition to OFDM-symbol and PRB

(Ericsson) Proposal 2:

  - FFS: Associate each NACK signal with a subset of bits in the NACK-only codebook, where multiple UEs use the same PUCCH resource for the NACK-only signal relating to the same subset and the subset size may reduce to 1. A UE transmits the NACK signal if at least one bit of the associated subset of bits in the NACK-only codebook is cleared, i.e. indicates a PDSCH decoding failure. The gNB accordingly retransmits the transport blocks of all HARQ processes of the subset.

### Round-1 (pending)

FL’s Comments

The discussions on PUCCH formats are quite diverse and more views/input/discussions are needed for FL to suggest proposals. One proposal regarding the number of HARQ-ACK bits for PUCCH format 0 and format 1 seems straightforward, which follows the same for NACK-only supported in V2X.

FL’s Proposal:

#### Proposal  3.1.1

For PUCCH format 0 or format 1 supported for NACK-only based HARQ-ACK feedback for multicast, only 1 HARQ-ACK bit is transmitted in one PUCCH.

Collect views:

| Company | Comments |
| --- | --- |
| CATT | OK with it. |
| Nokia, NSB | Support. |
| Samsung | Do not support. Maybe the intention is to say that the transmission is not modulated which is fine. For the case of >1 HARQ-ACK bits, a PUCCH can convey multiple HARQ-ACK bits (e.g. the issue discussed in proposal 3.2.1-2). |
| ZTE | OK. Maybe to clarify a little bit, we can have the following change. #### Proposal 3.1.1 For PUCCH format 0 or format 1 supported for NACK-only based HARQ-ACK feedback for multicast, only 1 HARQ-ACK bit is transmitted in one PUCCH before multiplexing. |
| Qualcomm | ok |
| Ericsson | Do not support. We propose multiple bits can be transmitted in one PUCCH in order to reduce the number of PUCCHs that are needed. |
| NTT DOCOMO | We would like to prioritize the discussion of Proposal 3.2.1-2 (more than one/two for NACK-only). Because which method is chosen in Proposal 3.2.1-2 will affect the number of HARQ-ACK bits in one PUCCH. |
| CMCC | Not support, this agreement conflicts with Alt 4 in Proposal 3.2.1-2 |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | Support. |
| MediaTek | OK |
| OPPO | Support |
| Apple | Don’t support. Why 2 bits can’t be transmitted using one PUCCH resource, instead using two PUCCH resources separately. |
| vivo | ok |
| Chengdu TD Tech, TD Tech | ok |

## Overlapping PUCCH/PUSCH

Submitted Proposals

“Overlap with other PUCCH/PUSCH”

Cases:

(ZTE) Proposal 11:

  - For the time domain overlapping between NACK-only PUCCH and unicast PUCCH/PUSCH, RAN1 study PUCCH multiplexing for the following scenes.

    - Unicast ACK/NACK/SR PUCCH F0/F1 overlaps with NACK-only PUCCH.

    - Unicast ACK/NACK/CSI PUCCH F2/F3/F4 overlaps with NACK-only PUCCH; unicast PUSCH and NACK-only PUCCH overlap.

(ZTE) Proposal 12:

  - If HARQ-ACK PUCCH for unicast and NACK-only PUCCH for multicast are determined in the same UL slot, then they should be multiplexed for the same priority (regardless of whether they overlap in the time domain).

    - The multiplexed PUCCH resource is determined based on the PRI in the last DCI corresponding to the unicast HARQ-ACK PUCCH.

    - NACK-only based HARQ-ACK needs to be transformed to ACK/NACK based HARQ-ACK bits.

Solution 1: NACK-only->ACK/NACK

(Huawei) Proposal 9:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, or the NACK-only feedback are available with other UCI feedback on the same PUCCH resource, or the PUCCH resources are overlapped with PUSCH transmission, the NACK-only feedback is transformed into the ACK/NACK based feedback.

(Nokia) Proposal 19:

  - If for a UE a scheduled group-common PUCCH resource for PTM NACK-only feedback overlaps in time with a UE-specific PUCCH resource for other UCIs or a PUSCH transmission with the same priority, this UE should multiplex the PTM HARQ-ACK feedback with the other UCIs on the UE-specific PUCCH resource or with the PUSCH transmission, by treating NACK-only feedback as a 1-bit ACK / NACK feedback.

(Nokia) Proposal 35:

  - When group-common NACK-only HARQ-ACK feedback is used as the HARQ-ACK scheme, in case the UE has UE-specific HARQ-ACK resource for unicast services along with group-common NACK-only resource for PTM, the UE utilizes the UE-specific PUCCH resource by constructing separate HARQ-ACK sub-codebooks, as if ACK / NACK based approach is being used for PTM.

(ZTE) Proposal 12:

  - If HARQ-ACK PUCCH for unicast and NACK-only PUCCH for multicast are determined in the same UL slot, then they should be multiplexed for the same priority (regardless of whether they overlap in the time domain).

    - The multiplexed PUCCH resource is determined based on the PRI in the last DCI corresponding to the unicast HARQ-ACK PUCCH.

    - NACK-only based HARQ-ACK needs to be transformed to ACK/NACK based HARQ-ACK bits.

(NEC) Proposal 4:

  - Support multiplexing between NACK-only based HARQ-ACK feedback and other UCIs/PUSCH.

(CMCC) Proposal 13:

  - When PUCCH resource for NACK-only feedback for multicast collides with PUCCH resources for HARQ-ACK feedback for unicast or other UCI,

    - For NACK-only based feedback with low priority, NACK-only feedback for multicast can be dropped.

    - For NACK-only based feeback with high priority, NACK-only feedback should turn into ACK/NACK based feedback.

(LGE) Proposal 11:

  - When different PUCCHs for NACK only HARQ-ACK and ACK/NACK based HARQ-ACK are overlapped, NACK only HARQ-ACK is changed to ACK/NACK based HARQ-ACK to be multiplexed with other ACK/NACK based HARQ-ACK.

(LGE) Proposal 13:

  - Discuss UE procedure for multiplexing NACK-only based HARQ-ACK and SR for PUCCH Format 0 and/or 1.

(Convida) Proposal 2:

  - When a PUCCH resource allocated for an MBS transmission (1st transmission) using NACK only HARQ feedback collides with a PUCCH resource allocated for a transmission (2nd transmission) using ACK/NACK HARQ feedback, the UE multiplexes the HARQ feedback for the 1st transmission on the PUCCH resource allocated for the 2nd transmission.

(Convida) Proposal 3:

  - When a UE multiplexes the HARQ feedback for the 1st transmission on the PUCCH resource allocated for the 2nd transmission, instead of gernerating the NACK only feedback, the UE generates the ACK/NACK based HARQ feedback for the 1st transmission and multiplexes it on the PUCCH.

(Convida) Proposal 4:

  - When a PUCCH resource allocated for an MBS transmission (1st transmission) using NACK only HARQ feedback collides with a PUSCH transmission, the UE multiplexes the HARQ feedback for the 1st transmission on the PUSCH resource.

(Convida) Proposal 5:

  - When a UE multiplexes the HARQ feedback for the 1st transmission on the PUSCH resource, instead of gernerating the NACK only feedback, the UE generates the ACK/NACK based HARQ feedback for the 1st transmission and multiplexes it on the PUSCH.

Solution 2:drop NACK-only

(DOCOMO) Proposal 12:

  - If a PUCCH for NACK-only based feedback overlaps with a PUCCH for SR/CSI or a PUSCH, NACK-only based feedback is dropped.

“more than two bits for NACK-only”

Cases:

(ZTE) Proposal 9:

  - RAN1 supports PUCCH multiplexing for the following scenarios for NACK only PUCCH.

    - One NACK-only PUCCH overlaps with another NACK-only PUCCH;

    - More than 2 NACK-only PUCCHs overlap with each other.

    - One NACK-only PUCCH overlaps with multiple non-overlapped NACK-only PUCCHs.

Solution 1: NACK-only->ACK/NACK

(Huawei) Proposal 9:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, or the NACK-only feedback are available with other UCI feedback on the same PUCCH resource, or the PUCCH resources are overlapped with PUSCH transmission, the NACK-only feedback is transformed into the ACK/NACK based feedback.

(LGE) Proposal 12:

  - When different PUCCHs for different NACK only HARQ-ACKs are overlapped, all NACK only HARQ-ACKs are changed to ACK/NACK based HARQ-ACK to be multiplexed.

(Apple) Proposal 3:

  - If the reported HARQ-ACK bits are larger than PUCCH payload size, UE switches to ACK/NACK based HARQ-ACK feedback mode.

(ZTE) Proposal 10:

  - When multiple NACK-only based feedbacks are indicated to be transmitted in the same slot,

    - If sub-slot based PUCCH is supported, UE transmits multiple sub-slot based PUCCHs with each corresponding to one NACK-only feedback;

    - Otherwise, consider the following options

      - Option 1: UE transforms multiple NACK-only feedback to ACK/NACK information bits;

      - Option 2: Multiple PUCCH resources are configured, where each PUCCH resource corresponds to one codebook.

Solution 2: Sub-slot and more PUCCH transmission

(Nokia) Proposal 5:

  - For a proper operation of NACK-only HARQ-ACK feedback for MBS, a UE can be optionally configured to support more than one PUCCH carrying HARQ-ACK feedback per slot with a method that is to be down-selected from the following options:

    - Option 1:  Allowing multiple HARQ-ACK feedback carrying PUCCHs per slot.

    - Option 2:  Based on Rel-16 sub-slot PUCCH mechanism.

    - Option 3:  DL-heavy slot configuration.

(ZTE) Proposal 10:

  - When multiple NACK-only based feedbacks are indicated to be transmitted in the same slot,

    - If sub-slot based PUCCH is supported, UE transmits multiple sub-slot based PUCCHs with each corresponding to one NACK-only feedback;

    - Otherwise, consider the following options

      - Option 1: UE transforms multiple NACK-only feedback to ACK/NACK information bits;

      - Option 2: Multiple PUCCH resources are configured, where each PUCCH resource corresponds to one codebook.

Solution 3: Combination of NACK-only

(Samsung) Proposal 7:

  - A UE is provided orthogonal PUCCH resources to select from according to combinations of NACK values for PDSCH/TB receptions.

(CMCC) Proposal 12:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, HARQ-ACK bits can be multiplexed in one PUCCH.

    - Different HARQ-ACK information of a number of group-common PDSCHs are feedbacked in different PUCCHs respectively.

(DOCOMO) Proposal 10:

  - For NACK-only based HARQ-ACK feedback for multicast, support multiplexing multiple HARQ-ACK bits in a PUCCH resource.

(DOCOMO) Proposal 11:

  - For multiple HARQ-ACK bits of NACK-only based HARQ-ACK feedback, associate a bit sequence of HARQ-ACK codebook to a PUCCH resource.

(Ericsson) Proposal 4:

  - Denoting the number of bits in the NACK-only codebook by N, downselect one or both from the following two variants to signal all information from the codebook in the same uplink slot:

- To convey N bits the UE transmits N PUCCH resources in the same slot, where each PUCCH resource represents one bit in the NACK-only codebook of N bits.

- To convey N bits the UE transmits one selected PUCCH resource   in a slot, from a set of M=2^N-1 configured PUCCH resources. The actually-used PUCCH resource therefore carries the required N information bits.

Solution 4: NACK if any NACK

(vivo) Proposal 14:

  - When more than one NACK-only based feedback is available for transmission in the same PUCCH slot, UE transmits NACK when any of the PDSCHs is NACK.

### Round-1 (pending)

“Overlap with other PUCCH/PUSCH”

FL’s Comments

When discussing the PUCCH transmission for NACK-only collides wither other PUCCH/PUSCH transmissions, there should be a basic assumption of the priority of NACK-only because the PUCCH transmission for other ACK/NACK or UCI or PUSCH is of either low or high priority depending on indication or default assumption. The priority of NACK-only is discussed in section 4.1 as well as the dropping rule for the case of PUCCHs overlapping for different priorities.

The discussion in this section can focus on at least the case of PUCCHs overlapping for the same priority.

At least for the case when NACK-only PUCCH overlaps with other PUCCH/PUSCH, the vast majority support multiplexing NACK-only with ACK/NACK and other UCI for unicast by transforming NACK-only into ACK/NACK bit.

FL’s Proposal:

#### Proposal  3.2.1-1

When PUCCH transmission for NACK-only feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback for unicast or other UCI for the same priority, support UE multiplexing the NACK-only with others by replacing the NACK-only with ACK/NACK HARQ bits.

Collect views:

| Company | Comments |
| --- | --- |
| - CATT | - OK with it. - Although dropping the NACK-only PUCCH is also a potential alterative to solve the collision issue, it is unfair for multicast when NACL-only PUCCH is always dropped. Thus, multiplexing the PUCCH for NACK-only with other PUCCH/PUSCH is preferred. |
| - Nokia, NSB | - Support. |
| - Samsung | Support - The PUCCH resource overhead is not an issue in that case and other alternatives are worse. |
| - ZTE | OK with the above proposal. |
| - Qualcomm | ok |
| Ericsson | Propose to change to: When PUCCH transmission for NACK-only feedback for multicast overlaps in time but not in frequency with PUCCH transmissions for HARQ-ACK feedback for unicast or other UCI for the same priority, support UE multiplexing the NACK-only with others by replacing the NACK-only with ACK/NACK HARQ bits. FFS: If one, several or all bits of the NACK-only codebook are replaced by ACK/NACK HARQ bits and multiplexed with the unicast or multicast codebook. The FFS is motivated like this: Multiple PUCCH resources may be configured to map the NACK-only codebook. In our proposed "variant a)", each bit it mapped to one PUCCH resource. UE can still send NACK-only signals on the non-overlapping NACK-only PUCCH resources. The gNB knows which NACK-only PUCCH resources overlap in time with other PUCCH and from this can know which bits the UE converts to A/N and multiplexes with other A/N bits |
| NTT DOCOMO | Clarification is needed on whether multiplexing also applies to cases where NACK-only feedback is not sent. The presence of NACK-only feedback depends on the result of UE processing. If multiplexing is applied only when NACK-only feedback is sent, then gNB will have to blind detect the UCI. Also, how to handle multiple HARQ-ACK bits in NACK-only feedback will affect what is appropriate to do when they overlap. For example, if Alt3 in Proposal 3.2.1-2 is chosen, the multiplexing/prioritizing procedure will become more complex. Therefore, we prefer to prioritize the discussion of Proposal 3.2.1-2. |
| CMCC | Ok |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | We agree with DOCOMO. Clarification on the detailed use cases is needed. |
| MediaTek | Support. |
| OPPO | OK |
| Apple | Just want to clarify, is the multiplexing is applied to both type-1 and type -2 HARQ codebook? So the type-1 HARQ codebook construction already consider the bits for NACK-only feedback? |
| vivo | Agree with DOCOMO. The proposal is not clear to us. There can be different understanding on the “PUCCH transmission for NACK-only feedback for multicast”. 1: PDSCH is decoded failed, NACK-only PUCCH would be transmitted and it conflicts with PUCCH transmission for HARQ-ACK feedback for unicast or other UCI. 2. Regardless whether PDSCH is decoded failed or not, UE checks the overlapping of NACK-only PUCCH and PUCCH transmission for HARQ-ACK feedback for unicast or other UCI. |
| Chengdu TD Tech, TD Tech | Ok. Another option is to discard the NACK-ONLY feedback |

“more than one/two for NACK-only”

FL’s Comments

The views on how to generate the HARQ feedback for more than one/two bits are diverse, including transforming NACK-only into ACK/NACK, supporting sub-slot based PUCCH and more PUCCH transmission in the same slot, defining combinations of NACK and associating with a specific sequence or PUCCH transmission, or UE always transmit one NACK-only as long as one TB is NACK-ed.

It is noted that multiplexing NACK-only with other ACK/NACK and UCI when PUCCH overlaps can also apply to solve the issue of more than one/two NACK-only by a unified solution with less specification efforts. At least at this stage, we can identify the proposed alternatives.

FL’s Proposal:

#### Proposal  3.2.1-2

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, down-select from the following alternatives:

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

- Alt2: Support sub-slot based PUCCH for this case.

- Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

- Alt5: NACK-only bundling

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Partly support. We believe that Alt 4 and 5 are not feasible. It would be too complex to define combination (of specific sequences) / bundling sizes that would fit different scenarios, in addition to allocation of significant amount of PUCCH resources to represent each combination / bundle. |
| Samsung | Support Alt. 4 – it works with Rel-16, provides best BLER, and has no impact on coverage. Alt. 1 defeats the purpose of having NACK-only. Alt. 2 relies on an unsupported feature that even if considered for Rel-17 multicast, cannot be assumed to be supported by Rel-16 UEs without significant additional HW impact. It will also shrink coverage and does not work in general. Alt. 3 has similar disadvantages with Alt.2 Alt. 5 will result to frequent full retransmissions – e.g. for a DDDSU and 10% BLER, the probability for a full transmission for a single UE is ~35%. For the many UEs in multicast, it is practically 100%. |
| ZTE | Alt.1, Alt.2 and Alt.4 (specific PUCCH transmission). From our perspective, Alt.2 is a straightforward solution to support multiple PUCCHs for HARQ-ACK in one slot. On top of that, we can also support Alt.1 and Alt.4 (specific PUCCH transmission, which requires less spec impact.) |
| Qualcomm | The down-selection depends on max number of NACK-only based feedback in a slot, which may be subject to UE capability. |
| Ericsson | We do not support of Alt1, except for the case that the PUCCH overlaps in time but not in frequency with another PUCH that the UE needs to use for transmitting other HARQ-feedback or UCI, see Proposal 3.2.1-1. Alt2: OK Alt3: OK Alt4: OK Alt5: OK if NACK-only bundling means what we express in Proposal 2 of our Tdoc: " Proposal 2 FFS: Associate each NACK signal with a subset of bits in the NACK-only codebook, where multiple UEs use the same PUCCH resource for the NACK-only signal relating to the same subset and the subset size may reduce to 1. A UE transmits the NACK signal if at least one bit of the associated subset of bits in the NACK-only codebook is cleared, i.e. indicates a PDSCH decoding failure. The gNB accordingly retransmits the transport blocks of all HARQ processes of the subset." Otherwise, we would like to have the FL understanding of the wording. |
| NTT DOCOMO | Support. We prefer Alt4. |
| CMCC | Support, prefer Alt 4 |
| LG | We are fine with this proposal. We prefer Alt 1. |
| OPPO | OK |
| Apple | Another alternative could be dropping the HARQ-ACK bits for NACK-only feedback. |
| vivo | Fine with this proposal. |
| Chengdu TD Tech, TD Tech | Alt 2 or Alt 3 |

## PUCCH resource

Submitted Proposals

“Resource sets/resources”

(Huawei) Proposal 7:

  - A separate PUCCH-Config or PUCCH-ConfigurationList for multicast can be but is not necessarily configured for NACK-only based HARQ-ACK feedback.

(Huawei) Proposal 8:

  - For a separate PUCCH-ConfigurationList for multicast that is configured for NACK-only based HARQ-ACK feedback,

    - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority and high priority feedback, respectively.

(vivo) Proposal 12:

  - For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for NACK-only based HARQ-ACK feedback.

(Nokia) proposal 4:

  - A decision is made by RAN1 group regarding PUCCH resource configuration of NACK-only feedback, down-selected from the following options:

    - 1-	NACK-only feedback has separate PUCCH-config besides that of ACK / NACK based feedback. Separate parameters for NACK-only feedback, e.g., power control parameters, can directly be signaled within the already existing PUCCH-config configuration.

      - For a separate PUCCH-ConfigurationList for multicast that is configured for NACK-only based HARQ-ACK feedback,

        - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority and high priority feedback, respectively.

        - FFS other configurations

    - 2-	NACK-only feedback uses the same PUCCH-config with ACK / NACK based feedback. Separate parameters for NACK-only feedback, e.g., power control parameters, are signaled additionally in the PUCCH-config configuration.

      - NACK-only feedback has separate PUCCH resource sets besides that of ACK / NACK based feedback.

      - NACK-only feedback uses the same PUCCH resource sets with that of ACK / NACK based feedback.

(Nokia) proposal 14:

  - When the UE supports both ACK / NACK based and NACK-only based feedback for multicast, the UE can be configured with separate PUCCH resources for both ACK / NACK based and NACK-only feedback.

(ZTE) Proposal 8:

  - Regarding NACK-only feedback for MBS, PUCCH resource is configured per MBS service.

(CATT) proposal 9:

  - NACK-only based feedback framework is designed based on Rel-15 NR ACK/NACK-based feedback mechanism by considering k1, PRI and DCI CCE index to indicate the PUCCH resource.

(CATT) proposal 10:

  - Up to 32 PUCCH resources can be configured for the PUCCH resource set for NACK-only feedback in MBS.

(CATT) proposal 11:

  - The PUCCH resource set can be used by all MBS services which applying NACK-only based HARQ-ACK feedback.

(OPPO) Proposal 7:

  - The PUCCH resource for NACK-only based HARQ-ACK feedback is configured per G-RNTI.

(Google) Proposal 1:

  - For NACK-only based HARQ-ACK feedback for multicast, it is up to network to configure multiple orthogonal PUCCH resources among UEs within the same group, where each PUCCH resource can be shared by plural UEs.

(CMCC) Proposal 10:

  - For UE supports both ACK/NACK based and NACK-only based feedback for multicast, UE can be configured either PUCCH resources for ACK/NACK based feedback or PUCCH resources for NACK-only based feedback.

(CMCC) Proposal 11:

  - PUCCH-ConfigurationList configured for NACK-only based HARQ-ACK feedback for multicast is separate from PUCCH-ConfigurationList for ACK/NACK based HARQ-ACK feedback for multicast.

    - •The separate PUCCH-ConfigurationList for NACK-only based HARQ-ACK feedback for multicast can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority and high priority feedback, respectively.

(LGE) Proposal 2:

  - Group common PUCCH resources for NACK only based HARQ-ACK are configured within UL CFR configured within UE’s active UL BWP.

    - Group common PUCCH resources are only configured by PUCCH-config for multicast.

    - UE specific PUCCH resources are configured on UE’s active BWP, as currently specified.

(LGE) Proposal 8:

  - Different group common PUCCH resources can be related to different DL RS e.g. in terms of symbol/slot and/or PRB and/or sequence for PUCCH.

(LGE) Proposal 10:

  - Only the first PUCCH resource set with PUCCH resource set ID = 0 in PUCCH-config for multicast can be configured for NACK only based HARQ-ACK.

(Apple) Proposal 1:

  - PUCCH resources for NACK-only are independently configured in UL CFR.

(Apple) Proposal 2:

  - PUCCH resource pool is defined for NACK-based feedback, UE select the PUCCH resource according to the subscribed MSB sessions.

(Ericsson) Proposal 3:

  - The PRI in the DCI is interpreted by UEs in NACK-only mode to select one of the PUCCH resource configurations for NACK-only.

- Power control for NACK-only

(Nokia) proposal 9:

  - A dedicated PUCCH-PowerControl parameter set, distinct from the PUCCH-PowerControl parameter set for UE-specific PUCCH resources, is used for configuration of power control of group-common PUCCH resources for NACK-only HARQ-ACK feedback.

(Nokia) proposal 10:

  - In the PUCCH-PowerControl parameter set for transmissions of NACK-only HARQ-ACK feedback on group-common PUCCH resources allow configuration of the p0-Set to comprise more than the current limit of 8 different p0-PUCCH-Values and indicate which values to use by signalling the p0-PUCCH-ID per MBS and HTID to the UEs via MAC-CE. The number of elements in the p0-Set is FFS; 32 different values, i.e., represented by 5 bits, in steps of 1dB would be provide sufficient dynamic range.

(Nokia) proposal 11:

  - The UEs run an autonomous transmit power control for NACK-only PUCCH transmission based on the observed reliability of NACK-only feedback, where the network only configures constraints such as maximum allowed transmit power and the NACK-only feedback reliability target.

(Nokia) proposal 12:

  - When running an autonomous transmit power control for NACK-only PUCCH transmissions the UE reports to the gNB when the limit of maximum allowed transmit power is reached / approached while the NACK-miss rate requirement is violated / close to being violated.

(Nokia) proposal 13:

  - RAN1 should discuss transmit power control for NACK-only HARQ FB on a per-HARQ-transmission index basis, considering the following options:

    - Based on NACK-only FB reliability measurements reported from the UEs to the gNB, the gNB adjusts the UEs’ transmit power control parameters for NACK-only FB.

    - Within constraints set by the gNB, the UE runs an autonomous transmit power control for NACK-only FB.

(OPPO) Proposal 12:

  - For a UE receiving group-common PDSCH transmitted with PTM scheme 1 a TPC-PUCCH-RNTI different from that for unicast should be configured.

(Qualcomm) Proposal 2:

  - For RRC_CONNECTED UEs, support separate PUCCH power control configuration for ACK/NACK-based and NACK-based multicast HARQ feedback.

    - FFS power control parameters

    - FFS signaling details

- other

(Chengdu TD Tech) Proposal 3:

  - For an MBS session with NACK-only based HARQ-ACK feedback, configure a group of PUCCHs. Each PUCCH has a fixed or dynamical CQI/MCS range. UE can select one PUCCH according to its CQI measurement result. Then send the NACK information with the selected PUCCH.

### Round-1 (closed)

FL’s Comments

For ACK/NACK-based HARQ-ACK feedback for multicast, we have agreed UE can be optionally configured with a separate PUCCH-Config/PUCCH-ConfigurationList for multicast from that configuration for unicast. Moreover, when UE is configured with a separate PUCCH-ConfigurationList, the list can include up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook.

Likewise, such discussion can be extended to NACK-only based feedback.

In addition, the discussion for NACK-only based feedback also includes whether the maximum number of PUCCH resources sets, the maximum PUCCH resource in the first PUCCH resource set, the maximum number of UCI information bits for the first PUCCH resource set that are configured to UE are unchanged or increased, especially when UE is configured with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.

FL’s Proposal:

#### Proposal  3.3.1-1

For NACK-only based feedback for RRC_CONNECTED UE receiving multicast, UE can be optionally configured with a separate PUCCH-Config/PUCCH-ConfigurationList from the configuration for unicast. If not configured, PUCCH-Config/ PUCCH-ConfigurationList for unicast applies.

Collect views:

| Company | Comments |
| --- | --- |
| CATT | Agree. |
| Nokia, NSB | Support. |
| Samsung | What unicast PUCCH-Config can support NACK-only based feedback? |
| ZTE | We agree that UE can be optionally configured with a separate PUCCH-Config/PUCCH-ConfigurationList for NACK-only from the configuration for unicast. However, it is not clear whether PUCCH-Config/PUCCH-ConfigurationList should be configured separately for NACK-only based feedback and ACK/NACK based feedback for multicast. This needs to be clarified first. Then, if PUCCH-Config/PUCCH-ConfigurationList is not configured for NACK-only, but configured for ACK/NACK for multicast and unicast, from our perspective, in this case, the PUCCH-Config/PUCCH-ConfigurationList for ACK/NACK for multicast applies. Thus, we propose the following proposal. #### Proposal 3.3.1-1 For NACK-only based feedback for RRC_CONNECTED UE receiving multicast, UE can be optionally configured with a separate PUCCH-Config/PUCCH-ConfigurationList from the configuration for unicast. If not configured, PUCCH-Config/ PUCCH-ConfigurationList for unicast applies when PUCCH-Config/ PUCCH-ConfigurationList for PUCCH for ACK/NACK-based feedback for multicast are not configured. If not configured, PUCCH-Config/ PUCCH-ConfigurationList for multicast applies when PUCCH-Config/ PUCCH-ConfigurationList for PUCCH for ACK/NACK-based feedback for multicast are configured. |
| Qualcomm | Fine with the main bullet but prefer FFS the subbullet. |
| Ericsson | OK |
| NTT DOCOMO | We would like to prioritize the discussion of Proposal 3.2.1-2 (more than one/two for NACK-only). Because what resources are needed for NACK-only feedback will depend on how multiple HARQ-ACK bits are handled. This will also affect whether PUCCH-Config can be shared between ACK/NACK based and NACK-only based. |
| CMCC | Similar concern with Samsung |
| LG | We are generally fine with this proposal. In addition, as we know, in PUCCH-config, especially for unicast, PUCCH format 0 and format 1 are only allowed for a resource in a first PUCCH resource set. PUCCH format2, format3 and format4 are only allowed for a resource in non-first PUCCH resource set. Thus, considering the current specification, if NACK only based HARQ-ACK is configured in PUCCH-config, it seems straightforward that only the first PUCCH resource set with PUCCH resource set ID = 0 in PUCCH-config for unicast can be configured for NACK only based HARQ-ACK. Moreover, we think that if both feedback options are configured for the UE, NACK-only based feedback and ACK/NACK based feedback share the same ‘separate PUCCH-Config/PUCCH-ConfigurationList from the configuration for unicast’. Thus, this proposal seems fine with this understanding. Considering potential different understanding, we propose to add the following FFS to clarify whether ACK/NACK based HARQ-ACK shares the same ‘separate PUCCH-Config/PUCCH-ConfigurationList from the configuration for unicast’ in this proposal. Thus, this proposal can be changed to: #### Proposal 3.3.1-1 For NACK-only based feedback for RRC_CONNECTED UE receiving multicast, UE can be optionally configured with a separate PUCCH-Config/PUCCH-ConfigurationList from the configuration for unicast. If not configured, PUCCH-Config/ PUCCH-ConfigurationList for unicast applies. - Only the first PUCCH resource set with PUCCH resource set ID = 0 in PUCCH-config (at least for unicast) can be configured for NACK only based feedback. - FFS: whether both NACK-only based feedback and ACK/NACK based feedback share the same PUCCH-Config/PUCCH-ConfigurationList from the configuration for unicast. |
| Lenovo, Motorola Mobility | Share same views with Samsung |
| MediaTek | OK |
| OPPO | Support |
| Apple | For NACK-only based feedback, to avoid misunderstanding, it could be better to clarify that only the first PUCCH resource set is configured or applied. |
| vivo | Fine in principle |

FL’s Proposal:

#### Proposal  3.3.1-2

For the separate PUCCH-ConfigurationList that is optionally configured to UE for  NACK-only based HARQ-ACK feedback for multicast,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority feedback and high priority feedback, respectively.

Collect views:

| Company | Comments |
| --- | --- |
| CATT | Agree. |
| Nokia, NSB | Support. |
| Samsung | Support. |
| ZTE | OK with the proposal. |
| Qualcomm | ok |
| Ericsson | OK |
| NTT DOCOMO | Support |
| CMCC | Support |
| LG | We are generally fine with this proposal. In addition, we think that if both feedback options are configured for the UE, NACK-only based feedback and ACK/NACK based feedback share the same ‘separate PUCCH-ConfigurationList from the configuration for unicast’. Thus, this proposal seems fine with this understanding. Considering potential different understanding, we propose to add the following FFS to clarify whether ACK/NACK based HARQ-ACK shares the same ‘separate PUCCH-ConfigurationList from the configuration for unicast’ in this proposal. #### Proposal 3.3.1-2 - FFS: whether both NACK-only based feedback and ACK/NACK based feedback share the same PUCCH-ConfigurationList from the configuration for unicast. |
| MediaTek | OK |
| OPPO | Support |
| Apple | For NACK-only based feedback, do we need to define high priority feedback? HARQ-ACK/NACK based feedback is more suitable for high priority feedback, the gNB could identify which UE send the NACK and could schedule the MBS service via PTM or PTP. |
| vivo | ok |
| Chengdu TD Tech, TD Tech | ok |

#### Proposal  3.3.1-3

For UE supporting NACK-only based HARQ-ACK feedback for multicast, the maximum values that are configured to UE as follows are unchanged:

- the maximum number of PUCCH resources sets,

- the maximum PUCCH resource in the first PUCCH resource set,

- the maximum number of UCI information bits for the first PUCCH resource set.

- Note:

  - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.

Collect views:

| Company | Comments |
| --- | --- |
| CATT | OK with it. |
| Nokia, NSB | Support. We would not need more than the current maximum numbers defined. If less is needed, that can be left for gNB implementation. |
| Samsung | The proposal does not seem applicable to NACK-only as there is no unicast equivalent. |
| ZTE | Only 1 bit can be transmitted in each PUCCH for NACK-only. In this sense, seems only 1 PUCCH resource set is needed. Thus, the first bullet is not needed. |
| Qualcomm | Ok in principle |
| Ericsson | This could be discussed later once we have defined NACK only formats/resources. |
| NTT DOCOMO | We would like to prioritize the discussion of Proposal 3.2.1-2 (more than one/two for NACK-only). Because how many resources are needed for NACK-only feedback will depend on how multiple HARQ-ACK bits are handled. |
| CMCC | Can be deprioritized after the discussion of issue 3.2.1-2 |
| LG | As we know, in PUCCH-config, especially for unicast, PUCCH format 0 and format 1 are only allowed for a resource in a first PUCCH resource set. PUCCH format2, format3 and format4 are only allowed for a resource in non-first PUCCH resource set. Thus, considering the current specification, if NACK only based HARQ-ACK is only configured in PUCCH-config, we would not need more than one PUCCH resource set, i.e. it seems straightforward that the maximum number of PUCCH resources set is ‘1’. Moreover in Proposal 3.1.1, for PUCCH format 0 or format 1 supported for NACK-only based HARQ-ACK feedback for multicast, only 1 HARQ-ACK bit is transmitted in one PUCCH. If this proposal is applied, the maximum number of UCI information bits for the first PUCCH resource set can be also ‘1’. #### Proposal 3.3.1-3 For UE supporting NACK-only based HARQ-ACK feedback for multicast, the maximum values that are configured to UE as follows are unchanged for PUCCH-config for multicast configuring NACK-only based HARQ-ACK feedback only: - FFS: the maximum number of PUCCH resources sets, - the maximum PUCCH resource in the first PUCCH resource set, - FFS: the maximum number of UCI information bits for the first PUCCH resource set. - Note: - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast. Meanwhile, if both NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK are configured in PUCCH-config, the number of PUCCH resources set can be more than 1 and only the first PUCCH resource set with PUCCH resource set ID = 0 in PUCCH-config for multicast can be configured for NACK only based HARQ-ACK. |
| OPPO | Can be discussed later when other details on NACK-only feedback are clear. |
| Apple | For NACK-only feedback, only first PUCCH resource set is needed. The intention of supporting multiple PUCCH resource sets is to switch to ACK/NACK based feedback? |
| vivo | Similar view with ZTE. |
| Chengdu TD Tech, TD Tech | More discussion is needed. |

### Round-2 (closed)

#### Proposal  3.3.2-1 (H)

For NACK-only based feedback for RRC_CONNECTED UE receiving multicast, UE can be optionally configured with a separate PUCCH-Config/PUCCH-ConfigurationList from the configuration for unicast.

- FFS: If not configured, PUCCH-Config/ PUCCH-ConfigurationList for unicast applies.

- Only the first PUCCH resource set in PUCCH-config can be configured for NACK only based feedback.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | I was intentionally separating the discussion from that for ACK/NACK based for multicast. I sees the comments how PUCCH-config for unicast is applied when PUCCH-config for multicast is not separately configured. At least in my understanding, if NACK-only feedback is not needed to be sent together with ACK/NACK based, NACK-only is still in use. Otherwise, UE can transform NACK-only into ACK/NACK. From network perspective, if the PUCCH-config of unicast can be used for NACK-only, probably the resources might be shared by some UEs, so it is up to gNB to schedule UE of the group to share this resources for NACK-only and it is also up to network to schedule UE not use the same resource when it is used for unicast feedback. This proposal is prioritized because it affect RRC parameters. When PUCCH-config is configured for NACK-only, how the PUCCH is used for the case of more than one NACK-only feedback is sent can be discussed later in RAN1. The proposal is updated per others’ comments. |
| Nokia, NSB | We support the previous version of the proposal with some additions for the relation of ACK / NACK and NACK-only PUCCH-Config. It is obvious, as we agreed on format 0 and format 1 PUCCH for NACK-only, that NACK-only resources are located in first PUCCH resource set. Therefore, we do not need the second sub-bullet above. We propose the following: For NACK-only based feedback for RRC_CONNECTED UE receiving multicast, UE can be optionally configured with a separate PUCCH-Config/PUCCH-ConfigurationList from the configuration for unicast. - FFS: If not configured, PUCCH-Config/ PUCCH-ConfigurationList for unicast applies. - FFS: whether both NACK-only based feedback and ACK/NACK based feedback share the same PUCCH-Config/PUCCH-ConfigurationList. - Only the first PUCCH resource set in PUCCH-config can be configured for NACK only based feedback. |
| Samsung | OK with the update from Nokia but should keep the FFS from the proposal. It is possible that the UE can be provided separate PUCCH-Config for NACK-only and ACK/NACK for multicast and, if not provided PUCCH-Config for NACK-only, the PUCCH-Config for ACK/NACK for multicast (not for unicast) applies. |
| LG | We support the last bullet point. We think that it is desirable to keep this to avoid potential different understanding. In addition, we think that same PUCCH-Config can be used for both NACK-only and ACK/NACK for multicast. We could avoid increased RRC message size by using the same PUCCH-Config for multicast with different feedback modes (or by using just one feedback mode). Considering the above comments from Samsung and Nokia, we propose to keep all of the followings in this proposal. - FFS: If not configured, PUCCH-Config/ PUCCH-ConfigurationList for unicast applies. - FFS: whether both NACK-only based feedback and ACK/NACK based feedback share the same PUCCH-Config/PUCCH-ConfigurationList. - Only the first PUCCH resource set in PUCCH-config can be configured for NACK only based feedback. |
| Lenovo, Motorola Mobility | The last sub-bullet is fine with us. Whether NACK-only based feedback and A/N based feedback are separately configured should be FFS. |
| vivo | We think both NACK-only based feedback and ACK/NACK based feedback can share the same PUCCH-Config/PUCCH-ConfigurationList. For the PUCCH-config for NACK only based feedback, the last bullet is ok. For the PUCCH-config for NACK only and ACK/NACK based feedback, the last bullet is not needed. |
| Chengdu TD Tech, TD Tech | More analysis is needed. |
| Qualcomm | Agree with Samsung to keep FFS for the first subbullet. |
| ZTE | Ok with the proposal for now. But as we discussed in the 1st round, we need to first discuss whether separate PUCCH-Config/PUCCH-ConfigurationList can be configured for NACK-only and ACK/NACK for multicast. If yes, then more discussion for the first FFS bullet is necessary. |
| Spreadtrum | Support. The last bullet seems to be common understanding. But it is fine to capture it here. Maybe we could change it as ‘Note: Only the first PUCCH resource set in PUCCH-config can be configured for NACK only based feedback.’. Also fine with FFS added by Nokia. |
| CMCC | Agree with Samsung to keep FFS. The issue of whether PUCCH-Config/ PUCCH-ConfigurationList for ACK/NACK based feedback and PUCCH-Config/ PUCCH-ConfigurationList for NACK-only based feedback are shared or separated should be discussed serepately. |
| OPPO | We suggest to agree on the main bullet only, and remove all the FFS, which have no RRC impact. We agree with Nokia that the last sub-bullet is not needed. |
| CATT | LG’s version is OK for us. |
| NTT DOCOMO | We support the main bullet and the 1st sub-bullet. Regarding the number of PUCCH resource sets for NACK-only based feedback, we would like to prioritize the discussion of Proposal 3.2.1-2 (more than one/two for NACK-only). Because multiple PUCCH resource sets may be needed to handle multiple HARQ-ACK bits. |
| MediaTek | Agree with majority view to keep the FFS in first sub-bullet. Regarding the last sub-bullet, Spreadtrum’s updated version is preferred. |
| Ericsson | We need to clarify the relationship between NACK-only PUCCH-Config and ACK/NACK PUCCH-Config. Can a UE be configured with both NACK-only PUCCH-Config and ACK/NACK PUCCH-Config? With also PUCCH-Config for unicast, does it mean UE can be configured with 3 PUCCH-Config, one for unicast, one for ACK/NACK multicast, one for NACK-only multicast? If we want to support DCI based switch between ACK/NACK based HARQ feedback and NACK-only HARQ feedback, it seems necessary to configure ACK/NACK PUCCH-Config and NACK-only PUCCH-config. In the current FFS wording, if NACK-only PUCCH-config not configured, unicast PUCCH-Config applies. However there could still be a configured ACK/NACK PUCCH configured. In that case, multicast should use the ACK/NACK PUCCH-config for multicast. |
| Apple | We are not sure the unicast PUCCH resource could be shared with NACK-only based feedback, due to the NACK-only PUCCH are shared among group of users in the CFR. The frequency domain location could be different for two type PUCCH resources. |

#### Proposal  3.3.2-2 (H)

For the separate PUCCH-ConfigurationList that is optionally configured to UE for NACK-only based HARQ-ACK feedback for multicast,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority feedback and high priority feedback, respectively.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | This proposal is not updated and is prioritized because it affects RRC parameters. Respond to LG’s comments: I am trying to separate the discussion from ACK/NACK and NACK-only mode switch so this discussion can be immune to other issues. Respond to Apple’s comments: The reason of supporting NACK-only is primarily for the sake of reducing PUCCH resource overhead and reducing UE power consumption in my recollection. From this sense, UE can also be configured with NACK-only for high priority, but your comments seems valid to me and it could be potentially a reason to have ACK/NACK and NACK-only dynamically switched? |
| Samsung | OK with the proposal. |
| vivo | ok |
| Chengdu TD Tech, TD Tech | Ok |
| ZTE | OK |
| CMCC | OK |
| OPPO | OK |
| CATT | OK |
| NTT DOCOMO | Support |
| Ericsson | Support. Still need be clear the relationship with ACK/NACK based PUCCH-ConfigurationList. Can they coexist. |

## Others

Submitted Proposals

(Ericsson) Proposal 2:

  - For NACK-only transmission of HARQ feedback for group scheduling, both semistatic and dynamic codebooks are supported

(Ericsson) Proposal 3:

  - The NACK-only codebook for a G-RNTI is constructed independently from codebooks constructed for other G-RNTIs that the UE may also receive and from the codebook that includes feedback for PDSCH scheduled by C-RNTI.

# HARQ-ACK feedback common

## Feedback priority

Submitted Proposals

“priority across M-cast and Unicast/other UCI”

(Nokia) Proposal 17:

  - The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK  for NACK-only based feedback.

(Nokia) Proposal 18:

  - For NACK-only based HARQ-ACK feedback for multicast, the prioritizing rule between the HARQ-ACK for multicast and SR / CSI can reuse Rel-16 prioritizing rule between the HARQ-ACK for unicast and SR / CSI.

“priority indication”

(Huawei) Proposal 6:

  - The priority index, if not included in the DCI formats, takes the default value that can be configured when G-RNTI is configured.

(CMCC) Proposal 14:

  - The priority index is optionally configured to be included in the DCI format 1_1 with CRC scrambled with G-RNTI. If not configured, the priority index is not included in the DCI format and is low priory by default.

(CMCC) Proposal 15:

  - The priority index is not included in the DCI format 1_0 with CRC scrambled with G-RNTI and is low priory by default.

(Intel) Proposal 10:

  - Scheduling of priority 1 MBS is possible using only DCI 1_1 or 1_2 (if supported).

(DOCOMO) Proposal 1:

  - Introduce a priority indicator field to DCI format 1_0 for multicast.

### Round-1 (closed)

“priority across Unicast/other UCI and M-cast”

FL’s Comments:

- Similar to what has been agreed for ACK/NACK based HARQ-ACK feedback, the priority comparison between  multicast (for NACK-only based) and unicast/other UCI needs to be discussed.

FL’s Proposal:

#### Proposal 4.1.1-1

The priority of multicast for NACK-only based feedback is the same as the priority of unicast for the same priority index of HARQ-ACK.

Collect views:

| Company | Comments |
| --- | --- |
| CATT | Agree. |
| Nokia, NSB | Support |
| Samsung | OK to simplify Rel-17 although the proposal is not necessarily valid. Unicast priorities can be different – e.g. priority 1 for a first UE is not same as priority 1 for a second UE. |
| ZTE | Ok with the proposal. |
| Qualcomm | ok |
| Ericsson | OK. |
| NTT DOCOMO | Support |
| CMCC | Support |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | OK |
| Spreadtrum | Fine |
| MediaTek | OK. |
| OPPO | OK |
| vivo | ok |
| Chengdu TD Tech, TD Tech | ok |

#### Proposal 4.1.1-2

For NACK-only based HARQ-ACK feedback for multicast, the prioritizing rule between the HARQ-ACK for multicast and SR / CSI can reuse Rel-16 prioritizing rule between the HARQ-ACK for unicast and SR / CSI.

Whether/how NACK-only is multiplexed with the HARQ-ACK for unicast and SR / CSI is discussed separately.

Collect views:

| Company | Comments |
| --- | --- |
| CATT | OK with it. |
| Nokia, NSB | Support |
| Samsung | The proposal is unclear. What Rel-16 prioritizing rule and for what? |
| ZTE | The proposal needs more clarification. Rel-16 doesn’t have NACK-only feedback, it is not clear how to reuse the prioritization rule. |
| Qualcomm | Main bullet seems ok. Is the subbullet related with Proposal 3.2.1-1? |
| Ericsson | OK. Subbullet should say “FFS” |
| NTT DOCOMO | Clarification is needed on whether prioritizing also applies to cases where NACK-only feedback is not sent. The presence of NACK-only feedback depends on the result of UE processing. If prioritizing is applied only when NACK-only feedback is sent, then gNB will have to blind detect the UCI. Also, how to handle multiple HARQ-ACK bits in NACK-only feedback will affect what is appropriate to do when they overlap. For example, if Alt3 in Proposal 3.2.1-2 is chosen, the multiplexing/prioritizing procedure will become more complex. Therefore, we prefer to prioritize the discussion of Proposal 3.2.1-2. |
| CMCC | Ok in principle |
| LG | We are generally fine with this proposal. In this proposal, we also want to clarify how multiplexing NACK only based HARQ-ACK and SR with PUCCH format 0 and/or 1. As we know, for unicast, UE procedure for multiplexing HARQ-ACK and SR with PUCCH format 0 and/or 1 has been specified in 38.213. However, it is not clear how to multiplex NACK-only based HARQ-ACK and SR for PUCCH format 0 and/or 1, especially because NACK-only based HARQ-ACK does not lead to PUCCH transmission in case of ACK. Thus, we propose to add FFS for multiplexing NACK-only based HARQ-ACK and SR for PUCCH format 0 and/or 1. #### Proposal 4.1.1-2 For NACK-only based HARQ-ACK feedback for multicast, the prioritizing rule between the HARQ-ACK for multicast and SR / CSI can reuse Rel-16 prioritizing rule between the HARQ-ACK for unicast and SR / CSI. FFS how to multiplex NACK-only based HARQ-ACK and SR for PUCCH format 0 or 1. Whether/how NACK-only is multiplexed with the HARQ-ACK for unicast and SR / CSI is discussed separately. |
| Spreadtrum | Fine in principle |
| OPPO | Support the proposal |
| Apple | The assumption for this proposal is more PUCCH resource sets are configured, even we only need the first PUCCH resource set for NACK-only feedback. The proposal can be discussed after the progress on PUCCH resource allocation. |
| vivo | Fine. (Assuming it is for NACK-only and other UCI with different priorities. The one with low priority is dropped and the other one is transmitted) |
| Chengdu TD Tech, TD Tech | The proposal needs clarifying |

“priority indication”

FL’s Comments:

Since DCI formats are discussed in AI 8.12.1 in general, the discussion for priority indication can be based on the agreement achieved wherein as follows:

Agreement:

For group-common PDCCH of Rel-17 MBS, support at least two DCI formats.

- DCI format 1_0 is used as the baseline for the first DCI format with CRC scrambled with G-RNTI.

- DCI format 1_1 or 1_2 is used as the baseline for the second DCI format with CRC scrambled with G-RNTI

  - FFS: Which of DCI format 1_1 or 1_2 is used as the baseline

- FFS: Details of the reuse (or not) of DCI format 1_0, 1_1 or 1_2 fields

FL’s Proposal:

#### Proposal 4.1.1-3

The priority index is,

- for the second DCI format with CRC scrambled with G-RNTI which takes DCI format 1_1 or 1_2 as the baseline, optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

  - Note: this is the same as what is specified for URLLC.

- for the first DCI format with CRC scrambled with G-RNTI which takes DCI format 1_0 as the baseline, Opt1 is supported from the following options:

  - Opt1: optionally configured to be included in the DCI format

  - Opt2: configured by RRC

  - Opt3: hard-coded in specification.

Collect views:

| Company | Comments |
| --- | --- |
| - CATT | - In RAN1#105-e meeting, it has been agreed in 8.12.1 that the existing fields in DCI format 1_0 and DCI format 1_1 can be reused as a baseline for the field of the first and second DCI, respectively. Until now the exact fields of both first and second DCI are still under discussion. We suggest postponing this proposal. |
| - Nokia, NSB | - Support |
| - Samsung | Support - There is no reason for different approaches in configurations between the first and second DCI formats and there is no need for different treatment. |
| - ZTE | For DCI format 1_0, a default priority is preferred (Opt3), which is aligned with existing mechanism. |
| - Qualcomm | Agree with first subbullet for the second DCI format. For the first DCI format, we think “low priority by default” is sufficient. |
| Ericsson | OK. But shouldn’t the option 2 and 3 be removed from the proposal, and only the text after “opt1” listed, since this is what is supported by the proposal? |
| NTT DOCOMO | Support. We prefer Opt1. Bits for unused fields (e.g., TPC command) in DCI format 1_0 should be utilized. |
| CMCC | Support second DCI format For first DCI format, we support Opt 3 as Rel-16 unicast design. |
| Spreadtrum | Generally fine |
| MediaTek | For the first DCI format, the existing default priority mechanism can be reused. |
| OPPO | Support the proposal |
| Apple | As supporting Opt1 is clearly mentioned in the proposal, Opt2 and Opt3 can be removed. |
| vivo | Same view with ZTE and CMCC. We prefer to reuse the existing mechanism. |
| Chengdu TD Tech, TD Tech | More discussion is needed |

### Round-2 (new)

We achieved the following agreement:

Agreement:

For UE supporting both unicast and multicast, the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList can be separately configured for multicast from that for unicast.

During the discussion of the proposal, there were questions/comments regarding the priority thing is that whether we mean different G-RNTIs by “priority” or we mean the priorities could be different for a single G-RNTI.

Before RAN2 agreed supporting multiple G-RNTI (May meeting), RAN1 agreed the following (at Apr. meeting),

Agreement:

Two priority indexes are introduced for multicast, with

- Index 0 meaning low priority and index 1 meaning high priority.

- Priority index can be included in DCI formats scheduling the group-common PDSCH.

  - FFS details for DCI formats.

- FFS: the priority comparison between multicast and unicast with the same priority index.

Hence, strictly the above RAN1 agreement should be applied to a single G-RNTI at least. Companies may think it is more nature different G-RNTIs have different priorities but at least we keep it is possible to indicate the “priority index” in the DCI scrambled by a single G-RNTI.

When UE is configured with multiple G-RNTI assuming a single priority for a single G-RNTI by network not configuring DCI to include the “priority index”, the question is whether all G-RNTIs have the same priority or can be different. The reasonable way should be configured per G-RNTI because different services corresponding to different G-RNTI may have different priorities naturally, so let’s try the following proposal because it also affects RRC parameters.

#### Proposal 4.1.2-3 (H)

When Priority index is not included in DCI scheduling the group-common PDSCH, the priority index is configured per G-RNTI.

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Not support. In our view, if priority index is not included in the DCI format, then it would mean low priority (as in current specs) and there is no need to change the current behavior. |
| Samsung | It is an optimization to save 1 bit and only when (a) a same G-RNTI cannot be used for different priorities and (b) the priority needs to be 1. |
| CMCC | We think default low priority as Rel-16 unicast can be applied to all G-RNTIs. |
| Lenovo, Motorola Mobility | One bit priority index can be always included in DCI format for multicast. At least discussion on default priority is a bit early. We can discuss the DCI field in AI 8.12.1. |
| LG | As FL said, different services corresponding to different G-RNTI may have different priorities. However, we would not need to configure the priority index per G-RNTI for DCI not including priority index. Low priority can be applied as default. |
| ZTE | The same handling as legacy UE is preferred, i.e., no priority index in DCI means lower priority. |
| Qualcomm | Similar view as majority companies. gNB can configure/indicate different priorities per G-RNTI by using second DCI format; otherwise, low priority can be assumed by default. |
| CATT | No OK with this proposal. In our understanding, if there is no priority index in DCI, the priority of the HARQ-ACK feedback corresponding to the scheduled PDSCH is low priority. |
| vivo | We prefer default low priority as Rel-16 unicast applied to all G-RNTIs. |
| NTT DOCOMO | We think it would be sufficient to reuse the existing rule, i.e., If a priority index is not provided, the priority index is 0. |
| OPPO | Share the view that legacy rule should be followed (low priority in default if there is no indication in DCI). |
| MediaTek | We support to reuse the current default rule. |

## Feedback mode selection

Submitted Proposals

“RRC configured”

(Samsung) Proposal 8:

  - A UE can be configured to use NACK-only reporting mode when the UE is configured ACK/NACK reporting mode and is indicated ‘disable’ of HARQ-ACK reporting by DCI.

(Qualcomm) Proposal 1:

  - For RRC_CONNECTED UEs, selection of ACK/NACK, NACK-only can be configured per HARQ process by unicast RRC signaling.

(CMCC) Proposal 16:

  - The HARQ-ACK feedback mode switching for multicast is RRC based.

(LGE) Proposal 9:

  - Select one of the following options to configure HARQ-ACK feedback scheme in PUCCH-config for multicast.

    - Option 1: Only one of NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK can be configured in a PUCCH-config for multicast.

      - FFS: Whether/how to support ACK/NACK based HARQ-ACK if NACK only based HARQ-ACK is configured in PUCCH-config for multicast

      - FFS: Whether to support more than one PUCCH-config for multicast

    - Option 2: Both NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK can be configured in PUCCH-config for multicast.

      - FFS: Whether NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK are can be jointly configured within a same PUCCH resource set or separately configured for different PUCCH resource sets.

(Intel) Proposal 1:

  - For RRC_CONNECTED UEs receiving multicast supporting both ACK/NACK based and NACK-only HARQ feedback:

    - ACK/NACK based feedback is used for delivery mode with PTP or PTM Scheme 2 (if supported)

    - NACK only feedback is used for delivery modes 1 and 2 with PTM Scheme 1

    - UEs within a group receiving multicast transmission can be configured with different HARQ feedback modes.

(Intel) Proposal 4:

  - For RRC_CONNECTED UEs, NR MBS supports both ACK/NACK based and NACK-only HARQ feedback. The configuration of ACK/NACK and NACK only mode can be done using the following options

    - Option 1: Semi-static RRC configuration of ACK/NACK or NACK only mode

    - Option 2: The configured PUCCH resource can contain additional indication that the UE is expected to transmit only NACK on the configured resource

    - Option 3: If UE has no dedicated PUCCH resource configuration, the UE uses cell-specific PUCCH resource and is expected to only transmit NACK

(DOCOMO) Proposal 4:

  - HARQ-ACK feedback scheme for multicast is configured per priority index.

“Dynamic switching”

(Nokia) Proposal 15:

  - As a third option to Alt. 2, a dynamic mechanism via MAC-CE is introduced to configure which HARQ-ACK feedback scheme is used by the UE.

(ZTE) Proposal 13:

  - Consider the following method to determine the feedback mechanism for the UE between ACK/NACK feedback and NACK-only feedback:

    - Both ACK/NACK PUCCH resources and NACK-only PUCCH resources for multicast are configured in the same PUCCH resource set.

    - PRI in DCI is used to indicate the feedback mechanism and PUCCH resources from the PUCCH resource set.

(Lenovo) Proposal 1:

  - Support dynamical HARQ-ACK feedback switch among Option 1 (Group NACK-only based feedback), Option 2 (UE-specific NACK-only based feedback), and Option 3 (UE-specific ACK/NACK-based feedback).

(MediaTek) Proposal 4:

  - Network can flexibly choose the HARQ-ACK mode and the HARQ feedback mode can be indicated dynamically by DCI field, e.g., “HARQ feedback option” field.

(Ericsson) Proposal 1:

  - We propose that the UEs can be dynamically reconfigured between NACK-only mode, ACK/NACK mode, and no feedback mode.

### Round-1

FL’s Comments

Now that we have agreed to support both ACK/NACK based and NACK-only based HARQ-ACK feedback for multicast, whether it is ACK/NACK or NACK-only based feedback for a given PUCCH transmission is a question needs to be discussed.

This issues was discussed in the last meeting which ended up no progress and how PUCCH is configured for ACK/NACK based or NACK-based needs to be discussed first. For example, whether UE can be configured with both PUCCH resources for ACK/NACK based and NACK-only based. If UE can be configured with both, it is possible for UE to dynamically switch between ACK/NACK based and NACK-only based upon network indication, e.g., DCI.

FL’s Proposal:

#### Proposal 4.2.1-1

For a given UE supporting both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast, it is up to network to configure PUCCH resources for ACK/NACK based and/or PUCCH resources for NACK-only based.

Collect views:

| Company | Comments |
| --- | --- |
| - CATT | - Not support. - In our opinion, for a given multicast transmission, a UE can be configured either PUCCH resources for ACK/NACK based feedback or PUCCH resources for NACK-only based feedback. And the feedback mode can be configured by RRC. |
| - Nokia, NSB | - Support |
| - Samsung | - The proposal is unclear. - If the understanding is same as CATT’s, we support CATT’s view. - If the understanding is that the NW can configure either ACK/NACK resources or NACK-only resources to a UE that can support both feedback modes, the proposal is OK but it seems unnecessary. |
| - ZTE | - Support the above proposal. From our perspective, network can configure both feedback modes simultaneously to the UE. |
| - Qualcomm | - Need further clarification. - Are we talking about the feedback modes for one multicast service or multiple multicast services? |
| Ericsson | OK. |
| NTT DOCOMO | Support |
| CMCC | Not support, similar view as CATT |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | Not support. Although it is up to gNB configuration, it should be clear whether the PUCCH resource is UE-specific or UE common for different modes. |
| MediaTek | Generally OK. |
| OPPO | Support |
| Apple | Ok with this proposal. |
| Chengdu TD Tech, TD Tech | ok |

#### Proposal 4.2.1-2

When UE is configured with both PUCCH resources for ACK/NACK based and PUCCH resources for NACK-only based HARQ-ACK feedback for multicast, the maximum values that are configured to UE as follows are unchanged:

- the maximum number of PUCCH resources sets,

- the maximum PUCCH resource in the first PUCCH resource set,

- the maximum number of UCI information bits for the first PUCCH resource set.

- Note:

  - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.

Collect views:

| Company | Comments |
| --- | --- |
| - CATT | - NOT support. - We do not see the benefit that UE is configured with both PUCCH resources for ACK/NACK based and PUCCH resources for NACK-only based. |
| - Nokia, NSB | - Support |
| - Samsung | - Not support – agree with CATT. Clarifications for NACK-only, for whether the “maximums” are joint or individual, and for whether there is a reason for a UE to be configured resources for both NACK-only and ACK/NACK based feedback are also needed. |
| - ZTE | - OK with the above proposal. |
| Ericsson | We believe this proposal should be postponed until more discussion has taken place on how to map NACK-only feedback to PUCCH resources.. |
| NTT DOCOMO | We would like to prioritize the discussion of Proposal 3.2.1-2 (more than one/two for NACK-only). Because how many resources are needed for NACK-only feedback will depend on how multiple HARQ-ACK bits are handled. |
| CMCC | Agree with Ericsson |
| LG | We are generally fine with this proposal. We could also clarify that both PUCCH resources for ACK/NACK based and PUCCH resources for NACK-only based HARQ-ACK feedback are configured in PUCCH-config for multicast in this proposal. #### Proposal 4.2.1-2 When UE is configured with both PUCCH resources for ACK/NACK based and PUCCH resources for NACK-only based HARQ-ACK feedback in a same PUCCH-config for multicast, the maximum values that are configured to UE as follows are unchanged for the PUCCH-config for multicast: - the maximum number of PUCCH resources sets, - the maximum PUCCH resource in the first PUCCH resource set, - the maximum number of UCI information bits for the first PUCCH resource set. - Note: - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast. |
| OPPO | Agree with the proposal. |
| Apple | The proposal 4.2.1-1 can be discussed first, then whether we need this proposal is clear. |
| vivo | Fine in principle. We think the first subbullet should be “the maximum number of PUCCH resources sets in each PUCCH-config” |
| Chengdu TD Tech, TD Tech | More discussion is needed. |

#### Proposal 4.2.1-3

When UE is configured with both PUCCH resources for ACK/NACK based and PUCCH resources for NACK-only based HARQ-ACK feedback for multicast, it is up to network to

- Configure the same or different PUCCH resource sets for ACK/NACK based and NACK-only based,

- Configure the same or different PUCCH resource in the same PUCCH resource set.

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | We need an agreement whether new optional PUCCH-config for ACK / NACK multicast is the same with the that of NACK-only multicast. For that, first proposals under 3.3.1 need to be agreed. If PUCCH-config for ACK / NACK multicast and NACK-only multicast are separate, anyway PUCCH resources and resource sets would be different, so we do not need the above proposal. Then, we can discuss what would happen if there are no separate PUCCH-configs for multicast than unicast. If PUCCH-config for ACK / NACK multicast and NACK-only multicast are the same, we can come back to above proposal. |
| Samsung | Further discussion is needed. Setting aside whether to support a UE being configured both above PUCCH resources, is PUCCH-Config separate for NACK-only and ACK/NACK. If yes, the proposal seems unnecessary. If no, further discussion is needed for having a same PUCCH-Config. |
| ZTE | Support the above proposal. |
| Qualcomm | Need to firstly discuss whether to support separate PUCCH-Config for NACK-only and ACK/NACK. |
| Ericsson | Motivation to configure the same PUCCH resource is not clear to us. Definitely should be possible to configure different PUCCH resource sets. |
| NTT DOCOMO | We would like to prioritize the discussion of Proposal 3.2.1-2 (more than one/two for NACK-only). Because what resources are needed for NACK-only feedback will depend on how multiple HARQ-ACK bits are handled. This will also affect whether PUCCH resources can be shared between ACK/NACK based and NACK-only based. |
| CMCC | Similar view as Samsung and Qualcomm to discuss whether the PUCCH-cofig for ACK/NACK and NACK-only is separated or not. |
| LG | We are fine with this proposal. |
| OPPO | Support the proposal. |
| vivo | Similar view with Ericsson. Different PUCCH resources should be configured for ACK/NACK and NACK-only. |
| Chengdu TD Tech, TD Tech | We don’t understand the following sentence. The same PUCCH set with a different PUCCH ? How to configure a same PUCCH set with a different PUCCH? - Configure the same or different PUCCH resource in the same PUCCH resource set. |

## Enable/disable HARQ-ACK feedback

Submitted Proposals

“Confirm WA”

(Huawei) Proposal 14:

  - Confirm the working assumption regarding the enabling/disabling HARQ-ACK feedback.

(ETRI) Proposal 2:

  - Confirm the working assumption:

(FUTUREWEI) Proposal 5:

  - Confirm the Working Assumption on the enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH including the following:

    - This WA is extended to apply to NACK-only based feedback.

    - This WA is extended to the enabling/disabling ACK/NACK based HARQ-ACK feedback and NACK-only retransmission.

    - The default UE behavior, prior to receiving of RRC signaling, is that ether ACK/NACK or NACK-only feedback is enabled.

(CMCC)  Proposal 17:

  - Confirm the working assumption:

(MediaTek) Proposal 6:

  - Confirming the working assumption:

(DOCOMO) Proposal 2:

  - Confirm the working assumption on HARQ feedback enabling/disabling.

“RRC signalling”

(vivo) Proposal 1:

  - For indicating HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH

    - RRC signaling configures the HARQ-ACK feedback option indication function of group-common DCI indicating the HARQ-ACK feedback option

      - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) HARQ-ACK options among ACK/NACK, NACK only and disabled feedback.

(vivo) Proposal 2:

  - HARQ-ACK feedback option for multicast for RRC_CONNECTED UEs should be configured/indicated per g-RNTI.

    - When RRC disables HARQ-ACK feedback option indication function of group-common DCI, for RRC_CONNECTED UE receiving multicast, the HARQ-ACK feedback option is configured per g-RNTI.

    - When RRC enables HARQ-ACK feedback option indication function of group-common DCI, for RRC_CONNECTED UE receiving multicast, the HARQ-ACK feedback option is implicitly indicated by g-RNTI.

- (Nokia) Proposal 37:

  - RRC-based enabling / disabling of HARQ-ACK feedback is used for MBS.

- (Nokia) Proposal 39:

  - Enabling / disabling of HARQ-ACK feedback is done per multicast / broadcast service.

- (Samsung) Proposal 3:

  - Enabling/disabling HARQ-ACK reporting by RRC is per G-RNTI and can apply for any HARQ-ACK reporting mode. Enabling/disabling HARQ-ACK reporting by DCI does not apply to NACK-only reporting.

- (CATT) Proposal 4:

  - State 1(ACK/NACK enabling) and State 3 (enabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback ACK/NACK) cannot be configured simultaneously.

- (OPPO) Proposal 9:

  - For HARQ feedback enabling/disabling of dynamic group-common PDSCH:

    - RRC signalling for enabling/disabling the function of group-common DCI or enabling/disabling HARQ-ACK feedback is configured per G-RNTI.

    - 2 bits in group-common DCI are used to indicate whether/which HARQ feedback is enabled;

- (ETRI) Proposal 1:

  - Support at least RRC configuration for UEs to indicate HARQ-ACK feedback scheme to use.

- (Qualcomm) Proposal 6:

  - For RRC_CONNECTED UEs, HARQ-ACK feedback can be enabled/disabled per HARQ process by unicast RRC signaling.

- (DOCOMO) Proposal 3:

  - Introduce an RRC parameter with 3 states {enabled, disabled, dynamic}.

    - When ‘enabled’, HARQ-ACK is enabled.

    - When ‘disabled’, HARQ-ACK is disabled.

    - When ‘dynamic’, if PRI is zero and PDSCH-to-HARQ feedback timing indicator is zero, HARQ-ACK feedback is disabled. Otherwise, HARQ-ACK feedback is enabled.

- (Ericsson) Proposal 8:

  - In a group of UEs configured with a DCI including a DCI-based enable/disable HARQ feedback command, a UE may be additionally configured with a UE specific RRC configuration to notify the UE whether the DCI command applies to the UE. If the UE is configured so that the DCI command does not apply, the UE follow the RRC configuration for the enabling/disabling of HARQ feedback and enabling/disabling NACK-only.

“DCI signalling”

(Spreadtrum) Proposal 6:

  - For both dynamic group-common PDSCH and SPS group-common PDSCH, if RRC configures enabling the function of group-common DCI indicating the enabling/disabling HARQ-ACK feedback, 1-2 bit DCI field can be applied to indicate the feedback back mode.

(CATT) Proposal 1:

  - For DCI indicating the HARQ ACK enabling/disabling, Alt 3 (A special state of an existing DCI field) and Alt 4(Implicit indication) can be supported.

(Lenovo) Proposal 2:

  - For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast, RRC signaling configures a non-numerical value in the K1 set and PDSCH-to-HARQ_timing indicator in the DCI indicates a numerical value or the non-numerical value for enabling or disabling the HARQ-ACK feedback.

(Lenovo) Proposal 5:

  - HARQ-ACK feedback enabling/disabling based on numerical k1 value or non-numerical k1 should be followed by UE.

(OPPO) Proposal 9:

  - For HARQ feedback enabling/disabling of dynamic group-common PDSCH:

    - RRC signalling for enabling/disabling the function of group-common DCI or enabling/disabling HARQ-ACK feedback is configured per G-RNTI.

    - 2 bits in group-common DCI are used to indicate whether/which HARQ feedback is enabled;

- (DOCOMO) Proposal 3:

  - Introduce an RRC parameter with 3 states {enabled, disabled, dynamic}.

    - When ‘enabled’, HARQ-ACK is enabled.

    - When ‘disabled’, HARQ-ACK is disabled.

    - When ‘dynamic’, if PRI is zero and PDSCH-to-HARQ feedback timing indicator is zero, HARQ-ACK feedback is disabled. Otherwise, HARQ-ACK feedback is enabled.

- (Ericsson) Proposal 5:

  - The enable/disable indicator in the PDCCHs scrambled by same RNTI which point to the same PUCCH for HARQ feedback must be the same.

- (Ericsson) Proposal 7:

  - DCI can be used to indicate not only ACK/NACK HARQ feedback or no HARQ feedback, but also NACK-only HARQ feedback.

    - The bit field for enable/disable/NACK only can be RRC configured with a configurable number of bits:

      - A two bits field can be configured in DCI to indicate ACK/NACK feedback, NACK-only feedback, no HARQ feedback.

      - A 1-bit field can be configured in DCI with the 0 and 1 value of the bit corresponding to 2 configured values selected from the 3 possible values of ACK/NACK enabled, disabled, or NACK only.

- “Applicability to NACK-only”

(Samsung) Proposal 3:

  - Enabling/disabling HARQ-ACK reporting by RRC is per G-RNTI and can apply for any HARQ-ACK reporting mode. Enabling/disabling HARQ-ACK reporting by DCI does not apply to NACK-only reporting.

(CATT) Proposal 2:

  - The enabling/disabling ACK/NACK-based HARQ-ACK feedback mechanism can be used for NACK-only based HARQ-ACK feedback.

(ETRI) Proposal 4:

  - Support DCI indication for enabling/disabling NACK-only based HARQ-ACK feedback.

(FUTUREWEI) Proposal 5:

  - Confirm the Working Assumption on the enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH including the following:

    - This WA is extended to apply to NACK-only based feedback.

    - This WA is extended to the enabling/disabling ACK/NACK based HARQ-ACK feedback and NACK-only retransmission.

    - The default UE behavior, prior to receiving of RRC signaling, is that ether ACK/NACK or NACK-only feedback is enabled.

(Lenovo) Proposal 3:

  - HARQ-ACK feedback enabling/disabling based on numerical k1 value or non-numerical k1 can be straightforwardly extended to NACK-only based feedback and multiple G-RNTI cases.

(MediaTek) Proposal 5:

  - The ACK/NACK-based enabling/disabling HARQ-ACK feedback mechanism can be extended to apply to NACK-only based feedback enabling/disabling.

(Intel) Proposal 3:

  - For NACK-only feedback RRC configures ON/OFF with no DCI based indication.

(Ericsson) Proposal 6:

  - Via RRC configuration each UE can be individually configured, for each G-RNTI, to use HARQ ACK/NACK, NACK-only or no HARQ feedback

(Ericsson) Proposal 7:

  - DCI can be used to indicate not only ACK/NACK HARQ feedback or no HARQ feedback, but also NACK-only HARQ feedback.

    - The bit field for enable/disable/NACK only can be RRC configured with a configurable number of bits:

      - A two bits field can be configured in DCI to indicate ACK/NACK feedback, NACK-only feedback, no HARQ feedback.

      - A 1-bit field can be configured in DCI with the 0 and 1 value of the bit corresponding to 2 configured values selected from the 3 possible values of ACK/NACK enabled, disabled, or NACK only.

- “Applicability to HARQ-ACK codebook”

(CATT) Proposal 6:

  - When Type-1 HARQ-ACK feedback is configured, State 3 (The function of group-common DCI based indication) is not supported.

(Qualcomm) Proposal 7:

  - For RRC_CONNECTED UEs, dynamic enabling/disabling function of GC-DCI is subject to UE capability.

    - Dynamic enabling/disabling in GC-DCI is applicable to Type-2 HARQ-ACK codebook, FFS Type-1 codebook

(Ericsson) Proposal 4:

  - The UE is only expected to follow the enable / disable indicator for HARQ feedback in PDCCH when the HARQ codebook type for the PUCCH resource is of type 2.

- “Applicability to retransmission”

(CATT) Proposal 3:

  - The indication of the enabling/disabling for retransmission depends on the RRC/DCI signaling same as initial transmission.

(Nokia) Proposal 40:

  - If HARQ-ACK feedback is enabled / disabled for a service, the UE provides / do not provide HARQ-ACK feedback also upon retransmissions.

(FUTUREWEI) Proposal 5:

  - Confirm the Working Assumption on the enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH including the following:

    - This WA is extended to apply to NACK-only based feedback.

    - This WA is extended to the enabling/disabling ACK/NACK based HARQ-ACK feedback and NACK-only retransmission.

    - The default UE behavior, prior to receiving of RRC signaling, is that ether ACK/NACK or NACK-only feedback is enabled.

(Lenovo) Proposal 4:

  - HARQ-ACK feedback enabling/disabling based on numerical k1 value or non-numerical k1 can be supported for multicast retransmission.

- “UE reaction”

(CATT) Proposal 5:

  - When only State 2 (RRC disabling) and State 3 (The function of group-common DCI based indication) are configured simultaneously, UE is allowed not to react to the DCI signaling but instead follow UE-specific RRC configuration for HARQ feedback.

- “Applicability to SPS GC-PDSCH”

(Spreadtrum) Proposal 6:

  - For both dynamic group-common PDSCH and SPS group-common PDSCH, if RRC configures enabling the function of group-common DCI indicating the enabling/disabling HARQ-ACK feedback, 1-2 bit DCI field can be applied to indicate the feedback back mode.

(Lenovo) Proposal 6:

  - HARQ-ACK feedback enabling/disabling based on numerical k1 value or non-numerical k1 can be supported for SPS group-common PDSCH transmission.

- “UE behavior”

(FUTUREWEI) Proposal 5:

  - Confirm the Working Assumption on the enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH including the following:

    - This WA is extended to apply to NACK-only based feedback.

    - This WA is extended to the enabling/disabling ACK/NACK based HARQ-ACK feedback and NACK-only retransmission.

    - The default UE behavior, prior to receiving of RRC signaling, is that ether ACK/NACK or NACK-only feedback is enabled.

(ETRI) Proposal 5:

  - The default operation when RRC configuration on HARQ-ACK feedback scheme is not configured is HARQ-ACK feedback off.

(MediaTek) Proposal 7:

  - The same HARQ-ACK feedback behaviour for all the UEs in the same MBS group are required.

(MediaTek) Proposal 8:

  - Disabling the feedback is as the default mode if the RRC signalling doesn’t configure the HARQ feedback function.

- “MAC-CE based”

(Nokia) Proposal 38:

  - If use cases that require dynamic enabling / disabling are found, MAC-CE is preferred.

(ZTE) Proposal 14:

  - Regarding enabling/disabling HARQ-ACK feedback for MBS, RAN1 confirms the working assumption reached in RAN1#105-e meeting and supports MAC-CE based enabling/disabling indication.

### Round-1 (closed)

FL’s comments:

Regarding enabling/disabling HARQ-ACK feedback, last meeting achieved the following working assumption:

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

6 companies explicitly propose to confirm the WA and no proposal from all companies objects confirming the WA.

Each of FFS from the list was discussed by some companies. From the discussion, at least this mechanism is conducted per G-RNTI seems agreeable.

The RRC signaling can be up to RAN2 and current RAN1 agreement regarding RRC signaling seems sufficient. Whether introducing a specific field or reusing the existing field of DCI for such enabling/disabling indication can be discussed later after discussion on DCI formats are more mature.

Whether the mechanism is applicable to NACK-only depends on how ACK/NACK-based or NACK-only HARQ-ACK feedback mode is configured/indicated to UE in general. We can proceed with this FFS after the issue how ACK/NACK-based or NACK-only HARQ-ACK feedback mode is configured/indicated to UE is clear.

The discussion on other FFS can be proceeded after more companies are interested.

FL’s Proposal:

#### Proposal 4.3.1-1

Confirm the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback.

Collect views:

| Company | Comments |
| --- | --- |
| - CATT | - Agree. |
| - Nokia, NSB | - Support |
| - Samsung | - Support |
| - ZTE | - OK |
| - Qualcomm | - From UE implementation point of view, we think whether to support GC-DCI enabling/disabling function should be subject to UE capability. Modification on WA is needed. Working assumption: For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH: - RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback subject to UE capability. - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling. - FFS details on RRC signaling and group-common DCI indicating. - … |
| Ericsson | OK |
| NTT DOCOMO | Support |
| CMCC | Support |
| Lenovo, Motorola Mobility | OK |
| LG | We are fine with this proposal. |
| Spreadtrum | Support, also fine with the updated version from Qualcomm |
| MediaTek | Support. |
| OPPO | OK |
| vivo | We prefer to confirm the WA after further discussion. We think NACK only and multiple g-RNTI should be considered when configure/indicate HARQ-ACK feedback mode. |
| Chengdu TD Tech, TD Tech | ok |

#### Proposal 4.3.1-2

RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI.

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support |
| Samsung | Support Need for HARQ-ACK can depend on number of UEs configured per G-RNTI. |
| ZTE | OK. We may need to further discuss whether to configure it per UE. #### Proposal 4.3.1-2 RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI. FFS: whether it can be per UE. |
| Qualcomm | We prefer to discuss if GC-DCI function is not enabled first. #### Proposal 4.3.1-2 If RRC signaling does not configure the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by RRC signaling - FFS how to use RRC signaling to configure the function of group-common DCI based indication |
| Ericsson | OK |
| NTT DOCOMO | Support. The required QoS level can be different for each multicast service, so the need for HARQ-ACK feedback can also be different for each service. Therefore, it is better to configure the enabling/disabling function per G-RNTI |
| CMCC | ok |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | OK |
| Spreadtrum | Fine |
| MediaTek | Support |
| OPPO | OK |
| Apple | We consider RRC signlaing enabling/disabling the HARQ-ACK feedback is enough. The benefit of dynamic enabling/disabling feedback via DCI is not clear, the PUCCH resources are reserved there whatever resources are used or not. |
| vivo | We think the configuration of RRC signaling configures the enabling/ disabling ACK/NACK based HARQ-ACK feedback is configured per UE per G-RNTI. For a certain g-RNTI, there is no clear motivation to change the HARQ-ACK feedback options dynamically. So, we prefer to modify the proposal as: RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per UE per G-RNTI. |
| Chengdu TD Tech, TD Tech | ok |

#### Proposal 4.3.1-3

For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, down-select from the following alternatives:

- Alt 1: Reuse the existing field in the group-common DCI.

- Alt 2: Introduce a new field in the group-common DCI.

Collect views:

| Company | Comments |
| --- | --- |
| - CATT | - Support Alt1. - Alt2 needs extra bit in DCI which will increase the DCI payload size. Since the group-common DCI targets a group UEs, whether this new field is needed in DCI for UEs which enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling should be further studied. This will bring extra work and high complexity on protocol design. Therefore, Alt 2 cannot be supported. - For Alt1, since the existing field e.g. PDSCH-to-HARQ feedback timing (k1) field and PUCCH resource indicator (PRI) field will be used to indicate the valid timing for PUCCH and the PUCCH resource when implementing the HARQ-ACK feedback, so a special state of the k1 field or PRI field can be used to indicate the HARQ-ACK feedback enabling/disabling. |
| - Nokia, NSB | - Support Alt 1. There are fields in the conventional DCI that are not needed and can be reused for the group-common DCI. |
| - Samsung | Support Alt. 1 - No need for new field given that several existing fields will not be used and that NR-U uses indication based on existing field. |
| - ZTE | Alt.1 is not clear, more clarification is needed. For example, how to reuse the existing field. Note that, we will have a new RNTI for DCI format, theoretically, all the fields are new, no such thing like reusing the existing field. |
| - Qualcomm | Fine to further study |
| Ericsson | Could you kindly explain Alt1, because we think there is no "existing field"for indicating enabled/disabled in group-common DCI, not even in unicast DCI. Should it say “repurpose”? otherwise, Alt2 is in our view the obvious remaining solution. |
| NTT DOCOMO | Support. We prefer Alt 1. Existing fields should be reused to minimize the DCI size. There are mechanisms to switch HARQ-ACK feedback enable/disable using existing DCI fields in NR-U and SL, so it is better to reuse either. |
| CMCC | Prefer Alt 1 |
| LG | We are fine with this proposal. |
| Spreadtrum | Fine |
| MediaTek | Support Alt.2. We are confused with Alt.1, how to understanding the “existing filed” for group-common DCI? We have agreed some fields in existing DCI format with C-RNTI can be reused for group common DCI scrambled by G-RNTI. However, not all field in legacy DCI can be used for MBS DCI, e.g., ‘Identifier for DCI formats’. We suggest these fields can be replaced by some new adding fields, which will not affect the DCI performance. |
| OPPO | Fine with the proposal, further study is needed for down selection. |
| vivo | We think for a certain g-RNTI, there is no clear motivation to change the HARQ-ACK feedback options dynamically. we think the reasonable way is to enable/disable HARQ-ACK feedback per UE per g-RNTI by RRC configuration, then UE determine the HARQ-ACK mode for a GC-PDSCH based on its associated g-RNTI. |
| Chengdu TD Tech, TD Tech | More discussion is needed |

### Round-2 (closed)

#### Proposal 4.3.2-1 (H)

Confirm the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback with the update in the first sub-bullet as follows:

RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback subject to UE capability.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | This is prioritized because it affects RRC parameters. The proposal is updated to incorporate QC’s comment. |
| Nokia, NSB | Ok. |
| Samsung | OK |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | We are OK with the proposal if it can be more generic to cover both A/N based feedback and NACK-only based feedback. RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling HARQ-ACK feedback subject to UE capability. |
| vivo | We cann’t accept the proposal. We think DCI indicating enabling/disabling via g-RNTI is enough. There is no need to enable/disable the HARQ-ACK feedback dynamically for a certain service, i.e. a certain g-RNTI. If it is not the common understanding, we can’t support the proposal. The motivation to support dynamic indication of enabling /disabling ACK/NACK based HARQ-ACK feedback for a certain g-RNTI is not clear to us. DCI based enabling / disabling would incur other issues, such as how to indicate, if new field is introduced, this filed would need fix in the group-common DCI or addition of RRC signaling is needed to tell a group of UE whether this filed exist or not, Since the RRC signaling configuring the enabling/ disabling function is UE-specific, but DCI size and fields needs to be common among a group of UEs. |
| Chengdu TD Tech, TD Tech | “subject to UE capability”？In the group, some UE can do while the other UEs can’t do? The proposal needs clarifying ” |
| Qualcomm | We think the function of dynamic enabling/disabling should not be mandatory for a UE supporting multicast. If the UE incapable of this function, gNB will not configure the function. Anyway it is unicast RRC configured. |
| ZTE | The newly added “subject to UE capability” is not clear. It is not clear whether “DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback” is subject to UE capability or “RRC signaling configures the enabling/ disabling function” is subject to UE capability. From our perspective, at least this part “enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling” doesn’t need to have an explicit UE capability. |
| Spreadtrum | Support. Re Chengdu TD Tech, for the listed case, the other UEs not supporting can ignore the field in DCI indicating enabling/disabling HARQ-ACK feedback. |
| CMCC | OK |
| OPPO | OK |
| CATT | We are not clear that the reason that why the ‘subject to UE capability’ is added. If it means enable/disable the HARQ-ACK feedback can be dynamically changed for a certain service, then we share the same vivo’s views. The motivation/benefit of supporting dynamic indication is not clear for us so far. Moreover, if it as Qualcomm mentioned, what kinds of UE that cannot support the function, the use case is not clear for us. |
| NTT DOCOMO | Support. The following agreements were made in RAN2#114-e. One-to-one mapping between G-RNTI and MBS session is supported in NR MBS. Other mappings FFS Multiple MBS QoS flows corresponding to the same MBS session can be mapped to one or more than one MBS radio bearers. Based on these agreements, TBs with different QoS levels can be sent with the same G-RNTI. The need for HARQ-ACK feedback will be high for TBs that require high reliability. On the other hand, TBs that do not require high reliability may not need HARQ-ACK feedback. Therefore, dynamic switching by group-common DCI will be useful. |
| MediaTek | We are confused with the new wording. If UE report doesn’t support this capability, which means RRC configure the function and gNB will configure the DCI enable/disable indicator or RRC will not configure the function? |

#### Proposal 4.3.2-2 (H)

RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | This is prioritized because it affects RRC parameters. Regarding ZTE’s comment whether it can be per UE: I guess whether it can be per UE depends on reusing an existing field or adding a new field in DCI as discussed in Proposal 4.3.1-3 that can be further discussed. Respond to QC’s comment: I created a new proposal as 4.3.2-4 (H) to discuss the case you cared about also because it also affects RRC parameters. |
| Nokia, NSB | Enabling / disabling HARQ-ACK feedback should be per UE (and this is what we interpret from the RRC signaling option of WA), because of several reasons we have been discussing since a couple of meetings. This is the reason why most companies wanted RRC configuration option, but not group-common DCI based option, since the RRC signaling is UE-specific. |
| Samsung | OK with the proposal (assuming the RRC signaling is UE-specific). |
| LG | We are fine with this proposal. |
| vivo | As comment in the 1st round, we don’t see the need to enable/disable ACK/NACK feedback dynamically for a certain g-RNTI. We think the proposal should be modifies as: RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per UE per G-RNTI. With this proposal, when a UE receive a PDSCH with certain g-RNTI, UE determine the ACK/NACK feedback for this PDSCH is enabled or disabled by its associated g-RNTI. |
| Chengdu TD Tech, TD Tech | Ok |
| Qualcomm | We prefer to further discuss this proposal. Other option can be to configure the function per HARQ process ID. It is up to gNB to use the HAQR process ID for same or different G-RNTIs in different time. |
| ZTE | Considering that the RRC signaling is per UE and group common DCI is per G-RNTI, isn’t the indicating per UE per G-RNTI? If that is the correct understanding, we would like to make it clear. |
| CMCC | Ok. In our understanding, since the RRC signalling is UE-specific, for a same G-RNTI, the function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback can be different for different UEs . |
| Lenovo, Motorola Mobility | We are OK with the proposal if it can be more generic to cover both A/N based feedback and NACK-only based feedback. RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling HARQ-ACK feedback. |
| OPPO | Support the proposal |
| CATT | OK |
| NTT DOCOMO | Generally support. We would like to change the wording for clarification. RRC signaling configures tThe enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI. |
| Ericsson | Support. If the function is not configured, we assume the UE consider the bit as “reserved” (i.e. the UE will not follow the DCI bit, but it is still there) |
| Apple | We support vivo’s update. |

#### Proposal 4.3.2-4 (H)

If RRC signaling does not configure the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by RRC signaling

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | This is prioritized because it affects RRC parameters. |
| Nokia, NSB | Support |
| Samsung | OK with the proposal (assuming the RRC signaling is UE-specific). |
| LG | We are fine with this proposal. |
| vivo | The proposal is not needed if the Proposal 4.3.2-2 is changed as RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per UE per G-RNTI. |
| Chengdu TD Tech, TD Tech | Ok |
| Qualcomm | ok |
| ZTE | Similar comment, isn’t it per UE per G-RNTI? If yes, we would like to make it clear. |
| CMCC | OK |
| Lenovo, Motorola Mobility | Not quite clear to us. Does it mean the first “RRC signaling” is group common RRC signaling and the second “RRC signaling” is UE-specific RRC signaling? |
| OPPO | support |
| CATT | OK |
| NTT DOCOMO | Support |
| MediaTek | Ok |
| Ericsson | We support in principle, but the proposal is complex to understand. Proposed formulation: The UE can be configured to use ACK/NACK, NACK-only, or no HARQ feedback by RRC configuration. Additionally, the configuration can be overridden by the HARQ enabling/disabling function in group common DCI If the UE is configured to follow the enabling/disabling function in group common DCI. |

## Retransmission

Submitted Proposals

“CBG-based retransmission”

(Nokia) Proposal 1:

  - CBG-based re-transmissions are not supported for PTM re-transmissions.

(CATT) Proposal 20:

  - CBG based PTP retransmission can be supported in multicast retransmission when a UE is configured with CBG transmission for unicast.

(Intel) Proposal 12:

  - For ACK/NACK based HARQ operation, support UE specific CBG based retransmission. Other advanced retransmission schemes are not precluded.

- “retransmission vs. Codebook”

(LGE) Proposal 6:

  - HARQ-ACK to PTP retransmission is treated as HARQ-ACK to normal unicast transmission for construction of HARQ-ACK codebook.

- “retransmission vs. HARQ-ACK feedback mode”

(MediaTek) Proposal 10:

  - PTM scheme 1 or PTP retransmission is supported for ACK/NACK based HARQ feedback mechanism.

(MediaTek) Proposal 11:

  - PTM scheme 1 retransmission is supported for NACK-only based HARQ feedback mechanism.

(Convida) Proposal 1:

  - PTM transmission scheme 2 should be supported for MBS retransmission.

### Round-1 (closed)

FL’s Comments

Whether UE supports simultaneously PTM scheme 1 an PTP for retransmission is currently discussed in AI 8.12.1 and whether UE can dynamic switch between ACK/NACK based and NACK-only based HARQ-ACK feedback is also to be discussed. The issues related in this section can be discussed later.

| Company | Comments |
| --- | --- |
| Samsung | OK |
| Ericsson | OK |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | OK |

# HARQ-ACK feedback for multicast SPS

Submitted Proposals

- “NACK-only for SPS activation/deactivation”

(Huawei) Proposal 12:

  - For multicast SPS activation/deactivation, NACK-only based HARQ-ACK feedback is not supported.

(vivo) Proposal 13:

  - NACK-only based HARQ-ACK feedback is not supported for multicast SPS activation/deactivation.

(ZTE) Proposal 15:

  - Regarding HARQ-ACK feedback for SPS-based MBS transmission, if NACK-only feedback is configured:

    - The NACK-only feedback mode applies to ‘PDSCH without scheduling PDCCH’ only

    - The feedback mode for ‘PDSCH with scheduling PDCCH’ can be,

- Option 1: fixed to ACK/NACK feedback

- Option 2: follow the configuration of feedback mode for DG-PDSCH

    - FFS: the feedback mode for deactivation PDCCH

(ETRI) Proposal 3:

  - Support NACK-only based HARQ-ACK feedback for multicast PDSCH with PDCCH scheduling.

(NEC) Proposal 3:

  - Support NACK-only based HARQ-ACK feedback for SPS activation/deactivation.

(Lenovo) Proposal 12:

  - For group-common SPS configuration, a UE-specific PUCCH resource is configured for each UE to transmit ACK upon reception of activation/deactivation DCI.

(Lenovo) Proposal 13:

  - For group-common SPS configuration, the UE-specific PUCCH resource for confirming reception of activation/deactivation DCI is also used for the UE to transmit ACK for the SPS PDSCH.

(Qualcomm) Proposal 8:

  - Support UE-specific ACK/NACK-based feedback for activation/release for SPS GC-PDSCH

  - Support configuration of no feedback, ACK/NACK-based or NACK-based feedback for SPS GC-PDSCH without PDCCH

(Google) Proposal 2:

  - For SPS activation with NACK-only based HARQ feedback, consider following options.

    - Option 1: MBS SPS is activated by a group common DCI. Handling the missing of group common DCI is based on network implementation.

    - Option 2: SPS is activated by RRC signaling (i.e. similar to the Type-1 configured grant)

    - Option 3: UE reports ACK/NACK based HARQ feedback to the SPS activation command in a UE specific PUCCH resource, and reports NACK-only base HARQ feedback to the SPS PDSCH transmission in group common PUCCH resource.

      - FFS: details

(Google) Proposal 3:

  - For SPS deactivation with NACK-only based HARQ feedback, consider following options

    - Option 1: MBS SPS can be deactivated by a group common DCI, handling the missing of the group common DCI missing is based on network implementation

    - Option 2: Network provides the MBS SPS transmission length information in the activating DCI. UE deactivates the SPS according to the indicated length.

    - Options 3: Apply ACK/NACK based HARQ feedback to the SPS deactivation command, and NACK-only base HARQ feedback to the SPS PDSCH transmission.

      - FFS: details

(CMCC) Proposal 18:

  - Support ACK/NACK based HARQ-ACK feedback for SPS activation/deactivation.

(LGE) Proposal 14:

  - For group common SPS, UE specific confirmation to group common SPS (de-)activation can be supported by PUCCH A/N.

    - UE specific PUCCH resource is allocated by DCI indicating SPS (de-)activation.

(MediaTek) Proposal 9:

  - MBS SPS activation/deactivation’s feedback mechanism only support ACK/NACK based HARQ feedback mode.

(Convida) Proposal 6:

  - NACK-only based HARQ-ACK feedback should be supported for MBS SPS PDSCH with PDCCH scheduling.

(Convida) Proposal 7:

  - Support HARQ-ACK feedback for MBS SPS activation DCI and deactivation DCI..

“HARQ-ACK option for SPS multicast”

(vivo) Proposal 3:

  - For MBS for RRC_CONNECTED UEs, HARQ-ACK feedback option for SPS group-common PDSCH is configured per SPS PDSCH configuration.

(ZTE) Proposal 15:

  - Regarding HARQ-ACK feedback for SPS-based MBS transmission, if NACK-only feedback is configured:

    - The NACK-only feedback mode applies to ‘PDSCH without scheduling PDCCH’ only

    - The feedback mode for ‘PDSCH with scheduling PDCCH’ can be,

- Option 1: fixed to ACK/NACK feedback

- Option 2: follow the configuration of feedback mode for DG-PDSCH

    - FFS: the feedback mode for deactivation PDCCH

(Samsung) Proposal 10:

  - A gNB can optionally provide separate configurations for the HARQ-ACK reporting mode for DCI activation/deactivation of SPS PDSCH and for SPS PDSCH receptions.

(CATT) Proposal 7:

  - For SPS group-common PDSCH, HARQ-ACK feedback for the first active SPS PDSCH is always enabled regardless of HARQ-ACK enabling/disabling configuration.

(CATT) Proposal 8:

  - If HARQ-ACK enabling/disabling is determined by DCI, the HARQ-ACK feedback of the PDSCH transmission without DCI (Case3) can be based on the DCI indication of the activation command.

(FUTUREWEI) Proposal 2:

  - For support of HARQ-ACK feedback for SPS multicast, use SPS-config configuration as a baseline for SPS multicast.

(OPPO) Proposal 10:

  - HARQ-ACK feedback enabling/disabling of SPS group-common PDSCH is configured by RRC signaling.

(OPPO) Proposal 11:

  - HARQ-ACK feedback enabling/disabling mechanism for dynamic group-common PDSCH is reused for the DCI activating/deactivating SPS configuration.

(Qualcomm) Proposal 8:

  - Support UE-specific ACK/NACK-based feedback for activation/release for SPS GC-PDSCH

  - Support configuration of no feedback, ACK/NACK-based or NACK-based feedback for SPS GC-PDSCH without PDCCH

Codebook related

(Huawei) Proposal 13:

  - When UE is configured with NACK-only for multicast SPS without PDCCH scheduling and when the HARQ-ACK bits for more than one multicast SPS configuration, for multicast SPS and unicast SPS, or for multicast SPS and dynamic scheduling of unicast/multicast, needs to be transmitted in the same PUCCH slot, the NACK-only based feedback for multicast SPS is transformed into the ACK/NACK based feedback.

(vivo) Proposal 7:

  - For the HARQ-ACK codebook for SPS PDSCHs, UE generates HARQ-ACK codebooks for SPS PDSCHs in the same way as that in Rel-16 regardless it is unicast SPS PDSCH or multicast SPS PDSCH.

(Intel) Proposal 9:

  - HARQ priority rules are also applicable to feedback for MBS SPS wherein SPS configuration of priority 0/1 maps to HARQ codebook priority 0/1 respectively.

- Further pursue NACK-only

(Nokia) Proposal 41:

  - Enhancements to SPS activation / deactivation mechanisms are needed to have reliable SPS grant in case of NACK-only feedback is used, in particular a mechanism for the gNB to be certain that all UEs have received the SPS grant would be desirable.

(Nokia) Proposal 42:

  - For NACK-only HARQ operation a mechanism should be used, in which UEs are made aware via RRC signalling that SPS might be used for an MBS and request retransmission of an SPS activation PDCCH only if they have not received it in a certain amount of time.

(Nokia) Proposal 43:

  - While gNBs can send SPS deactivation commands, that are in NACK-only mode not acknowledged by UEs, UEs can assume that SPS has been deactivated if they have not been able to decode a PDSCH for a certain period of time.

(Nokia) Proposal 44:

  - In NACK-only HARQ operation, a method is supported for UEs to check with the gNB whether an SPS (re-)activation has been sent by the gNB but missed by the UE. Options include:

    - (a) Option 1: Using a group-common uplink resource

    - (b) Option 2: Using UE-specific signalling (MAC-CE or RRC message)

    - (c) Other methods are not precluded.

(LGE) Proposal 18:

  - For group common SPS, group common PUCCH resources used for NACK only based HARQ-ACK is semi-statically configured per SPS configuration for SPS PDSCH transmissions.

- activation/deactivation related

(LGE) Proposal 15:

  - For group common SPS activation/deactivation to multiple UEs in a group, (de)activation DCI can be repeated on multiple CORESETs with same TCI state or different TCI states.

(LGE) Proposal 16:

  - For a UE not confirming SPS activation, gNB can schedule PTP initial transmission of missed TB(s).

(LGE) Proposal 17:

  - After group common SPS activation, all UEs autonomously release the group common SPS right after a pre-determined slot

    - The pre-determined time is determined by RRC and/or DCI.

- “HARQ for retransmission”

(LGE) Proposal 19:

  - For group common SPS retransmission, PUCCH resource is allocated by DCI of which CRC is scrambled by G-CS-RNTI

(LGE) Proposal 20:

  - Either NACK only based HARQ-ACK or UE specific ACK/NACK based HARQ-ACK is used for SPS PDSCH retransmission.

(LGE) Proposal 22:

  - For multiplexing HARQ-ACKs for dynamically scheduled multicast and unicast SPS, for multicast SPS and dynamically scheduled unicast, or for dynamically scheduled multicast and multicast SPS, UE determines a PUCCH resource based on one of the following options:

    - Option 1: The PRI of the last DCI with dynamic scheduling.

    - Option 2: SPS (e.g. if SPS is unicast)

    - Option 3: priority index

(LGE) Proposal 23:

  - For multiplexing HARQ-ACKs for multicast SPS and unicast SPS or for multicast SPS PDSCHs, UE determines a PUCCH resource based on one of the following options:

    - Option 1: the lowest SPS configuration index

    - Option 2: the highest priority of SPS between SPS configurations

    - Option 3: unicast SPS configuration (or multicast SPS configuration)

## Round-1 (pending)

FL’s Comments

The issues discussed the most are:

- whether NACK-only is supported for multicast SPS activation/deactivation

  - Supportive: 4 (ETRI, NEC, Google, Convida)

  - Not supportive: 7 (HW, vivo, ZTE, [Lenovo], QC, CMCC, MediaTek).

    - The concern for applying NACK-only to SPS activation/deactivation is that NW cannot differentiate whether DCI is missing or decoded successfully by UE. AI 8.12.1 is also discussing how to guarantee the reliability of the group-common PDCCH activation of SPS group-common PDSCH. Overall, solving the issue of DCI missing cannot be perfect, so this issue should be avoided as much as possible.

- how ACK/ACK-based or NACK-only is configured/indicated for multicast SPS. This issue is firstly discussed and solutions are diverse from proposals. Maybe this meeting we can aim to identify alternatives if cannot converge to one solution.

FL’s Proposal:

#### Proposal 5.1-1

For multicast SPS activation/deactivation, NACK-only based HARQ-ACK feedback is not supported.

Collect views:

| Company | Comments |
| --- | --- |
| - CATT | - Agree. - If NACK-only is supported, only the NACK information is feedback when SPS activation/deactivation command is decoded incorrectly. Without feedback information, the gNB cannot know whether the command is missing or all UEs decode it successfully. |
| - Nokia, NSB | Not support. Use of UE-specific ACK / NACK also for SPS activation / deactivation grants, even when using NACK-only feedback for PDSCHs without DCI is inefficient, because it requires configuration of UE-specific PUCCH resources to provide a feedback only once. - Instead, we proposed in Error! Reference source not found. a mechanism that uses NACK-only feedback and solves the above mentioned concern. |
| - Samsung | Needs further discussion/consideration. The reasoning is understood but SPS use-cases are a main reason for introducing NACK-only. If ACK/NACK can be used for such cases, it can practically always be used. |
| - ZTE | From our perspective, it can be up to network implementation. |
| - Qualcomm | Agree. Furthermore, we think neither NACK-only nor no feedback can be used for multicast SPS activation/deactivation. It is necessary to conform the activation/deactivation by using UE-specific ACK/NACK. |
| Ericsson | OK |
| NTT DOCOMO | Support |
| CMCC | OK |
| LG | We are fine with this proposal. |
| Lenovo, Motorola Mobility | It is better to say ACK/NACK is supported. |
| Spreadtrum | Support |
| OPPO | Further discussion is necessary. |
| Apple | Support |
| vivo | ok |
| Chengdu TD Tech, TD Tech | ok |
| Google | Support |

#### Proposal 5.1-2

For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS, down-selection from the following alternatives:

- Alt1: HARQ-ACK feedback option is configured per SPS PDSCH configuration

- Alt2: separate configuration of HARQ-ACK feedback option for SPS activation/deactivation from that for SPS PDSCH receptions

- Note: enabling/disabling HARQ-ACK feedback for multicast SPS can be discussed separately.

Collect views:

| Company | Comments |
| --- | --- |
| - CATT | - In our opinion, the NACK-only based is not supported for multicast SPS activation/deactivation, as discussed in Proposal 5.1-1. And both ACK/NACK-based and NACK-only based for multicast SPS may be supported for UE. Thus, Alt2 is preferred. |
| - Nokia, NSB | - Support |
| - Samsung | - Is Alt. 2 meaningful if, combined with 5.1-1, SPS activation supports only ACK/NACK? |
| - ZTE | - Seems Alt.1 and Alt.2 are not mutually exclusive, do we mean further discuss the following alternatives instead of down-selection. |
| - Qualcomm | - Shall we focus on SPS GC-PDSCH without PDCCH here, separate from SPS activation/deactivation discussed in Proposal 5.1-1? |
| Ericsson | We propose to keep only Alt1 and modify it to: HARQ-ACK feedback option is configured per SPS PDSCH configuration, expect for multicast SPS activation/deactivation, where HARQ feedback is always ACK/NACK based. |
| NTT DOCOMO | It seems to us that Alt1 and Alt2 are not exclusive options. We interpreted Alt1 as making the feedback option(i.e., ACK/NACK or NACK-only) configurable per SPS-Config. And we interpreted Alt2 as allowing the feedback option to be configured separately for SPS activation/deactivation and other SPS PDSCH. Alt1 and Alt2 seem to be compatible. |
| CMCC | Discuss this issue separately from feedback for SPS activation/deactivation |
| LG | We think that Alt 1 means that HARQ-ACK feedback option is configured per SPS configuration index. In addition, both Alt 1 and Alt 2 can be supported. For instance, ACK/NACK based feedback is supported for SPS activation/deactivation while NACK only based feedback can be supported for SPS PDSCHs associated to a particular SPS configuration index. |
| Lenovo, Motorola Mobility | OK |
| Spreadtrum | Fine with the revised version from Ericsson |
| OPPO | support |
| vivo | Similar view with DOCOMO. |
| Chengdu TD Tech, TD Tech | Alt 1 and alt 2 |
| Google | Support. We share the similar view with DOCOMO and LG. |

# PDSCH repetition

Submitted Proposals

(vivo) Proposal 15:

  - For PDSCH repetition of group-common PDSCH, if a UE can be configured with multiple g-RNTIs,

- The PDSCH aggregation factors for group-common PDSCHs with different g-RNITs should be separately configured.

(Nokia) Proposal 36:

  - For Config A, pdsch-AggregationFactor is per multicast / broadcast service.

(Qualcomm) Proposal 9:

  - Support semi-static and dynamic slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs.

    - Repetition configuration for SPS GC-PDSCH can be different than that of dynamic GC-PDSCH.

(LGE) Proposal 24:

  - For a group common SPS configuration, UE can be optionally configured with either pdsch-AggregationFactor or TDRA table with repetitionNumber as part of the TDRA table.

## Round-1 (closed)

FL’s Comments

FL suggests discussing these issues when more companies are interested.

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Companies support that enabling/disabling HARQ-ACK feedback is done per G-RNTI, since different multicast services have different requirements. This logic is the same also when configuring the number of blind repetitions. Thus, we expect a common understanding from the group, if a proposal would be raised by the FL. |
| Samsung | OK |

# CSI feedback and trigger

Submitted Proposals

(ZTE) Proposal 16:

  - UE supports reporting multiple candidate {CQI, PMI, RI} sets in one CSI report for MBS

(ZTE) Proposal 17:

  - Regarding CSI subband determination for MBS transmission, further study how to align different UE’s CSI subband size.

(CATT) Proposal 21:

  - CSI feedback enhancement for MBS can be further studied and discussed but with low priority.

(Qualcomm) Proposal 10:

  - For RRC_CONNNECTED UES, configure the CSI-RS resource per MBS CFR.

    - CSI-RS bandwidth is limited within the MBS CFR.

    - CSI-RS power is associated with GC-PDSCH power.

(Qualcomm) Proposal 11:

  - Support GC-PDCCH to trigger A-CSI-RS transmission in MBS CFR.

(DOCOMO) Proposal 13

  - Include an aperiodic CSI report trigger in DCI format 1_1 for multicast.

(DOCOMO) Proposal 14:

  - Support aperiodic CSI reporting on PUCCH.

(Ericsson) Proposal 12:

  - The existing Rel. 15/16 framework of CSI feedback is reused for multicast/PTM.

## Round-1 (closed)

FL’s Comments

Since we have agreed that existing CSI feedback can be used for multicast transmission and whether enhancement is needed needs further study, the situation does change much comparing to previous meetings.

FL does not see probability to converge to a point more than FFS, so suggest coming back later when situation changes.

Collect views:

| Company | Comments |
| --- | --- |
| Samsung | OK – although we support several of the listed proposals. |
| ericsson | OK |

# Other miscellaneous proposals

Submitted Proposals

(Nokia) Proposal 6:

  - A mechanism is adopted to disable HARQ-ACK feedback (and optionally CSI reporting) of the outlier UEs. This is down-selected from the following:

    - The UE detects itself that it is an outlier UE (e.g. if the reliability criteria cannot be met for a specific amount of time) and disables its own HARQ-ACK feedback mechanism.

    - The UE detects itself that it is an outlier UE and sends a request to the gNB to disable its feedback (e.g. using a specific CQI value such as CQI 0).

(Nokia) Proposal 7:

  - An outlier UE’s NACK-only HARQ-ACK feedback on group-common PUCCH resources can be re-activated by using one of the following methods:

    - The gNB can assign dedicated ACK / NACK resources to the UE and / or the UE can keep reporting CSI feedback, so that the gNB can decide when to re-activate the NACK-only based HARQ-ACK feedback of the UE.

    - The UE can be configured with a QoS criterion, such as an average BLER, and by satisfying such a criterion, the UE can either directly re-activate its feedback mechanism, or request gNB to allow the UE to utilize the group-common HARQ-ACK resource for NACK-only HARQ-ACK feedback.

(Nokia) Proposal 8:

  - When NACK-only HARQ-ACK feedback on group-common PUCCH resources is used, UEs can be configured to measure and report the share of NACKs that they sent but that were not honoured by the respective requested retransmission (e.g., ratio of missing DL retransmissions vs. corresponding number of NACKs sent by the UE). This measuring and reporting

    - is done per HARQ transmission index,

    - may be sent individually per HARQ transmission index or as a set periodically, upon request from the gNB or event-triggered on the UE side.

(Nokia) Proposal 33:

  - RAN1 decides whether the UE produces HARQ-ACK bits for all the multicast services that the UE had indicated interest or only for services that the UE has received PDSCH(s) when constructing multiplexed Type-1 codebook for FDM-ed unicast / multicast case.

(Nokia) Proposal 34:

  - To overcome the problem of HARQ-ACK codebook size ambiguity, the constructed HARQ-ACK sub-codebooks are prepended by the UE with the information (with a known size) about which service the following sub-codebook belongs to using one of the following methods:

    - i.	The received services can be indexed, and that index can be prepended to the HARQ-ACK sub-codebook:

      - The services can be indexed based on the order of RNTIs. For instance, unicast can be of index 0, the service with minimum configured G-RNTI value is index 1, service with second minimum G-RNTI is index 2 ….

      - A mapping of RNTI to index can be directly signaled by the gNB via RRC signaling, e.g., during RNTI configuration.

      - Same indexing with the concatenation order of the sub-codebooks can be used.

(Nokia) Proposal 45:

  - On top of MIMO precoding as widely applied in NR unicast that is transparent to the receiver, non-transparent diversity techniques such as LD-CDD known from LTE are supported for group-common PDSCH transmission.

(CATT) Proposal 18:

  - To support multi-beam transmission in MBS, gNB can transmit same MBS data on all SSB beams.

(CATT) Proposal 19:

  - UE can receive MBS data from neighbor SSB-beam, and the soft-combination is used to improve the reliability of MBS receptions.

(FUTUREWEI) Proposal 3:

  - For the same service, all RRC_CONNECTED UEs receiving multicast are configured with the same MCS table.

(FUTUREWEI) Proposal 4:

  - For MBS, FBRM should be used to improve performance for small sized TBs.

(LGE) Proposal 25:

  - Discuss whether different TCI states can be configured for group common SPS received by different UE, e.g. different slots of group common SPS PDSCH repetitions or different SPS configurations can be associated to different TCI states for the same group of UEs.

(Intel) Proposal 2:

  - For the case when unicast and MBS PDSCH partially overlap in time on different PRBs, out-of-order HARQ feedback may be supported wherein the HARQ feedback corresponding to the PDSCH ending earlier in time may be transmitted after the HARQ feedback for the PDSCH ending later in time

FL’s Comments

The proposals summarized in this section are either interested by only a single company or can be discussed in other section/agenda. Hence, FL does not plan to handle the proposals in this section in this meeting.

Collect views:

| Company | Comments |
| --- | --- |
| Samsung | OK – several of the proposals may be possible to discuss through other proposals. |

# Proposals for 2nd checkpoint and GTW on 25th (closed)

The proposals affect RRC parameters are slightly prioritized for current discussion:

## Part 1 (closed)

Part 1 has been discussed for a couple of rounds, so supposed to be stable and expected to be discussed firstly on GTW if not approved for the 2nd checkpoint (24th). Please continue the discussion if you still have strong concerns using each table.

### Round-1 (closed)

#### Proposal 2.7.2-1-r1 (H)

For UE supporting ACK/NACK-based HARQ-ACK feedback for multicast, the maximum values that are configured to UE as follows are unchanged compared to unicast in Rel-16:

- the maximum number of PUCCH resources sets in each PUCCH-config,

- the maximum number of PUCCH resources in a PUCCH resource set,

- the maximum number of UCI information bits for the first PUCCH resource set.

- Note:

  - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.

  - The case of NACK-only based is discussed separately.

| Company | Comments |
| --- | --- |
| Samsung | One aspect that requires further discussion in the second sub-bullet. In Rel-16 for unicast, the maximum can be 32, where the CCE index is used. It may be better to first discuss how the UE determines the PUCCH resource for ACK/NACK in case of multicast. FL’s response: Not sure whether the intention was make clear. Let’s repeat again: I intended to mean UE supporting ACK/NACK for multicast does not increase the max number of sets/resources because procedure of ACK/NACK for multicast is pretty much like to that for unicast. Therefore, the max resources of set 0 for ACK/NACK is 32 and unchanged which is used for unicast and multicast. I assume there is no difference for UE determining the PUCCH resource for ACK/NACK for multicast comparing to that for unicast, or could Aris elaborate a bit more why how UE determining the PUCCH resource for ACK/NACK for multicast may affect the second sub-bullet? [Samsung2]: Thank you for the clarification – no further comment. Fine with the proposal. |
| Lenovo, Motorola Mobility | Regarding the note, “This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.”, does it imply the maximum number of PUCCH resources in the 1st PUCCH resource set is not changed even if the UE is configured to receive multiple multicast services and unicast service? FL’s response: Yes. Regardless UE supporting unicast only, multicast only, multiple multicast, or unicast and multicast, all these max values are unchanged. |
| LG | We are generally fine with this proposal. We wonder if the maximum number of PUCCH resources corresponds to the first PUCCH resource set only. We could change to: • the maximum number of PUCCH resources in a the first PUCCH resource set, |
| vivo | Generally fine. |
| Lenovo 2 | We are OK with this proposal. Based on LG’s modification, does “ the max number of PUCCH resources in a PUCCH resource set” mean “the max number of PUCCH resources in a PUCCH resource set in each PUCCH-config” - the maximum number of PUCCH resources in a PUCCH resource set in each PUCCH-config, |
| Nokia, NSB | We are fine with the proposal with Lenovo’s modification. |
| ZTE | Just some editorial issues. These values are not configured by RRC, thus the wording is a bit misleading. Propose the following editorial changes. Hope it is not too late. For UE supporting ACK/NACK-based HARQ-ACK feedback for multicast, the maximum values that are configured to UE as follows following values are unchanged compared to unicast in Rel-16: - the maximum number of PUCCH resources sets in each PUCCH-config, - the maximum number of PUCCH resources in a PUCCH resource set, - the maximum number of UCI information bits for the first PUCCH resource set. - Note: - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast. - The case of NACK-only based is discussed separately. |
| Qualcomm | ZTE’s suggestion is ok to us |
| Ericsson | If it follows from R-16, maybe it is good to mention the total number of PUCCH resources from all PUCCH-Config/PUCCH-ConfigurationList is not changed as well. |
| NTT DOCOMO | Support |
| Spreadtrum | Support |

#### Proposal 3.3.2-2-r1 (H)

For the separate PUCCH-ConfigurationList that is optionally configured to UE for NACK-only based HARQ-ACK feedback for multicast,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority feedback and high priority feedback, respectively.

- FFS: how to handle the case when separate PUCCH-ConfigurationList is not configured to UE for NACK-only based HARQ-ACK feedback for multicast.

| Company | Comments |
| --- | --- |
| vivo | Support |
| Samsung | OK. |
| ZTE | Previously, we discussed that PUCCH-ConfigurationList for NACK-only can be shared with that for unicast or multicast ACK/NACK. Considering that we are now only discussing the separate configuration case here, we suggest to add an FFS for this issue. FFS: how to handle the case when separate PUCCH-ConfigurationList is not configured to UE for NACK-only based HARQ-ACK feedback for multicast. FL’s responses: updated. |
| Lenovo, Motorola Mobility | OK. |
| vivo | ok |
| Nokia, NSB | Support. |
| Qualcomm | fine |
| Ericsson | Support |
| NTT DOCOMO | Support |
| Spreadtrum | Support |

#### Proposal 2.1.3-1-r1 (H)

When UE is configured with the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList for multicast, it is applied to all G-RNTIs configured to UE.

| Company | Comments |
| --- | --- |
| FL’s notes | Based on the discussion in section 2.1.3, it seems the vast majority support the codebook type is applied to all G-RNTI configured to UE. The proposal is updated accordingly. |
| vivo | support |
| Samsung | Support. |
| Lenovo, Motorola Mobility | OK |
| Nokia, NSB | Support. |
| ZTE | Support |
| Qualcomm | Does it apply to both ACK/NACK and NACK-only-based feedbacks? |
| Ericsson | Support |
| NTT DOCOMO | Support |
| Spreadtrum | Suppport |

#### Proposal 4.1.2-3-r1 (H)

The priority index is,

- for the second DCI format with CRC scrambled with G-RNTI which takes DCI format 1_1 or 1_2 as the baseline, optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

- for the first DCI format with CRC scrambled with G-RNTI which takes DCI format 1_0 as the baseline, optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

| Company | Comments |
| --- | --- |
| FL’s notes | Based on the discussion in section 4.1.2, the proposal is updated. Now for both DCI formats, the proposal is that the priority index can be configured to be included in DCI. If not included in DCI, the default is low priority. The proposal is split into two bullets for easier discussion in case there are two different views for two different formats. The point is clear. If there is no controversy whether treat two formats as the same way, no problem to merge the two sub-bullets into one. |
| vivo | We support the first sub bullet. For the second sub bullet, we prefer to wait the decision on the field design on the first DCI in AI 8.12.1. We haven’t agree that priority indicator can be optionally configured to be included in the DCI format. |
| Samsung | Support. |
| Lenovo, Motorola Mobility | We are OK with this proposal. |
| MediaTek | We have the similar view with vivo. In last meeting, we agreed that “As a baseline, reuse existing fields in DCI format 1_0 with CRC scrambled by C-RNTI for the fields of first DCI format with CRC scrambled with G-RNTI”. The existing DCI format 1_0 with CRC scrambled by C-RNTI does not have “priority index” field, and at least there is also no proposal to add the new filed during the discussion. Thus, the “optionally configured” is confused for us. We suggest to only keep the first sub-bullet at current stage. |
| CATT | We share the view with vivo and MediaTek. Since the fields in the first DCI is still under discussion, whether priority index in the first DCI is “optionally configured” should wait the decision on the field design in AI 8.12.1 |
| Nokia, NSB | We support the proposal. |
| ZTE | We are not sure why we need to add the priority index field in the first DCI format. We would prefer to follow the legacy UE behavior, i.e., no priority index for DCI format 1_0. |
| Qualcomm | We think the first DCI format is similar as fall-back DCI with basic scheduling information for GC-PDSCH. No need to add priority index in the first DCI format. If priority index is needed, just use the second DCI format. So we prefer to delete “optionally configured to be included in the DCI format. If not configured,” in the second subbulet. |
| Ericsson | Support |
| NTT DOCOMO | Support |
| Spreadtrum | Support the first bullet, not ok for the second bullet. |

#### Proposal 4.3.2-1-r1 (H)

Confirm the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback with the update in tracking change as follows:

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

- UE capability is introduced and FFS details.

RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback subject to UE capability.

| Company | Comments |
| --- | --- |
| vivo | As our comment in previous round. We disagree to confirm the WA. First. We think there is no need to enable/disable ACK/NACK feedback for a given G-RNTI. Regarding RAN2’s following agreements, after further check with our RAN2 delegate, RAN2 only support one-to-one mapping between G-RNTI and MBS session (i.e, MBS service) in NR MBS. The multiple MBS QoS flows corresponding to the same MBS session belong to the same MBS service. In a word, one G-RNTI corresponds to one MBS service. For one service, the QoS requirement is time-independent. Thus, DCI dynamic enabling/disabling ACK/NACK feedback for a given G-RNTI is not needed at all. One-to-one mapping between G-RNTI and MBS session is supported in NR MBS. Other mappings FFS Multiple MBS QoS flows corresponding to the same MBS session can be mapped to one or more than one MBS radio bearers. Second, DCI dynamic enabling/disabling ACK/NACK feedback will make UE behavior more complex. Because for a certain g-RNTI, UE needs to determine whether ACK/NACK feedback is needed or not based on the indication in DCI. If RRC is used for the enabling/disabling of ACK/NACK feedback, when UE detects a DCI with a certain g-RNTI, whether to feedback ACK/NACK is deterministic. Third, DCI dynamic enabling/disabling ACK/NACK feedback will introduce more work load. As Proposal 4.3.3 proposed, we need to discuss how to indicate, e.g., existing field or new field. This issue is still very controversial. For the new field option, to keep the common understanding on the fields of group-common DCI, this new filed needs to be fixed or addition configuration signaling. FL’s responses: The 3GPP procedure for handling WA is that WA should be configured as long as there is no fundamental issue that makes the WA not work. All the comments from vivo don’t comply to this procedure. On the other hand, this WA was made was because some companies see the use cases and benefits and it was the compromise. It is not good to revert all the discussion to the very beginning which is not helpful to the group for progress. |
| Samsung | OK to confirm the WA. In response to the comments by Vivo, it is noted that the function exists in SL and a similar one has been introduced in NTN (HARQ-ACK disabling per HARQ process – UE doesn’t know whether ACK/NACK is needed before decoding the DCI, at least for Type-2 CB). Whether it is an existing field or a new field, the issue is trivial – it is just making a choice although RAN1 has a habit of spending too much time on such things. |
| ZTE | Ok to confirm the WA. |
| Lenovo, Motorola Mobility | Ok to confirm it. |
| vivo2 | Thanks for Samsung’s response, I agree that the function exists in SL and a similar one has been introduced in NTN. However, I think for SL, there is no RRC configuration, so HARQ-ACK feedback disabling has to be done via DCI. For NTN, RTT delay is too long and will have impact on HARQ operation, it is better to do PDSCH repetition rather than retransmission based on HARQ-ACK feedback. But for some MAC CE transmission, it is better to get the HARQ-ACK feedback from UE. That is why they support HARQ-ACK disabling per HARQ process. But for MBS, we don’t’ have MAC CE transmission on GC-PDSCH. So, we can disable HARQ-ACK feedback for all HARQ process simply by RRC. Thanks for FL’s response. The WA was agreed in quite late in the last meeting. Although we think this WA is not very clear, for the process, we made compromise with the comments on multiple g-RNTIs. There is one serious issue for DCI dynamic indication, that is DAI counting issue. Since the RRC enabling/disabling is per UE, and the DCI indication is group-common, then there is only one DAI indicator in the group-common DCI. Then we don’t know how to indicate the DAI value for GC-PDSCH3 as shown in the following case. In addition, we think the 3GPP procedure for handling WA includes confirm the WA with update if the WA is not very reasonable. We suggest the following update. Confirm the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback with the update in red as follows: Working assumption: For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH: - RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback per g-RNTI. - If RRC signaling configures the function of group-common DCI based indication multiple g-RNTIs, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling. - FFS details on RRC signaling and group-common DCI indicating. - FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases. - FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction. - FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission. - FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback. - FFS whether/how to apply it to SPS group-common PDSCH. - UE capability is introduced and FFS details. |
| MediaTek | The motivation of newly added wording “UE capability is introduced and FFS details” is unclearly for us. Does it mean “enabling/disabling ACK/NACK-based HARQ-ACK feedback via dynamic group-common PDSCH” is UE capability behavior? If UE doesn’t support this capability, whether UE support HARQ-ACK? Considering the meeting progress, we can live with the following version: - FFS: UE capability is introduced and FFS details. |
| vivo2 | Thanks for your response. Think until now, we have different understanding on the following two aspects: - How to indicate C-DAI/T-DAI value in a PDCCH if the PDSCH is indicated with ‘disabling’. - Understanding 1: C-DAI/T-DAI value should not be increased by 1 since the PDSCH does not need to feed back ACK/NACK(this is my understanding) - Understanding 2: C-DAI/T-DAI value should be increased by 1 since the C-DAI/T-DAI is used for the counting of number of PDCCHs regardless ACK/NACK is needed for the PDSCH is needed or not. - How to generate HARQ-ACK codebook according to the DAI indication in each PDCCH. - Understanding 1: UE generate HARQ-ACK codebook according to the C-DAI/T-DAI value, for each DAI value UE sets corresponding ACK/NACK only when ACK/NACK feedback is enabled. So a ‘real’ DAI that without counting the PDCCHs indicated with ‘disabling’ may be needed for codebook generation or we need to change the current spec for type 2 codebook generation(This is my understanding. the ‘real’ DAI here is ‘another DAI’ mentioned in my previous comment) - Understanding 2: UE generate HARQ-ACK codebook according to the C-DAI/T-DAI value, for each DAI value UE sets corresponding ACK/NACK regardless when ACK/NACK feedback is enabled and set NACK when ACK/NACK feedback is disabled. (this is not meaningful to disabled ACK/NACK in this case) Then, the issue can be further explained in the following figure. As I explained in before, different C-DAI/T-DAI values are needed for GC-PDSCH3 if we based on Understanding 1 for the first issue. However, some companies think we should set C-DAI/T-DAI based on Understanding 2 for the first issue. Then we need to generate HARQ-ACK codebook for UE2 based on Understanding 1 for the second issue. If “enable/disable” is configured by RRC purely per UE, there is no such issue. Because, there are only two subgroups of UEs for a MBS group of one service： - Sub-group 1：ACK/NACK is disabled by RRC. C-DAI/T-DAI indication is not available for these UEs. The DAI indication is ignored. - Sub-group 2：ACK/NACK is enabled by RRC. C-DAI/T-DAI indication is available for these UEs and is kept same among these UEs. We think companies still have different understanding on the WA. Some companies think the RRC is per MBS service, and other companies think it is per service per UE. In addition, regarding the issue we raised, I only get two company's reply. People seem have different understandings on how set the DAI value and how to generate HARQ-ACK in such case. We want to hear more voice before we agree to confirm the WA |
| Samsung2 | Regarding the issue raised by “Vivo2”, if a UE is disabled HARQ-ACK feedback by RRC, the UE behavior for fields related to HARQ-ACK feedback (DAI, PRI, HARQ-ACK timing) can remain undefined as the UE is not transmitting respective PUCCH. If a UE is enabled HARQ-ACK feedback by RRC, the simplest approach is for the UE to expect same enabling/disabling indication by different DCIs for a same Type-2 HARQ-ACK codebook. There is no apparent reason to support different indications for a same Type-2 HARQ-ACK codebook - no need to create new UE behaviors. If for some reason different indications for enabling/disabling are to be allowed for a same Type-2 HARQ-ACK CB, the UE can disregard everything about HARQ-ACK in the DCI indicating disabling (DAI, PRI, HARQ-ACK timing) as for the case HARQ-ACK is disabled by RRC. How the DAI is set in DCIs indicating enabled HARQ-ACK is up to the gNB (proper behavior would be to not count DCIs indicating disabled HARQ-ACK) – the procedure can be based on the DAI values as in Rel-16. |
| Lenovo 2 | I think the current discussed cases are deadlocked on whether there are different HARQ-ACK feedback enabling/disabling configurations for different UEs in same group. Assuming UE-specific RRC signaling is used to enable or disable HARQ-ACK feedback (although I don’t think this is an economical way), we think a reasonable gNB will configure same HARQ-ACK feedback options for the group of UEs, i.e., enabling HARQ-ACK feedback for each of the UEs in the group or disabling HARQ-ACK feedback for each of the UEs. This is because enabling/disabling HARQ-ACK feedback option is based on the reliability requirement of the multicast service and the number of available PUCCH resources in one slot for multicast. Arbitrary enabling/disabling HARQ-ACK feedback does not make any sense and leads to more complexity. |
| MediaTek | We basically agree with Lenovo’s view that “A reasonable gNB will configure same HARQ-ACK feedback options for the group of UEs”. If different UE with in the same group have different understanding of HARQ-ACK feedback, it will make the network scheduling and UE processing more complicated. E.g., if some UEs disable the HARQ-ACK and some UEs enable HARQ-ACK, and the PTM transmission scheme is used for retransmission, the HARQ-ACK disable UE will also receive the retransmission even though they do not feed back the HARQ-ACK (why not feedback the HARQ-ACK since it still can receive retransmission). Thus, we support that “A reasonable gNB will configure same HARQ-ACK feedback options for the group of UEs”. |
| CATT | Copied our views from the inline discussion: In our understanding, UEs in a group can have same RRC configuration or different RRC configurations, it depends on gNB implement, as shown below table1, where there are 7 cases that a group UEs can be configured. Case 1; RRC enabling; OK. Case 2; RRC disabling; OK. Case 3; DCI indicating; OK. Case 4; RRC enabling+ RRC disabling; OK. Case 5; RRC enabling+ DCI indicating; NOT OK. Case 6; RRC disabling+ DCI indicating; OK. Case 7; RRC enabling+ RRC disabling+ DCI indicating; NOT OK. Regarding the DAI accounting issue that Lina mentioned, in our understanding, it is really existed when some UE are enabled by RRC and other UEs are disabled by DCI indication function (i.e. Case5/7). For the enabled UEs, the DAI is expected to increase 1, and DAI for the disabled UEs is expected to keep unchanged. Thus, the cases that RRC enabling and DCI indicating cannot be configured simultaneously, which is also illustrated in our tdoc. However, other cases(Case1/2/3/4/6) are still workable based on WA. Moreover, per our understanding, the Case5/7 can be avoided by gNB implement. Thus, we suggest to first confirm the WA, and other issues that WA may bring can be further studied. |
| Nokia, NSB | We support the WA. We do not think that the scenario that vivo illustrated will occur, as Samsung pointed out. DAI is used to provide one HARQ-ACK codebook for several PDSCH TB receptions. Why would the gNB indicate enable for some of those TBs and disable for some others, if the feedback will be provided as Type-2 HARQ-ACK codebook? |
| vivo3 | Regarding the 7 cases listed by Miaomiao. I am wondering which case is reasonable and which is not reasonable. If case 4 and case 6 are reasonable cases, why aren’t case 5 and case 7 reasonable? I think there is one use case for case 4/5/6/7. For example, for the cell-center UEs, PDSCHs are almost always decoded correctly. gNB can RRC disable the ACK/NACK feedback to save PUCCH resources. For the cell-edge UEs, PDSCHs are almost always decoded incorrectly, gNB can also RRC disable the ACK/NACK feedback to save PUCCH resources and retransmit the TB directly (e.g. by PTP), where the initial PTM transmission is just used for soft combining. For the retransmission issue raised by Xuanbo, we think PTP can also be used for the retransmission. gNB can retransmit the TB by PTP for the UE who feedback NACK. From this perspective, it is still possible to configure different HARQ-ACK feedback options for the group of UEs. In addition, even gNB retransmits the TB via PTM scheme, we think it is ok. For the UEs decoding the TB successfully (including the UEs who sends ACK and the UEs who are RRC configured with disabling), the retransmission is ignored. |
| Qualcomm | Considering vivo’s concern, the dynamic enabling/disabling in GC-DCI needs more discussion. |
| Ericsson | We also think we need to clarify how DCI based solution works when DCI is missed by a UE. This may result into codebook size ambiguity. We think it is better to mention the enable/disable indicator in DCI does not change for those PDCCHs point to the same PUCCH. We also wish to point out that with only two HARQ feedback states, e.g. ACK/NACK and no feedback, the meaning of enable/disable is clear i.e. enabled=HARQ ACK/NACK is used, disabled=no HARQ feedback. But with also NACK-only feedback we have three HARQ feedback states, with possible transitions between any of these. It seems therefore more natural to use something like “change of HARQ feedback state”, rather than “enable/disable”. For the case of just using RRC configuration (i.e. not configuring the DCI function) our interpretation is that a UE may be RRC configured to use any of the three HARQ feedback states and can be RRC reconfigured to any other state. We assume other companies share this understanding. If not, RAN1 needs to discuss this. Also, when RRC configures the DCI function it should be possible to change HARQ feedback state via DCI between any of the three states: ACK/NACKNACK-only, ACK/NACK no HARQ feedback, NACK-only no HARQ feedback. |
| NTT DOCOMO | We believe that feedback enable/disable should be related to the reliability requirements of the multicast service. So we believe that the RRC configuration for enabling/disabling should be the same for all UEs in a group. |
| Lenovo 3 | We think the HARQ-ACK feedback enabling/disabling including possible HARQ-ACK feedback options need to be designed dynamically to adapt the reliability requirements, the number of UEs in the group, and the number of PUCCH resources available in one slot for multicast reception according to the number of UEs in the group. Since the number of UEs in the multicast group may be variable from time to time, e.g., some UEs may leave the group and some new UEs may join the group at any time, which makes the number of UEs in the multicast group variable. Considering the number of available PUCCH resources is predetermined, the variable number of UEs in the multicast group requires a dynamic HARQ-ACK feedback option. E.g., when the number of UEs is too large, then gNB can disable the HARQ-ACK feedback or configure NACK-only based HARQ-ACK feedback; otherwise, each UE can be configured with two PUCCH resources for enabling UE-specific ACK/NACK feedback. As a result, in order to fully exploit the benefit of each HARQ-ACK feedback option, it is necessary to further allow the gNB to dynamically enable/disable HARQ-ACK feedback option for a given MBS transmission. In that sense, gNB can flexibly adjust the feedback options based on the number of interested UEs in connected mode or the PUCCH resource capacity on the serving cell or the reliability requirement of QoS so as to reach a tradeoff between transmission reliability and resource overhead. |
| CATT2 | Echo vivo’s comments. In Case 4 (RRC enabling+ RRC disabling) and Case 6（RRC disabling+ DCI indicating）, the DAI will be invalid for the UEs which are configured by RRC disabling,. Thus, no DAI issue will happen in Case4 and Case6. We believe that some companies may need more time to discuss the 7 cases that mentioned above. Also, a common understanding that whether different RRC can be configured among the group UEs should also be achieved. However, we think all this issues can be further studied after we confirm the WA. Moreover, how to construct the HARQ-ACK codebook is included in the sub-bullet for further study, as shown below. - ‘FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction. ’. Thus, we suggest confirming this WA at this stage because enabling/disabling HARQ-ACK feedback dynamically is necessary for multicast services. The remaining issues can be further studied. |
| Spreadtrum | In our memory, when we discuss to support feedback enable/disable function, one main justification is to reduce PUCCH resource overhead except for reliability, especially when orthogonal PUCCHs are limited. In our understanding, dynamic enable/disable could provide flexibility for gNB to ensure the PUCCH resources orthogonal among member UEs in a group, e.g., some member UEs could share the same PUCCH resource in TDM by leveraging the feature of DCI indicating feedback enabled/disabled per UE. We are fine to confirm the WA. But given that there exists divergence among companies, we are also fine to delay this issue, and leave more time for people to study, e.g., based on the cases listed by CATT. |

#### Proposal 4.3.2-2-r1 (H)

RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI per UE.

| Company | Comments |
| --- | --- |
| vivo | We think group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback for a certain g-RNTI is not needed. The reasonable option is that RRC signaling configures the enabling/ disabling of ACK/NACK based HARQ-ACK feedback per G-RNTI per UE, and UE determines whether to feed back ACK/NACK or not based on the G-RNTI of PDSCH. We suggest the following update. RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI per UE. FL’s responses: vivo only cares about RRC configuring ACK/NACK which is what proposal 4.3.2-4-r2 tries to handle. |
| Samsung | OK with the proposal. Per G-RNTI configuration may be an overkill but can also be useful depending on the number of UEs receiving each MBS service (e.g. for a service with many Ues NACK-only; else, ACK/NACK). In any case, it is trivial and easy to support (practically 0 spec impact as everything will be per G-RNTI). |
| ZTE | OK |
| Lenovo, Motorola Mobility | We think using common RRC signaling per G-RNTI is enough for multicast service. Per UE configuration can anyway work but not that economical. |
| CATT | We are OK with this proposal. But a small modification is needed. It seems that there are two predicate verbs (i.e. ‘configures’ and ‘is’) in the proposal. We suggest the proposal can be changed as following: #### Proposal 4.3.2-2-r1 (H) RRC signaling configures tThe enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI per UE. |
| Vivo2 | Regarding Samsung’s comment, we don’t think per G-RNTI configuration may be an overkill. We think it is overflexible to enable/disable ACK/NACK with dynamic DCI indication. There is no motivation to have such operation for the real system. |
| MediaTek | We have the similar view with Lenovo. |
| Nokia, NSB | We support the proposal. We have been discussing / showing by simulations since several meetings that there are scenarios where disabling some of the UEs’ HARQ-ACK feedback can be beneficial. E.g., in case there are UEs in very good conditions, their feedback can be disabled, and they can rely on the retransmissions originated by the feedback of the UEs in bad channel conditions. This would save PUCCH overhead. Although the proposal covers only ACK / NACK feedback, also for NACK-only feedback, UE-specific enabling / disabling of HARQ feedback is needed. A unified solution can be agreed for both mechanisms, if we agree per UE enabling / disabling also for ACK / NACK. E.g., in case there are some UEs in very bad channel conditions that utilize the group-common NACK-only PUCCH resource, they would always send NACKs and the overall spectral efficiency of the group would be reduced due to retransmissions. Those UEs’ feedback can be disabled. |
| Qualcomm | We don’t support the proposal, which needs further discussion. As mentioned in previous round, the configuration can per HARQ process ID per UE, similar as NTN agreement. It is up to gNB to use the HAQR process ID for same or different G-RNTIs. Regarding Lenovo’s comment, the multicast configuration especially related with HARQ-ACK feedback will be via unicast RRC signaling for RRC_CONN UEs. So, it is per UE. It is not feasible to use common RRC (SIB/MCCH), which results in unnecessary impact on IDLE/INACTIVE UEs. |
| Ericsson | Support |
| NTT DOCOMO | Similar view with Lenovo. |

#### Proposal 4.3.2-4-r2 (H)

If RRC signaling does not configure the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback, whether UE feedbacks ACK/NACK or not is configured per G-RNTI per UE by RRC signalling.

| Company | Comments |
| --- | --- |
| Samsung | OK. |
| ZTE | OK |
| Lenovo, Motorola Mobility | We think using common RRC signaling per G-RNTI is enough for multicast service. Per UE configuration can anyway work but not that economical. |
| LG | We are fine with the proposal. We assume that per UE signaling is used for multicast. |
| CATT | OK |
| Nokia, NSB | Support. |
| Ericsson | Support |
| NTT DOCOMO | Similar view with Lenovo. |

### Round-2 (closed)

#### Proposal 2.7.2-1-r2 (H)

For UE supporting ACK/NACK-based HARQ-ACK feedback for multicast, the following values are unchanged compared to unicast in Rel-16:

  - the maximum number of PUCCH resources sets in each PUCCH-Config,

  - the maximum number of PUCCH resources in a PUCCH resource set in each PUCCH-Config,

  - the maximum number of UCI information bits for the first PUCCH resource set.

  - the total number of PUCCH resources from all PUCCH-Config/PUCCH-ConfigurationList.

  - Note:

    - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.

    - The case of NACK-only based is discussed separately.

#### Proposal 2.1.3-1-r3 (H)

- When UE is configured with the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList for ACK/NACK based feedback for multicast, it is applied to all G-RNTIs configured to UE.

#### Proposal 3.3.2-2-r1 (H)

For the separate PUCCH-ConfigurationList that is optionally configured to UE for NACK-only based HARQ-ACK feedback for multicast,

  - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority feedback and high priority feedback, respectively.

  - FFS: how to handle the case when separate PUCCH-ConfigurationList is not configured to UE for NACK-only based HARQ-ACK feedback for multicast.

#### Proposal 4.1.2-3-r2 (H)

The priority index is,

- for the second DCI format for GC-PDCCH, optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

- for the first DCI format for GC-PDCCH, down-select from:

    - Alt1: optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

    - Alt2: always low priority.

#### Proposal 4.3.2-1-r1 (H)

Confirm the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback with the update in tracking change as follows:

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

- UE capability is introduced and FFS details.

#### Proposal 4.3.2-2-r1 (H)

RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI per UE.

#### Proposal 4.3.2-4-r2 (H)

If RRC signaling does not configure the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback, whether UE feedbacks ACK/NACK or not is configured per G-RNTI per UE by RRC signalling.

## Part 2 (closed)

This part 2 is added to treat more proposals.

#### Proposal 4.1.1-1

The priority of multicast for NACK-only based feedback is the same as the priority of unicast for the same priority index of HARQ-ACK.

| Company | Comments |
| --- | --- |
| FL’s note | This proposal is unchanged since stable from Round-1 |
| Samsung | OK |
| CMCC | OK |
| Lenovo, Motorola Mobility | OK |
| CATT | OK |
| Nokia, NSB | Support. |
| vivo | ok |
| LG | We are fine with this proposal. |
| ZTE | OK |
| Ericsson | OK |
| NTT DOCOMO | Support |
| Spreadtrum | Support |

#### Proposal 3.2.1-2

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, down-select from the following alternatives:

  - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

  - Alt2: Support sub-slot based PUCCH for this case.

  - Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

  - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

  - Alt5: NACK-only bundling

| Company | Comments |
| --- | --- |
| FL’s note | This proposal is unchanged since stable from Round-1. Solutions are quite diverse and a step forward is identifying alternatives for consensus and continue the discussion based on the alternatives. Regarding Ericsson’s comment in 1st round: Alt 5 means when more than one TBs with NACK-only based feedback are available for transmission in the same PUCCH slot, as long as there is one TB is NACK-only, what is transmitted in the PUCCH is one NACK-only bit/sequence. Respond to Apple: Not sure I get the point of “dropping NACK-only”, this proposal does not talk about overlapping with other PUCCH with ACK/NACK or PUSCH. Not sure why NACK-only is dropped. I tend to not expand the list of alternatives too much if there is not explicit support. |
| Samsung | OK. Some debate seems necessary to capture the trade-offs. We support Alt. 4 – same principle as for PUCCH format 0 and is sufficient for the typical case of 1 PDSCH/slot and a DDDSU configuration. There are limitations in case of >1 PDSCH/slot and DDDSU, but those limitations exist at some point for any approach and there is no need to optimize for such cases as it is unlikely to have a group of UEs what all support >1 PDSCH/slot - can consider simple suboptimal solutions for those cases. Alt. 1 is against having NACK-only (e.g. there is a resource overhead problem for 1 bit but there isn’t for >1 bits?), Alt. 2/3 have many problems including not being supported by (practically all) UEs and resulting to significant coverage reductions, while Alt. 5 practically always results to retransmissions unless the target BLER is very low which will then penalize data rates and/or coverage. |
| CMCC | Ok. We also support Alt 4. |
| Lenovo, Motorola Mobility | We are OK with this proposal and support Alt 4. Alt 1: Not sure whether there are sufficient PUCCH resources for such switching. Alt 2: Dependent on UE capability of supporting sub-slot PUCCH Alt 3: Not consistent with the design principle of PUCCH format 0 where sequence based PUCCH is designed for low PAPR and coverage. Alt 4: Keep NACK-only design principle and maintain the low PAPR in PUCCH Alt 5: more unnecessary retransmissions are caused which leads to DL performance degradation. |
| Nokia, NSB | Ok with the proposal. Support 1/2/3, not support 4/5. We believe that Alt 4 is not feasible. It would be too complex to define all combinations (of specific sequences) that would fit different scenarios, in addition to allocation of significant amount of PUCCH resources to represent each combination. Assume that the UE needs to provide NACK feedback for 10 different TBs in a slot, how do the supporters of Alt 4 believe all the combinations can be configured? In some other cases, only 2-3 TBs need to be NACK-ed in the same slot, how will the configuration be dynamically changed to represent the changes in number of TBs to be NACK-ed? This kind of specification is not necessary and other alternatives (such as Alt 2) can be agreed that require less impact. |
| vivo | We are OK with this proposal and support Alt 5. Alt 1: PUCCH for 2bits ACK/NACK for UE1 will collide with PUCCH for NAC-only for UE2. Alt 2: Dependent on UE capability of supporting sub-slot PUCCH Alt 3: New feature is introduced and dependent on UE capability Alt 4: more sequences or PUCCH resources are reserved. Alt 5: simplest |
| LG | We are fine with this proposal. We support Alt 1, thinking that the solution should be general enough to be used for other mux cases (e.g. with unicast feedback) with basic UE capability. In our view, Alt 2 and 3 would require UE supports more than basic multicast capability and Alt 4 and 5 are very specific to NACK only case. |
| ZTE | We support Alt.1, Alt.2 and Alt.4. From our perspective, Alt.1 is a common solution for all scenarios even when NACK-only needs to be multiplexed with other UCI. Alt.4 is also feasible when only NACK-only are overlapping. Alt.2 is also fine if UE supports sub-slot based HARQ-ACK. |
| Qualcomm | Ok to list up the alternatives for further study |
| Ericsson | Ok with the proposal. We support Alt 2/3/4/5, not support Alt 1. |
| NTT DOCOMO | Support. We prefer Alt4. |
| Spreadtrum | Ok |

#### Proposal 5.1-2-r1 (H)

For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS PDSCH without PDCCH scheduling, support:

  - Alt1: HARQ-ACK feedback option is configured per SPS configuration index

  - Note: enabling/disabling HARQ-ACK feedback for multicast SPS can be discussed separately.

| Company | Comments |
| --- | --- |
| FL’s note | This proposal is updated per the comments in Round-1. Since ACK/NACK based feedback is supported for multicast SPS and whether NACK-only is supported for activation/deactivation, this proposal focuses on PDSCH without PDCCH scheduling only. |
| Samsung | OK. |
| CMCC | OK |
| Lenovo, Motorola Mobility | OK |
| CATT | OK with this proposal. |
| Nokia, NSB | Ok. |
| vivo | ok |
| LG | We are fine with this proposal. |
| ZTE | We have a proposal that pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList for multicast is applied to all G-RNTIs configured to UE. To us, NACK-only based codebook and ACK/NACK based codebook are different codebooks, if we follow the same approach, we need to apply the same feedback mechanism for all SPS configurations. #### Proposal 2.1.3-1-r1 (H) - When UE is configured with the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList for multicast, it is applied to all G-RNTIs configured to UE. |
| Qualcomm | We think it may be too early to discuss HARQ-ACK feedback mode selection for SPS GC-PDSCH, since we haven’t discussed the case of dynamic GC-PDSCH yet. If ACK/NACK or NACK-only is configured for a G-RNTI and SPS-config is associated with the G-RNTI (agreed in 8.12.1), no further configuration in SPS-config is needed. Regarding ZTE’s comment, it is not clear to us. If all G-RNTIs share same CB configurations, does it mean configuration of ACK/NACK or NACK-only are common for all G-RNTIs as well? |
| Ericsson | Ok |
| NTT DOCOMO | Support |
| ZTE | @Qualcomm, yes, configuration of ACK/NACK or NACK-only is common for all G-RNTIs as well. Based on our understanding, type-1 ACK/NACK, type-2 ACK/NACK and NACK-only are different feedback options. If RAN1 is going to agree that the same configuration of type-1 ACK/NACK or type-2 ACK/NACK is configured for all G-RNTIs, we can follow the same logic here and support the same HARQ-ACK feedback option for all SPS configurations. |
| Spreadtrum | Support |

#### Proposal 2.4.1-1

When UE supports and is configured with more than one G-RNTI,

  - for Type-2 codebook construction, DAI is separately counted per G-RNTI.

  - Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each RNTI following the ascending order of the G-RNTI value.

| Company | Comments |
| --- | --- |
| FL’s note | This proposal is not changed from Round 1 which is straightforward and stable. |
| Samsung | OK. |
| CMCC | ok |
| Lenovo, Motorola Mobility | OK |
| CATT | OK |
| Nokia, NSB | Ok. |
| vivo | ok |
| LG | We are fine with this proposal. |
| ZTE | OK |
| Qualcomm | fine |
| Ericsson | Ok |
| NTT DOCOMO | Support |
| Spreadtrum | OK |

# Proposals for 3nd checkpoint for GTW on 27th (closed)

The remaining proposals in this section are expected to progress, so please continue the discussion.

#### Proposal 4.3.2-1-r3 (H)

Confirm the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback with the update in tracking change as follows:

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

- UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details.

- Note: It is up to network implementation to avoid the C-DAI/T-DAI mismatch between different UEs in the same multicast group

| Company | Comments |
| --- | --- |
| FL’s note | ZTE suggested a note to address ZTE’s concern and also hopefully can address others concerns as well. Let’s see more views for this round |
| vivo | Sorry, we cannot agree with this updated proposal from ZTE. Until now, the solution to avoid this DAI issue we heard either is to prohibit gNB to configure case 5/7, or to limit that the DCI indicators should be the same when they pointed to the same PUCCH slot. Neither solution make sense to us. Why case 5/7 is not a reasonable configuration without considering the DAI issue comparing to the case 4/6? From our point of view, case 5/7 can be typical cases, e.g, gNB enables HARQ-ACK feedback for some UEs based on channel condition and enable the DCI function for the other UEs. For the second solution, we cannot understand why the enabling/disabling should be the same in a PUCCH slot level. We think gNB can indicate the enabling/disabling per HARQ process. The current logic seems to preclude all cases by gNB where we find the WA is not workable. If we find more cases that have some issues with the WA, will we preclude these cases too? I just think it is a little tricky. The WA is questionable in some cases, we just to preclude the cases, rather than to update the WA to make it more reasonable. In addition, we really cannot find any meaningful use case for the DCI indication. In the last meeting, we had concern on the WA, but for process we made compromise considering that in the multiple G-RNTI case, if HARQ-ACK enabling/disabling is RRC configured per G-RNTI, then UE determines whether to feed back HARQ-ACK based on G-RNTI. It can be treated as group-common DCI indicates implicitly whether ACK/NACK based HARQ-ACK feedback is enabled/disabled. But now we find companies totally have different views and understandings on this WA. There are many disadvantages for the RRC+ DCI dynamic indication. - No apparent use cases for DAI dynamic indication for a certain service are found. - is not workable in some RRC configuration cases, i.e. the DAI issue - for the DCI dynamic indication, it will be very complex to handle at the UE side and requires a lot of implementation effort as the UE needs to accommodate two scenarios for each case which will complicate the implementation. - Make the spec more complicate. - Introduce more workload to design the DCI indication mechanism. So, we really hope companies can reconsider the reasonability of the WA. |
| Qualcomm | We think the “enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback” is subject to UE capability, because the CB generation need to be changed, considering dynamically disabled ACK/NACK feedback. Only if all the UEs in a group are capable of supporting the enabling/disabling function and are configured with the function, GC-DCI based enabling/disabling is applied to all UEs and it may be possible to avoid DAI mismatch between different UEs by gNB scheduling; otherwise, it cannot. Before confirming WA, the following revision is needed: - … - Notallow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback. - FFS whether/how to apply it to SPS group-common PDSCH. - UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details. - Note: It is up to network implementation to avoid the C-DAI/T-DAI miss-match between different UEs in the same multicast group |
| Nokia, NSB. | We support the WA. The logic of using DAI fields is to group several HARQ bits into one Type-2 codebook. Thus, why would the gNB configure some of those corresponding PDSCH transmissions with “HARQ feedback enable” and some others with “HARQ feedback disable”? This does not make any sense. If PDSCHs are being grouped and counted by DAI, we see no reason why the gNB would disable feedback for one of these PDSCHs. Why wouldn’t case 5 and case 7 work is also not clear to us. WA states that some UEs can be configured to use DCI indication. If some other UEs are not configured to use DCI indication, those UEs are configured via RRC signalling whether to send HARQ feedback. Therefore, in case 5, RRC enabling is for the latter UEs, and DCI indicating is for the former. We think case 7 is not more problematic, too. Since we have a group-common DCI, the DAI field should be arranged for the UEs whose HARQ feedback is enabled. Another solution would not work. If one thinks that case 5 and 7 cannot work, we wonder how companies think case 4 would. Also, in case 4, same group-common DCI is received by the UEs and some UEs have their HARQ feedback enabled, some others have their HARQ feedback disabled. The current WA supports both cases 5 and 7. We also believe that the DCI based group-common enabling / disabling is not an ideal method, since we found out that there are several cases where the UEs should be individually configured to send feedback or not due to several reasons mentioned in our Tdocs and some other companies herein (e.g. Lenovo above). However, this WA brings compromise for both RRC and DCI supporters, since we spent several meetings and neither side persuaded the other. |
| ZTE | Based on our understanding, we have the following way forwards to address the issue here. W1: Revert the working assumption and only allow RRC-based HARQ-ACK enabling/disabling. W2: Confirm the working assumption and specify some mechanisms to address the DAI miss-match issue between different UEs in the same multicast group. W3: Confirm the working assumption and it is up to network implementation to address the DAI miss-match issue between different UEs in the same multicast group. Considering that the current working assumption is a compromised solution, W1 may not be a good way to go. Considering the late stage of Rel-17, W2 may not be an appropriate way to go. Then, it seems W3 is the only compromised way to go. Thus, we are fine with the FL proposal above. |
| FL’s response | Updated per QC’s some comments. The comment that “not allow” UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback may need further discussion, it can be FFS after WA is confirmed. |

#### Proposal 4.3.2-2-r2 (H)

The enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI per UE.

| Company | Comments |
| --- | --- |
| FL’s note | This proposal is not changed but with minor editorial adjustment and I don’t plan to make substantial change either for example changing the function of … into HARQ-ACK states as some company suggested in case causing more ambiguity or more round discussion for clarification though the basic meaning is unchanged. To me, this proposal is quite clear. Even though the signaling is RRC per UE, if some companies think it is not reasonable to configure different something different for different UE within the group, then network still have the freedom to configure the same for all UEs. From FL’s perspective, it is a good enough compromise for everybody to move forward. |
| Qualcomm | In NTN, it has been agreed that the enabling/disabling HARQ-feedback is configured per HPID. The dynamic enabling/disabling can be implicitly indicated by the scheduled HPID, no need to add additional field in DCI. Same logic can be applied to GC-DCI. Also, it also ensure same enabling/disabling for initial/retx per HPID. For multicast HARQ-ACK for CONN UEs, we have to use unicast RRC signaling. For the function configuration aligned among UEs, we think it is similar as CFR configuration, where gNB will indicate common parameters required for multicast via unicast RRC. We suggest revising “per G-RNTI per UE” to “per HARQ process ID via unicast RRC signalling” |
| Nokia, NSB. | We support the proposal. We do not support the per HARQ process ID solution from Qualcomm. We believe it does not bring any technical benefits on top of what is in the current proposal. |
| LG | We are fine with this proposal. |
| FL’s response | The proposal is not updated and I guess the vast majority may tend to keep the proposal as is. |

#### Proposal 4.3.2-4-r4 (H)

If the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is not configured, whether UE feedbacks ACK/NACK or not is configured per G-RNTI by unicast RRC signalling.

| Company | Comments |
| --- | --- |
| FL’s note | This proposal is not changed but with minor editorial adjustment. Similar to the previous one, to me, this proposal is quite clear. Even though the signaling is RRC per UE, if some companies think it is not reasonable to configure different something different for different UE within the group, then network still have the freedom to configure the same for all UEs. |
| Qualcomm | Fine in general. Minor comment: “per UE by RRC signalling” can be simplified to “by unicast RRC signalling” |
| Nokia, NSB | Support. |
| LG | We are fine with this proposal. |
| FL’s response | The proposal is updated per QC’s suggestion. |

#### Proposal 5.1-2-r1 (H)

For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS PDSCH without PDCCH scheduling, down-select from the following alternatives:

  - Alt1: HARQ-ACK feedback option is configured per SPS configuration index.

  - Alt2: HARQ-ACK feedback option is indicated in the SPS activation DCI.

  - Note: enabling/disabling HARQ-ACK feedback for multicast SPS can be discussed separately.

| Company | Comments |
| --- | --- |
| FL’s note | This proposal was not discussed on GTW and let’s see whether it can be agreed in this meeting. Please continue the discussion if you have not provide views or you want to keep following-up. |
| Samsung | OK. |
| CMCC | OK |
| Lenovo, Motorola Mobility | OK |
| CATT | OK with this proposal. |
| Nokia, NSB | Ok. |
| vivo | ok |
| LG | We are fine with this proposal. |
| ZTE | We have a proposal that pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList for multicast is applied to all G-RNTIs configured to UE. To us, NACK-only based codebook and ACK/NACK based codebook are different codebooks, if we follow the same approach, we need to apply the same feedback mechanism for all SPS configurations. #### Proposal 2.1.3-1-r1 (H) - When UE is configured with the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList for multicast, it is applied to all G-RNTIs configured to UE. FL’s response: this proposal only talks about how ACK/NACK or NACK-only based is chosen for SPS. it seems straightforward because for SPS PDSCH without PDCCH scheduling , there is no way to switch between them dynamically. The only way is to be configured by RRC per SPS configuration. |
| Qualcomm | We think it may be too early to discuss HARQ-ACK feedback mode selection for SPS GC-PDSCH, since we haven’t discussed the case of dynamic GC-PDSCH yet. If ACK/NACK or NACK-only is configured for a G-RNTI and SPS-config is associated with the G-RNTI (agreed in 8.12.1), no further configuration in SPS-config is needed. Regarding ZTE’s comment, it is not clear to us. If all G-RNTIs share same CB configurations, does it mean configuration of ACK/NACK or NACK-only are common for all G-RNTIs as well? FL’s response: it is a bit different from dynamical scheduling which could possibly have RRC or DCI based switch. However, for SPS PDSCH without PDCCH scheduling, there is no other way except RRC seemingly. Also, SPS-config should have its own G-CS-RNTI not sure what you meant by “associated with G-RNTI so no further configuration is needed” [QC] Sorry for the confusion. What I mean is a SPS-config is associated with G-CS-RNTI (as agreed in 8.12.1). If we configure HARQ-ACK feedback mode per G-CS-RNTI by RRC signalling, no need to add additional feedback mode configuration in SPS-Config associated with the G-CS-RNTI. If HARQ-ACK feedback mode can be dynamically indicated in GC-DCI, it is possible to use SPS activation GC-PDCCH indicate the feedback selection for the scheduled SPS GC-PDSCH without PDCCH. So we think it is too early to agree with Alt1. For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS PDSCH without PDCCH scheduling, support: - Alt1: HARQ-ACK feedback option is configured per SPS configuration index - Note: enabling/disabling HARQ-ACK feedback for multicast SPS can be discussed separately. |
| Ericsson | Ok |
| NTT DOCOMO | Support |
| ZTE | @Qualcomm, yes, configuration of ACK/NACK or NACK-only is common for all G-RNTIs as well. Based on our understanding, type-1 ACK/NACK, type-2 ACK/NACK and NACK-only are different feedback options. If RAN1 is going to agree that the same configuration of type-1 ACK/NACK or type-2 ACK/NACK is configured for all G-RNTIs, we can follow the same logic here and support the same HARQ-ACK feedback option for all SPS configurations. FL’s response: ZTE supported feedback HARQ-ACK or not feedback can be configured per G-RNTI, why here seemingly meant applied to G-RNTI? |
| Spreadtrum | Support |
| ZTE | @FL, we don’t have a strong concern here since it any gives more flexibility for network. We just want to understand why companies prefer to apply different logics for configuring the same feedback option (type-1 vs type-2 codebook) for DG PDSCH for all G-RNTIs, while configuring different feedback option (type-1 vs type-2 codebook vs NACK-only) for CG PDSCH for each G-CS-RNTI? |
| FL’s response | @ZTE I see your point now. Multiplexing different CB is the concern to configure CB type per G-RNTI. However, choice between ACK/NACK or NACK-only may be different from CB type configuration. @all, Proposal is updated to address QC’s comment. Now we have two alternatives for further down-selection. |

# Proposals for 3rd checkpoint via email (new)

Based on the online discussion today on GTW, there are two remaining related proposals which has been discussed since the beginning of the meeting. Let’s try whether we can converge before the official meeting deadline 27th UTC 16:59

#### Proposal 4.3.2-4-r4 (H)

If the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is not configured, whether UE feedbacks ACK/NACK or not is configured per G-RNTI by unicast RRC signalling.

| Company | Comments |
| --- | --- |
| FL’s note | This proposal was discussed online and Lenovo commented the signaling should not be unicast RRC signaling, but instead should be SIB or MCCH. Please companies express your views. |
| LG | We are find with this proposal. To our understanding, UE in RRC_CONNECTED is mostly configured by dedicated signalling. In addition, when Rel-15 UE cannot receive broadcast SIB, if required (e.g. when UE active BWP cannot accommodate broadcast SIB1), gNB can provide SIB based cell specific information by dedicated signalling. In any case, whether to use SIB or MCCH at all times instead of UE dedicated signalling should be left to RAN2 decision. |
| Nokia, NSB | We think that the enabling / disabling should be done per UE, because there are various use cases that would significantly benefit from this, as we have been discussing since several meetings, e.g., disabling the HARQ feedback of outlier UEs, disabling the HARQ feedback of UEs in very good channel conditions, etc. . In our view, the DCI size problem does not exist and only option is that the UE ignores the fields related to HARQ-ACK feedback, when their HARQ-ACK feedback is disabled. Since the DCI is a group-common DCI, those fields would anyway be signaled for the other UEs whose HARQ feedback is enabled, so there is no decrease in spectral efficiency of the system. Based on the need for per UE configuration and no impact on downlink spectral efficiency, we believe that the current proposal should be agreed. |

#### Proposal 4.3.2-2-r2 (H)

The enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI per UE.

| Company | Comments |
| --- | --- |
| FL’s note | There was comment that the configuration can be per HARQ process. Based on the agreed update of the working assumption. Let’s check whether this proposal can be agreeable. |
| LG | We are fine with this proposal. |
| Nokia, NSB | We support the proposal. |
| Lenovo | Since HARQ-ACK feedback is quite relevant to DCI format design and DCI format design is discussed in AI8.12.1, it may be better to discuss in AI8.12.1 on each possible DCI field then we can discuss whether the HARQ related fields are configured per UE or per G-RNTI. |
| LGE | As you replied to my previous email, we can check how RAN2 signaling will be designed. Meanwhile, I would like to point out that if SIB or MCCH is not transmitted within UE active BWP but UE in RRC_CONNECTED needs to receive SIB or MCCH for this configuration, UE receiving multicast in one CFR associated to UE active BWP may need to be configured with additional CFR for reading SIB or MCCH. Otherwise, this UE may need to switch to initial BWP to receive SIB or MCCH. |
| FL‘s responses | Regarding the debating whether the signaling is per UE RRC signaling or could be SIB/MCCH, I checked with RAN2 and get agreement from RAN2 as follows: Basically, it means for UEs receiving multicast in RRC_CONNECTED states, the relevant MBS configuration is sent by gNB via RRC Reconfiguration message, which clearly is per UE RRC signaling. According to RAN2’s agreement, the Proposal 4.3.2-4-r4 (H) and Proposal 4.3.2-2-r2 (H) should be agreeable. |
| Lenovo | @Jinhuan: Based on this RAN2 agreement, it said, when a UE in connected mode joined the multicast session when the session is started, gNB sends RRC reconfiguration message to the UE with MBS configuration, it is not clear whether the MBS configuration including the HARQ-ACK enabling/disabling configuration is per UE or per G-RNTI. Assuming the multicast configuration includes the HARQ-ACK enabling/disabling configuration per G-RNTI (not per UE), when a new UE joins the session, gNB sends the multicast configuration to the UE via dedicated RRC signaling. It does not imply this dedicated RRC signaling will configure a different HARQ-ACK configuration to the UE from other UEs which are ongoing in the session. On the other hand, using dedicated RRC signaling does not imply the configuration is per UE. In TS38.331, UE dedicated RRC signaling can be also used to indicate cell-specific configuration with same info as common signaling. @David: Assuming there are 100 UEs in a group where one UE is configured with HARQ-ACK feedback and other 99 UEs are not configured with HARQ-ACK feedback, then all the 99 UEs have to monitor a DCI with 10 HARQ related fields which are totally useless to them. What’s benefit can these 99 UEs obtain? On the other hand, from each UE’s perspective, if it is configured without HARQ-ACK feedback, how can it know other UEs’ HARQ-ACK feedback option with per UE configuration? Should it always assume HARQ-ACK fields included in the DCI? |
| MTK | I totally agree with Haipeing’s view. UE dedicated RRC signaling also can be used to configure the common information for all group UE. Regarding the David comment that “We think that the enabling / disabling should be done per UE….”, I am confused that why legacy UE cannot support this option, however, for MBS UE, does it need to support this option? Just because it is for group communication? (Note: For Rel-16 NR V2X, it also supports groupcast communication, however, the HARQ feedback mechanism is the same for all UEs within the same group) |
| FL’s response | Thanks for the comments and discussion. How about the proposals go as follow? Proposal 4.3.2-4-r4 (H) If the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is not configured, whether UE feedbacks ACK/NACK or not is configured per G-RNTI by unicast RRC signalling. Proposal 4.3.2-2-r3 (H) The enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI per UE by unicast RRC signalling. |
| Ericsson | Thanks Jinhuan for the proposal. The proposals are fine with us. For Proposal 4.3.2-2-r3 (H) , we could add an FFS regarding further RRC configuration of the DCI indicator. Once the enabling disabling function has been configured, the “HARQ feedback states” (i.e. NACK only, ACK/NACK, disabled) need to be mapped to the DCI indicator (unless we use more than 1 bit). |
| FL’s response | Thanks for the comment. I hesitate to add the FFS you suggested is because whether the DCI indicator is one existing field or newly added field is still open. If it is the former, probably no need to have such RRC configuration. It can/will be discussed anyway. We are running very late and approaching deadline soon, so I suggest we just try the following proposals based on the agreed update of the WA. If there is still objection, I will give them up for this meeting. Thanks for the understanding. Proposal 4.3.2-4-r4 (H) If the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is not configured, whether UE feedbacks ACK/NACK or not is configured per G-RNTI by unicast RRC signalling. Proposal 4.3.2-2-r3 (H) The enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI per UE by unicast RRC signalling. |
| Lenovo | Thanks for the continuing effort. Regarding Proposal 4.3.2-4-r4 (H) and Proposal 4.3.2-2-r3 (H), we don’t support per UE configuration. Our concern was provided last night while not getting any feedback. |
| Conclusion from Chair | Even though we may be quite close to consensus on the two proposals listed by Jinhuan, I am not comfortable endorsing these without giving more time for everyone to check. I think it is best to wait until the next meeting and hopefully we can make agreements on these smoothly at the next meeting. And the discussions should provide enough insight on the general direction so that the progress in the next meeting will not be slowed down due to the lack of agreement on these this time. Thanks to all of you for you hard work and perseverance during this meeting. We can now close this thread. |

# References

R1-2106439	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2106624	Discussion on mechanisms to improve reliability for RRC_CONNECTED Ues	vivo

R1-2106663	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2106717	Mechanisms to improve MBS reliability for RRC_CONNECTED Ues	Spreadtrum Communications

R1-2106746	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2106913	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2106946	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT, CBN

R1-2106997	HARQ-ACK feedback scheme for multicast of RRC_CONNECTED Ues	ETRI

R1-2107094	Further Discussions on Reliability for RRC_CONNECTED UEs	FUTUREWEI

R1-2107138	Discussion on Reliability Improvements for RRC_CONNECTED UEs	NEC

R1-2107161	On reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2107230	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

R1-2107370	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2107383	Discussion on MBS reliability for RRC_CONNECTED UEs	Google Inc.

R1-2107426	Discussion on reliability improvement	CMCC

R1-2107457	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2107515	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	MediaTek Inc.

R1-2107612	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

R1-2107764	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

R1-2107882	Discussion on mechanisms to improve reliability for multicast for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2107951	Reliability related discussion for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2108027	Discussion on reliability enhancement for RRC_CONNECTED UEs	Convida Wireless

R1-2108171	Mechanisms to improve reliability for RRC_CONNECTED Ues	Ericsson

# Appendix Agreements summary for AI 8.12.2

## 102e

Agreements:

For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

## 103e

Agreements:

For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources.

  - FFS details including conditions for it to be used

- NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, further down-select between:

    - FFS: PUCCH resource configuration for NACK only feedback.

  - FFS details including conditions for it to be used

- To decide in RAN1#104-e whether or not to support only one or both of the above schemes

  - If both are supported, FFS configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

Agreements:

For RRC_CONNECTED UEs receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

Agreements:

For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Agreements:

Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- FFS: Option 4: MAC-CE indicates enabling/disabling

- FFS: Option 5: RRC configures the enabling/ disabling function and MAC-CE indicates enabling /disabling

Agreements:

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI

- FFS: Opt 4: by MAC-CE

- FFS: Opt 5: by RRC+MAC-CE

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Agreements:

From the perspective of RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 initial transmission, retransmission supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

Agreements:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

- SRS configuration

Agreements:

For ACK/NACK based HARQ-ACK feedback if supported, both Type-1 and Type-2 HARQ-ACK codebook are supported for RRC_CONNECTED UEs receiving multicast,

- FFS details of HARQ-ACK codebook design.

- FFS whether enhanced Type-2 and/or Type-3 HARQ-ACK codebook is supported or not.

## 104e

Agreement:

For ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies.

Agreement:

The priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast can be,

- Lower, higher than or equal to the HARQ-ACK feedback for unicast

  - FFS: How to reflect the priority in specification, e.g., whether it is configured or indicated to the UE

  - FFS: The total number of priorities across multicast and unicast

- FFS the priority between HARQ-ACK feedback for multicast and other UCI for unicast (SR, CSI) or PUSCH for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1,

- DAI for unicast and DAI for multicast are separately counted.

- Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported.

  - FFS details on   the codebooks.

- FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

Agreement:

For RRC_CONNECTED UEs receiving multicast, support the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - It is up to network to configure orthogonal PUCCH resources among UEs within the same group.

- FFS: NACK-only based HARQ-ACK feedback for multicast,

  - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group.

- FFS details.

Agreement:

For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource,

- Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot.

  - FFS whether it is subject to UE capability.

- FFS the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot.

- FFS whether sub-slot based PUCCH transmission for HARQ-ACK is supported.

- FFS the case of HARQ-ACK feedback for multicast and other UCI for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast.

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported.

- FFS: whether/how to optimize the Type-1 codebook construction to reduce the HARQ-ACK feedback payload size.

Agreement:

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast,

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- FFS: whether down-selection between option 3 and option 2 is needed or support the both options.

- FFS: enabling/disabling by MAC-CE.

Agreement:

For slot-level repetition for group-common PDSCH for RRC_CONNECTED UEs receiving multicast,

- (Config A) UE can be optionally configured with pdsch-AggregationFactor.

- (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table.

- If UE is configured with Config B, UE does not expect to be configured with Config A for the same group-common PDSCH.

## 104be

Agreement:

Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

Agreement:

Two priority indexes are introduced for multicast, with

- Index 0 meaning low priority and index 1 meaning high priority.

- Priority index can be included in DCI formats scheduling the group-common PDSCH.

  - FFS details for DCI formats.

- FFS: the priority comparison between multicast and unicast with the same priority index.

Agreement:

For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively.

- FFS other configurations

Agreement:

For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource,

- The first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast.

- FFS: The number of Type-2 HARQ-ACK sub-codebooks for multicast.

- Note: The case of SPS PDSCH will be discussed separately.

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;

## 105e

Agreement:

- The signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast, respectively, at least for the case of different priorities, at least for Type-2 HARQ codebook

- FFS: The case for the same priority.

- FFS: The case of Type-1 HARQ codebook

- FFS: Whether this applies to separate PUCCH transmissions only

Agreement:

The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback.

Agreement:

NR supports at least the following cases for UE supporting multicast:

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast with different priorities in a slot subject to UE capability.

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast with different priorities, respectively, in a slot subject to UE capability.

Agreement:

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast with the same priority from the same TRP, support

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

- FFS: If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4).

  - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

Agreement:

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on down-selecting one of the two alternatives as follows:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

- Companies are encouraged to continue discussion of pros and cons for each alternative for further down-selection in the next meeting.

Agreement:

For ACK/NACK based HARQ-ACK feedback for multicast, the multiplexing/prioritizing rule between the HARQ-ACK for multicast and SR/CSI can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and SR/CSI.

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

- Agreement:

- Support PUCCH format 0 and format 1 for NACK-only based HARQ-ACK feedback for multicast.

Agreement:

Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling.

- FFS for SPS activation/deactivation.

Conclusion:

PUCCH resource for NACK-only can be shared by UEs transmitting the NACK-only based HARQ-ACK feedback.

Agreement:

For support of ACK/NACK based HARQ-ACK feedback for SPS multicast,

- the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH is included in the configuration for SPS multicast.

  - UE determines a priority index from the HARQ-ACK codebook index

- UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS multicast configurations. Otherwise, a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations.