3GPP TSG RAN WG1 #106-e	R1-2108438

e-Meeting, August 16th – 27th, 2021

Title:	Draft Reply LS on Resource Reselection Trigger sl-reselectAfter

Response to: 	R2-2106625

Release:	Release 16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	   Chunxuan Ye

Email Address:  chunxuan_ye@apple.com

Send any reply LS to:	 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# Overall description

RAN1 thanks RAN2 for the LS on resource reselection trigger sl-reselectAfter. RAN1 would like to provide answers to the following questions:

Question 1: In NR V2X, whether “unused retransmission opportunities in case of HARQ feedback is enabled” shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection?

Reply to Question 1: Unused retransmission opportunities, due to the reception of sidelink ACK (or, no reception of sidelink NACK in case of NACK-only HARQ feedback), shall NOT be counted towards consecutive unused transmission opportunities for resource reselection trigger.

Question 2: In NR V2X, according to RAN1 agreement, is it the correct understanding that only the resources already indicated in SCI shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection?

Reply to Question 2: Only if none of the resources of a grant in a resource reservation period is used for (re)transmission, no matter whether they are indicated in SCI, it shall be counted towards one of consecutive unused transmission opportunities to trigger resource reselection.

# Actions

To RAN WG2:

ACTION:  RAN1 kindly asks RAN2 to take above information into account for their future work.

# Dates of Next TSG-RAN WG1

TSG RAN WG1 Meeting #106b-e		  11 -- 19 October, 2021		    eMeeting

TSG RAN WG1 Meeting #107-e		  11 -- 19 November, 2021		  eMeeting