3GPP TSG RAN WG1 #106-e	R1-2108633

e-Meeting, August 16th – 27th, 2021

Title:	LS on Rel-17 inter-cell multi TRP

Response to:

Release:	Rel-17

Work Item:	NR_FeMIMO

Source:	RAN1

To:	RAN2

CC:

# Contact Person:

# Name: Rakesh Tamrakar

E-mail Address: rakesh@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1. Overall Description:

For inter-cell multi TRP operation in Rel-17 FeMIMO, RAN1 has reached following agreements and conclusions in past few meetings:

Agreement from RAN1#103-e

Agreement

The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information

Agreements from RAN1#104-e

Agreement

Non-serving cell information at least includes non-serving cell PCI to support inter-cell multi-DCI multi-TRP operation

- FFS: Whether the indication of PCI is implicit or explicit

Conclusion

Reuse Rel-15/16 QCL rule between the source and target RS/channel for non-serving cell RS/channel.

Agreement

At least following non-serving cell SSB information are needed in inter-cell MTRP operation

- SSB time domain position

- SSB transmission periodicity

- SSB transmission power

FFS: Other non-serving cell information

FFS: Whether indication of these information is implicit or explicit

Agreements from RAN1#104b-e

Agreement

- For intercell MTRP operation, 1 additional PCI different from the serving cell PCI is supported per CC

  - The additional PCI is the one associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH, per CC.

  - Applicable at least for non-cross carrier QCL indication

    - FFS: Cross carrier scheduling QCL indication

- RAN1 to decide on the maximum number of PCIs different from the serving cell PCI per CC and/or across all CCs that can be RRC-configured for multi-DCI based inter-cell multi-TRP

- Above should be specified by reusing R15 QCL rules as concluded in RAN1#104-e

Conclusion

Configuration of CSI-RS for mobility as QCL source for intercell MTRP operation is not supported from Rel-17 specification point of view

Agreements from RAN1#106-e

Agreement

Introduce a new RRC indicator/signaling (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with, where the new indicator/signaling is not the exact PCI value

- Detailed signaling design is up to RAN2

Agreement

- Rel. 17 inter-cell MTRP, the maximum number of additional RRC -configured PCIs  per CC is denoted X and can be reported as a UE capability

  - For the report value of X, multiple candidate values including 1 is supported.

    - FFS : Which values to support other than 1.

    - Values larger than 7 are precluded

    - RAN1 needs to agree on value(s) of X other than 1

- Down-select one of the following alternatives:

  - Alt 1: A single value of X is reported as UE capability for any possible SSB time domain position and periodicity

  - Alt 3: At least Two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB

- The serving cell PCI is always associated with active TCI states, only 1 additional PCI can be associated with the active TCI States

Agreement

- For inter-cell mTRP, one PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with one CORESETPoolIndex , another PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with another CORESETPoolIndex

  - FFS: The association between PCI and CORESETPoolIndex when switching between intra-cell mTRP and inter-cell mTRP

RAN1 is aware of the terminology “non-serving cell” is not proper as mentioned in respective LS in R2-2106787, the terminology “non-serving cell” above means “cell having TRP with different PCI from serving cell”.

# 2. Actions:

To RAN2

ACTION:

RAN1 would like to respectfully request RAN2 to take the above agreements into account while designing necessary signaling.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #106b-e     11th – 19th Oct. 2021                                emeeting

TSG-RAN WG1 Meeting #107-e      11th – 19th Nov. 2021                                emeeting