3GPP TSG RAN WG1 #106bis-e                                        R1-2108850
e-Meeting, October 11th – 19th, 2021

Source: 	ZTE Corporation

Title: 	Performance impacts of residual frequency error for joint channel estimation of PUSCH and PUCCH transmissions

Agenda item:	8.8.4

Document for:	Discussion/Decision

Introduction

In RAN1#104-e, it was agreed to take residual frequency error into account when performing evaluation on joint channel estimation [1].

Agreements: - For joint channel estimation. - Take into account the residual frequency error, e.g., +/- 0.1 ppm as upper bound. - Companies can report other values and frequency error model.

Based on RAN4 agreements in [2], it is agreed in RAN4 that the tolerance of phase continuity and amplitude consistency for joint channel estimation will be further investigated.

In this contribution, we provide some evaluation results regarding the tolerance of phase continuity and amplitude consistency for joint channel estimation for PUSCH and PUCCH transmissions. Note that, this contribution is a re-submission of R1-2106744 in RAN1#106-e.

Evaluation results

  - Impacts of residual frequency error

- Evaluation model

Denote  as the baseband signal and  as the signal after modulation to the carrier frequency. Then,  can be given as

Assuming the residual frequency error (RFE) due to imperfect oscillator at the receiver is , the received signal  after demodulation to the carrier frequency would be:

Regarding, Uniform distribution with [-0.1,+0.1] ppm is assumed in our simulation. For PUSCH/PUCCH transmissions with repetition, the RFE is added per repetition basis. That is, for each repetition, a separate RFE is randomly generated from a uniform distribution and added to (multiplied by ) the received time domain signal.

- Evaluation results of PUSCH

The simulation results for cross-slot joint channel estimation with and without RFE for PUSCH are given in this section. PUSCH with 8 repetitions are provided, and no frequency error correction at receiver side is assumed in our simulations.

In Figure 1 and Figure 2, the performance impact with taking RFE into account is provided for 700MHz Rural scenario and 4GHz Urban scenario respectively. The detailed simulation assumptions are provided in Table A-1 in the appendix.

Figure 1. Performance of joint channel estimation for PUSCH w/ or w/o RFE in 700MHz Rural scenario

As can be observed in Figure 1, the performance difference between the case with or without RFE in 700MHz Rural scenario is very small. That is, the performance impact due to RFE is negligible in 700MHz Rural scenario.

Observation 1: The performance impact due to residual frequency error for joint channel estimation for PUSCH is negligible in 700MHz Rural scenario.

Figure 2. Performance of joint channel estimation for PUSCH w/ or w/o RFE in 4GHz Urban scenario

As can be observed in Figure 2, the performance degradation due to RFE in 4GHz Urban scenario is very large under the used simulation assumptions. At target BLER of 0.1, the performance degradation is about 4 dB, and it would get larger with a lower BLER target.

Observation 2: The performance impact due to residual frequency error for joint channel estimation for PUSCH could be significant in 4GHz Urban scenario, with about 4 dB performance loss @BLER = 0.1 observed.

- Evaluation results of PUCCH

The simulation results for joint channel estimation with and without RFE for PUCCH are shown in Figure 3 and Figure 3 for 700MHz and 4GHz respectively. No frequency error correction at receiver side is assumed in our simulations.

Figure 3: Performance of joint channel estimation for PUCCH w/ or w/o RFE in 700MHz Rural scenario

In 700MHz Rural scenario, 8 PUCCH repetitions are assumed, and different bundle size (BS) are simulated. When the BS=1, it means no joint channel estimation is applied. BS = {1,2,4,8} are simulated when frequency hopping (FH) is disabled, while BS = {2,4} when FH is enabled. From the simulation results shown in Figure 3, the performance degradation due to RFE is negligible for all simulated cases in 700MHz Rural scenario.

Observation 3: The performance impact due to residual frequency error for joint channel estimation for PUCCH is negligible in 700MHz Rural scenario.

Figure 4: Performance of joint channel estimation for PUCCH w/ or w/o RFE in 4GHz Urban scenario

In 4GHz Urban scenario, 4 repetitions are assumed and different bundle size are simulated. BS = {1,2} are simulated when FH is disabled, while BS=2 when FH is enabled. From the simulation results shown in Figure 4, the performance degradation due to RFE is significant. About 1~2.5dB performance loss is observed @ BLER=0.01 in our simulation for the simulated cases.

Observation 4: The performance impact due to residual frequency error for joint channel estimation for PUCCH is significant in 4GHz urban scenario, with about 1~2.5dB performance loss @BLER = 0.01 observed.

  - Phase tolerance level

Given the potential large impact due to RFE, the acceptable tolerance from network performance perspective should be evaluated in a quantitative manner. In this section, the simulation cases listed in Table 1 are evaluated to check the impacts of phase discontinuity on the PUSCH performance and detailed simulation assumptions are listed in the appendix.

Table 1. Evaluation cases for phase continuity

| Configurations | Simulation cases |
| --- | --- |
| Phase discontinuity across 2 repetitions 700MHz/4GHz | 2 repetitions Phase error for the first repetition: 0o Relative phase error for the second repetition : 10o: 20o: 150o. The baseline is when the relative phase error is 0o. |

Preliminary simulation results for PUSCH at 700MHz and 4GHz are provided in Figure 5 and Figure 6 respectively. It can be found that phase error could cause significant performance degradation for PUSCH, and the acceptable phase error between different repetitions should be at least within 30o for both 700MHz and 4GHz.

Figure 5. The performance impact of phase error on joint channel estimation for PUSCH for 700MHz

Figure 6. The performance impact of phase error on joint channel estimation for PUSCH for 4GHz

Observation 5: Phase error could cause significant performance degradation for PUSCH, and the acceptable phase error between different repetitions should be at least within 30o for 700MHz and 4GHz.

Proposal 1: The acceptable phase error between different repetitions for PUSCH should be at least within 30o for both 700MHz and 4GHz.

  - Amplitude tolerance level

In this section, the performance impact of power inconsistency (i..e., amplitude tolerance) across different repetitions is evaluated. The simulation results for the cases listed in Table 2 are provided. Detailed simulation assumptions are listed in the appendix.

Table 2. Evaluation cases for phase continuity

| Configurations | Simulation cases |
| --- | --- |
| Power inconsistency across 2 repetitions 700MHz/4GHz | 2 repetitions Amplitude error for the first repetition: 0o Relative amplitude error for the second repetition : 0dB: 0.5dB: 2dB, where the baseline is when the relative amplitude error is 0. |

Preliminary simulation results for PUSCH at 700MHz and 4GHz are provided in Figure 7 and Figure 8 respectively. It can be found that 0.5dB amplitude error will cause large performance degradation (e..g at least 1.5-2dB deterioration @10% BLER), the acceptable amplitude error between different repetitions should be less than 0.5dB.

Figure 7. The performance impact of amplitude error on gNB receiver performance for 700MHz

Figure 8. The performance impact of amplitude error on gNB receiver performance for 4GHz

Observation 6: Amplitude error could cause significant performance degradation for PUSCH, and 0.5dB amplitude error could cause about 1.5-2dB deterioration at 10% BLER for both 700MHz and 4GHz.

Proposal 2: The acceptable amplitude error between different repetitions should be at least less than 0.5dB for both 700MHz and 4GHz.

Conclusion

According to the analysis given above, we have the following observations and proposals:

Observation 1: The performance impact due to residual frequency error for joint channel estimation for PUSCH is negligible in 700MHz Rural scenario.

Observation 2: The performance impact due to residual frequency error for joint channel estimation for PUSCH could be significant in 4GHz Urban scenario, with about 4 dB performance loss @BLER = 0.1 observed.

Observation 3: The performance impact due to residual frequency error for joint channel estimation for PUCCH is negligible in 700MHz Rural scenario.

Observation 4: The performance impact due to residual frequency error for joint channel estimation for PUCCH is significant in 4GHz urban scenario, with about 1~2.5dB performance loss @BLER = 0.01 observed.

Observation 5: Phase error could cause significant performance degradation for PUSCH, and the acceptable phase error between different repetitions should be at least within 30o for 700MHz and 4GHz.

Proposal 1: The acceptable phase error between different repetitions for PUSCH should be at least within 30o for both 700MHz and 4GHz.

Observation 6: Amplitude error could cause significant performance degradation for PUSCH, and 0.5dB amplitude error could cause about 1.5-2dB deterioration at 10% BLER for both 700MHz and 4GHz.

Proposal 2: The acceptable amplitude error between different repetitions should be at least less than 0.5dB for both 700MHz and 4GHz.

Reference

3GPP RAN1#105-e, Chairman’s notes.

3GPP RAN4#99-e, R4-2107881 WF on phase continuity and power consistency for PUCCH and PUSCH repetition, RAN4.

- Appendix-Simulation assumption

- Table A-1 Simulation assumption for performance impacts of residual frequency error for PUSCH

| Parameter | Value |
| --- | --- |
| Carrier frequency | 700MHz / 4GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 for Rural scenario and Urban scenario |
| UE speed | 3 km/h |
| BS antenna configuration | 4Rx |
| UE antenna configuration | 1 Tx |
| Sub-carrier spacing | Fc = 4GHz, 30kHz for TDD, Fc = 700MHz, 15kHz for FDD. |
| Occupied RB | 4 |
| MCS | 0/1/4 |
| Waveform | DFT-s-OFDM |
| Number of OS per repetition | 14 (PUSCH mapping type A) |
| DMRS overhead | 2 DMRS symbols |
| Number of repetitions | 8 |
| Frequency hopping | Enabled |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Frequency offset model | Uniform distribution with [-0.1, +0.1] ppm |

- Table A-2 Simulation assumption for performance impacts of residual frequency error for PUCCH

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 3, 11bits |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |
| Antenna configuration | 1Tx , 2Rx |
| DMRS configuration | Additional DMRS is not configured |
| SCS | Fc = 4GHz, 30kHz for TDD, Fc = 700MHz, 15kHz for FDD. |
| Repetitions | 4,8 |
| Bundle size(BS) for joint CE | 1,2,4,8 |
| FH | Enabled/ disabled |
| Channel model | TDL-C, 300ns |
| UE speed | 3km/h |
| FO | +/- 0.1ppm, uniform distribution |

- Table A-3 Simulation assumption for phase tolerance level and amplitude tolerance level for PUSCH

| Parameters | Values |  |
| --- | --- | --- |
| - Scenario | - Urban , O2I | - Rural, O2I |
| - Duplexing scheme and frequency | - 4GHz TDD | - 700MHz FDD |
| - Frame structure for TDD | - DDDSUDDSUU (S: 10D:2G:2U) for 4GHz |  |
| - Transmission bit rate for data channel (bit/s) | - 1 Mbps for eMBB | - 100 kbps for eMBB |
| - Subcarrier spacing | - 30kHz | - 15kHz |
| - Channel model for link-level simulation | - TDL-C | - TDL-C |
| - Pathloss model (select from LoS or NLoS) | - NLos | - NLos |
| - Delay Spread | - 300ns | - 300ns |
| - UE velocity | - 3 km/h | - 3 km/h for O2I |
| - Number of RF chains for BS | - 4 | - 4 |
| - Number of UE antennas | - 1 | - 1 |
| - Number of RF chains for UE | - 1 | - 1 |
| - Number of occupied RBs | - 15 | - 4 |
| - MCS | - MCS4 | - MCS2 |
| - DMRS overhead | - Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data |  |
| - Frequency hopping | - Disabled |  |
| - Channel estimation | - Practical |  |
| - Receiver type | - MMSE |  |
| - BLER Target | - 10% iBLER for eMBB |  |
| - Waveform | - DFT-s-OFDM |  |
| - Number of repetitions | - 2 repetitions with JCE |  |