3GPP TSG RAN WG1#106bis-e    		                                         R1-2108918

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.7.2

Source:	Spreadtrum Communications

Title:	Discussion on power saving techniques for connected-mode UE

Document for:	Discussion and decision

# Introduction

In the past meetings, power saving techniques for connected-mode UE and the corresponding signaling design were discussed. In this contribution, we discuss the power saving techniques and the corresponding signaling design.

# Power saving techniques

It is common understanding that there are mainly two power saving techniques for PDCCH monitoring reduction in connected mode, such as SSSG switching and PDCCH skipping for a duration.

It can be observed that these two techniques are similar. In our previous evaluation [1], these two techniques have the similar power saving gain. From the agreement in RAN1#104, these two techniques should be both supported.

## Rel-16 SSSG switching technique for unlicensed operation

### Wakeup indication and fallback timer

In Rel-16 SSSG switching technique for unlicensed operation, DCI format 2-0 can be used for triggering switching from “dense” SSSG (may not have large BD/CCE) to “sparse” SSSG (may not have small BD/CCE). For explicit triggering, there is 1 toggle bit for SSSG switching in DCI format 2-0.

It should be noted that DCI format 2-0 is very important for Rel-16 unlicensed operation, since COT information, frequency location information, guard-band information and SSSG switching information are all conveyed by DCI format 2-0. If DCI format 2-0 is missed by UE, the whole COT will be out of order. So, gNB should guarantee the reliability of DCI format 2-0 in Rel-16 unlicensed operation.

To mitigate the impact of miss-detection of DCI (e.g. DCI format 2-0) triggering switching from the “dense” SSSG to the “sparse” SSSG, the fallback timer is specified.

### State machine

Rel-16 SSSG switching technique for unlicensed operation has an “elegant” state machine. There are two states: the first one is the monitoring of the “dense” SSSG, and the second one is the monitoring of the “sparse” SSSG. For explicit triggering, the 1 bit in DCI format 2-0 can trigger transition between the two states, i.e. “go-to-sleep” indication and wakeup indication. For implicit triggering, detection of DCI in the “dense” SSSG can trigger transition from the first state to the second state. In our view, gNB should guarantee the reliability of triggering bit or triggering PDCCH, otherwise, the whole state machine will be out of order.

Furthermore, in CA case, such state machine will operate in each activated SCell. It is too complicated at UE side. However, it was not optimized in R16 NR-U, since CA is not so necessary in sub-6GHz unlicensed band (UE can using 100MHz BWP to achieve the target data rate).

### Application delay

The application delay is defined in Rel-16 SSSG switching technique for unlicensed operation. In our view, at least (may include bandwith/guard-band adaptation time), the pipeline switching is assumed in Rel-16 SSSG switching technique for unlicensed operation.

## Rel-17 SSSG switching technique for power saving

For SSSG switching, power saving gain comes from the sparse PDCCH monitoring occasions in a SSSG after indicating “go-to-sleep”. The source and target SSSG after indicating “go-to-sleep” can be called as “normal” SSSG and “dormant” SSSG. It is noted this is different from that of Rel-16 SSSG switching technique, in which the “dense” SSSG and the “sparse” SSSG should be configured.

### Wakeup indication and fallback timer

Wakeup indication and fallback timer are similar as that of Rel-16 SSSG switching technique. As the conclusion of Rel-16 power saving study item (in RAN1_AH_1901), the wakeup indication should have the target miss detection rate equal to 0.1%.

Agreements: The performance evaluation of the power saving signal/channel should target the miss detection at X% and the false alarm rate at Y% with the following aspects identified for the proposed power saving signal/channel - The target of miss detection X% and the false alarm rate at Y% as baseline for evaluation - For power saving signal/channel for wake-up purpose, X=[0.1] and Y=[1] - For power saving signal/channel for go-to-sleep purpose, X=[1] and Y=[0.1]

However, unfortunately, there was no discussion on the miss detection rate of the wakeup indication for Rel-16 SSSG switching technique in NR-U standardization. In our view, the wakeup indication for Rel-17 SSSG switching technique can be down prioritized, and UE should mainly rely on the fallback timer to wake up.

Proposal 1: The wakeup indication for Rel-17 SSSG switching technique should be down prioritized.

### State machine

Rel-17 SSSG switching technique can have an “elegant” state machine like Rel-16 SSSG switching technique for unlicensed operation, but we suggest not introducing such “elegant” state machine. The reason is also the target miss detection rate of 0.1%. If the target miss detection rate cannot be met, the power saving gain will be zero (even negative) or scheduling delay will be too large. Furthermore, if gNB and UE do not align in their own state machine due to miss detection and false alarm, UE will be out of control. It may not be critical for small cell (e.g. in NR-U), but definitely critical for macro cell.

Furthermore, in CA case, the state machine will operate in each activated SCell. It is too complicated at UE side.

Proposal 2: The state machine for Rel-17 SSSG switching technique should be down prioritized, or simplified as much as possible.

### Application delay

For SSSG switching, if the full-blown SSSG switching is supported like Rel-16 SSSG switching, UE should switch the pipeline dynamically, and the application delay is needed for the pipeline switching. In Rel-17 power saving, the “dormant” SSSG still has almost full flexibility (fully depends on gNB configuration). So, the application delay could be that defined in Rel-17 SSSG switching.

Proposal 3: The application delay for Rel-17 SSSG switching technique can be defined.

### Rel-16 SSSG switching is not compatible to Rel-17 SSSG switching

We would like to emphasize that Rel-16 SSSG switching is not compatible to Rel-17 SSSG switching.

Design purpose

For unlicensed operation, the purpose of SSSG switching in Rel-16 NR-U is to adapt different PDCCH monitoring characteristics inside COT and outside COT. Outside COT, UE can monitor the mini-slot level PDCCH to avoid missing scheduling or COT information when gNB gains the channel. Inside COT, UE can monitor the slot level PDCCH to reduce PDCCH monitoring. However, the purpose of SSSG switching for Rel-17 power saving is to control UE to monitor the “dormant” SSSG when packet is not arriving.

Observation 1: The purpose of Rel-16 SSSG switching is different from that of Rel-17 SSSG switching.

Explicit triggering

For explicit triggering, DCI format 2-0 is used for unlicensed operation. DCI format 2-0 may not be suitable for licensed operation, since the triggering bit in DCI format 2-0 is only used for unlicensed operation together with other bits like bits informing COT structure. It is general view that DCI format 2-0 is used for SFI in licensed operation, and gNB should transmit DCI format 2-0 only if SFI is changed. Moreover, the UE-specific like indication should be addressed in group common DCI to support indicating a specific UE, like DCI format 2-6.

Observation 2: For explicit triggering, DCI format 2-0 may not be suitable for Rel-17 SSSG switching.

Implicit triggering

For implicit triggering, UE can autonomously switch SSSG with assistance of a timer. Specifically, gNB can configure a timer by RRC signaling, and UE should switch to the “sparse” SSSG upon detection of PDCCH in the “dense” SSSG, and after timer expired UE should switch back to the “dense” SSSG. Behind implicit triggering, DCI format 2-0 is usually configured in the “dense” SSSG, but DCI format 2-0 is not mandatory in licensed band.

Observation 3: Implicit indication may not be suitable for Rel-17 SSSG switching.

## Rel-17 PDCCH skipping technique for power saving

For Rel-17 PDCCH skipping technique, power saving gain comes from skipping PDCCH monitoring occasions after indicating “go-to-sleep”.

### Wakeup indication and fallback timer

There is no wakeup indication in Rel-17 PDCCH skipping technique, and fallback time is necessary.

Proposal 4: The wakeup indication for Rel-17 PDCCH skipping technique is not supported.

### State machine

Rel-17 PDCCH skipping technique has no state machine.

Proposal 5: The state machine for Rel-17 PDCCH skipping technique is not supported.

### Application delay

For Rel-17 PDCCH skipping technique, the application delay can be zero.

Proposal 6: The application delay for Rel-17 PDCCH skipping technique is zero.

## Some thoughts on the state machine

In Rel-16, RAN2 had discussed PDCCH skipping and summarized in [2]. As summary, RAN2 had consensus that PDCCH skipping has smaller time scale than DRX operations and PDCCH skipping is independent of MAC operations.

Observation 1: DCI-based PDCCH skipping and DRX operate on different time scales. Observation 2: DCI-based PDCCH skipping and DRX have different functionality. Observation 3: A UE is expected to operate in different power states when in DRX-OFF and when executing a DCI-based PDCCH skipping, however this is not RAN2 expertise and also depends on UE implementation. Observation 4: DCI-based PDCCH skipping does not duplicate the DRX functionality. Proposal 1: DCI-based PDCCH skipping is supported in Rel-16. Proposal 2: DCI-based PDCCH skipping has no effect on the drx-onDurationTimer. Proposal 3: DCI-based PDCCH skipping has no effect on the drx-InactivityTimer. Proposal 4: DCI-based PDCCH skipping has no effect on the drx-RetransmissionTimerUL. Proposal 5: DCI-based PDCCH skipping has no impact on the drx-RetransmissionTimerDL. Proposal 6: DCI-based PDCCH skipping has no impact on the ra-ContentionResolutionTimer. Proposal 7: DCI-based PDCCH skipping does not trigger the MAC entity to exit DRX Active Time during an SR procedure. Proposal 8: DCI-based PDCCH skipping has no impact on MAC. Proposal 9: The UE applies the PDCCH skipping during DRX Active Time irrespective of the event that triggered the Active Time. Proposal 10: MAC has no impact on DCI-based PDCCH skipping.

In other words, PDCCH skipping is pure PHY control, which is not so reliable. It was stated in LTE TS 36.300. As suggested in 36.300, the state machine should be performed by RRC.

Table B.1-1: Summary of the difference between MAC and RRC control MAC control; RRC control. Control entity; MAC; RRC. Signalling; PDCCH; MAC control PDU; RRC message. Signalling reliability; ~ 10-2 (no retransmission); ~ 10-3 (after HARQ); ~ 10-6 (after ARQ). Control delay; Very short; Short; Longer. Extensibility; None or very limited; Limited; High. Security; No integrity protection No ciphering; No integrity protection No ciphering; Integrity protection Ciphering. The main difference between MAC and RRC control lies in the signalling reliability. Due to the signalling reliability, signalling involving state transitions and radio bearer configurations should be performed by RRC. Basically, all signalling performed by RRC in UTRA should also be performed by RRC also for E-UTRA.

Therefore, the state machine for PHY layer control should be simple in general design.

In NR, only BWP switching and Rel-16 SSSG switching for NR-U has the state machine for PHY layer control. For BWP switching, it has large time scale, even larger than DRX operations, and it can be protected by HARQ feedback. For Rel-16 SSSG switching for NR-U, we think it is suitable for small cell and DCI format 2-0 can be configured reliably. Further, Rel-16 SSSG switching for NR-U is not commercialized in real world. Hence, BWP switching and Rel-16 SSSG switching for NR-U cannot be taken as a reference for the state machine.

## Comparison

We compare the above three techniques in the following table.

Table 1: Comparison of the above three techniques

|  | Rel-16 SSSG switching technique for unlicensed operation | Rel-17 SSSG switching technique for power saving | Rel-17 PDCCH skipping technique for power saving |
| --- | --- | --- | --- |
| Wakeup indication and fallback timer | Explicit triggering: 1 bit wakeup indication; Fallback timer | We suggest the wakeup indication is down prioritized; Fallback timer | No wakeup indication; Fallback timer |
| State machine | State machine with two states | We suggest the state machine is down prioritized or simplified as much as possible | No state machine |
| Application delay | Non zero (at least for pipeline switching) | Non zero (for pipeline switching) | Zero |

From the above table, we think Rel-17 PDCCH skipping technique is more friendly for UE implementation.

Observation 4: Rel-17 PDCCH skipping technique is more friendly for UE implementation.

# Signaling design

## Striving common design of signaling

It was agreed in RAN1#104e [3] to strive for a common design to support functionalities inclusive of both SSSG switching and PDCCH skipping for a duration.

Agreements: - Strive for a common design for DCI based PDCCH monitoring adaptation in active time for an active BWP to support functionalities inclusive of both SSSG switching and PDCCH skipping for a duration. - Details FFS

Then, it was agreed in RAN1#104e [3] that two alternatives of signalling design can be considered.

Agreements: - The following alternatives can be considered for DCI based PDCCH monitoring adaptation in active time for an active BWP for power saving - Alt 1: Enhancement of Rel-16 SSSG switching to support PDCCH monitoring adaptation including skipping for a duration - Alt 2a: Enhancement of DCI(s) utilized for Rel-16 power saving adaptation for supporting both skipping PDCCH monitoring for a duration and SSSG switching - Alt 2b: Enhancement of DCI(s) utilized for Rel-16 power saving adaptation for supporting both skipping PDCCH monitoring for a duration and PDCCH monitoring periodicity adaptation - Others not precluded

It should be noted that both Alt 1 and Alt 2a can support both PDCCH skipping for a duration and SSSG switching.

In RAN1#105e, it seems that companies cannot achieve a common design of signalling, and cannot achieve a consensus for down selection from Alt 1 and Alt 2a.

## Two understandings of common design

The first understanding

Some companies think specifying one RRC configuration is common design. For example, if one RRC configuration within SSSG switching framework is specified, the “dormant” SSSG (or “empty” SSSG) should be defined to emulate PDCCH skipping. An example for the first understanding can be described in details:

- RRC: Configure {the “dormant” SSSG, the “normal” SSSG}

- DCI: Trigger SSSG#0 or SSSG#1

The second understanding

In our view, a set of two RRC configurations is also common design. gNB can choose one of two RRC configuration for a given UE. In this sense, RAN1 can still design two frameworks (SSSG switching and PDCCH skipping) in parallel, which correspond to two separate UE capabilities. An example for the second understanding can be shown as follows:

- RRC:

  - Configure { the “dormant” SSSG, the “normal” SSSG }, if UE supports Rel-17 SSSG switching, or

  - Configure {no PDCCH skipping, PDCCH skipping for a duration}, if UE supports Rel-17 PDCCH skipping

- DCI:

  - Trigger the “dormant” SSSG or the “normal” SSSG, if { the “dormant” SSSG, the “normal” SSSG } is configured

  - Trigger non PDCCH skipping or PDCCH skipping for a duration, if {no PDCCH skipping, PDCCH skipping for a duration} is configured

From RAN1#105e discussion and conclusion, the above two understandings are both applicable. In our view, the second understanding can give more flexible signalling design. Otherwise, we have to let the SSSG switching emulate PDCCH skipping. We will discuss the signaling design under the second understanding.

## Scheduling DCI

In RAN1#105e [4], it was agreed that PDCCH skipping technique can be supported by the scheduling DCI (DCI format x-1 and DCI format x-2).

Agreement: - PDCCH schedules data and also indicates PDCCH monitoring adaptation by SSSG switching and PDCCH skipping for a duration is supported. - At least DCI format(s) 1-1, 0-1, 1-2 and 0-2 can be used for the indication(s)

In RAN1#106e [5], it was agreed that at most 2 bits indication in self-scheduling DCI can be specified.

Agreement - At most 2 bit indication in self-scheduling DCIs (i.e., DCI format 1-1/0-1/1-2/0-2) can be specified for triggering the PDCCH monitoring adaptation in a single cell - FFS: the bit size of the indication is configurable - FFS: bit mapping to the PDCCH monitoring behaviour - FFS: details of indication of multiple cells case

For the bits size, we support it can be configured by gNB for flexibility.

Proposal 7: The bits size for indication in self-scheduling DCI is configurable.

For bit mapping to the PDCCH monitoring behaviour, PDCCH skipping and Rel-17 SSSG switching should separately triggered by DCI. Otherwise, easily (1% probability) gNB and UE have different knowledge on the current PDCCH monitoring behaviour, and the error can be propagate for long time leading to out of order.

Proposal 8: PDCCH skipping and Rel-17 SSSG switching should be separately triggered by DCI.

For indication of multiple cell case, PDCCH skipping and Rel-17 SSSG switching cannot be indicated in cross-cell scheduling DCI. The indication of PDCCH skipping and Rel-17 SSSG switching usually implies the pending of traffic, so the indication in cross-scheduling DCI is meaningless. Further, the indication across the cells is too complicated and easily cause out of order.

The cross-cell scheduling usually means the PDSCH/PUSCH in the target cell, which does not mean the PDCCH is not transmitted in the source cell for a duration.

Proposal 9: The indication of PDCCH skipping and Rel-17 SSSG switching in cross-cell scheduling DCI means that PDCCH skipping or Rel-17 SSSG switching occurs in the source cell.

### Supporting PDCCH skipping technique

For supporting PDCCH skipping technique, there were two alternatives of signalling design agreed in RAN1#105e [4] shown as follows.

Agreement: - At least one of Alt 1 and Alt 2 is supported, to be decided in RAN1#106, - Alt 1: Supporting SSSG switching to emulate PDCCH skipping functionality, - Alt 1-1: by an ‘empty’ SSSG which no SS set(s) is configured for the ‘empty’ SSSG, UE does not monitoring PDCCH on the ‘empty’ SSSG, - Alt1-2: by a ‘dormant SSSG’ which may have associated SS sets, and monitored conditionally (e.g., depending on HARQ NACK or RTT/ReTx timers) - Alt 2: PDCCH schedules data and also indicates PDCCH monitoring adaptation by PDCCH skipping for a duration is supported. - FFS details, including - e.g., joint / separate indication of SSSG switching and PDCCH skipping - Determination of the duration(s) for PDCCH skipping, e.g., - by RRC signaling, - by DCI indication - Implicitly, to the end of C-DRX active time

For Alt-1, some companies prefer to use SSSG switching signalling framework to emulate PDCCH skipping technique.

For Alt-2, some companies prefer to use “simple” or “native” PDCCH skipping signalling framework to support PDCCH skipping technique.

In our view, SSSG switching has many different aspects from PDCCH skipping. And UE capabilities for SSSG switching and PDCCH skipping are totally different and needs to be separate in the spec. PDCCH skipping has lower UE complexity. There is no need to use SSSG switching signaling framework to support a separate capability (PDCCH skipping). Therefore, we support Alt-2.

In RAN1#106e discussion, companies cannot achieve consensus on Alt-1 or Alt-2. So, companies agreed the PDCCH monitoring behaviours.

Agreement Select either package 1 or package 2 Package 1 · UE behavior after receiving PDCCH indication of monitoring adaptation can be one of the followings, - Working Assumption: Beh 1: PDCCH skipping is not activated - Beh 1A: PDCCH skipping means stopping PDCCH monitoring for a duration X - FFS the possible values for X - FFS: Whether and how to support more than one skipping duration(s) - FFS: whether to continue monitoring PDCCH scrambled by C-RNTI for Type 0/1/1A/2 CSS or not - Beh 2: stop monitoring SS sets associated with SSSG#1 and SSSG#2 (if confirmed) and monitoring of SS sets associated to SSSG#0 (legacy behaviour) - Beh 2A: stop monitoring SS sets associated with SSSG#0 and SSSG#2 (if confirmed) and monitoring of SS sets associated to SSSG#1 (legacy behaviour) - Working Assumption: Beh 2B(if confirmed): stop monitoring SS sets associated with SSSG#0 and SSSG#1 and monitoring of SS sets associated to SSSG#2 (if confirmed) · Note: The number of supported SSSG is left to UE feature discussion. · FFS: UE capability of supported UE behaviors · Indication of Beh 1A when SSSG(s) are not configured is supported. · Working assumption: Indication of Beh 1A for current SSSG when two SSSG(s) are configured is supported · FFS: Indication of Beh 1A when three SSSG(s) (if supported) are configured · Y bits is configured for scheduling DCIs (i.e., DCI format 1-1/0-1/1-2/0-2) indicating PDCCH schedules data and also PDCCH monitoring adaptation - FFS how the UE behavior(s) defined above mapping to Y bits § Note: at most Y = 2 · Working Assumption at most 3 SSSGs is supported to be configured. - FFS: whether or how SSSG can be configured to be monitored conditionally (e.g., depending on HARQ NACK or RTT/ReTx timers) - FFS: whether or how non-default SSSG to another non-default SSSG · FFS details of timer(s) for switching between SSSG(s) - UE fallbacks to default SSSG (i.e., SSSG#0) after timer expiration. - R16 timer for SSSG switching and the corresponding behavior is as baseline · FFS whether the timer(s) is configured per SSSG, or per BWP or other approaches. · FFS whether the skipping duration(s) is configured per SSSG, per BWP, or other approaches. · FFS PDCCH monitoring adaptation indicated by non-scheduling DCI · PDCCH based monitoring adaptation is limitedapplied to USS and type-3 CSS. Agreement Package 1 in above agreement is selected.

As we support the indications of PDCCH skipping and Rel-17 SSSG switching are separate, we support Behaviour 1, and we support multiple skipping durations to fill 4 codepoints, including Behaviour 1 and up to 3 skipping durations for Behaviour 1A.

Proposal 10: Confirm the working assumption that Behaviour 1 is supported.

Proposal 11: Up to 4 codepoints, including Behaviour 1 and up to 3 skipping durations for Behaviour 1A, are supported for PDCCH skipping.

### Supporting SSSG switching technique

In RAN1#105e [4], it was agreed that SSSG switching technique can be supported by the switching between two SSSGs.

Agreement: At least SSSG#0 and SSSG#1 switching is supported for Rel-17 SSSG switching indicated by PDCCH scheduling data and/or timer.

### DCI fields

In addition to new DCI field, the legacy DCI fields can be reused as well.

- Indication of cross-slot scheduling: Rel-16 support 1 bit indication for the minimum scheduling offset in scheduling DCI. When gNB indicates the larger one of the minimum scheduling offsets, it means the scheduling delay can be relaxed for UE power saving purpose. At the same time, PDCCH skipping can be conducted. It can reduce the DCI overhead.

- Indication of SCell dormancy: Rel-16 support up to 5 bits indication for the switch between normal BWP and dormant BWP in SCell(s) in scheduling DCI. When gNB indicates switching to the dormant BWP in SCell, it is likely that traffic packet does not arrive, and thereby “go-to-sleep” for both SCell (SCell dormancy) and PCell (PDCCH skipping) can be triggered. It can reduce the DCI overhead if SCell dormancy is configured.

We think the above DCI fields can be all considered.

Proposal 12: Consider also the following DCI fields for PDCCH that schedules data and also indicates PDCCH monitoring adaptation by SSSG switching and PDCCH skipping.

- Reusing the Rel-16 indication of cross-slot scheduling

- Reusing the Rel-16 Indication of SCell dormancy

## Non-scheduling DCI with C-RNTI scrambling

In Rel-16, SCell dormancy can be supported by the non-scheduling DCI with C-RNTI scrambling (i.e. SCell dormancy case 2 like), i.e. DCI format x_0 and x_1 with the disabled frequency resource allocation. With such non-scheduling DCI, UE can be indicated to switch to dormant BWP in SCell(s) without being scheduled. It has the following benefits.

- It can carry large number of DCI bits.

- It can support the HARQ feedback for the indicating PDCCH shown below (TS 38.213).

A UE is expected to provide HARQ-ACK information in response to a detection of a DCI format 1_1 indicating SCell dormancy after N symbols from the last symbol of a PDCCH providing the DCI format 1_1.

So, we support the non-scheduling DCI with C-RNTI scrambling that indicates PDCCH monitoring adaptation by both SSSG switching and PDCCH skipping.

Proposal 12: Support the non-scheduling DCI with C-RNTI scrambling that indicates PDCCH monitoring adaptation by SSSG switching and PDCCH skipping.

## DCI format 2-0

In our view, DCI format 2-0 is not mandatory for licensed operation. On the other hand, SFI in DCI format 2-0 usually has large periodicity in real deployment. Therefore, it may not be suitable for Rel-17 SSSG switching or Rel-17 PDCCH skipping.

## DCI format 2-6

In Rel-16, DCI format 2-6 can carry the wakeup indication. Whether it can carry the “go-to-sleep” indication is questionable. Firstly, DCI format 2-6 should be reliable (i.e. with low miss detection rate) so that it cannot carry too many DCI bits. Secondly, the use case that gNB want to start onDurationTimer but also want UE skips PDCCH monitoring at the beginning may be a corner case. Therefore, it may not be suitable for Rel-17 SSSG switching or Rel-17 PDCCH skipping.

# Conclusion

In the contribution, we have the following proposals.

Rel-17 SSSG switching technique

Proposal 1: The wakeup indication for Rel-17 SSSG switching technique should be down prioritized.

Proposal 2: The state machine for Rel-17 SSSG switching technique should be down prioritized, or simplified as much as possible.

Proposal 3: The application delay for Rel-17 SSSG switching technique can be defined.

Rel-17 PDCCH skipping technique

Proposal 4: The wakeup indication for Rel-17 PDCCH skipping technique is not supported.

Proposal 5: The state machine for Rel-17 PDCCH skipping technique is not supported.

Proposal 6: The application delay for Rel-17 PDCCH skipping technique is zero.

Scheduling DCI

Proposal 7: The bits size for indication in self-scheduling DCI is configurable.

Proposal 8: PDCCH skipping and Rel-17 SSSG switching should be separately triggered by DCI.

Proposal 9: The indication of PDCCH skipping and Rel-17 SSSG switching in cross-cell scheduling DCI means that PDCCH skipping or Rel-17 SSSG switching occurs in the source cell.

Proposal 10: Confirm the working assumption that Behaviour 1 is supported.

Proposal 11: Up to 4 codepoints, including Behaviour 1 and up to 3 skipping durations for Behaviour 1A, are supported for PDCCH skipping.

DCI fields

Proposal 12: Consider also the following DCI fields for PDCCH that schedules data and also indicates PDCCH monitoring adaptation by SSSG switching and PDCCH skipping.

- Reusing the Rel-16 indication of cross-slot scheduling

- Reusing the Rel-16 Indication of SCell dormancy

Non-scheduling DCI with C-RNTI scrambling

Proposal 12: Support the non-scheduling DCI with C-RNTI scrambling that indicates PDCCH monitoring adaptation by SSSG switching and PDCCH skipping.

# Reference

- R1-2100815, “Discussion on power saving techniques for connected-mode UE”, Spreadtrum, RAN1#104e, Jan. 25th - Feb. 5th, 2021.

- R2-1908072, “Report on [105bis#27][NR/Power Saving] – PDCCH skipping”, CATT, RAN2#106, May 13th - 17th, 2019.

- 3GPP RAN1#104e Chairman’s Notes, Jan. 25th - Feb. 5th, 2021.

- 3GPP RAN1#105-e Chairman’s Notes, May 10th - 27th, 2021.

- 3GPP RAN1#106-e Chairman’s Notes, August 16th - 27th, 2021.