3GPP TSG RAN WG1#106b-e			R1-2109474

e-Meeting, 11th – 19th October, 2021

Agenda item:	8.1.4

Source: 	Samsung

Title: 	Views on Rel-17 CSI enhancements

Document for:	Discussion and Decision

# Introduction

This contribution provides Samsung’s view on the two Rel. 17 CSI enhancement items included in the WID [1]: CSI enhancements for NC-JT transmission, and CSI enhancements for FDD reciprocity.

# NC-JT CSI enhancements

In Rel-16 CSI framework, a CSI-RS resource can be used as a CMR and/or NZP-IMR. To accurately measure CSI for NC-JT transmission, CMR and NZP-IMR should be measured together as a pair depicted in Figure 1. To reduce the RS overhead, it is desirable to reuse CMR as NZP-IMR as well. Rel-16 specification already allows such reuse only when CSI-RS is precoded, since NZP-IMR needs to be precoded such that each port corresponds to the interference transmission layer. To provide more flexibility on CSI resource configuration, it is beneficial to allow non-precoded CSI-RS for CMR to be reused as IMR.

Figure 1. CMR and IMR configuration for NC-JT CSI measurement.

Proposal 1: On CSI enhancements for multi-TRP, support CMR to be re-used as IMR for both non pre-coded and pre-coded CSI-RS

In RAN1#103-e [2], it was agreed to study CSI report configured by a single CSI reporting settings as follows:

Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report - two RIs, two PMIs, two LIs and one CQI per codeword, for single-DCI based NCJT when the maximal transmission layers is less than or equal to 4 - FFS: Maximal transmission layers larger than 4 - FFS: Whether/how a subset of above reporting quantities are allowed to be configured to the UE - FFS: whether/how to support two RIs, two PMIs, two LIs and two CQIs, for multi-DCI based NCJT - FFS: whether/how to support CRI(s) to be reported in a CSI - FFS: restrictions among reported CSI quantities, e.g. among reported RIs and PMIs - FFS: whether/how to support non-PMI based port-selection - FFS: whether/how to support single value of reported LI Note that other NCJT CSI measurement/reporting enhancement for other scenarios is not precluded, e.g. for HST-SFN

When coordinated scheduler is available for multi-TRP, NW would dynamically switch between NC-JT and non-NC-JT transmission according to the traffic condition and channel quality. For such operation, non-NC-JT CSI report can be configured as well as NC-JT CSI report but it is redundant since only one of those reports are utilized for data scheduling. Another approach is to allow UE to choose only one of those reports according to channel condition and omit the others from reporting. If {CMR, IMR} pairs for non-NC-JT CSI and those for NC-JT CSI are configured together within a same resource setting, such omission can be done implicitly by CRI. Further, since NC-JT CSI would be burdensome for UE to calculate appropriate precoder, non-PMI based port-selection in order to reduce the burden of UE side and CSI report quantity.

Figure 2. Example of CSI resource setting for dynamic NC-JT CSI reporting

Also, optimized UCI structure for dynamics NC-JT CSI report is also beneficial and can be extended by using Rel-16 two parts UCI structure as a baseline. For example, for CRI based solution, the amount of UCI for the proposed CSI report can vary much according to the selected CRI value. If the selected CRI value is for NC-JT CSI, the UCI would contain two sets of {RI, PMI} and a CQI for cooperating TRPs. Otherwise, the UCI would contain one set of {RI, PMI, CQI} for a single TRP. To handle the varying amount of UCI, we can extend two-part UCI structure in Rel-16 for the NC-JT CSI report. For example, UCI comprises a two parts (UCI#1, UCI#2), where

- UCI#1 is always reported, has fixed payload, and comprises (1) partial CSI for two TRPs and (2) an indication about remaining CSI for two TRPs included in UCI2. Note that (2) determines the payload of UCI2; and

- UCI#2 has a variable payload, and comprises remaining CSI for two TRPs.

Proposal 2: For NC-JT CSI reporting enhancement, support and study followings:

- Support non-PMI based port-selection

- Study UCI structure optimized for dynamic NC-JT CSI report

The following agreements were made for mTRP CSI enhancement.

Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, the UE can be configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N ≥ 1 NZP CSI-RS resource pairs whereas each pair is used for a NCJT measurement hypothesis - Configure UE with two CMR groups with Ks=K1+K2 CMRs. CMR pairs are determined from two CMR groups by following method(s). - K1 and K2 are the number of CMRs in two groups respectively. FFS K1=K2 or different K1/K2. - Note that CMRs in each CMR group can be used for both NCJT and Single-TRP measurement hypotheses - N CMR pairs are higher-layer configured by selecting from all possible pairs - signalling mechanism can be discussed further, e.g. using a bitmap - FFS: Whether MAC-CE or RRC+MAC CE indication is needed - FFS: how to support NCJT measurement hypotheses in FR2 - Support N=1 and Ks =2, FFS other maximal values of N>1 and Ks>2 - Note: for CPU/resource/port occupation, NCJT hypothesis is considered separately from single TRP hypothesis Agreement Whether a NZP CSI-RS resource can be referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis: - It is feasible in both FR1 and FR2 but subject to UE capability for FR2. If a UE supports and the sharing is also enabled by gNB, two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, otherwise they cannot. Agreement For CSI measurement associated with a CSI-ReportConfig for NC-JT, support following Alt: - Alt 3: For CMRs configured in the CSI-RS resource set, support RRC signalling to enable/disable single-TRP measurement hypothesis using CMRs configured within CMR pairs for NCJT measurement hypothesis

Based on the above agreements for CSI enhancement for NCJT-based mTRP, a higher layer parameter shall configure two groups of NZP CSI-RS resources corresponding to two TRPs in a NZP CSI-RS resource set. Furthermore, another higher layer parameter selects N CMR pairs by pairing CSI-RS resources from the two groups for CSI measurement for NCJT measurement hypothesis. Moreover, RAN1 concluded that there is no consensus on supporting MAC-CE based dynamic update of the CMR pairs [7]. Additionally, in Rel. 15/16 CSI, the number of CSI-RS resource sets configured for periodic and semi-persistent CSI resource setting is limited to S=1 which doesn’t give much flexibility for resource configuration for NCJT. In light of the above points, the RRC configuration for NCJT CSI reporting should be made flexible so that the gNB activates/triggers one of the reporting configurations depending on the channel condition, CSI processing unit (CPU) overbooking status, traffic load, etc. One way of achieving flexible CSI framework for NCJT is by configuring the CMR pairs per CSI-ReportConfig rather than in CSI-ResourceConfig. This allows a NZP CSI-RS resource set to be referred by multiple CSI-ReportConfigs for NCJT. Fig. 3 shows 3 CSI-ReportConfigs with different level of CPU requirement. Based on the agreed enabling/disabling of single-TRP measurement hypothesis using CMRs configured for NCJT measurement, the three configurations in Fig. 3 correspond to 8, 6 and 4 measurements, respectively.

Proposal 3: Support flexible RRC configuration for CSI measurement and reporting for mTRP NCJT CSI by configuring the CMR pairs for NCJT measurement hypothesis and the CMR sharing in CSI-ReportConfig.

Figure 3. Flexible RRC configuration for NCJT CSI measurement and reporting

In RAN1#104-e, it was discussed and agreed that the CSI-RS resource configuration and CSI measurement hypotheses for a single reporting setting as follows:

Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, the UE can be configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N ≥ 1 NZP CSI-RS resource pairs whereas each pair is used for a NCJT measurement hypothesis - Configure UE with two CMR groups with Ks=K1+K2 CMRs. CMR pairs are determined from two CMR groups by following method(s). - K1 and K2 are the number of CMRs in two groups respectively. FFS K1=K2 or different K1/K2. - Note that CMRs in each CMR group can be used for both NCJT and Single-TRP measurement hypotheses - N CMR pairs are higher-layer configured by selecting from all possible pairs - signalling mechanism can be discussed further, e.g. using a bitmap - FFS: Whether MAC-CE or RRC+MAC CE indication is needed - FFS: how to support NCJT measurement hypotheses in FR2 - Support N=1 and Ks =2, FFS other maximal values of N>1 and Ks>2 - Note: for CPU/resource/port occupation, NCJT hypothesis is considered separately from single TRP hypothesis Agreement Whether a NZP CSI-RS resource can be referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis: - It is feasible in both FR1 and FR2 but subject to UE capability for FR2. If a UE supports and the sharing is also enabled by gNB, two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, otherwise they cannot.

The proposed CSI report requires additional CSI computational complexity to take into account the mutual interference in NC-JT. As CPU occupation rule in the current spec is designed for the single-TRP CSI, the proposed CSI report needs new CPU occupation rule considering the additional CSI computational complexity. Practical implementation aspects should be taken into account in designing the new CPU occupation rule.

Proposal 4: Design new CPU occupation rule for dynamic NC-JT CSI report

In RAN1#104bis-e [6], it was agreed to support a 2-part CSI report in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis as below:

Agreement A 2-part CSI report is supported in Rel-17 for a CSI reporting configuration associated with NCJT measurement hypothesis with following clarifications: - Within CSI part 1 - CRI, RI, WB CQI and SB CQI for the first CW are reported with consistent payload and zero padding (if needed). FFS further details - FFS whether RI can be shared between NCJT CSI and single-TRP CSIs to reduce CSI feedback overhead - FFS whether additional field is needed, at least for Option 2 - Within CSI part 2: - FFS further compression/omission/Sharing of PMI among Single-TRP and NCJT hypotheses

Further, it was also agreed to study whether to support PMI/RI sharing mechanisms between NC-JT and sTRP CSI in RAN1#105-e [3]. It was also agreed in RAN1#103-e at least ‘typeI-SinglePanel’ codebook is supported. Nevertheless, other codebook types are not precluded. In the following, we discuss the possible compression/omission/sharing of PMI among single-TRP and NCJT hypotheses by giving an example based on the Type II codebook while the same concept can be applied to ‘typeI-SinglePanel’ codebook.

Agreement For Option 1 CSI reporting associated with NCJT and X single-TRP measurement hypotheses, study whether to support following PMI/RI sharing mechanisms between NCJT CSI and single-TRP CSI(s): - Enabling/Disabling PMI, RI sharing via higher-layer configuration - Dynamic indication of PMI, RI sharing in the CSI report - FFS: other details - FFS: applicable conditions/restrictions of CMR sharing among Single-TRP and NCJT hypotheses, if above PMI/RI sharing mechanism can be applied Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, [at least for multi-DCI based and single-DCI based schemes (scheme 1a)], NZP CSI-RS resources for channel measurement are associated to different TRPs/TCI states at resource level - CMRs corresponding to different TRPs respectively shall be configured within the same resource set (i.e. scheme 1-2) and have the same number of ports among CMRs. - At least ‘typeI-SinglePanel’ codebook is supported - FFS: Other codebook types - Note that RAN1 shall strive to finalize NCJT CSI enhancement with single reporting setting firstly. - The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI

Consider an NCJT operation wherein  and  denote the received signal at UE’s antenna panel 1 and 2 corresponding to TRP1 and TRP2, respectively, is given as

- (1)

where   and denote the precoding matrix and data vector, respectively, corresponding to the  TRP. Whereas, the channel matrix   represents the channel between the i-th TRP and j-th receive antenna panel at the UE. Moreover, denotes a noise vector associated with the j-th UE panel.

Similarly, a UE may operate in a single-TRP mode when the channel of a particular TRP is significantly better than the other or based on other conditions such as traffic loads at the TRPs. In this case, the received signal from the i-th TRP can be modeled as

(2)

where ,  and  denote the channel matrix, precoding matrix and data vector, respectively, corresponding to the   TRP. From (1) and (2), it is clear that the main difference in the precoders selection for NCJT (, ) and sTRP hypotheses (, ) is the consideration of the cross-link interference channels, i.e.,  and , in the NCJT case. In other words, when the channels corresponding to the two TRPs are nearly orthogonal, i.e.,  for  wherein  denotes Frobenius norm, the PMI for sTRP hypotheses can be reused for NCJT hypothesis too. This can be considered as a full compression/omission/Sharing of PMI among Single-TRP and NCJT hypotheses.

Moreover, considering the codebook format of type II codebook as an example, i.e., , the same beam-group selector matrix  can be reported for both NCJT and single-TRP hypotheses while mitigating cross-link interference by a separately reporting for the second part (). As  corresponds to the dominant angles wherein the received power is concentrated, it doesn’t make much sense to report different PMI components for  corresponding to NCJT and single-TRP hypotheses. The above concept can also be extended to other codebook types and can be considered as partial compression/omission/Sharing of PMI among Single-TRP and NCJT hypotheses. The PMI components that could be shared by PMI reporting for single-TRP and NCJT hypotheses can be configured to the UE by higher layer configuration. This way supports flexible trade-off between CSI overhead and precoding performance. In fact, it is reasonable for the UE to dynamically decide which PMI components to be omitted/shared among single-TRP and NCJT hypotheses based on the channel and inter-TRP interference measurement. In other words, upon reporting PMIs for single-TRP and NCJT hypotheses, the UE may dynamically decide on the PMI reporting overhead.

Proposal 5: Support full and/or partial compression/omission/Sharing of PMI among single-TRP and NCJT hypotheses.

Proposal 6: Support the dynamic variation on the level of compression/omission/Sharing of PMI and the associated payload of PMI for single-TRP and NCJT hypotheses.

In RAN1#106-e [4], six alternatives, shown below, were agreed in regards to RI restriction for Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting. According to the agreement, at most one alternative will be selected in RAN1#106bis-e.

Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support RI restriction by selecting at most one alternative from the following in RAN1#106bis-e: - Alt 1: One RI restriction is configured per CodebookConfig, whereas the RI restriction is applied to both Single-TRP and NCJT measurement hypotheses. - If rank restriction of X is configured, reported rank is X for a Single-TRP measurement hypothesis and sum of two reported ranks is X for a Multi-TRP measurement hypothesis. - Alt 2: Two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to one CMR group in a CMR resource set respectively, i.e. per TRP. - If rank restriction of (X, Y) is configured, reported rank is X for the CMR in the first CMR group and Y for the CMR in the second CMR group, regardless single-TRP and NCJT measurement hypotheses. - Alt 3: Multiple RI restrictions can be configured per CodebookConfig, whereas RI restriction is applied to per each CMR in CMR pair for NCJT and per each CMR for Single-TRP. - Alt 4: Two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to all Single-TRP measurement hypotheses, and another one is applied to all NCJT measurement hypotheses. - If rank restriction of (X, Y) is configured, reported rank is X for all single-TRP measurement hypotheses and reported rank (1 out of 4 possible rank combinations) is Y for all NCJT measurement hypotheses. - Alt 5: Three RI restrictions can be configured per CodebookConfig, whereas two RI restrictions are applied to two CMR groups in a CMR resource set respectively for Single-TRP measurement hypothesis, and the third one is applied to all NCJT measurement hypotheses. - If rank restriction of (X1, X2, Y) is configured, reported rank is X1, X2 for each CMR group respectively for single-TRP measurement hypotheses and reported rank (1 out of 4 possible rank combinations) is Y for all NCJT measurement hypotheses. - Alt 6: Switch between Alt 4 and Alt 5 where gNB can configure via RRC signaling which alternative to use Note that if none of above Alternatives is agreed in Rel-17, RI restriction is only applied for Single-TRP measurement hypotheses and no RI restriction is applied for Multi-TRP measurement hypotheses. Conclusion Whether to support interference measurement based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis, in addition to CSI-IM Alt 2: No, it is not supported Agreement Support the indication of following RI combinations by a joint RI field for a NCJT measurement hypothesis in CSI part 1, when the maximal transmission layers is less than or equal to 4: - {1, 1}, {1, 2}, {2,1}, {2,2} - FFS: CBSR and/or RI restrictions per TRP or across TRPs

One purpose of RI restriction is to limit the CSI reported by a UE to the low ranks in case the UE is a cell-edge UE with ‘bad’ channel quality. Another purpose is to give the gNB flexibility in terms of managing the transmission layers in MU-MIMO scenario. It is to be recalled that the case for simultaneous occurrence of MU-MIMO and Multi-TRP NCJT operations is precluded by RAN1 from Multi-TRP CSI enhancement. This is apparent from the conclusion above, as RAN1 has agreed not to support interference measurement based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis. Moreover, it is agreed in RAN1#104bis-e [6], shown above, that the possible RIs for NCJT measurement are {1,1}, {1,2},{2,1} and {2,2}. Considering the possible rank pairs for NCJT are already for low ranks per each TRP and owing to the common understanding that simultaneous occurrence of NCJT and MU-MIMO operations is not supported, it is plausible to configure RI restriction only for sTRP measurement hypotheses with no RI restriction applied for Multi-TRP measurement hypothesis.

Proposal 7: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support RI restriction for single –TRP measurement hypotheses and no RI restriction is applied for Multi-TRP measurement hypothesis.

In RAN1#106-e [4], the following agreement was made on whether to restrict the two CMRs within a same CMR pair configured for NCJT measurement hypothesis to be within the same downlink slot or CDRX active time.

Agreement For CSI measurement associated with a CSI-ReportingConfig for NC-JT, study following restriction(s) for two CMRs within the same CMR pair configured for NCJT measurement hypothesis: - FFS: two resources are restricted within the same DL slot - FFS: two resources are restricted with the same CDRX active time

Figure 4. CMR-pair reception (a) under the same receiver setting (b) under different receiver (oscillator) setting

The issue as pointed out in [8] is whether a relative random phase rotation introduced by DL-UL-DL switching, i.e., if the UE switches to UL in between the reception of the two CMRs in a CMR pair for NCJT measurement hypothesis, impacts the CSI quality. In particular, as such relative noise rotation is not expected when a PDSCH is scheduled, owing to the SDMed transmission of the two PDSCH from the two TRPs, it was agreed to study whether the CSI computed by the UE under different receiver (oscillator) setting has impact. In this regard, consider the channel matrices. H1∈CNr×Nt and. H2∈CNr×Nt . corresponding to the TRP1 and TRP2, respectively. Moreover, consider an random phase rotation matrix  where the random phase rotation at the i-th UE antenna is denoted as . Then, if the two CMRs in a CMR pair are received with the same receiver (oscillator) setting, i.e., without relative phase rotation, the UE computes CSI based on , otherwise, the CSI is computed based on . In this regard, the optimal precoders based on  are given as

(3)

Similarly, the optimal precoders based on  are given as

(4)

where  is a utility function. To illuminate light on the impact of inter-TRP interference, we chose  to be   akin to spectral efficiency.

Fig. 5 illustrates the PMI indices for the two scenarios based on Type I single panel codebook. As shown in the figure, the PMIs for the two scenarios calculated based on (3) and (4) are identical in almost all channel realizations. Furthermore, the utility  is shown to be identical for the two cases in all channel realization. In light of these observations, it can be concluded that restricting the two CMRs of a CMR pair to be within a single slot is not advantageous. It is also to be noted that the IMR in Rel-15/16 is not restricted to be within the same slot as the corresponding CMR.

Figure 5. Performance (a) PMI indices for different channel realization (a) Utility value (C(.)) as defined above for different channel realization; Type I Single Panel [N1,N2,O1,O2]=[4,2,4,4]

Observation 1: Relative random phase rotation as defined above has negligible impact on optimal PMIs selection when NCJT CSI is computed based on CMRs within a CMR pair.

Observation 2: Relative random phase rotation as defined above has negligible impact on signal-to-inter-TRP-interference ratio when NCJT CSI is computed based on CMRs within a CMR pair.

Observation 3: IMR in Rel-15/16 is not restricted to be within the same slot as the corresponding CMR.

Proposal 8: Strive for less restriction on the transmission of CMRs in a CMR pair for NCJT measurement hypothesis.

In RAN1#106-e [4], the following agreement was made:

Agreement To confirm the order of UCI payload construction for reported CSIs, study following Alternatives and down-select one or more Alternative(s) for required specification changes in RAN1 106bis: - Alt 1: modify priority equation, i.e., Section 5.2.5 in 38.214. - Alt 2: modify the table of priority reporting levels for Part 2 CSI, i.e., Table 5.2.3-1 in 38.214. - Alt 4: modify mapping order of CSI fields of one CSI report, i.e., Table 6.3.2.1.2-3/4/5 in 38.212

Since the UCI payloads for NCJT and sTRP CSI reports are different, it is preferable if the CSI for NCJT and sTRP are reported separately as distinct reports. Accordingly, the priority equation in Section 5.2.5 in 38.214 can be modified by giving the CSI report for NCJT a higher priority. An example for mapping CSI fields of one CSI report in 38.212, for both NCJT and sTRP CSI, is given in Fig. 6. Akin to Rel. 15/16 CSI report, a report can be divided in two parts. The first part of a CSI report for NCJT may consist of CRI and RI, if they are reported, WB and SB differential CQIs, and an indication for sharing of CRI, RI and PMI with sTRP CSI. Moreover, CSI part 2 for NCJT CSI can be subdivided in two parts, i.e., WB and SB parts, and the corresponding fields as illustrated in Fig. 6 can be reported.

Figure 6. UCI structure for CSI report for NCJT and sTRP CSI

Proposal 9: Report NCJT and sTRP CSIs as separate reports.

- Modify priority equation in Section 5.2.5 in 38.214 by giving the CSI report for NCJT a higher priority.

- Include an indication for sharing of CRI, RI and PMI with sTRP measurement hypotheses in CSI part 1 for NCJT.

CSI enhancements for FDD reciprocity

The following agreements were made in RAN1#105-e [3] and RAN1#106-e [4] about the Rel. 17 codebook enhancement based on FDD reciprocity.

RAN1#105-e Agreement For Wf in CN3*Mv, Mv=2 is supported for R17 PS codebook - FFS: whether further dependence/restriction, i.e. conditioned on the number of CSI-RS ports, can be applied to Mv=2 - FFS: Whether Mv=4 can be supported for # of CSI-RS ports, e.g. 4 or 8 Agreement For Rel-17 port selection codebook, study following Alternatives and down-select in RAN1 106e: - Alt 1: Wf OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length N3. Wf as an all-one vector of length 1 is not needed - Alt 2: Wf OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length 1, i.e., a scalar. Wf as an all-one vector of length N3 is not needed. - Alt 3: Keep both Wf OFF and Wf ON with Mv=1. - If PMI format is SB, Wf is an all-one vector of length N3 - Informative note: this case is considered as “Wf ON with Mv=1” in the agreement in RAN1 104e - If PMI format is WB, Wf is an all-one vector of length 1, i.e., a scalar - Informative note: this case is considered as “Wf OFF” in the agreement in RAN1 104e - Note: N3 = NCQISubband*R. - FFS: the case when no SB size is configured.

RAN1#106-e Agreement Following working assumption is confirmed (with revision in RED): - At least for rank 1 and 2, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix, i.e. Alt 1. - FFS other restrictions, e.g. value(s) of N, if the value of N3 is small - FFS other restrictions, e.g. when the number of CSI-RS ports is small Conclusion For Rel-17 PS codebook, there is no consensus on the support of Mv>2 for Wf. Agreement For Rel-17 PS codebook, support reporting of the position, [il*, fl*], of the strongest coefficient (SCI) of layer l, using ceil(log2(K1*Mv)) bits Agreement For Rel-17 PS codebook, support layer-common port selection for rank 2. Agreement Support parameter combinations represented by (alpha, Mv, beta) with K1 = alpha*P for Rel-17 PS codebook - The candidate values of alpha are {1/2, 3/4, 1} - Note that exact parameter combination will be discussed from RAN1 106bis: - based on trade-off among UPT performance, feedback overhead, and complexity - based on all supported ranks - Limit total number of parameter combinations comparable to Rel-16 eType II - Mv={1, 2} and beta = {[1/4], 1/2, 3/4, 1} are from previous agreements Agreement For Rel-17 PS codebook with Rank 2, support layer-specific bitmap for indicating non-zero coefficient selection of W2. Agreement Support rank 3 and 4 for Rel-17 PS codebook with following: - Supporting ranks 3 and 4 is optional with separate UE capability (same as Rel-16 PS codebook) - The maximal CSI overhead of rank 3 and 4 is comparable to rank 2 - FFS: use a smaller K1 (or alpha) or beta for ranks 3 and 4, or limit the maximum number of non-zero coefficients across all layers to 2K0 and per layer to K0 with the same beta - FFS: limit Mv=1 for ranks 3 and 4 PMI Agreement At least for rank 1/2 and Mv > 1, for relationship between N and Mv, support following alternative - Alt 2-1: N >= Mv, Wf is layer-common and reported by UE for N>Mv. - For Mv=2, N=2 and one value from {3, 4, 5} - RAN1 to select one value from {3, 4, 5} in RAN1#106bis-e - FFS: how to report Wf in terms of reporting mechanism and associated bits when Mv=2 and N=one value from {3, 4, 5} Note: Wf is layer-common for N=Mv Note: For all alternatives, a layer-common window/set of size N is configured. Agreement If a bitmap for indicating non-zero coefficients can be absent, down-select one Alt from the following for Rel-17 PS codebook: - Alt 1: At least for rank 1 PMI, the bitmap of indicating non-zero coefficients is not needed if Mv=1 and Beta=1. - FFS the need for Mv>1 and/or Beta<1 - Alt 2: For rank 1 /2 PMI, the bitmap(s) of indicating non-zero coefficients for corresponding layer(s) is absent if reported KNZ=K1*Mv*rank - Where KNZ is the number of non-zero coefficients - Alt 3: In addition to Alt 2, additional field is reported by UE to inform whether the bitmap of indicating non-zero coefficients for specific layer is absent if rank>1. - Alt 4: The bitmap of indicating non-zero coefficients is not needed if the number of coefficients is sufficiently small, i.e. K1Mv ≤ δ Note: If none of above Alternative is agreed in RAN1#106bis-e, the bitmap for indicating non-zero coefficient is always present by default. Agreement For Rel-17 PS codebook, following values of R are supported: - R = 1 and - At most one value from {2, D* NPRBSB} - FFS: which one is to be decided in RAN1#106bis if support, and applicable conditions, e.g. whether the support of this feature when Mv=1 - D is the density of CSI-RS in frequency domain and NPRBSB is the subband size in PRBs - Note that this R is optional if supported

In this subsection, we first provide our views related to the codebook components. In particular, the following open issues have been discussed. The simulation assumptions in this section are according to the agreed EVM in RAN1#102-e, which are also copied in Table 2 in the appendix A. The Rel. 16 PS T2 with paraComb = 1 is considered as reference, unless stated otherwise.

## Wf OFF vs Wf ON, and PMI reporting format

According to agreement in RAN1#104-e [5], the Wf component of the codebook can be turned OFF by gNB. The exact mechanism about how Wf OFF is configured is still open, and there are three alternatives (according to the agreement copied above) to down-select from. We provide our view about this issue below.

Comment 1: spec-related issues

Agreement (RAN#104-e) For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, support codebook structure W=W1W2 WfH where - W1 is a free selection matrix, with identity matrix as special configuration - FFS polarization-common/specific selection - Wf is a DFT based compression matrix in which N3 = NCQISubband*R and Mv>=1 - At least one value of Mv>1 is supported - Decide on the value(s) of Mv, e.g. Mv=2, in RAN1# 104bis-e - Working assumption: Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters - FFS candidate value(s) of R, mechanism for configuring/indicating to the UE and/or mechanism for selecting/reporting by UE for Wf - Wf can be turned off by gNB. When turned off, Wf is an all-one vector (FFS; the length of all-one vector) - FFS other signaling/CSI reporting mechanism for trade-off among signaling overhead, UE complexity and UPT gain

Based on the agreement,

- when Wf is OFF, Wf is an all-one vector

- a single precoding matrix indicated by the PMI for all SBs or for the entire CSI reporting band

- WB pmi

- (A) When cqi format = WB, both cqi and pmi are essentially WB  freq granularity of CSI = WB

Spec related issue 1:

- (B) If pmi format is not provided (similar to R16), then otherwise (yellow highlighted) of the following text from 38.214 spec applies (SB freq granularity).

- (B) is contradictory to (A) which is based on agreement, when Wf OFF (WB pmi) and cqi format = WB.

- The correct text is 1st subbullet (green highlighted).

- Note: this is also an issue in R16 codebook also since Mv=1 is also possible there, though such cases can be argued to be corner cases.

- Pmi format = WB is straightforward solution to avoid the contradictory specification

Section 5.2.1.4, 38.214 A CSI Reporting Setting is said to have a wideband frequency-granularity if - reportQuantity is set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', cqi-FormatIndicator is set to 'widebandCQI' and pmi-FormatIndicator is set to 'widebandPMI', or - reportQuantity is set to 'cri-RI-i1' or - reportQuantity is set to 'cri-RI-CQI' or 'cri-RI-i1-CQI' and cqi-FormatIndicator is set to 'widebandCQI', or - reportQuantity is set to 'cri-RSRP' or 'ssb-Index-RSRP' or 'cri-SINR', or 'ssb-Index-SINR' otherwise, the CSI Reporting Setting is said to have a subband frequency-granularity.

Spec related issue 2:

- N3 is defined as the total number of precoding matrices (cyan text).

- If N3 >1, we can’t say “N3 precoding matrices” for Mv=1 since there is only one precoding matrix indicated by the PMI.

- We can say one of the following:

  - A single precoding matrix is indicated by the PMI

  - N3 precoding matrices indicated by the PMI, but they are the same when Mv=1

Section 5.2.2.2.5, 38.214 The parameter R is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband. This parameter controls the total number of precoding matrices N3 indicated by the PMI as a function of the number of configured subbands in csi-ReportingBand, the subband size configured by the higher-level parameter subbandSize and of the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2, as follows

Observation 4: according to current 38.214 specification,

- If pmiFormat is not provided, the frequency granularity of the CSI reporting is SB

  - This is against the agreement when Wf is OFF (meaning WB PMI) and cqiFormat = WB

- N3 is defined as the total number of precoding matrices

  - This is incorrect when Wf is OFF since there is only one precoding matrix

Comment 2: Wf OFF vs Wf ON

When Wf is turned OFF,

- the frequency granularity of PMI reporting is WB since there is only precoding matrix reported for the whole CSI reporting band. This implies that there is no need to associate SB with the PMI reporting, and hence the length of the all-one vector (FFS in RAN1#104-e agreement) should be one;

- both WB and SB CSI can be supported, depending on whether CQI reporting format is WB or SB. Note that in case of WB CSI reporting, BWP < 24 PRBs can be configured, which is not possible when Wf is turned ON since there is no SB size associated with small BWPs. The more details about BWP < 24 PRBs are discussed later; and

- the UE complexity is according to a WB precoder (or PMI) calculation.

On the other hand, when Wf is turned ON,

- the frequency granularity of PMI reporting is SB since there are N3 precoding matrices reported in total, one for each FD unit. The columns of Wf matrix correspond to N3 components;

- only SB CSI can be supported; and

- the UE complexity depends on the SB size for PMI and #SBs for precoder calculation, which in turn depends on the R value.

Now, we agree that Wf OFF and Wf ON with Mv = 1 can achieve the same functionality, and Wf can be fixed to an all-one vector. Hence, we don’t need to support both configurations. The only open issue then is the length of the all-one vector. In our view, it depends on the CSI format (which can also depend on the BWP size, cf. next subsection), in particular, whether a SB size is configured. The length should be one when the CSI is WB (i.e., when SB size is not configured), and more than one when the CSI is SB (i.e. when SB size is configured).

Observation 5: Regarding turning Wf ON/OFF

- When Wf is OFF, PMI reporting format is WB

- When Wf is ON, PMI reporting format is SB,

- Wf OFF and Wf ON with Mv=1 are the same, and Wf can be fixed to an all-one vector.

We therefore propose the following.

Proposal 10: Regarding turning Wf OFF,

- Wf OFF and Wf ON with Mv=1 are the same, and Wf is an all-one vector of length N3.

- pmiReportingFormat = WB

- support one of the following

  - Alt1: N3 = 1

  - Alt2: one of the following is included in the codebook description

    - Alt2-1: a single precoding matrix is indicated by the PMI

    - Alt2-2: N3 precoding matrices indicated by the PMI, but they are the same when Mv=1

## BWP < 24 PRBs

In the current specification, the CSI reporting is restricted to WB CSI using Rel.15 Type I codebook when the BWP size < 24 PRBs. The reason for supporting WB only CSI is due to the fact there is no SB size supported for such small BWPs. In our view, the WB CSI reporting should be supported for Rel.17 codebook for all BWPs, both BWP < 24 and BWP >= 24 PRBs. Some rationales behind this are as follows:

- Rel.17 CB can be used for any BWP sizes since gNB performs some frequency compression via beamformed CSI-RS so that the beamformed channel is less sensitivity to frequency selectivity when compared with Rel.16 codebook. This is also corroborated by the fact that the performance gains of the Rel. 17 codebook shrink quickly with large Mv value.

- Rel.17 codebook with Wf turned OFF essentially corresponds to WB PMI reporting, hence, can be configured for WB CSI reporting for BWP size < 24.

- Performance gain: the UPT vs overhead trade-off comparing Rel.17 codebook and Rel.15 Type I codebook for BWP = 20 PRBs is shown in Figure 1. A large performance gain (up to 16% in avg. UPT) can be observed.

- Reduced capacity or low cost UEs: since the complexity of Rel.17 codebook (especially when Wf is OFF) is small (comparable to Rel.15 Type I), and it achieves large UPT gain, it can be quite attractive for use cases such as low cost UEs, reduced capability UEs, NR railways, or more futuristic use cases.

- Finally, only low-resolution (Type I) codebook is supported for BWP size < 24, it is beneficial from the perspective of the overall system to also support a high-resolution (Type II) codebook for these BWP sizes. The performance gain can be large, and the CSI overhead and UE complexity are reasonable.

Observation 6: for BWP size = 20 PRBs, Rel.17 codebook can achieve up to 16% gain in avg. UPT over the current specification support (i.e. based on Rel.15 Type I codebook)

Proposal 11: support Rel.17 codebook for BWP size < 24 PRBs with the current restriction in the specification, i.e. support only WB CSI implying Wf is turned OFF

Figure 1

## SCI

In RAN1#106-e, it was agreed to support reporting of the position, [il*, fl*], of the strongest coefficient (SCI) of layer l, using ceil(log2(K1*Mv)) bits. It was discussed whether, similar to Rel. 16 codebook, a shift/remapping operation needs to specified in frequency domain. Our view about the shift/remapping operations are provided below.

- This is applicable only when N > Mv = 2, and is not needed for two other supported configurations Mv = 1 and N=Mv=2.

- Wf shift

  - If the purpose of Wf shift is to ensure that the FD basis (if reported) is w.r.t. to the DC component (FD basis 0), this can be done by the UE since there is no restriction on how UE determines Wf. The UE is free to apply shift before reporting (similar to the fact that the UE is free to measure FD basis outside the window).

  - The benefit of this shift is marginal (at most 1 bit saving in Wf payload).

- W2 shift or column index remapping (similar to Rel.16)

  - The purpose of this shift or remapping is to ensure that the column index of the SCI is FD basis fl*=0 (similar to Rel.16).

  - In our view, this is not needed since the column index of the SCI is reported anyway.

  - For prioritization in UCI omission, the reported column index of the SCI can be prioritized. Besides, UCI omission is a rare event and is only an emergency scheme which is not the codebook designed for.

- Bitmap remapping: similar to W2 shift, this is not needed in our view.

- Specification and UE operations: we prefer to simply specification so that the UE implementation is simple. Therefore, any operations if not necessary should be avoided.

Observation 7: regarding shift/remapping operations,

- There is no need for specifying Wf shift, W2 shift, and bitmap remapping.

- Shift/remapping for UCI omission is merely an optimization of a rare-event, hence doesn’t have any benefits.

We therefore propose the following.

Proposal 12: shift/remapping operations are not supported in Rel. 17 codebook.

## Value(s) of R

Figure 2

The value of R determines #FD components (to determine the #rows of the Wf matrix) and the SB size for the PMI or precoding matrix calculation. A lower/smaller of R is preferable for UE implementation complexity. The need for larger values of R requires justification. Similar to other codebook parameters, the tradeoff among UE complexity, CSI overhead, potential-specification impact, and the UPT performance should be considered in order to justify the larger value(s) of R.

One important aspect is related to the assumption about the CSI-RS beamforming for different R values. In particular, whether the CSI-RS beamforming is the same regardless of the R values or it can be different, e.g. the CSI-RS beamforming is according to the R values. This is illustrated in Figure 2.

- In example 1, the CSI-RS beamforming at the gNB follows the R value, shown in the middle of the figure. So, CSI-RS beamforming resolution in FD improves with increasing R values.

- In example 2, the CSI-RS beamforming at the gNB follows the largest R value (or per PRB), shown in right side of the figure. So, the CSI-RS beamforming resolution in FD remains the same regardless of the R values.

The FD basis vectors at the UE side as shown in left side of the figure are obtained according to the R value. In this example, a length-8 FD basis vector is considered, which can be obtained from a DFT vector as follows.

- When R=1/2, we have a length-2 DFT vector v0, v1 T, and a length- 8 FD basis vector is given by: v0,v0,v0,v0,v1,v1,v1,v1 T.

- When R=1, we have a length-4 DFT vector v0, v1,v2,v3 T, and a length-8 FD basis vector is given by: v0,v0, v1,v1, v2,v2,v3,v3 T.

- When R=2, a length-8 FD basis vector is v0,v1,v2, v3,v4, v5,v6,v7T, which is a DFT vector.

We provide simulation results comparing different R values for the examples of CSI-RS beamforming in Figure 3, Figure 4, and Figure 5. Results in Figure 3 assume the CSI-RS beamforming (BF) example 1, i.e., when R=1, the CSI-RS BF is based on length-NSB BF vectors, and when R=4, the CSI-RS BF is based on length-4NSB BF vectors. That is, the CSI-RS BF vectors are assumed to be different. The simulation results for the CSI-RS BF example 2, when the CSI-RS BF vectors are fixed to be of length-NRB, are shown in Figure 4 and Figure 5 for DFT based and ideal BF respectively. The value of Mv is fixed to 2, and multiple points correspond to different K1 (or L) values. In these simulation results, the number of RBs is assumed to be 48. We can observe the following.

Observation 8:

- when CSI-RS beamforming is according to the R value, R=4 shows small gain over R=1; however, R=4 has more UE complexity and CSI overhead than R=1

- when CSI-RS beamforming is the same for different R values, then

  - there is no performance gain with increasing R values (or increasing the length of DFT vectors comprising Wf)

  - R=1/4 achieves the best performance among R=1/4, ½, 1, and 4.

Based on these observations, we think R=1 is sufficient for Rel. 17 codebook. If it has to be supported, then it should apply only to the case when Wf is ON, i.e., Mv=2, since there is no need to configure for any R value when Wf is OFF.

Proposal 13: R>1, if supported, is configured only when Mv=2.

Figure 3: CSI-RS beamforming example 1 (according to R value)

Figure 4: DFT CSI-RS beamforming example 2 (according to max R or per PRB)

Figure 5: Ideal CSI-RS beamforming example 2 (according to max R or per PRB)

In order to the study the impact of CSI-RS beamforming example 2, we provide additional analysis based on the distribution of power candidate FD basis vectors across users. In particular, for CSI-RS beamforming example 2, distribution of 3 strongest FD components (from all possible FD components as shown in Appendix B) across UEs are shown in Figure 6 for the following parameter settings.

- NRB=48, NCQI,SB=12, CQI SB size=4

- N3=NCQI,SB×R

- R=14,12,1,4 implying N3={3,6,12,24,48} and 93 FD basis vectors in total.

- CDF of normalized coefficient power for 3 strongest FD basis vectors for each R value.

When the gNB beamforms CSI-RS at per PRB level, the owing to the angle-delay reciprocity, the frequency selectivity of the beamformed channel can be reduced significantly, to the extent that there are no performance benefits with increasing FD resolution due to increasing R values at the UE side.

Observation 9: when the CSI-RS beamforming resolution is per PRB level, there is no noticeable difference between distributions for three strongest FD components for different R values;

- Implying reduced frequency selectivity of the beamformed channel, which leads to no performance gain with increasing R values

Figure 6

## Mv > 1

As discussed in past RAN1 meetings, the support of Mv > 1 should be based on the tradeoff among UPT performance, CSI overhead, and UE complexity, as mentioned in WID [1]. This tradeoff for Mv=2 is described in Appendix C based on the results from companies during RAN1#104bis-e and RAN1#105-e. It is evident that this tradeoff is attractive for small #CSI-RS ports, but not so for high #CSI-RS ports. In fact, for 32 CSI-RS ports, there is hardly any UPT gain, but the CSI overhead and UE complexity burden are very high. The very small UPT gain is due to the fact that when the number of CSI-RS ports is large (e.g. 32), then with high probability, all dominant SD-FD pairs can be beamformed by the gNB via CSI-RS ports, and hence the resultant beamformed DL channel may not have any strong FD components, hence, the Wf component in the codebook may not bring any performance benefits. Therefore, the UE supporting Mv > 1 should not be forced to support this feature for all values of number of CSI-RS ports (P). Such a UE should be allowed to provide the information whether it UE support P > t (threshold), where t = 12, for example.

Proposal 14: Regarding Mv = 2,

- UE reporting its capability to support Mv = 2 shall also report whether it supports Mv=2 for P > 12 CSI-RS ports

## Wf window size and UE reporting

As agreed, the Wf component (when turned ON by setting Mv = 2) is determined based on a single window. There are a few open points regarding the window.

- The first point is regarding the window size when N3 is small (e.g. 3), in particular when N3 < N. A simple solution for this can be setting N = N3.

- The second point is whether FD basis index 0 is always included in the reported FD basis indices, i.e.

  - When N=3, the reported FD basis indices are from {(0,1), (0,2)}.

  - When N=4, the reported FD basis indices are from {(0,1), (0,2), (0,3))}.

  - When N=5, the reported FD basis indices are from {(0,1), (0,2), (0,3), (0,4)}.

Similar to Rel.16 codebook, the UE can apply a rotation to the FD basis vectors without any loss in performance such that the rotated FD basis vectors always include FD basis with index 0. Therefore, we can adopt Re.16 design and include FD basis index 0 in the reported FD indices.

- The third point is selecting one N value from {3,4,5}. Since FD basis 0 is included in the reported FD basis indices, N=3 or 5 can be supported if the payload of Wf reporting is 1 or 2 bits respectively, i.e., N=4 can be excluded.

Proposal 15: when N>Mv=2,

- window-size = N3 if the configured N>N3

- the reported FD indices include FD index 0

- Support N=3 or 5

## Bitmap absent

Another point for discussion is whether/how the bitmap for indicating non-zero coefficients for W2 can be absent for CSI reporting. In our view, if β=1 is supported for Rel. 17 codebook, then the maximum value of the number of coefficient for a layer can be KlNZ=K1Mυ, which corresponds to an all-one bitmap, hence can be omitted. Such omission can be beneficial since the overhead saving can be large. There are four alternatives to down-select from.

- Alt 1: whether bitmap is absent is determined based on parameters. For example, when β=1 or Mυ=1, the bitmap is absent.

- Alt 2: whether bitmap is absent is determined by the UE. For example, this can be based on KNZ value in UCI part 1, e.g. KNZ=K1Mυυ, bitmap is absent; otherwise, bitmap is present.

- Alt 3: in addition to Alt 2, an additional field to report whether bitmap is absent for specific layer(s).

- Alt 4: the bitmap is absent if the number of coefficients is sufficiently small, i.e. K1Mv ≤ δ.

Between these alternatives, Alt 1 is not desired since it restricts UE implementations by forcing the UE to report all coefficients. Alt 4 is unclear since the parameter δ is not defined, and also, it is different from other alternatives since it is for the case when the number of coefficients is small (other alternatives are about the case when the number of coefficients is large). Alt 2 and Alt 3 both are based on Rel. 16 design, i.e., the UE determines #NZ coefficients for reporting (e.g. based on channel measurement), and is not forced to always report all coefficients by configuration. However, Alt 2 is simpler since it doesn’t require any new UCI parameter (required in Alt 3) and is likely to perform similar to Alt 3.

Proposal 16: for bitmap being absent, support Alt 2 (whether bitmap is absent is determined by the UE, e.g., based on KNZ value in UCI part 1).

## Rank 3-4 design

In this section, we provide our view about the rank 3-4 codebook design.

- It is desired to keep the UE complexity and CSI payload of the rank 3-4 CSI reporting reasonable, if possible, comparable to rank 2 CSI reporting. One way to achieve this is by turning Wf OFF, i.e., the Mυ value for rank 3-4 can be fixed to Mυ=1. Also, similar to Rel.16, total number of NZ coefficients can be upper bounded by 2K0, and number of NZ coefficients for each layer can be upper bounded by K0.

- A layer-common (PS) W1 reporting has been agreed for rank 2. For rank 3-4, however, layer-common SD basis vectors are likely to incur performance loss (when compared with layer-specific or layer-pair-specific basis vectors) since multiple layers are likely to have different SD profiles. We therefore propose layer-specific or at least layer-pair-specific SD basis vectors for rank 3-4.

- The W2 reporting can be similar to Rel.16, i.e., a layer-specific bitmap, SCI, and amplitude/phase of NZ coefficients should be supported in Rel.17.

- Finally, the same quantization scheme is used for all layers.

Based on this, we propose the following.

Proposal 17: For rank 3-4, support

- Mv=1 only

- reuse Rel.16 design: KNZ≤2K0 and KlNZ≤K0

- at least layer-pair-specific W1: one W1 for layer 1-2 and another W1 for layer 3-4

- W2:

  - layer-specific bitmap, SCI, and amplitude/phase of NZ coefficients

  - same quantization scheme for all layers

## Parameter combinations

Our view about the codebook parameters are as follows:

- #CSI-RS ports (PCSIRS): the maximum value of PCSIRS has been agreed to be 32. Regarding the minimum PCSIRS value, in our view, PCSIRS=2 should be supported in addition to {4,8,12,16,24,32}. This can be beneficial for scenarios in which the UL-DL reciprocity is strong (including the TDD scenario) and the channel has very few (1 or 2) strong clusters. Note that a 2-port beamformed codebook (Class B, K=1) is supported in LTE.

- Since the special configuration of W1 being identity is agreed, the value K1 belongs to {2,4,8,12,16,24,32}. Hence, at least one configuration with α=1 needs to be supported. Now, for a given PCSIRS, the value K1 should not be too small since similar performance can be achieved with a lower value of PCSIRS and the overhead and complexity can also be reduced. For example, the performance similar to the configuration PCSIRS=32 and K1=16 (α=1/2) is likely to be achieved with the configuration PCSIRS=24 and K1=16 (α=3/4) or PCSIRS=16 and K1=16 (α=1). Also, since the total number of parameter combinations in Rel.17 needs to be comparable to that (8) in Rel. 16 and the special configuration needs to be supported, one way to achieve both is by supporting α∈1,34.

- Similarly, the parameter β can also be configured from 1,34, since the performance similar to β=12 and K1Mυ=A can be achieved by another configuration in which β=34 and K1Mυ~23A.

- Therefore, the parameter combinations can be triples (α,Mυ,β), where

  - α=1,34

  - β=1,34

  - Mυ=1, 2

The simulation results for different parameter combinations (α∈12,34, 1, Mυ∈1,2,β∈12,34, 1) are shown in Figure 7 and Figure 8 for rank 1 and rank 2, respectively. The three dots on each curve correspond to β∈12,34, 1. We can observe the following (as shown in the dotted red ovals):

Observation 10: based on the performance-overhead tradeoff

- parameter combinations with α=34, 1 are better than those with α=12

- parameter combinations with β=12 is either worse than or comparable to those with β=34, 1

Proposal 18: Regarding Rel. 17 codebook parameters,

- support P from {2,4,8,12,16,24,32}

- parameter combinations correspond to triples (α,Mυ,β), where

  - α=1,34

  - β=1,34

  - Mυ=1, 2.

Figure 7

Figure 8

## Other issues

Some of the other issues are discussed in this subsection.

- RI restriction: similar to Rel.16, the RI restriction should be supported via higher layer signalling.

- UCI omission: we prefer to reuse the Rel.16 UCI omission, with possible simplifications such as the following:

  - Shifting/remapping: as discussed earlier, there is no need for any shift/remapping operations in frequency domain.

  - Permutation: since the maximum value of Mv is 2, which is subject to UE capability, then there is no need for any FD permutation.

  - Bitmap partition: since the bitmap size is likely to be much smaller than Rel.16, there is no need for bitmap partitioning, hence it can be reported in G0 or G1 of UCI part 2.

  - FD basis indicator: for rank 3 and 4, we can fix Mv=1, thereby avoid the need for any FD basis indicator reporting.

Proposal 19: for Rel.17 codebook,

- Support RI restriction: reuse Rel.16 design

- simplify Rel.16 UCI omission mechanism and consider the following potential simplifications

  - no shifting/remapping operations in frequency domain

  - no FD permutation

  - no bitmap partition, i.e. bitmap is included in G0 or G1

  - no FD basis indicator for rank 3-4

Conclusion

In this contribution, the following observations and proposals are made:

NC-JT CSI enhancements

Proposal 1: On CSI enhancements for multi-TRP, support CMR to be re-used as IMR for both non pre-coded and pre-coded CSI-RS

Proposal 2: For NC-JT CSI reporting enhancement, support and study followings:

- Support non-PMI based port-selection

- Study UCI structure optimized for dynamic NC-JT CSI report

Proposal 3: Support flexible RRC configuration for CSI measurement and reporting for mTRP NCJT CSI by configuring the CMR pairs for NCJT measurement hypothesis and the CMR sharing in CSI-ReportConfig.

Proposal 4: Design new CPU occupation rule for dynamic NC-JT CSI report.

Proposal 5: Support full and/or partial compression/omission/Sharing of PMI among single-TRP and NCJT hypotheses.

Proposal 6: Support the dynamic variation on the level of compression/omission/Sharing of PMI and the associated payload of PMI for single-TRP and NCJT hypotheses.

Proposal 7: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support RI restriction for single –TRP measurement hypotheses and no RI restriction is applied for Multi-TRP measurement hypothesis.

Observation 1: Relative random phase rotation as defined above has negligible impact on optimal PMIs selection when NCJT CSI is computed based on CMRs within a CMR pair.

Observation 2: Relative random phase rotation as defined above has negligible impact on signal-to-inter-TRP-interference ratio when NCJT CSI is computed based on CMRs within a CMR pair.

Observation 3: IMR in Rel-15/16 is not restricted to be within the same slot as the corresponding CMR.

Proposal 8: Strive for less restriction on the transmission of CMRs in a CMR pair for NCJT measurement hypothesis.

Proposal 9: Report NCJT and sTRP CSIs as separate reports.

- Modify priority equation in Section 5.2.5 in 38.214 by giving CSI for NCJT a higher priority.

- Include an indication for sharing of CRI, RI and PMI with sTRP measurement hypotheses in CSI part 1 for NCJT.

FDD CSI enhancements

Observation 4: according to current 38.214 specification,

- If pmiFormat is not provided, the frequency granularity of the CSI reporting is SB

  - This is against the agreement when Wf is OFF (meaning WB PMI) and cqiFormat = WB

- N3 is defined as the total number of precoding matrices

- This is incorrect when Wf is OFF since there is only one precoding matrix

Observation 5: Regarding turning Wf ON/OFF

- When Wf is OFF, PMI reporting format is WB

- When Wf is ON, PMI reporting format is SB,

- Wf OFF and Wf ON with Mv=1 are the same, and Wf can be fixed to an all-one vector.

Observation 6: for BWP size = 20 PRBs, Rel.17 codebook can achieve up to 16% gain in avg. UPT over the current specification support (i.e. based on Rel.15 Type I codebook)

Observation 7: regarding shift/remapping operations,

- There is no need for specifying Wf shift, W2 shift, and bitmap remapping.

- Shift/remapping for UCI omission is merely an optimization of a rare-event, hence doesn’t have any benefits.

Observation 8:

- when CSI-RS beamforming is according to the R value, R=4 shows small gain over R=1; however, R=4 has more UE complexity and CSI overhead than R=1

- when CSI-RS beamforming is the same for different R values, then

  - there is no performance gain with increasing R values (or increasing the length of DFT vectors comprising Wf)

  - R=1/4 achieves the best performance among R=1/4, ½, 1, and 4.

Observation 9: when the CSI-RS beamforming resolution is per PRB level, there is no noticeable difference between distributions for three strongest FD components for different R values;

- Implying reduced frequency selectivity of the beamformed channel, which leads to no performance gain with increasing R values

Observation 10: based on the performance-overhead tradeoff

- parameter combinations with α=34, 1 are better than those with α=12

- parameter combinations with β=12 is either worse than or comparable to those with β=34, 1

Proposal 10: Regarding turning Wf OFF,

- Wf OFF and Wf ON with Mv=1 are the same, and Wf is an all-one vector of length N3.

- pmiReportingFormat = WB

- support one of the following

  - Alt1: N3 = 1

  - Alt2: one of the following is included in the codebook description

    - Alt2-1: a single precoding matrix is indicated by the PMI

    - Alt2-2: N3 precoding matrices indicated by the PMI, but they are the same when Mv=1

Proposal 11: support Rel.17 codebook for BWP size < 24 PRBs with the current restriction in the specification, i.e. support only WB CSI implying Wf is turned OFF

Proposal 12: shift/remapping operations are not supported in Rel. 17 codebook.

Proposal 13: R>1, if supported, is configured only when Mv=2.

Proposal 14: Regarding Mv = 2,

- UE reporting its capability to support Mv = 2 shall also report whether it supports Mv=2 for P > 12 CSI-RS ports

Proposal 15: when N>Mv=2,

- window-size = N3 if the configured N>N3

- the reported FD indices include FD index 0

- Support N=3 or 5

Proposal 16: for bitmap being absent, support Alt 2 (whether bitmap is absent is determined by the UE, e.g., based on KNZ value in UCI part 1).

Proposal 17: For rank 3-4, support

- Mv=1 only

- reuse Rel.16 design: KNZ≤2K0 and KlNZ≤K0

- at least layer-pair-specific W1: one W1 for layer 1-2 and another W1 for layer 3-4

- W2:

  - layer-specific bitmap, SCI, and amplitude/phase of NZ coefficients

  - same quantization scheme for all layers

Proposal 18: Regarding Rel. 17 codebook parameters,

- support P from {2,4,8,12,16,24,32}

- parameter combinations correspond to triples (α,Mυ,β), where

  - α=1,34

  - β=1,34

  - Mυ=1, 2.

Proposal 19: for Rel.17 codebook,

- Support RI restriction: reuse Rel.16 design

- simplify Rel.16 UCI omission mechanism and consider the following potential simplifications

  - no shifting/remapping operations in frequency domain

  - no FD permutation

  - no bitmap partition, i.e. bitmap is included in G0 or G1

  - no FD basis indicator for rank 3-4

# References

- 3GPP RAN, RP-193133, Samsung, New WID: further enhancements on MIMO for NR

- Chairman’s notes, RAN1#103-e

- Chairman’s notes, RAN1#105-e

- Chairman’s notes, RAN1#106-e

- Chairman’s notes, RAN1#104-e

- Chairman’s notes, RAN1#104bis-e

- R1-2108357 Summary of CSI enhancements for MTRP and FDD (Round 1)

- R1-2106469, Discussion on CSI Enhancements for Rel-17, Huawei

Appendix A

Table 2: Simulation assumptions for FDD reciprocity

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL |
| Inter-BS distance |  | 200m |
| Reciprocity model |  | Based on Section 5.3 of TR 36.897, to generate FDD DL and UL channels |
| Antenna setup and port layouts at gNB |  | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 44 dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz with 15kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO, rank 1 only |
| MIMO layers |  | Up to 4 layers |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Overhead |  | CSI-RS, DMRS |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% for SU/MU-MIMO, rank 1 only |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | User throughput vs CSI feedback overhead |
| Baseline for performance evaluation |  | Rel-16 regular and PS eTypeII codebooks |
| SRS modeling for UL channel estimation |  | SRS periodicity with 5ms SRS error modeling in Table A.1-2 in 36.897 with Δ=9 BW: same as CSI-RS Number of ports = 2 Tx power = 23 dBm |
| FDD DL/UL calibration error model at gNB |  | According to R1-144943, with amplitude error (expressed in decibel of x=20log10a) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation, respectively. Both amplitude/phase errors are assumed to be constant during a simulation drop at time, and constant per 4 PRB at frequency. |

Appendix B

Figure 9: distribution of normalized power of FD components across UEs (for CSI-RS beamforming example 2)

Appendix C

WID:

Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

RAN1#104bis-e

Companies: HW, Nokia, CATT, MTK, Fraunhofer, QCM, SS, E///

Results comparing Mv=2 with Wf OFF (Mv=1)

| Company | 8 ports | 16 ports | 32 ports |
| --- | --- | --- | --- |
| C1 |  |  | No comparison |
| C2 |  |  | No comparison |
| C3 |  |  | Small gain (1%) |
| C4 |  | Mod gain (2-3%) |  |
| C5 |  | Small gain (~1%) | Very small gain (<0.5%) |
| C6 |  |  | No gain |
| C7 |  | Small gain (1-2%) | No gain |
| C8 | Gain (~4%) | Small gain (~1%) | No gain |
| Performance | Gain | Small gain | No gain |
| Overhead | Not too high | High | Highest |
| Complexity | Not too high | High | Highest |
| Overall trade-off among 3 metrics | Yes | No | No |

RAN1#105-e

Companies: HW, CATT, Fraunhofer, Nokia/NSB, SS, MTK, E///

Results comparing Mv=2 with Wf OFF (Mv=1)

| Company | 8 ports | 12 ports | 16 ports | 24 ports | 32 ports |
| --- | --- | --- | --- | --- | --- |
| C1 |  | 3.58% | 3.26% | 2.31% | 0.87% |
| C2 |  |  | <3% |  | <1.3% |
| C3 |  |  | ~1% |  | ~1% |
| C4 |  |  |  |  | No gain in lower overhead regime, very small gain in high overhead regime; New results: gain when there SRS BW < ½ CSIRS BW |
| C5 |  |  | Small gain (1-2%) |  | No gain |
| C6 |  |  | Mod gain (3-4%) |  |  |
| C7 | Mod gain (2-3%) |  | Mod gain (2-3%) |  | No gain |
| C8 |  |  |  |  |  |
| Performance | Gain |  | Small gain |  | No gain, very small gain |
| Overhead | Not too high |  | High |  | Highest |
| Complexity | Not too high |  | High |  | Highest |
| Overall trade-off among 3 metrics | Yes |  | No |  | No |