3GPP TSG RAN WG1 #106bis-e			R1-2109657

e-Meeting, October 11th – 19th, 2021

Source:	Intel Corporation, Qualcomm, NTT DOCOMO, Ericsson, Bosch

Title:	Rel-17 TEI proposal for mitigating half-duplex issue in NR V2X groupcast NACK-only case

Agenda item:	8.18

Document for:	Discussion and Decision

Introduction

Release 16 NR introduced support of sidelink V2X communication for unicast, groupcast, and broadcast V2X traffic services. One of the main differences of NR comparing to LTE sidelink is physical layer support of HARQ feedback and retransmissions. HARQ feedback is realized in two regimes: ACK-NACK information and NACK-only information, where the latter is assumed to be used for connection-less groupcast mode using communication range information.

During the normative and maintenance work on specification of NACK-only feedback regime it was identified that NACK-only groupcast regime is susceptible to half-duplex collision on sidelink control channel which could limit the achievable performance even at very short distances, i.e. on high SNR links [1]-[4].

In this TEI proposal, the technical issue of half-duplex collision in NACK-only HARQ feedback regime is first illustrated by evaluation and then proposed to be solved by a limited specification work.

# Motivation

In V2X sidelink resource allocation Mode-1 and Mode-2, UEs can use groupcast transmissions with NACK only feedback. In this mode of operation, receivers within target communication range from the transmitter provide NACK feedback in case of unsuccessful reception and do not provide ACK in case of successful reception. The susceptibility to half-duplex collisions can be illustrated by the following simple example:

- UE1, UE2, and UE3 operate with groupcast NACK only feedback and are group members (i.e., within target communication range from each other).

- UE1 and UE2 selected/were granted with resources in the same slot and transmitted in slot ‘n’.

- UE3 has successfully received UE1 and UE2 transmissions and thus has not provided HARQ feedback.

- UE1 and UE2 were not able to receive each other transmissions.

- Due to lack of NACK feedback UE1 and UE2 assume successful reception by UEs within target communication range.

  - In Mode-1, the UE reports ACK to gNB, and gNB considers successful transmission on SL, thus do not grant retransmissions

  - In Mode-2, the UE reports ACK to higher layer, and the higher layer does not grant retransmissions

- UE1 and UE2 stop transmissions of TBs without receiving each other transmissions.

The above problem was also confirmed by system level evaluations [1][3][4]. Figure 1 shows comparative analysis of the current Rel.16 design vs scenario when two blind retransmissions are used for groupcast communication with NACK only feedback according assumptions listed in Annex B. As it can be seen, the Rel.16 solution does not achieve PRR equal to 1 even at short communication distances.

Figure 1: Illustration of the Rel.16 groupcast communication with NACK only feedback

Since the half-duplex collision is a fundamental issue in distributed communication systems, it may not be possible to completely avoid it. But it is possible to apply a simple enhancement which reduces the issue dramatically. Such an enhancement is to allow a UE to transmit at least two TB (re-)transmissions without considering the feedback (Figure 2), thus increasing the chances that at least one of the two control channels were successfully received, as illustrated in the analysis above.

Figure 2: Illustration of mitigation of half-duplex for NACK-only feedback regime by ignoring first N feedbacks

Observation

- Rel-16 groupcast sidelink communication with NACK-only is susceptible to half-duplex issue which could limit the achievable reliability even at very high SNR links

# Possible Solutions

To mitigate the illustrated half-duplex problem for groupcast NACK only sidelink communication, there could be different solutions with difference spec impact, as was analyzed in the previous submissions [5][6].

Analyzing the comments received in two rounds of 3GPP TEI discussion, it seems the aspect of enabling/disabling it requires a bit more attention. In particular, it is discussed whether to keep Release 16 behavior in specification, either by configuration or by UE implementation. In our understanding, it is a matter of details of defining the feature in specification. For completeness we also discuss this aspect in current revision of the document.

Using Options 2 and 4 from the previous round of discussion [6] as a starting point, the following further detailed alternatives can be considered as part of Release 17 TEI work:

- Alternative 1: Introduce a configurable minimum number of transmissions N performed w/o considering the feedback from receivers

  - The (pre-)configuration may be provided per resource pool and per priority, similar to the maximum number of retransmissions specified in Release 16. If it is not provided, then Release 16 behavior is applied. Further, this feature may only be applicable to NACK-only feedback case or to other cases as well. Possible values for N include 1, 2, and 3.

- Alternative 2: Introduce a fixed number of minimum two transmissions performed w/o considering the feedback from receivers

  - To support both the proposed behavior and Release 16 behavior, introduce a (pre-)configuration to enable/disable this option, also considering UE capability for Rel.17

In our view, the proper way to introduce this feature is Alt.1 which gives the most flexible control. If the configurability of the minimum number of transmissions is not agreeable, then Alt. 2 can be used. The example text proposals for both alternatives are provided in Annex A.

Proposal

- Agree on Release 17 TEI work to introduce mitigation of half-duplex issue for sidelink V2X communication in groupcast NACK-only feedback regime by introducing a minimum number of retransmissions performed without considering the feedback from receivers

# Conclusions

In this contribution, we have shown a technical problem of NR sidelink groupcast communication with NACK only feedback and have shown simple changes that can address it with minimal changes in the Rel.16. Based on discussion we have the following proposal:

Proposal

- Agree on Release 17 TEI work to introduce mitigation of half-duplex issue for sidelink V2X communication in groupcast NACK-only feedback regime by introducing a minimum number of retransmissions performed without considering the feedback from receivers

References

- R1-1910650, “Resource Allocation Mode-2 for NR V2X Sidelink Communication”, Intel Corporation, October 2019 - R1-2103766, “Correction to PSFCH reception procedure for NACK-only case to mitigate half-duplex issue”, Intel Corporation, April 2021 - R1-2100673, “On Feasibility and Benefits of Inter-UE Coordination for Sidelink Mode-2 Design”, Intel Corporation, January 2021

- R1-2101910, “Reliability and Latency Enhancements for Mode 2”, Qualcomm Incorporated, January 2021 - R1-2105778, “Rel-17 TEI proposal for mitigating half-duplex issue in NR V2X groupcast NACK-only case regime”, Intel Corporation, Qualcomm Inc., May 2021 - R1-2108349, “Rel-17 TEI proposal for mitigating half-duplex issue in NR V2X groupcast NACK-only case”, Intel Corporation, Qualcomm Inc., NTT DOCOMO, Ericsson, Bosch, August 2021

# Annex A: Example Text Proposals for TS 38.213, section 16.3.1

In this section, we provide examples of text proposals that can be used as a starting point for further TEI discussion.

Text Proposal for Alternative 1

### 16.3.1 UE procedure for receiving HARQ-ACK on sidelink A UE that transmitted a PSSCH scheduled by a SCI format 2-A or a SCI format 2-B that indicates HARQ feedback enabled, attempts to receive associated PSFCHs according to PSFCH resources determined as described in clause 16.3. The UE determines an ACK or a NACK value for HARQ-ACK information provided in each PSFCH resource as described in [10, TS 38.133]. The UE does not determine both an ACK value and a NACK value at a same time for a PSFCH resource. For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report to higher layers. For generating the HARQ-ACK information, the UE can be indicated by a SCI format to perform one of the following - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "10" - report to higher layers HARQ-ACK information with same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "01" - report an ACK value to higher layers if the UE determines an ACK value from at least one PSFCH reception occasion from the number of PSFCH reception occasions in PSFCH resources corresponding to every identity MID of UEs that the UE expects to receive corresponding PSSCHs as described in Clause 16.3; otherwise, report a NACK value to higher layers - if the UE receives a PSFCH associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11" - if the UE is provided with [RRC parameter for the minimum number of ReTX] and if the transmitted PSSCH scheduled by the SCI format 2-A or SCI format 2-B is the initial transmission or a retransmission with index smaller than [RRC parameter for the minimum number of ReTX] - 1, report to higher layers a NACK value ; otherwise - report to higher layers an ACK value if the UE determines absence of PSFCH reception for the PSFCH reception occasion; otherwise, report a NACK value to higher layers

Text Proposal for Alternative 2

### 16.3.1 UE procedure for receiving HARQ-ACK on sidelink A UE that transmitted a PSSCH scheduled by a SCI format 2-A or a SCI format 2-B that indicates HARQ feedback enabled, attempts to receive associated PSFCHs according to PSFCH resources determined as described in clause 16.3. The UE determines an ACK or a NACK value for HARQ-ACK information provided in each PSFCH resource as described in [10, TS 38.133]. The UE does not determine both an ACK value and a NACK value at a same time for a PSFCH resource. For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report to higher layers. For generating the HARQ-ACK information, the UE can be indicated by a SCI format to perform one of the following - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "10" - report to higher layers HARQ-ACK information with same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "01" - report an ACK value to higher layers if the UE determines an ACK value from at least one PSFCH reception occasion from the number of PSFCH reception occasions in PSFCH resources corresponding to every identity MID of UEs that the UE expects to receive corresponding PSSCHs as described in Clause 16.3; otherwise, report a NACK value to higher layers - if the UE receives a PSFCH associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11" - if a UE is provided with [RRC parameter enabling this R17 TEI feature] and if the transmitted PSSCH scheduled by the SCI format 2-A or SCI format 2-B is the initial transmission and the maximum number of retransmissions for the TB provided by higher layers is larger than one, report to higher layers a NACK value ; otherwise - report to higher layers an ACK value if the UE determines absence of PSFCH reception for the PSFCH reception occasion; otherwise, report a NACK value to higher layers

Annex B: Evaluation Assumptions

In this section, we provide the list of evaluation assumptions used for analysis in this contribution. The set of main parameters used for NACK only evaluation assumptions is provided in Table 1.

Table 1: System level evaluation assumptions for NACK only groupcast communication studies

| Parameter | Value |
| --- | --- |
| Deployment scenario | - Highway Option A scenario from NR V2X methodology - Vehicle speed = 70 km/h |
| Channel model | TR 37.885, NR V2X Channel Model |
| Spectrum allocation | Carrier frequency: 6GHz Simulated Bandwidth: 40 MHz |
| Subcarrier spacing | 30 kHz |
| Tx/Rx Antenna Ports | 1Tx/2Rx |
| Communication Mode(s) | Groupcast, connection-less |
| UE-to-UE Association | - UE is considered as member of the group based on location principle - Each UE may be a member of a multiple groups |
| Traffic model | Aperiodic variable packet size traffic (TR 37.885 Aperiodic Model 1 traffic): - Packet size: uniform in the range [200..2000] B with quantization step of 200 B - Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms - Latency requirement: 50 ms |
| TTI structure | NR Slot TTI: - Slot w/o PSFCH zone: 1 AGC symbol, 1 GAP symbol, 2 PSSCH symbols w/ pilots - Slot w/ PSFCH zone: 1 AGC symbol, 2 GAP symbol, 2 PSSCH symbols w/ pilots, 2 symbols for PSFCH |
| PSFCH Zone Allocation | Enabled, in every 2nd slot |
| Sidelink control TX parameters | 64 Bits QPSK Modulation |
| SCI/Data frequency resource allocation | - PSCCH: 10 PRB - PSSCH: 35 PRB |
| SCI/Data time resource allocation | - PSCCH: 3 Symbols - PSFCH: 2 Symbols - PSSCH: Remaining symbols in slot |
| Data Packet Tx parameters | Aperiodic variable packet size evaluations: - 200 B packet: QPSK, 2 TTI - 400 B packet: 16-QAM, 2 TTI - 600 B packet: 16-QAM, 2 TTI - 800 B packet: 16-QAM, 2 TTI - 1000 B packet: 16-QAM, 3 TTI - 1200 B packet: 16-QAM, 3 TTI - 1400 B packet: 64-QAM, 3 TTI - 1600 B packet: 64-QAM, 3 TTI - 1800 B packet: 64-QAM, 4 TTI - 2000 B packet: 64-QAM, 4 TTI |
| HARQ Feedback | Enabled, NACK-only |
| Feedback reporting distance | UEs within 200m report HARQ feedback |