3GPP TSG-RAN WG1 Meeting #106bis			R1-2109707

E-Meeting, October 11th – 19th, 2021

Agenda item:	8.17.2

Title:	Views on Rel-17 UE features for supporting NR from 52.6 GHz to 71 GHz

Source:	NTT DOCOMO, INC.

Document for:	Discussion and Decision

- Introduction

This contribution describes our views on UE features for supporting NR from 52.6 GHz to 71 GHz in Rel-17, captured in R1-2108679 [1].

- Discussion

2.1. General views

A general issue would be how to consider FR-related differentiation. In this WI, companies discuss on various types of new features and enhancements of the existing NR functionalities to extend NR to 52.6 – 71 GHz frequency range. Any UE feature to be supported in this WI will then be applicable at least for 52.6 – 71 GHz. However, companies may or may not argue that some of the UE features to be specified in this WI could be technically beneficial even in another frequency range, and then desire to discuss whether/how to expand the applicability of such UE features to other frequency ranges. It may consume quite a lot of time in RAN1 to discuss the range of applicability one by one. Note that 52.6 – 71 GHz frequency range at least include unlicensed bands, while licensed bands may also be identified in the future. The WI also target the support for both licensed and unlicensed band in this frequency range. This fact may also make the discussion more complex.

Also, frequency range notation itself should be carefully considered. In the latest WID for supporting NR from 52.6 GHz to 71 GHz [2], two important aspects on frequency range definition regarding beyond 24 GHz are captured; one is to extend the definition of FR2 up to 71GHz, and the other is to introduce new FR sub-labels, FR2-1 and FR2-2, to be used for differentiating 24.25 – 52.6 GHz and 52.6 – 71 GHz if needed. Therefore, as well as FR1/2 differentiation, FR2-1/2-2 differentiation may also need to be considered.

With the consideration above, how to have FR-related differentiation would depend on each UE feature in our view. We see the following alternatives at this stage.

- One potential approach to easily solve this issue could be to define all the UE features to be specified in this WI per-band (or per BC). With this, UE can report its capability regarding functionalities for 52.6 – 71 GHz operation per band that the UE supports, which means vendors have a freedom for its own implementation. However, it may increase the overhead for UE capability signalling depending on the number of bands/band combinations to be specified.

- For UE features which can be applied regardless of licensed or unlicensed band, extending per-FR capability signalling may be another possibility. For example, by enabling per-FR capability signalling to differentiate FR2-1 and FR2-2, it would be possible to indicate a certain UE feature is applicable for FR2-2 only if needed. Or, if a UE feature is applicable to both FR2-1 and FR2-2 without any difference, just to use the existing per-FR capability signalling would also be possible. By defining in this manner, vendors still have a freedom to implement a certain feature for a certain frequency range, while overhead for capability signalling can be suppressed.

- To decrease signalling overhead more, per-UE signalling with some Notes can also be considered. This approach, however, may be applicable to particular UE features only, for which the targeted FR is crystal clear. For example, if a UE feature is clearly applicable for FR2-2 unlicensed band only, it could be possible to define it as a UE feature with per-UE capability signalling with a Note saying i.e., “this is applicable only for unlicensed band in FR2-2”. While this approach achieves much less overhead on UE capability signalling, an issue may be less implementation flexibility.

Table 1. Comparison of FR differentiation approaches

| FR differentiation | Flexibility for implementation | UE capability signalling overhead |
| --- | --- | --- |
| Per-band | Very flexible | Heavy |
| Per FR | Less flexible | Relatively light |
| Per UE | Much less flexible | Light |

Proposal 1: For the discussion on Rel-17 UE features at least regarding 52.6 – 71 GHz WI, the following alternatives can be considered in case-by-case manner, in terms of FR differentiation.

- Alt 1: define as per-band

- Alt 2: define as per-FR

  - Differentiation of FR2-1/2-2 may or may not be needed

- Alt 3: define as per-UE

  - A fixed limitation (e.g., as a Note) on applicable frequency range may be needed

2.2. Views on the preliminary list

Since a number of discussions are still on-going in RAN1, only a few UE features are captured in the list now. In general, we believe most of the discussion points regarding UE features need to wait for RAN1 progress a bit more.

However, even in the current situation, we think an important UE features to be clearly supported in this WI is missing in the list; that is, the ones for supporting multi-PDSCH/PUSCH scheduling. As the support of this functionality had already been agreed in RAN1, there would be no reason not to capture it even now. Given that only 120 kHz SCS is to be mandatory for UEs supporting NR in 52.6 – 71 GHz, whether to support multi-PDSCH/PUSCH scheduling in 52.6 – 71 GHz should be reported per SCS that the UE supports.

Proposal 2: Capture the functionality of multi-PDSCH/PUSCH scheduling as a Rel-17 UE feature for supporting NR from 52.6 to 71 GHz:

- It could be per SCS

- For example, capture “Support multi-PUSCH[/PDSCH] scheduling by single DCI” in FG 24-1, 24-4 and 24-5

- Conclusion

Our proposals/observations are summarized as follows:

Proposal 1: For the discussion on Rel-17 UE features at least regarding 52.6 – 71 GHz WI, the following alternatives can be considered in case-by-case manner, in terms of FR differentiation.

- Alt 1: define as per-band

- Alt 2: define as per-FR

  - Differentiation of FR2-1/2-2 may or may not be needed

- Alt 3: define as per-UE

  - A fixed limitation (e.g., as a Note) on applicable frequency range may be needed

Proposal 2: Capture the functionality of multi-PDSCH/PUSCH scheduling as a Rel-17 UE feature for supporting NR from 52.6 to 71 GHz:

- It could be per SCS

- For example, capture “Support multi-PUSCH[/PDSCH] scheduling by single DCI” in FG 24-1, 24-4 and 24-5

- Appendix

- NR_ext_to_71GHz

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | General FR2-2 support | 1. Support 120KHz SCS transmission and reception 2. Support multi-RB PUCCH format 0/1/4 3. PRACH with 120KHz SCS and length 139/571/1151 4. Support multi-PUSCH[/PDSCH] scheduling by single DCI for the operation with 120 kHz SCS |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB based stand-alone support | 1. Support 480KHz SSB for initial access | 24-1 |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB based stand-alone support | 1. Support 480KHz SSB for initial access | 24-1, 24-2, 24-4 |  |  |  |  |  |  |  | From WID: - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) |  |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support | 1. 480KHz SCS for UL transmission 2. 480KH SCS for DL reception 3. 480KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 480KHz with X=4 5. PRACH with 480KHz and length 139/[571] 6. Multi-PUSCH/PDSCH scheduling by single DCI for the operation with 480 kHz SCS |  |  |  |  |  |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. |  |
| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support | 1. 960KHz SCS for UL transmission 2. 960KH SCS for DL reception 3. 960KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 960KHz with X=8 5. PRACH with 960KHz and length 139 6. Multi-PUSCH/PDSCH scheduling by single DCI for the operation with 960 kHz SCS |  |  |  |  |  |  |  |  | - From WID - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. |  |
| 24. NR_ext_to_71GHz | 24-? | Cat 3 or Cat 4 LBT support for FR2-2 unlicensed operation | 1. Cat 3 or 4 LBT support (not agreed yet if CW is supported, so it can be either Cat 3 or Cat 4 LBT for now. Will update when we have agreement) | 24-1 |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-? | Cat 2 LBT support for FR2-2 unlicensed operation | 1. Support Cat 2 LBT | 24-1 |  |  |  |  |  |  |  |  |  |

- References

- R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, 3GPP RAN1#106-e, August 2021

- RP-212637, “Revised WID: Extending current NR operation to 71 GHz”, Qualcomm, Intel, 3GPP RAN#93-e, Sept. 2021