3GPP TSG RAN WG1 #106bis-e			R1-2109710

e-Meeting, October 11th – 19th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on UE features for RedCap

Agenda Item:	8.17.6

Document for: 	Discussion and Decision

- Introduction

The preliminary RAN1 UE features list for Rel-17 NR is available in [1] for the discussion in RAN1#106bis-e. In this contribution, we present our views regarding the UE features for RedCap.

- Discussion

As discussed in our companion contribution [2],	the preliminary RAN1 UE features for RedCap in [1] is well constructed considering the RedCap UE type, i.e.,

- FGs 28-2 to 28-4 correspond to the capability candidates which may be captured to the RedCap UE type as follows

  - FG28-2: Supports either 1 or 2 Rx branches and corresponding maximum DL MIMO layers

  - FG28-3: Supports either FD-FDD or Type A HD-FDD operation for FR1 FDD bands

  - FG28-4: Supports either DL up to 64 QAM or up to 256 QAM for FR1

- The note of FG28-1 captures that RedCap UEs do not support CA or DC

- FG 28-1 is the prerequisite FG of FGs 28-2 to 28-4 and hence, these capabilities can be used only for RedCap UEs

Therefore, the 2nd and 3rd objectives to define the RedCap UE type, i.e., constraining the use of those RedCap capabilities only for RedCap UEs and preventing RedCap UEs from using capabilities not intended for RedCap UEs including at least CA, DC and wider BWs, are satisfied even if RedCap UE type is defined only by FG28-1. In that sense, we propose to keep FGs 28-1 to FG28-4. More details of our views on each FG are summarized as below:

- FG 28-1: RedCap UE

  - FG 28-1 should be kept for “RedCap UE” as basic FG for RedCap UEs

    - In “mandatory/optional” column, it should be clarified that “RedCap UEs must indicate this FG is supported.”

  - Components of FG 28-1 should be kept

  - Report type of FG 28-1 should be per UE

  - FDD/TDD and FR1/FR2 differentiation are not necessary for FG 28-1

  - Note of FG 28-1 should be kept

- FG 28-2: Number of UE Rx branches and DL MIMO layers for RedCap UE

  - FG 28-2 should be kept for “Number of UE Rx branches and DL MIMO layers for RedCap UE”

  - Components of FG 28-2 should be kept

  - FG 28-1 should be the prerequisite FG of FG 28-2

  - Report type of FG 28-2 should be per band

    - FDD/TDD and FR1/FR2 differentiation are not applicable to FG 28-2

  - Note of FG 28-2 should be kept

- FG 28-3: Half-duplex FDD operation for RedCap UE

  - FG 28-3 should be kept for “Half-duplex FDD operation for RedCap UE”

  - Component of FG 28-3 should be kept

  - FG 28-1 should be the prerequisite FG of FG 28-3

  - Report type of FG 28-3 should be per band

    - FG 28-3 is applicable only to FR1 FDD bands

- FG 28-4: DL 256QAM support for RedCap UE

  - Currently DL 256QAM is supported/reported as mandatory with capability signalling for FR1 and optional with capability signalling for FR2 using different fields pdsch-256QAM-FR1 and pdsch-256QAM-FR2, respectively. Therefore, FG 28-4 should be used only for FR1 and pdsch-256QAM-FR2 is reused for FR2.

  - FG 28-4 should be kept for “DL 256QAM support for RedCap UE for FR1”

  - Components of FG 28-2 should be kept with following revisions

    - 1. Support of 256QAM for PDSCH for RedCap UE for FR1

    - 2. Support of 256QAM MCS table (Table 5.1.3.1-2 in TS 38.214) for PDSCH for RedCap UE for FR1

    - 3. Support of CQI table 2 (Table 5.2.2.1-3 in TS 38.214) for RedCap UE for FR1

  - FG 28-1 should be the prerequisite FG of FG 28-4

  - Report type of FG 28-4 should be per band

    - FDD/TDD differentiation is not applicable to FG 28-2

    - FG 28-4 is applicable only to FR1

  - Note of FG 28-4 should be kept

- FG 28-5: UL 256QAM support for RedCap UE

  - Currently DL 256QAM is supported/reported as optional with capability signalling for both FR1 and FR2 using pusch-256QAM per band and hence, it can be used for RedCap UEs as well. In that sense, we don’t see any motivation to introduce FG 28-5 in addition to pusch-256QAM.

  - FG 28-5 should be removed

References

- Moderators (AT&T, NTT DOCOMO, INC.), R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Aug. 2021.

- NTT DOCOMO, INC., R1-2109687, Discussion on RAN1 aspects for RAN2-led features for RedCap, Oct. 2021.