3GPP TSG RAN WG1 #106bis-e		R1-2110029

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.3.2

Source:	Apple

Title:	URLLC uplink enhancements for unlicensed spectrum

Document for:	Discussion/Decision

# Introduction

A work item on enhanced IIoT and URLLC was approved in RAN#86, and the WID was further updated in RP-201310 (RAN#88, June 2020) [1]. One objective is about the uplink enhancements for URLLC in unlicensed spectrum:

- Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]:

  - Specify support for UE-initiated COT for FBE with minimum specification effort

  - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

In this contribution, we discuss the remaining issues and share our views based on the agreements made in previous meetings (see Appendix A).

# UE-initiated COT for FBE

## Configuration parameters

In terms of enabling UE-initiated COT, there are a few options, and some options may be combined:

- Option 1: it is enabled per UE.

  - This can be easily done by reusing the FFP periodicity and offset configuration. That is, when the FFP periodicity and offset are configured for a UE, UE-initiated COT is enabled for the UE.

- Option 2: it is enabled per type of channel (e.g. PUSCH, PUCCH, SRS)

- Option 3: it is enabled per type of signal (e.g. PUSCH/data, HARQ-ACK, SR, P-CSI, SRS)

- Option 4: it is enabled per PHY priority

- Option 5: it is enabled per configuration for configured UL transmission.

More granularity certainly means better control of which transmissions can use UE-initiated COT. But the tradeoff between overhead and flexibility should also be considered.

It makes sense to always allow HARQ-ACK and SR to initiate a COT once UE-initiated COT is configured for the UE. However, it may be desirable to provide the flexibility to disable UE-initiated COT for P-CSI and/or SRS, for UE/gNB power saving purpose.

For PUSCH transmission, from channel access perspective, if there is no overlapping in the resources allocated to UL transmissions from different UEs (i.e., there is no collision), it always provides more channel access opportunities by enabling UE-initiated COT for all the UL transmissions, without much need to differentiate different types of transmissions.

Whether there is a need to introduce per-CG configuration to enable/disable UE-initiated COT depends on whether it makes sense to configure overlapping resources for multiple UEs. Overlapping CG PUSCH resources for multiple UEs was considered a valid use case in Rel-16, because it increases the channel utilization efficiency (larger probability that at least one of the UEs can access the channel). In Rel-16, if multiple UEs are configured with overlapping resources for UL transmissions (i.e., CG PUSCH) to increase the probability of UEs using the resources, the collision issue has been handled by random offset for UEs’ LBT so that one UE’s transmission can prevent other UEs from starting their transmission. However, for UE-initiated COT with FBE, the FFP boundary is pre-defined and cannot move. The Rel-16 mechanism does not apply any more.

Assume that we want to continue to support the configuration of overlapping resources for multiple UEs when UE-initiated COT is enabled in Rel-17. When the UEs share gNB’s COT, Rel-16 mechanism can be reused to resolve the collision. But when the UEs initiate their own COT, it would be better if only one UE or a few UEs (e.g. with orthogonal frequency resources) are allowed to initiate the COT to avoid collision. In this case, it can be useful to introduce configuration for each CG configuration to enable or disable UE-initiated COT.

With these considerations, we propose the following:

Proposal 1-1: UE-initiated COT is considered enabled once the FFP periodicity and offset are configured for a UE. Introduce a RRC parameter to disable UE-initiated COT for P-CSI and/or SRS.

- FFS whether to introduce a RRC parameter to disable UE-initiated COT for each CG configuration, which overrides the per-UE configuration for this CG.

## gNB sharing UE’s COT

The following has been agreed in case a DL transmission shares a UE-initiated COT. The remaining issue is whether broadcast or transmission to other UEs can be included in such a DL transmission burst.

Agreement

In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include at least scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

- FFS whether/how the DL transmission burst can include transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission

The issue here is that if such a DL transmission burst includes either broadcast signaling (e.g. SSB, SIB, etc) or transmission to other UEs, those UEs may assume this belongs to gNB-initiated COT and may transmit UL sharing gNB’s COT. For Rel-17 UEs, it is possible to introduce some new signaling so that the Rel-17 UEs knows whether a DL transmission belongs to a gNB’s COT or UE’s COT. However, any enhancements are not supported by Rel-16 legacy UEs, so the issue still exists for legacy UEs. In order to prevent this from happening also for the legacy UEs, we propose:

Proposal 1-2: When the gNB shares a UE’s COT for a DL transmission, to prevent another UE from mistakenly assuming this is gNB’s COT, adopt one of the following alternatives:

- Alt 1: It is left to gNB implementation.

- Alt 2: The DL transmission shall not include broadcast or UE-specific signaling/channels that can be detected by other UEs.

Alt 1 may be less restrictive in the sense that gNB may still include the broadcast or transmission to other UEs if the other UE(s) do not have any configured UL transmissions to transmit.

# Configured grant enhancements

One issue that should be considered is the orphan symbol handling. In Rel-16 URLLC, if an actual repetition consists of a single symbol, it is not transmitted. On unlicensed spectrum, if the orphan symbol is not transmitted, the UE would need to perform another LBT when continuing the transmission of the next repetition. This behavior can be modified to avoid the LBT.

Proposal 2-1: For PUSCH repetition Type B enhancements on unlicensed spectrum, orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted.

Regarding whether segmentation is performed around the idle period, the following two alternatives have been considered:

- For PUSCH repetition Type B on unlicensed spectrum, select one of the following:

  - Alt 1: If a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT for the nomial repetition or associated to UE’s FFP in case UE assumes UE-initiated COT for the nomial repetition, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16.

    - Segmentation before and/or after the idle period is applied when applicable.

      - FFS on handling the misalignment between gNB and UE on the COT initiator associated to an idle period

  - Alt 2: If a nominal repetition overlaps with an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT, the nominal repetition is dropped (i.e. no segmentation around the idle period).

The issue for Alt 1 is the FFS point, how to handle the misalignment between gNB and UE on the COT initiator. For example, as shown in Fig. 1, assuming the gNB had initiated the COT but UE did not detect it, the UE would initiate its own COT. According to Alt 1, the UE would segment nominal rep#2 around the idle period of UE’s FFP, resulting in 3 actual repetitions. On the other hand, the gNB knows it has initiated the COT and would assume no segmentation, i.e., only two actual repetitions (same as the two nominal repetitions). In this case, the signals from actual rep#2 and #3 would corrupt the buffer at the gNB. If we follow Alt 2, the UE would not do segmentation and simply drop nominal rep#2, and the gNB can use DMRS detection to detect whether the repetition has been transmitted or not.

In Rel-16 PUSCH repetition Type B design, the principle was that the segmentation itself is done based on semi-static configuration/signaling only, while the dynamic signaling is used only to decide whether to transmit or drop an actual repetition. This principle should also be followed here. Alt 2 appears to be the easiest and most straightforward solution, and we do not see the need to further optimize.

Figure 1 Example for PUSCH repetition Type B in case of misalignment between gNB and UE on the COT initiator

Proposal 2-2: Adopt Alt 2: If a nominal repetition overlaps with an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT, the nominal repetition is dropped (i.e. no segmentation around the idle period).

# Conclusion

In this contribution, we discussed the UE-initiated COT for FBE and potential configured grant enhancements for unlicensed spectrum, and proposed the following:

Proposal 1-1: UE-initiated COT is considered enabled once the FFP periodicity and offset are configured for a UE. Introduce a RRC parameter to disable UE-initiated COT for P-CSI and/or SRS.

- FFS whether to introduce a RRC parameter to disable UE-initiated COT for each CG configuration, which overrides the per-UE configuration for this CG.

Proposal 1-2: When the gNB shares a UE’s COT for a DL transmission, to prevent another UE from mistakenly assuming this is gNB’s COT, adopt one of the following alternatives:

- Alt 1: It is left to gNB implementation.

- Alt 2: The DL transmission shall not include broadcast or UE-specific signaling/channels that can be detected by other UEs.

Proposal 2-1: For PUSCH repetition Type B enhancements on unlicensed spectrum, orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted.

Proposal 2-2: Adopt Alt 2: If a nominal repetition overlaps with an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT or associated to UE’s FFP in case UE assumes UE-initiated COT, the nominal repetition is dropped (i.e. no segmentation around the idle period).

# Reference

- RP-210854, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, Nokia, Nokia Shanghai Bell, RAN#91e, March 2021.

- ETSI EN 301 893 V2.1.1, 5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU.

# Appendix A: Previous Agreements

## RAN1#102-e

Agreements:

- For semi-static channel access mode,

- If sensing is needed, it is performed immediately before the configured/scheduled transmission opportunity.

- For operation with semi-static channel access, the Rel-16 random starting offsets for UL configured grants with Full BW allocation when UE initiates a COT, is not supported.

Agreements:

- For semi-static channel access mode,

  - When gNB operates as an initiating device

    - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initates a COT

  - When a UE operates as an initiating device

    - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initates a COT

  - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB

    - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB

  - When the gNB shares a COT initiated by a UE during an FFP associated with the UE

    - The gNB is not allowed to transmit during the idle period of that the FFP in which the gNB shares the COT initiated by the UE

  - FFS whether/how to support additional restrictions to the idle period

Agreements:

- For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode

  - FFS the case when the UE is IDLE/INACTIVE mode

Agreements:

- A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9us immediately before the UL transmission burst.

Agreements:

- Conditions on the channel access procedures with respect to sensing duration and transmission gap for UE-initiated COT with UE-to-gNB COT sharing is similar as those for gNB initiated COT and gNB-to-UE COT sharing in Rel-16 by exchanging UE and gNB roles.

Agreements:

- UE-to-gNB COT sharing in semi-static channel access mode is supported.

  - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP.

    - FFS details

  - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP.

    - FFS whether/how UE to gNB COT sharing when the gap is >16us

Agreements:

For semi-static channel access mode,

o    Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT.

o    FFS: FFP Periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT.

Agreements:

- For semi-static channel access mode,

- FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling.

  - FFS on to be provided by SIB-1

- FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

Agreements:

- At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

Conclusion:

Further study and decide how to harmonize the CG features for Rel-16 URLLC and Rel-16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision.

## RAN1#103-e

Agreements:

- In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200ms

Conclusion:

- For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB.

Agreements:

  - UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16us is supported

Conclusion:

If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X.

If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X  is the FFP associated with Y.

Note 1: One of the devices X and Y is a UE and the other is its serving gNB.

Note 2: Whether or not there is additional restriction on idle period is still FFS.

Update on 11/11

Agreements:

- The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s).

  - Note: UE initiated FBE configuration is configured per serving cell

Agreements:

In semi-static channel access mode,  FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT.

o    Note: Any value for the period, shall be at least 1ms and at most 10ms.

o    Note: Aim for low complexity operation to handle gNB and UE COT interactions

Agreements:

In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT.

- UE determines the initiator of a COT based on at least one of the following alternatives:

  - Alt 1: Introduce additional bit field in the scheduling DCI

  - Alt 2: Based on ChannelAccess-CPext field in DCI

  - Alt. 3: Based on a predetermined rule(s)

  - Alt. 4: Based on RRC signalling

  - Alt. 5: Based on MAC CE

  - FFS other alternatives

- FFS on overriding possibility and/or the assumption

- Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreements:

In semi-static channel access mode:

- When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

  - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration.

- When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE:

  - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

- FFS on other conditions for determining the corresponding UE or gNB initiated COT

- Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreements:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

- Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

- Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions.

## RAN1#104-e

Agreement:

- PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG

  - FFS whether/how to enhance

Agreement:

- In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5,10}.

  - FFS on other values

Agreement:

- In semi-static channel access mode:

- An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT

- FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability

- FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary.

  - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT

    - FFS on X (e.g. X=0, or X= even index number)

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Agreement:

- In semi-static channel access mode, sharing a UE initiated COT through the gNB to other intra-cell UEs for UL transmissions, is not supported.

## RAN1#104b-e

Agreements:

- Support explicit RRC configuration for the UE-FFP parameters including period and offset in RRC connected mode.

Agreements:

- For semi-static channel access mode, the offset value for configuration of a UE-FFP for a serving cell has a symbol level granularity.

Agreement:

- For semi-static channel access mode, in addition to the agreed set of period values for configuration of a UE-FFP for a serving cell:

  - Do not support any additional period value

Agreement:

- For semi-static channel access mode, the starting point of first UE FFP for a serving cell

  - is relative to the boundary of the radio frame of even index number (i.e. X=even indexed number in RAN1#104-e agreement).

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The UL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI  DL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The DL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

Agreement:

- Select one of the following options (aiming for RAN1#105-e):

  - Option 1: Do not support PUSCH repetition Type Bwhen using based on NR-U Rel-16 based CG for unlicensed band operation.

  - Option 2: Support enhancements of PUSCH repetition Type B when using based on NR-U Rel-16based CG for unlicensed band operation. FFS whether/how to enhance

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP.

  - FFS on details

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details

Conclusion:

- In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT

  - Note: the gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI.

Agreement:

- Option 2-b and option 3 are not considered further for the agreement in RAN1#103-e regarding CG harmonization

## RAN1#105-e

Agreement:

- Option 1 is taken in the following agreement:

Agreement:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

  - If cg-RetransmissionTimer-r16 is configured, “CG-UCI based procedures” should also be enabled by X.

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions

- Alt-a is taken in the following agreement:

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT..

- Alt-a is taken in the following agreement:

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

## RAN1#106-e

Agreement

In semi-static channel access mode, the content in a scheduling DCI that indicates the assumption on the COT-initiator for the scheduled transmission is determined based on the channel access field in the DCI.

Agreement

In semi-static channel access mode,

- The inclusion of the channel access field in Rel-16 DCI 0_1 and 1_1 in Rel-17 DCI 0_2 and 1_2, respectively, is supported.

Agreement

In semi-static channel access mode, the size of channel access field in a scheduling DCI with format 0_0/1_0, 0_1/1_1, 0_2/1_2 is 2 bits.

Conclusion

Any UL or DL transmission that is expected to occur, should be associated to a Channel Occupancy (CO) with a corresponding FFP. When a transmission is associated to a CO with a corresponding FFP:

- The association of the transmission to a CO with corresponding FFP is based on either of the following assumption:

  - “Initiating COT”: This assumption implies that the transmission would initiate a CO corresponding the FFP.

  - “Sharing COT”: This assumption implies that the transmission would share a CO corresponding to the FFP.

- The association assumption is validated as follows:

  - “Initiating COT” assumption is validated if the transmission would start at the FFP boundary and would end before idle period of the FFP.

  - “Sharing COT” assumption is validated if the transmission would start after the FFP boundary and would end before idle period of the FFP and the CO corresponding to the FFP is initiated.

- A transmission based on a CO association assumption can occur if the CO association assumption is validated and if the following sensing conditions are met:

  - For CO association assumption as “Initiating COT”:

    - If a CCA is successful before the transmission.

  - For CO association assumption as “Sharing COT”

    - If the gap between the beginning of the transmission and the end of previous one sharing the same CO in that FFP is more than 16us and if a CCA is successful before the transmission.

    - IF the gap between the beginning of the transmission and the end of previous one sharing the same CO in that FFP is at most 16us

Agreement

In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include at least scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

- FFS whether/how the DL transmission burst can include transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission

Agreement

In semi-static channel access mode, the content of the channel access field in a DCI scheduling a UL transmission for a UE determines an index to a row in Table 1 with Alt-1 (Option 1)

TABLE 1

| Bit field mapped to index | Channel Access Type | The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211] | Initiator of a channel occupancy associated to UL transmission described in Clause x.x in TS 37.213 |
| --- | --- | --- | --- |
| 0 | No sensing as defined in Clause 4.3 in TS 37.213 | 0 | Alt-1:gNB Alt-2: UE-initiated COT if condition A, otherwise gNB’s COT |
| 1 | No sensing as defined in Clause 4.3 in TS 37.213 | 2 | Alt-1:gNB Alt-2: UE-initiated COT if condition A, otherwise gNB’s COT |
| 2 | 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 3 | 9us sensing [within a 25us interval] as defined in Clause 4.3 in TS 37.213 | 0 | UE |

Note: The last row in Table 1 is only applicable when the UE can operate as an initiating device as configured by gNB.

Agreement

In semi-static channel access mode, when the gNB schedules by a DCI a UL transmission in a later g-FFP that is different from the g-FFP that carries the scheduling DCI:

- The UE follows the indicated COT initiator as the following:

  - If the UE validates the indicated COT initiator assumption and satisfies the applicable sensing conditions, the transmission occurs. Otherwise, the transmission is dropped.

Conclusion

There is no consensus in RAN1 to support UE-initiated COT for semi-static channel occupancy in IDLE/INACTIVE mode.

Agreement

Replace “9us sensing [within a 25us interval] as defined in Clause 4.3 in TS 37.213” with “9us sensing as defined in Clause x.x in TS 37.213” in the last row of Table 1 in the previous agreement and add the following notes to Table 1:

- Note 1: The intention of Clause x.x above is to describe the LBT procedure from a UE perspective when this operates as initiating device.

- Note 2: A UE operating as initiating device may transmit an UL transmission burst(s) within its u-FFP immediately after sensing the channel to be idle for at least a sensing slot duration  if the gap between the UL transmission burst(s) and any previous transmission burst is more than

Agreement

When a UE operates as an initiating device, and the gNB shares a UE’s FFP for DL transmission, regardless of the gap between any UL and DL bursts, no restriction is imposed on the maximum duration of each of the DL bursts such that each can continue until the UE FFP idle period starts.

- Note: The applicability of the EDT calculation based on the UE’s transmit power to the UE COT initiation in accordance to the UL-DL gap duration and/or the content of the DL burst is separately discussed

Agreement

Do not support PUSCH repetition Type B based on NR-U Rel-16 CG for unlicensed band operation.