3GPP TSG-RAN WG1 #106-bis-e	R1-2110177

e-Meeting, October 11th – October 19th, 2021

Agenda item:    8.2.6

Source:              Qualcomm Incorporated

Title:                  Channel Access Mechanisms for NR in 52.6 to 71GHz band

Document for:  Discussion and Decision

# Introduction

At the end of RAN1#106e, multiple discussion points had not reached convergence and consensus. In this paper we present potential proposals better designed for consensus The relevant agreements, FFS elements and guidelines for different considerations are captured in the section that discusses them.

# Energy Detection and related computation aspects

### Pout working assumption

The working assumption on the definition of Pout to be used in energy detection threshold computation requires clarification.

Proposal from Discussion: (RAN1#106e) Confirm the working assumption on Pout definition in RAN1 #104bis-e with the following updates: - For Pout in EDT determination, define Pout to be at least the maximum of mean EIRP of each transmission burst during the COT at the node initiating the COT.

The ‘at least’ formulation leaves open the room for (1) selection of a larger Pout to be conservative as implementation

(2) Allow for design of medium access for COT sharing case if COT sharing node is using a larger EIRP than the COT initiating node

Proposal 1:  Confirm the working assumption on Pout definition in RAN1 #104bis-e with the following updates:

- For Pout in EDT determination, define Pout to be at least the maximum of mean EIRP of each transmission burst during the COT at the node initiating the COT.

### Adjustment to ED computation and threshold for beamformed transmission and beamformed sensing

We have maintained that the criterion for LBT based silencing, namely comparison of any quantity related to sensed energy with a threshold, needs to be cognizant of transmit beam and beamformed sensing to be universal, fair and unambiguously implemented.

On further adjustment of ED Threshold based on the sensing beam and the transmission beam – such that further adjustments do not violate EDT requirements per regulation, support for additional adjustment to ED computation/threshold that includes the relationship between transmit beamforming and sensing beam, has a support of majority of companies.

In the discussion from RAN1#105e/106e meeting, we support Alt A captured below.

Proposal from Discussion: (RAN1#105e) [2] On further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations), please provide your view for the following - Alt A: Support additional adjustment to Energy Detection computation/threshold to include transmit beamforming and/or sensing beam - Alt B: No additional adjustment to Energy Detection computation introduced (Energy measurement directly compared with baseline EDT agreed no matter which transmit beamform(s) and sensing beam(s) are used Proposal from Discussion: (RAN1#106e) [3] The value of the adjustment to ED threshold based on the sensing beam and the transmission beam should be zero if - Alt 1. Same beam is used for transmission or reception. - Alt 2. Pseudo-omni beam is used for sensing - Alt 3. When 0dBi sensing beam is used - Alt 4. When TX antenna gain matches max EIRP

In general, the matched scenario, namely the use of same antenna and beam for sensing and for transmission, should be ideally taken as a baseline scenario, where beam-based adjustment to energy sensing is not necessary.  While keeping the transmission beam the same, if the sensing beam is different than that used for transmission, the measured values of energy are likely to be different than matched beam case, and hence an adjustment to the comparison equation is needed to maintain that the LBT backoff is commensurate with the transmission beam.

Proposal 2:  Support additional adjustment to Energy Detection computation/threshold whenever the sensing beam has a lower beamforming gain than the transmission beam.

Additional discussion is provided in the section on directional sensing.

# LBT Bandwidth aspects

The key agreement related to bandwidth used for sensing, reached in RAN1#106-e is as follows:

Agreement: - For LBT for single carrier transmission, gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) (Alt SC.1. in earlier agreements) - For LBT for multi-carrier transmission in intra-band CA, gNB/UE performs multiple LBT, one for each channel bandwidth separately (Alt CA.1. in earlier agreements) - FFS: Additional support of performing single LBT over all CCs (Alt CA.2. in earlier agreements)

The outcome of a single LBT over all CC’s can be obtained by logical AND of LBT over individual CCs without any ambiguity. We believe additional support for performing single LBT over all CCs need not be specified.

Proposal 3:  For LBT for multi-carrier transmission in intra-band CA, the support for single LBT over all CC is not required.

# Sensing Structure aspects

For energy measurement during deferral period, the Ran1#106e discussions converged to selection between one of the following alternatives without reaching consensus.

Agreement: For energy measurement in 8us deferral period, at least a single measurement within 8us is performed, and the measurement duration is selected from one of the following alternatives: - Alt 1: At least 3+X us (FFS X, such as X=1). - Alt 2: At least X us, where X is the same as the minimum measurement duration in a 5 us observation slot and is within the 5 us observation slot. - Alt 3: At least a contiguous duration of X+Y us where the Y us part of the measurement is done at the end of the first 3 us and X is the same as the minimum measurement duration in a 5 us observation slot and is at the beginning of the 5 us duration. Agreement: For energy measurement in 8us deferral period, Alt 2 is supported while Alt 1 and Alt 3 can be considered as gNB/UE implementation (Alt. 1/2/3 are defined as per previous agreement)

The discussion on minimum requirements for sensing has been FFS. A simplified choice for the minimum duration of sensing is suggested to be 1us.

Proposal 4: Minimum requirement for sensing for both 5us and 8us slots should be 1us irrespective of bandwidth.

# COT Sharing aspects

Although ETSI regulation 302 567 does allow a COT to be shared with responding node without a requirement on sensing. On the other hand, there are regional regulations that do not permit such a sensing free transmission, e.g., Japan. Ability for NR procedures to support operation in unlicensed bands requires a configurable design. Therefore, there is a strong incentive to support a design that works for both alternatives. This also is a strong incentive to specify Cat 2 LBT.

Agreement: On COT sharing from an initiating device transmission to responding device transmission, support both of the following two alternatives - Alt 1: No maximum gap defined between the initiating device transmission and responding device transmission. A responding device transmission can occur without LBT with any gap within the maximum COT duration - Alt 3: Define a maximum gap Y, such that a responding device transmission can occur without LBT only if the transmission starts within Y from the end of the initiating device transmission. If the responding device transmission starts after Y from the end of the initiating device transmission, a Cat 2 LBT is needed before the responding device transmission. - The Cat 2 LBT uses the same sensing structure as the 8 us initial deferral period as in eCCA - Further downselect between the following options: - Option 1: Y=8 us (motivated by need to operate in all regions) - Option 2: Y=a multiple number of OFDM symbols - Option 3: gNB determines Y (for example, according to local regulation) - Cat. 2 LBT is a UE capability - The usage of the two alternatives is a gNB choice and depends at least on local regulations. Note: Alt. 3 is motivated by the regulations in Japan but use of Cat. 3 LBT is also an option for operation in Japan and Cat. 2 LBT is not restricted for use only in Japan. Note: Maximum gap allowed without Cat 2 LBT between two initiating device transmissions is to be separately discussed Note: Other use cases of Cat 2 LBT will be separately discussed

Among the alternatives for FFS, we propose Option 3 as a compromise solution for Alt-3.

Proposal 5: For Alt-3 for COT sharing, gNB determines the value of Y and is transparent to UE.

Note that whether to use Cat 2 LBT is decided per UE separately under the agreement.

# Cat 2 LBT aspects

In RAN1#104e, RAN1 reached the following agreements regarding Cat 2 LBT.

From RAN-104-bis-e Agreement: For Cat 2 LBT, downselect from the following alternatives - Alt 1: Do not introduce Cat 2 LBT for 60GHz unlicensed band operation - Alt 2: Introduce Cat 2 LBT for 60GHz unlicensed band operation Agreement: If Cat 2 LBT is introduced, the following use cases can be further studied: - Resume transmission after a gap Y: Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y) - COT sharing: Cat 2 LBT may be used before transmission by a responding node sharing a COT - Multi-Beam LBT: Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z) - Rx-Assistance: Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signaling Other use cases not precluded. FFS if Cat 2 LBT is mandated for each use case or not.

Cat 2 LBT is a single measurement of energy for a specified duration. Cat2 LBT, also called one-shot LBT (Type 2A, 2B for Rel .16) is not required by regulation and hence the principal question is whether defining such a measurement/sensing and tying it to medium access procedures brings benefits. As agreed in RAN1-106e, Cat 2 LBT is a UE Capability. Primary use case agreed for Cat 2 LBT on the UE side is COT Sharing, with a a few that at least fixed duration sensing is a regulatory requirement in regions such as Japan. A Cat 2 LBT support in NR facilitates unlicensed operation in those regions.

Multiple companies have proposed the use of Cat 2 LBT as a component of other medium access procedures described in the language of the agreement from RAN1-104e. Following set of reasons collectively should be considered in favor of specifying Cat 2 LBT.

- Cat 2 LBT provides a predictable time for channel sensing compared to the variability of time for the CAT4 LBT

- Cat 2 LBT, with duration properly selected, can be readily implemented with little extra added complexity at any device that implements Cat 4 LBT. Note that as part of the eCCA procedure, sensing requirements for a 5us slot and an 8 us deferral period are already needed to be identified. It is relatively straightforward to adopt them to define a Cat 2 LBT procedure.

- Predictability permits scheduling of sensing in time, that is favorable for sensing procedure that includes beam sweeping.

We propose to extend the use of it as possible part of procedures for the use case of Multi-Beam LBT where Cat 2 LBT is performed before  switching to another beam in a COT.

A few candidate durations for Cat 2 LBT are (1) equal to Cat 4 deferral period, i.e., 8us (2) equal to Cat 4 deferral + 1 observation slot, i.e., 13 us (3) multiples of OFDM symbol duration for some subcarrier spacing.

Proposal 6: Introduce Cat 2 LBT for the use case of Multi-Beam LBT.

# Rx Assistance aspects

The agreement from RAN1#106-e stands as follows:

Agreement: For receiver to provide assistance in channel access, channel sensing and reporting need to be performed. The following schemes can be further considered. Target down-selection by RAN1 #106bis-e - Scheme 1: L1-RSSI based receiver assistance - Resource used for RSSI measurement - Alt 1: RSSI measurement is based on the time/frequency resources configured for ZP-CSI-RS - FFS: any enhancement needed for ZP-CSI-RS for this purpose (e.g., ZP-CSI-RS over all REs in BWP over one or more symbols). - Alt 2: Energy measurement on operating BW over indicated or specified number of symbols or time interval - L1-RSSI is reported in an AP-CSI report - L1-RSSI trigger in UL grant - FFS if L1-RSSI trigger can also be carried in DL grant - Timeline for L1-RSSI reporting is at least equal to AP-CSI reporting and RAN1 strives to tighten the timeline - Note: If L1-RSSI reporting timeline cannot be tighter than AP-CSI reporting timeline, this scheme is not needed - FFS: How to indicate the measurement beam for L1-RSSI - FFS: What is included in the L1-RSSI report, such as the value of RSSI measurement, comparison outcome with Energy Detection threshold, etc - Scheme 2: CCA or eCCA based receiver assistance with existing phy channel/signals - Scheme 2-1: gNB schedules/triggers UL PUCCH/SRS transmission with the DL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUCCH (or SRS in the case of 1-bit Rx-assistance) to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, the downlink data transmission happens. - FFS if the downlink data transmission can be granted with the same DL DCI that schedules/triggers the first UL PUCCH/SRS transmission, in which case, the CCA or eCCA is performed for at least the first UL PUCCH/SRS transmission - Scheme 2-2: gNB schedules/triggers UL transmission PUSCH with the UL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUSCH to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, the downlink data transmission happens. - Scheme 3: CCA or eCCA based receiver assistance with new RTS/CTS type transmission - New RTS/CTS-like signaling introduced. - gNB sends RTS-like signaling to UE. UE performs CCA or eCCA and if LBT passes, transmits CTS-like signaling to explicitly indicate the LBT outcome. gNB detects the CTS-like signaling to identify if the UE passed CCA or eCCA. After detecting the CTS-like signal, the data transmission happens - Scheme 4: Legacy L3-RSSI with potential enhancements - FFS potential enhancements, e.g., supporting gNB indicating the beam used for UE RSSI measurement, supporting gNB indicating new reference SCS and measurement bandwidths - Note: The schemes listed above are not mutually exclusive and should be discussed separately.

At this point, among scheme 1, enhancement to RSSI measurement and reporting and scheme 2,  eCCA and CCA based class A Rx-assistance,  it can be argued that,  scheme 1 which closely follows NR framework, can approximate approach much of the performance offered by  scheme 2. Further, a version of scheme 2, which relies on eCCA and Configured Grant UL Transmission, can be implemented already under current NR-U framework. In that light we propose to prioritize scheme 1 L1-RSSI enhancements.

Proposal 7: Among  Rx-Assistance schemes, prioritize and adopt L1-RSSI enhancements to AP-CSI framework.

Further design of L1-RSSI enhancements is discussed below.

AP-CSI Enhancements are concurrently being considered in the ongoing Rel. 17 WI on Enhanced IIOT and URLLC. The most important enhancement under consideration is reporting of AP-CSI on PUCCH that is triggered by DL control signaling. Yet the content of CSI enhancements being considered in the ongoing discussions, namely, statistical CQI, Interference Standard Deviation, worst case CQI, increasing granularity of subband CQI and CQI-only update, are orthogonal to the form of Rx-Assistance being considered in the 52-71GHz WI. The question of processing time reduction for CQI is being addressed only in the context of CQI reporting.

In the context of unlicensed operation in the 52-71GHz, the enhancement considered for Rx-Assistance does not overlap with the enhancements in the IIOT/URLLC WI. The key report of interest is a stand-in for energy measurement as done in Cat2/Type 2 LBT, that capitalizes on the NR CSI framework.

Proposal 8: L1-RSSI enhancements to AP-CSI framework should be considered independently of Rel 17 IIOT/URLLC AP-CSI enhancements.

The following list can be identified as the components of the design needed for energy measurement.

- The metric for reporting: An enhancement that introduces L1-RSSI as a component of aperiodic CSI report. L1-RSSI   Some of the options available are (1) encoding of raw RSSI value with known quantization (2) 1 bit decision on the RSSI value when compared with a threshold that is configured

- The trigger: For Rx-Assistance for DL, the appropriate trigger is DCI for UL grant.

- The timeline and resource for measurements:  ZP-CSI-RS based measurements conforming to existing Z1 requirements can be used as a baseline the L1-RSSI measurement

- The timeline for reporting: Under the current specification Z2 constraints as applicable for AP-CSI for this band need to be applied as a baseline while allowing PUCCH to carry the L1-RSSI report. On the other hand the L1-RSSI measurement is expected to be considerably simpler than full AP-CSI, of the same level of complexity as a L1-RSRP report for CSI-RS and hence expected to be sent faster than Z2 . A more appropriate baseline reporting timeline can be based on Z3’ parameter, that is derived from the UE Capability IE BeamReportTiming indicating the  minimum number of OFDM symbols between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing the beam report. BeamReportTiming  takes values of 14, 28 or 56 OFDM symbols for SCS 120KHz  depending on UE capability, and can be taken as a lower bound on the timeline of reporting of L1-RSSI. This lower bound though, is considerably smaller than the CSI computation delay requirements. For example the Z2’ requirement for SCS 120KHz, is 140 OFDM symbols, which is much looser compared to the worst case capability for BeamReportTiming, namely 56 OFDM symbols.

Proposal 9: Consider the use of RSSI compared to a configurable threshold as part of the L1-RSSI report.

Proposal 10: Consider use of UL grant DCI for trigger of Beam Specific L1-RSSI measurement and reporting for enhanced AP-CSI in PUSCH.

Proposal 11: L1-RSSI trigger should also be carried in DL grant. Consider use of PUCCH for sending Beam Specific L1-RSSI measurement and reporting for enhanced AP-CSI.

Proposal 12: Use Rel. 16 AP-CSI timelines as baseline for enhanced AP-CSI reporting with L1-RSSI and study further possible tightening of the timelines.  Use worst case UE capability for BeamReportTiming for 120KHz SCS, namely 56 OFDM symbols, as a guideline for setting the minimum requirement for L1-RSSI reporting timeline.

Beam Specific L1-RSSI:

Further, the RSSI measurement could be beam specific.

Proposal 13: Beam Specific L1-RSSI measurement and reporting should be supported.

The framework for beam based RSSI measurement, and reporting could follow the configuration, measurement and reporting analogous to L1-RSRP for CSI-RS. In particular the rule for determining the TCI state for L1-RSSI measurement can be mimicked. Further the resource for RSSI measurement could come from CSI-RS measurement config, where CSI-RS sent is virtual/ i.e. carries zero power.

Proposal 14: Consider the design of timeline, triggering and beam indication mechanisms of L1-RSSI to be analogous to CSI-RS based L1-RSRP reporting in AP-CSI.

# Multi-Beam LBT aspects

The agreements from RAN1#104e regarding Multi-Beam LBT are listed below.

Agreement: For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both) - Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT Agreement: Within a COT with TDM of beams with beam switching, downselect one or more of the following LBT operations - Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - FFS: Details on the definition of "cover" - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT - Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch Agreement: - SSB transmission with LBT is supported, at least when the conditions for contention exempt short control signaling based SSB transmission is not met - Note the channel access for SSB with LBT may not be different from a normal COT with multiple beams - FFS: If any difference from a multi-beam COT LBT needs to be introduced Agreement: For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed in TDM fashion - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams Agreement: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed one after another in time domain - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

The key debates on Multi-Beam COT are capture below on two separate use-cases. The first is a COT where beams are sent in SDM way and not changed during the COT.

Proposals for discussion from RAN1-106e: Proposal 2.7.1-1 For a COT with MU-MIMO (SDM) transmission, support both Alt 1 and Alt 2 below: - Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT Proposal 2.7.1-3 Within a COT with TDM of beams with beam switching, downselect to one of the following LBT operations - Alt A: Support both Alt-1 and Alt 2 - Alt B: Support both Alt-1 and Alt 3

Here the proposal has near uniform support, i.e., both, wide single beam LBT and independent per beam LBT should be supported.

For SDM transmissions, independent per-beam sensing and channel access decision provides the maximum flexibility to nodes contending for the medium.  On the other hand, a single sensing beam valid to be used with multiple beams with potentially different beam gains is attractive for simplicity and beam selection flexibility so long as the chosen beams can be used with the sensing beam. For SDM transmission, we propose supporting both Alt 1 and Alt 2 in the specification.

Proposal 15:  For SDM transmission, support both (Alt1) single LBT sensing with wide beam covers all beams used in the COT and (Alt 2) independent per beam sensing.

Especially when a node has capability to do separate sensing operations for multiple beams, the following proposal to enable separate per beam sensing should be readily agreeable for both type of Multi-Beam COTs – namely

Proposals for discussion from RAN1-106e: Proposal 2.7.1-2 For a COT with MU-MIMO (SDM) transmission if Alt 2 is supported (independent per beam LBT), and if the node has the capability to perform simultaneous sensing in different beams, simultaneous per-beam LBT for different beams is supported. Proposal 2.7.1-4 Within a COT with TDM of beams with beam switching, if Alt 2 or Alt 3 is supported (independent per beam LBT), and if the node has the capability to perform simultaneous sensing in different beams, simultaneous per-beam LBT for different beams is supported.

We support both these proposals:

Proposal 16: For a COT with MU-MIMO (SDM) transmission if independent per beam LBT is supported, and if the node has the capability to perform simultaneous sensing in different beams, simultaneous per-beam LBT for different beams is supported.

Proposal 17: Within a COT with TDM of beams with beam switching, if independent per beam LBT is supported, and if the node has the capability to perform simultaneous sensing in different beams, simultaneous per-beam LBT for different beams is supported.

We also consider accumulation of channel access rights by clearing LBT for different beams within a COT at different times.  In particular:

Proposal 18: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement is considered),  select,  Alt A-2, namely, the node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam.

The mode of operation for TDM of beams with beam switching, we support the mode Alt A which corresponds to – supporting both, single wide beam LBT and per beam LBT at the start of the COT.  Among per beam LBT options, our preference is towards Alt-A-2.

Proposal 19: Within a COT with TDM of beams with beam switching, downselect to the following LBT operations

Alt A:  Support both Alt-1 and Alt-2, where Alt-1 and Alt -2 are part of earlier agreement as follows:

- Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

  - FFS: Details on the definition of "cover”

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

# Multi-Channel Channel Access aspects

By meeting RAN1#104e, RAN1 reached the following agreements and listed for-further-study the following considerations related to Multi-Channel LBT.

Agreement: (RAN1#104e)

Define Type A and Type B multi-channel channel access as:

- Type A: Perform independent eCCA for each channel

- Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot

Down-selection between

- Alt1: Support Type A multi-channel channel access only

- Alt2: Support both Type A and Type B multi-channel channel access.

Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed

As part of the discussion in RAN1#105e, further distillation was reached for the following proposals.

Proposal for discussion: (RAN1#105e) - Type A multi-channel channel access is supported - If Cat 2 LBT is introduced, type B multi-channel channel access is supported

Type B multi-channel access from Rel 16 NR-U eCCA on primary channel and a Cat 2 LBT in other channels in a last observation slot permits tying the medium access on multiple channels together, allowing increased predictability and helps in scheduling, cross-channel use of control signaling and data transmissions tied to the COT. On the other hand, the channel access on the secondary channel done via Cat 2 LBT may violate ETSI Bran medium access requirements of eCCA in that channel, even if the duration of Cat 2 LBT is chosen to be large. Given this regulatory obstacle, we propose restricting to Type A only channel access.

Proposal 20: Adopt Alt-1 for multi-channel access, i.e., support Type A multi-channel access only.

# Directional LBT aspects

RAN1#106e discussion recommended the following discussion proposal as a starting point of the directional LBT convergence.

Agreement: 3GPP specification consider defining at least the relative relationship between all applicable sensing beam(s) and the transmission beam(s) to define sensing beam for LBT, where at least sensing beam(s) “covers” the transmission beam(s), considering following alternatives. Target down-selection by RAN1 #106bis-e - Alt 1: Specify necessary requirement/test procedure to guarantee sensing beam “covers” the transmission beam - Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam. - Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain - Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams. - Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well - Alt 2. Extending the beam correspondence framework and QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s) - On gNB side sensing beam selection for a DL transmission beam, - Option 1: The selection of eligible sensing beam for a transmission beam is left for gNB implementation - No testing or enforcement introduced in 3GPP spec for this option - Option 2: Beam correspondence at gNB side is assumed. Supporting one or more of the following behaviors - A1. For a gNB transmission beam corresponding to TCI state A for a certain UE, the gNB can use the same beam for sensing - A2. If TCI B is used as QCL source (Type D) for TCI A for a certain UE, then gNB transmission beam corresponding to TCI B can be used as the sensing beam for transmission with TCI A. - A3. If TCI C is NOT used as QCL source (Type D) for TCI A for any UE, then gNB cannot use the transmission beam corresponds to TCI C as the sensing beam for transmission with TCI A. - FFS: How and if to support sensing with a beam without corresponding RS sent? For example, how to use quasi-Omni beam for sensing if there is no SSB transmitted with quasi-omni beam - On UE side sensing beam selection for a UL transmission beam - Beam correspondence is assumed at UE - FFS: What if beam correspondence is not supported at UE. - Supporting one or more of the following behaviors - If the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing - Assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing - FFS: How and if to support a wider sensing beam (such as pseudo-omni beam, which is supported in WiFi) to be used for a narrower transmission beam under QCL/TCI framework - Option 0: Not supported - Option 1: UE implementation. - No testing or enforcement introduced in 3GPP spec for this option - Option 2: gNB indication. - FFS details. - FFS: How and if to support multiple sensing beams to be used for a transmission beam under QCL/TCI framework - Note: Supporting both alternatives or a combination of the two alternatives is not precluded

### Alt 1 for Directional LBT

Among the FFS items list in Alt 1, we consider Alt 1-D be a sensible option which combines simplicity without assuming structure or imposing constraints on the choice of transmission beams. It aims to achieve sufficiently high sensing gain along the directions of high interference footprint of the transmission beam.

Alt 1-D provides a testable path to defining directional sensing. It is proposed in Proposal 26. The promising alternative is not to compare the sensing beam gain with transmission beam gain, but instead compare the sensing beam gain with sensing beam gain in the direction of the peak transmit EIRP or with peak sensing beam gain.

Proposal 21: If Alt 1 is chosen, Support Alt -1-D namely the sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain.

If testing simplicity is a consideration Alt-1-D can be modified to consider the following criterion for eligibility of sensing beam. This formulation avoids the need to specify the direction of peak sensing beam gain at the device via conducting tests only along directions of high transmit beam EIRP.

Proposal 22: Alt-1-D Alternative formulation:  The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions.

Figure 1  Sketch depicting candidate relationship between transmission beam EIRP (left Y axis) and Sensing beam gain (Right Y axis). The figure shows Transmission Beam’s interference footprint (left), an eligible sensing beam (middle) and an ineligible sensing beam (right)

The criterion is tested by (1) sampling of the points with high EIRP (within A dB of the highest value), (2) determining the energy level of the jammer in the peak EIRP direction of the transmitter can silence the transmitter (LBT/adaptivity conformance test), (3) placing the jammer at sampled directions and increasing the power of the jammer until the transmitter is silenced . If the power of the jammer is kept within X dB, the test is successful in determining that the sensing beam gain is sufficiently high. This is depicted in Figure 2 below.

Figure 2  Testing Directional Sensing: Left: Jammer along peak EIRP direction  θ* causes LBT failure at the DUT at power Z0, Right: Jammer along strong direction  θ needs power Zi to silence the same transmitter beam

### Considerations for Alt 2 for Directional LBT

The Alternative 2 goes beyond defining the relationship between sensing and transmission beam and define signaling and test requirements based on enhancements to beam correspondence and TCI signaling framework. The TCI/QCL Framework as it stands is quite limiting.  On the positive side, TCI/QCL framework connects two transmissions –as quasi collocated and their relationship can be signaled. On the other hand, the framework connects only two transmitter side beams. Extra step is needed to connect a transmitter beam with a sensing beam.  In general, the sensing beam to transmitter beam is a many-to-many relationship, i.e., a single sensing beam can be used for multiple transmission beams in a COT and potentially the specification should permit the use of any ‘eligible’ sensing beams to be used for a given transmission beam.  The current QCT/TCI framework assumes a tree structured relationship which does not permit many-to-many QCL relationships.

The concept of connecting a transmission beam with reception/sensing beam has been previously visited in the context of beam correspondence where a UE with beam correspondence capability can identify a good beam for uplink transmission based on DL reception beam, without undergoing uplink beam sweep.  This indeed specifies a sufficiently good transmission beam as matching the reception beam.  In case of directional sensing the problem is dual. The equivalent ‘reception beam’ would be the sensing beam.  In this case the transmission beam of interest is already identified and the eligibility of sensing beam for that transmission beam is in question. Beam correspondence in the original sense is testable via the quality of transmission beam. On the other hand, the eligibility of sensing beam is not tested as there is no signal to measure but an LBT criterion needs to be satisfied instead.

In the listed options for Alt-2, the following behaviors on gNb and UE side are readily understandable and specify  clearly identified relationships in sensing and transmission under the TCI/QCL and beam correspondence framework on the gNB and UE side.

Proposal 23: If Alt -2 is chosen, adopt gNB behaviors A1 and A2 for sensing at gNB.

Proposal 24: If Alt -2 is chosen, and UE has beam correspondence and if the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing.

Proposal 25: If Alt -2 is chosen, and UE has beam correspondence, assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing.

The drawbacks under Alt 2 can be listed as follows.

- It is unclear how a UE without beam- correspondence can use the framework.

- On gNB side the effort involves either specifying beam correspondence or using an untestable language in the specifications.

- It is unclear how to support a sensing beam that does not have an RS transmission associated with it. This is especially relevant as a common design choice for sensing beams is Quasi-Omni/Pseudo-Omni beam, which is often, not necessarily the best beam for reception in a TCI state or for transmission of SRS.

Having recognized that many desirable behaviors can be identified in the framework of Alt-2, it is conceivable to move forward with a combined solution that retains the benefits of both alternatives. RAN1 can undertake the support and specifications of cleanly identifiable behaviors in Alt-2 and involve Alt-1 approach to define ‘cover’ via RAN4 only in the other situations.

Combined way forward for relationship between sensing and transmission beams:

Proposal 26: Adopt Alt 2 behaviors in the known-beam situations i.e. cases where sensing beam and transmission beams can be connected by TCI/QCL/beam correspondence relationship for the UE, and the behaviors A1, A2 hold for gNB in Alt -2 . In other situations specify requirement/test procedure to guarantee sensing beam “covers” the transmission beam, via RAN4.

# No-LBT aspects

Cell Specific and UE Specific indication of LBT mode/ vs No LBT mode was adopted as an agreement in RAN11-105e. The main question open is whether it is useful to have that indication beam specific. Although potential use cases of CPE like devices in consideration, with multi-panel antennas, higher power, and high beamforming gain, making certain interference directional, we believe the desired functionality can be achieved by UE specific indication of LBT mode. Per beam indication of the mode would add limited value while requiring higher complexity in signaling, design and testing.

Proposal 27: Do not support per beam indication of the decision on applying LBT mode or no-LBT mode.

# Contention Exemption and Short Control Signaling aspects

During RAN1#105, the following proposal for discussion was put forth for contention exempt uplink transmissions, which majority companies support:

Proposal for discussion: (RAN1#105e) - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signaling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1 /msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1 /msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

Alt 1, although simpler to specify and verify/enforce, is more restrictive than that required by regulation.

Alt1 restrictions on cell wide configurations of control signaling, may result in some of the configurations that are allowed for NR licensed operation to be ineligible for operation in the unlicensed band.

On the other hand, use of Alt 2 is compliant with regulation and is less restrictive.  it requires control signaling procedure to be designed with the consideration that in case short control signaling budget is exceeded, LBT is used at the UE.

Proposal 28:  Support Alt 2. Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and msgA for the 2-step RACH for all supported SCS. The 10% over any 100ms interval restriction is applicable from the perspective of the UE in accordance with per device requirement set by regulation.

Proposal 29:  SRS should be included towards contention exempt transmissions.

Proposal 30:  PUCCH should be included towards contention exempt transmissions.

Proposal 31:  PUSCH without user plane data, such as CSI or Ack/Knack, and msg3 should be included towards contention exempt transmissions.

For downlink we support contention exempt transmissions for all control signaling under the duty cycle restrictions imposed by regulations, including PDSCH transmission not carrying user-plane data.

Proposal 32:  Under the restrictions of duty cycle for short control signaling, allow SS/PBCH, PDCCH, CSI-RS and PRS for contention exempt transmission.

# CWS and CAPC aspects

We reiterate our position that sophisticated procedures for CWS adjustment or channel access priority class (CAPC) may not be necessary for higher band channel access. Based on current requirements on deferral period and contention window, it is possible to serve any traffic with worst case delay of the order of 25 us. That is   sufficiently small for envisioned high priority traffic to be served with low latency overhead.  Roughly symmetric behaviour in aggressor and victim observed in Sub6 unlicensed bands motivates the CWS adjustment type procedures introduced in Rel. 16 NR-U.

On the other hand, unlike Sub-7GHz, there is large asymmetry between interferer link and the interfered link in 60GHz spectrum. Contention Window adjustment procedure – namely extension of the window on collision/HARQ NAK based schemes will affect primarily the victim’s ability to access the medium instead of increasing the fairness of channel access. Therefore, we maintain the following proposal:

Proposal 33:  CWS adjustment need not be introduced for 60GHz band.

Proposal 34:  CAPC need not be introduced for 60GHz band.

# Conclusions

We have the following observations and proposals:

Proposal 1:  Confirm the working assumption on Pout definition in RAN1 #104bis-e with the following updates:

- For Pout in EDT determination, define Pout to be at least the maximum of mean EIRP of each transmission burst during the COT at the node initiating the COT.

Proposal 2:  Support additional adjustment to Energy Detection computation/threshold whenever the sensing beam has a lower beamforming gain than the transmission beam.

Proposal 3:  For LBT for multi-carrier transmission in intra-band CA, the support for single LBT over all CC is not required.

Proposal 4: Minimum requirement for sensing for both 5us and 8us slots should be 1us irrespective of bandwidth.

Proposal 5: For Alt-3 for COT sharing, gNB determines the value of Y and is transparent to UE.

Proposal 6: Introduce Cat 2 LBT for the use case of Multi-Beam LBT.

Proposal 7: Among  Rx-Assistance schemes, prioritize and adopt L1-RSSI enhancements to AP-CSI framework.

Proposal 8: L1-RSSI enhancements to AP-CSI framework should be considered independently of Rel 17 IIOT/URLLC AP-CSI enhancements.

Proposal 9: Consider the use of RSSI compared to a configurable threshold as part of the L1-RSSI report.

Proposal 10: Consider use of UL grant DCI for trigger of Beam Specific L1-RSSI measurement and reporting for enhanced AP-CSI in PUSCH.

Proposal 11: L1-RSSI trigger should also be carried in DL grant. Consider use of PUCCH for sending Beam Specific L1-RSSI measurement and reporting for enhanced AP-CSI.

Proposal 12: Use Rel. 16 AP-CSI timelines as baseline for enhanced AP-CSI reporting with L1-RSSI and study further possible tightening of the timelines.  Use worst case UE capability for BeamReportTiming for 120KHz SCS, namely 56 OFDM symbols, as a guideline for setting the minimum requirement for L1-RSSI reporting timeline.

Proposal 13: Beam Specific L1-RSSI measurement and reporting should be supported.

Proposal 14: Consider the design of timeline, triggering and beam indication mechanisms of L1-RSSI to be analogous to CSI-RS based L1-RSRP reporting in AP-CSI.

Proposal 15:  For SDM transmission, support both (Alt1) single LBT sensing with wide beam covers all beams used in the COT and (Alt 2) independent per beam sensing.

Proposal 16: For a COT with MU-MIMO (SDM) transmission if independent per beam LBT is supported, and if the node has the capability to perform simultaneous sensing in different beams, simultaneous per-beam LBT for different beams is supported.

Proposal 17: Within a COT with TDM of beams with beam switching, if independent per beam LBT is supported, and if the node has the capability to perform simultaneous sensing in different beams, simultaneous per-beam LBT for different beams is supported.

Proposal 18: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement is considered),  select,  Alt A-2, namely, the node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam.

Proposal 19: Within a COT with TDM of beams with beam switching, downselect to the following LBT operations

Alt A:  Support both Alt-1 and Alt-2, where Alt-1 and Alt -2 are part of earlier agreement as follows:

- Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

  - FFS: Details on the definition of "cover”

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

Proposal 20: Adopt Alt-1 for multi-channel access, i.e., support Type A multi-channel access only.

Proposal 21: If Alt 1 is chosen, Support Alt -1-D namely the sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain.

Proposal 22: Alt-1-D Alternative formulation:  The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions.

Proposal 23: If Alt -2 is chosen, adopt gNB behaviors A1 and A2 for sensing at gNB.

Proposal 24: If Alt -2 is chosen, and UE has beam correspondence and if the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing.

Proposal 25: If Alt -2 is chosen, and UE has beam correspondence, assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing.

Proposal 26: Adopt Alt 2 behaviors in the known-beam situations i.e. cases where sensing beam and transmission beams can be connected by TCI/QCL/beam correspondence relationship for the UE, and the behaviors A1, A2 hold for gNB in Alt -2 . In other situations specify requirement/test procedure to guarantee sensing beam “covers” the transmission beam, via RAN4.

Proposal 27: Do not support per beam indication of the decision on applying LBT mode or no-LBT mode.

Proposal 28:  Support Alt 2. Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and msgA for the 2-step RACH for all supported SCS. The 10% over any 100ms interval restriction is applicable from the perspective of the UE in accordance with per device requirement set by regulation.

Proposal 29:  SRS should be included towards contention exempt transmissions.

Proposal 30:  PUCCH should be included towards contention exempt transmissions.

Proposal 31:  PUSCH without user plane data, such as CSI or Ack/Knack, and msg3 should be included towards contention exempt transmissions.

Proposal 32:  Under the restrictions of duty cycle for short control signaling, allow SS/PBCH, PDCCH, CSI-RS and PRS for contention exempt transmission.

Proposal 33:  CWS adjustment need not be introduced for 60GHz band.

Proposal 34:  CAPC need not be introduced for 60GHz band.

# References

- Chairperson’s notes: RAN1-106-e, Agenda 8.2.6

- Chairperson’s notes: RAN1-105-e, Agenda 8.2.6

- R1-2108554, Feature lead summary of channel access mechanism for 52.6GHz-71GHz band, ver03, Qualcomm Incorporated,

- ETSI EN 302 567, Harmonised European standard for Multi-Gigabit radio equipment in 60GHz, v2.2.0 (2020-12)

- R1-2107335, Channel Access Mechanisms for NR in 52.6 to 71GHz band, RAN1-106-e contribution, Qualcomm Incorporated

- R1-2104662, Channel Access Mechanisms for NR in 52.6 to 71GHz band, RAN1-105-e contribution, Qualcomm Incorporated

- R1-2103162, Channel Access Mechanisms for NR in 52.6 to 71GHz band, RAN1-104bis-e contribution, Qualcomm Incorporated