| 3GPP TSG-RAN WG1 Meeting #106bis-e e-Meeting, October 11th – 19th, 2021 | R1-2110234 |
| --- | --- |

Agenda item:	8.17.16

Source: 	Qualcomm Incorporated

Title: 	UE features for DL 1024QAM for NR FR1

Document for:	Discussion/Decision

# Introduction

In [1], the preliminary UE features for Rel-17 NR are provided by WI rapporteurs. It is understood that the provided list is only for information for preparation of RAN1#106bis-e and all formal discussion/decision on Rel-17 UE features will start from RAN1#106bis-e.  In this contribution, we discuss our views on UE feature for DL 1024 QAM for NR FR1.

# Discussion

The support of the 1024-QAM PDSCH in NR is limited only to FR1.  The field of “Need of FR1/FR2 differentiation” should explicitly states ‘FR1 only’.  Also, there is no need to differentiate between TDD and FDD band in FR1.  In addition, the gNB should be aware of the UE support of that feature.  Finally, the support of the 256-QAM should be prerequisite for the support of the 1024-QAM as the MCS/CQI tables are derived based on the 256-QAM MCS/CQI tables.

Proposal 1: For FG 36-1 on the support of 1024QAM for PDSCH, the following updates/modifications are recommended as shown in Table-1 in the appendix.

- FR1 only.

- Optional with capability signalling

- No differentiation is needed between TDD and FDD

- gNB should be aware of UE support of this feature.

- Add FG 1-4, 256QAM for PDSCH, as prerequisite FG.

# Conclusion

In this contribution, we discussed our views on UE features for the support of the DL 1024QAM in NR FR1. Our views are summarized in Table-1 in the appendix.

# Reference

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO,INC.).

# Appendix

Table 1: UE feature for 1024-QAM

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_DL1024QAM_FR1 | 36-1 | 1024QAM for PDSCH for FR1 | Support 1024QAM for PDSCH for FR1 including 1024QAM modulation scheme as defined in TS 38.211, MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214. | FG 1-4 | Yes |  |  | Per Band | No | FR1 Only |  | Note from WI objective: DL PDSCH 1024QAM for FR1 should be defined as a per-band UE capability | Optional |