3GPP TSG RAN WG1 #106bis	R1- 2110267

e-Meeting, 11 – 19 October, 2021

Agenda item:		8.17.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR NTN

Document for:		Discussion and Decision

# Introduction

Initial drafts on NR and LTE UE features have been made available already on RAN1 reflector [1, 2]. In this contribution we present our views on the UE features for NR NTN.

# Discussion

Detailed comments on UE features are listed below. For reference, the feature list is available in the Annex.

- 26-1, 26-2, 26-3, 26-6:

  - Instead of ”Mandatory”, it should say ”Optional with capability. This is a basic feature for UEs supporting NR over NTN.” In any case, considering these features are expected to be signalled together and with same signalling granularity, it is better to combine them into a single FG.

- 26-3:

  - Replace text on ”Consequence if…” column with ”Release 17 UE cannot access NTN”. No need for lengthy explanations why it cannot access NTN.

- 26-4:

  - Component 1: It is unclear what is signalled here. We propose to change the text to “Support UE reporting of information about the UE specific TA pre-compensation”

  - Component 2: remove component 2 or change the text to make it clear all report frequencies are supported.

  - Component 3 (which is numbered ”4”): remove component 3 or change the text to make it clear all report granularities are supported

- 26-6:

  - It is not clear what enhancement the FG refers to. We propose to change the text to: ” Support type-1 and type-2 HARQ codebook enhancements for feedback-disabled HARQ processes”

- 26-7:

  - FG name is not descriptive, and we propose to change it to, e.g. ” Extended slot aggregation”

  - Add candidate values to notes column. Since X=8 it is the same as legacy operation, this value is not needed as a capability.

- 26-8:

  - FG name is misleading, as the FG is not about the signalling, but about the UE being able to apply the information it receives in the signalling. We propose to change it to, e.g. “Utilization of polarization information in NR NTN”

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for NR NTN, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- R1-2108679, ”Preliminary RAN1 UE features list for Rel-17 NR”, Moderators (AT&T, NTT DOCOMO, INC.), Aug. 2021

- R1-2108678, ”Preliminary RAN1 UE features list for Rel-17 LTE”, Moderators (AT&T, NTT DOCOMO, INC.), Aug. 2021

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops |  | No | No | Release 17 UE cannot access NTN |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. | Mandatory |
| 26. NR_NTN_solutions | 26-2 | Uplink Frequency/Doppler pre-compensation | - In RRC_IDLE and RRC_INACTIVE states calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. - in RRC_CONNECTED states, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. |  | No | No | Release 17 UE cannot access NTN |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 | An NR NTN UE shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. | Mandatory |
| 26. NR_NTN_solutions | 26-3 | Start of RAR window | - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow - delaying the start of msgB-ResponseWindow | 26-2 | No | No | Due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble. |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Mandatory |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | - The exact content of UE reporting of information about the UE specific TA pre-compensation - frequency of the reports - granularity of the reported content | 26-2 | Yes | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | [Optional] |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | - The maximal supported HARQ process number is 32 for both UL and DL - Support on the maximal HARQ process number is up to UE capability |  | Yes | No | Increasing the number of HARQ processes avoids HARQ stalling, |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional |
| 26. NR_NTN_solutions | 26-6 | HARQ CODEBOOK Enhancement | - Enhancement on Type-2 HARQ codebook in NTN - Enhancement on Type-1 HARQ codebook in NTN |  | Yes | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Mandatory |
| 26. NR_NTN_solutions | 26-7 | NTN Performance enhancement | - The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] FFS: X = 8, 16 or 32 |  | Yes | No | Particularly beneficial in case HARQ feedback is disabled |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional |
| 26. NR_NTN_solutions | 26-8 | Support of polarization signalling in NR NTN | - UE polarization capability (RHCP, LHCP, Linear) - Polarization information for UL may be indicated in SIB by the network - UE assumes a same polarization for UL and DL, when the UL polarization information is absent. |  | No | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional |