3GPP TSG RAN WG1 #106bis-e			                                                                    R1-2110679

e-Meeting, October 11th – 19th, 2021

Agenda Item: 8.7

Source: MediaTek Inc.

Title: Summary of email discussion on Rel-17 RRC parameters for UE power saving

Document for: Discussion and Decision

# Initial List of RRC Parameters

The following draft RRC parameter table for Rel-17 UE power saving enhancements is based on the email discussion summary post RAN1#106-e [1] and includes coordinator’s suggested revisions for clarification based on [2]. The revisions are marked in red color for ease of reference.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parent IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.7.1.1 Paging Enh. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Paging enhancement | 38.213 |  |  |  | subgroupsNumPerPO | new |  | Number of subgroups per Paging Occasion (PO) for UE to read subgroups indication from physical-layer singling | TBD with maximum of 8 |  | per cell | cell-specifc |  | Agreement: For UE subgroups indication in physical layer, maximum of 8 subgroups per PO is supported. |
| 8.7.1.2 TRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TRS resources for idle/inactive UEs | 38.214 |  |  |  | [TRS-ResourceConfig] | new |  | RS configuration of TRS occasion(s) for idle/inactive UE(s). FFS in terms of a list of N>=1 NZP TRS resource set(s) or TRS resources | TBD |  | per cell | cell-specifc |  | The configuraiton structure (i.e. whether to support common parameter(s) per resoruce set(s)) is still under discussion |
|  | TRS resources for idle/inactive UEs | 38.214 |  |  |  | powerControlOffsetSS | new |  | Power offset (dB) of NZP CSI-RS RE to SSS RE | {-3, 0, 3, 6} |  | TBD(e.g. per configuration, per TRS resource set, or per TRS resource) | cell-specifc |  |  |
|  | TRS resources for idle/inactive UEs | 38.214 |  |  |  | scramblingID | new |  | scrambling ID of TRS with length of 10 bits. | 0 to 1023 |  | TBD(e.g. per configuration, per TRS resource set, or per TRS resource) | cell-specifc |  |  |
|  | TRS resources for idle/inactive UEs | 38.214 |  |  |  | firstOFDMSymbolInTimeDomain | new |  | The index of the first OFDM symbol in the PRB used for TRS in a slot | 0 to 9 |  | TBD(e.g. per configuration, per TRS resource set, or per TRS resource) | cell-specifc |  | firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4 |
|  | TRS resources for idle/inactive UEs | 38.214 |  |  |  | startingRB | new |  | PRB index where corresponding TRS resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid. | 0 to 274 |  | TBD(e.g. per configuration, per TRS resource set, or per TRS resource) | cell-specifc |  | Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP. |
|  | TRS resources for idle/inactive UEs | 38.214 |  |  |  | nrofRBs | new |  | Number of PRBs across which corresponding TRS resource spans | 24 to 276 |  | TBD(e.g. per configuration, per TRS resource set, or per TRS resource) | cell-specifc |  | Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP. |
|  | TRS resources for idle/inactive UEs | 38.214 |  |  |  | ssb-Index | new | QCL source | Index of reference SSB with which quasi-collocation information is provided as specified in TS 38.214 subclause 5.1.5. | SSB-Index: 0 to 63 |  | TBD(e.g. per configuration, per TRS resource set, or per TRS resource) | cell-specifc |  | a QCL type can be determined implicitly without explict configuration, as - ‘typeC’ with an SS/PBCH block and, when applicable, ‘typeD’ with the same SS/PBCH block |
|  | TRS resources for idle/inactive UEs | 38.214 |  |  |  | periodicityAndOffset | new |  | periodicity and slot offset (slot) for periodicTRS, with values from IE 'CSI-ResourcePeriodicityAndOffset' | {slots10, slots20, slots40, slots80, [slots160], [slots320], [slots640]} |  | TBD(e.g. per configuration, per TRS resource set, or per TRS resource) | cell-specifc |  |  |
|  | TRS resources for idle/inactive UEs | 38.214 |  |  |  | frequencyDomainAllocation | new |  | for row1 to indicate the offset of the first RE to RE#0 in a RB | {0, 1, 2, 3} |  | TBD(e.g. per configuration, per TRS resource set, or per TRS resource) | cell-specifc |  |  |
| 8.7.2 DCI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PDCCH monitoring adaptation | 38.213 |  |  |  | [searchSpaceGroupIdList-r17] | new |  | Add in SearchSpace. If configured, it provides a list of search space group IDs to which the search space set is associated. maxSearchSpaceGroup-r17 is [3]. | Any combination of {0,…,maxSearchSpaceGroups-r17-1} |  | Per BWP | UE-specific |  | This parameter is needed only if the following working assumption is confirmed. - Working Assumption at most 3 SSSGs is supported to be configured. |
|  | PDCCH monitoring adaptation | 38.213 |  |  |  | [searchSpaceSwitchingTimer-r17] | new |  | Timer (slot) to control the UE behavior to switch from search space group X back to search space group 0 [as specified in clause 10 of TS 38.213] | TBD |  | TBD | UE-specific |  | FFS details of timer(s) for switching between SSSG(s) • UE fallbacks to default SSSG (i.e., SSSG#0) after timer expiration. • R16 timer for SSSG switching and the corresponding behavior is as baseline FFS whether the timer(s) is configured per SSSG, per BWP or other approaches. |
|  | PDCCH monitoring adaptation | 38.213 |  |  |  | PDCCHSkippingDuration-r17 | new |  | [Duration X that UE will skip monitoring PDCCH for X slot(s) after receiving DCI indication for PDCCH skipping] | TBD |  | TBD | UE-specific |  | Agreed Beh 1A saying PDCCH skipping means stopping PDCCH monitoring for a duration X. • FFS the possible values for X • FFS: Whether and how to support more than one skipping duration(s) • FFS whether the skip duration(s) is configured per SSSG, or per BWP or other approaches. |

Based on the above table, companies please review and provided needed revision(s) in Table 1 to clarify and complete the RRC list for the RAN1 related power saving enhancement features:

Table 1: Companies’ views and suggested revisions on the above draft RRC parameter table

| CATT | Since several issues are still in on-going discussion, it is not clear how the RRC parameters would be discussed at this stage. The additional RRC parameters at this stage are as follows, 8.7.1.1 Paging enhancement - RRC parameter of the PEI_offset relative to PO to derive the location of PEI monitoring occasion - RRC parameter of the number of POs associated with one PEI - RRC parameter of PEI index within a PEI frame 8.7.1.2 TRS - RRC parameter of configuration of TRS Availability indication in paging DCI - RRC parameter of Bit map of TRS resource set 8.7.1.3 PDCCH adaptation - RRC parameter of configuration of PDCCH adaptation bits in DCI formats 1_1, 1_2, 0_1 or 0_2 - RRC parameter of mapping of PDCCH skipping and/or SSSG swtiching to the configured bits in DCI format The detailed parameters could be discussed after further agreements in RAN1. |
| --- | --- |
| Moderator | For the discussion of RRC parameters, it still depends on what agreements we can achieved in GTW2 online on 10/19. However, the due time of this email discussion is also 10/19. Therefore, I would like to suggest companies to provide inputs to the following email thread that will last after RAN1#106-bis-e: [106bis-e-R17-RRC] Email discussion on Rel-17 RRC parameters for LS to RAN2 – Sorour (Ericsson) - Email discussion to start on October 18 - LS to RAN2 to be finalized and endorsed on October 22 |

# Summary

The email discussion doesn’t change the initial RRC parameters collected in [1]. Since companies shift to discuss and update RRC parameters for R17 UE power saving enhancements for NR in the email thread [106bis-e-R17-RRC] Email discussion on Rel-17 RRC parameters for LS to RAN2, session chair finally close the email discussion thread on 10/22 UTC, as quoted below.

From: xuxiaodong@chinamobile.com [mailto:xuxiaodong@chinamobile.com] Sent: Friday, October 22, 2021 4:32 PM To: Weide Wu (吳威德); 3gpp_tsg_ran_wg1 Subject: Re: [106bis-e-R17-RRC-PowSav] Email discussion on Rel-17 RRC parameters for UE power saving Dear all, Since all companies have provided their inputs to Sorour's thread, this email thread is closed. Best Regards, Xiaodong Xu China Mobile(CCSA) Mobile:+86 13601035215 China Mobile Research Institute, 32 Xuanwumen West Street, Xicheng District, Beijing, 100053, China

# Reference

- R1-2108683, “[Post-106-e-Rel17-RRC-07] Summary of email discussion on RRC parameters for UE Power Saving Enhancements”, Rapporteur (MediaTek), RAN1#106-e

- R1-2110415, “Recommendations for RAN1 RRC Parameter Preparation”, Coordinator (Ericsson), RAN#106bis-e