3GPP TSG RAN WG1 Meeting #106bis-e		R1-2110686

e-Meeting, 11th – 19th October 2021

Agenda Item:	8.18

Source:	Rapporteur (Ericsson)

Title:	List of RAN1 agreements for Rel-17 WI on NR DL 1024QAM

Document for:	Information

# Introduction

This contribution lists RAN1 agreements made so far for the Rel-17 WI on the following WI (WID in [1]). It also includes related information (LS from other WGs, UE feature), and relevant references.

Title of WI: ‘Revised WID on Introduction of DL 1024QAM for NR FR1’

WI code: NR_DL1024QAM_FR1

- Agreements in RAN1

RAN1#103e:

Agreements:

- Introduce new RRC signaling to indicate use of 1024-QAM CQI table.

- For supporting 1024-QAM in NR downlink, adopt the LTE 1024-QAM constellation.

- 1024-QAM MCS table can be used only with DCI format with CRC scrambled by C-RNTI or CS-RNTI.

Agreements:

- Introduce new RRC signaling to indicate use of 1024-QAM MCS table for at least DCI format 1_1

  - FFS : support of 1024-QAM MCS table for DCI format 1_2

    - Note: If 1024-QAM MCS table for DCI format 1_2 is supported, separate RRC signaling is used for each of the two DCI formats 1_1 and 1_2, respectively

  - FFS : whether the RRC signaling is only introduced in PDSCH-Config or it can also be separately configured in SPS-Config

Agreements:

- RRC signaling (mcs-Table-r17) to indicate use of 1024-QAM MCS table for DCI format 1_1 is present only in PDSCH-config

- When UE is configured with mcs-Table-r17 set to ‘qam1024’ in PDSCH-Config,

  - UE uses 1024-QAM MCS table for PDSCH scheduled with a DCI format 1_1 with CRC scrambled by C-RNTI,

  - UE uses 1024-QAM MCS table for PDSCH scheduled with the DCI format 1_1 with CRC scrambled by CS-RNTI if the UE is not configured with mcs-Table in SPS-Config

- Note: If 1024-QAM MCS table for DCI format 1_2 is supported, similar approach is used for 1024-QAM MCS table usage with DCI format 1_2

Agreements:

- Adopt following TP for in 38.212, subclause 5.4.2.1 for TBS_LBRM determination.

5.4.2.1	 Bit selection

The bit sequence after encoding  from Clause 5.3.2 is written into a circular buffer of length  for the -th coded block, where  is defined in Clause 5.3.2.

For the -th code block, let  if  and  otherwise, where, ,  is determined according to Clause 6.1.4.2 in [6, TS 38.214] for UL-SCH and Clause 5.1.3.2 in [6, TS 38.214] for DL-SCH/PCH, assuming the following:

-	maximum number of layers for one TB for UL-SCH is given by X, where

-	if the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter

-	elseif the higher layer parameter maxRank of pusch-Config of the serving cell is configured, X is given by the maximum value of maxRank across all BWPs of the serving cell

-	otherwise, X is given by the maximum number of layers for PUSCH supported by the UE for the serving cell

-	maximum number of layers for one TB for DL-SCH/PCH is given by the minimum of X and 4, where

-	if the higher layer parameter maxMIMO-Layers of PDSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter

-	otherwise, X is given by the maximum number of layers for PDSCH supported by the UE for the serving cell

-	if the higher layer parameter mcs-Table-r17 given by a pdsch-Config for at least one DL BWP of the serving cell is set to 'qam1024', maximum modulation order  is assumed for DL-SCH, elseif the higher layer parameter mcs-Table given by a pdsch-Config for at least one DL BWP of the serving cell is set to 'qam256', maximum modulation order  is assumed for DL-SCH; otherwise else a maximum modulation order  is assumed for DL-SCH;

Unchanged parts are omitted

Agreements:

- Adopt following text proposal for TS 38.201, subclause 4.2.2.

-------------------------------------------------------- Omitted ----------- 4.2.2 Physical channels and modulation -------------------------------------------------------- Omitted --------------- The modulation schemes supported are - in the downlink, QPSK, 16QAM, 64QAM, and 256QAM and 1024 QAM - in the uplink, QPSK, 16QAM, 64QAM and 256QAM for OFDM with a CP and π/2-BPSK, QPSK, 16QAM, 64QAM and 256QAM for DFT-s-OFDM with a CP -------------------------------------------------------- Omitted ------

Agreements:

- Adopt following TP for 38.211 to reflect the agreed 1024-QAM constellation.

-------------------------------------------------------- Omitted -----------------------------------------------------

5.1.7	1024QAM

In case of 1024QAM modulation, 10-tuplets of bits, b10i,b10i+1,b(10i+2),b(10i+3),b(10i+4),b(10i+5),b(10i+6),b(10i+7),b(10i+8),b(10i+9), are mapped to complex-valued modulation symbols d(i) according to

di=1682 1-2b10i16-1-2b10i+28-1-2b10i+44-1-2b10i+62-(1-2b10i+8)+j1-2b10i+116-1-2b10i+38-1-2b10i+54-1-2b10i+72-(1-2b10i+9)

Agreements:

- Adopt following TP to 38.211, subclause 7.3.1.2, to reflect 1024-QAM support for PDSCH

Section 7.3.1.2

Table 7.3.1.2-1: Supported modulation schemes.

| Modulation scheme | Modulation order |
| --- | --- |
| QPSK | 2 |
| 16QAM | 4 |
| 64QAM | 6 |
| 256QAM | 8 |
| 1024QAM | 10 |

Agreements:

- Adopt following TP for 38.214, subclause 5.2.2.1, reflecting the 1024-QAM CQI table usage based on corresponding RRC parameter as follows.

  - Note : RAN1 to further align with the RAN2 signaling design

5.2.2.1   Channel quality indicator (CQI)

-------------------------------------------------------- Omitted ----------------------------------------------------------------------------------------------------------------------

The CQI indices and their interpretations are given in Table 5.2.2.1-2 or Table 5.2.2.1-4 for reporting CQI based on QPSK, 16QAM and 64QAM. The CQI indices and their interpretations are given in Table 5.2.2.1-3 for reporting CQI based on QPSK, 16QAM, 64QAM and 256QAM. The CQI indices and their interpretations are given in Table 5.2.2.1-5 for reporting CQI based on QPSK, 16QAM, 64QAM, 256QAM and 1024 QAM.

Based on an unrestricted observation interval in time unless specified otherwise in this Clause, and an unrestricted observation interval in frequency, the UE shall derive for each CQI value reported in uplink slot n the highest CQI index which satisfies the following condition:

-    A single PDSCH transport block with a combination of modulation scheme, target code rate and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CSI reference resource, could be received with a transport block error probability not exceeding:

-    0.1, if the higher layer parameter cqi-Table in CSI-ReportConfig configures ‘table1’ (corresponding to Table 5.2.2.1-2), or ‘table2’ (corresponding to Table 5.2.2.1-3), or if the higher layer parameter cqi-Table-r17 in CSI-ReportConfig configures ‘table4’ (corresponding to Table 5.2.2.1-5)

-    0.00001, if the higher layer parameter cqi-Table in CSI-ReportConfig configures ‘table3’ (corresponding to Table 5.2.2.1-4).

-------------------------------------------------------- Omitted -------------------------------------------------------------------------------------------------------------------------

Agreements:

- Companies are encouraged to use below link-level simulation assumptions for assessing at least transition point between 256-QAM and 1024-QAM.

| Parameter | Value |
| --- | --- |
| Carrier frequency, SCS, System BW | 3.5GHz, 30kHz, 100 MHz |
| Channel model | AWGN, CDL-B or CDL-C in TR 38.901 with up to 30ns delay spread |
| UE speed | 3km/h, 0km/h |
| Number of UE antennas | 1T4R, 2T4R or 4T4R |
| Number of gNB antennas | 32T32R or 64T64R or 2T or 8T |
| Tx EVM | 0, 2% |
| Rx EVM | 0, 3% |
| MCS | 256 QAM, 1024 QAM Coding Rate*: 0.70, 0.75, 0.80, 0.85, 0.90, 0.925 Other coding rates are not precluded and, if simulated, to be reported by each company |
| DMRS type | DM-RS type 1 |
| Number of DMRS symbols | 1 |
| Number of scheduled RBs | 273 |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| Rank | Rank1, Rank 2, |
| Channel estimation | Realistic channel estimation |
| Metric | Crossover SNR at transition points between 256-QAM and 1024-QAM |
| Note*: Coding rates are used for 1024QAM, while coding rates for 256QAM are selected from TS38.214 MCS table 2 |  |

RAN1#104-e:

Agreement

Reuse LTE CQI table with 1024-QAM entries

- Note: for CQI index 14, the SE value (from LTE) is slightly updated from 8.3321 to 8.3301

- Has 2 CQI entries for 1024-QAM

| CQI index | modulation | code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | Out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.377 |
| 3 | QPSK | 449 | 0.877 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3301 |
| 15 | 1024QAM | 948 | 9.2578 |

Agreement

For supporting 1024-QAM in NR downlink, adopt a five-bit MCS table with 1024-QAM entries:

- Remove M explicit MCS entries (from MCS indices 0-27) from the NR 256QAM MCS table and add M new entries for 1024QAM

- M=5

- Add one implicit MCS entry corresponding to 1024-QAM

- Add M-1 explicit MCS entries corresponding to 1024-QAM as follows:

| modulation | code rate x 1024 | Efficiency |
| --- | --- | --- |
| 10 | 805.5 | 7.8662 |
| 10 | 853 | 8.3301 |
| 10 | 900.5 | 8.7939 |
| 10 | 948 | 9.2578 |

Agreement

For supporting 1024-QAM in NR downlink, remove following M (=5) MCS entries from the NR 256-QAM MCS table

- 2, 4, 6, 8, 10

Agreement

Introduce separate RRC signaling to indicate use of 1024-QAM MCS table for DCI format 1_2.

Agreement

- The text proposals in R1-2102023 are endorsed for TS38.214.

- The text proposals in R1-2102087 are endorsed for TS38.214.

Agreement

LS to RAN2 and RAN4 to inform them of RAN1 decisions on 1024QAM and related RRC parameters, CQI/MCS tables is endorsed in R1-2102088.

- LS from other WG

RAN1#105e:

R1-2104166	Reply LS on Introduction of DL 1024QAM for NR	RAN2, Ericsson

- UE features for 1024-QAM

RAN1#106bis-e:

Agreement

The agreements listed in Section 6 of R1-2109919 are endorsed.

# References

RP-202886, New WID on Introduction of DL 1024QAM for NR FR1, Nokia, RAN#90e, December 2020

R1-2102023, First Set of Agreed TPs for support of 1024QAM in NR DL, Moderator (Ericsson), RAN1#104e, Jan-Feb 2021

R1-2102087, Second Set of Agreed TPs for support of 1024QAM in NR DL, Moderator (Ericsson), RAN1#104e, Jan-Feb 2021

R1-2102088, LS on Introduction of DL 1024QAM for NR, RAN1, RAN1#104e, Jan-Feb 2021

R1-2104166, Reply LS on Introduction of DL 1024QAM for NR, RAN2, RAN1#105e, May 2021

R1-2109919, Summary of UE features for DL 1024QAM for NR FR1, Moderator (AT&T), RAN1#106bis-e, Oct 2021

R1-2009799, Summary of Email discussion [103-e-NR-1024QAM-01], Moderator (Ericsson), RAN1#103e, Oct-Nov 2020.

R1-2102089, Final Summary for introduction of 1024QAM for NR DL, Moderator (Ericsson), RAN1#104e, Jan-Feb 2021.