3GPP TSG RAN WG1 #107e	                                                                  R1-2110972

e-Meeting, November 11th – 19th, 2021

Source:	vivo

Title:	Remaining issues on Rel-17 Tx switching enhancements

Agenda Item:	5.1

Document for:	Discussion and Decision

- Introduction

In RAN#106e, following agreements have been reached with the detailed options to be down-selected.

Agreement

Down select one of the following options in RAN1 #106bis-e

- Option 1:

  - For a UE configured with UL Tx switching via uplinkTxSwitching, the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources is used to determine the operation mode, i.e. either 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

    - 2Tx-2Tx switching mode: when the maximum number is 2 for both all uplinks configured with uplinkTxSwitching

    - 1Tx-2Tx switching mode: when the maximum number is 1 for any one uplink configured with uplinkTxSwitching

    - the switching gap duration for a triggered uplink switching is equal to the switching time capability value reported for the switching mode

      - Note: If the switching time capability value for 1Tx-2Tx switching mode is not reported by the UE, the value reported for 2Tx-2Tx switching mode is applied.

    - If any of the above SRS resources is configured with usage “noncodebook”, then the max number of 2 antenna ports are counted for the SRS resources during the determination of operation mode.

      - FFS how to determine the number of antenna ports for SRS resources.

- Option 2:

  - For a UE configured with UL Tx switching via uplinkTxSwitching, a new RRC parameter is used to indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

In this contribution, we provide our view on the down-selection of the above issue.

- Discussion

In [1], it was discussed how to determine the 1Tx-2Tx or 2Tx-2Tx switching modes if a UE supports both. Since the switching time capability between the two switching modes could be different therefore ambiguity would exist if there is no common understanding between NW and UE on the switching mode determination. Down-selection are to be made between the following two options, as agreed in RAN#106e.

- Option 1:

  - For a UE configured with UL Tx switching via uplinkTxSwitching, the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources is used to determine the operation mode, i.e. either 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

    - 2Tx-2Tx switching mode: when the maximum number is 2 for both all uplinks configured with uplinkTxSwitching

    - 1Tx-2Tx switching mode: when the maximum number is 1 for any one uplink configured with uplinkTxSwitching

    - the switching gap duration for a triggered uplink switching is equal to the switching time capability value reported for the switching mode

      - Note: If the switching time capability value for 1Tx-2Tx switching mode is not reported by the UE, the value reported for 2Tx-2Tx switching mode is applied.

    - If any of the above SRS resources is configured with usage “noncodebook”, then the max number of 2 antenna ports are counted for the SRS resources during the determination of operation mode.

      - FFS how to determine the number of antenna ports for SRS resources.

- Option 2:

  - For a UE configured with UL Tx switching via uplinkTxSwitching, a new RRC parameter is used to indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

Between the two options, the key controversy is how to count the UE Tx state (i.e. 1Tx or 2Tx) when an SRS resource with usage “non-codebook” is configured for a carrier. It is understood that for non-codebook based UL transmission, it is up to UE implementation to use either 1Tx or 2Tx. We think such UE implementation flexibility should be maintained in the UL Tx switching feature, which is allowed by Option1, since the worst case is assumed by gNB. In Option 2, the UE implementation (1Tx or 2Tx) for non-codebook based UL transmission is controlled by the NW, which is different from the case without UL Tx switching, therefore not desirable.

Although not concluded, but based on the RAN1#106bis-e discussion, it was clarified that the determination of the Tx switching mode is only intended for determine the corresponding switching time so that UE and gNB keep the same understanding, while there is no intention to put any restriction on UE implementation regarding actual transmission (i.e. 1Tx or 2Tx).  Therefore, the updated version of option 2 was proposed as following

Updated Option 2

- For a UE configured with UL Tx switching via uplinkTxSwitching, a new RRC parameter is used to indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

- Note 1: gNB would not configure 1Tx-2Tx mode where the assumed 1Tx CC is also configured with non-codebook based MIMO.

- Note 2: The new RRC configuration would not conflict with other active RRC configurations on Tx states.

- Note 3: This RRC parameter is used to differentiate the table (mapping between UL transmission ports and Tx chain) and switching delay for 1Tx-2Tx vs 2Tx-2Tx for UE, and it doesn’t imply any restriction on application of non-codebook transmission together with UL Tx switching.

The updated option 2 has addressed the concern on UE implementation flexibility for non-codebook transmission therefore is a workable solution. However, with these clarification and configuration restrictions, the Option 1 and updated Option 2 became equivalent from functional wise. Option 2 may have the benefit of cleaner operation and less ambiguous specification, while with the cost of additional RRC configuration. We would be fine with either option 1 or updated option 2 and a down-selection should be made in RAN#107e.

Proposal:  Down-select between option 1 and updated option 2 in RAN1#107e

Option 1

- For a UE configured with UL Tx switching via uplinkTx Switching, the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources is used to determine the operation mode, i.e. either 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

  - 2Tx-2Tx switching mode: when the maximum number is 2 for both all uplinks configured with uplinkTxSwitching

  - 1Tx-2Tx switching mode: when the maximum number is 1 for any one uplink configured with uplinkTxSwitching

  - the switching gap duration for a triggered uplink switching is equal to the switching time capability value reported for the switching mode

    - Note: If the switching time capability value for 1Tx-2Tx switching mode is not reported by the UE, the value reported for 2Tx-2Tx switching mode is applied.

  - If any of the above SRS resources is configured with usage “noncodebook”, then the max number of 2 antenna ports are counted for the SRS resources during the determination of operation mode.

    - FFS how to determine the number of antenna ports for SRS resources.

Updated Option 2

- For a UE configured with UL Tx switching via uplinkTxSwitching, a new RRC parameter is used to indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

- Note 1: gNB would not configure 1Tx-2Tx mode where the assumed 1Tx CC is also configured with non-codebook based MIMO.

- Note 2: The new RRC configuration would not conflict with other active RRC configurations on Tx states.

- Note 3: This RRC parameter is used to differentiate the table (mapping between UL transmission ports and Tx chain) and switching delay for 1Tx-2Tx vs 2Tx-2Tx for UE, and it doesn’t imply any restriction on application of non-codebook transmission together with UL Tx switching.

- Conclusion

In this contribution, we discuss the remaining RAN1 issues to support the Rel-17 enhanced UL Tx switching.

Proposal:  Down-select between option 1 and updated option 2 in RAN1#107e

Option 1

- For a UE configured with UL Tx switching via uplinkTx Switching, the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources is used to determine the operation mode, i.e. either 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

  - 2Tx-2Tx switching mode: when the maximum number is 2 for both all uplinks configured with uplinkTxSwitching

  - 1Tx-2Tx switching mode: when the maximum number is 1 for any one uplink configured with uplinkTxSwitching

  - the switching gap duration for a triggered uplink switching is equal to the switching time capability value reported for the switching mode

    - Note: If the switching time capability value for 1Tx-2Tx switching mode is not reported by the UE, the value reported for 2Tx-2Tx switching mode is applied.

  - If any of the above SRS resources is configured with usage “noncodebook”, then the max number of 2 antenna ports are counted for the SRS resources during the determination of operation mode.

    - FFS how to determine the number of antenna ports for SRS resources.

Updated Option 2

- For a UE configured with UL Tx switching via uplinkTxSwitching, a new RRC parameter is used to indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode.

- Note 1: gNB would not configure 1Tx-2Tx mode where the assumed 1Tx CC is also configured with non-codebook based MIMO.

- Note 2: The new RRC configuration would not conflict with other active RRC configurations on Tx states.

- Note 3: This RRC parameter is used to differentiate the table (mapping between UL transmission ports and Tx chain) and switching delay for 1Tx-2Tx vs 2Tx-2Tx for UE, and it doesn’t imply any restriction on application of non-codebook transmission together with UL Tx switching.

References

- R1-2110566, [106bis-e-NR-R17-TxSwitching-01] Summary of email discussion on Rel-17 uplink Tx switching, Moderator (China Telecom)