3GPP TSG RAN WG1 #107-e	    R1-2112602

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.10.1

Source:	Moderator (AT&T)

Title:	Feature Lead Summary #1 of 8.10.1

Document for:	Discussion/Approval

# Introduction

This contribution provides a summary of the discussion in RAN1#107-e for the following email discussion:

[107-e-NR-eIAB-01] Email discussion on enhancements to resource multiplexing between child and parent links of an IAB node – Thomas (AT&T)

- 1st check point: November 15

- Final check point: November 19

# Simultaneous Operation of Access and Backhaul Links

From the eIAB WID:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx)

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

Summary of input contributions:

| Company | Proposals |
| --- | --- |
| Nokia, Nokia Shanghai Bell R1-2110775 | Observation 2.1: It is important to limit the total number of frequency domain resource configurations to limit signaling overhead Proposal 2.1: Limit the number of RB sets, M, configurable per DU Cell - The value of M can be discussed with M=4 proposed a likely candidate. Observation 2.2: Signaling configuration for only those RB sets whose configurations differ from Rel-16 config would have limited benefit if the number of RB sets is constrained and may complicate signaling design. Proposal 2.2: If resources configured with Rel-17 H/S/NA overlap with resources configured with Rel-16 H/S/NA, apply Rel-17 H/S/NA config. Proposal 2.3: A single DCI format 2_5 carries the indications for all RB sets regardless of semi-static signaling details. Proposal 2.4: For FDM dynamic indication, IAB nodes should use the legacy Rel-16 resourceAvailability table entries provided in DCI format 2_5 applied to each successive RB set in a slot. Proposal 2.5: For dynamic indication of Rel-17 FDM soft resources, extend DCI format 2_5 payload to maximum DCI payload size. Proposal 2.6: Beam restrictions indicated by the parent DU are applied only to resources configured as semi-static soft by the CU. Observation 2.3: Interference measurements from child nodes can be performed via implementation and do not require sharing child SRS configurations. Proposal 2.7: Beam restrictions should be provided for resources configured as semi-static soft over a set of slot indexes consistent with FDM dynamic resource availability indication. Proposal 2.8: DL RS ID is used as indication for both recommended IAB MT DL and IAB MT UL beams Observation 2.4: Unless additional agreement is made a node operating in either case#6 or case#7 timing mode and changing link direction would necessitate a change in timing mode as well. Observation 2.5: For an IAB node switching from either Rel-17 enhanced multiplexing mode (e.g., case A or case B) requires only a single indication of required guard symbols when switching to Rel-16 TDM operation. Observation 2.6: When an IAB node switches between Rel-16 TDM multiplexing and Rel-17 case A multiplexing indication of desired guard symbols for changing operation within either the MT or DU is unnecessary. Observation 2.7: Required guard symbols for switching operation between Rel-17 case A multiplexing mode and Rel-16 TDM mode can be determined by the parent node via Rel-16 guard symbol indication and timing control for case #1 and case #6 timing mode. Proposal 2.9: Enhance MAC-CE for guard symbol indication to indicate, in symbols, the slot boundary offset of the UL Tx for the child node switching between case#7 and case#1 and switching between case#7 and case#6 timing only. |
| Huawei R1-2110834 | Proposal 1: For simultaneous MT Rx/DU Rx, the IAB node can report some combinations of the following conditions to the parent node per (MT CC, DU cell) pair: - Desired DL TX power adjustment - MT’s DL beam (e.g. TCI state id) - Orthogonal DMRS ports - Guard band Proposal 2: For simultaneous MT Tx/DU Tx, the IAB node can report some combinations of the following conditions to the parent node per (MT CC, DU cell) pair: - UL PSD range - MT’s UL beam (e.g., SRI id) - Timing mode (Case#6) - Orthogonal DMRS ports - Guard band Proposal 3: All required conditions and parameters which have been directly indicated/requested to the parent node (e.g. via MAC-CE) are implicitly acknowledged by the parent node. Proposal 4: TCI state ID and SRS ID can be adopted for the reporting of recommended DL beams and UL beams, respectively. Proposal 5: To support the parent node indicating restricted beams to IAB node, dedicated sounding reference signals (SRS) should be defined/configured. Proposal 6: Reuse Rel-16 DCI format 2_5 to indicate availability of all the soft resources which correspond to the same time resources of the child IAB-DU cell. Proposal 7: For intra-CC FDM, the guard band should be placed in MT resources. Proposal 8: If an IAB-MT is additionally provided TDD-UL-DL-ConfigDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon. Proposal 9: To increase the resources for simultaneous operation, the specification should allow the collision between tdd-UL-DL-ConfigurationDedicated-IAB-MT and cell-specific signals/channels. In the slots with the collision, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT. The list of cell-specific signals/channels includes: - SS/PBCH block - CORESET for Type0-PDCCH CSS set - PRACH Proposal 10: Donor CU can provide two sets of resource configurations to each DU cell: a basic resource configuration for access UEs and one additional resource configuration for child IAB node MTs, and the DU cell should maintain two sets of DU resource configurations simultaneously. Proposal 11: The MAC-CE signaling of Desired/Provided Guard Symbols is enhanced to optionally indicate the number of guard symbols required for switching between at least the following cases: - Case#6 MT Tx and DU Tx/Rx - Case#7 MT Tx (to support Case #7 at parent node) and DU Tx/Rx. |
| vivo R1-2111033 | Proposal 1: Support both semi-static and dynamic adaptation of an IAB-node’s multiplexing operation on a given set of time/frequency resources. Proposal 2: Support to indicate the allowance of a given multiplexing case on a set of time/frequency resources by parent node/CU. Proposal 3: Support IAB node to report the expected multiplexing case (e.g., simultaneous operation or TDM operation) on a set of time/frequency resources to parent node/CU. Proposal 4: Rel-17 multipelxing case(s) can be applied to the symbol(s) which is configured/indicated as flexible for IAB DU and/or IAB MT. Proposal 5: For a given symbol configured/indicated as flexible for DU and/or MT, the associated Rel-17 multipelxing case(s) operated by IAB node can be any multiplexing case assuming that the flexible symbol is either DL or UL. Proposal 6: If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot. - The application of the Rel-17 frequency domain H/S/NA configuration should base on the supported multiplexing mode in the time resource, but not base on whether there is MT scheduling. Proposal 7: For DCI format 2_5, support multiple availability indication fields in the DCI format to indicate resource availibilities in different RB set(s), respectively. Proposal 8: The child node reports the recommended beam(s) per multiplexing case. Proposal 9: The child node reports SSB index/CRI for DL beam recommendation. Proposal 10: The child node reports SSB index/CRI/SRI for UL beam recommendation. Proposal 11: When child node reports recommended TCI(s) to parent node, the associated per beam interference is included in the beam reporting. - The associated interference includes MT-to-MT CLI in case of simultaneous to DU RX and MT TX at IAB node. - The associated interference includes self-interference from IAB DU to IAB MT in case of simultaneous to DU TX and MT RX. Proposal 12: The DU DL beam restriction is at least applied to the occasions of simultaneous DU TX and MT TX. - FFS only applied to a subset of slots from the slots where simultaneous DU TX and MT TX is assumed. Proposal 13: The beam restriction is not applied cell-specific signaling transmission, e.g., SSB and periodic CSI-RS. Proposal 14: Multiple sets of restricted DU DL beams are indicated to child node, where each set may correspond to a given UL MT beam. - FFS how to activate a given set of restricted DU DL beams, e.g., explicit signaling or implicit activation via UL beam indication when scheduling the IAB MT. Proposal 15: Parent node indicates a set of restricted beams in the form of SSB index only, i.e., STC index is not needed. Proposal 16: The guard symbol for swithing between Case#6 MT Tx and Case #7 DU Tx/Rx is not separately defined, i.e., only define guard symbol for Case#6 MT Tx and Case# 1 or Case# 6 DU [Tx]/Rx. Proposal 17: IAB-node is also indicated by parent node, when Case 7 timing is performed at the IAB-node. Proposal 18: The indication of timing mode is associated with indication of multiplexing mode, i.e., Case 6 timing is associated to multiplexing case A and Case 7 timing is associated to multiplexing case B. |
| Intel R1-2111512 | Proposal 1: Consider the following two alternatives for a single DCI format 2_5 to indicate multiple soft RB sets availability in one slot. - Alt. 1: Extend resourceAvailability mapping table so that the indication of availability can be applied to indicate multiple soft RB sets in a slot - Alt. 2: Extend RRC signaling AvailabilityCombination definition to include multiple soft RB sets indication within a slot in a resourceAvailability indication Proposal 2: Limit the maximum number of semi-static configured soft RB sets per slot. Proposal 3: An IAB-node’s SSB ID, used by a parent node to indicate IAB-DU restricted beams, should be related to the STC configuration for access only. No STC index is needed. Proposal 4: For recommended beam indication from an IAB-MT to its parent node, TCI state ID for DL beam indication and spatial relation info for UL beam indication defined in current specification can be re-used. |
| ZTE, Sanechips R1-2111591 | Proposal 1: One frequency domain configuration can be used to indicate the H/S/NA resource types of all the RB sets at a given time. Proposal 2: If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, the following is supported: • Rel-17 frequency domain H/S/NA configuration is applied. Proposal 3: The carrier corresponding to the reference SCS in the carrier list of the IAB-node DU cell is used as the reference carrier to determine RB sets for frequency domain H/S/NA configuration. Proposal 4: A parent node should be made aware of the SCS-specific carriers and frequency domain H/S/NA configuration of its child node DU’s cell(s). Proposal 5: The Rel-16 AvailabilityCombination is reused to include all the RB sets in a resourceAvailabilty indication. Proposal 6: Update resourceAvailability mapping table defined in TS38.213 as shown in Table 1. Proposal 7: There is no need to extend the maximum payload size of DCI format 2_5. Proposal 8: It is not necessary to include STC index in restricted beams indication information of an IAB-DU cell. Proposal 9: The restricted beam indication from the parent node to the IAB node may be indicated (or specified) to be associated with time resources (e.g., slot index in a period). Proposal 10: For the recommended beam indication from IAB-MT to the parent node, IAB-MT DL beams are defined by TCI state ID, IAB-MT UL beams are defined by SRI. |
| Samsung R1-2111756 | Proposal 1: The following WA is confirmed as agreement: Working Assumption If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected: - Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied. Proposal 2: Guard band can be generated by each IAB node or parent IAB. Proposal 3: At most one H, one S, and one NA resource type for different RB sets at a given time are supported. Proposal 4: The following H/S/NA resource types are supported for FDM - H type for frequency resource: the IAB DU can assume it can use the frequency resource regardless of the MT’s configuration on the frequency resource - S type for frequency resource: the IAB DU can assume it can use the frequency resource without an impact on the MT’s configuration on the frequency resource - NA type for frequency resource: the IAB DU cannot assume it can use the frequency resource Proposal 5: There is no need to extend AvailabiltyCombination to include multiple RB sets in a resourceAvailabilty indication. Proposal 6: "RB set” in the existing mapping table and the relevant sentences of TS38.213 can be added. Proposal 7: There is no need to extend the maximum payload size of DCI format 2_5 to increase the number of IAB-DU cells. Proposal 8: The beam restrictions are applicable only to simultaneous transmissions in IAB node. Proposal 9: The beam restrictions can be applied only on the Soft resource. Proposal 10: STC index is not indicated for the beam restrictions. Proposal 11: For UL Tx beam in the beam recommendation, spatial relation information and/or Rel-17 joint DL/UL TCI ID are used. Proposal 12: Only one set of signaling for Desired/Provided Guard Symbols considering maximum values of all possible timing switching can be supported. |
| AT&T R1-2111804 | Proposal 1: To support dynamic operation of non-TDM multiplexing modes, a child IAB node cannot operate under a given mode until all conditions and parameters which are indicated from the child node are acknowledged or provided by the parent node including the following: - Case 6/7 timing mode usage - DL Tx power adjustment - IAB-MT DL/UL beam recommendations - Guard symbols Proposal 2: DCI format 2_5 message is extended to support indication of availability of the soft resources corresponding to a given time resource of the child IAB-DU cell for an RRC configured subset of RB sets within a given a resourceAvailability indication. Proposal 3: The recommended beam indication from the IAB-MT to the parent node via MAC-CE includes the following associated configurations/resources of the child IAB node: - For DL: via DL TCI state id, For UL: via spatial relation - Multiplexing mode - {MT CC, DU cell} pair - Resource type (H, S, or H and S) - Starting slot index (to enable) or ending slot (to disable) Proposal 4: The restricted beam indication from the parent node to the child node via MAC-CE includes the following associated configurations/resources of the child IAB node: - RS ID: SSB ID, STC Index, and CSI-RS configuration - Multiplexing mode - {MT CC, DU cell} pair - Resource type (H, S, or H and S) - Starting slot index (to enable) or ending slot (to disable) |
| Apple R1-2111892 | Proposal 1: Alt1 is more preferred given that it requires less signaling overhead in comparison with Alt2. Proposal 2: Confirm the working assumption, that is an IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied. |
| Huawei, HiSilicon, R1-2111929 | Proposal 1: Number of guard symbols for MT and DU flexible symbols = min(Number of guard symbols for MT and DU downlink switching, Number of guard symbols for MT and DU uplink switching) Proposal 2: Introduce negative value of Ng to support efficient resource utilization. |
| Lenovo, Motorola Mobility R1-2111952 | Proposal 1: Support joint application of time-domain and frequency-domain H/S/NA configurations. Proposal 2: Specify rules for determining H/S/NA resource types for time-frequency resources based on separate configurations in time and frequency domains. Proposal 3: Allow multiple frequency-domain H/S/NA configurations for non-overlapping and overlapping RB sets provided that the configurations do not assign conflicting H/S/NA attributes to any given resource. Proposal 4: Extend DCI 2-5 for frequency-domain availability indication by allowing a unit of N PRBs in addition to the unit of 1 slot. Proposal 5: Support conditional availability indication as a balance between existing best-effort non-TDM and TDM-only availability indication. Proposal 6: The beam restriction indication from the parent node includes indication of time and frequency resources (e.g., slots/symbols and RB sets) of the IAB-DU over which the beam restriction is applied. No need to include information related to H/S resource attributes and relationship with IAB-MT beams. Proposal 7: Support indication of recommended beams to the parent node as part of dynamic signaling of “conditions/parameter” agreed in RAN#105-e. Proposal 8: Support per-panel granularity by allowing dynamic indication of restricting/using beams and their associated antenna panels for specific time-frequency resources. Proposal 9: Support early beam/panel indications for the upstream link in addition to downstream beam indications. Proposal 10: Support signaling IAB node capabilities for timing alignment and multi-panel and baseband characteristics. Proposal 11: Support dynamic signalling for communicating operation constraints such as beam, power, interference, and timing alignment constraints along with information of time, frequency, and spatial resources. Proposal 12: Support MAC CE signaling for activation/deactivation of Case 6 timing. Proposal 13: Support MAC CE signaling for activation/deactivation of Case 7 timing. Proposal 14: Support RRC configuration for Case 6 and Case 7 timing alignment in addition to dynamic signaling. Proposal 15: Signalling for timing alignment indication is linked to multiplexing capability of the IAB node as well as whether the IAB node requires enhanced timing alignment. Proposal 16: Support a capability signalling, e.g., number of IFFT/FFT windows, to indicate whether the IAB node requires timing alignment between IAB-MT and IAB-DU operations. If negative, the IAB node can transmit unaligned OFDM symbols (Case A) and receive/process unaligned OFDM symbols (Case B), which simplifies signalling and configuration significantly. |
| LG Electronics (R1-2111983) | - Simultaneous operation schemes between MT and DU Proposal 1: To reduce signaling overhead for frequency domain H/S/NA configuration, there can be up to M configurations where each configuration corresponds to H/S/NA resource type of one or multiple consecutive RB sets. Proposal 2: For an IAB-node with simultaneous operation, frequency domain availability is indicated independently in addition to the Rel-16 time domain soft resource availability indication. Proposal 3: For a specific slot, one of Rel-16 time domain soft resource availability and Rel-17 frequency domain soft resource availability is applied depending on the TDM/non-TDM operation mode in the slot. Proposal 4: Discuss whether the soft resource availability for time domain and frequency domain are indicated using different fields or the same field. Proposal 5: DU beam restriction is applied only to time resources in which the IAB node operates in non-TDM. Proposal 6: The restricted beam information of the DU can be associated with MT’s UL beam. Proposal 7: When IAB node operates in non-TDM, DU beam restriction is not applied for Hard resource or Soft resource with availability indication. Proposal 8: When IAB node operates in non-TDM, DU operation with the restricted beam is allowed in for Soft resource without availability indication. Further discuss whether to allow DU operation with the restricted beam in NA resource. Proposal 9: Support to describe that UE/IAB-MT does not assume that DL Tx power adjustment (if provided) is applied to the SSB index (if supported) indicated as QCLed reference signal in DL TCI state ID. Proposal 10: Support to use SRI for indicating UL recommended Tx beam to the parent node. Proposal 11: Support separation of CDM group between DU and MT for SDM operation of IAB-node. Proposal 12: For IAB-nodes that require timing alignment for simultaneous operation, - If MT is configured Case#6 timing for MT UL Tx, MT and DU can perform simultaneous Tx/Tx when the MT has UL Tx direction and the DU has DL Tx direction within the time resource applying Case #6 timing. - If Case#7 timing is allowed for DU UL Rx, MT and DU can perform simultaneous Rx/Rx when the MT has DL Rx direction and the DU has UL Rx direction within the time resource applying Case #7 timing. - When the MT and DU of the IAB-node have a Tx/Rx resource direction combination that is not capable of simultaneous operation within the time resource applying Case #6 or Case #7 timing, the MT and DU can be operated in TDM-manner. Proposal 13: For the case of an IAB-node that does not require any timing alignment for simultaneous operation, the IAB-node can perform simultaneously without restriction on time resource or within the explicitly allowed time resource if the combination of Tx/Rx direction between MT and DU is capable of. Proposal 14: It is supported to report the applicability of the given multiplexing operation mode to the parent node. Proposal 15: The number of desired/provided guard symbols for switching cases ‘Case#6 MT Tx to DU Tx’ and ‘DU Tx to Case #6 MT Tx’ are assumed to 0 without MAC CE signaling. |
| ETRI R1-2111995 | Proposal 1: Support the following multiplexing modes for DU/MT simultaneous operations: - Multiplexing mode A: DU Tx / MT Tx - Multiplexing mode B: DU Rx / MT Rx - Multiplexing mode C: DU Rx / MT Tx - Multiplexing mode D: DU Tx / MT Rx - Multiplexing mode E: DU/MT FDM required - Multiplexing mode F: DU/MT SDM required - Multiplexing mode G: Inclusive set of non-TDM multiplexing modes Proposal 2: The configuration/indication of Rel-17 frequency domain resource(s) should reflect the latest report/request on the simultaneous operation capabilities. Proposal 3: Support Alt.2 (i.e. the reported conditions/parameters are implicitly acknowledged by the Rel-17 frequency domain resource configuration/indication from parent node), if RAN1 does not have a consensus among Alt.1 (explicit ACK) and Alt.2 (implicit ACK) during RAN1#107-e meeting. Proposal 4: Introduce L2 (MAC CE) based report/indication on conditions/parameters to facilitate adaptation between multiplexing operation modes, where the report/indication includes the following contents, at least: - Supported multiplexing mode (FDM and/or SDM). - Required timing mode. - Required DL/UL power control support. - Information on the “non-preferred” beam. - Information on the “recommended” beam. - Required number of guard symbols for switching of multiplexing mode. - Required guard band for FDM Proposal 5: Introduce higher-layer parameter(s) to configure the following information: - Minimum periodicity (or prohibit time) of report/indication on conditions/parameters to facilitate adaptation between multiplexing operation modes. - Recommendations on the conditions/parameters from parent node perspectives. Proposal 6: The number of different frequency domain configurations at a given time can be 1 or 2. Proposal 7: The WA on Rel-16/-17 HSNA configurations can be confirmed with the following definitions on non-TDM multiplexing modes: - Multiplexing mode A: DU Tx / MT Tx - Multiplexing mode B: DU Rx / MT Rx - Multiplexing mode C: DU Rx / MT Tx - Multiplexing mode D: DU Tx / MT Rx - Multiplexing mode E: DU/MT FDM required - Multiplexing mode F: DU/MT SDM required - Multiplexing mode G: Inclusive set of non-TDM multiplexing modes Proposal 8: Extend the mapping between values of resourceAvailability elements and types of soft symbols/resources for the time/frequency resources configured with both Rel-17 frequency domain H/S/NA configuration and the Rel-16 time domain H/S/NA configuration Value; Indication. 0; No indication of availability for soft symbols. 1; DL soft symbols are indicated available No indication of availability for UL and Flexible soft symbols. 2; UL soft symbols are indicated available No indication of availability for DL and Flexible soft symbols. 3; DL and UL soft symbols are indicated available No indication of availability for Flexible soft symbols. 4; Flexible soft symbols are indicated available No indication of availability for DL and UL soft symbols. 5; DL and Flexible soft symbols are indicated available No indication of availability for UL soft symbols. 6; UL and Flexible soft symbols are indicated available No indication of availability for DL soft symbols. 7; DL, UL, and Flexible soft symbols are indicated available. 8; DL soft resources in the configured RB sets are indicated available No indication of availability for UL and Flexible soft symbols. 9; UL soft resources in the configured RB sets are indicated available No indication of availability for DL and Flexible soft symbols. 10; DL and UL soft resources in the configured RB sets are indicated available No indication of availability for Flexible soft symbols. 11; Flexible soft resources in the configured RB sets are indicated available No indication of availability for DL and UL soft symbols. 12; DL and Flexible soft resources in the configured RB sets are indicated available No indication of availability for UL soft symbols. 13; UL and Flexible soft resources in the configured RB sets are indicated available No indication of availability for DL soft symbols. 14; DL, UL and Flexible soft resources in the configured RB sets are indicated available. 15; Reserved. Proposal 9: Support CSI-RS ID to indicate restricted beams in addition to the agreed SSB ID. Proposal 10: Support different beam restrictions H and S resources. Proposal 11: The following information can be associated with the restricted beam indication: - Multiplexing modes including: - Multiplexing mode A: DU Tx / MT Tx - Multiplexing mode D: DU Tx / MT Rx - Multiplexing mode E: DU/MT FDM required - Multiplexing mode F: DU/MT SDM required - Multiplexing mode G: Inclusive set of non-TDM multiplexing modes - MT’s DL beam by TCI ID and SSB/CSI-RS ID - DU resource configuration type (H and/or S) - Note: Slot index is covered by the associated multiplexing mode. Proposal 12: The recommended beam indication from the IAB-MT to the parent node are provided via MAC-CE: - For DL Rx beam(s): using one or more of the following: - DL TCI state ID - SSB ID - CSI-RS ID - For UL Tx beam(s): using one or more of the following: - Spatial relation (SSB ID, CSI-RS ID, SRS ID) Proposal 13: For the recommended beam report the UE/IAB-MT may not assume the DL power adjustment on SSB and PDCCH. Proposal 14: For the recommended beam report the UE/IAB-MT may assume the same DL power adjustment according to the QCL/TCI chain rule. Proposal 15: Support a report on non-preferred MT beams for simultaneous operations, in addition to the agreed report on recommended MT beams. Proposal 16: Support X>1, where X is the maximum numbers of the recommended/ restricted beams for simultaneous operations. - Consider X=4, at least. Proposal 17: Clarify IAB DU/MT behaviors when the beam indicated by Rel-16 simultaneous TCI/spatial relation update is not included in the recommended beams (is included in the restricted beams). - E.g., introduce a higher layer configuration to indicate the applicable multiplexing mode for Rel-16 simultaneous TCI/spatial relation update. Proposal 18: Clarify that there is no impact on IAB-MT behavior due to conflicts between cell-specific signals/channels and other resource configurations of the IAB-MT, when the IAB node supports SDM between DU and MT resources at least. |
| NTT DOCOMO R1-2112124 | Proposal 1: Confirm the following working assumption. - If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected: - Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied. Proposal 2: Whether an IAB node is operating in TDM or non-TDM mode in a slot can be determined at least based on following rules. - IAB node is operating in TDM mode in a slot and time domain H/S/NA is applied, - If MT does not Tx/Rx. - Or if the simultaneous MT/DU operation corresponding to the MT/DU DL/UL configuration - at the symbol is not supported, e.g., the symbol is configured as MT DL and DU DL, but MT-Rx/DU-Tx is not supported - Or if corresponding timing mode for simultaneous operation is not indicated, e.g., case6 timing is not indicated for simultaneous MT-Tx/DU-Tx. - Otherwise, IAB node is operating in non-TDM mode and frequency domain H/S/NA is applied. Proposal 3: AvailabiltyCombination can be extended to include multiple resourceAvailabilty, where each resourceAvailabilty includes availability indication for one RB set. Proposal 4: Restricted DU beam indication is only applicable for simultaneous MT-Tx/DU-Tx. Proposal 5: Restricted DU beams are indicated to be associated with MT UL beam, where MT UL beam is indicated using RS ID. Proposal 6: The indication of restricted DU beams does not need to be associated with slot index. Proposal 7: With parent node’s indication of restricted beam, whether a beam is used by DU is up to DU implementation. Proposal 8: Recommended beams are indicated per multiplexing case. Proposal 9: For recommended beam indication from IAB-MT, DL Rx beam is indicated using RS ID, UL Tx beam is indicated using RS ID. Proposal 10: For recommended beam indication from IAB-MT, a RS resource set can be configured as candidate MT beam pool. Proposal 11: The recommended beam indication from IAB-MT can be triggered by periodic timer. Proposal 12: Parent node indicates whether Case#6 or Case#7 timing is performed via UL scheduling DCI. Proposal 13: On a DU flexible symbol, parent node can configure/indicate IAB node MT Tx/Rx, and IAB node DU will Tx/Rx on the symbol based on MT transmission/reception direction and its multiplexing capability. Proposal 14: The MAC-CE signaling of Desired/Provided Guard Symbols is enhanced to optionally indicate the number of guard symbols required for switching between at least the following cases: - Case#6 MT Tx and [case #7] DU Tx/Rx - Case#7 MT Tx (to support Case #7 at parent node) and DU Tx/Rx Proposal 15: Guard band can be handled by IAB node implementation. |
| CEWiT | Proposal 1: Mechanism to inform multiplexing capability of parent nodes to IAB node is supported Proposal 2: Signalling of multiplexing capability and supported modes from IAB node to CU and parent-DU is supported Proposal 3: Following alternatives are proposed for configuring active mode of operation at an IAB node - Alt1: Donor configuring the active mode of operation of IAB node - Alt2: Parent node configuring the active mode of operation of IAB node Proposal 4: Dynamic switching between multiplexing modes of IAB node is supported under the control of parent node Proposal 5: Mechanism to fall back to TDM mode from simultaneous mode at IAB node is supported Observation 1: For an IAB node - The TA associated with UL-Tx at IAB-MT and guard required for switching to/from UL-Tx changes based on active mode of operation of IAB node and its parent node - The timing of UL-Rx at IAB-DU and guard required for switching to/from UL-Rx changes based on active mode of operation of IAB node and its child node Proposal 6: Parent node update TA and guard symbols required for switching to/from UL-Tx before mode switching at IAB node or its parent node Proposal 7: IAB node inserts additional guard symbols for switching to/from UL-Rx at IAB-DU before mode switching at IAB node or its child node Proposal 8: An IAB node cannot operate under a given non-TDM multiplexing mode until all required conditions and parameters which have been directly requested to the parent node are explicitly acknowledged by the parent node. |
| Qualcomm R1-2112235 | Frequency-domain resource management Observation 2.1: The reason an IAB-node is expected to adhere to a frequency-domain H/S/NA configuration, provided by the CU, may be to accommodate the needs of a child-node who wishes to operate in a non-TDM multiplexing mode. Therefore, basing the choice of selection between the IAB-node’s time-domain and frequency-domain H/S/NA configuration only on the IAB-node’s operation mode is not right. Observation 2.2: The parent-node and/or other IAB-nodes/IAB-donors need to know the IAB-DU cell configuration, within each slot, to avoid any conflict, better utilize the available resources, and/or manage interference. In case, IAB-DU is provided with both time-domain and frequency-domain resource configurations and there is a dynamic decision to select one of the two configurations, the other nodes should be informed about such a decision, and this may lead to undesired (sometime infeasible) signalling overhead and/or latency. Proposal 2.1: If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, the Rel-17 frequency domain H/S/NA configuration is adopted for that RB set and that slot. Observation 2.3: Even if the parent-node does not grant the desired/recommended configurations, indicated by the IAB-node, the IAB-node may still be able to support an enhanced multiplexing (no-TDM) mode, possibly with some limitations and/or reduced performance. Observation 2.4: The indications of desired/recommended configurations, by the IAB-node to the parent-node, are optional. In case the IAB-node does not provide such indications, the parent-node does not necessarily know whether the IAB-node can or cannot support an enhanced multiplexing (non-TDM) mode. Proposal 2.2: The decision about whether an IAB-node operates under a given non-TDM multiplexing mode is made by the IAB-node. Other nodes, including the parent-node, may not exactly know the multiplexing mode of the IAB-node within a slot. Proposal 2.3: Extend the AvailabiltyCombination to provide multiple resourceAvailability indications, wherein each resourceAvailability indication is associated with one RB set. A resourceAvailability indication, for a given RB set, follows the same design as in Rel-16 and is configured to be a sequence of INTEGER (0..7) elements associated with a sequence of consecutive slots. Proposal 2.4: Update resourceAvailability mapping table defined in TS38.213 so that the indication of availability can be applied over soft resources in frequency-domain for DL or UL or Flexible symbols. - E.g., replacing “soft DL/UL/Flexible symbols” with “soft resources in DL/UL/Flexible symbols” Adaptation of multiplexing mode Observation 3.1: For a supported enhanced multiplexing operation mode at an IAB-node, the assistance information on whether FDM is required or not can be beneficial - for the donor-CU to determine more efficient H/S/NA resource pattern, - for the parent node to make a better decision on coordinating soft resources with its child node. Proposal 3.1: Support indication of whether FDM is required or not for an enhanced multiplexing operation mode to donor-CU and/or parent node. Timing management Proposal 3.2: An IAB-DU decides about its UL RX timing reference (e.g., whether to adopt Case 1, or Case 7) without any indication from its parent-node. Observation 3.2: An IAB-MT’s Case 6 UL TX timing is to facilitate enhanced duplexing at the IAB-node, while IAB-MT’s Case 7 UL TX timing is to facilitate enhanced duplexing at the parent-node. An IAB-node cannot map Case 7 UL TX timing (of IAB-MT) to any subset of its time resources, without explicit indication from the parent-node. It is desired to have a unified design for indication of different UL TX timing cases. Proposal 3.3: An IAB-MT is provided with Timing Case Indication that explicitly indicates a list of slots and their associated UL TX timing cases (i.e., one of {Case 1, Case 6, Case 7} for each slot). Timing Case Indication is provided via MAC-CE. Proposal 3.4: Timing Case Indication received from a serving cell is applicable to all other cells in the same timing advance group (TAG). Proposal 3.5: IAB-MT indicates, to its parent-node, whether it desires Case 6 UL TX timing, wherein this indication is associated with some combination (one or multiple) of the following IAB-node’s configurations: - Multiplexing mode, - MT’s UL beam (e.g., SRI id) - (MT CC, DU cell) pair, - DU resource configuration (e.g., whether the concurrent MT’s and DU’s signals are FDMed or overlap in the frequency domain). Beam management Observation 4.1: Indication of “not-preferred” IAB-MT’s beam(s) (in addition to recommended beam(s)) may provide more efficient and flexible assistance information. It can also effectively be used to indicate required fallback to TDMing, when an enhanced multiplexing mode of operation is not supported. Proposal 4.1: Further support indication of not-preferred IAB-MT’s beam(s) (for both TX and RX) to the parent-node. Proposal 4.2: The recommended and/or not-preferred IAB-MT’s beam(s), indicated by the IAB-MT to its parent-node, is indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations: - Multiplexing mode, - (MT CC, DU cell) pair, - DU resource configuration (e.g., whether the concurrent MT’s and DU’s signals are FDMed or overlap in the frequency domain). Proposal 4.3: The indication of the restricted IAB-DU’s beam(s), provided by the parent-node to the IAB-MT, is only applicable to the IAB-DU’s DL TX beams. Proposal 4.4: SSB ID and STC index, based on the IAB node’s DU configurations, are used by a parent node to indicate beams of an IAB-DU in the direction of which simultaneous operation is restricted. Proposal 4.5: The restricted beam indication from the parent node to the IAB node may be indicated (or specified) to be associated with some combination (one or multiple) of the following IAB node’s parameters: - Multiplexing mode, - MT’s DL beam (e.g. TCI state id) or MT’s UL beam (e.g., SRI id), - DU resource configuration (e.g. soft resources, and/or whether concurrent MT’s and DU’s signals are FDMed or overlap in the frequency domain). Proposal 4.6: The restricted IAB-DU’s beam(s), indicated by the parent-node, - is considered as a recommendation, without any mandated behaviour at the IAB-DU, within the IAB-DU’s Hard resources. - is expected to be followed by the IAB-DU, within the IAB-DU’s soft resources. |
| Ericsson R1-2112356 | Proposal 1 The frequency domain granularity, i.e., the N RBs in an RB set is related to MT’s BWP and number of PRBs in an RBG, NRB(RBG), as given in Table 1. Proposal 2 Support per-BWP frequency domain H/S/NA configuration, including the RB set configuration. Proposal 3 RB sets which are non-overlapping with the IAB-MT’s active BWPs, are treated as Hard DU resources. Proposal 4 RAN1 should discuss the indexing of RB sets (including complete and incomplete RB sets) and how many incomplete RB sets can be configured for each IAB-DU cell. Proposal 5 In case that an IAB-DU cell is overlapping with multiple IAB-MT’s serving cells, the IAB-DU cell is configured with one N value which is derived based on the largest RBG size of all IAB-MT cells and is common to all the MT-BWPs. Proposal 6 If a resource is configured as Hard, or explicitly indicated as Soft-IA the IAB-DU can transmit, receive, or either transmit or receive according to its configuration only if it does not impact the IAB-MT’s actual ability to operate in any other resource that is either configured Not Available, or configured Soft and not explicitly indicated available. Proposal 7 If a resource is configured as Soft (including implicitly indicated available), the IAB-DU can transmit, receive, or either transmit or receive only if it does not impact the IAB-MT’s actual ability to operate in any resource, either configured Not Available, or configured Soft and not explicitly indicated available. Proposal 8 The parent node can be made aware of the Rel-17 frequency domain resource configurations of the child IAB-node, including the RB set configuration. Proposal 9 Extend the semi-static IAB node multiplexing capability to include FDM-required for multiplexing modes MT RX/DU RX and MT TX/DU TX in the Multiplexing Info IE in 3GPP TS 38.473 [9]. Proposal 10 Confirm that working assumption that if both Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, an IAB node applies the frequency domain H/S/NA if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied. Proposal 11 The IAB-node may report to the parent node a set of multiplexing conditions on the required timing mode, power levels and beam indication: • For simultaneous MT-TX/DU-TX, the multiplexing conditions include whether the IAB-node requires: o Case-6 timing o Parent node indication of restricted IAB-DU beams o IAB-node indication to its parent node on its MT’s minimum UL TX power level • For simultaneous MT RX/DU RX, the multiplexing conditions include whether the IAB-node: o Supports adjustment of child node UL TX timing in Case-7 timing o Supports adjustment of UL TX timing for parent node in Case-7 timing o Requires IAB-node indication of recommended IAB-MT beams o Requires IAB-node indication to support parent node maximum DL power level Proposal 12 Whether or not to switch to FDM/SDM should be based on the explicit or implicit ACK/NACK response from the parent IAB-node. Proposal 13 RAN1 selects the following two signaling alternatives for the fallback request to TDM operation mode: Alt1: UCI-based Alt2: MAC-CE based Proposal 14 Fallback to TDM does not need an ACK response from the parent IAB-node. Proposal 15 For explicit availability indication of frequency-domain Soft resources, each availabilityCombinationId of availabilityCombinations can indicate availability for a single or multiple RB sets. Proposal 16 Rel-17 enhancement on DCI format 2_5 should at least consider extension of the maximum payload size of DCI format 2_5 to increase the number of IAB-DU cells that can be provided with availability information for Soft resources. Proposal 17 Introducing group indexing bits in DCI format 2_5 to further increase the number of IAB-DU cells that can be indicated AI. Proposal 18 One resourceAvailability element of the availabilityCombinations table is reserved to signal to the IAB-DU that “No new Availability Indication is provided” in DCI format 2_5. Proposal 19 SDM, including beam restrictions/recommendations, is restricted to Soft resources. Proposal 20 Explicit availability indication overrides SDM beam restrictions in Soft resources. Proposal 21 The restricted beam indication from the parent node to the IAB node may be indicated to be associated to IAB-MT’s UL TX beam, via the parent node’s DL RS ID (e.g., SSB ID). Proposal 22 RS ID (including SSB index and STC index) is sufficient for a parent node to indicate beams of an IAB-DU in the direction in which simultaneous operation is restricted. Proposal 23 RS ID (parent node’s SSB ID and CSI-RS ID) is used by a child node to indicate a set of preferred beams to the parent node for both IAB-MT UL TX beam and DL RX beam. Proposal 24 Desired/Provided Guard Symbols are signaled to cover all relevant switching combinations among Case #1, Case #6 and Case #7 timing alignments in Figure 6c): Guard symbols may be needed when the IAB-node transitions: • Case #6 MT UL TX -> Case #1 DU UL RX • Case #6 MT UL TX -> Case #7 DU UL RX • Case #7 DU UL RX -> Case #6 MT UL TX Proposal 25 Include new RRC parameters which are needed to support dynamic indication of frequency domain soft resource availability to the list of upper layer parameters for eIAB physical layer operation. Proposal 28 Include the following for the value range of P02 in the RRC parameters for eIAB: N is at least the # PRBs corresponding to the MT’s configured # PRB of an RBG. Proposal 28 Include the following for the value range of P02 in the RRC parameters for eIAB: N is at least the # PRBs corresponding to the MT’s configured # PRB of an RBG. |

ISSUE 2.1: FREQUENCY DOMAIN MULTIPLEXING

Proposal 2.1.1a: Confirm the following RAN1#106bis-e working assumption:

Working Assumption

If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected:

- Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.

Discussion: Views on proposal 2.1.1a?

| Company | Comments |
| --- | --- |
| ETRI | Support the proposal. |
| NTT Docomo | Support |
| LG Electronics | We support the proposal. |
| ZTE, Sanechips | Do not support. As pointed at previous meetings, if such dynamic HSNA configuration selection were supported, It is difficult to make a consistent understanding on the selected HSNA configuration on the IAB node among the IAB node, its parent node(s), CU, neighbour nodes(for CLI). Alt 1 itself is not clear, e.g., whether a NA RB set can fallback to a Hard RB set? From our point of view, applying the frequency domain H/S/NA configuration is a precondition to operate in a FDM mode, but not vice versa; We propose to support the most simple solution, i.e., An IAB node applies the frequency domain H/S/NA when it is provided. |
| Vivo | Support |
| Huawei, HiSilicon | Not support. We think dynamic switching between two semi-static H/S/NA resource configurations will result in several issues. The fundamental issue is that whether to operate with TDM or non-TDM mode is a scheduling decision at IAB node which may not be known by the parent node. From this perspective, there is no way that the parent node and IAB node can share the same understanding on the semi-static H/S/NA resource configuration for the IAB-DU. This would results in scheduling conflictions between the IAB node and parent node. In addition, as pointed out by several other companies, there are information exchanges of H/S/NA configurations among nodes for CLI handling or for DC operations. Dynamical switching between configurations makes information exchange extremely complicated if not possible at all. |
| Samsung | Support in principle. However, some update to the working assumption may be possible to address concerns from other companies. In our view, the whole procedure is as follows. If the IAB node is allowed to operate in a non-TDM multiplexing mode by an indication from the parent, IAB node applies the FDM H/S/NA. Otherwise, the IAB node applies TDM H/S/NA. During the non-TDM multiplexing mode, the parent IAB can indicate the TDM operation to the IAB node. Or if the IAB node decides that the non-TDM multiplexing mode is not possible or the non-TDM multiplexing mode cannot provide more performance gains than the TDM operation, the IAB node can fall back from the non-TDM multiplexing mode to the TDM operation. The fallback to the TDM operation can be dynamically indicated by the IAB node to the parent. |
| Lenovo, Motorola Mobility | Do not support. As explained in Lenovo/MM tdoc, the working assumption is invalid as it depends on “a non-TDM multiplexing mode,” which has not been agreed. |
| Ericsson | Support. With ZTE’s proposal, we would still need some fallback for the case when FDM may not be feasible, due to, e.g., timing or power mismatch. To our understanding, if this fallback is not provided, there would need to be some other resolution mechanism in place. It is the donor CU’s responsibility to provide a TDM/FDM configuration allowing fallback to TDM. |
| Nokia | Do not support. If an IAB node receives both a Rel-16 and Rel-17 H/S/NA configuration and makes its own determination on multiplexing mode of operation, the parent node and CU will be unaware of which resource configuration is being applied, and therefore may be unable to appropriately schedule resources for other nodes connecting on the same cell. Additionally, the parent node may always be required to provide a Rel-16 and Rel-17 DCI format 2_5 since the parent would be unaware of which resource configuration is applied. For these reasons, a more straightforward prioritization such as applying the Rel-17 config in all scenarios where Rel-16 and Rel-17 are provided in the same resource is preferred. |
| Qualcomm | We do not support this proposal. In our view, (1) it is not clear how exactly this scheme works without some additional required signaling and coordination, and hence more discussions and specification work will be needed to assure a proper operation, (2) the real benefits of such a scheme are not clear, (3) somehow this scheme ignores the principal role of the CU to provide central coordination, by allowing local/dynamic switching across semi-static configurations, (4) there are a few major issues with this scheme, explained by other companies and a subset of them reiterated below, Given all above, and the issues, complexities, and more required spec work for this scheme, and since there is already a super simple and efficient scheme (i.e., apply FD HSNA when provided), we strongly believe there is no justification to this proposal. Regarding the issues of this scheme, and in addition to what other companies pointed out, we should note that a reason that CU provides an IAB-node with a FD HSNA may really be due to the needs (e.g., FDMrequired as suggested in proposal 2.3.7a) of a child-node who wishes to operate in a non-TDM multiplexing mode. Therefore, in such a case and in the context of the above proposal, the choice between the IAB-node’s time-domain and frequency-domain H/S/NA configuration should have been based on the child-node’s operation mode, and NOT the IAB-node’s operation mode. Therefore, so we conclude in case the CU decides to provide a semi-static configuration (e.g., FD HSNA) to an IAB-node, it is sensible for the IAB-node to just follow what CU has indicated. |
| Intel | We support the proposal. In previous Alt.2, freq-domain HSNA must be applied once it is semi-static provided, which means FDM or non-TDM multiplexing mode is required/decided semi-statically at the IAB-node. This is against the dynamic adaptation of multiplexing mode between IAB-node and its parent. |

Proposal 2.1.2a: Limit the maximum number of RB sets, M, configurable per DU cell

- M=[4]

- If multiple frequency-domain H/S/NA configurations contain overlapping RB sets the configurations should not assign conflicting H/S/NA attributes to any given resource within a slot

- A given frequency domain configuration can be associated with a BWP of a co-located MT

Discussion: Views on proposal 2.1.2a?

| Company | Comments |
| --- | --- |
| ETRI | We think the value M=2 could be supported, at least. Re the second and third bullets, we think these are implementation issues, i.e. no need to be clarified in the specifications. |
| NTT Docomo | Support the main bullet and first sub bullet We fail to understand the motivation of the second sub bullet. Why there will be multiple configuration containing overlapping RB sets. |
| LG Electronics | We are ok with the proposal in principle. But, we don’t support the last bullet, since the frequency domain configuration can be applied regardless of a BWP of MT in our view. In order to commonly apply the frequency domain configuration to all MT BWPs, it is preferable that the frequency domain resource is configured based on the CRB of the DU. |
| ZTE, Sanechips | Support the main bullet and OK with the first sub-bullet For the second sub-bullet, we prefer to use a single frequency-domain H/S/NA configuration to indicate the respective H/S/NA attributes of the RB sets within a slot. For the last sub-bullet, we think this has been discussed many times, the DU configuration should not be coupled with the BWP(s) of IAB MT, on the other hand, as pointed in our contribution R1-2111591, the RB sets on a DU cell is not clear, since multiple SCS-specific carriers may be configured on a DU cell, the reference carrier for these RB sets should be clearly defined. Based on above, we proposed to update the proposal as: Limit the maximum number of RB sets, M, configurable per DU cell - M=[4] - If multiple frequency-domain H/S/NA configurations contain overlapping RB sets the configurations should not assign conflicting H/S/NA attributes to any given resource within a slot if the RB sets could contain overlapping RBs. - A given frequency domain configuration can be associated with a BWP of a co-located MT - The RB sets on a DU cell is associated with a reference carrier based on the Reference SCS of Frequency Domain H/S/NA Configuration. |
| vivo | Configuring multiple frequency H/S/NA may be over-optimized, the motivation should be clarified. |
| Huawei, HiSilicon | We are fine with the first sub-bullet and keep the value in bracket. For the second sub-bullet, we don’t see the necessity or motivation to provide multiple frequency domain H/S/NA configuration to an IAB node, and thus the solution is not necessary. For the third issue, we think the following agreement implies that the size of RB set can be configured independently hence this bullet is not needed. Agreement A single value for the RB set size, N, is configured for a given IAB-DU cell’s Rel-17 frequency domain H/S/NA configuration |
| Samsung | For the first bullet, our preference is to have at most 3 i.e., one H, one S, one NA. For the second bullet, as other companies commented, there is another option to allow single freq. domain H/S/NA configuration to any given resource within a slot. |
| Lenovo, Motorola Mobility | Fine with the main line and the first and second bullet items. The third bullet needs to be justified. |
| Ericsson | Support in principle. We think #RB sets M=4 is a bit too small considering the flexibility in the RB set size, N, does not match the relative inflexibility in #RB sets. In our understanding, this would result in a very crude differentiation of H, S and NA sizes. However, the maximum number of RB set groups should not need to exceed 4, i.e., one each for H, S, NA. For second bullet, for simplicity and without any real loss, we would prefer if RB sets are non-overlapping. We think the above agreement should be combined with the following agreement: Any DU resource outside the IAB-MT’s active BWP is considered Hard. If companies cannot agree to the third bullet, we think the following is needed, in addition to what is in the FL Proposal: DU carrier information should be provided to the parent node. |
| Nokia | We support the proposal in principle. We understand that second bullet to refer to handle scenarios in which multiple RB sets are configured over an overlapping subset of physical resources (i.e., RB set 1 is configured from RB 0:N-1, while RB set 2 is configured from RB 0:2N-1. In this scenario multiple conflicting H/S/NA configurations may cause ambiguity for the IAB-DU. We are fine with this sub-bullet. We don’t understand the motivation of the third sub-bullet, since to our understanding, RB set is a contiguous set of RBs that are indicated as H/S/NA for a DU. An association with a BWP is unnecessary. |
| Qualcomm | We support the first two bullets, and do not support the last bullet. We also believe similar discussions were made before, and the conclusion was that the IAB-DU resource confirmation does not need to be associated with IAB-MT’s resources. Nonetheless, the intention and potential benefits of such a proposal are not even clear, and what is clear is its unnecessarily imposed limitations/complexity. |
| Intel | We are fine with the proposal in principle. However, we think the most important thing is to limit the number of soft RB sets in a slot or in a D/U/F resource type, which directly decides the DCI format 2_5 payload. |

Proposal 2.1.3a: For a DCI format 2_5 indicating availability for the soft resources of the respective RB sets corresponding to a given time resource of the child IAB-DU cell:

- Alt. 1. The legacy Rel-16 resourceAvailability table entries provided in DCI format 2_5 applied to each successive RB set in a slot.

- Alt 2. AvailabiltyCombination can be extended to include multiple resourceAvailabilty, where each resourceAvailabilty includes availability indication for one RB set.

- Alt. 3. Extend resourceAvailability mapping table so that the indication of availability can be applied to indicate multiple soft RB sets in a slot

  - FFS: One resourceAvailability element of the availabilityCombinations table is reserved to signal to the IAB-DU that “No new Availability Indication is provided” in DCI format 2_5.

  - FFS: Introducing group indexing bits in DCI format 2_5 to further increase the number of IAB-DU cells that can be indicated AI

Discussion: Views on proposal 2.1.3a?

| Company | Comments |
| --- | --- |
| ETRI | Opened for Alt.2 or Alt.3 (but cannot agree on Alt.1) From our understanding, the latest version of specification already permits two different types of soft resources (i.e. one S by Rel-17 configuration and the other S by Rel-17 configuration) at the same time. And we think availability indication on the subset of those soft resources at least from interference management perspectives. Otherwise, it will mean that Rel-16 soft resource always overrides Rel-17 soft resource (since Rel-16 soft resource includes Rel-17 soft resource in this case), which contradicts with the previous agreements. Therefore, Alt.1 should not be the way to go. Given that this is the last official RAN1 meeting for Rel-17, we think we need to agree on the extended table directly to resolve this issue in time. |
| NTT Docomo | We support Alt.2 |
| LG Electronics | We are ok with the proposal, and support Alt 2. It would be better if we can remove Alt 3 for the progress. |
| ZTE, Sanechips | Alt1 with the following updates is preferred. - Alt. 1. The legacy Rel-16 AvailabilityCombination with The the legacy Rel-16 resourceAvailability table entries provided in DCI format 2_5 is applied to each successive all the respective RB sets in a slot given time resource. |
| vivo | We support Alt.1. Alt.1 provide more flexibility to indicate resource availability per RB set, e.g., Assuming 2 RB sets are configured, when configuring 2 AvailabiltyCombination in high layer, Alt.1 can indicate 4 Availabilty patterns; while alt. 2 indicate only 2 Availabilty patterns. |
| Huawei, HiSilicon | We support Alt.1 with the understanding that the indication is applied for all RB sets in a slot. |
| Samsung | Alt.1 is preferred. In our view, the indication can be applied for all RB sets in a slot and then Rel-16 mechanisms can be fully reused. |
| Lenovo, Motorola Mobility | Alt. 1 is preferred. |
| Ericsson | Support Alt.3. Alt.1 is not practical due to the limited number of the DCI payload (128 bits). Alt.2 is not practical given the limited number of AvailabilityCombinations, (512 rows). Also, considering the discussion about restricting the number of RB sets, without such restriction it is not feasible to allow availability indication per RB set. |
| Nokia | Support Alt. 2 |
| Qualcomm | We support Alt 2. |
| Intel | Open for Alt.2 and Alt.3. Against Alt.1 |

Proposal 2.1.4a: For a DCI format 2_5 indicating availability for the soft resources of the respective RB sets corresponding to a given time resource of the child IAB-DU cell extend DCI format 2_5 payload to maximum DCI payload size.

Discussion: Views on proposal 2.1.4a?

| Company | Comments |
| --- | --- |
| ETRI | Does it mean that Rel-17 DCI format 2_5 will always have the maximum DCI payload size? |
| ZTE, Sanechips | Do not support. Since we prefer to reuse Rel-16 solutions, as the updated option 1 in 2.1.3a, the motivation and benefit of this proposal is not clear to us. |
| Huawei, HiSilicon | No support. The current maximum payload for DCI format 2_5 is 128. Even though RAN2 has over- designed the signaling to support 512 DU cells, we don’t think an IAB node will have many DU cells in practice, hence the need to extend the DCI payload size is not critical. |
| Samsung | Our preference is to keep current DCI size and share a view with HW. |
| Ericsson | In principle, we support extending the amount of information the DCI format 2_5 can provide since we believe the currently limited number of cells that can be addresses is not sufficient for future deployments. The specification allows for 512 cells and presently, only 14 cells are feasible. An enhancement would be reasonable. Enhancing the maximum payload size seems to help but will only increase the number of cells to 15 cells, why we think a more capable solution is necessary. One way to flexibly extend the number of DU cells that can be referred to without providing colliding (in time) availability indication is indicated in the two FFSs in Alt-3 in the FL Proposal 2.1.3a above. |
| Nokia | We can support the proposal. The benefit may be limited but given the added overhead for AI on each RB set, this may help reduce the number of decodes required for DCI format 2_5. |
| Qualcomm | We do not support this proposal and prefer to reuse Rel-16 framework as much as possible. |
| Intel | There is a chance we don’t need to extend the DCI 2_5 payload size if we limit the maximum # of soft RB sets in a slot or in a resource type. |

ISSUE 2.2: SPATIAL DOMAIN MULTIPLEXING

Proposal 2.2.1a: In addition to SSB ID, STC Index, and CSI-RS ID may be additionally used as the RS ID for a restricted beam indication from the parent node to the IAB node.

Discussion: Views on proposal 2.2.1a?

| Company | Comments |
| --- | --- |
| ETRI | Support the proposal. |
| NTT Docomo | We think SSB ID is sufficient. |
| LG Electronics | In general, it can be assumed that beam width used for SSB transmission is wider than beam width for CSI-RS. Hence, when SSB index is indicated as a restricted beam to the IAB node, the IAB node can select CSI-RSs corresponding to SSB index. In this sense, we think SSB ID sufficient. In addition, STC index seems not necessary. |
| ZTE, Sanechips | STC index alone cannot be used as an RS ID, I think the motivation is to say an SSB ID + an STC index can be used as an RS ID. We share similar view with NTT and LG, it is not necessary to indicated the STC index. |
| vivo | STC index is not needed, OK with SSB ID and CRI. |
| Huawei, HiSilicon | We are supportive to include CSI-RS ID but we are not sure about STC index. In addition, we think SRS can also be supported. Because IAB-DU always have a co-located IAB MT, which can transmit SRS. And the QCL relationship between SRS of IAB-MT and PDSCH of IAB-DU can be linked by IAB node. Besides, a parent node can always have the capability of receiving SRS, but it may not be able to receive SSB or CSI-RS. For example, when the parent node is a donor node. |
| Samsung | We think SSB ID is sufficient. |
| Ericsson | Support in principle. In Rel-16, SSBs were specified for inter-IAB-node measurements. It is not clear how a CSI-RS can be measured by a parent IAB-node. Similar to IAB-specific SSBs, IAB-specific CSI-RS would be required. In our understanding, an STC Index is required because the IAB-node can have up to 4 independent STC configurations, which does not make an SSB ID alone a sufficient information to resolve the mapping from RS ID to beam, i.e., what beam (in terms of direction) has been measured and reported by the parent IAB-node. Alternatively, the parent IAB-node could also indicate which of its SMTC indices has been used on parent side for an SSB measurement. We support SSB ID plus STC or SMTC index for restricted beam indication. |
| Nokia | STC is not necessary since beam indication is only provided for simultaneous operation, not SSBs used for discovery. We may agree to CSI-RS ID, but it does not seem there is a currently a method for an IAB node to provide CSI-RS configurations to a parent DU. |
| Qualcomm | We support this proposal and believe indication of STC index along with SSB index is needed. An IAB-DU cell may be configured with up to 5 STC, and effectively send up to 5 sets of SSBs (possibly with different beamforming configurations). Our question to the companies, who think STC index is not needed, is when a parent-node indicates to the IAB-DU its SSB index X is restricted, how does the IAB-DU know which exact SSB the parent-node is referring to? Because X^th SSB in each STC window could be sent toward a different beam direction. |
| Intel | STC index is not needed, OK with SSB ID and CRI. |

Proposal 2.2.2a: The restricted beam indication from the parent node to the IAB node may be indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

- Multiplexing mode [including FDM required/not required]

- Association with IAB-MT’s DL Rx beam or UL TX beam, via the parent node’s DL RS ID

- (MT CC, DU cell) pair

- Soft resources of the DU configuration

- Slot index

Discussion: Views on proposal 2.2.2a?

| Company | Comments |
| --- | --- |
| ETRI | Support the proposal in general. We think the target slot index is already included since the multiplexing mode will be configured/indicated for a specific time duration only. We think the bracket should be removed now. Maybe we can consider the following: - Multiplexing mode, at least [including FDM required/not required] |
| NTT Docomo | We agree that restricted DU beam indication is associated with multiplexing mode, MT beam, MT CC, DU cell. Meanwhile, we think restricted DU beam indication is associated only with simultaneous MT Tx/DU Tx mode. For DU resource type, we are open to discuss. But we think we also need to discuss DU behavior regarding the restricted DU beam indication (e.g. up to DU implementation, or DU cannot use the restricted DU beams). These issues are related. If up to DU implementation, we don’t think it needs to associated with any resource type. For slot index, we think it is not needed. A restricted DU beams at a given time can be determined from the association with MT beam and MT beam used at the time. |
| LG Electronics | We support the proposal 2.2.2a in general. We think multiplexing mode (i.e., sim Tx/Tx, Rx/Tx) should be indicated when the restricted beam is indicated because required restricted beam could be different depending on multiplexing mode Regarding the soft resource of the DU configuration, the applicability of the restricted beam would depend on the availability of the source resource. In our view, soft resource with availability indication should be prioritized to DU. Thus, the DU should be able to perform the desired scheduling, and the DU operation should not be interrupted by the MT operation. On the other hand, in the soft resource without availability indication, MT operation without any restriction should be guaranteed, so the restricted beam considering MT scheduling and beam resource should be applied for the DU operation. Overall, DU beam restriction is applied to the soft resource without availability indication. ‘Slot index’ in the last bullet seems not necessary. The time/frequency resource in which the DU will operate may be determined according to the multiplexing mode and the DU H/S/NA configuration. In this case, the restricted beam of the DU may be determined in consideration of the beam direction of the MT. Therefore, I think the explicit association with a slot index is not necessary. |
| ZTE, Sanechips | It is straightforward to use the slot index to indicate when the indicated beam(s) is restricted. And for the fourth sub-bullet, it is not necessary to couple restrict beam with only Soft resources. |
| vivo | We have concern on 1st and 2nd sub-bullet For 1st bullet, we understand the multiplexing mode is equal to multiplexing case, if it refer to slot where FDM/SDM is applied, then slot index is sufficient. In later proposal ‘FDM required/not required’ is also a condition as beam indication, it is complicated to associated with one condition to another. For 2nd bullet, We think restricted beam indication is only associated with simultaneous transmission, so it can be associated with UL beam. But if you say it is directly associated with SSB ID/CRI/SRI, we are not sure how to use the restricted beam, do you mean if the MT scheduling information indicate a given RS ID on a given resource, then the corresponding restricted beams is activated? We think further discussion is needed. - Multiplexing mode [including FDM required/not required] - Association with IAB-MT’s DL Rx beam or UL TX beam, via the parent node’s DL RS ID - (MT CC, DU cell) pair - Soft resources of the DU configuration - Slot index Moreover, the 4th bullet seems over-optimized, if lots of additional spec. effort is required, we think the 4th bullet can be removed. |
| Huawei, HiSilicon | More discussion needed. For the multiplexing mode, we think the restricted beam indication is intended for simultaneous TX, but there is no need to include whether FDM is required or not from parent node to IAB node. It can be included a condition reported by IAB node but not other way around since the parent node can schedule the IAB-MT in the given resources where FDM H/S/NA resources are configured without the need to inform the IAB node. There is no need to associate the restricted beam indication to IAB-MT’s DL Rx beam since this is for simultaneous Tx. The “slot index” seems already cover the “Soft resources of the DU configuration” hence “Soft resources of the DU configuration” can be removed. |
| Samsung | We have the following comments. Multiplexing mode: we think beam restriction is associated with simultaneous TXs. Association with beam: we think DL RX beam is not needed for simultaneous TXs |
| Lenovo, Motorola Mobility | Support the main line and bullet items 2, 3, 5. Do not support “multiplexing mode” as it is not defined. Soft resources of the DU configuration should be justified. |
| Ericsson | We support restricted beam indication per (MT CC, DU cell) pair and in association to UL TX beam, via the parent node’s DL RS ID. We do not support Multiplexing mode association, since only the Case-A operation (Tx-Tx) is of relevance. Please note that the restricted beam is an IAB-DU Tx beam and in case of Case-C (MT-Rx DU-Tx) there is not impact on the parent IAB-node. The application of the restricted beam is only relevant to IAB-DU Soft resources. Therefore, it does not need to be indicated additionally. If a restricted beam is associated to a slot index, we do not need association to DL Rx or UL Tx beam or CC/cell pair. Since association to UL Tx beam has already been agreed, it seems that slot index association becomes overhead. |
| Nokia | Beam restrictions provided by the parent DU should only be applied to semi-static soft resources, since otherwise, it can be assumed that the CU is configuring the resources, and unaware of beam indications communicated between the IAB node and the parent DU. Additionally, slot index may also be included since relevant modes of operation may change from slot to slot. |
| Qualcomm | We partially agree with the proposal and have the following comments/questions. - We share the same view that restricted beam indication may be more relevant to simultaneous TX. However, we believe the signaling should be flexible enough to be applicable to other multiplexing modes such as MT-RX/DU-TX. We did agree not to do any special optimizations/treatments to a subset of Rel-17 enhanced multiplexing modes (namely those related to “full-duplex” operation) and use other legacy/new features (defined primarily for other purposes) when applicable. Now, completely ignoring these modes, when there is a low-cost opportunity to have a more flexible and future-proof feature, is not advised and is essentially against the Rel-17 eIAB WI scope. - The first bullet needs clarification. “FDM required/not required” is a capability and not a multiplexing mode. In our view, “FDMed/not-FDMed” is a better term to refer to the cases where simultaneous MT’s and DU’s signals are non-overlapping or overlapping in the frequency-domain. It should be clear that the two scenarios may have different limitations, and it makes sense to support indication of different associated configurations. “Soft resources of the DU configuration” requires clarification. Is the intention to indicate the restrictions can be associated with soft resource or not? And if not, does it mean the restriction should be applied to all resources (hard and soft)? But, as pointed out by others, an IAB-DU should have flexibility/priority over its hard resources, and the parent-node should not impose any restriction over IAB-DU’s hard resources. On the other hand, if this is to be adopted, it needs to be clarified how the frequency-domain soft resources should be treated. |

Proposal 2.2.3a: The recommended beam indication from the IAB-MT to the parent node are provided using one or more of the following

- For DL Rx beam(s)

  - DL TCI state ID, or

  - RS ID: SSB ID and/or CSI-RS ID

- For UL Tx beam(s)

  - SRI ID, or

  - RS ID: SSB ID and/or CSI-RS ID

Discussion: Views on proposal 2.2.3a?

| Company | Comments |
| --- | --- |
| ETRI | Support the proposal. |
| NTT Docomo | We support RS ID for both DL Rx beam and UL Tx beam. |
| LG Electronics | Because we think using one ID is enough rather than using multiple IDs for recommendation of beam, it seems an intention of this proposal to use one or more ID (i.e., RS ID and DL TCI state ID for DL Rx beam, RS ID and SRI ID for UL Tx beam) is not clear. We think CSI-RS ID is appropriate for indication of DL Rx beam(s) because network may use MT specific narrow beam for MT specific data transmission, and the MT may select matched DL Rx beam corresponding the CSI-RS resource. And we think SRI ID is feasible for indication of UL Tx beam(s) because SRI is directly corresponded to MT UL Tx beam. |
| ZTE, Sanechips | From our point of view, TCI state ID for DL and SRI ID for UL is sufficient, we can also live with RS ID if the majority view is to support it. |
| vivo | Support the proposal, at least SRI/SSB ID/CRI is used for UL beam indication, which is used for configuring UL spatial relation info. |
| Huawei, HiSilicon | For DL Rx beams, we support DL TCI ID since it covers both SSB and CSI-RS. For UL Tx beams, we support SRI because the PUSCH transmission will eventually be associated to SRI. |
| Samsung | Our preference is DL TCI state ID for DL Rx beam and SRI ID for UL Tx beam. |
| Ericsson | We prefer RS ID for both DL Rx beam and UL Tx recommended beam indication. Both SSB and CSI-RS are acceptable to us. |
| Nokia | We believe there may be some value in using a common signaling framework with power control indication, especially given working assumption considerations on when non-TDM multiplexing may be used. |
| Qualcomm | We share the same views as Huawei and Samsung. However, we are also OK with the majority view to get this agreed and progress. |

Proposal 2.2.4a: The recommended beam indication from the IAB node to the parent node may be indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

- Multiplexing mode [including FDM required/not required]

- (MT CC, DU cell) pair

- Soft resources of the DU configuration

- Slot index

Discussion: Views on proposal 2.2.4a?

| Company | Comments |
| --- | --- |
| ETRI | Support the proposal in general. We think the target slot index is already included since the multiplexing mode will be configured/indicated for a specific time duration only. We think the bracket should be removed now. Maybe we can consider the following: Multiplexing mode, at least [including FDM required/not required] |
| NTT Docomo | We agree that recommended beam indication is associated with multiplexing mode, MT CC, DU cell. For DU resource type, we think it is not needed. With the recommended beam indication, how parent node configure MT beam is up to parent node. For slot index, we think it is not needed. |
| LG Electronics | Similar to our comment in Proposal 2.2.2a, the recommended beam should be different for soft resource with availability indication and without availability indication. In the soft resource without availability indication, the MT can operate without restrictions, whereas the operation of MT should not interfere the operation of the DU in the soft resource with availability indication. Therefore, we think that the recommended beam is for soft resource without availability indication. In addition, by applying the recommended beam to the hard resource, DU hard resource also can be utilized for SDM between the DU and the MT without causing a problem in the operation of the DU. We don't think the ‘slot index’ is necessary as commented in Proposal 2.2.2a. |
| ZTE, Sanechips | It is straightforward to use the slot index to indicate when the indicated beam(s) is recommended. For the second sub-bullet, it is not clear how the parent node can determine the recommended IAB MT beam at a given time if it is provided per (MT CC, DU cell) pair, since the parent node may not know which DU cell is operated at the IAB node at a given time. And for the third sub-bullet, it is not necessary to couple restrict beam with only Soft resources. |
| vivo | We have concern on 1st and 3rd sub-bullet For 1st bullet, why we support so many time domain information, slot index can be used instead of the slot of FDM/non-FDM. For 3rd bullet, if DU and MT share panel, the beams used for MT operation is common regardless of the resource type |
| Huawei, HiSilicon | On the first bullet, we share a similar view with ETRI that the bracket can be removed since this is condition that the IAB node would know while the parent node does not. For the last two bullets, we think they are not needed since the recommendation from the IAB node to its parent node on what conditions are required to operate a certain multiplexing mode does not really need to include the time domain resources. Since they can be applied on any type of time domain resources. |
| Samsung | Share a view with HW regarding the last two bullets. |
| Lenovo, Motorola Mobility | Support the main line and bullet items 2, 4. Do not support “multiplexing mode” as it is not defined. Soft resources of the DU configuration should be justified. |
| Ericsson | Support in general. Similar to restricted beams, if a recommended beam is associated with a slot index, we do not need association to multiplexing mode or resource configuration. We can agree to either option but don’t think both should be specified. To align with functionality for restricted beams, we tend to prefer multiplexing mode to slot index. |
| Nokia | Similar to discussions on beam restriction we feel that beam preference should be associated with soft resources for a MT CC, DU cell for a given slot index. |
| Qualcomm | We have similar comments as in proposal 2.2.2a. We reiterate it is important to distinguish cases/slots where the semi-static resource configurations at the IAB-DU and parent-node are such that the MT’s and DU’s signals are non-overlapping in the frequency-domain (so called “FDMed”), and those where the MT’s and DU’s signals may overlap in the freq-domain (so called “not-FDMed”). Different conditions/limitations may be applied to these two cases. For the indication of the recommended beams, we do not think “slot index” or “soft resources of the DU configuration” is needed. |

Proposal 2.2.5a: The maximum number of recommended/restricted beams in a given indication is X.

- X=[4].

Discussion: Views on proposal 2.2.5a?

| Company | Comments |
| --- | --- |
| ETRI | Support the proposal. |
| NTT Docomo | Support in principle. We think we should separate the discussion of number of recommended MT beams and restricted DU beams because the number can be different e.g., X1/X2. And we should make it clear whether the maximum number of recommended MT beams/restricted DU beams in per MT cell/per DU cell. And we are fine to further discuss the number. |
| ZTE, Sanechips | Agree in principle. |
| Huawei, HiSilicon | We are fine with the proposal but have a preference to a slightly larger number. |
| Samsung | Fine with the proposal and open to discuss exact number |
| Ericsson | Support in general although we think more restricted beams may be needed than recommended beams. We propose 8 restricted beams. We also note there is a relation between recommended and restricted beams, if both features are used, in that restricted beams will depend on which recommended beam is being used. |
| Nokia | We are fine with this proposal. |
| Qualcomm | We agree with Docomo’s comment. Additionally, we should clarify the following aspects to avoid later ambiguities in defining the related MAC-CEs. - Does one indication comprise a list of up to X beams and a single set of configurations to be commonly associated with all the beams in the list? - For the set of configurations, shall we support a list of values for each configuration? i.e., a list of slot indices, or a list of (MT CC, DU cell) pairs can be indicated? |

Proposal 2.2.6a: Explicit availability indication from a parent node overrides beam restrictions in Soft resources at the child IAB node.

Discussion: Views on proposal 2.2.6a?

| Company | Comments |
| --- | --- |
| ETRI | Do not support. We think such mismatch is a misconfiguration/indication from parent node side. |
| NTT Docomo | We are open to discuss it. But similar as we commented in 2.2.2a, we think we also need to discuss DU behavior regarding the restricted DU beam indication (e.g. up to DU implementation, or DU cannot use the restricted DU beams). If up to DU implementation, we think this proposal is not needed. |
| LG Electronics | DU beam restriction should not be applied for soft resource with availability indication. Soft resource with availability indication should be prioritized to DU. Thus, DU should be able to perform the desired scheduling, and the DU operation should not be interrupted by the MT operation. |
| ZTE, Sanechips | More clarification is needed, the motivation for this proposal is not clear to us. And from our point of view, indicating restricted beam for CLI management should be also considered, overriding beam restrictions only based on AI may be not suitable. |
| vivo | We do not see much benefit to do so. |
| Huawei, HiSilicon | Not support. We think defining such a rule is not needed considering frequency domain H/S/NA configuration. An IAB DU configured with soft resources in frequency domain may still need the beam restriction to operate FDM even when the soft resources in certain RB sets are indicated as available by its parent node. Therefore, whether to apply the beam restriction in the soft indicated as available resources can be handled by IAB node by implementation. |
| Samsung | Tend to agree that the beam restriction on soft resource indicated as available is up to IAB DU implementation. |
| Lenovo, Motorola Mobility | Do not support. Parent node handles both signaling and the IAB node follows. No need for this rule. |
| Ericsson | Agree. Our understanding is that only Soft resources are used for SDM. To align with other use of Soft-IA resources, the IAB-DU should be given unconditional access to those resources. That includes not having any spatial constraints on such resources. |
| Nokia | We understand this proposal to mean that soft resources indicated as not available for an IAB DU may be assumed to be available except on beams indicated as restricted. In general, we may be supportive of this proposal, but there are still issues that must first be addressed, such proposals related to when non-TDM operation is allowed, and what an IAB node may do if unable to transmit at its indicated power level. |
| Qualcomm | We agree with the intent, that perhaps can be captured differently such as “The IAB-DU is expected to apply the indicated beam restriction within its soft resources that are not explicitly indicated as available. It is up to IAB-node’s implementation whether to apply the restricted IAB-DU beams within IAB-DU hard and soft resources that are explicitly indicated as available.” |

ISSUE 2.3: ADAPTATION OF SIMULTANEOUS OPERATION

Proposal 2.3.1a: An IAB node (or parent node) cannot operate under a given non-TDM multiplexing mode until (downselect one of the following alternatives):

- Alt. 1: All required conditions and parameters which have been directly indicated/requested to the parent node (e.g. via MAC-CE) are explicitly acknowledged by the parent node.

- Alt. 2: All required conditions and parameters which have been directly indicated/requested to the parent node (e.g. via MAC-CE) are implicitly acknowledged by the parent node or implicitly determined at the child node

Discussion: Views on proposal 2.3.1a?

| Company | Comments |
| --- | --- |
| ETRI | Support the proposal. Since this is the last meeting, we need to analyze the outcome from lack of details on this issue. We assume that if RAN1 cannot make a clear decision on this issue, then it implies that the IAB node can operate the configured/indicated non-TDM multiplexing mode at any time after the IAB node decodes the configuration/indication on that non-TDM multiplexing mode, unless any collision between the configurations / indications / requests is clarified. In other words, the misaligned configurations / indications / requests on the simultaneous operations will be an implementation issue at the parent node in this case. Introducing a minimum time duration (or a prohibit time) of the report on the required conditions/parameters can be an alternative for this issue, we think. Aligning the periodicity of “report from the IAB node” and “configuration/indication from the parent node” (e.g. by the parent node) would be a way to minimize such collision / ambiguity of simultaneous operations. |
| NTT Docomo | Support in general. But it is not very clear to us what “explicit acknowledge”/ “implicit acknowledge” means. We are fine with this proposal and further discuss the details. |
| ZTE, Sanechips | Share similar view with NTT Docomo, further discussion/clarification may be needed for this proposal. |
| vivo | At least for the case that DU and MT share RF/or IAB use some advanced receiver, the timing mode and PSD balancing should be required to apply the multiplexing case A and case B. so, timing case indication, and power control parameter indication should be met to enable case A and case B. However, beam indication is not mandatory. In some other case, e.g., DU and MT use different panel, any condition(s) is not mandatory to enable a given multiplexing case. Then Rel-16 non-optimized simul. operation is performed. We are fine with the acknowledge of each request/indicated condition, either explicit or implicit. But shall we discuss how to acknowledge, e.g., via indication of multiplexing case, via ACK, via indication transmission parameters (power, timing…) |
| Huawei, HiSilicon | Need more details related to “implicit and explicit” to perform the down selection. In addition, this proposal can be discussed after the conclusion of Proposal 2.1.1a. |
| Samsung | Support |
| Lenovo, Motorola Mobility | Do not support. “non-TDM multiplexing mode” is not defined. |
| Ericsson | Share view with Docomo and ZTE. We think both options may be needed since not all requests may be either explicitly or implicitly acknowledged. To be efficient, we can agree that both implicit and explicit may be allowed. |
| Nokia | We are generally supportive of Alt. 2 as it seems preferred that decision to operate in non-TDM mode should be under the control of the IAB node without explicit indication. |
| Qualcomm | We do not agree with this proposal. First, we are not clear about the implication of such a proposal. At the end, an IAB-node may or may not/can or cannot operate in a non-TDM multiplexing mode based on several conditions, configurations, scheduling decisions, etc. Why do we need an agreement for this aspect? We suspect, as Huawei said, this may be related to Proposal 2.1.1a, and that is exactly one of our concerns, mentioned before, i.e., Proposal 2.1.1a has major issues and cannot work without extra signaling/specification work. And this proposal is possibly an example of such extra work, where itself has issues: - Even if the parent-node does not grant the desired/recommended configurations, indicated by the IAB-node, the IAB-node may still be able to support an enhanced multiplexing (no-TDM) mode, possibly with some limitations and/or reduced performance (e.g., it can reduce its MCS on the child BH links). - The indications of desired/recommended configurations, by the IAB-node to the parent-node, are optional. In case the IAB-node does not directly indicate/request anything, the parent-node does not necessarily know whether the IAB-node can or cannot support an enhanced multiplexing (non-TDM) mode. |

Proposal 2.3.2a: The required conditions for a given non-TDM multiplexing mode include the following:

- Desired/provided guard symbols

- [Timing case indication (Case 1, Case 6, Case 7)]

- DL Tx power adjustment

- Desired UL PSD range

- Restricted beam indication

- Recommended beam indication

- [FDM required/not required]

Discussion: Views on proposal 2.3.2a?

| Company | Comments |
| --- | --- |
| Moderator (AT&T) | Note: All contents not in brackets are already agreed as separate MAC-CE signaling (do not need to be revisited) |
| ETRI | Support the proposal. |
| NTT Docomo | Support. And we think the timing case indication and FDM required/not required are needed. |
| LG Electronics | We are ok with the proposal. We think timing case indication is necessary. |
| ZTE, Sanechips | The Timing case indication should be additionally supported. Further discussion on whether [FDM required/not required] is needed. |
| vivo | (copy comment as proposal 2.3.1a) At least for the case that DU and MT share RF/or IAB use some advanced receiver, the timing mode and PSD balancing should be required to apply the multiplexing case A and case B. so, timing case indication, and power control parameter indication should be met to enable case A and case B. However, beam indication is not mandatory. In some other case, e.g., DU and MT use different panel, any condition(s) is not mandatory to enable a given multiplexing case. Then Rel-16 non-optimized simul. operation is performed. |
| Huawei, HiSilicon | Further clarification needed. The conditions can be different for different multiplexing case. For timing case indication, we are not sure why case 1 and case 7 timing are needed if the case 1 timing and case 7 timing here means the DU Rx timing. |
| Samsung | Support in principle. Bracket for Timing case indication can be removed. |
| Lenovo, Motorola Mobility | Non-TDM multiplexing mode is not defined. |
| Ericsson | We share the view that timing mode should be reported to the parent node. FDM required should be reported to the donor CU. |
| Nokia | Even though MAC-CE’s have been agreed for indication of the listed parameters, it is unclear what the implications of this proposal are. For instance, it is unclear how recommended beam indication is understood in this context. It does not seem that a recommended beam indication from an IAB node to a parent DU is necessary for non-TDM operation. Additionally, DL Tx power adjustment does not seem necessary non-TDM operation and has already been agreed as optional. |
| Qualcomm | The intention of this proposal is not clear. If the intention is to additionally support the conditions in the brackets, that are not yet agreed, then we should directly have separate proposals to define the required signaling for each of them. If the intention is somehow to link this proposal to the proposal 2.3.1a, we already made a comment that we do not agree with proposal 2.3.1a and hence this proposal may not stand by itself. Further to our comments to 2.3.1a, it should be noted again that some of these conditions are called “desired” or “recommended” for a good reason that, in our view, they may not be “required”. |

Proposal 2.3.3a: The MAC-CE signaling of Desired/Provided Guard Symbols is enhanced to optionally indicate the number of guard symbols required for switching between the additional following cases:

- Case #6 MT TX -> Case #7 DU RX

Discussion: Views on proposal 2.3.3a?

| Company | Comments |
| --- | --- |
| NTT Docomo | Do not support. We think [case#7] should be removed. If the intention is different DU Rx timing may result in different guard symbol, we also need case#6 MT Tx -> case#1 DU Rx, case#6 MT Tx -> case#6 DU Rx. However, we think DU Rx timing modes do not need to be differentiated and it is not useful because parent node will not know which timing mode is used by DU Rx at a given time. |
| LG Electronics | We don’t support the proposal. |
| vivo | Fine with any additional guard symbol. |
| Huawei, HiSilicon | Not support. We share similar view with DCM that the parent node is not aware of the UL timing case of IAB-DU, and thus the reporting is not needed. |
| Samsung | Support. We think differentiation of DU Rx timing mode may be useful for parent node because it may be related to symbols for DL power control from parent node for MT Rx. |
| Ericsson | It is a bit unclear to us what cases have been agreed considering some alternatives are within brackets. We think all of the following should be considered: - Case #6 MT TX -> Case #7 DU RX - Case #6 MT TX -> Case #1 DU RX Case #7 DU RX -> Case #6 MT TX |
| Nokia | Support the proposal. |
| Qualcomm | We do not support this proposal, for the same reasons mentioned by Docomo and Huawei. |

Proposal 2.3.4a: Support negative values of Ng for signaling of Desired/Provided Guard Symbols.

Discussion: Views on proposal 2.3.4a?

| Company | Comments |
| --- | --- |
| NTT Docomo | Do not support. similar as in Rel-16, if guard symbol is not required, the value can be set to 0. Negative value is not needed. |
| LG Electronics | We support the proposal to reduce the resource waste. The problem of resource waste due to timing misalignment would be more significant for timing case #6 and #7. |
| ZTE, Sanechips | Do not support, share similar view with NTT. |
| vivo | Share view as DCM. |
| Huawei, HiSilicon | Support. The timing difference (T_m) between IAB-MT and IAB-DU can be quite large in some case; therefore, negative guard symbol can be used to increase the resource utilization. An example is shown in figure below. |
| Samsung | Our preference is to keep current value range with 0 as a minimum because it seems not that critical. |
| Ericsson | Disagree |
| Nokia | Although this may be feasible, more consideration is necessary, and it does not appear to be necessary for enhanced multiplexing operation. Additionally, it increases the scope of study on guard symbol enhancement too significantly for the current Rel-17 schedule. |
| Qualcomm | We do not support to this proposal. The proposal may offer some optimization in terms of resource utilization. However, spending time in the very last RAN1 meeting to specify unnecessary aspects is not advised. |

Proposal 2.3.5a: An IAB-MT is provided with a Timing Case Indication via MAC-CE that explicitly indicates a list of slots and their associated UL TX timing cases (i.e., one of {Case 1, Case 6, Case 7} for each slot). This indication is associated with some combination (one or multiple) of the following IAB-node’s configurations:

- Multiplexing mode [including FDM required/not required]

- MT’s UL beam [via SRI ID]

- (MT CC, DU cell) pair

Discussion: Views on proposal 2.3.5a?

| Company | Comments |
| --- | --- |
| NTT Docomo | We think UL Tx timing case can be indicated in each UL Tx scheduling DCI so that the UL Tx timing case can be dynamically decided and indicated by parent node. But we can be OK to accept majority view on this issue. |
| LG Electronics | We are fine with the proposal in principle. However, could you elaborate the reason that the timing case indication is associated with MT's UL beam. In our view, the indicated UL Tx timing should be applied in common regardless of MT’s UL beam. |
| ZTE, Sanechips | We support the main bullet that a list of slots and their associated UL TX timing cases are explicitly indicated. But for the sub-bullets, we think they are not needed. |
| vivo | Case 6 timing is associated with multiplexing case A, at least we would say if case 6 timing is indicated, then multiplexing case A is enabled, or say if multiplexing case A is indicated, then case 6 timing is assumed by IAB node. Otherwise, why we need this proposal. This proposal is tight related with 2.3.1a, can be discussed after it, e.g., how to acknowledge the required condition. Moreover, we are confused with the 1st and 2nd bullet, which should be clarified. |
| Huawei, HiSilicon | We think the multiplexing mode is needed but “FDM required/not required” is not needed since this is the decision at the IAB node. The last two bullets are also not needed since MT UL beam is indicated in DCI and there is no need to differentiate among different DU cells for the same MT CC. |
| Samsung | Support the main bullet and multiplexing mode in the first sub-bullet. |
| Lenovo, Motorola Mobility | Support the main line and bullet items 2, 3. Do not support “multiplexing mode” as it is not defined. |
| Ericsson | We support the main part of the proposal but, similar to ZTE, we don’t think the three listed parameters are needed. Depending on an IAB node’s capability and resource configuration, it and its parent node can conclude the timing mode. For Case-6 operation, an IAB node should be able to request this timing mode from its parent node for a list of slots, potentially ACK/NACKed. For Case-7 operation, considering IAB-DU seniority to its child MT, the child MT must follow the configuration of the IAB node provided the child MT has indicated Case-7 capability. |
| Nokia | Support for case #6 and case #7 timing does not seem to require dynamic indication for its use, rather the use of case #6 and case #7 timing should be used only with the relevant non-TDM mode of operation and this should be managed by semi-static configuration. |
| Qualcomm | We support the main clause. There is no need to further associate the indicated timing cases to other configurations. The parent-node explicitly indicates which case to use for each slot, and parent-node presumably makes such a decision already considering the other listed configurations. |

Proposal 2.3.6a: A Timing Case Indication received from a serving cell is applicable to all other cells in the same timing advance group (TAG).

Discussion: Views on proposal 2.3.6a?

| Company | Comments |
| --- | --- |
| LG Electronics | We are ok with the proposal. |
| ZTE, Sanechips | Agree in principle. |
| Huawei, HiSilicon | Not support. We think the parent node can indicate the timing mode for each MT CC, and thus the rule is not necessary and may be detrimental. For example, there may be cases that the IAB node adopt different timing case for cells in same TAG. |
| Samsung | Not necessary since it is too much restriction in our view. |
| Lenovo, Motorola Mobility | Not necessary. |
| Ericsson | Agree |
| Nokia | Support the proposal. |
| Qualcomm | We support this proposal, and believe this is necessary to comply with the legacy spec and concept of TAG: “[38.213] Upon reception of a timing advance command for a TAG, the UE adjusts uplink timing for PUSCH/SRS/PUCCH transmission on all the serving cells in the TAG based on a value that the UE expects to be same for all the serving cells in the TAG and based on the received timing advance command where the uplink timing for PUSCH/SRS/PUCCH transmissions is the same for all the serving cells in the TAG.” |

Proposal 2.3.7a: Support indication (i.e. via Multiplexing Info IE in 38.473) of whether FDM is required or not for an enhanced multiplexing operation mode to donor-CU and/or parent node.

Discussion: Views on proposal 2.3.7a?

| Company | Comments |
| --- | --- |
| ETRI | Support the proposal. |
| NTT Docomo | Support |
| LG Electronics | We are fine with the proposal. |
| ZTE, Sanechips | Agree in principle. |
| Huawei, HiSilicon | We think whether FDM is required or not can defined as conditions contained in MAC-CE because whether FDM is required or not is not a semi-static capability. |
| Ericsson | Agree, send LS to RAN3. |
| Nokia | The purpose of this proposal is unclear. Is the purpose to indicate that an IAB node is incapable of SDM operation? It doesn’t seem that this should be necessary for proper operation. |
| Qualcomm | We support this proposal. |

# Resource allocation for dual-connectivity scenarios (i.e. IAB-MT with concurrent BH links with two parent nodes)

From the eIAB WID:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx)

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

Summary of input contributions:

| Nokia, Nokia Shanghai Bell R1-2110775 | Proposal 3.1: A donor CU may provide the intersection of all per-child NA resource configurations of an IAB node to the parent DU of the IAB node. Proposal 3.2: Draft an LS to RAN 3 informing them of Rel-17 specification enhancements with RAN 3 impact. |
| --- | --- |
| Huawei R1-2110834 | Proposal 12: Add per-child MT link-NA resource configuration into higher layer parameter list. |
| vivo R1-2111033 | Proposal 19: For an IAB MT in DC, the mode 2 semi-static power control is performed based on both TDD configuration and per-child MT link-NA resource configuration. Proposal 20: For an IAB MT in DC, when performing mode 2 semi-static power control, when e, if NA is assumed for overlapped UL symbol, the maximum transmission power of the UL transmission is restricted by PCMAX; Otherwise, restricted by configured value PMCG or PSCG. |
| Lenovo, Motorola Mobility R1-2111952 | Proposal 17: Support signaling from an IAB node in the DC mode to its parent node for informing the parent node of the status of availability of soft resources. Further discuss specification of mechanisms to handle availability indication collisions by two parent nodes in the DC mode. Proposal 18: RAN1 to discuss resource conflicts in the DC mode due to conflicting Case-7 timing. |
| LG Electronics R1-2111983 | - Dual-connectivity to support multiple parent DUs Proposal 16: the conflict of symbols configured as semi-static flexible by one parent node but not by the other in inter-donor DC scenarios is left as a network configuration error case without specification impact. Proposal 17: Discuss how to handle the case where the MT cannot perform simultaneous Tx/Tx or simultaneous Rx/Rx for two serving cells due to timing misalignment in CA/DC environment. |
| ETRI R1-2111995 | Proposal 19: Regarding the two IAB MT behaviors on D-U direction conflict in DC scenarios, explicitly clarify both in the specifications. |
| Ericsson R1-2112356 | Proposal 26 For directional collision handling in IAB inter-donor and intra-donor DC scenarios, introduce the following capabilities and RRC parameters • simultaneousRXTXIABInterDonorDC capability • simultaneousRXTXIABIntraDonorDC capability • half-DuplexTDD-IABNRDC-r17 capability Proposal 27 For directional collision handling in IAB inter-donor and intra-donor DC scenarios, RAN1 decides whether or not a new RRC signal should be introduced: Alt1: reuse the Rel-16 RRC parameter directionalCollisionHandling-r17 Alt2: introduce the new RRC parameter IABDCdirectionalCollisionHandling-r17 |

Proposal 3.1.1a: A donor CU may provide the intersection of all per-child NA resource configurations of an IAB node to the parent DU of the IAB node.

Discussion: Views on proposal 3.1.1a?

| Company | Comments |
| --- | --- |
| NTT Docomo | we think it is not necessary. Legacy per DU resource configuration provided to parent node is sufficient. |
| LG Electronics | We have the same view with NTT Docomo. |
| ZTE, Sanechips | The benefit of the additional signalling is not justified. |
| vivo | The agreement of per-child NA configuration is not clear. In our reading of the agreement, the per-child NA is informed to parent node and IAB node, IAB node is the one in DC. Before discussing this proposal, the agreement should be clarified. |
| Huawei, HiSilicon | We share similar view with DCM. |
| Lenovo, Motorola Mobility | Not necessary. Agree with NTT Docomo’s comment. |
| Ericsson | Disagree. The fact that an IAB-DU resource is Not Available for a certain link (to a child IAB-MT) does not change the principal operation of the IAB-DU (e.g., to UEs). It is therefore of no relevance for the parent IAB-node in supporting (e.g., by providing DCI 2_5) resource coordination between the IAB-MT and IAB-DU. Hence, the additional signaling is not justified. |
| Qualcomm | We do not support this proposal. This effectively means info about IAB-DU resources that are NA for all the child BH communications is provided to the parent-node. However, those resources could still be hard or soft and be available to the IAB-DU to serve its UEs. Hence, the indication of this info to the parent-node does not seem to offer any real benefit, nor resulting in a meaningful expected behavior at the parent-node. |

Proposal 3.1.2a: For an IAB-MT in DC operation, the mode 2 semi-static power control is performed based on both TDD configuration and per-child MT link-NA resource configuration (if provided).

Discussion: Views on proposal 3.1.2a?

| Company | Comments |
| --- | --- |
| NTT Docomo | it seems not critical |
| ZTE, Sanechips | Agree with NTT, it seems an optimization issue and not critical. |
| Vivo | The parent per-child MT NA can be provided to child IAB node operating in DC. In case of UL-H/UL-H slot for MCG/SCG, then no change to DC power sharing. In case of UL-H/UL-NA slot for MCG/SCG, then UL transmission power of a leg can reach Pcmax. Which is beneficial for power control. |
| Samsung | Our preference is to just follow current specifications for power control without any further discussion. Maybe, whether or not mode 2 semi-static power control only is up to NW choice. |
| Ericsson | Agree with Docomo and ZTE. |
| Qualcomm | We see the benefit of this straightforward proposal. However, since this is an optimization, we are OK to not spend additional time on it, if the majority view does not support the proposal. |

Proposal 3.1.3a: Specify handling of case(s) where the IAB-MT cannot perform simultaneous Tx/Tx or simultaneous Rx/Rx for two serving cells due to conflicting timing alignment with the parent nodes.

Discussion: Views on proposal 3.1.3a?

| Company | Comments |
| --- | --- |
| NTT Docomo | We are fine to discuss this issue. |
| LG Electronics | We support the proposal. |
| ZTE, Sanechips | Do not support. It can be handled by proper network implementation, and the proposal seems to propose specify an open issue without solutions, it is better not to discuss such issue at this late stage of Rel-17. |
| Huawei, HiSilicon | Do not support. We are not sure whether there is a sufficient time to address the support of simultaneous operation and DC at the same time at this late stage of the WI. |
| Samsung | Issue is unclear. If the timing mis-alignment is caused by different timings between two serving cells (e.g., Case 1 for serving cell 1, Case 6 for serving cell 2), it can be concluded that the combination of simultaneous operation and dual connectivity is not considered in Rel-17. |
| Lenovo, Motorola Mobility | Support |
| Ericsson | It is not clear to us what timing alignment is conflicting, i.e., if it is related to (MT/DU or MT’s CCs) simultaneous operation and what is IAB specific in this problem since similar problems arise in legacy-DC, CA or multi-TRP. We think simultaneous (MT/DU) operation in DC should be down prioritized. |
| Nokia | It is not clear why this would be a concern for inter-carrier DC operation. |
| Qualcomm | Realistically there is not time left to open up a new aspect for discussions, and hence we do not support this proposal. |

# Summary

Proposed Agreement: Support indication (i.e. via Multiplexing Info IE in 38.473) of whether FDM is required or not for an enhanced multiplexing operation mode to donor-CU and/or parent node.

Proposed Agreement: An IAB-MT is provided with a Timing Case Indication via MAC-CE that explicitly indicates a list of slots and their associated UL TX timing cases (i.e., one of {Case 1, Case 6, Case 7} for each slot).

- Alt. 1 This indication is associated with some combination (one or multiple) of the following IAB-node’s configurations:

  - (MT CC, DU cell) pair

- Alt. 2. A Timing Case Indication received from a serving cell is applicable to all other cells in the same timing advance group (TAG).

Proposed Agreement: The maximum number of non-overlapping RB sets configurable per DU cell is M

- where, M=[4]

- In case of M > 1, DU carrier information should be provided to the parent node.

Proposal Agreement: The required conditions for a given non-TDM multiplexing mode include the following:

- Desired/provided guard symbols

- [Timing case indication (Case 1, Case 6, Case 7)]

- DL Tx power adjustment

- Desired UL PSD range

- Restricted beam indication

- Recommended beam indication

- [FDM required/not required]

Proposed Agreement: An IAB node (or parent node) cannot operate under a given non-TDM multiplexing mode until:

- Alt. 1: All required conditions and parameters which have been directly indicated/requested to the parent node (e.g. via MAC-CE) are explicitly acknowledged by the parent node.

OR

- Alt. 2: All required conditions and parameters which have been directly indicated/requested to the parent node (e.g. via MAC-CE) are implicitly acknowledged by the parent node or implicitly determined at the child node.

Proposed Agreement: Confirm the following RAN1#106bis-e working assumption:

Working Assumption

If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected:

- Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.