3GPP TSG RAN WG1 #107bis-e	R1-2200125

e-Meeting, January 17th – 25th, 2022

Source:	Fujitsu

Title:	Considerations on partial sensing and DRX in NR Sidelink

Agenda item:	8.11.1.1

Document for:	Discussion and Decision

Introduction

During the RAN1#106-e and RAN1#107-e meetings, the following agreements have been achieved w.r.t partial sensing schemes. For contiguous partial sensing (CPS), the following agreement was made.

Agreement When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n, TA and TB for the CPS monitoring window is defined according to one of the followings: - n+TA is M logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. - By default, M is 31 unless (pre-)configured with another value.

Furthermore, when resource (re)selection procedure is triggered by aperiodic transmission (Prsvp_TX=0), the Approach 1 from last meeting is finally agreed in RAN1#107-e meeting but there are still some FFSs need to be solved in future meeting.

Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, the general design framework in Approach 1 from RAN1#106bis-e in below is adopted. Note that, the details can still be updated. - Approach 1: (SA is initialized based on at least slots with PBPS and/or CPS results and guarantee a minimum of M slots for CPS) - The UE selects a set of Y’s’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - FFS how to handle the case if the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min without dropping the aperiodic transmission - FFS whether the Y’ candidate slots for aperiodic transmission is the same as the Y candidate slots in PBPS for periodic transmission of another TB(s) - FFS whether/how to prioritize/select resources based on partial sensing results. - FFS: How to select Y’ in case of CPS only - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results for a minimum of M consecutive logical slots before ty0, where ty0 is the first slot of the selected Y’ candidate slots. - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority - FFS the range of (pre-)configured M from a TBD lowest value up to 30 - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed - FFS: RSW in case of CPS only Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - If the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min, - How UE includes other candidate slots is up to UE implementation - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results starting at M consecutive logical slots before ty0 and ending at Tproc,0 + Tproc,1 slots earlier than ty0. - FFS: By default, M is 31 unless (pre-)configured with another value, or where M is (pre-)configured based on transmission priority - FFS: The range of (pre-)configured M from a TBD lowest value up to 30 - When the minimum M slots for CPS cannot be guaranteed, support both - Option A, the UE ensures the Y’min criterion is fulfilled - Option B: UE performs random resource selection - When the UE performs Option A or Option B is up to UE implementation

Also, the following agreements have been further achieved for PBPS and CPS based re-evaluation/pre-emption.

Agreement When UE is triggered to perform re-evaluation and pre-emption checking for periodic transmission (Prsvp_TX≠0) in slot n, - During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starts from slot tyiSL and ends at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to [q x Prsvp_Tx + tySL], where tySL is a slot index of Y candidate slots used in the initial resource (re)selection. - tyiSL is the first candidate slot after slot n+T3. - FFS whether/how to handle the case when number of the remaining Y candidate slots is less than Ymin. - Scheme 1: - UE performs PBPS for the remaining Y candidate slots according to ty'-k×PreserveSL, where ty'SL is a slot belong to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection. - UE performs CPS starts from M logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. - By default, M is 31 unless (pre-)configured with another value.

For the SL DRX impact on resource selection, option 2 is selected among the three options in the working assumption from RAN1#106bis-e.

Working Assumption (RAN1#106bis-e) When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, a restriction is applied in PHY layer so that at least a subset of candidate resources reported to MAC layer is located within the indicated active time of the RX UE. The following options will be further discussed in RAN1 to restrict resources for candidate resource selection taking into account the indicated active time from MAC layer: - Option 1: PHY layer selects and reports candidate resources only within the indicated active time of the RX UE - Option 2: PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE - FFS: Details on when the number of subsets of candidate resource is less than the threshold - FFS: The subset of candidate resource outside of the active time should consider each inactive time period - FFS: UE selection of resource selection window to overlap with indicated RX UE active time - FFS: Whether it is up to UE implementation to report candidate resources only within the indicated active time of the RX UE - Option 3: PHY layer selects and reports an additional candidate resource set of candidate resources within the indicated active time of the RX UE

In this contribution, we will share some further considerations on the remaining issues or details of PBPS and CPS. Meanwhile, we also would like to further discuss the impact of SL DRX on sensing and resource selection procedure in this contribution, to resolve some FFSs left over from the agreements of the previous meetings.

Discussion on partial sensing schemes

Remaining details on periodic-based partial sensing

The main purpose of periodic-based partial sensing design is to eliminate interference from other UEs which performing periodic transmission, this is like the mechanism in LTE V2X. Thus, it is agreed that the LTE V2X partial sensing mechanism should be a baseline for periodic-based partial sensing design in Rel-17 sidelink, which can reduce the specification efforts. The enhancement work may only need to be deployed when some features mentioned above cannot be supported by the existing mechanism.

In Rel-14 V2X, the maximum HARQ transmission number is 2, whereas in Rel-16 V2X, this number has been extended to 32. If Rel-17 P-UEs also want to support such a larger HARQ transmission number, the issue on partial sensing may arise that, the number of determined candidate slots in the selection window should ensure the target HARQ transmission number. Even though the HARQ transmission number is not noticed to physical layer explicitly, some implicit mapping rule can be made b/w this number and priority value, and/or CBR range, based on implementation. Therefore, we propose that the UE determine the minimum value for Y based on the priority and/or CBR value, from a range of values (pre-)configured in a resource pool.

When performing periodic-based partial sensing, UE should determine the minimum value of “Y” based on the priority and/or CBR value, from a range of values (pre-)configured in a resource pool.

Furthermore, when PSFCH is configured for the resource pool, a P-UE should also ensure the HARQ RTT related minimum time gap between any two selected resources of a TB. This timing restriction may have some impacts on the candidate slots determined in the partial sensing procedure. As illustrated in Figure 1, the P-UE selects 9 consecutive slots as its candidate slots. Since a HARQ RTT minimum time gap needs to be guaranteed for HARQ-ACK feedback, some slots (the slots colored as yellow) are not usable. In this case, the actual number of candidate slots is reduced to nearly half in comparison with the original determined number. This results in the number of candidate resources to be limited to a half, accordingly, after the step-1 in the resource selection procedure. As a result, the reliability performance may be degraded due to the limited number of candidate resources. A simple solution to address this issue is to determine a set of candidate slots for each transmission opportunity, with a minimum time gap preserved between any two sets of candidate slots. Note that, the minimum time gap is larger than or equal to the HARQ RTT minimum time gap.

Figure 1: An example of a reduction in candidate slots due to HARQ RTT relate timing restriction.

When PSFCH is configured for the resource pool, the impact of the HARQ RTT related timing restriction should also be considered when UE determines the “Y” candidate slots.

Contiguous partial sensing

For CPS monitoring window, a point that needs to be further discussed is the value of M for aperiodic transmission (Prsvp_TX=0). The functionality of CPS monitoring window is to detect aperiodic reservation of other UEs. It should offer enough sensing time to acquire all aperiodic reservation information. Thus, M should be 31 by default to ensure sufficient sensing results regardless of the traffic type (periodic or aperiodic) of the transmission that triggers the resource (re)selection procedure. Moreover, for aperiodic transmission, it is not justified that M needs to be (pre)configured based on priority. The (pre)configuration of M can be the same as the periodic transmission case.

- When UE performs at least contiguous partial sensing for aperiodic transmission (Prsvp_TX=0), for CPS monitoring window, by default, M is 31 unless (pre-)configured.

Re-evaluation/pre-emption

In the agreements from last meeting, there is an FFS: “whether/how to handle the case when number of the remaining Y candidate slots is less than Ymin”. In our view, when this case happens, in order to ensure sufficient randomness during resource selection, additional slots with incomplete sensing results can be included in the candidate resource set. For example, maybe only CPS can be done for these additional slots due to the time limitation, but there is no room to perform PBPS for these slots. In such case, we do not prefer to do random selection instead because for the resource reselection triggered by re-evaluation/pre-emption, random selection has higher possibility to select a bad resource again, which will make re-evaluation and pre-emption meaningless.

When number of the remaining Y candidate slots is less than Ymin, additional slots with incomplete sensing results can be included in the candidate resource set.

- E.g., these slots may only have the corresponding CPS sensing results.

  - Resource selection

For the resource selection based on full sensing, a received SCI can be assumed to reserve Q (Q>1) resources within the next Q periodicities. A parameter Tscal is used to determine the Q value. More specifically, Q=TscalPrsvp_RX if Prsvp_RX< Tscal. Here, Prsvp_RX is the periodicity indicated by the SCI and Tscal equals to the selection window size T2, i.e., the interval between slot n (the triggering slot) and slot n+T2 (the last slot of the selection window). Similarly, partial sensing also needs the Q value (Q>1) to determine how many resources have been reserved in the resource selection. Different from full sensing, partial sensing slots can be after slot n. Therefore, it is not appropriate to set  Tscal=T2 for partial sensing. For partial sensing, the last slot n+TB of the contiguous sensing window is the last opportunity where a UE can perform sensing, and the last slot ty'SL of the selected Y slots is the last slot where the UE can perform resource selection. Then, it would be sufficient to just avoid collisions within the interval between slot n+TB and slot ty'SL no matter whether partial sensing slots are before slot n or not. To this end, Tscal can be set to equal to the aforementioned interval, i.e., Tscal=ty'SL-n-TB. An example is provided in Figure 2 to show the definition of Tscal.

More complete resource selection procedures as illustrated below. These procedures at least apply to the situation where a UE performs resource selection for periodic transmission based on contiguous partial sensing and periodic-based partial sensing, and only the most recent sensing occasion for a given periodicity is monitored. The procedures to utilize the aforementioned Tscal are as follows.

The UE shall exclude any candidate single-slot resource Rx,y if it meets the following conditions:

- the measured RSRP is higher than the threshold, and

- the SCI received in slot tmSL or the same SCI which is assumed to be received in slot(s) tm+q×Prsvp_RXSL determines the set of resources which overlaps with Rx,y+j×Prsvp_TX for q=1, 2, …, Q and j=0, 1, …, Cresel-1. If Prsvp_RX< Tscal and n'-m≤Prsvp_RX where tn'SL=n+TB, then Q=TscalPrsvp_RX. Otherwise, Q=1.

Figure 2: An example for re-defining Tscal.

- When a UE performs resource selection for periodic transmission based on contiguous partial sensing and periodic-based partial sensing where only the most recent sensing occasion for a given periodicity is monitored, instead of using Tscal=T2, the parameter Tscal=ty'SL-n-TB is used in the resource selection. Here, slot ty'SL is the last slot of the Y slots, and slot n+TB is the last slot of the contiguous partial sensing window. More specifically, the UE shall exclude any candidate single-slot resource Rx,y if it meets the following conditions:

- the measured RSRP is higher than the threshold, and

- the SCI received in slot tmSL or the same SCI which is assumed to be received in slot(s) tm+q×Prsvp_RXSL determines the set of resources which overlaps with Rx,y+j×Prsvp_TX for q=1, 2, …, Q and j=0, 1, …, Cresel-1. If Prsvp_RX< Tscal and n'-m≤Prsvp_RX where tn'SL=n+TB, then Q=TscalPrsvp_RX. Otherwise, Q=1.

Discussion on the impact of SL DRX

According to the WID, SL DRX is supported as a power saving mechanism in Rel-17. During the discussion of recent RAN2 meetings, some progress has been made on it. A LS [1] has been sent to RAN1 after RAN2#112-e meeting to notice RAN1 there may be some impacts on RAN1 procedure after SL DRX is introduced. Thus, the impacts of sidelink DRX on some RAN1 features, e.g., sensing and resource selection, need to be discussed in RAN1. We will discuss the impacts of SL DRX on partial sensing and resource selection, respectively, in this section.

The impact of SL DRX on partial sensing

As agreed in RAN1#106-e meeting, sensing is permitted during SL DRX inactive time, but one leftover issue is whether sensing in SL DRX inactive time should be subject to specification, or up to UE implementation. During RAN1#106bis-e meeting, for sensing in the SL DRX inactive time of SL DRX, there are three options as below drafted by FL for down-selection, but no conclusion is reached in the end of the meeting.

- Option 1: UE performing PSCCH reception and RSRP measurement for sensing based on defined rules / processes in the specification regardless of SL DRX active and inactive time of the UE. - Option 2: UE performing PSCCH reception and RSRP measurement for sensing during its SL DRX inactive time according to one or more specified rules / conditions (e.g., total number of sensing slots in DRX inactive time is greater than a threshold, using different set of (pre-)configured settings for sensing during SL DRX inactive time, only the most recent PSO is monitored, only after resource (re)selection trigger, etc.) - Option 3: Up to UE implementation

In our view, firstly, it is not good to leave the sensing operation during the SL DRX inactive time to implementation. From the system point of view, this will result in inability to control the balance between PRR performance and power saving. Secondly, we think option 2 is better than option 1 because it can better balance reliability and power saving gain.

Sensing in SL DRX inactive time should be subject to specification.

UE performing PSCCH reception and RSRP measurement for sensing during its SL DRX inactive time according to one or more specified rules / conditions.

If some rules and conditions need to be defined for sensing during SL DRX inactive time, it is necessary to consider the specification workload which should not be increased too much. According to section 2, there are two types of partial sensing which has been approved in Rel-17. Therefore, we would like to discuss the specification impact of sensing during SL DRX inactive time separately for each type of partial sensing as follow.

The specification impact on periodic-based partial sensing (PBPS)

Refer to the agreement in last meeting, the most recent one or two sensing occasions need to be monitored for PBPS. Of course, a UE can perform sensing during the SL DRX active time, then if the periodic-based sensing occasions can align with the SL DRX active time as much as possible, the number of monitoring slot in inactive time would be rare and the power efficiency could be improved.

An example is shown in Figure 3, for a candidate slot ym and a given reservation periodicity P1, both the most recent two sensing occasions are configured to be monitored in the resource pool. For the corresponding most recent sensing occasion of the given reservation periodicity P1, it does not overlap with SL DRX active time of the Rx UE, then addition power consumption for sensing in the corresponding slot will be required. However, for the corresponding monitoring slot of the last periodic sensing occasion prior to the most recent one for the given reservation periodicity, it overlaps with SL DRX active time and no additional power consumption for sensing is needed. In this case, we propose that the sensing of the most recent occasion can be skipped, so that it can provide better balance between reliability and power saving. In another example as Figure 4, neither of these two sensing occasions overlap with the SL DRX active time of the Rx UE. In this case, the UE is allowed to only monitor the most recent one for power saving and ensure the sensing accuracy at the same time.

Figure 3: An example where only one sensing occasion overlaps with SL DRX active time.

Figure 4: An example where both sensing occasions do not overlap with active time

When both the most recent two sensing occasions are configured to be monitored in the resource pool for PBPS and SL DRX is configured simultaneously,

- If only one sensing occasion overlaps with SL DRX active time, the monitoring of the other sensing occasion which overlaps with inactive time can be skipped;

- If both sensing occasions do not overlap with SL DRX active time, only the most recent sensing occasion is monitored for PBPS.

The specification impact on contiguous partial sensing (CPS)

- As mentioned in section 2.2, an additional continuous partial sensing before/after resource (re)selection trigger is agreed, which can be regarded as an approach to eliminate the interference from co-existing aperiodic transmission. From our point of view, continuous partial sensing (CPS) should be performed regardless of whether the corresponding monitoring slots overlap with SL DRX active time or not. The reason for doing this is, contiguous partial sensing needs to be able to obtain the sensing results in time, and the possibility of interference caused by aperiodic transmission is relatively higher.

For CPS, when SL DRX is configured, the corresponding slots shall be monitored regardless of whether they overlap with SL DRX active time or not.

The impact of SL DRX on resource selection

- Furthermore, it is too restrictive to limit all the selected resources within the current active time of the Rx UE and the congestion condition will be worse if such restrictions are adopted. Therefore, some enhancements should be done in both RAN1 and RAN2. One possible method is, only limiting initial transmission within the current active time of the Rx UE, and the subsequent (re)transmissions can be achieved since the active time can be further extended by the initial transmission. To make sure there are enough candidate resources for the resource selection of initial transmission, a standalone resource exclusion procedure can be done on the subset of the candidate resource, as in the agreed option 2, which is within the SL DRX active time of Rx UE. And the obtained resources can be reported to MAC layer separately for initial transmission resource selection. An example is shown in Figure 5. This operation can avoid the “not enough candidate resources” case in the first FFS in the WA from RAN1#107-e, as much as possible.

- In addition, resources before the current active time cannot be selected because these resources cannot be covered by the current/extended active time, that is, the start of the selection window needs to be postponed to the starting time of the current active time as shown in Figure 5. This operation can address the 3rd FFS in the WA from RAN1#107-e.

Figure 5: Standalone resource exclusion procedure for a subset of the candidate resource or a separate candidate resource set

If SL DRX is configured for the Rx UE, the starting time of the resource selection window of Tx UE should be aligned with the starting time of SL DRX active time of Rx UE.

A standalone resource exclusion procedure can be done on a subset of the candidate resource (i.e., in option 2) within the SL DRX active time of Rx UE.

- The resources obtained from the resource exclusion procedure can be reported to MAC layer separately for at least initial transmission resource selection.

- In the following, we would like to provide our system-level evaluation results to compare two potential options for resource selection, in the case where SL DRX is configured for Rx UE:

- Option A: Both initial transmission and retransmission are confined within Rx UE’s On-Duration.

- Option B: Only initial transmission is confined within Rx UE’s On-Duration (the mechanism in proposal 14).

- The following two figures are used for illustrating the above two options. In Figure 6 for showing option A, the selection windows of initial transmission and retransmission in the Tx UE are same and should be fully or partially aligned with the DRX On-Duration of the Rx UE depending on the packet arrival time. In Figure 7 for showing option B, only the selection window of initial transmission is fully or partially aligned with the DRX On-Duration, but the DRX configuration of the Rx UE does not limit the selection window of retransmission resource(s).

Figure 6: An example for the working mechanism of option A.

Figure 7: An example for the working mechanism of option B.

- The following Figure 8 and Table 1 have shown the PRR performance and power consumption comparison of the above two options, in which the system-level simulation results are provided in urban scenario. The simulation assumptions can refer to the Annex, in which 50% vehicle UEs transmit V2P periodic packets in the scenario and the remaining 50% vehicle UEs transmit V2V periodic packets as background interference. The PRR and power consumption calculation are based on TR 37.885 [2] and the RAN1 agreements on evaluation from previous RAN1 meetings. In the simulation, DRX periodicity is set as 100ms, while the SL DRX On-Duration in each DRX cycle is set as 10ms, 20ms and 30ms separately, and the result of “DRX not configured” case is also provided as a reference.

Figure 8: The PRR performance for Rx UE’s DRX On-Duration=30,20,10ms.

Table 1: Power Consumption Reduction ratio for Rx UE’s DRX On-Duration=30,20,10ms.

| On-Duration [ms] | Average relative power |  | Reduction ratio [%] |  |
| --- | --- | --- | --- | --- |
|  | Option A | Option B | Option A | Option B |
| No DRX | 31003234 |  | 0.00 |  |
| 30 | 8923231 | 10023298 | 71.22 | 67.67 |
| 20 | 5757060 | 7324634 | 81.43 | 76.37 |
| 10 | 2662008 | 4558702 | 91.41 | 85.30 |

- According to the simulation results in the above figure, it can be found that the PRR performance of option B is improved compared with option A. Since option B only confines the initial transmission of a TB within DRX On-Duration, the probability of collision can be reduced and then the PRR performance will be improved. As the UE-to-UE distance increases, the degree of the improvement becomes greater. As an example, for On-Duration=20ms and when the UE-to-UE distance is less than 200m, the increasing rate is small, e.g., it is nearly 2% when the distance=150m; but when the UE-to-UE distance is equal to or larger than 200m, PRR performance is improved by larger than 6%.

- It can also be found that if option B is adopted, a smaller On-Duration value can be set so that the PRR performance is not significantly degraded, meanwhile the power saving gain can be obtained. For example, the PRR performance of Option B with On-Duration=20ms, is very close to Option A with On-Duration=30ms, while still saving about 5.2% of power consumption.

- Based on the above analysis, Option B can be considered as a trade-off method b/w PRR performance and power consumption, since it can improve the power efficiency by setting a shorter DRX On-Duration, while maintaining the PRR performance is not significantly degraded.

- Compared with confining both initial transmission and retransmission within Rx UE’s On-Duration, confining only initial transmission within Rx UE’s On-Duration can improve the PRR performance when the DRX On-Duration value is same.

- Compared with confining both initial transmission and retransmission within Rx UE’s On-Duration, confining only initial transmission within Rx UE’s On-Duration can improve the power efficiency by setting a shorter DRX On-Duration, while maintaining a similar PRR performance.

- When a TX UE transmits to a RX UE configured with DRX, the resources selected by the TX UE should be within the active time (e.g., on duration timer or inactivity timer, or retransmission timer) at the RX UE. However, a selected resource can be dropped due to prioritization/congestion control or can be re-selected due to pre-emption/re-evaluation/dropping. Furthermore, the dropping or re-selection of a resource can result in that another resource is not within the active time anymore. One example is shown in Figure 9 where resource C and resource D are within the retransmission timers. If resource C is dropped, the associated retransmission timer will not be initiated at the RX UE. Therefore, resource D will not be within the active time anymore. As one option, resource D can be dropped too. As another option, resource D (resource C) can be re-selected. Similarly, if resource C is re-selected, it may happen that resource D is outside of the active time depending on where the re-selected resource C is. Anyway, if a selected resource is not within the active time anymore, dropping or re-selection at the TX UE can be triggered.

Figure 9: An example for dropping of resource #1 makes resource #2 outside of the active time.

Dropping or re-selection of resource(s) at the TX UE is triggered if dropping or re-selection of a resource results in that another resource is not within the active time of the RX UE anymore.

Conclusions

In this contribution, we have expressed our views on the resource allocation from the partial sensing and SL DRX perspective. The observations and proposals are summarized as follows.

- Compared with confining both initial transmission and retransmission within Rx UE’s On-Duration, confining only initial transmission within Rx UE’s On-Duration can improve the PRR performance when the DRX On-Duration value is same.

- Compared with confining both initial transmission and retransmission within Rx UE’s On-Duration, confining only initial transmission within Rx UE’s On-Duration can improve the power efficiency by setting a shorter DRX On-Duration, while maintaining a similar PRR performance.

- When performing periodic-based partial sensing, UE should determine the minimum value of “Y” based on the priority and/or CBR value, from a range of values (pre-)configured in a resource pool.

- When PSFCH is configured for the resource pool, the impact of the HARQ RTT related timing restriction should also be considered when UE determines the “Y” candidate slots.

- When UE performs at least contiguous partial sensing for aperiodic transmission (Prsvp_TX=0), for CPS monitoring window, by default, M is 31 unless (pre-)configured.

When number of the remaining Y candidate slots is less than Ymin, additional slots with incomplete sensing results can be included in the candidate resource set.

- E.g., these slots may only have the corresponding CPS sensing results.

- When a UE performs resource selection for periodic transmission based on contiguous partial sensing and periodic-based partial sensing where only the most recent sensing occasion for a given periodicity is monitored, instead of using Tscal=T2, the parameter Tscal=ty'SL-n-TB is used in the resource selection. Here, slot ty'SL is the last slot of the Y slots, and slot n+TB is the last slot of the contiguous partial sensing window. More specifically, the UE shall exclude any candidate single-slot resource Rx,y if it meets the following conditions:

- the measured RSRP is higher than the threshold, and

- the SCI received in slot tmSL or the same SCI which is assumed to be received in slot(s) tm+q×Prsvp_RXSL determines the set of resources which overlaps with Rx,y+j×Prsvp_TX for q=1, 2, …, Q and j=0, 1, …, Cresel-1. If Prsvp_RX< Tscal and n'-m≤Prsvp_RX where tn'SL=n+TB, then Q=TscalPrsvp_RX. Otherwise, Q=1.

Sensing in SL DRX inactive time should be subject to specification.

UE performing PSCCH reception and RSRP measurement for sensing during its SL DRX inactive time according to one or more specified rules / conditions.

When both the most recent two sensing occasions are configured to be monitored in the resource pool for PBPS and SL DRX is configured simultaneously,

- If only one sensing occasion overlaps with SL DRX active time, the monitoring of the other sensing occasion which overlaps with inactive time can be skipped;

- If both sensing occasions do not overlap with SL DRX active time, only the most recent sensing occasion is monitored for PBPS.

For CPS, when SL DRX is configured, the corresponding slots shall be monitored regardless of whether they overlap with SL DRX active time or not.

If SL DRX is configured for the Rx UE, the starting time of the resource selection window of Tx UE should be aligned with the starting time of SL DRX active time of Rx UE.

A standalone resource exclusion procedure can be done on a subset of the candidate resource (i.e., in option 2) within the SL DRX active time of Rx UE.

- The resources obtained from the resource exclusion procedure can be reported to MAC layer separately for at least initial transmission resource selection.

Dropping or re-selection of resource(s) at the TX UE is triggered if dropping or re-selection of a resource results in that another resource is not within the active time of the RX UE anymore.

References

- R1-2100021, “LS to RAN1 on SL DRX design”, RAN2.

- 3GPP TR 37.885 V15.3.0.

Annex

The system level simulation in Section 3.2 is performed based on the simulation assumptions, listed in Table 2.

Table 2: SLS simulation assumptions.

| Attributes | Values or Assumptions |
| --- | --- |
| Number of drops | 5 |
| Simulation length | 5000[slots](5s) + warmup(3000[slots]) |
| Scenario | base on Urban grid case of option A of 3GPP TR 37.885 V15.1.0 |
| Channel model | Pathloss：Table 6.2.1-1 of TR 37.885 Shadowing：STD 3dB, Decorrelation distance 25m Fast fading：Section 6.2.3 in TR 37.885 |
| Speed of vehicle | 3 km/h for P-UE and 60 km/h for V-UE |
| UE dropping | Option A (seed=1201: 145 V-UE, 145 P-UE) |
| Average number of P-UEs | 200 on average |
| Carrier frequency | 5.9[GHz] |
| Bandwidth | 20 [MHz] (100RBs, 1200subcarriers) |
| Subcarrier spacing | 15[kHz] |
| Slot length | 1[ms] (14symbols) |
| Transmission power | 23[dBm] |
| TX Antenna Configuration | 1 antenna |
| RX Configuration | 4 antennas with λ/2 spacing |
| Antenna pattern | Nothing |
| Antenna height | 1.6 [m] (option A, type 2) |
| Antenna gain | 3 [dBi] |
| Noise figure | 9 [dB] |
| Subchannel type | PSCCH+PSSCH scheme |
| Size of sub-channel | 25 RBs |
| Modulation and Code rate Error curve type of PSCCH | QPSK, Polar coding |
| Modulation and Code rate Error curve type of PSSCH | QPSK, LDPC 190byte: 25RB, R=0.339367(without PSFCH), 0.513699(with PSFCH) 300byte: 25RB, R=0.490196(without PSFCH), 0.742009(with PSFCH) |
| Traffic mode | Traffic: Model-1 (low traffic intensity) - traffic generate vehicle rate：50% -Inter-packet arrival time: V-UE 100 ms, P-UE -, -Delay budget: 100 ms |
| Resource selection scheme | - Mode-2 (with SCI decoded, PSSCH-RSRP) |
| Threshold for excluding SCI decoded resources | -128[dBm] |
| SL_RESOURCE_RESELECTION_COUNTER | 1. The counter decremented by one after every transmission 2. Resource reselection is triggered if the counter reaches to zero (probResourceKeep=0.8) |
| Repetition | Chase combining with the same number of sub-channels as initial Tx |
| HARQ Scheme | HARQ-ACK based retransmission |
| Number of retransmissions | 1 |
| Sensing Window Length | 1000 slots |
| Selection Window Length | 100 slots |
| Number of 1st Stage SCI resources | 3 symbols |
| DRX period | 100 slots |
| On-Duration | 10,20,30 slots |