v3GPP TSG RAN WG1 #107-bis-e	R1-2200164

e-Meeting, January 17 – 25, 2022

Agenda item:		8.8.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Approaches and solutions for Type A PUSCH repetitions for Msg3

Document for:		Discussion and Decision

# Introduction

In this contribution we express our views on the repetition indication for Msg3 re-transmission, with specific focus on which MCS indices can be signaled via the repurposed MCS field of DCI format 0_0 with CRC scrambled by TC-RNTI.

# Discussion

In RAN1 #107-e, the following was agreed

Agreement For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. - If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. - FFS: [8] MCS index to be used for Msg3 re-transmission

Additionally, it was agreed that the following higher layer parameter is used by NW to configure four candidate MCS indexes for PUSCH transmission scheduled by RAR UL grant.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value | Per (UE, Cell, TRP…) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Type A PUSCH repetitions for Msg3 | mcs-Msg3Repetition | new | Configure four candidate MCS indexes for PUSCH transmission scheduled by RAR UL grant. | SEQUENCE (SIZE (4)) OF INTEGER (0..15) | {0, 1, 2, 3} | in SIB1 | Cell-specific | 38,331 |

Based on the above, the missing aspect to complete the design of the feature is related to the MCS indices for PUSCH transmission scheduled by DCI 0_0 with CRC scrambled with TC-RNTI.

According to existing agreements, up to 8 MCS indices can be indicated via the 3 bits of the re-purposed MCS field of the DCI format 0_0 with CRC scrambled by TC-RNTI dedicated to MCS index indication. In this context, the need of supporting MCS indices larger than MCS#7 is unclear, as shown in our companion paper [2]. Indeed, larger MCS indices would not be suitable for coverage shortage scenarios. This would also yield overall simpler signalling structure for SIB1, given that no additional higher layer parameters to configure a set of 8 cell specific MCS indices would be needed (differently for the UL RAR case).

At the same time, it could be argued that MCS indices from 0 to 7 are associated to the same modulation order as MCS#8 and MCS#9, i.e., 2, as shown in the table below.

| MCS Index IMCS | Modulation Order Qm | Target code Rate x [1024] R | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |

While the fact that MCS#8 and MCS#9 are associated to the same modulation as MCS indices from #0 to #7 is irrefutable, so it is the impact of the larger code rate, i.e., larger than 2/3, on the expected MCL/MPL, as shown in our companion paper [2]. Therein, in fact, results for MCS#8 (and small number of allocated PRBs) were almost always worse than their lower MCS index counterpart. In this sense, we are not sure that such a limited potential advantage may justify the introduction of a new higher layer parameter to allow the cell specific configuration of MCS indices for Msg3 repetitions scheduled via DCI 0_0 with CRC scrambled with TC-RNTI. Therefore, we suggest taking the simpler route and support MCS#0 to MCS#7 as the only possible configuration option in this case.

Proposal 1. For the indication of the MCS index for the re-transmission of Msg3 with repetitions over PUSCH, support the 8 indices from MCS#0 to MCS#7.

Proposal 2. No additional higher layer parameter is introduced in SIB1 to configure cell specific MCS indices for the re-transmission of Msg3 with repetitions over PUSCH.

# Conclusion

In this contribution, we expressed our views on the repetition indication for Msg3 re-transmission, with specific focus on the which MCS indices can be signaled via the repurposed MCS field of DCI 0_0 with CRC scrambled with TC-RNTI.

We made the following proposals:

Proposal 1. For the indication of the MCS index for the PUSCH re-transmission of Msg3 repetitions, support the 8 indices from MCS#0 to MCS#7.

Proposal 2. No additional higher layer parameter is introduced in SIB1 to configure cell specific MCS indices for the re-transmission of Msg3 with repetitions over PUSCH.

# References

- RP-202928	New WID on NR coverage enhancements, China Telecom (moderator), December 2020.

- R1-2200165	Performance of Type A PUSCH repetitions for Msg3 for different MCS index configurations, Nokia/NSB, January 2022.